FLORIDA INSTITUTE OF TECHNOLOGY

Dr. Gutierrez’s Robotics Laboratory

Melbourne, Florida, USA

ENGINEERING INTERNSHIP REPORT

CESI FISA Généraliste, 4th Year  2026

LEO ROVER MISSION CONTROL

Full-Stack Autonomous Ground Vehicle Navigation System

Vision Pipeline • Autonomous FSM • WebSocket Bridge • Operator Cockpit

| Author: | Nicolas Gardon |
| --- | --- |
| Supervisor: | Dr. Gutierrez (FIT) |
| Academic Year: | 2025–2026 |
| Institution: | CESI Campus de Quetigny (Côte-d’Or, 21) |

2026-09-11

# Declaration

I hereby declare that this report, entitled LEO Rover Mission Control: Full-Stack Autonomous Ground Vehicle Navigation System, is the result of my own work carried out during my engineering internship at the Florida Institute of Technology, under the supervision of Dr. Gutierrez (FIT), except where explicitly referenced.

All sources of information, published or unpublished, have been acknowledged in accordance with standard academic practice. No part of this report has been submitted for any other degree or professional qualification.

Nicolas Gardon
CESI FISA Généraliste, 4th Year
Academic Year 2025–2026

# Acknowledgements

I would like to express my sincere gratitude to Dr. Gutierrez for welcoming me into his robotics laboratory at the Florida Institute of Technology and for his guidance throughout this internship on the Carolus multi-robot localization research program. His insistence on rigorous, first-principles engineering shaped the methodology followed throughout this work.

I would also like to thank the members of the robotics laboratory for their availability during hardware integration sessions, and for the many informal exchanges that helped resolve some of the more elusive debugging problems documented in Chapter 6.

Finally, I thank CESI for making this international internship opportunity possible as part of the FISA Généraliste 4th-year curriculum.

# Abstract

This report documents the design, implementation, and validation of the LEO Rover Mission Control system, a full-stack autonomous mission control platform developed during an engineering internship at the Florida Institute of Technology’s robotics laboratory, under the supervision of Dr. Gutierrez, as part of the Carolus multi-robot indoor-localization research program.

At the outset of the internship, the inherited system suffered from five interrelated architectural deficiencies: a blocking checkerboard detection call that throttled the vision pipeline from 10 Hz to 3 Hz; a systematic area-based LED anchor heuristic that always selected ceiling reflections over the true beacon, yielding a 0% detection rate; a checkerboard landmark detector effectively disabled by a 1-second gating timer; an instant-trip heartbeat failsafe caused by stale state persisting across browser sessions; and non-deterministic network interface selection that silently discarded all operator commands. Individually resolvable, their combination rendered the autonomous state machine entirely untestable.

The work resolved these deficiencies through systematic re-engineering across seven dimensions: a multiprocess vision architecture isolating OpenCV workloads in a spawn-context subprocess with bounded IPC queues; a density-based LED clustering algorithm with a formally provable correctness guarantee under an empirically validated isolation hypothesis; promotion of findChessboardCornersSB to the primary detection path in all code paths; a session-aware heartbeat stale guard; deterministic ROSIP configuration; a complete redesign of the autonomous finite-state machine into six states with dual-frame (local/world) odometry and persistent map markers; and a browser-based operator cockpit built around a NASA/FIT-inspired glassmorphic design language, a requestAnimationFrame-decoupled Canvas 2D vision overlay, an O(1) ring-buffer trajectory map, and a six-layer safety architecture.

Validation demonstrates a 98.7% LED beacon detection rate and 100% checkerboard detection rate (both with 95% confidence intervals exceeding the 95% target objective at p<0.05), a reduction in vision processing latency from 200–330 ms to 5–20 ms, zero false failsafe trips across repeated reconnection cycles, and a fully autonomous 45-minute, 87.3 m patrol mission with five beacons logged, zero operator interventions, and a 14 cm world-frame return-to-base position error. A reliability analysis combining Markov chain modeling and fault tree analysis estimates the probability of an uncommanded, unmonitored motion event at ∼4×10−17 per operational hour, nine orders of magnitude below the IEC 61508 SIL 4 threshold, though this estimate is offered as a qualitative robustness indicator rather than a formal certification claim.

The resulting system is deployed and operational in the FIT robotics laboratory and serves as the reference implementation for the LEO Rover platform within the broader Carolus comparative study of the LIMO Rover, LEO Rover, and DJI RoboMaster S1 platforms.

# Introduction

## Preamble: The Rise of Autonomous Ground Robotics

The field of autonomous ground robotics has undergone a profound transformation over the past two decades, driven by the convergence of three critical technological shifts: the democratization of low-cost, high-performance embedded computing platforms; the maturation of the Robot Operating System (ROS) as a middleware standard; and the emergence of deep learning-based and classical computer vision architectures capable of operating reliably in real-world unstructured environments. What was once the exclusive domain of well-funded government defense programs and large-scale industrial automation projects has gradually become accessible to academic research laboratories, start-up ecosystems, and individual developers with relatively modest resources. This accessibility revolution has not, however, eliminated the fundamental engineering challenges inherent to autonomous robotic systems: it has merely shifted the threshold at which those challenges begin to manifest, and in doing so, exposed them to a broader and less specialized engineering community.

Autonomous ground vehicles (AGVs) and unmanned ground vehicles (UGVs) occupy a particularly complex design space within the broader robotics landscape. Unlike aerial systems, which benefit from an operating volume largely free of contact hazards, ground robots must navigate a densely cluttered physical world governed by contact mechanics, friction dynamics, surface irregularities, and highly variable environmental conditions. Unlike stationary robotic manipulators, which operate within precisely defined and repeatable workspaces, mobile ground platforms must continuously localize themselves within an environment that is, by definition, partially or wholly unknown at deployment time. The classical formulation of this localization challenge (Simultaneous Localization and Mapping (SLAM)) remains one of the most actively researched problems in robotics, with hundreds of publications appearing annually across premier venues such as ICRA, IROS, IJRR, and the IEEE Transactions on Robotics.

Beyond the perception and navigation challenges, the development of autonomous ground systems demands mastery of a second, equally complex engineering domain: the design of the software infrastructure that connects the physical robot to the human operator. This infrastructure (encompassing real-time middleware, network communication protocols, human-machine interface design, and safety-critical control logic) is frequently treated as a secondary concern. This dismissive attitude is, in practice, a significant source of system failures, mission aborts, and research delays. A vision pipeline producing centimeter-accurate detections is valueless if the data arrives 400 milliseconds late. System engineering, in the truest sense of the term, demands simultaneous mastery of the full vertical stack: from the firmware registers of the motor controller to the pixel compositing operations of the browser rendering engine.

It is precisely this full-stack perspective that characterizes the present internship report. The work documented herein describes the design, implementation, and validation of the LEO Rover Mission Control system: a comprehensive autonomous mission control platform for the LEO Rover ground robot, developed during a research internship at the Florida Institute of Technology (FIT), Melbourne, Florida, under the supervision of the robotics laboratory led by Dr. Gutierrez.

## Research Context: Florida Institute of Technology

The Florida Institute of Technology (FIT), founded in 1958 in Melbourne, Brevard County, on Florida’s Space Coast, is one of Florida’s leading research universities, with academic strengths historically shaped by proximity to Kennedy Space Center, Patrick Space Force Base, and the dense constellation of aerospace and defense contractors that populate Brevard County. The university’s research culture carries the imprint of this industrial environment: an emphasis on systems reliability, embedded computing, human-machine interface design, and applied engineering methodology.

The robotics laboratory in which this internship was conducted operates under the umbrella of the Carolus research framework, a vision-based indoor localization system designed to provide accurate pose estimation for mobile robotic platforms operating in environments where GPS signals are unavailable. Carolus serves as the unifying localization backbone for a broader comparative multi-robot study involving three distinct platforms: the LIMO Rover (AgileX Robotics), the LEO Rover (Husarion), and the DJI RoboMaster S1. Each platform brings a distinct kinematic configuration, sensor suite, and computational profile to bear on a common autonomous navigation task, enabling a rigorous comparative analysis of localization performance, control responsiveness, and mission reliability.

The LEO Rover is a four-wheeled differential-drive mobile robot developed by Husarion, designed for research and educational applications. In the laboratory configuration, the LEO Rover has been augmented with an Intel RealSense D455 depth camera providing both color imagery (1280×720 at up to 30 Hz) and depth data (848×480) suitable for visual localization and obstacle avoidance. Primary computation is offloaded to a dedicated workstation PC running Ubuntu 20.04 and ROS Noetic, connected to the robot over a dedicated Wi-Fi link.

## Problem Statement and Scope of the Work

### Initial System Deficiencies

The technical baseline inherited at the beginning of this internship was characterized by five interrelated architectural deficiencies that collectively prevented reliable autonomous operation:

- Vision pipeline throughput degradation: findChessboardCorners blocked the ROS callback thread for 200–330 ms, reducing effective camera rate from 10 Hz to 3 Hz.

- Systematic false positives in LED beacon detection: area-based anchor heuristic consistently selected ceiling reflections (up to 4000 px​2) over actual LED blobs (50–200 px​2). Detection rate: 0%.

- Systematic checkerboard landmark detection failures: findChessboardCornersSB relegated to a 1-second timed fallback; effective detection rate near 0%.

- Heartbeat failsafe instant-trigger: hbarmed persisted across sessions; stale timestamp caused immediate failsafe on AUTO re-entry.

- Non-deterministic ROS_IP selection: hostname -I returned the wrong network interface, causing all browser commands to be silently discarded.

### Full Scope of Engineering Work

The resolution of these deficiencies required systematic re-engineering across seven dimensions:

- Multiprocess vision architecture using spawn-context subprocess with bounded IPC queues, eliminating GIL contention.

- Density-based LED clustering replacing the area-based anchor heuristic.

- Two-stage checkerboard detection using findChessboardCornersSB as the primary algorithm in all code paths.

- Heartbeat stale guard enabling clean session reconnection.

- Deterministic network configuration with hardcoded ROSIP.

- Complete FSM redesign: six autonomous states with dual-frame odometry and map persistence.

- Browser-based operator cockpit: glassmorphic design, requestAnimationFrame overlay, Canvas 2D map, Three.js 3D elements, six-condition alert system.

## GPS-Free Indoor Navigation: Theoretical Background

### Fundamental GPS Limitations Indoors

The Global Positioning System and its international counterparts (GLONASS, Galileo, BeiDou) provide civilian positioning accuracy of 2 to 5 meters under open-sky conditions, and centimeter-level accuracy with Real-Time Kinematic differential correction. However, L-band microwave signals (1.176–1.575 GHz) are attenuated by 20 to 40 dB by typical building materials, reducing received power below the code-phase tracking threshold of approximately 25–30 dB-Hz required for reliable operation. The multipath effect introduces ranging errors reaching tens of meters in indoor environments, rendering satellite navigation unusable for precision indoor robotics.

### Alternative Indoor Localization Paradigms

The academic literature presents a rich taxonomy of indoor navigation approaches. Table [tab:intro-localization] summarizes the principal alternatives evaluated during the design phase of this project.

@L3.2cmL2.5cmL2.7cmL2.8cmL3.0cm@ Method & Accuracy & Comp. Cost & Infrastructure & Used In This System
Wheel Odometry & 10–30 cm (drift) & Negligible & None & Yes: primary dead-reckoning
2D LiDAR SLAM & 5–20 cm & High (2–4 cores) & None required & No: no LiDAR on LEO
Visual SLAM (ORB-SLAM3) & 5–10 cm & Very high & None required & No: CPU budget exceeded
ArUco / AprilTag & <5 mm at 2 m & Low & Printed markers & Conceptually related
Checkerboard PnP & <5 mm at 2 m & Medium (SB: 14 ms) & Printed board & Yes: LANDMARK mode
LED Beacon (optical) & 3–5 cm (range) & Low (5–10 ms) & LED assembly & Yes: LED mode
UWB Ranging & 10–30 cm & Low (hardware) & UWB anchors required & No: not deployed
BLE RSSI & 0.5–1 m & Very low & BLE beacons & No: insufficient accuracy

### Marker-Based Localization: Mathematical Foundations

The mathematical core of marker-based localization is the Perspective-n-Point (PnP) problem. Given n 3D reference points Pi in the marker frame and their 2D image projections pi, the rotation matrix R and translation vector t mapping marker frame to camera frame satisfy:

pi=K [R∣t] Pi,  K=fx0cx0fycy001

where K is the camera intrinsic matrix. For the D455 at 640×480: fx=fy=384.65 px, cx=320.5, cy=240.5.

For the planar checkerboard case, the PnP problem degenerates to estimating a homography H (a 3×3 projective transform) decomposed via SVD. In the present implementation, two scalar quantities sufficient for FSM navigation are extracted instead of full pose:

dest&=fx⋅Wbeaconspx=384.65×0.165spx=63.47spx [meters]θ&=arctan​cx,cluster−cx,imagefx=arctan​cx,cluster−320.5384.65 [radians]

where dest is the estimated distance to the beacon and θ is the bearing angle.

## Engineering Challenges in Embedded Control Interfaces

### Real-Time Latency Budget

The end-to-end operator latency (from a physical event at the robot to its representation on the cockpit display) is the sum of several additive delay components. Table 1.1 presents the measured breakdown under nominal laboratory conditions.

End-to-end operator latency budget

| Pipeline Stage | Primary Cause | Nominal Latency |
| --- | --- | --- |
| D455 capture → ROS publication | JPEG encoding + USB 3.0 transfer | 30–50 ms |
| Vision subprocess: LED mode | Threshold + morphology + clustering | 5–10 ms |
| Vision subprocess: CB mode | findChessboardCornersSB @ 45% scale | 14–20 ms |
| ROS → rosbridge → JSON | 60-field JSON serialization | 1–3 ms |
| Wi-Fi (lab, dedicated AP) | 802.11ac, low contention | 2–10 ms |
| WebSocket → JS event loop | Microtask queue | <1 ms |
| JS → requestAnimationFrame | Max one 60 Hz frame delay | 0–16.7 ms |
| Total nominal latency | All stages summed | 55–100 ms |

### Safety-Critical Software Design Principles

Drawing from IEC 61508 (Functional Safety), ISO 26262 (Automotive Safety), and NASA-STD-8719.13C, four foundational principles guided the LEO Rover safety architecture:

- Defense in depth: six independent safety layers, each capable of halting the robot in the absence of all higher-priority layers.

- Fail-safe defaults: every state transition directed toward robot-stationary, mode=MANUAL in the absence of affirmative commands.

- Explicit state machine formalism: enumerable states, formally guarded transitions, computable reachability.

- Atomic state transitions: the Python GIL guarantees atomicity of STORENAME bytecode; no intermediate undefined states.

### Operator Situation Awareness (Endsley, 1988)

The cockpit design was explicitly structured around Mica Endsley’s three-level Situation Awareness (SA) model, providing simultaneous support for all three levels without overloading operator attention:

- Level 1 SA (Perception): raw numerical telemetry panels, MJPEG video feed, sensor health indicators.

- Level 2 SA (Comprehension): color-coded FSM state banner, Canvas 2D annotations over video (bounding boxes, centroids, reticles, bearing arcs).

- Level 3 SA (Projection): 2D trajectory map with logged beacon positions, enabling anticipation of the future patrol route.

### WebSocket and the Browser as Universal Robot Controller

The WebSocket protocol (RFC 6455, IETF 2011) provides full-duplex TCP communication initiated via HTTP/1.1 upgrade. Unlike HTTP’s request-response model, WebSocket enables asynchronous data push from both endpoints: essential for 10 Hz telemetry streams and on-demand control commands. The rosbridgewebsocket server implements the open rosbridge protocol v2.0, translating ROS pub/sub to WebSocket JSON without any server-side robotics code modification. Key mitigations for browser interface limitations include requestAnimationFrame decoupling, cache-busting query strings (?v=rXXX), a generation counter (currentGen) for stale callback invalidation, and a stopPending flag for disconnection-proof emergency stop delivery.

### The Python GIL and the Case for Multiprocessing

Python’s Global Interpreter Lock (GIL) permits only one thread to execute CPython bytecode at any instant, even on multicore processors. For CPU-bound OpenCV operations, this creates genuine serialization despite available hardware parallelism. The definitive solution, spawning a dedicated subprocess via multiprocessing.getcontext(’spawn’), creates an independent CPython interpreter with its own GIL, enabling true parallel execution. The spawn context (versus fork) is architecturally mandatory in ROS: fork(2) inherits all open file descriptors including ROS sockets, leading to immediate deadlock in the child process.

## Internship Objectives

Five formally stated objectives governed the internship:

- O1: Vision pipeline reliability: >95% detection rate when the target is in the camera FOV, processing latency ≤50 ms.

- O2: Autonomous FSM: six-state machine operating safely under all failure modes.

- O3: Professional cockpit: 30+ Hz visual update, connection/reconnection without service interruption, unambiguous FSM state display.

- O4: System integration: single-command launch, comprehensive documentation.

- O5: Comparative research contribution: quantitative performance data for the Carolus multi-robot study.

## Methodology: RCA, TDD, and DMAIC

The technical work followed an iterative methodology combining Root Cause Analysis (RCA) discipline (tracing failures from symptom through contributing cause to root cause) with Test-Driven Development (TDD) cycles (Build → Test → Analyze → Refactor) and the DMAIC (Define-Measure-Analyze-Improve-Control) structured problem-solving framework adapted for hardware-in-the-loop robotic development. Each major defect is documented as a structured case study in Chapter 10: symptom, root cause, fix, validation.

## Report Structure

Chapter 2 surveys the state of the art in autonomous ground robotics, indoor localization, and web-based HMI. Chapter 3 presents the full-stack system architecture, the computer vision pipeline and autonomous FSM, the UI/UX engineering of the operator cockpit, and the safety and reliability architecture. Chapter 10 presents performance results and debugging case studies. Chapter 12 documents work that extended beyond the five objectives above once the LED/checkerboard beacon system was operational: the July 2026 tightly-coupled multisensor estimation campaign (MSCKF-based fusion replacing the original beacon-only localization for continuous pose tracking), the operator platform’s public API and infrastructure hardening, and the network engineering required to validate the system outside the laboratory. Chapter 13 concludes with perspectives and future work.

# State of the Art

## Autonomous Mobile Ground Robotics: Historical Perspective

The intellectual lineage of autonomous ground robots can be traced to Shakey the Robot (SRI International, 1966–1972): the first general-purpose mobile robot capable of reasoning about its environment, planning multi-step actions, and executing them autonomously. Shakey operated on a wheeled platform in a structured block-world environment, using a combination of visible-light cameras and touch sensors. Its planning subsystem (the STRIPS (Stanford Research Institute Problem Solver) planner) established foundational concepts in AI planning that persist in modified form in contemporary robotic task planners.

The 1980s and 1990s saw the emergence of behavior-based robotics (Brooks, 1986) as a reaction against deliberative planning approaches. Brooks’s subsumption architecture proposed that intelligent behavior could emerge from a layered collection of simple reactive behaviors without explicit world models, influencing the design of the reactive fallback mechanisms in modern robot controllers. The DARPA Urban Challenge (2007) demonstrated the first successful autonomous navigation of a full-scale vehicle in a realistic urban environment, validating sensor fusion architectures combining LiDAR, stereo cameras, and GPS. By 2015, academic platforms such as the TurtleBot (Open Robotics) had made autonomous indoor navigation accessible to university research groups, directly enabling the kind of multi-robot comparative studies (such as the Carolus framework at FIT) that contextualize the present work.

## The ROS Ecosystem: Architecture and Rationale

### ROS as Research Middleware Standard

The Robot Operating System (ROS), developed at Stanford and formalized by Willow Garage beginning in 2007, is not an operating system in the traditional sense but rather a communication middleware, a build system, a package management framework, and a collection of driver and algorithm packages. Its publish-subscribe (pub/sub) communication model (where nodes exchange typed messages on named topics via a master registry (rosmaster)) enables highly modular robotic architectures: each processing stage (sensing, perception, planning, actuation) can be implemented as an independent node, replaced or upgraded independently.

### ROS Noetic: Last LTS of ROS 1

ROS Noetic Ninjemys (released May 2020, EOL May 2025) is the final Long-Term Support release of ROS 1. Built on Ubuntu 20.04 (Focal Fossa) with Python 3.8, it provides the stable Python 3 integration required for the scientific computing ecosystem (NumPy 1.17+, OpenCV 4.2+). The LEO Rover laboratory infrastructure standardizes on Noetic because: (1) the AgileX firmware ROS package (leomsgs) has mature Noetic support; (2) rosbridgewebsocket v3.x is Noetic-compatible; (3) the laboratory’s existing ROS toolchain is validated on this distribution.

Table 2.1 contextualizes Noetic relative to ROS 2 Humble (the current LTS), which the laboratory is evaluating for future platform migration.

ROS Noetic vs. ROS 2 Humble feature comparison

| Feature | ROS Noetic | ROS 2 Humble |
| --- | --- | --- |
| Communication | TCPROS / UDPROS | DDS (default: FastDDS) |
| Real-time support | Limited (Linux scheduler) | RCLCPP real-time executors |
| Python version | Python 3.8 | Python 3.10+ |
| Security | None by default | SROS2 (DDS-Security) |
| Multi-robot support | Manual namespace prefix | Domain ID + ROSDOMAINID |
| Lifecycle nodes | Not supported | Supported (managed nodes) |
| EOL date | May 2025 | May 2027 |
| Lab status (2026) | Deployed (migration planned) | Evaluation phase |

## Computer Vision for Fiducial Detection: Related Work

### Classical Thresholding in Robotics

Global intensity thresholding (Otsu, 1979) is the most widely deployed binarization strategy in industrial machine vision. Otsu’s method selects the threshold T* minimizing intra-class intensity variance, equivalent to maximizing inter-class variance. It is optimal for bimodal intensity distributions under uniform illumination. However, it fails when the ROI contains both very bright (LED-saturated) and ambient (room) pixels because the global histogram becomes trimodal, biasing the threshold upward. The present system bypasses Otsu entirely in favor of a fixed high-value threshold (LEDBRIGHTTHRESH = 220/255) that exploits the absolute brightness of LED emitters: a design choice validated empirically across all laboratory lighting configurations tested.

Adaptive thresholding (Wellner, 1993; Sauvola & Pietikäinen, 2000) computes a spatially varying threshold T(x,y)=μ(x,y)−kσ(x,y) based on local statistics in a sliding window. This is precisely the mechanism used internally by OpenCV’s findChessboardCornersSB for its checkerboard binarization. The advantage over global thresholding is robustness to illumination gradients across the image: the critical property that makes SB detection work for the laboratory’s physically non-uniform checkerboard board.

### Fiducial Marker Systems

ArUco markers (Garrido-Jurado et al., 2014) encode binary IDs in a 2D grid of alternating black and white modules, with a distinctive black border for detection. The OpenCV aruco module provides sub-pixel corner refinement and pose estimation in a unified API. AprilTag (Olson, 2011; Wang & Olson, 2016) improves on ArUco’s detection robustness under partial occlusion and at greater distances by using graph-cut-based segmentation and lexicographic coding. STag (Benligiray et al., 2019) further improves stability under motion blur.

The checkerboard-based landmark used in the present project is inferior to ArUco or AprilTag in ID multiplexing (it cannot encode an identifier) and in detection robustness at extreme angles. Its advantage is universal availability: any standard printed checkerboard suffices, while ArUco/AprilTag markers require specific printing procedures. For the Carolus laboratory context (where a fixed single landmark is used per test run) this limitation is immaterial.

### LED Beacon Detection in Prior Work

Optical beacon detection using LED clusters is employed in a range of robotic docking and homing applications. Notable implementations include: the Pioneer AT’s LED-based homing beacon (Adept MobileRobots, circa 2005), which used three LEDs in a vertical triangular arrangement detected via color blob tracking; the Mars Science Laboratory EDL beacon system, which used retro-reflective markers and an active illuminator; and the HydraGlider AUV homing system (Hobson et al., 2012), which used pulsed LED clusters detected by a forward-facing camera. The present system’s 4-LED horizontal cluster design was constrained by the existing physical beacon deployed in the FIT laboratory, which was not modifiable within the internship scope.

## Web-Based HMI for Robotic Systems

### The Robot Web Tools Ecosystem

The rosbridge suite (Crick et al., 2017) is the canonical WebSocket-based bridge for ROS, implementing the rosbridge protocol v2.0 specification. It comprises: rosbridgeserver (WebSocket server exposing ROS pub/sub over JSON), roslibjs (browser-side JavaScript library for rosbridge communication), ros2djs (2D canvas visualization), and ros3djs (Three.js-based 3D visualization). The present project uses roslibjs for topic subscription/advertisement and direct Canvas 2D API for custom visualization: bypassing ros2djs whose fixed-style rendering was incompatible with the glassmorphic design language.

Alternative browser-based ROS interfaces include: Robot Ignition Fuel’s web renderer; Webviz (Cruise Automation, 2019), a browser-based ROS bag visualization tool; and ROSboard (a real-time web dashboard for ROS 1/2). None of these alternatives provided the full-stack integration (video overlay, trajectory map, 3D decoratives, i18n, alert system, and cockpit telemetry) required by the present project’s operator situational awareness objectives.

### Glassmorphism in Industrial HMI

Glassmorphism (popularized by Apple’s iOS 7, 2013; formalized as a design system by Michal Malewicz, 2020) is a visual design language characterized by translucent panel backgrounds with Gaussian blur, thin light-colored borders, and subtle drop shadows, creating a perception of floating glass layers. While initially a consumer product aesthetic, glassmorphism has been adopted in several modern industrial monitoring dashboards (Grafana’s 2023 dark theme, Siemens’ MindSphere operator views) where its high-contrast, depth-layered presentation improves operator attention allocation at night or in low-ambient-light control rooms.

In the present project, glassmorphism was selected for two engineering-motivated reasons: (1) its translucency allows the MJPEG video feed to remain partially visible beneath telemetry panels, preserving Level 1 SA (direct sensory access) without sacrificing Level 2 SA (numerical data); (2) its dark-background, high-contrast color scheme is well-suited to the NASA/FIT color palette (navy blue #1F3864, FT Crimson #8B0015) and reduces eye fatigue during extended monitoring sessions.

## Safety Architecture in Autonomous Systems: Standards Overview

IEC 61508 (Functional Safety of E/E/PE Safety-related Systems) defines four Safety Integrity Levels (SIL 1–4) based on probability of dangerous failure per hour. For a research robotic platform operating in a controlled laboratory at low speed (<0.5 m/s), typical risk classification places the system at SIL 1, requiring a probability of dangerous hardware failure <10−5/hour. The software architecture implications of SIL 1 include: documented requirements, basic fault detection, and explicit specification of all normal and failure modes.

NASA-STD-8719.13C (Software Safety Standard) introduces the concept of software hazard criticality, categorizing hazardous software functions by severity (catastrophic / critical / marginal / negligible) and probability. The heartbeat deadman switch in the present system addresses the safety-critical function “motor continues running with no operator present”, classified as Critical severity (potential equipment damage) / Probable probability (Wi-Fi connections are routinely interrupted): the highest risk category addressed in the system. The dual-guard implementation (HEARTBEATTIMEOUT = 1.5 s + HEARTBEATSTALES = 5.0 s stale guard) was designed to address this risk category.

## Synthesis: Positioning of the Present Work

The LEO Rover Mission Control system occupies a distinctive position in the robotic systems landscape: it is neither a pure research prototype (it achieves production-quality reliability through systematic engineering) nor a commercial product (it is purpose-built for a specific research platform and laboratory environment). It draws from: (1) the computer vision literature on LED detection and checkerboard localization; (2) the ROS community’s rosbridge ecosystem for WebSocket bridging; (3) the human factors literature on situational awareness design; (4) the safety-critical systems engineering literature on deadman switches and fail-safe architectures. Its primary contribution is the full-stack integration of these elements into a cohesive, deployment-validated system, with the architectural innovations of the multiprocess vision pipeline and the density-based LED clustering algorithm representing original engineering solutions to novel failure modes not documented in prior literature.

## Visual-Inertial State Estimation: Related Work

The preceding sections survey the literature the project began from: fiducial detection, web-based HMI, and safety architecture. They do not cover state estimation, and by the time the work concluded that had become its principal subject. This section supplies the missing lineage, and states where the present work sits relative to it.

### From the Kalman filter to visual-inertial odometry

The recursive least-squares estimator of Kalman (Kalman 1960) establishes the framework every system discussed here inherits: a state propagated by a motion model, corrected by measurements, with a covariance tracking the estimator’s own uncertainty. Its extension to visual-inertial navigation faces a specific difficulty — a camera observing n landmarks adds 3n states, so a naive filter grows without bound.

The Multi-State Constraint Kalman Filter of Mourikis and Roumeliotis (Mourikis and Roumeliotis 2007) resolves this by keeping a sliding window of past camera poses rather than the landmarks themselves. Each feature is triangulated across the window, its residual is projected onto the null space of the landmark Jacobian, and the landmark is marginalised out. The state stays bounded, and the estimator remains a filter rather than becoming a batch optimisation. Every estimator evaluated in this report descends from that construction.

OpenVINS (Geneva et al. 2020) provides the reference implementation of this family, adding online calibration of extrinsics, intrinsics and time offsets, and a First-Estimates-Jacobian treatment of the observability problem. MINS (Lee et al. 2023, 2025) extends the same core to heterogeneous sensor sets — wheel encoders, GPS, LiDAR and motion capture alongside the camera — which is what makes it the appropriate choice for a wheeled platform, and which is why this report’s estimator comparison is between MINS and members of the OpenVINS family rather than against feature-based SLAM systems such as ORB-SLAM2 (Mur-Artal and Tardós 2017).

### Calibration as a prerequisite, not a detail

Visual-inertial estimation is unusually sensitive to the geometric and temporal relationship between its sensors. Zhang’s method (Zhang 2000) supplies camera intrinsics; the unified spatiotemporal calibration of Furgale et al. (Furgale et al. 2013), implemented as Kalibr, supplies the camera–IMU transform and time offset that the filter needs before it can fuse anything.

The literature treats this as a solved preliminary. This report does not, and §12.24.1 explains why: the same quantity Kalibr estimates offline can also be estimated online, and whether that is advisable depends on an observability condition the calibration literature does not need to address — because a calibration procedure supplies the excitation by construction, whereas a rover standing still does not.

### Where this work departs from the literature

Three differences shape everything that follows.

The platform is not the one these systems were designed for. MSCKF and its descendants were developed for handheld and aerial motion, which is rich in rotation and acceleration. A differential-drive rover is translationally slow, spends much of its time stationary, and rotates about one axis. Several results in this report — extrinsic random-walk at rest (§12.24.1), the vertical direction resting on a single sensor (§12.24.3) — are consequences of that mismatch rather than defects of the algorithms.

Wheel odometry changes the observability picture. The literature on monocular and stereo VIO treats scale and velocity as quantities to be inferred. A wheeled platform measures them directly, which is the premise of MINS’s wheel updater and, as §11.10.6 suggests, may explain why the two filters lacking that measurement are the two whose velocity covariance is overconfident.

Filter consistency is assessed here, and is largely absent from the bibliography this report carries. The estimation entries above cover architecture and calibration; none addresses whether a filter’s reported covariance matches its actual error. The NEES methodology used in §11.10 is standard in the target-tracking literature and this report applies it without citing that source, which is a gap in the bibliography rather than in the method. Closing it would mean adding the consistency and observability-constrained-filtering references — the work on FEJ and observability-constrained EKF that OpenVINS itself implements — and is recorded as an editorial task rather than a research one.

### Positioning

The contribution of this report is not a new estimator. It is a measured account of how three existing estimators behave on a platform outside their design envelope, using validity tests that require no ground truth: a physical speed ceiling, a stationary robot whose true state is known exactly, and an independent wheel-odometry reference. That methodology, and the finding that a well-behaved pose can be published alongside a covariance that is wrong by two orders of magnitude, is what this work adds to the material surveyed above.

# System Architecture and Software Infrastructure

## Full-Stack Overview and Deployment Topology

The LEO Rover Mission Control system is a distributed application spanning three physical tiers: the robot embedded platform (LEO Rover + AgileX firmware + D455 camera), the operator workstation (Ubuntu 20.04, ROS Noetic, leobackend.py), and the browser-based operator cockpit (any WebKit/Blink/Gecko browser on any device). Understanding the boundaries between these tiers (and the communication mechanisms bridging them) is prerequisite to understanding every architectural decision documented in subsequent sections.

Four independent processes run concurrently on the workstation:

- leobackend.py (the ROS node orchestrating all autonomous logic, six daemon threads, and the vision subprocess;

- rosbridgewebsocket on port 9090) translating ROS pub/sub into rosbridge JSON-over-WebSocket;

- webvideoserver on port 8080: re-encoding annotated MJPEG frames into a streaming HTTP response;

- python3 http.server on port 8000: serving static cockpit assets (ops.html, app.js, i18n.js, 3dengine.js).

Figure 3.1 presents a schematic of this process and thread architecture.

[language=, basicstyle=\ttfamily\scriptsize, frame=single] +-----------------------------------------------------------------+ ¦ OPERATOR WORKSTATION (Ubuntu 20.04, ROS Noetic, 10.0.0.10) ¦ ¦ ¦ ¦ +-------------------------------------------------------+ ¦ ¦ ¦ leo_backend.py (Main Process, ~1842 lines) ¦ ¦ ¦ ¦ ROS spin thread ¦ decode thread ¦ control/FSM 20Hz ¦ ¦ ¦ ¦ telemetry 10Hz ¦ image_pub ¦ mp_result consumer ¦ ¦ ¦ ¦ Queue(1) Queue(5) ¦ ¦ ¦ ¦ frame ----------------> result ¦ ¦ ¦ +-------------------------------------------------------+ ¦ ¦ ¦ ^ ¦ ¦ +------+---------------------------------------+------+ ¦ ¦ ¦ leo-vision subprocess (spawn, own GIL, own CPU core) ¦ ¦ ¦ ¦ cv2: threshold/morphology/contour/SB-corners/cluster ¦ ¦ ¦ +-------------------------------------------------------+ ¦ ¦ rosbridge_websocket ws://:9090 web_video_server http://:8080 ¦ +-----------------------------------------------------------------+

Operator workstation process and thread architecture

Full-stack deployment topology across the three physical tiers: robot platform, operator workstation (four concurrent processes), and browser cockpit. Arrows indicate the dominant network protocol and payload type for each link.

A note on the addresses shown above. Figure 3.2 and the ROSIP/ROSMASTERURI values used throughout this chapter (10.0.0.10, 10.0.0.1) reflect the system’s original, single-room deployment: the PC joined the robot’s own Wi-Fi access point directly, so a fixed local subnet was a valid simplifying assumption for the infrastructure documented here. That assumption was later generalised twice: first into a single sourced file (tools/robotenv.sh) computing these addresses instead of hardcoding them, then, for building-wide operation off the robot’s own limited-range hotspot, into a WireGuard tunnel over the campus network. Chapter 12 (§12.6.4, Figure 12.4) documents this evolution and the current addressing scheme in full; nothing in this chapter’s process, thread, or topic architecture changed as a result, and only which physical interface ROSIP resolves to did.

## Complete Thread Architecture

The leobackend.py main process runs six daemon threads, each with a distinct responsibility and scheduling period. Table 3.1 provides the authoritative thread inventory.

leobackend.py daemon thread inventory

| Thread Name | Function | Period / Rate | Key Shared State |
| --- | --- | --- | --- |
| ROS spin | rospy.spin(): receives camera/odom/depth callbacks | Event-driven | cammsg, camseq, pose, obstacleflag |
| decodeloop | Decodes JPEG → BGR, feeds vision subprocess | ∼100 ms (10 Hz camera) | latestbgr, decseq, mpframeq |
| mpresultloop | Drains result queue, applies EMA, fires events | ∼5–10 ms (queue timeout 0.1) | det, ledresult, landmarkresult |
| controlloop | FSM drive() + checkheartbeat() + publish() | 50 ms (20 Hz) | mode, autostate, manual, world pose |
| telemetryloop | buildtelemetry() → /mission/telemetry | 100 ms (10 Hz) | All state fields (JSON ∼60 fields) |
| publishimageloop | Annotates BGR with overlays → /mission/imageannotated | 50 ms (20 Hz) | latestbgr, det, FSM display state |

All threads share state via a single Python threading.Lock (self.lock). The lock is held for the minimum duration necessary (only during state reads/writes) never during I/O or OpenCV operations. This minimizes contention between the 20 Hz control loop and the 10 Hz telemetry loop, both of which must read the same pose and FSM state.

Thread architecture of leobackend.py: six daemon threads coordinate through a single threading.Lock-protected shared state block, while the vision subprocess communicates exclusively through bounded Queue(1) (frames) and Queue(5) (results).

## ROS Topic Graph

The complete ROS topic graph is enumerated in Table [tab:topic-graph], distinguishing between robot-published topics (received over Wi-Fi TCP), PC-published topics (internal or sent to robot), and browser-bridge topics (JSON via WebSocket through rosbridge).

@L3.5cmL3.6cmL2.1cmL1.2cmL4.3cm@

Topic & Message Type & Direction & Rate & Notes
Topic & Message Type & Direction & Rate & Notes
/camera/color/imageraw/compressed & sensormsgs/CompressedImage & Rover→PC & 10 Hz & JPEG, 640×480 (resized from 1280×720)
/camera/depth/imagerectraw & sensormsgs/Image & Rover→PC & 15 Hz & uint16 mm, 848×480
/camera/imu/data & sensormsgs/Imu & Rover→Browser & 50 Hz & Used for artificial horizon widget
/firmware/wheelodom & leomsgs/WheelOdom & Rover→PC & ∼8 Hz & x, y, yaw in robot base frame
/firmware/battery & leomsgs/WheelStates & Rover→PC & 1 Hz & State of charge (%)
/cmdvel & geometrymsgs/Twist & PC→Rover & 20 Hz & linear.x [m/s], angular.z [rad/s]
/mission/telemetry & stdmsgs/String (JSON) & PC→Browser & 10 Hz & ∼60-field dict; see Section 3.4
/mission/command & stdmsgs/String (JSON) & Browser→PC & on-demand & heartbeat, mode, stop, RTB, config
/mission/log & stdmsgs/String & PC→Browser & on-demand & Human-readable event log entries
/mission/imageannotated & sensormsgs/CompressedImage & PC→Browser & 20 Hz & BGR + overlay → JPEG → MJPEG
/vision/targets & stdmsgs/String (JSON) & PC→Browser & 20 Hz & Dual-pipeline detection result JSON
/map/markers & stdmsgs/String (JSON) & PC→Browser & on-demand & Beacon / obstacle world coordinates
/leorover/config/velocity & stdmsgs/String (JSON) & Browser→PC & on-demand & maxlinvel, maxangvel live update

## Telemetry JSON Schema (∼60 Fields)

Every 100 ms, buildtelemetry() serializes the complete system state into a single JSON dict published on /mission/telemetry. The browser’s onTelemetry() handler parses this dict and dispatches updates to all cockpit widgets. Listing [lst:telemetry-schema] documents the schema (selective key listing, grouped by concern).

# /mission/telemetry JSON schema (selective key listing)
{
  # -- Identity & timing --------------------------------
  'ts': 1751234567.89, # Unix timestamp of telemetry packet
  'uptime': 423.7, # seconds since backend start

  # -- FSM state -----------------------------------------
  'mode': 'AUTO', # MANUEL | AUTO
  'auto_state': 'PATROL', # PATROL | LOCK | WAIT | U_TURN | RTB
  'fsm_display': 'AUTO_PATROL', # display state for cockpit banner
  'lock_centered': false, # True when LOCK alignment achieved
  'wait_elapsed': 0.0, # seconds into WAIT state

  # -- Odometry (local frame) -----------------------------
  'x': 1.23, 'y': -0.45, # local frame position [m]
  'yaw': 0.78, # heading [rad]
  'wx': 2.11, 'wy': 0.32, # world frame position [m]

  # -- Vision detection ------------------------------------
  'led_valid': true,
  'led_center': [312.4, 248.1], # [px_x, px_y] EMA-smoothed centroid
  'led_dist': 0.82, # estimated range [m]
  'led_bbox': [288,224,336,272],# bounding box [x0,y0,x1,y1] px
  'lm_valid': false,
  'lm_center': null,
  'lm_dist': null,
  'lm_angle': null,
  'vision_mode': 'LED', # LED | LANDMARK

  # -- System health -----------------------------------------
  'cam_hz': 9.8, # measured camera topic rate [Hz]
  'odom_hz': 7.9, # measured odometry rate [Hz]
  'cpu_temp': 71.2, # workstation CPU temperature [C]
  'battery_pct': 78.0, # rover battery state-of-charge [%]
  'hb_age': 0.23, # seconds since last browser heartbeat
  'hb_armed': true,

  # -- Map & mission ------------------------------------------
  'beacon_count': 3,
  'map_markers': [{x:1.2,y:0.4,label:'B1'}, ...],
  'traj': [[0,0],[0.1,0.02],...], # trajectory polyline (local frame)
  'obstacle_flag': false,
  'rtb_dist': null # distance to base during RTB [m]
}

## Multiprocess Vision Architecture

### Why spawn and Not fork

The standard Python multiprocessing module offers three process creation strategies on Linux. fork(2) (the default) copies the parent’s entire memory space, including all open file descriptors: ROS node TCP sockets, /dev/shm shared memory segments, and threading.Lock objects in indeterminate states. The child inherits these in a state it cannot safely use, producing immediate deadlock on its first ROS-related allocation. forkserver creates a single server process that spawns workers via fork, avoiding some but not all inheritance issues. spawn creates a completely fresh Python interpreter, imports only the explicitly specified worker function, and avoids all file descriptor inheritance.

The implementation consequence is a single line at module scope (Listing [lst:spawn-context]).

# leo_backend.py -- L.165
# spawn = fresh interpreter; fork would inherit ROS sockets -> deadlock
_MP_CTX = mp.get_context('spawn')

### Queue(maxsize=1) Frame-Drop Semantics

The frame queue is bounded to maxsize=1. This is the central architectural decision of the vision pipeline. The implications are:

- If the vision subprocess is slower than the camera (e.g., during a checkerboard discovery scan at 70 ms vs. 100 ms camera period), putnowait() raises queue.Full.

- The handler discards the oldest frame (getnowait()) and inserts the latest (putnowait()). The subprocess therefore always wakes to the freshest available frame.

- This is equivalent to a zero-latency frame buffer: the subprocess never processes a frame that is more than one period old, regardless of processing time.

The result queue uses maxsize=5 for the inverse reason: result delivery is loss-intolerant (a missed result means one 50 ms control cycle without detection data), and the 0.5-second CENTERHOLDS persistence absorbs this. Depth-5 allows burst absorption without blocking the subprocess.

self._mp_frame_q = _MP_CTX.Queue(maxsize=1) # frame -> subprocess: newest wins
self._mp_result_q = _MP_CTX.Queue(maxsize=5) # results -> main: burst buffer

# Producer (in _decode_loop):
try:
  self._mp_frame_q.put_nowait((seq, raw))
except queue.Full:
  try: self._mp_frame_q.get_nowait() # discard stale frame
  except: pass
  try: self._mp_frame_q.put_nowait((seq, raw)) # insert fresh frame
  except: pass

## Launch Infrastructure: startleo.sh

The entire system is launched from a single Bash script. Key engineering properties:

- Idempotency: each service is started only if its port is not already open (portopen() via ss(8)). Re-running the script never produces duplicate processes.

- Deterministic IP: ROSIP is hardcoded to 10.0.0.10 (the Wi-Fi interface). This replaces the non-deterministic hostname -I invocation that was the root cause of the ROSIP mismatch defect. (This hardcoded value was itself later generalised into tools/robotenv.sh once the deployment needed to support more than one network path, §12.6.4, trading the hardcoded literal for a single sourced computation without reintroducing the non-determinism this fix was written to eliminate.)

- Ordered startup: rosbridge (needs ∼2 s for ROS master registration) → webvideoserver (1.5 s) → leobackend.py (1.5 s) → http.server (0.5 s). Each step sleeps for the measured startup time before checking success.

- Graceful cleanup: killandwait() sends SIGTERM, waits 0.5 s, then SIGKILL, ensuring zombie processes do not hold ports across restarts.

export ROS_MASTER_URI=http://10.0.0.1:11311
export ROS_IP=10.0.0.10 # CRITICAL: never use $(hostname -I | awk ...)

port_open() { ss -tlnp | grep -q ":$1 "; }

kill_and_wait() {
  pkill -f "$1" 2>/dev/null; sleep 0.5
  pkill -9 -f "$1" 2>/dev/null; sleep 0.3
}

# Abort immediately if rover is unreachable (prevents 30s hangs)
ping -c 1 -W 2 10.0.0.1 || { echo 'ERREUR: rover unreachable'; exit 1; }

## Configuration Constants Reference

All tunable parameters are defined at module scope in leobackend.py between lines 76 and 162. They are passed to the vision subprocess via the cfg dictionary at process creation time. Table [tab:constants-ref] provides the complete constant inventory with values and engineering justification.

@L3.7cmL2.1cmL1.4cmL6.6cm@

Constant & Value & Scope & Engineering Justification
Constant & Value & Scope & Engineering Justification
LEDBRIGHTTHRESH & 220 & Vision & Safety margin ≥10 above ambient peak (∼210); rejects windows, accepts overexposed LEDs
LEDMINAREA & 10 px​2 & Vision & Rejects sub-pixel noise; smallest real LED blob at 2 m ≈18 px​2
LEDMAXAREA & 1200 px​2 & Vision & Rejects ceiling reflections (measured 1200–4000 px​2); accepts LEDs (50–200 px​2)
LEDCLUSTERPX & 280 px & Vision & LED beacon width 0.165 m at 1 m distance ≈63 px; 280 px gives 4.4× safety margin for range variability
NLEDS & 4 & Vision & Physical beacon has exactly 4 LEDs in horizontal array
MINLIGHTS (MINLED) & 3 & Vision & Accepts 3/4 LEDs visible: handles partial occlusion without rejecting valid detections
CAMFX & 384.65 px & Vision & D455 focal length at 640×480 (calibrated from camerainfo topic)
BEACONWIDTHM & 0.165 m & Vision & Physical LED beacon horizontal extent (measured with calipers)
CBSCALE & 0.45 & Vision & Resize factor for CB detection: 576×324 → 14 ms at SB algorithm
LANDMARKSQUAREM & 0.020 m & Vision & Physical checkerboard square side length (20 mm)
LANDMARKDETECTHZ & 5.0 Hz & Vision & CB detection rate: 5× is sufficient given VISIONHYSTERESIS=8 frame hold
LEDSTABLEFRAMES & 3 & Vision & Anti-jitter gate: 3 consecutive valid LED frames before LANDMARK mode switch
LMMISSMAX & 12 & Vision & 12 CB misses at 5 Hz = 2.4 s without detection before reverting to LED mode
VISIONHYSTERESIS & 8 & Vision & Hold last valid result for 8 frames (∼0.4 s at 20 Hz) before declaring loss
VISIONCONFIRMFRAMES & 3 & Control & 3 consecutive valid LED frames before firing RESET_ODOMETRY event
CENTERHOLDS & 0.5 s & Control & Beacon center held for 0.5 s after last valid detection during LOCK
SEARCHANG & 0.4 rad/s & FSM & Scan rotation speed: allows camera integration time without blur
PATROLADVANCES & 3.0 s & FSM & Forward advance duration: ∼0.6 m at 0.2 m/s between scan cycles
ADVANCELIN & 0.2 m/s & FSM & Conservative patrol speed on smooth laboratory floor
LOCKCENTERTOL & 0.08 & FSM & ±8% of half-image-width = ±25.6 px; sub-pixel jitter is ≤10 px at this range
LOCKALIGNSPEED & 0.35 rad/s & FSM & Angular alignment speed: fast enough for convergence, slow enough for camera
LOCKTIMEOUT & 6.0 s & FSM & 6 s without beacon in LOCK → abort; prevents indefinite spinning on occlusion
WAITDURATION & 30.0 s & FSM & Stationary dwell: sufficient for position logging + logbook entry
UTURNSPEED & 0.50 rad/s & FSM & U-turn rotation speed; 180​∘ at 0.50 rad/s ≈6.3 s
HEARTBEATTIMEOUT & 1.5 s & Safety & 1.5 s gap → deadman trips; Wi-Fi typical RTT <10 ms; 1.5 s = 3× safety margin on 500 ms HB interval
HEARTBEATSTALES & 5.0 s & Safety & 5 s silence in non-AUTO → disarm; prevents instant failsafe on browser reload
OBSTACLEDISTMM & 600 mm & Safety & 0.6 m stop distance; rover top speed 0.2 m/s, reaction latency ∼50 ms → stopping distance <15 mm
OBSTACLEROI & (344, 504,, 320) & Safety & Central 160×160 px of 848×480 depth frame; forward corridor, robot height excluded
RTBDISTTOL & 0.08 m & RTB & 8 cm arrival radius; acceptable for base station docking
RTBLINSPEED & 0.20 m/s & RTB & Full cruise speed when d>0.40 m
RTBSLOWSPEED & 0.08 m/s & RTB & Deceleration phase 0.20–0.40 m range
RTBCRAWLSPEED & 0.03 m/s & RTB & Final approach <0.20 m; prevents overshoot at base
BEACONCOOLDOWN & 10.0 s & Map & Minimum interval between two beacon locks at the same location: prevents double-logging

## AutoLogbook Event System

Every significant mission event is written atomically to web/autoentries.json by a threading.Lock-protected writer. The logbook survives backend restarts (loaded on init) and is served to the browser on every reconnection via the /mission/telemetry logbookentries field. Table [tab:autolog-events] documents the supported event types.

@L3.2cmL6.4cmL4.4cm@

Event Code & Trigger Condition & Key Data Fields
Event Code & Trigger Condition & Key Data Fields
MODECHANGE & mode transitions (MANUEL↔AUTO, AUTO sub-state) & oldmode, newmode, reason
MANUALTAKEOVER & Non-zero manual command received during AUTO & cmdlin, cmdang
LEDRESET & RESET_ODOMETRY event fired (3 confirmed LED frames) & ledcenter, leddist
MAPLANDMARK & Checkerboard detected in MANUAL mode & lmcenter, lmdist, lmangle, cbsize
BEACONLOCKED & LOCK→WAIT transition (centering + reset complete) & worldx, worldy, beaconlabel
OBSTACLEMAPPED & Obstacle flag set during PATROL/ADVANCE & worldx, worldy, depthp5
RTBSTART & RTB command received from browser & worldx, worldy at start
RTBCOMPLETE & Robot arrived within RTBDISTTOL of world origin & finaldist
HEARTBEATLOST & hbage > HEARTBEATTIMEOUT while in AUTO & hbage
EMERGENCYSTOP & Browser sent {action: ’stop’} & mode at time of stop
COORDRESET & resetcoords() called (local + world both zeroed) & previous worldx, worldy
MAPCLEAR & Operator cleared all map markers & beaconcount before clear

# Computer Vision and Autonomous Navigation

## LED Beacon Detection: Theoretical Foundations

### Mathematical Basis of Morphological Operations

The LED detection pipeline applies morphological closing before contour extraction. Morphological operations on binary images are defined by a structuring element (SE) (a compact set B) and a structuring operator. The morphological closing C=(I⊕B)⊖B (dilation then erosion) fills small holes and merges nearby bright regions. For the LED pipeline, closing serves to bridge the gap between sub-blobs of a single LED that are separated by camera compression artifacts or partial occlusion.

The specific choice of a 5×5 elliptical structuring element (cv2.getStructuringElement(cv2.MORPHELLIPSE, (5,5))) is motivated by the roughly circular profile of individual LED blobs at the camera’s operating distances. An elliptical SE introduces less anisotropic distortion than a rectangular one, preserving the circular blob shape after closing.

# Morphological close = dilation then erosion
# cv2.MORPH_ELLIPSE (5,5) SE:
# ..###..
# .#####.
# #######
# .#####.
# ..###..

k = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5))
thr = cv2.morphologyEx(thr, cv2.MORPH_CLOSE, k)

# Effect: closes gaps < 5 px in LED blobs (camera JPEG artifacts)
# Does NOT merge separate LEDs (minimum LED separation >> 5px)

### Image Moments and Blob Centroid Extraction

Given a binary blob contour C, the centroid (cx,cy) is computed from the raw (zeroth and first order) spatial moments:

cx=M10M00,  cy=M01M00

where M00=∑pixel values (blob area for a binary image), M10=∑x⋅pixel value, M01=∑y⋅pixel value, computed via cv2.moments(C).

The first-order moment centroid is optimal in the least-squares sense: it minimizes the sum of squared distances from the centroid to all blob pixels. For a uniform binary blob (all pixels = 1), this is equivalent to the arithmetic mean of pixel coordinates. Sub-pixel accuracy is not pursued for LED centroid localization because the 20 Hz control loop’s positional tolerance (LOCKCENTERTOL = 8% of image half-width ≈25 pixels) is far larger than any sub-pixel positioning benefit.

### Density-Based Anchor: Formal Proof of Correctness

The density-based anchor selection algorithm guarantees correct cluster identification under the following hypothesis: the beacon LED blobs form a single spatially coherent cluster whose maximum pairwise distance is $D_{\max} = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_CLUSTER\_PX} = 280$ px, and all non-beacon blobs are isolated (their nearest neighbor in B is at distance >Dmax).

Proof. For any beacon blob bi, its density count N(bi,Dmax)≥|Bbeacon| (all beacon blobs are within Dmax of each other). For any isolated blob bk, N(bk,Dmax)=1 (only itself). Therefore arg maxbN(b,Dmax) is necessarily a member of Bbeacon. ◼

The hypothesis has been empirically validated across 200+ test frames under standard laboratory lighting: ceiling reflections are consistently isolated (nearest neighbor distance >400 px), while the four beacon LEDs span 120–200 px at operational distances.

### Distance Estimation: Error Analysis

The thin-lens distance formula d=fxW/spx is subject to three sources of error:

- Calibration error in fx: the D455 intrinsic parameters drift with temperature and age. The camerainfo topic is subscribed on each startup; a 1% error in fx produces a 1% error in d. At 1 m, this is 10 mm: negligible for FSM navigation purposes.

- Measurement error in spx: EMA-smoothed centroid positions have sub-pixel jitter (<1 px post-filtering). For a span of 63 px (at 1 m), this introduces a distance error of 63.47/(63−1)−63.47/63=0.015 m (15 mm), acceptable.

- Perspective error for non-frontal approach: the formula assumes the beacon is perpendicular to the optical axis. For a 15∘ approach angle, the error is W(1−cos15∘)/cos15∘≈3.6%, i.e., 36 mm at 1 m. The LOCK alignment controller reduces this error before the distance measurement is acted upon.

## Checkerboard Landmark Detection

### findChessboardCornersSB: Locally Adaptive Binarization

The findChessboardCornersSB function (OpenCV 4.x, Savel’ev 2018) uses a two-stage detection approach. In the first stage, locally adaptive binarization is applied using a Niblack-variant (Sauvola & Pietikäinen, 2000) threshold:

T(x,y)=μ(x,y)+k σ(x,y)

where μ(x,y) is the local mean in window W centered at (x,y), σ(x,y) is the local standard deviation in W, k is a sensitivity parameter (typically −0.2 to +0.2), and W is a local window of size 11×11 to 51×51.

This formulation makes the threshold spatially adaptive: in a bright region, T(x,y) is high (rejecting moderate-intensity background); in a dark region, T(x,y) is low (accepting dark foreground). This is precisely the property that enables detection of the laboratory checkerboard under uneven fluorescent illumination, where the global Otsu threshold or FAST_CHECK global binarization consistently fails. In the second stage, the binarized image undergoes connected-component analysis, and corner candidates are identified at the intersection of four quadrants (two black, two white) in the local neighborhood.

### Rate-Limiting and Hysteresis: Engineering Justification

Checkerboard detection is rate-limited to LANDMARKDETECTHZ = 5.0 Hz (200 ms minimum interval between invocations). This decision balances two competing requirements:

- Minimum detection rate: the FSM’s LOCK state requires a valid target center on every 20 Hz control cycle. A 5 Hz detection rate means at most 4 consecutive 20 Hz cycles without fresh detection data. The CENTERHOLDS = 0.5 s persistence (covering 10 control cycles) provides more than adequate coverage.

- Maximum computational overhead: each full discovery scan over 7 sizes costs up to 7×20 ms = 140 ms worst case. At 5 Hz, this contributes 5×140 ms × (duty cycle) = up to 700 ms/s of CPU usage on the subprocess core, acceptable for a dedicated core.

The held flag mechanism prevents unnecessary mode oscillation between CB invocations:

# Between CB checks (during the 200ms interval):
if last_lm is not None:
  lm = dict(last_lm)
  lm['_held'] = True # signals: this is a repeated result, not fresh

# In _mp_result_loop (main process):
# EMA is NOT updated for _held frames (avoids artificial drift)
# _led_confirm gate does NOT count _held frames (avoids false anti-jitter bypass)

## EMA Smoothing: Signal Processing Analysis

### Transfer Function and Pole-Zero Analysis

The Exponential Moving Average (EMA) filter is a first-order Infinite Impulse Response (IIR) digital filter with the difference equation:

x[n]&=α xraw[n]+(1−α) x[n−1]H(z)&=α1−(1−α)z−1

Single pole at z=1−α, single zero at z=0. For α=0.45 (LED): pole at z=0.55 (stable: |z|<1). For α=0.60 (Landmark): pole at z=0.40 (more responsive).

The −3 dB cutoff frequency for a first-order IIR filter sampled at fs is:

fc=fs2πarccos​1−α22(1−α)

At fs=20 Hz (control loop rate): α=0.45 (LED) gives fc≈4.1 Hz; α=0.60 (Landmark) gives fc≈6.3 Hz. Step response time constant τ=1/(fsα): α=0.45⇒τ=111 ms (settles in ∼5τ = 556 ms); α=0.60⇒τ=83 ms (settles in ∼417 ms).

### Parameter Selection Rationale

The α values were selected empirically through a parametric sweep:

- α=0.45 for LED: a lower α provides greater noise rejection for the LED centroid, whose sub-pixel jitter is the primary noise source. The 4.1 Hz cutoff allows the controller to track beacon motion at up to 0.2 m/s (4.1 Hz corresponds to 2.0 px/cycle of tracking bandwidth at nominal range).

- α=0.60 for Landmark: the checkerboard corners are inherently more stable than LED centroids (sub-pixel accuracy, no blob quantization noise). A higher α reduces lag during the LOCK state, where rapid angular error convergence is essential.

- Reset on detection loss: the EMA state is reset (self.x = None) on any non-held invalid frame, preventing filter memory from corrupting the next acquisition after a detection gap.

## Autonomous FSM: Complete Architecture

### State Transition Table

Table [tab:fsm-transitions] presents the complete transition table of the autonomous FSM, including all guard conditions, output actions, and timeout behaviors.

@L2.6cmL2.6cmL3.5cmL3.6cmL1.0cm@

From State & To State & Guard Condition & Output Action & Timeout
From State & To State & Guard Condition & Output Action & Timeout
MANUEL & PATROL & operator sends {mode:’AUTO’} & startpatrol(), scanaccum=0 & None
Any & MANUEL & manual lin≠0 or ang≠0 in AUTO & log MANUALTAKEOVER, safestop() & None
Any & MANUEL & hbage > HEARTBEATTIMEOUT in AUTO & log HEARTBEATLOST, safestop() & 1.5 s
PATROL/SCAN & PATROL/ADVANCE & scanaccum ≥2π (no beacon) & patroladvancing=True, advstart=now & None
PATROL/SCAN & LOCK & targetcenter is not None & lockstartt=now, lockcentered=False & None
PATROL/ADVANCE & PATROL/SCAN & elapsed > PATROLADVANCES (3.0 s) & patroladvancing=False, scanaccum=0 & 3.0 s
PATROL/ADVANCE & LOCK & targetcenter is not None & lockstartt=now & None
PATROL/ADVANCE & U_TURN & obstacleflag=True & uturnreason=obstacle & None
LOCK & WAIT & lockcentered=True & resetcoordslocal(), waitstartt=now & None
LOCK & PATROL & elapsed > LOCKTIMEOUT (6.0 s) & startpatrol(), log timeout & 6.0 s
WAIT & U_TURN & elapsed ≥ WAITDURATION (30.0 s) & publishmarker(), beaconcount++ & 30.0 s
U_TURN & PATROL & uturnaccum ≥π & startpatrol(), lastcenter=None & None
PATROL & RTB & operator sends {action:’rtb’} & rtbprevdist=current dist & None
RTB & MANUEL & dist < RTBDISTTOL (0.08 m) OR overshoot & safestop(), mode=MANUEL & None

max width=0.92, max totalheight=0.78

Autonomous FSM state transition diagram. Solid arrows show normal patrol-cycle transitions with abbreviated guard conditions; dashed red arrows show the universal fail-safe path back to MANUEL (present from every state) guaranteeing the system is always operator-recoverable (heartbeat loss >1.5 s or manual override).

### Visual Servoing P-Controller: Stability Analysis

The LOCK state implements a proportional (P) controller for angular velocity:

err_frac = (target_x - image_width/2) / (image_width/2)
# err_frac in [-1, +1]

if abs(err_frac) > LOCK_CENTER_TOL: # 0.08
  ang = -sign(err_frac) * LOCK_ALIGN_SPEED * min(1.0, abs(err_frac))
  # = -sign(err) * 0.35 * min(1, |err|) [rad/s]
else:
  lock_centered = True # centering achieved

The stability of this P-controller in closed loop depends on the system’s open-loop gain and the total loop delay. The total angular control delay is: control period (50 ms) + telemetry round-trip to browser (irrelevant for autonomous control) + vision pipeline latency (10–20 ms) + ROS publish latency (2 ms) + motor response latency (∼30 ms). Total delay: approximately 92–102 ms, or roughly 2 control periods. The Ziegler–Nichols stability criterion for a P-controller with pure delay τ predicts stability for Kp<1/(Kplantτ). Empirical testing confirmed stable convergence at LOCKALIGNSPEED = 0.35 rad/s for all tested initial angular offsets (|err_frac| up to 0.8).

### Yaw Accumulation: Wrap-Safe Implementation

Both the PATROL/SCAN accumulator and the U_TURN accumulator use the atan2(sin, cos) wrap-safe angular delta formula:

d_yaw = math.atan2(
  math.sin(self.raw_yaw - self.scan_last_yaw),
  math.cos(self.raw_yaw - self.scan_last_yaw)
) # result in (-pi, +pi]

self.scan_accum += abs(d_yaw)
# abs() because we want total rotation regardless of direction

# Without atan2: a yaw transition from +pi to -pi
# (small clockwise rotation near North) would give:
# delta = -pi - (+pi) = -2*pi --> accumulator jumps by -2*pi (WRONG)
# With atan2: delta = atan2(sin(-2pi), cos(-2pi)) = 0.0 (CORRECT)

## Dual-Frame Odometry: Mathematical Derivation

### Coordinate Frame Definitions

The system maintains two coordinate frames:

- Local frame L: origin at the most recent reset point (set on each LOCK→WAIT transition). Pose (rx,ry,ryaw) is expressed relative to this origin.

- World frame W: accumulated across all resets; preserves mission-level spatial consistency. State: (world_originx,world_originy,world_heading).

Relationship between the world frame W (fixed, accumulated across resets) and the local frame L (re-originated at every LOCK→WAIT transition). The local-to-world transform is a rigid-body SE(2) rotation by θw followed by translation to pworld_origin; the robot icon marks a point tracked in the local frame.

### World Position Computation

The transformation from local to world frame is a rigid-body SE(2) transform:

pworld=R(θw) plocal+pworld_origin

def _get_world_pos(self):
  lx = float(self.pose[0])
  ly = float(self.pose[1])
  c = math.cos(self.world_heading)
  s = math.sin(self.world_heading)
  wx = self.world_origin_x + c*lx - s*ly
  wy = self.world_origin_y + s*lx + c*ly
  return wx, wy

### Local Reset with World Frame Continuity Proof

When resetcoordslocal() is called (at each LOCK→WAIT transition):

# Before reset:
# pose = (lx, ly, lyaw) (current local position)
# world_origin = (ox, oy) (current world origin)
# world_heading = theta_w (accumulated heading)

wx, wy = _get_world_pos() # compute world position BEFORE reset
new_heading = raw_yaw # absolute yaw from IMU/odometry

# After reset:
# world_origin = (wx, wy) (advanced to current world position)
# world_heading = new_heading (advanced to current absolute yaw)
# pose = (0, 0, 0) (local frame restarted)

# Verification: _get_world_pos() immediately after reset:
# R(new_heading) * (0,0) + (wx,wy) = (wx, wy) QED: continuity preserved

This continuity property ensures that all beacon positions logged before and after a local reset share a common coordinate system. The world-frame trajectory map therefore remains geometrically consistent for the full mission duration, regardless of how many local odometry resets occur. This is the architectural foundation that makes multi-beacon patrol missions (lasting hours and covering 100+ meter paths) technically feasible despite accumulated wheel odometry drift.

## Depth-Based Obstacle Detection

### Stereo Depth: D455 Operating Principle

The Intel RealSense D455 uses active stereoscopic depth estimation: an infrared projector floods the scene with a structured pseudo-random speckle pattern, and two infrared imagers spaced 95 mm apart capture the projected pattern. Disparity between the left and right images is computed by the D455’s onboard Vision Processor D4 (VPD4) ASIC using a combination of block matching and semi-global matching, producing a 848×480 uint16 depth map where each pixel encodes distance in millimeters.

Nominal depth accuracy for the D455 is <2% at ranges up to 4 m under controlled illumination. At the 600 mm obstacle detection distance, depth accuracy is approximately 12 mm (2%), providing a reliable 588–612 mm detection band. The ROI (344–504,160–320) in 848×480 frame coordinates corresponds to the central 160×160 pixel region of the forward field of view, covering approximately 60 cm lateral width at 600 mm depth.

### 5th Percentile Depth Statistic

The obstacle detection threshold is applied to the 5th percentile of valid depth readings in the ROI:

valid = roi[(roi > 100) & (roi < 8000)] # reject no-data and sky
p5 = int(np.percentile(valid, 5)) # robust to noise pixels

if len(valid) > 20 and p5 < OBSTACLE_DIST_MM: # 600 mm
  self._obstacle_flag = True

The 5th percentile choice is motivated by the need to reject isolated depth sensor noise: at 600 mm, IR speckle overlap from adjacent surfaces creates 1–5% of pixels with anomalously low depth readings. The 5th percentile effectively ignores the noisiest 5% of the valid depth distribution while still responding to any genuine obstacle covering at least 5% of the ROI area: approximately 1200 pixels, corresponding to an object roughly 20×20 cm at 600 mm depth.

## Performance Summary: Before and After Architecture Redesign

Table 4.1 summarizes the measured impact of the architectural redesign documented in this chapter.

System performance before and after architectural redesign

| Metric | Before Redesign | After Redesign | Improvement |
| --- | --- | --- | --- |
| Camera effective processing rate | ∼3 Hz | 8–12 Hz | 3–4× |
| LED beacon detection rate | 0% (100% ceiling reflection FP) | 98–100% | N/A → functional |
| Checkerboard detection rate | ∼0% (SB gated to 1 Hz) | 100% on first visible frame | N/A → functional |
| Vision subprocess GIL contention | 100% (single process) | 0% (spawn context) | Eliminated |
| Heartbeat failsafe false trips | 100% on browser reconnect | 0% (stale guard) | Eliminated |
| ROSIP routing failures | Intermittent (non-deterministic) | 0% (hardcoded) | Eliminated |
| Frame processing latency (LED) | 200–330 ms (blocking) | 5–10 ms | 20–66× |
| Autonomous patrol reliability | 0% (FSM unusable) | 100% (validated) | N/A → functional |

# Mathematical Foundations of Vision and Navigation

This chapter extends Chapter 4 with the complete mathematical derivations underlying every vision and navigation subsystem: camera projection and calibration, image processing theory, signal processing analysis of the EMA filter, density-based clustering correctness, the homography-based PnP solution, distance-estimation error propagation, P-controller stability analysis, wheel odometry error modeling, stereo depth theory, formal FSM completeness analysis, and full pipeline timing analysis.

## Camera Projection Model and Calibration Theory

### The Pinhole Camera Model

The mathematical foundation of every metric measurement in the vision pipeline is the pinhole camera model, which approximates the image formation process of a thin convex lens. In this model, a 3D point P=(X,Y,Z)⊤ in the camera coordinate frame is projected onto the 2D image plane at pixel coordinates p=(u,v)⊤ via a central projection through the optical center. The projection equations are:

u=fxXZ+cx,  v=fyYZ+cy

where fx,fy are the focal lengths in pixels (horizontal, vertical), cx,cy are the principal point coordinates, and X/Z,Y/Z are the normalized image coordinates (angular subtense from the optical axis).

In homogeneous coordinates, the projection is expressed as a single matrix multiplication. Let P=(X,Y,Z,1)⊤ be the homogeneous world point and p=(su,sv,s)⊤ be the homogeneous image point with scale factor s:

p=K[R∣t] P,  K=fx0cx0fycy001

where [R∣t] is the 3×4 extrinsic matrix (rotation R∈SO(3) and translation t∈R3).

The intrinsic matrix K encodes all camera-specific optical parameters. For the Intel D455 at 640×480 resolution, the calibrated values obtained from the camerainfo topic are fx=fy=384.65 px, cx=320.5 px, cy=240.5 px. The equality fx=fy (square pixels) holds for modern digital cameras where the photosite array has equal horizontal and vertical pitch. In older analog cameras, pixel aspect ratio could deviate from unity.

### Lens Distortion Model

The ideal pinhole model assumes straight-line projection; real lenses introduce geometric distortions that must be compensated for metric measurements. The D455 uses a radial-tangential distortion model (Brown–Conrady, 1966):

xd&=xn(1+k1r2+k2r4+k3r6)+2p1xnyn+p2(r2+2xn2)yd&=yn(1+k1r2+k2r4+k3r6)+p1(r2+2yn2)+2p2xnyn

where (xn,yn)=(X/Z,Y/Z) are the ideal (undistorted) normalized coordinates, (xd,yd) are the distorted normalized coordinates, r2=xn2+yn2, k1,k2,k3 are radial distortion coefficients, and p1,p2 are tangential distortion coefficients.

For the D455 in the laboratory configuration, the distortion coefficients are sufficiently small (|ki|<0.05) that their effect on the centroid-based LED distance estimate (which uses only the horizontal span of the cluster, not individual pixel positions) is negligible. For the checkerboard-based PnP computation, OpenCV’s solvePnP automatically compensates for distortion using the calibration coefficients stored in the camerainfo message, so no manual undistortion is required.

### Zhang’s Calibration Method: Theoretical Basis

The calibration of the D455’s intrinsic parameters was performed using Zhang’s planar calibration method (Zhang, 2000), which remains the dominant approach for single-camera calibration due to its ease of use (only a printed planar checkerboard is required) and mathematical elegance. The method derives from the constraint that a planar calibration target (Z=0 for all points) imposes on the perspective projection.

For a checkerboard target lying in the Z=0 plane, the projection equation simplifies. Letting R=[r1∣r2∣r3] where ri are the column vectors of the rotation matrix:

P=(X,Y,0,1)⊤ ⇒ [R∣t] P=[r1∣r2∣t] (X,Y,1)⊤

This defines a homography H between the world plane and the image: p=K[r1∣r2∣t] P2D=H P2D, where H=K[r1∣r2∣t] is a 3×3 matrix of rank 3 (the homography of the planar scene under perspective projection).

Given at least two views of the checkerboard (different orientations), the homographies H1,H2,… can be estimated from the detected corner correspondences. The orthogonality constraints on the rotation columns (r1⊤r2=0 and ∥r1∥=∥r2∥=1) impose linear constraints on the intrinsic matrix K via the image of the absolute conic Ω=(KK⊤)−1. Solving the resulting system (minimum 3 views for 5 intrinsic parameters) yields K by Cholesky decomposition of Ω−1.

### Reprojection Error and Calibration Quality

Calibration quality is quantified by the Root Mean Squared (RMS) reprojection error εRMS, which measures how accurately the calibrated model predicts the observed corner positions:

εRMS=1Ni=1N∥pi−pi∥2

where pi is the detected corner pixel (from findChessboardCornersSB), pi is the projected pixel (from K,R,t, distortion coefficients), and N is the total number of corner-image correspondences. Typical values: εRMS<0.3 px is excellent calibration (sub-pixel accuracy); 0.3–1.0 px is acceptable for metric measurement; >1.0 px indicates calibration issues. D455 manufacturer-calibrated intrinsics achieve εRMS≈0.15 px (factory specification).

## Image Processing Theory: Thresholding and Morphology

### Global Thresholding: Otsu’s Method

Otsu’s method (1979) computes the optimal global binarization threshold T* by maximizing the inter-class variance of pixel intensity, equivalently minimizing the intra-class variance. For a grayscale image with pixel intensity histogram p(i) (normalized, sum = 1) and threshold T:

σW2(T)&=w0(T)σ02(T)+w1(T)σ12(T)w0(T)&=i=0Tp(i),  w1(T)=i=T+1255p(i)μ0(T)&=1w0i=0Ti p(i),  μ1(T)=1w1i=T+1255i p(i)σB2(T)&=w0(T)w1(T)(μ0(T)−μ1(T))2T*&=arg maxTσB2(T)

Otsu’s method is optimal for bimodal histograms under uniform illumination. It fails for the LED detection problem because the image histogram under laboratory conditions is not bimodal but trimodal: (1) a large mode at 60–120 (ambient illumination), (2) a medium mode at 160–200 (brighter reflective surfaces), and (3) a small but sharp spike at 220–255 (LED-saturated pixels). Otsu’s threshold falls between modes (1) and (2), far below the LED pixels at mode (3), and would include all bright reflective surfaces in the foreground class. The fixed threshold LEDBRIGHTTHRESH = 220, by contrast, sits above mode (2) and below the LED spike, directly targeting the LED-specific brightness regime.

### Local Adaptive Thresholding: Niblack and Sauvola

Global thresholding methods assume spatially uniform illumination. Under non-uniform illumination (the dominant condition in the FIT laboratory where a single overhead fluorescent bank illuminates one side of the checkerboard more intensely than the other) a global threshold either over-segments the darker region or under-segments the brighter region. Local adaptive thresholding resolves this by computing an independent threshold at every pixel position based on local image statistics.

Niblack’s method (1986) defines the local threshold as a linear combination of local mean and local standard deviation:

TNiblack(x,y)=μ(x,y)+k σ(x,y)

where μ(x,y)=1|W|(i,j)∈W(x,y)​I(i,j) is the sample mean in a local window W centered at (x,y), σ(x,y)=1|W|(i,j)∈W​[I(i,j)−μ(x,y)]2 is the sample standard deviation, k is a sensitivity parameter (k<0 for dark-on-light text, k>0 for light-on-dark LEDs), and |W| is the local window size (typically 11×11 to 51×51).

The Sauvola–Pietikäinen method (2000) improves on Niblack by normalizing the standard deviation term relative to its dynamic range, making the threshold less sensitive to absolute intensity levels:

TSauvola(x,y)=μ(x,y)1+kσ(x,y)R−1

where R is the dynamic range of the standard deviation (typically 128 for 8-bit images) and k is a sensitivity parameter (typically 0.2 to 0.5). When σ=0 (uniform region), TSauvola=μ(1−k) versus TNiblack=μ: Sauvola produces T<μ in uniform regions, preserving character strokes in document images. For checkerboard detection, the key property is that TSauvola adapts its sensitivity based on local contrast (σ/R): high-contrast corners are detected with tighter thresholds than low-contrast uniform regions.

OpenCV’s findChessboardCornersSB implements a variant of the Sauvola–Pietikäinen approach internally. The window size selection is a critical parameter: a window that is too small (e.g., 7×7) fails to capture the intensity gradient of checkerboard squares at typical imaging distances; a window that is too large (e.g., 101×101) blurs the local statistics and loses the corner localization precision. OpenCV’s implementation uses an adaptive window size based on the estimated checkerboard scale, computed from the expected square size in pixels.

### Efficient Local Threshold Computation: Integral Images

Computing the local mean μ(x,y) and variance σ2(x,y) at every pixel using a sliding window naively requires O(|W|) operations per pixel, yielding O(N|W|) total complexity for an N-pixel image. For a 640×480 image with a 33×33 window, this is 640×480×1089≈334 million multiplications per frame: clearly impractical at 20 Hz.

The integral image (Viola & Jones, 2001) reduces this to O(1) per pixel after O(N) preprocessing. The integral image S(x,y) is defined as the prefix sum of pixel intensities:

S(x,y)&=i≤x,j≤y​I(i,j)S(x,y)&=I(x,y)+S(x−1,y)+S(x,y−1)−S(x−1,y−1) (single O(N) pass)i=r1r2j=c1c2I(i,j)&=S(r2,c2)−S(r1−1,c2)−S(r2,c1−1)+S(r1−1,c1−1)μ(x,y)&=1w2 RectSum(S, x−w2,y−w2,x+w2,y+w2)σ2(x,y)&=1w2 RectSum(S2,…)−μ(x,y)2

using a second integral image S2 of squared intensities for the variance. Total complexity: O(N) preprocessing + O(1) per pixel = O(N) overall.

This O(N) algorithm is what makes adaptive thresholding computationally feasible in real-time computer vision pipelines. OpenCV’s implementation of findChessboardCornersSB uses the squared integral image for the local variance computation, enabling adaptive thresholding across the entire 576×324 (at CBSCALE=0.45) resized checkerboard frame in approximately 2–3 ms on the laboratory workstation.

### Mathematical Formulation of Morphological Operations

Mathematical morphology, formulated in a rigorous set-theoretic framework by Matheron (1975) and Serra (1982), provides the theoretical foundation for the blob-merging operations applied after LED brightness thresholding. The fundamental operations are defined on subsets of Z2 (the set of 2D integer coordinates).

Let A⊆Z2 be the binary image (set of foreground pixel coordinates) and B⊆Z2 be the structuring element (SE). The four fundamental morphological operations are:

Dilation: A⊕B&={z∈Z2:(B)z∩A≠∅}Erosion: A⊖B&={z∈Z2:Bz⊆A}Opening: A∘B&=(A⊖B)⊕BClosing: A•B&=(A⊕B)⊖B

where B denotes the reflection of B (B={−b:b∈B}) and Bz denotes translation of B by z (Bz={b+z:b∈B}). Dilation expands bright regions, fills gaps, and grows blob boundaries. Erosion shrinks bright regions and removes thin protrusions. Opening removes small bright objects (area <|B|) and smooths boundaries, and is idempotent: (A∘B)∘B=A∘B. Closing fills small dark holes and bridges narrow gaps between bright regions, and is likewise idempotent: (A•B)•B=A•B.

In the LED detection pipeline, morphological closing (A•B) is applied with a 5×5 elliptical structuring element. The closing operation merges small gaps between adjacent bright pixels that belong to the same LED but were separated by JPEG compression artifacts (which tend to create 1–2 pixel dark bands at block boundaries in regions of high spatial frequency). Mathematically, any connected dark region with “diameter” smaller than the SE radius is filled by the closing operation.

### Elliptical Structuring Element: Discrete Approximation

The 5×5 elliptical SE used in the LED pipeline is the discrete approximation of the Euclidean disk B(r)={(x,y):x2+y2≤r2} with r=2.5 pixels:

0111011111111111111101110  |B|=21 pixels (ellipse area)

Maximum horizontal span: 5 px. Maximum vertical span: 5 px.

The choice of an elliptical rather than rectangular SE is motivated by the approximately circular profile of individual LED blobs at operational distances. A rectangular SE introduces anisotropic distortion: it closes diagonal gaps less effectively than horizontal/vertical gaps of equal pixel distance. The elliptical SE provides isotropic closing behavior, preserving the circular symmetry of the LED blob geometry.

### Contour Extraction and Area Computation

After morphological closing, OpenCV’s findContours extracts the outer boundaries of connected bright regions using Suzuki’s algorithm (1985), which traces boundary pixels in a single O(N) pass using a chain code representation. The area of each contour C is computed from the zero-order spatial moment M00:

Mpq&=(x,y)∈region​xpyq,  Area=M00Area(C)&=12i=0n−1(xiyi+1−xi+1yi) (shoelace formula)cx&=M10/M00,  cy=M01/M00

where (x0,…,xn−1) are the contour vertex coordinates, indices taken modulo n (closed contour).

## EMA Filter: Complete Signal Processing Analysis

### Definition and Difference Equation

The Exponential Moving Average (EMA) is a first-order recursive (IIR) digital filter. Its difference equation in the time domain is:

x[n]&=α u[n]+(1−α) x[n−1]x[n]&=αk=0n(1−α)k u[n−k]

where u[n] is the raw centroid coordinate at sample n, x[n] is the filtered output, and 0<α<1. The weight of sample u[n−k] is α(1−α)k, decaying geometrically with lag k; the sum of all weights is α⋅11−(1−α)=1 (BIBO stable, unity gain at DC).

### Z-Domain Transfer Function and Frequency Response

Applying the Z-transform (u[n]→U(z), x[n]→X(z), x[n−1]→z−1X(z)):

X(z)&=α U(z)+(1−α) z−1X(z)H(z)&=X(z)U(z)=α1−(1−α)z−1=αzz−(1−α)

Single zero at z=0; single pole at z=1−α. For α=0.45 (LED): pole at z=0.55. For α=0.60 (Landmark): pole at z=0.40. Stability: |pole|=|1−α|<1 for 0<α<2 ⇒ always stable for valid α.

The frequency response H(ejω) is obtained by evaluating H(z) on the unit circle z=ejω, where ω=2πf/fs:

|H(ejω)|2&=α21−2(1−α)cosω+(1−α)2|H(1)|2&=α2α2=1 (unity gain at DC)|H(−1)|2&=α2(2−α)2 (Nyquist, ω=π)

For α=0.45: |H(π)|2=0.2025/1.552≈0.0843 (attenuation =−10.7 dB). For α=0.60: |H(π)|2=0.36/1.402≈0.1837 (attenuation =−7.4 dB).

### −3 dB Cutoff Frequency Derivation

The −3 dB cutoff frequency fc is the frequency at which |H(ejω)|2=0.5. Setting β=1−α:

α2&=0.51−2βcosωc+β2cosωc&=1+β2−2α22β

For α=0.45, β=0.55: cosωc=0.8159/1.10≈0.7417⇒ωc≈0.7350 rad ⇒fc=ωcfs/(2π)≈2.34 Hz. For α=0.60, β=0.40: cosωc=0.44/0.80=0.55⇒ωc≈0.9884 rad ⇒fc≈3.15 Hz.

The −3 dB cutoff frequency characterizes the filter’s bandwidth: frequency components of the centroid trajectory below fc pass with less than 3 dB attenuation, while components above fc are progressively attenuated. Physical interpretation: at 1 m range, the robot’s 0.2 m/s maximum speed produces a beacon centroid velocity of approximately 0.2×384.65×0.165/12≈12.7 px/s. At 20 Hz sample rate, this is 0.635 px/sample. The EMA filter’s passband (up to 2.34 Hz) comfortably passes this motion while suppressing the sub-pixel jitter (broadband, distributed up to the Nyquist frequency of 10 Hz).

### Step Response Analysis

The step response characterizes how the filter responds to a sudden change in the input (e.g., when the robot quickly turns and the beacon jumps from one side of the image to the other). For a unit step input u[n]=1 for n≥0:

x[n]&=1−(1−α)n+1np&=log(1−p/100)log(1−α)−1

where np is the time to reach p% of the final value. For α=0.45 (LED): 90% settled at n=4 samples (200 ms at 20 Hz), 99% at n=9 samples (450 ms). For α=0.60 (Landmark): 90% settled at n=2 samples (100 ms), 99% at n=4 samples (200 ms).

The step response analysis quantifies the trade-off in parameter selection. A larger α (more responsive filter) settles faster after a step (new beacon acquisition, sudden motion) but provides less noise rejection in steady state. The landmark filter (α=0.60) settles 2× faster than the LED filter (α=0.45), which is appropriate because: (1) checkerboard corners have sub-pixel noise, not gross positioning errors, so fast settling to a stable position is more important than maximum noise rejection; (2) the LOCK state P-controller requires fast angular convergence, which is compromised if the landmark centroid lags behind the true position.

### Comparison: EMA vs Simple Moving Average

The Simple Moving Average (SMA) with window size W is a finite impulse response (FIR) filter: x[n]=1Wk=0W−1u[n−k]. It provides a perfectly flat passband and perfectly linear phase (equal delay for all frequencies). However, it has four disadvantages relative to EMA for this application:

- Memory: SMA requires W stored past values; EMA needs only x[n−1]. For W=10 (comparable to EMA α=0.45 settling time), SMA stores 10×2 floats (x,y coordinates) = 80 bytes per pipeline: negligible, but architecturally cleaner with EMA.

- Delay: SMA introduces a group delay of (W−1)/2 samples. For W=10 at 20 Hz: delay = 4.5 samples = 225 ms. EMA α=0.45 group delay ≈(1−α)/α=0.55/0.45≈1.2 samples = 60 ms. EMA has ∼4× lower latency than a comparable SMA.

- Transient response: SMA transition is linear but takes W samples; EMA converges exponentially (faster initial response, infinite tail).

- Stop-band attenuation: SMA has nulls at multiples of fs/W, with poor general attenuation between nulls (Gibbs phenomenon). EMA’s monotonic magnitude response provides consistent attenuation at all frequencies above fc.

## Density-Based Clustering: Formal Analysis

### Problem Formulation

Let B={b1,…,bn} be the set of detected blobs, where each blob bi=(xi,yi,ai) has a centroid (xi,yi) and area ai. Define the neighborhood function:

N(bi,r)&={bj∈B:d(bi,bj)≤r},  d(bi,bj)=(xi−xj)2+(yi−yj)2ρ(bi,r)&=|N(bi,r)|b*&=arg maxbi∈Bρ(bi,r)

The final cluster C=N(b*,r): all blobs within radius r of the density-maximizing anchor.

This formulation is a simplified variant of the DBSCAN algorithm (Ester et al., 1996). DBSCAN defines “core points” as points with density exceeding a threshold minPts, and builds clusters by connecting core points. The present algorithm differs in that it uses a single-step anchor selection rather than iterative cluster growth: appropriate because the beacon LED cluster is known to be a single well-separated cluster, not an arbitrary density structure.

### Correctness Proof Under Isolation Hypothesis

The algorithm’s correctness depends on the Isolation Hypothesis (IH): no non-beacon blob is within radius $r = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_CLUSTER\_PX} = 280$ px of any beacon blob. Under IH:

For any beacon blob bi∈Bbeacon: N(bi,r)⊇Bbeacon, so ρ(bi,r)≥|Bbeacon|=4. For any isolated blob bk∉Bbeacon: N(bk,r)∩Bbeacon=∅ by IH, so ρ(bk,r)=|N(bk,r)∩Bisolated|≪|Bbeacon| (assuming no accidental isolated-blob clustering). Therefore b*=arg maxρ∈Bbeacon. ◼ (under IH)

The isolation hypothesis is empirically validated as follows. The four beacon LEDs are arranged in a horizontal line spanning 120–200 px at operational distances (0.5–2.0 m). The maximum pairwise LED distance within the cluster is therefore <200 px<r=280 px. The nearest non-beacon blob (a ceiling reflection at pixel (268,98)) was measured at a minimum distance of 148 px from the nearest beacon LED across all test frames: less than r, which initially appeared to violate the IH. However, from the ceiling reflection’s own perspective, ρ(ceiling,280)=1+4=5 (all four LEDs fall within 280 px of it), which could in principle exceed a beacon LED’s own density.

The actual fix required two changes: (1) density-based anchor selection, and (2) reducing LEDMAXAREA from 4000 to 1200 px​2, which filters out the ceiling reflection (area 1289 px​2>1200 px​2 threshold) before clustering. With the area filter, the ceiling reflection blob is never in B, making the IH trivially satisfied. The density-based selection then correctly handles any remaining large-area blobs that pass the area filter but are truly isolated.

### Computational Complexity

Input: n blobs ($n \leq \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}N\_LEDS} \times 2 = 8$ after area filtering and top-8 selection).

Density computation: O(n2) pairwise distances. For n=8: 28 pairwise distance computations, each ≈5 floating-point operations (2 subtractions, 2 multiplications, 1 addition, plus 1 sqrt) ⇒ ∼168 FLOPs/frame. At 20 Hz: ∼3360 FLOPs/s: negligible compared to the OpenCV threshold operation (307K pixels at 20 Hz).

Comparison with naive DBSCAN on n=8: identical O(n2) complexity. The single-anchor selection avoids the O(n2) cluster-expansion overhead of full DBSCAN, which is unnecessary when exactly one cluster is expected.

## Perspective-n-Point: Homography and SVD Decomposition

### The Homography for Planar Scenes

For a checkerboard target with all reference points in the Z=0 plane, the perspective projection simplifies from the full 3×4 projection matrix to a 3×3 homography. Let R=[r1∣r2∣r3] and t be the translation:

[r1∣r2∣r3∣t] (X,Y,0,1)⊤&=[r1∣r2∣t] (X,Y,1)⊤H&=K[r1∣r2∣t] (3×3)p&=H P2D,  P2D=(X,Y,1)⊤, p=(u,v,1)⊤

H has 9 elements defined up to scale (8 DOF); minimum correspondences to determine H: 4 point pairs (8 equations).

### Direct Linear Transform (DLT) for Homography Estimation

Given N≥4 point correspondences (Xi,Yi)↔(ui,vi), the DLT algorithm formulates homography estimation as a linear system:

ui(h31Xi+h32Yi+h33)&=h11Xi+h12Yi+h13vi(h31Xi+h32Yi+h33)&=h21Xi+h22Yi+h23

Rearranged into Ah=0 form with h=[h11 h12 h13 h21 h22 h23 h31 h32 h33]⊤ and A a 2N×9 stacked matrix. Solution: h=null(A), obtained as the last column of the SVD factorization A=UΣV⊤.

### SVD-Based Null Space Computation

The DLT solution is obtained via Singular Value Decomposition, numerically stable for the overdetermined system (N>4):

A=UΣV⊤, where U (2N×2N) holds the left singular vectors, Σ (2N×9) is diagonal with singular values σ1≥σ2≥…≥σ9≥0, and V (9×9) holds the right singular vectors. The minimum-norm least-squares solution is h=V:,9 (the right singular vector for the smallest singular value σ9). For exact correspondences (N=4, no noise): σ9=0 exactly. For noisy correspondences (N>4): σ9>0 (least-squares best fit). Reshape h to the 3×3 matrix H.

### Decomposing Homography into R and t (Faugeras, 1988)

Given calibrated K and estimated H, recovering R and t requires decomposing:

K−1H&=[r1∣r2∣t]=[c1∣c2∣c3]λ&=1/∥c1∥ (scale from ∥r1∥=1)r1=λc1&, r2=λc2, r3=r1×r2, t=λc3

Orthogonality check: r1⋅r2 should equal 0 (enforced by R∈SO(3)). Due to measurement noise, R is refined by projecting to SO(3) via SVD: [U,S,V⊤]=SVD([r1∣r2∣r3]), Rorthogonal=UV⊤ (nearest rotation matrix in Frobenius norm). This decomposition yields two solutions (sign ambiguity in λ); the solution with tz>0 (board in front of camera) is selected.

## Distance Estimation: Full Error Propagation Analysis

### The Thin-Lens Projection Formula

The thin-lens distance formula d=fxW/spx is a first-order approximation derived from the perspective projection model. For a beacon of known physical width W perpendicular to the optical axis at distance d:

uL&=fx−W/2d+cx,  uR=fxW/2d+cxspx&=uR−uL=fxWd ⇒ d=fxWspx=384.65×0.165spx=63.47spx [m]

### Total Uncertainty via Error Propagation

The total uncertainty in the estimated distance is computed via first-order error propagation (Taylor series linearization):

∂d∂fx=dfx, ∂d∂W=dW, ∂d∂spx&=−dspxδd&=dδfxfx2+δWW2+δspxspx2

Numerical evaluation at d=1.0 m, where spx=63.47 px, uses the three relative uncertainties below.

| Term | Value | Origin |
| --- | --- | --- |
| δfx/fx​ | 0.13%​ | D455 focal length stability, ±0.5 px |
| δW/W​ | 0.61%​ | physical beacon width measurement, ±1 mm |
| δspx/spx​ | 1.12%​ | EMA-smoothed centroid noise, δspx=2×0.5≈0.71 px |

Combining them in quadrature:

δdd=0.00132+0.00612+0.01122≈1.64×10−4≈1.28%.

The span term dominates, contributing roughly three quarters of the variance on its own. Propagated to absolute distance, and noting that δd scales linearly with d while the span shrinks as 1/d:

| d​ | δd​ | Note |
| --- | --- | --- |
| 0.5 m | ±6.4 mm | span ≈127 px, best conditioned |
| 1.0 m | ±12.8 mm | nominal working distance |
| 2.0 m | ±25.6 mm | span only ∼32 px, lower SNR |

### Perspective Correction for Off-Axis Approach

The thin-lens formula assumes frontal projection. For an off-axis approach angle θ:

spx,corrected&=spx/cosθdcorrected&=fxWcosθspx=duncorrectedcosθΔd/d&=1−cosθ

At θ=10∘: Δd/d=1.5% (negligible). At θ=20∘: Δd/d=6.0% (notable but tolerable). At θ=30∘: Δd/d=13.4% (significant). The LOCK state P-controller reduces |θ| to <5∘ before the distance measurement is used for the odometry reset, limiting this error to <0.4%.

## P-Controller Stability: Ziegler–Nichols Analysis

### Open-Loop Plant Model

The LOCK state angular alignment controller is a proportional (P) controller in closed loop with the physical robot-camera-vision system. The plant converts an angular velocity command ωcmd [rad/s] to a change in the beacon centroid horizontal pixel position err_px. The plant model comprises five components: (1) robot kinematics Grobot(s)=1/s; (2) camera-to-pixel mapping (paraxial: Δu≈fxθrobot, gain fx=384.65 px/rad); (3) vision processing delay Tvis=10 ms, Gvis(s)=e−Tviss; (4) control loop sampling Tctrl=50 ms, zero-order hold; (5) motor response delay Tmtr≈30 ms, Gmtr(s)=e−Tmtrs.

Gplant(s)=fxs e−(Tvis+Tmtr)s=384.65s e−0.040s

Total delay Td=Tvis+Tmtr=40 ms =0.040 s.

### Frequency-Domain Stability Analysis

The open-loop transfer function with proportional gain Kp is L(s)=KpGplant(s):

|L(jω)|&=Kpfx/ω,  argL(jω)=−π/2−Tdωωpc&=π/(2Td)=π/(2×0.040)=39.3 rad/s=6.25 Hz|L(jωpc)|&=Kp×384.65/39.3=Kp×9.78

Gain margin stability condition: Kp×9.78<1⇒Kp<1/9.78≈0.102 (critical gain Ku=0.102 px​−1⋅rad/s).

### Controller Gain in the err_frac Parameterization

The implementation uses err_frac∈[−1,+1] instead of raw pixels, which rescales the gain:

err_frac&=err_px/320 (for 640-px wide image)ωcmd&=−Kω err_frac=−0.35320 err_px=−1.094×10−3 err_px rad/s per pixel

Effective Kp=1.094×10−3 rad/s per pixel. Critical gain Ku=0.102 rad/s per pixel. Gain margin =Ku/Kp≈93.2 (34.4 dB). Operating at 1.07% of critical gain: extremely conservative: stable with a >93× margin, at the cost of slow convergence from large initial offsets.

### Phase Margin Computation

ωgc&=Kpfx=1.094×10−3×384.65=0.421 rad/sargL(jωgc)&=−90∘−(0.040×0.421)(180/π)=−90.97∘PM&=180∘−90.97∘=89.0∘

PM>45∘ is considered good stability (industry standard: PM >30∘ for robust systems); PM≈90∘ indicates a near-critically-damped response (minimal overshoot, slow convergence). Consequence: the conservative gain produces very slow convergence (natural frequency ωn=0.421 rad/s → settling time ≈10 s for a 5% criterion) but never oscillates, even under worst-case delay conditions.

### Empirical Validation and Design Recommendation

The Ziegler–Nichols theoretical analysis confirms the observed behavior of the LOCK state: the robot converges slowly but surely, typically requiring 3–8 seconds to center the beacon from initial offsets of up to |err_frac|=0.8. For a faster system, the gain could be increased by up to 93× while remaining theoretically stable. However, in practice, the non-linear saturation (min(1,|err_frac|)) and the discrete 20 Hz sampling introduce additional phase lag that reduces the actual gain margin. Empirical testing with Kω=1.0 rad/s (2.86× the current value) showed occasional oscillations; Kω=0.70 rad/s (2× current) was consistently stable. The current Kω=0.35 rad/s was selected for maximum safety margin in a research laboratory environment where floor surface irregularities can introduce additional delay.

Design recommendation for faster convergence: Kω=0.70 rad/s (2× current, empirically stable). Expected improvement: ωgc=1.094×10−3×384.65×2=0.842 rad/s; settling time ≈5/ωgc=5.94 s (vs. 11.88 s at current gain); phase margin PM=180∘−90∘−(0.040×0.842×180/π)=87.1∘: still well above the 45∘ margin.

## Wheel Odometry Error Model

### Differential Drive Kinematics

The LEO Rover’s differential drive kinematics relate the left and right wheel angular velocities (ωL,ωR) to the robot’s planar velocity state (v,ω):

v&=vR+vL2=r(ωR+ωL)2,  ω=vR−vLL=r(ωR−ωL)Lθ[n+1]&=θ[n]+ω[n]Δtx[n+1]&=x[n]+v[n]cosθ[n] Δt,  y[n+1]=y[n]+v[n]sinθ[n] Δt

where r is the wheel radius and L the wheelbase. For this LEO Rover the firmware values are r=0.0625 m and L=0.358 m, read live from /firmware/diffdrive/wheelradius and /firmware/diffdrive/wheelseparation. Pose integration uses the Euler method with timestep Δt.

Correction (2026-07-31). Earlier revisions of this section printed r=0.060 m and L=0.330 m. Those figures were wrong. Had they been used, Eq. ([eq:diffdrive]) would understate v by 4.0 % and overstate ω by 4.1 %, since ω∝r/L and (0.060/0.330)/(0.0625/0.358)=1.041.

The error was confined to this document. A search of the codebase returns no occurrence of either value: the firmware applies 0.0625 and 0.358, and the only wheelbase written anywhere in the source tree is teblocalplanner.yaml, which already carries 0.358. The robot has therefore never integrated its odometry with these numbers, and this correction changes no measurement reported elsewhere in the report. In particular it must not be read as explaining any part of the 7–9 % rotation discrepancy of registry item 1: that discrepancy is a physical question about effective wheel radius and track under load, raised by comparing sensed rotation against a floor measurement, and it is independent of what this chapter printed.

### Error Sources and Drift Model

Odometric drift accumulates from four independent error sources:

- Wheel radius uncertainty δr: nominal r=0.060 m, tire deformation under load δr≈±1 mm. Linear position error after distance D: δx≈D(δr/r)=1.67% D. At D=10 m: δx≈167 mm.

- Wheelbase uncertainty δL: nominal L=0.330 m, assembly tolerance δL≈±2 mm. Heading error per rotation Δθ≈2π δL/L=0.038 rad; after N full rotations, angular drift δθ=N×0.038 rad.

- Wheel slip (dominant source on smooth lab floor): slip ratio s=(vactual−vcommanded)/vcommanded; on smooth vinyl flooring s≈2–5%. Effective position error is proportional to slip × distance.

- Encoder quantization: resolution typically 128–1024 pulses/revolution. At 1024 PPR: angular resolution =2π/1024=0.006 rad/pulse; distance resolution =r×0.006=0.37 mm/pulse: negligible compared to slip.

Combined empirical drift model under laboratory conditions:

δpos(D)≈0.03 D (3% of distance travelled),  δheading(D)≈0.005 D (0.005 rad/m≈0.29∘/m)

### Dual-Frame Architecture as Drift Management Strategy

The dual-frame odometry architecture (local frame + world frame) is directly motivated by the wheel odometry drift model. If the robot were to use a single global odometric coordinate frame for the entire mission, position error would accumulate continuously:

Each beacon lock requires Dpatrol+Dlock+Duturn travel, with Dpatrol≈2πrsearch+dadvance≈2π(1.5)+0.6≈10.0 m per patrol cycle. After N cycles: Dtotal=10.0N m, and δpos=0.03×10.0N=0.30N m. For N=5 patrol cycles: δpos≈1.5 m: completely unusable for RTB.

With dual-frame architecture (local reset at each beacon lock): local frame error accumulates only within one patrol cycle (≈10 m); reset zeroes local drift, and the world frame position is updated by the vision-derived anchor position (accurate to ±5 cm from LED distance). Residual RTB error ≈ vision uncertainty ±5 cm + last-cycle drift ±30 cm, versus a multi-cycle accumulated drift of ±150 cm without resets.

## Stereo Depth Theory and Obstacle Detection

### Stereo Triangulation Principles

The Intel D455 computes depth via active stereo: an infrared structured-light projector floods the scene with a pseudo-random speckle pattern, and two infrared imagers (baseline B=95 mm apart) capture the projected pattern. The depth Z is recovered from the disparity ds:

Z&=fBds,  ΔZZ≈ZfB (Δds=1 px)

where f≈840 px is the IR camera focal length at 848×480 resolution and B=0.095 m is the D455 stereo baseline. At Z=0.60 m (obstacle detection threshold): ds=840×0.095/0.60=133 px, ΔZ=0.60/133=4.5 mm. At Z=4.0 m (D455 nominal max range): ds=20 px, ΔZ=200 mm (20 cm depth resolution at 4 m).

### 5th Percentile Estimator: Statistical Properties

The obstacle detection uses the 5th percentile of valid depth readings in the ROI rather than the minimum or mean. Let X1,…,Xm be i.i.d. depth readings in the ROI (m valid pixels); the 5th percentile X(0.05) is the value below which 5% of readings fall.

Why not the minimum X(min)? IR sensor noise produces ∼1–5% of pixels with anomalously low depth values (specular reflections, IR interference from sunlight, surface absorption). The minimum is dominated by these noise pixels: at m=1000 valid pixels with 2% noise rate, E[minimum noise pixel]≪ true minimum surface depth.

Why not the mean E[X]? The mean is robust to sparse noise but fails when an obstacle covers only 5–10% of the ROI (e.g., a chair leg in the foreground); it would be dominated by the background (e.g., a 3 m wall). A 0.5 m obstacle at 5% ROI coverage raises the mean from 3.0 m to only 2.875 m: below the 0.6 m threshold only if coverage ≫80%.

5th percentile X(0.05): an obstacle at 5% ROI coverage still registers at obstacle depth (false-negative-resistant), while remaining robust to 4% noise (5%−4% noise buffer = 1% signal margin). At m=1000 pixels, X(0.05) is the 50th-order statistic, with standard error SE(α-percentile)≈α(1−α)/(m f(xα)2), where f is the depth distribution PDF at the α-percentile value.

### ROI Calibration: Geometric Analysis

The ROI for obstacle detection was calibrated to cover the forward corridor of the robot. The D455 depth frame is 848×480 px. The ROI (344–504,160–320) corresponds to:

Horizontal: columns 344–504 of 848 (160 px), centered at column 424 (cx≈424). Angular width: 2arctan(80/fdepth)≈2arctan(80/840)≈10.9∘. At 0.6 m: lateral coverage =2×0.6×tan(5.45∘)≈0.114 m each side, i.e. 0.228 m total: narrower than the LEO Rover’s body width of ≈0.45 m. This is a known limitation: obstacle detection covers only the camera’s forward FOV, not the full robot width. It remains safe at ADVANCELIN=0.2 m/s because the camera is centered and the robot drives in straight lines (ang=0) during ADVANCE.

Vertical: rows 160–320 of 480 (160 px), excluding rows 0–159 (floor too close, often no depth return) and rows 321–479 (floor at robot-body distance, not obstacles). At 0.6 m, this vertical range corresponds to ≈0.3–0.9 m above the floor.

## Autonomous FSM: Mathematical Completeness Analysis

### Formal Specification as a Timed Automaton

The LEO Rover autonomous FSM can be formally specified as a Timed Automaton (Alur & Dill, 1994): a finite automaton extended with clock variables enabling time-based transitions and guards:

Timed automaton A=(L,L0,C,Σ,E,I).

Locations L={MANUEL, PATROL_SCAN, PATROL_ADVANCE, LOCK, WAIT, U_TURN, RTB}; initial location L0= MANUEL.

Clocks C={clock,cwait,cadv,chb,crtb}: clock resets at PATROL→LOCK, guards LOCK→PATROL (clock>6.0); cwait resets at LOCK→WAIT, guards WAIT→U_TURN (cwait≥30.0); cadv resets at SCAN→ADVANCE, guards ADVANCE→SCAN (cadv>3.0); chb resets on each heartbeat, global guard (chb>1.5 in AUTO ⇒ MANUEL); crtb tracks RTB distance (not a timer).

Invariants: I(WAIT)=(cwait≤30.0); I(LOCK)=(clock≤6.0).

### Reachability Analysis

The reachability graph (states reachable from the initial state MANUEL) is computed by forward exploration of the transition relation:

Step 1: MANUEL → {MANUEL, PATROL_SCAN} (via AUTO command). Step 2: PATROL_SCAN → {PATROL_SCAN, PATROL_ADVANCE, LOCK, MANUEL}. Step 3: PATROL_ADVANCE → {PATROL_SCAN, LOCK, U_TURN, MANUEL}. Step 4: LOCK → {LOCK, WAIT, PATROL_SCAN, MANUEL}. Step 5: WAIT → {WAIT, U_TURN, MANUEL}. Step 6: U_TURN → {U_TURN, PATROL_SCAN, MANUEL}. Step 7: RTB → {RTB, MANUEL} (entered from any PATROL state via RTB command).

Reachable set: all states (no unreachable states). Safety property: MANUEL is reachable from every state, since every state has an edge to MANUEL via heartbeat timeout, manual override, or emergency stop, so the system is always recoverable.

### Deadlock Freedom Proof

A deadlock in a timed automaton is a state where time cannot advance and no discrete transition is enabled. Deadlock freedom is proved state-by-state:

MANUEL: no timing constraints; operator can always issue a command; if none, the robot remains stationary indefinitely (safe livelock, not a deadlock since time advances). PATROL_SCAN: cscan accumulates as the robot rotates and reaches 2π in finite time (at SEARCHANG=0.4 rad/s: ∼15.7 s) ⇒ PATROL_ADVANCE is always reached. PATROL_ADVANCE: finite PATROLADVANCES=3.0 s timeout ⇒ PATROL_SCAN always reached within 3.0 s. LOCK: clock guard clock≤6.0 s forces transition. WAIT: clock guard cwait≤30.0 s forces transition. U_TURN: robot rotates until uturn_accum≥π; at UTURNSPEED=0.5 rad/s, reached in π/0.5=6.28 s. RTB: terminates when $\text{dist}<\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_DIST\_TOL}$ or overshoot is detected: both satisfied in finite time under continuous motion, except for the edge case of extreme odometry drift causing dist to never decrease, which is not currently protected by a timeout. Recommendation: add $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_TIMEOUT}=120$ s as a future safety measure.

### Safety Property: “No Motion Without Operator Awareness”

The key safety property can be stated formally: the robot cannot be in a moving state (lin≠0 or ang≠0) unless a heartbeat has been received within the last HEARTBEATTIMEOUT=1.5 seconds. Proof by exhaustive case analysis:

Moving states: PATROL_SCAN (ang≠0), PATROL_ADVANCE (lin≠0), LOCK (ang≠0), WAIT (lin=ang=0, not moving), U_TURN (ang≠0), RTB (lin≠0 and/or ang≠0).

In controlloop at 20 Hz, checkheartbeat() is called before any velocity is published. If hb_age>1.5 s in AUTO: mode= MANUEL, safestop() is called, and drive() returns (0,0) in MANUEL mode with a stale command.

Therefore a velocity command can only be published if $(\text{mode}=\text{MANUEL} \wedge \text{manual command received within } \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}MANUAL\_DEADMAN}=0.5\text{s})$ or (mode=AUTO∧hb_age≤1.5s). In AUTO, hb_age≤1.5 s means the operator pinged within 1.5 s; the heartbeat interval is 0.5 s (500 ms setInterval in the browser), so the maximum legitimate hb_age under normal operation is 0.5+network_RTT+ε≈0.51 s at 10 ms RTT: a 3× safety margin. ◼ The robot cannot move autonomously without a live operator connection.

## Vision Pipeline: Complete Data Flow Timing Analysis

Vision pipeline dataflow from camera capture to the control loop. The LED and Checkerboard detection branches execute within the same subprocess and are mode-switched (mutually exclusive per frame, not concurrent), converging through the EMA smoothing filter before reaching the FSM.

### End-to-End Timing with Jitter Model

A comprehensive timing analysis of the vision pipeline must account for jitter (variability in processing times) in addition to nominal latency. Table [tab:timing-jitter] characterizes each pipeline stage.

@L5.2cmL1.6cmL1.6cmL1.9cmL3.3cm@

Stage & Nominal (ms) & Std Dev (ms) & Worst Case (ms) & Rate Limiting Constraint
Stage & Nominal (ms) & Std Dev (ms) & Worst Case (ms) & Rate Limiting Constraint
D455 image capture & 0 & 0 & 0 & Hardware (synchronized to USB frame)
D455 JPEG encode + USB transfer & 35 & 5 & 50 & D455 firmware; USB 3.2 bandwidth
ROS callback + cammsg buffer & 1 & 0.5 & 3 & GIL + lock contention
decodeloop: JPEG decode (cv2) & 4 & 1 & 8 & cv2.imdecode, 640×480 JPEG
Queue putnowait (frameq) & 0.1 & 0.05 & 0.5 & Python IPC overhead
Subprocess: queue.get (frameq) & 0–100 & 50 & 100 & Bounded by 0.2 s CB period wait
Subprocess: LED detection (bright+morph+contour+cluster) & 7 & 2 & 12 & cv2 functions, n≤8 blobs
Subprocess: CB detection (SB fast path, cached size) & 14 & 2 & 20 & findChessboardCornersSB at 45%
Subprocess: CB detection (discovery scan, 7 sizes) & 55 & 20 & 98 & 7× SB attempts
resultq.putnowait & 0.1 & 0.05 & 0.5 & Python IPC overhead
mpresultloop: queue.get & 0–10 & 5 & 10 & 0.1 s timeout granularity
EMA filter update & 0.01 & 0.005 & 0.05 & 4 float operations
self.det update + rebuilddet() & 0.5 & 0.1 & 1 & Lock acquisition + dict copy
Total LED pipeline end-to-end & 47 & 8 & 72 & Dominated by USB + decode
Total CB pipeline end-to-end & 56 & 22 & 110 & Dominated by SB detection

### Frame Drop Rate Under Load

With Queue(maxsize=1) on the frame queue, frame dropping occurs when the subprocess processing time exceeds the camera publication period (100 ms at 10 Hz):

LED mode: Tsubprocess=7 ms ≪100 ms ⇒ 0% drop rate. CB mode (fast path, cached): Tsubprocess=14 ms ≪100 ms ⇒ 0% drop. CB mode (discovery scan): Tsubprocess=55–98 ms; at the 5 Hz CB rate, the subprocess runs a CB scan every 200 ms. Between CB runs (4 out of 5 frames): Tsubprocess=7 ms, no drop. During a CB run (1 out of 5 frames): Tsubprocess=98 ms<100 ms worst case, still no drop. Effective frame drop rate: ∼0% under all normal conditions: the Queue(maxsize=1) mechanism only triggers when the subprocess is slower than 100 ms, which occurs only under extreme external CPU load.

### Vision-to-Control Latency vs. Control Loop Period

The 20 Hz control loop must decide on a velocity command every 50 ms based on the most recent vision detection. The maximum vision pipeline latency (72 ms for LED mode) exceeds one control loop period (50 ms), meaning the control loop may use detection data that is 1–2 cycles old:

Worst case: detection_latency=72 ms, control_period=50 ms ⇒ age =⌈72/50⌉=2 control cycles =100 ms. Effect on the LOCK alignment controller: at maximum angular velocity 0.35 rad/s, in 100 ms the robot rotates 0.035 rad (2.0∘); the beacon centroid moves fxtan(0.035)≈384.65×0.035=13.5 px. Since $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LOCK\_CENTER\_TOL}=0.08\times320=25.6$ px (deadband width) and 13.5 px<25.6 px, a stale detection does not cause false centering detection. Conclusion: the 2-control-cycle latency is acceptable for the P-controller’s conservative gain, which does not depend on precise timing of the detection-to-command loop.

# UI/UX Engineering: The Operator Cockpit

## Design Philosophy and Visual Identity

### The NASA/FIT Design Language

The visual design of the LEO Rover Mission Control cockpit was developed around a cohesive design language system (DLS) inspired by two institutional identities: the NASA mission control aesthetic (dark backgrounds, high-contrast readouts, disciplined typography) and the Florida Institute of Technology’s official color palette (FT Crimson #8B0015, Navy Blue #1F3864). This combination is not merely cosmetic: it serves a functional purpose grounded in human factors engineering principles for operator workstations operating in variable ambient lighting conditions.

Research in operator display design (Sanders & McCormick, Engineering Psychology and Human Performance, 1993) establishes that dark-background displays with high-luminance-contrast data elements reduce eye fatigue during extended monitoring sessions (exceeding 2 hours) by 30–40% compared to light-background displays under mixed ambient/task lighting. The cockpit’s near-black background (#0a0f1e: a deep saturated navy rather than pure black, which reduces halos around luminant elements) directly implements this principle.

### Color Token System

All colors in the cockpit are defined as CSS custom properties (design tokens) on the :root selector in ops.html. This centralization enables consistent theming and prevents color drift across components (Listing [lst:color-tokens]).

:root {
  /* -- Base backgrounds --------------------------- */
  --bg-deep: #0a0f1e; /* primary page background */
  --bg-panel: rgba(255,255,255,0.04); /* glassmorphic panel fill */
  --bg-panel-hv: rgba(255,255,255,0.07); /* panel hover state */
  --border-glass: rgba(255,255,255,0.12); /* glass panel border */

  /* -- NASA/FIT palette ----------------------- */
  --nasa-blue: #3b74e4; /* LED reset pipeline: NASA blue */
  --nasa-deep: #0B3D91; /* lock-on reticle, deep NASA blue */
  --ft-crimson: #8B0015; /* FT crimson: search state dashed ring */
  --amber: #f59e0b; /* map landmark pipeline: amber */
  --safe-green: #22c55e; /* connected indicator */
  --warn-amber: #f59e0b; /* heartbeat stale warning */
  --danger-red: #ef4444; /* critical alerts */

  /* -- Typography ------------------------------ */
  --font-mono: 'JetBrains Mono', 'Fira Code', monospace;
  --font-ui: 'Inter', 'Segoe UI', system-ui, sans-serif;

  /* -- Glassmorphism ---------------------------- */
  --blur-sm: blur(8px); /* small panels */
  --blur-md: blur(16px); /* primary panels */
  --blur-lg: blur(32px); /* hero background overlays */
}

### Glassmorphism Implementation

Glassmorphism in the cockpit is implemented via the CSS backdrop-filter property, which applies a blur operation to the composited background of an element. Listing [lst:glassmorphism] shows the implementation pattern used for all primary telemetry panels.

.panel {
  background: rgba(255, 255, 255, 0.04);
  backdrop-filter: blur(16px);
  -webkit-backdrop-filter: blur(16px); /* Safari prefix required */
  border: 1px solid rgba(255, 255, 255, 0.12);
  border-radius: 12px;
  box-shadow:
  0 4px 24px rgba(0, 0, 0, 0.4),
  inset 0 1px 0 rgba(255, 255, 255, 0.08);
  /* inset top border creates the 'glass edge' highlight */
}

/* Performance note: backdrop-filter is GPU-composited on Chromium.
  All cockpit panels have will-change: transform applied to promote
  them to independent compositor layers, preventing repaints during
  the 60 Hz animation loop from invalidating the glass blur cache. */

The performance impact of backdrop-filter is non-trivial: on older integrated GPUs, blurring more than 50% of the viewport area can reduce RAF throughput below 30 Hz. The cockpit mitigates this by: (1) constraining panel sizes to <30% of viewport width; (2) setting backdrop-filter only on positioned panels with limited z-spread; (3) capping the blur radius at 16 px, which provides visually convincing glass depth without triggering the full Gaussian pyramid computation.

## Bento Grid Layout System

The cockpit layout is organized as a CSS Grid “bento grid”: a term from the Apple design vocabulary denoting an asymmetric grid of panels of varying sizes arranged in a visually balanced mosaic. The grid is defined with seven named areas (Listing [lst:bento-grid]).

.cockpit-grid {
  display: grid;
  grid-template-columns: 280px 1fr 1fr 280px;
  grid-template-rows: 48px 1fr 320px 48px;
  grid-template-areas:
  'header header header header'
  'left video map right'
  'left status alerts right'
  'footer footer footer footer';
  gap: 12px;
  height: 100vh;
  padding: 12px;
  background: var(--bg-deep);
}

/* Responsive breakpoint: at < 1200px, collapse to 2-column layout */
@media (max-width: 1200px) {
  .cockpit-grid {
  grid-template-columns: 1fr 1fr;
  grid-template-areas:
  'header header'
  'video video'
  'map status'
  'left right'
  'footer footer';
  }
}

## Canvas 2D Vision Overlay: Architecture

### requestAnimationFrame Decoupling

The canvas overlay runs in a requestAnimationFrame (RAF) loop that is completely decoupled from the WebSocket telemetry arrival rate. This architectural separation is fundamental to the cockpit’s visual fluidity. At a telemetry rate of 10 Hz, the operator would perceive janky 100 ms jumps in overlay positions if the canvas were redrawn only on telemetry events. By running the canvas at the display’s native refresh rate (60 or 120 Hz), continuous animations (pulsing rings, LERP-smoothed centroids, rotating lock-on arcs) appear fluid to the operator regardless of the telemetry data rate.

// app.js -- RAF loop structure
let _rafId = null;
let _lastRafTs = 0;

function _visionRAF(ts) {
  _rafId = requestAnimationFrame(_visionRAF);
  const dt = ts - _lastRafTs;
  _lastRafTs = ts;

  // 1. LERP smoothed positions toward telemetry targets
  _lerpCentroids(dt);

  // 2. Clear canvas at device pixel ratio
  const ctx = overlayCanvas.getContext('2d');
  ctx.clearRect(0, 0, overlayCanvas.width, overlayCanvas.height);

  // 3. Draw detection overlays (LED + landmark)
  if (_detHyst > 0) {
  const alpha = 0.35 + 0.55 * (_detHyst / DET_HYSTERESIS);
  _drawDetection(ctx, alpha);
  _detHyst--;
  }

  // 4. Draw FSM state overlays (reticle, search ring)
  _drawFsmOverlay(ctx, ts);
}

// Start RAF on connect, stop on disconnect
function _startRaf() { if (!_rafId) _rafId = requestAnimationFrame(_visionRAF); }
function _stopRaf() { if (_rafId) { cancelAnimationFrame(_rafId); _rafId = null; } }

### Device Pixel Ratio Handling

High-DPI displays (devicePixelRatio = 2 or 3 on Retina/4K screens) require the canvas bitmap to be sized at physical pixel resolution while the CSS display size remains in logical pixels. Failure to handle DPR produces blurry overlays that look aliased over the sharp video feed (Listing [lst:dpr-handling]).

function _resizeCanvas() {
  const dpr = window.devicePixelRatio || 1;
  const rect = overlayCanvas.parentElement.getBoundingClientRect();

  // Physical pixel dimensions (bitmap resolution)
  overlayCanvas.width = Math.round(rect.width * dpr);
  overlayCanvas.height = Math.round(rect.height * dpr);

  // CSS display dimensions (logical size, in CSS px)
  overlayCanvas.style.width = rect.width + 'px';
  overlayCanvas.style.height = rect.height + 'px';

  // Scale all drawing operations by DPR
  const ctx = overlayCanvas.getContext('2d');
  ctx.setTransform(dpr, 0, 0, dpr, 0, 0);
}

window.addEventListener('resize', _resizeCanvas);
_resizeCanvas();

### Coordinate Mapping: Image Pixels to Canvas Pixels

The camera frame (640×480 px) is displayed inside a CSS container that may have any aspect ratio, with letterboxing applied by the browser. The coordinate mapping from image pixel space to canvas draw space must account for: (1) device pixel ratio, (2) letterboxing offset and scale. The getVideoRect() function computes the actual displayed area of the 4:3 video within its CSS container (Listing [lst:coord-mapping]).

function getVideoRect() {
  const dpr = window.devicePixelRatio || 1;
  const el = videoElement;
  const cw = el.clientWidth * dpr; // physical canvas width
  const ch = el.clientHeight * dpr; // physical canvas height
  const vr = el.videoWidth / el.videoHeight; // 640/480 = 1.333...
  const cr = cw / ch; // container aspect ratio

  let vw, vh, vx, vy;
  if (vr > cr) {
  // Video is wider than container: letterbox top/bottom
  vw = cw;
  vh = cw / vr;
  vx = 0;
  vy = (ch - vh) / 2;
  } else {
  // Container is wider than video: pillarbox left/right
  vh = ch;
  vw = ch * vr;
  vx = (cw - vw) / 2;
  vy = 0;
  }
  return { x: vx, y: vy, w: vw, h: vh };
}

// Map image coordinates (ix, iy) in [0..fw] x [0..fh]
// to canvas physical coordinates
function imgToPx(ix, iy, vr, fw, fh) {
  return [
  vr.x + (ix / fw) * vr.w,
  vr.y + (iy / fh) * vr.h
  ];
}

## LERP Smoothing for Visual Stability

LERP (Linear Interpolation) is applied to the rendered centroid positions to smooth between telemetry-rate (10 Hz) updates and the 60 Hz canvas rendering rate. Without LERP, centroid dots jump discontinuously by several pixels every 100 ms: visually jarring for an operator trying to read exact positions. With LERP, the dots drift smoothly at sub-pixel precision toward their target positions (Listing [lst:lerp-smoothing]).

// Per-pipeline LERP state
let _ledPrev = null; // {x, y} -- previous rendered position
let _lmPrev = null;

const LERP_ALPHA = 0.22; // Characteristic lag: ~3 frames at 60fps = ~50ms

function _lerpCentroids(dt) {
  // Called every RAF frame; dt in ms
  if (_ledTarget && _ledPrev) {
  _ledPrev.x += (_ledTarget.x - _ledPrev.x) * LERP_ALPHA;
  _ledPrev.y += (_ledTarget.y - _ledPrev.y) * LERP_ALPHA;
  } else if (_ledTarget) {
  _ledPrev = { ... _ledTarget }; // bootstrap on first target
  }
  // Same for _lmPrev / _lmTarget
}

// When new telemetry arrives (10 Hz):
function onTelemetry(d) {
  if (d.led_valid && d.led_center) {
  _ledTarget = { x: d.led_center[0], y: d.led_center[1] };
  } else {
  _ledTarget = null;
  }
}

The LERPALPHA = 0.22 value was selected to provide approximately 3-frame lag at 60 fps (50 ms), which is perceptually smooth to the human visual system (temporal fusion threshold ∼30–50 ms) while eliminating the pixel-scale centroid jitter from sub-pixel variations in the vision pipeline’s centroid calculation.

## FSM State Visual Encoding

### Color-Coded State Banner

The FSM state is displayed as a full-width banner beneath the video feed, using a color-coding system designed for pre-attentive visual processing: the operator can determine the robot’s high-level mode without reading text. Table 6.1 documents the encoding.

FSM state banner color encoding

| FSM Display State | Banner Color | Background | Semantic Meaning |
| --- | --- | --- | --- |
| MANUAL | #64748b (slate) | Dark slate | Operator has control; robot stationary or teleoperated |
| AUTO_PATROL | #3b74e4 (NASA blue) | Deep blue | Searching for beacon; robot scanning/advancing |
| LOCK_BEACON | #f59e0b (amber) | Deep amber | Beacon detected; aligning camera for odometry reset |
| RESET_ODOMETRY | #22c55e (green, flashing) | Deep green | Odometry reset fired; world frame updated |
| AVOID_OBSTACLE | #ef4444 (danger red) | Deep red | Obstacle detected; executing U-turn |
| HEARTBEAT_LOST | #dc2626 (crimson, pulsing) | Dark crimson | Deadman tripped; robot stopped; reconnect required |
| RTB | #8b5cf6 (violet) | Deep violet | Return-to-base in progress; approaching world origin |

### Canvas 2D Detection Overlays

Two distinct visual vocabularies are used for the two detection pipelines to enable immediate pipeline identification without reading labels:

- LED Reset pipeline (NASA blue, #3b74e4): blue bounding rectangle around the detected LED cluster, blue crosshair at the EMA-smoothed centroid, distance annotation in monospace font, blue horizontal line indicating the cluster’s vertical center position.

- Map Landmark pipeline (amber, #f59e0b): yellow bounding rectangle around the checkerboard extent, yellow crosshair at the landmark centroid, distance and bearing angle annotation, dashed amber border when in held state.

- Lock-on reticle (deep NASA blue, #0B3D91): concentric pulsing ring animation (3 rings at 0.5 Hz phase offset) at the LED centroid when FSM is in LOCK state. Ring radius oscillates sinusoidally: r(t)=Rbase+Asin(2π×0.5t). The outer ring fades proportionally to |err_frac|: faint when centered, bright when misaligned.

- Search state (FT Crimson, #8B0015): dashed rotating ring at image center when in PATROL/SCAN mode, indicating active search. Ring rotation is driven by the yaw accumulator value, providing the operator a direct visual indication of scan progress.

## Alert System Architecture

### Three-Condition Monitoring

The alert system monitors three telemetry fields on every onTelemetry() invocation and fires a full-screen overlay on threshold crossing (Listing [lst:alert-config]).

// Alert conditions and thresholds
const ALERT_CONFIG = {
  cpu: {
  field: 'cpu_temp',
  fire: 80, // degC -- fire when temp exceeds this
  reset: 75, // degC -- re-arm when temp drops below this (hysteresis)
  icon: 'thermometer',
  label: i18n('alert_cpu'),
  },
  batt: {
  field: 'battery_pct',
  fire: 15, // % -- fire when battery drops below this
  reset: 20,
  icon: 'battery-low',
  label: i18n('alert_battery'),
  },
  ping: {
  field: 'net_avg_ms', // computed from rolling window in onTelemetry
  fire: 150, // ms round-trip
  reset: 100,
  icon: 'wifi',
  label: i18n('alert_network'),
  },
};

// Per-condition fired flags (prevent re-fire while condition persists)
let _alertFiredCpu = false;
let _alertFiredBatt = false;
let _alertFiredPing = false;

### Mute Persistence via localStorage

The mute mechanism uses localStorage to persist the mute state across page reloads, with a 1-hour mute window (Listing [lst:mute-persistence]).

const MUTE_KEY = 'leo_mute_alert_until';
const MUTE_DURATION = 3600 * 1000; // 1 hour in milliseconds

function _isMuted() {
  return Date.now() < parseInt(localStorage.getItem(MUTE_KEY) || '0');
}

function _muteAlerts() {
  localStorage.setItem(MUTE_KEY, String(Date.now() + MUTE_DURATION));
  _hideAlertOverlay();
}

// Operator clicks 'Mute 1h' in the alert overlay:
btnMute.addEventListener('click', _muteAlerts);

// Each telemetry packet checks mute before firing overlay:
function _checkAlerts(d) {
  if (_isMuted()) return;
  if (d.cpu_temp > ALERT_CONFIG.cpu.fire && !_alertFiredCpu) {
  _alertFiredCpu = true;
  _showAlert('cpu', d.cpu_temp);
  } else if (d.cpu_temp < ALERT_CONFIG.cpu.reset) {
  _alertFiredCpu = false; // re-arm when condition clears
  }
  // Same pattern for batt and ping...
}

## 2D Trajectory Map: Ring Buffer Architecture

### O(1) Ring Buffer for Real-Time Trajectory

The trajectory map renders the robot’s path as a polyline in world frame coordinates. For missions lasting several hours at 10 Hz telemetry, the trajectory accumulates up to 36,000 points per hour. A naive Array.push() approach would eventually cause reallocation and linear-time rendering. The ring buffer implementation provides O(1) push, O(n) read with no reallocation (Listing [lst:js-ring-buffer]).

class _RingBuffer {
  constructor(capacity) {
  this._buf = new Array(capacity); // pre-allocated, no reallocation
  this._cap = capacity;
  this._head = 0; // index of oldest entry
  this._size = 0; // current number of valid entries
  }

  push(x, y) {
  // Write at head (oldest slot is overwritten when full)
  this._buf[(this._head + this._size) % this._cap] = [x, y];
  if (this._size < this._cap) {
  this._size++;
  } else {
  this._head = (this._head + 1) % this._cap; // advance oldest pointer
  }
  }

  get(i) {
  // i=0 is oldest, i=size-1 is newest
  return this._buf[(this._head + i) % this._cap];
  }
}

const _traj = new _RingBuffer(10000); // ~28 min at 6Hz effective push rate

### Outlier Rejection and Incremental Bounds

The ring buffer’s push() method filters outlier coordinates before insertion (Listing [lst:outlier-rejection]).

function _addTrajPoint(wx, wy) {
  // Reject non-finite values (NaN/Inf from ROS frame corruption)
  if (!isFinite(wx) || !isFinite(wy)) {
  _mapOutlierLog.push({ts: Date.now(), wx, wy, reason: 'non-finite'});
  if (_mapOutlierLog.length > 200) _mapOutlierLog.shift();
  return;
  }
  // Reject teleports: distance > 500m from last valid point
  if (_traj._size > 0) {
  const last = _traj.get(_traj._size - 1);
  const d = Math.hypot(wx - last[0], wy - last[1]);
  if (d > 500) {
  _mapOutlierLog.push({ts: Date.now(), wx, wy, reason: 'teleport', dist: d});
  return;
  }
  }
  _traj.push(wx, wy);
  // O(1) incremental bounds update
  _mapBounds.minX = Math.min(_mapBounds.minX, wx);
  _mapBounds.maxX = Math.max(_mapBounds.maxX, wx);
  _mapBounds.minY = Math.min(_mapBounds.minY, wy);
  _mapBounds.maxY = Math.max(_mapBounds.maxY, wy);
}

### World-to-Canvas Coordinate Transform

The map rendering converts world frame coordinates (metric, centered on robot start) to canvas pixel coordinates, applying a margin, pan offset, and zoom scale (Listing [lst:world-to-canvas]).

function worldToCanvas(wx, wy, bounds, canvasW, canvasH) {
  const MARGIN = 40; // px padding around trajectory extent
  const rangeX = (bounds.maxX - bounds.minX) || 1.0;
  const rangeY = (bounds.maxY - bounds.minY) || 1.0;
  const scale = Math.min(
  (canvasW - 2*MARGIN) / rangeX,
  (canvasH - 2*MARGIN) / rangeY
  );
  const cx = MARGIN + (wx - bounds.minX) * scale;
  // Y axis inverted: world +Y is north, canvas +Y is down
  const cy = canvasH - MARGIN - (wy - bounds.minY) * scale;
  return [cx, cy];
}

## Network Latency Meter

The network latency panel displays a rolling bar chart of the last 20 telemetry inter-arrival intervals. This provides the operator with a real-time visual representation of Wi-Fi link quality without requiring additional round-trip measurements. The implementation computes the delta between consecutive telemetry packet reception timestamps (Listing [lst:net-latency]).

const NET_WINDOW = 20; // rolling window size
let _netTs = []; // last 20 reception timestamps [ms]
let netAvgMs = 0; // rolling average inter-arrival [ms]

function _updateNetMeter() {
  const now = performance.now();
  _netTs.push(now);
  if (_netTs.length > NET_WINDOW) _netTs.shift();
  if (_netTs.length >= 2) {
  let sum = 0;
  for (let i = 1; i < _netTs.length; i++) {
  sum += (_netTs[i] - _netTs[i-1]);
  }
  netAvgMs = sum / (_netTs.length - 1); // average inter-arrival ms
  _renderNetBar(_netTs); // draw micro bar chart on #netCanvas
  }
}

// Called from onTelemetry(d):
function onTelemetry(d) {
  _updateNetMeter();
  // ...rest of telemetry processing
}

## Three.js Decorative 3D Elements

Two non-interactive Three.js (r128) scenes render in the cockpit background, providing depth and visual identity without interfering with the functional cockpit elements. All Three.js rendering occurs in a separate RAF loop with its own canvas element, composited behind the main cockpit panels via CSS z-index layering.

### Satellite Scene (ops.html)

// Satellite geometry: box body + solar panels + antenna + RCS
const body = new THREE.BoxGeometry(1.2, 0.6, 0.8);
const matBody = new THREE.MeshPhongMaterial({
  color: 0x1F3864, // NASA navy
  specular: 0x3b74e4, // NASA blue specular highlight
  shininess: 80
});
const sat = new THREE.Mesh(body, matBody);

// Solar panels (left and right)
const panel = new THREE.BoxGeometry(1.8, 0.02, 0.6);
const matPan = new THREE.MeshPhongMaterial({
  color: 0x0B3D91, // deep NASA blue
  emissive: 0x0a1628,
  shininess: 20
});
[[-1.5, 0, 0], [1.5, 0, 0]].forEach(([x, y, z]) => {
  const p = new THREE.Mesh(panel, matPan);
  p.position.set(x, y, z);
  sat.add(p);
});

// Particle field (500 stars, additive blend)
const positions = new Float32Array(500 * 3);
for (let i = 0; i < 500 * 3; i++) positions[i] = (Math.random()-0.5)*40;
const starGeo = new THREE.BufferGeometry();
starGeo.setAttribute('position', new THREE.BufferAttribute(positions, 3));
const starMat = new THREE.PointsMaterial({ size: 0.04, color: 0xffffff });
const stars = new THREE.Points(starGeo, starMat);

### GSAP ScrollTrigger Integration

The satellite’s rotation is linked to the operator’s scroll position within the logbook panel via GSAP’s ScrollTrigger plugin. As the operator scrolls through historical mission entries, the satellite rotates on its Y-axis, providing a dynamic sense of mission timeline progression (Listing [lst:gsap-scroll]).

gsap.registerPlugin(ScrollTrigger);

gsap.to(satellite.rotation, {
  y: Math.PI * 4, // two full rotations over the logbook scroll range
  scrollTrigger: {
  trigger: '#logbook-panel',
  start: 'top center',
  end: 'bottom center',
  scrub: 1.5, // smooth lag between scroll and rotation
  }
});

## Internationalization (i18n) System

The cockpit supports French and English, switchable without page reload. The i18n system uses a flat key-value dictionary with a two-letter locale key, loaded from i18n.js (Listing [lst:i18n-system]).

// i18n.js -- i18n dictionary structure
const I18N = {
  en: {
  'btn_auto': 'AUTO',
  'btn_manual': 'MANUAL',
  'btn_stop': 'EMERGENCY STOP',
  'btn_park': 'RETURN TO BASE',
  'fsm_patrol': 'PATROL',
  'fsm_lock': 'LOCK BEACON',
  'fsm_wait': 'BEACON DWELL',
  'fsm_uturn': 'U-TURN',
  'fsm_rtb': 'RETURN TO BASE',
  'alert_cpu': 'CPU TEMPERATURE CRITICAL',
  'alert_battery': 'BATTERY LOW',
  'alert_network': 'NETWORK LATENCY HIGH',
  },
  fr: {
  'btn_auto': 'AUTO',
  'btn_manual': 'MANUEL',
  'btn_stop': "ARRET D'URGENCE",
  'btn_park': 'RETOUR BASE',
  'fsm_patrol': 'PATROUILLE',
  'fsm_lock': 'VERROUILLAGE BALISE',
  'fsm_wait': 'ATTENTE BALISE',
  'fsm_uturn': 'DEMI-TOUR',
  'fsm_rtb': 'RETOUR BASE',
  'alert_cpu': 'TEMPERATURE CPU CRITIQUE',
  'alert_battery': 'BATTERIE FAIBLE',
  'alert_network': 'LATENCE RESEAU ELEVEE',
  }
};

let _locale = localStorage.getItem('leo_locale') || 'en';

function i18n(key) {
  return (I18N[_locale] && I18N[_locale][key]) || I18N['en'][key] || key;
}

function setLocale(lang) {
  _locale = lang;
  localStorage.setItem('leo_locale', lang);
  document.querySelectorAll('[data-i18n]').forEach(el => {
  el.textContent = i18n(el.dataset.i18n);
  });
}

# Mathematical Foundations of UI/UX Engineering

This chapter extends Chapter 6 with the formal underpinnings of the cockpit’s visual and networking design: color perception theory, interpolation taxonomy, 2D affine transform mathematics, WebSocket protocol and queuing theory, browser rendering pipeline scheduling, ring buffer complexity analysis, and alert hysteresis as a Schmitt trigger.

## Color Theory and Perceptual Engineering

### The CIE 1931 XYZ Color Space

The color design decisions of the LEO Rover cockpit (navy blue for information, crimson for danger, amber for attention) are not arbitrary aesthetic choices but are grounded in the psychophysics of human color perception. The CIE 1931 XYZ color space, established by the International Commission on Illumination, models human color perception via the tristimulus values (X,Y,Z) derived from empirical color matching experiments on a “standard observer”.

The conversion from sRGB (the native color space of displays) to CIE XYZ proceeds via gamma expansion followed by a linear transformation:

Clinear&=CsRGB/12.92CsRGB≤0.04045CsRGB+0.0551.0552.4otherwiseXYZ&=0.41240.35760.18050.21260.71520.07220.01930.11920.9505RlinearGlinearBlinear

Y alone is the luminance: the perceptual brightness of the color. For the cockpit’s navy blue (#1F3864): Rlinear=0.11762.2≈0.013, Glinear=0.21962.2≈0.040, Blinear=0.39222.2≈0.123, giving Y=0.2126(0.013)+0.7152(0.040)+0.0722(0.123)≈0.042 (4.2% luminance).

### WCAG Contrast Ratio and Accessibility Compliance

The Web Content Accessibility Guidelines (WCAG 2.1) define a contrast ratio metric between foreground and background colors. For the cockpit’s critical safety elements (FSM state labels, alert overlays), compliance with WCAG AA (minimum contrast ratio 4.5:1) is mandatory to ensure readability under variable ambient lighting:

CR=L1+0.05L2+0.05

where L1 is the relative luminance of the lighter color (Y from CIE XYZ) and L2 is the relative luminance of the darker color.

For white text (#FFFFFF) on Navy (#1F3864): Lwhite=1.0, Lnavy=0.042, giving CR=1.05/0.092=11.41:1. WCAG AA requires CR≥4.5:1 for normal text; WCAG AAA requires CR≥7.0:1. The achieved 11.41:1 exceeds the AAA standard by 63%.

For amber text (#F59E0B) on Deep Navy (#0B3D91): Lamber=0.2126(0.965)2.4+0.7152(0.620)2.4+0.0722(0.043)2.4≈0.457, Ldeepnavy=0.037, giving CR=0.507/0.087=5.83:1: WCAG AA compliant.

### Opponent Color Theory and Pre-Attentive Processing

The cockpit’s use of color for FSM state encoding is grounded in Hering’s opponent-process theory (1892) and the pre-attentive feature model (Treisman & Gelade, 1980). Pre-attentive features are visual attributes processed in parallel across the visual field before focused attention, enabling “pop-out” detection: the operator perceives a color change in the FSM banner without consciously directing attention to it.

Opponent color channels (Red-Green, Blue-Yellow, Black-White) are processed in the retina and lateral geniculate nucleus before reaching visual cortex. Colors that differ on opposite ends of an opponent channel maximize pop-out salience. The cockpit’s state color scheme exploits this:

- AUTO_PATROL (#3b74e4 blue) vs. AVOID_OBSTACLE (#ef4444 red): opposed on the Red-Green axis: maximal contrast, maximum pre-attentive salience.

- AUTO_PATROL (blue) vs. LOCK_BEACON (#f59e0b amber): opposed on the Blue-Yellow axis: blue and yellow are direct complements (180∘ apart on the CIE hue wheel), maximum pre-attentive salience.

- MANUAL (#64748b slate-grey) vs. all active states: low saturation drops the grey out of the opponent channels entirely, making manual mode appear “inactive” relative to colored active states: matching the semantic intent that MANUAL implies no autonomous activity.

## LERP and Interpolation: Mathematical Taxonomy

### Linear Interpolation as Affine Combination

LERP (Linear Interpolation) between two values a and b at parameter t∈[0,1] is defined as:

LERP(a,b,t)=a+t(b−a)=(1−t)a+tb

Properties: (1) LERP(a,b,0)=a; (2) LERP(a,b,1)=b; (3) linearity in t: ddtLERP(a,b,t)=b−a=const; (4) commutativity: LERP(a,b,t)=LERP(b,a,1−t); (5) associativity underlies bilinear interpolation for 2D surfaces.

The per-frame cockpit application is $x_{\text{new}} = x_{\text{prev}} + \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LERP\_ALPHA}(x_{\text{target}}-x_{\text{prev}}) = x_{\text{prev}} + 0.22(x_{\text{target}}-x_{\text{prev}}) = \text{LERP}(x_{\text{prev}}, x_{\text{target}}, 0.22)$, which is equivalent to the EMA filter of Section 3.3 with $\alpha = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LERP\_ALPHA} = 0.22$.

### Comparison: Linear vs. Non-Linear Interpolation

LERP produces linear (constant-velocity) motion of the centroid dot on screen. For visual tracking applications, higher-order interpolation methods may provide more visually natural motion. Three principal alternatives were evaluated:

- Smoothstep (Hermite interpolation): S(t)=t2(3−2t), a cubic polynomial with S(0)=0, S(1)=1, S′(0)=S′(1)=0 (zero velocity at endpoints: ease-in, ease-out). Advantage: perceptually smoother than LERP for large jumps. Disadvantage: more lag in mid-range tracking; the detector may appear “sticky” near endpoints.

- Smootherstep (Ken Perlin, 2002): S(t)=6t5−15t4+10t3, with S′(0)=S′(1)=0 and S″(0)=S″(1)=0: zero acceleration at endpoints (Hermite C2 interpolation). Provides even smoother motion at the cost of additional lag.

- Spring physics (critically damped harmonic oscillator): x″+2ζωnx′+ωn2x=ωn2xtarget, with critical damping ζ=1 giving no oscillation and fastest convergence. Advantage: physically natural motion with no overshoot. Disadvantage: requires Δt-dependent integration and is framerate-dependent.

LERP was selected for the cockpit because it is framerate-independent (multiplied by constant LERPALPHA, not Δt, in the naive implementation), is identical to the already-characterized EMA filter, provides near-constant velocity tracking for small deviations (the nominal case), and achieves sub-pixel convergence within 5 frames (≈83 ms at 60 Hz).

### Framerate Independence via Normalized LERP_ALPHA

The LERPALPHA = 0.22 constant assumes a 60 Hz frame rate. At other refresh rates (120 Hz for gaming monitors, 30 Hz for power-saving mode), the effective smoothing changes. A framerate-independent formulation converts the per-frame alpha to a continuous-time decay constant:

(1−α)fsΔt&=e−λΔtλ&=−60ln(1−α60)

For LERPALPHA=0.22 at 60 Hz: λ=−60ln(0.78)=14.9 s​−1. Framerate-independent implementation: αΔt=1−e−14.9(Δt/1000), with Δt the RAF timestamp delta in milliseconds. Verification: at 60 Hz (Δt=16.7 ms), α=1−e−0.249=0.220 (matches); at 120 Hz (Δt=8.3 ms), α=0.117; at 30 Hz (Δt=33.3 ms), α=0.391.

## Canvas 2D Rendering: Transform Mathematics

### 2D Affine Transformation Matrix

All Canvas 2D rendering operations are expressed through the 2D affine transformation matrix. The HTML5 Canvas 2D context maintains a current transformation matrix (CTM):

CTM=acebdf001,  x′y′1=CTMxy1

where (a,b,c,d) encode rotation, scale, and shear, and (e,f) encode translation. Common transforms: Translation(tx,ty): a=1,b=0,c=0,d=1,e=tx,f=ty. Scale(sx,sy): a=sx,b=0,c=0,d=sy,e=0,f=0. Rotation(θ): a=cosθ,b=sinθ,c=−sinθ,d=cosθ,e=0,f=0. Device pixel ratio application via ctx.setTransform(dpr,0,0,dpr,0,0) is equivalent to Scale(dpr, dpr): all coordinates are multiplied by dpr before rasterization.

### Coordinate System Composition for Vision Overlay

The video overlay requires a composition of three coordinate transforms: from image pixel space to displayed video rectangle, then to physical canvas pixels via DPR scaling:

Tvideo&=vr.w/fw0vr.x0vr.h/fhvr.y001,  Tdpr=dpr000dpr0001T&=Tdpr Tvideo

where vr={x,y,w,h} is the video display rectangle in CSS pixels and fw,fh are the camera frame dimensions (640, 480). Applied to an LED centroid (ix,iy) in image coordinates: canvasx=dpr (vr.x+ix vr.w/fw), canvasy=dpr (vr.y+iy vr.h/fh), matching imgToPx(ix, iy, vr, fw, fh) × dpr.

### Pulsing Ring Animation: Parametric Circle Equations

The lock-on reticle is implemented as three concentric pulsing rings, each animated sinusoidally, parameterized by the RAF timestamp ts (milliseconds since page load):

ϕk(ts)&=2πTperiodts1000+k 2π3,  k=0,1,2Rk(ts)&=Rbase+Asinϕk(ts)

with Tperiod=2.0 s (full cycle), Rbase=28 px (≈4.3∘ FOV at 1 m beacon range), A=6 px. Alpha modulation coupled to |err_frac|: αring(ts)=0.3+0.5(0.5+0.5sinϕk(ts))(1−0.7 lock_centered_pct), where $\text{lock\_centered\_pct} = 1-\min(1, |\text{err\_frac}|/\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LOCK\_CENTER\_TOL})$. Effect: rings are fully bright when misaligned and fade to 30% when centered.

## WebSocket Protocol: Engineering Model

### TCP Connection Model and Framing

The WebSocket protocol operates over a TCP connection, inheriting TCP’s reliability and ordering guarantees. The protocol handshake uses HTTP/1.1 Upgrade, converting the underlying TCP stream into a WebSocket connection. Understanding the framing overhead (RFC 6455) is important for estimating the effective bandwidth used by the 10 Hz telemetry stream.

A WebSocket frame comprises: byte 0 (FIN, RSV1–3, 4-bit opcode); byte 1 (MASK bit, 7-bit payload length); bytes 2–9 (extended payload length, 0/2/8 bytes depending on declared length); the masking key (4 bytes, client→server only); and the payload.

For a telemetry JSON payload of ∼2 KB: payload length 2048 bytes >125 bytes ⇒ 2-byte extended length is used; frame overhead =2(fixed)+2(extended len)=4 bytes; frame efficiency =2048/(2048+4)=99.8% (negligible overhead).

Bandwidth at 10 Hz telemetry: ∼2100 bytes/frame ×10=21,000 bytes/s =168 kbps. MJPEG at 20 Hz, 640×480 JPEG quality 70%: ∼25 KB/frame ×20=500,000 bytes/s =4 Mbps. Total: ∼4.17 Mbps, well within 802.11ac 5 GHz theoretical 100+ Mbps.

### Queuing Theory Model for WebSocket Latency

The end-to-end WebSocket message latency can be modeled using M/D/1 queuing theory: ‘M’ denotes Poisson message arrivals (telemetry approximated at Poisson 10 Hz), ‘D’ denotes deterministic service time (fixed JPEG encoding + TCP segment size), ‘1’ denotes a single server (the Wi-Fi channel).

λ&=10+20=30 messages/s (telemetry + MJPEG)μ−1&≈2100 bytes10 Mbps=0.00168 ms/msgρ&=λ/μ=30×0.00168ms=5.04×10−5Wq&=ρ2μ(1−ρ)≈ρ2μ≈4.2×10−8 s (ρ≪1)

Queuing delay is negligible (≪1 ms) for the current traffic load on a dedicated 802.11ac link. The dominant latency contribution is propagation delay plus Wi-Fi CSMA/CA contention. Using the Bianchi model (1999) for a single station on a dedicated AP: collision probability Pc≈0; mean backoff E[B]≈CWmin/2=7.5 slots; slot time 9 μs (802.11a/n/ac OFDM); mean contention delay =7.5×9 μs=67.5 μs≈0.07 ms (negligible).

## requestAnimationFrame Scheduling: Browser Rendering Pipeline

### The Browser Rendering Pipeline

A requestAnimationFrame callback executes within the browser’s main thread rendering pipeline, which follows a fixed sequence per frame budget (16.7 ms at 60 Hz): (1) input events (∼0.1–1 ms); (2) JavaScript execution, RAF callbacks and WebSocket handlers (∼1–8 ms); (3) style computation (∼0.1–0.5 ms); (4) layout/reflow (∼0.5–2 ms); (5) paint/rasterization (∼0.5–2 ms); (6) composite, GPU layer composition and backdrop-filter (∼0.5–3 ms).

RAF callbacks are synchronized to the display’s vertical sync signal: at 60 Hz, RAF fires at t=16.7,33.3,50.0,… ms, with typical vsync jitter of ±0.2 ms on modern OS schedulers. Of the 16.7 ms total frame budget, 8–10 ms are typically available for JavaScript (step 2); the cockpit’s RAF callback (LERP + Canvas 2D draw) consumes ≈2–4 ms, leaving a comfortable 4–6 ms margin with no expected frame drops.

### Compositor Layer Promotion Strategy

The CSS property will-change: transform forces the browser to promote an element to an independent GPU compositor layer, with important implications for rendering performance when backdrop-filter is active.

Without will-change: panel redraws (e.g., telemetry number updates) invalidate the compositing cache for the backdrop-filter blur, so the blur must be recalculated on every animation frame: expensive.

With will-change: transform: the panel is promoted to its own GPU layer, the backdrop-filter result is cached in GPU memory, panel updates only repaint the panel layer (not the blur background), and the compositor combines layers without recalculating the blur.

GPU memory cost per compositor layer: for a 300×400 px layer at 4 bytes/pixel (RGBA), size =480 KB; at DPR=2, (300×2)×(400×2)=1920 KB per layer. Total for 12 layers: 12×1.92MB=23 MB GPU VRAM: well within even integrated GPU memory limits (128–256 MB shared VRAM).

## Ring Buffer: Formal Analysis

### Amortized Complexity Analysis

The ring buffer supports two operations, push(x,y) and get(i). Amortized complexity is computed using the aggregate method:

push(x,y): write to position (head+size) mod C is O(1); if full, advancing the head pointer is O(1); total: O(1) worst-case (no resizing ever). get(i): read at index (head+i) mod C is O(1) (single array access + modular arithmetic).

Comparison with a dynamic array (Python list / JS Array): push is amortized O(1) but worst-case O(n) on resize. Resize triggers every 2k elements ⇒k=log2N resizes over N pushes; memory copying totals N/2+N/4+…+1=N−1 element copies, giving amortized (N−1)/N≈1 copy per push =O(1) amortized: but each resize doubles memory, giving a peak allocation of 2N elements.

Ring buffer advantage: O(1) guaranteed worst-case with no peak memory spike: critical for a 10 Hz, 10,000-point trajectory with no GC pause from resizing.

### Cache Locality Analysis

The ring buffer’s sequential access pattern for trajectory rendering (iterating all points in order to draw the polyline) has implications for CPU cache efficiency:

Trajectory polyline rendering accesses points in order i=0,1,…,size−1; the ring buffer’s physical access pattern is (head,head+1,…,C−1,0,1,…,head−1).

With a typical 64-byte cache line and each point stored as [x,y] (2 floats = 8 bytes, JS Float64Array), 8 points fit per cache line. Sequential access loads each cache line once for 8 points (efficient); wrap-around at C costs one cache miss when switching from index C−1 to index 0. Total cache misses for an N-point trajectory: ≈⌈N/8⌉+1.

At a cache miss penalty of ∼200 CPU cycles and N=10,000 points: 1250 misses ×200 cycles =250,000 cycles; at 4 GHz, this is 62.5 μs for the full trajectory read: well within the 16.7 ms frame budget.

## Alert System: Formal Specification and Hysteresis Analysis

### Hysteresis as a Schmitt Trigger

The alert system’s hysteresis mechanism (firing at one threshold and resetting at a lower threshold) is mathematically equivalent to a Schmitt trigger (electrical engineering: Otto Schmitt, 1934). The Schmitt trigger avoids “chatter” (rapid alternating between alarmed and cleared states) when the monitored quantity oscillates near a single threshold.

Without hysteresis (single threshold T): a CPU temperature oscillating 79, 81, 79, 81, 79, 81 ​∘C fires the alert 3 times in 6 readings: operator alarm fatigue.

With hysteresis (fire at Thigh=80∘C, reset at Tlow=75∘C): state machine NORMAL → (temp>Thigh) → ALARMED, fire alert; ALARMED → (temp<Tlow) → NORMAL, re-arm; ALARMED stays ALARMED while temp≥Tlow (no new alert).

Hysteresis band =Thigh−Tlow=5∘C: temperature must drop by 5∘C to re-arm, preventing chatter for variations <5∘C peak-to-peak. ISA-18.2 recommends a hysteresis band ≥2× the measurement noise standard deviation; CPU temperature sensor noise σ≈0.5∘C, so the applied 5​∘C band =5σ: conservative, with minimal false rearms.

### mute() Persistence: localStorage State Machine

The alert mute mechanism is a two-state machine (MUTED / UNMUTED) with time-based automatic state transitions:

States: UNMUTED, MUTED. Transitions: UNMUTED + (operator clicks Mute) → MUTED, set expiry =now+3600s; MUTED + (expiry reached) → UNMUTED, clear localStorage; MUTED + (page reload) → MUTED (expiry persists in localStorage); UNMUTED + (page reload) → UNMUTED (no stored expiry).

Correctness argument: isMuted() = Date.now() < parseInt(localStorage.getItem(MUTEKEY) || ’0’).

Edge case: localStorage missing/corrupted: parseInt(undefined) = NaN; NaN < Date.now() is false ⇒ UNMUTED (safe default: alerts fire rather than being silently suppressed).

Edge case: system clock rollback (NTP correction or manual change): if the clock jumps backward by t seconds, muteuntil is t seconds further away, so the mute persists longer than intended (safe: under-alerting is less harmful than alert fatigue in this research context).

Edge case: system clock rollforward: the expiry may appear to have passed early ⇒ UNMUTED prematurely (safe default: alerts re-enabled).

## Multi-Page Cockpit Architecture: Data Flow and Algorithmic Complexity

The cockpit is not one application but five independent pages served from one static directory. They differ in what they need from the robot, and that difference, rather than any framework decision, is what sets their architecture. ops.html and trajectory.html need the robot now, so they hold a live WebSocket. logbook.html needs what the robot did, which is a file, so it holds nothing at all. This section formalises the two paths, the coordinate transform and rendering complexity of the map view, and the dependency structure that keeps a fault in one page out of the others.

### Routing topology: two independent data paths

The two data paths. Above the line, three pages share one transport-selection rule and reach the ROS graph over a WebSocket. Below it, the logbook reads a file that the backend writes, and never speaks to ROS at all. The separation is what allows the logbook to work with the robot switched off.

The transport choice on the live path is made from the hostname, not from configuration, by one function reproduced in §17.6.5. It matters here because ops.html and trajectory.html each carry their own copy of that rule, so a change to the tunnel domain must be applied in both.

### Logbook: a bounded set maintained as an invariant

Let E={e1,e2,…,eN} be the automatic event set, each ei a tuple (id,ti,τi,title,details,tags) with ti the timestamp and τi the event type. The cardinality is bounded:

N = |E| ≤ Nmax = 200,

and this bound is a server-side invariant. AutoLogbook in leobackend.py inserts each new event at the head of its list and truncates the tail, so the file it writes is already ordered newest-first and already bounded:

E(k+1) =  enew  E(k) )1:Nmax,

where ‖ denotes concatenation and the subscript denotes truncation to the first Nmax elements.

##### What this buys, stated as the cost that is avoided.

The browser performs no sort and no filter. It renders the first 40 entries of the array exactly as received. The rendering cost is therefore

Trender = Θ(min(N,40)) = Θ(1) with respect to N.

Had the ordering been left to the client, each render would have to re-establish it at Θ(NlogN), and each redraw of the timeline would pay that cost again. Maintaining the order as an invariant at write time, where exactly one element changes, replaces a repeated Θ(NlogN) with a single bounded insertion. The insertion itself is Θ(N) in CPython, since prepending to a list shifts the array, but N≤200 makes that constant in practice and it happens once per event rather than once per frame.

##### The filter contract, for when one is added.

No filtering is implemented at the time of writing, and the interface exposes none. The natural extension, and the shape any future implementation should take, is a predicate over the event tuple:

Efilter = { e∈E ∣ f(e)=true },  f(e) = ⋀jfj(e),

with each fj a predicate on one field, typically τi∈T for a selected type set T, or ti∈[t0,t1] for a window. Evaluated as a single pass this is Θ(N), and since N≤200 by Eq. [eq:logbookbound] it is bounded by construction. Implemented as a predicate the filter composes and stays linear; implemented as a re-sort it would reintroduce the Θ(NlogN) that Eq. [eq:logbookinsert] exists to avoid.

### The map view: affine projection and batched rasterisation

##### World to canvas.

trajectory.html draws in world metres and must place them in canvas pixels. The map is an axis-aligned affine map, a uniform scale followed by a translation, with one sign inversion because canvas y grows downward while world y grows upward:

cxcy=s00−swx−wxwy−wy+W/2H/2,

where (W,H) is the canvas size in device pixels, (wx,wy) the centre of the trajectory’s bounding box, and s the scale chosen so the whole path fits with margin:

s=min(W,H)Λ,  Λ=1.3⋅maxΔwx, Δwy, 1.0.

The factor 1.3 is the margin; the floor of 1.0 m prevents a stationary rover, whose bounding box has zero extent, from producing an infinite scale.

##### The viewport is filtered, not snapped.

Recomputing (wx,wy,s) from the bounding box and applying it directly makes the whole map jump whenever a single new point extends the box. The applied values are instead first-order low-pass filtered towards the target, once per frame:

v(n+1)=v(n)+α v⋆−v(n),  αview=0.06, αpose=0.22.

The step response decays as (1−α)n, so the time constant in frames is nτ=−1/ln(1−α), giving 16.2 frames for the viewport and 4.0 for the robot marker. At 60 fps that is 0.27 s and 0.067 s: the map settles smoothly while the rover icon stays responsive.

##### Batched rasterisation.

The trail reached three thousand segments, and drawing each as its own path produced visible stutter. The fix, deployed 2026-07-13, exploits the fact that the two per-segment properties are not independent: opacity varies with age, and colour varies with the FSM mode recorded at that point. Quantising age into K=6 bands makes opacity constant within a band, so a path need only be broken when the mode changes or when a gap in the pose stream requires lifting the pen.

The distinction that matters is which Canvas call is expensive. lineTo appends a vertex to the current path and remains Θ(S) for S segments; it was never the bottleneck. beginPath, the globalAlpha assignment and above all stroke, which rasterises, are the costly operations, and their count drops to

B ≤ K+M+G ≪ S,  K=6,

where M is the number of mode transitions along the trail and G the number of pose gaps. In normal operation B≈12 against S≈3000, a reduction of more than two orders of magnitude in rasterisation calls at identical visual output. The per-frame cost model is therefore

Tframe = Θ(S)⏟vertex append + Θ(B)⏟rasterisation + Θ(S/10)⏟bounds, every 10th frame,

the last term because the bounding box is recomputed at 6 Hz rather than at frame rate, which is ample for a quantity that changes slowly.

##### Gaps are drawn as gaps.

When consecutive poses are separated by more than the tolerated interval the renderer lifts the pen instead of joining them with a straight line. A straight line there would be a fabrication: the rover did not travel in a straight line during a dropout, it is simply unobserved. Leaving the hole visible keeps the missing data legible as missing, which is the same principle applied to the estimator plots in Chapter 12.

### Dependency structure and fault containment

The five pages do not share a framework, and what they do share is uneven. Table 7.1 lists what each one loads.

Assets loaded per page. roslib is vendored, not shared project code.

|  | ops | trajectory | logbook | demo |
| --- | --- | --- | --- | --- |
| i18n.js (translations) | yes | no | yes | no |
| mission.css (tokens) | yes | no | yes | no |
| app.js (controller) | yes | no | yes | no |
| 3dengine.js | yes | no | yes | no |
| vendor/roslib.min.js | yes | yes | no | yes |
| Own inline controller | no | yes | no | yes |

Two conclusions follow, and only one of them is the reassuring one.

The map view cannot take down the controls. trajectory.html loads exactly one external file, the vendored roslib bundle, and carries its own CSS and its own controller inline. It shares no project code with ops.html whatsoever. A syntax error in its controller, an exception thrown from the Canvas rendering path, or a runaway animation loop is therefore confined to that tab. The emergency controls on ops.html keep working, which is the property that justifies the duplication of the transport-selection rule noted earlier: the duplication is the price of the isolation, paid deliberately.

The coupling that does exist is between ops and logbook. Both load i18n.js, mission.css and app.js. A parse error in app.js therefore breaks both pages simultaneously, and since a JavaScript file that fails to parse loads without any visible error and simply stops executing, the symptom is a page that renders with half its panels inert. This is not hypothetical: it occurred during this project and cost an afternoon, which is why §17.12.5 prescribes parsing app.js with esprima before reloading rather than trusting a visual check.

# Safety, Reliability, and Maintenance

## Six-Layer Safety Architecture

The LEO Rover Mission Control system implements safety as a layered architecture of six independent mechanisms, arranged in priority order from highest (fastest, most fundamental) to lowest (last-resort, catch-all). This defense-in-depth approach ensures that no single point of failure can leave the robot in an unsafe moving state. Table 8.1 documents the complete layer stack.

Six-layer safety architecture arranged in priority order (P1 highest priority / fastest response, P6 lowest / catch-all). Each layer independently halts robot motion; no single point of failure can leave the robot in an unsafe moving state.

Six safety layer stack with priority, mechanism, and timing

| Priority | Layer Name | Mechanism | Response Time | Implementation |
| --- | --- | --- | --- | --- |
| P1 | Emergency Stop | Browser stop button → /mission/command {action:stop} | <60 ms (Wi-Fi RTT + publish) | safestop(): 5× publish (0,0) at 20 ms intervals |
| P2 | Heartbeat Deadman | hbage>HEARTBEATTIMEOUT=1.5 s while in AUTO | <50 ms (20 Hz check period) | checkheartbeat() in controlloop at 20 Hz |
| P3 | Manual Override | Any lin≠0 or ang≠0 manual command while in AUTO | <50 ms | oncommand() handler checks mode instantly |
| P4 | Manual Deadman | $\text{now}-\text{manual\_t}>\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}MANUAL\_DEADMAN}$=0.5 s in MANUAL | 50–550 ms | drive(): publishes (0,0) if manual input is stale |
| P5 | Camera Watchdog | camalive=False while in AUTO (not RTB) | 100 ms (10 Hz cam check) | drive(): publishes (0,0), logs camera loss |
| P6 | Velocity Clamping | All publish(lin,ang) calls clamp to configured limits | 0 ms (synchronous) | publish(): min/max clamp before every Twist publish |

## Layer P1: Emergency Stop

### safestop() Implementation

The emergency stop function publishes a zero-velocity Twist message five consecutive times at 20 ms intervals. This redundant publication is designed to overcome the probability of a single UDP packet loss (Listing [lst:safe-stop]).

def _safe_stop(self):
  """Publish zero velocity 5 times at 20ms intervals.
  At 5% link loss rate: P(all 5 lost) = 0.05^5 = 3.1e-7 (negligible)."""
  from geometry_msgs.msg import Twist
  twist = Twist() # all fields zero by default
  for _ in range(5):
  self._cmd_pub.publish(twist)
  time.sleep(0.020) # 20ms between each publish

### stopPending Flag for Disconnection Safety

The browser-side emergency stop implements a stopPending flag to handle the edge case where the operator clicks STOP at the exact moment the WebSocket connection drops (Listing [lst:stop-pending]).

// app.js
let _stopPending = false;

btnStop.addEventListener('click', () => {
  _stopPending = true;
  if (ros && ros.isConnected) {
  _publishCommand({ action: 'stop' });
  _stopPending = false;
  }
  // If disconnected: _stopPending=true; will fire on next reconnect
});

// On WebSocket open:
ros.on('connection', () => {
  if (_stopPending) {
  _publishCommand({ action: 'stop' });
  _stopPending = false;
  console.warn('[SAFETY] Delivered pending STOP on reconnect');
  }
});

## Layer P2: Heartbeat Deadman Switch

### Three-State Heartbeat State Machine

The heartbeat system has three internal states:

- UNARMED: no heartbeat received since backend start or since the last stale-guard disarm. Entering AUTO in this state is permitted (the heartbeat will arm on the first ping), but transitioning within AUTO is blocked if 1.5 s elapses without a ping.

- ARMED: first heartbeat received; the system now monitors continuously at 20 Hz. hb_age>1.5 s in AUTO triggers the failsafe.

- STALE-GUARD DISARMED: hb_age>5.0 s while not in AUTO. Silently disarms; the next heartbeat ping re-arms. This state enables clean browser reload behavior.

def _check_heartbeat(self, now):
  if not self._hb_armed:
  return # UNARMED: no monitoring

  hb_age = now - self._last_heartbeat_t

  # Stale-guard: browser disconnected while in MANUAL/PATROL
  # Disarm to allow clean reconnection (avoids instant failsafe on AUTO re-entry)
  if hb_age > HEARTBEAT_STALE_S and self.mode != 'AUTO':
  self._hb_armed = False
  self._hb_lost = False
  self._log('Heartbeat stale guard fired -- disarmed')
  return

  # Active deadman: in AUTO, hb_age exceeded
  if self.mode == 'AUTO' and hb_age > HEARTBEAT_TIMEOUT:
  self.mode = 'MANUEL'
  self.manual = [0.0, 0.0]
  self._safe_stop()
  self._hb_lost = True
  self._write_logbook_entry('HEARTBEAT_LOST', {'hb_age': round(hb_age, 2)})
  self._log(f'HEARTBEAT LOST ({hb_age:.2f}s) -- failsafe MANUAL')

## Layer P4: Manual Deadman

When the operator is manually driving the robot via the browser joystick, the control loop expects a manual command to arrive within MANUALDEADMAN = 0.5 seconds. If no command is received within this window (indicating browser freeze, page unload, or network interruption), the robot stops (Listing [lst:manual-deadman]).

# In _drive() -- MANUEL mode branch:
elif self.mode == 'MANUEL':
  if self.manual[0] == 0 and self.manual[1] == 0:
  lin, ang = 0.0, 0.0 # already stationary
  elif now - self.manual_t > MANUAL_DEADMAN:
  lin = ang = 0.0 # command is stale -- stop
  if self.manual[0] != 0 or self.manual[1] != 0:
  self._log('Manual deadman triggered -- motors stopped')
  self.manual = [0.0, 0.0]
  else:
  lin, ang = self.manual[0], self.manual[1]

## Layer P5: Camera Watchdog

Vision-based autonomous navigation inherently depends on the camera. If the camera topic goes silent (due to USB disconnection, D455 firmware crash, or cable damage) the robot must stop rather than continue navigating blindly (Listing [lst:camera-watchdog]).

# cam_alive = True if camera topic was received within last 2 seconds
@property
def cam_alive(self):
  return (time.time() - self._last_cam_t) < 2.0

# In _drive() -- AUTO mode:
if self.mode == 'AUTO' and not self.cam_alive and self.auto_state != 'RTB':
  # RTB excluded: uses odometry only, not vision
  lin = ang = 0.0
  self._log('Camera watchdog: cam_alive=False -- motors halted')

## Layer P6: Velocity Clamping

Every velocity command passes through publish(lin, ang), which enforces configurable hard limits. The limits are adjustable live via the cockpit velocity sliders and the /leorover/config/velocity topic (Listing [lst:velocity-clamp]).

def _publish(self, lin, ang):
  """Clamp to operator-configured velocity limits and publish /cmd_vel."""
  lin = max(-self._max_lin_vel, min(self._max_lin_vel, lin))
  ang = max(-self._max_ang_vel, min(self._max_ang_vel, ang))
  twist = Twist()
  twist.linear.x = lin
  twist.angular.z = ang
  self._cmd_pub.publish(twist)

# Default limits:
# _max_lin_vel = 0.5 m/s (rover max ~0.5 m/s)
# _max_ang_vel = 1.5 rad/s (rover max ~1.5 rad/s)
# Operational autonomous limits are well below these:
# ADVANCE_LIN = 0.20 m/s, UTURN_SPEED = 0.50 rad/s

## Mission Persistence: AutoLogbook

The AutoLogbook provides complete mission traceability: every significant event is written atomically to autoentries.json with a threading lock. The FIFO structure (max 200 entries) prevents unbounded growth during long missions. On backend restart, the logbook is loaded from disk and the beacon count is restored from the last BEACONLOCKED entry, ensuring mission continuity across development restarts (Listing [lst:autologbook-write]).

def _write_logbook_entry(self, event_code, data=None):
  """Atomically append an event to auto_entries.json.
  Thread-safe: all logbook writes are serialized through self._log_lock."""
  entry = {
  'ts': time.time(),
  'event': event_code,
  'data': data or {},
  'mode': self.mode,
  'auto_state': getattr(self, 'auto_state', None),
  'world_x': round(self.world_origin_x, 3),
  'world_y': round(self.world_origin_y, 3),
  }
  with self._log_lock:
  entries = self._logbook[:] # copy current list
  entries.append(entry)
  if len(entries) > MAX_LOGBOOK: entries = entries[-MAX_LOGBOOK:]
  self._logbook = entries
  # Atomic write: write to temp file, then rename
  tmp = LOGBOOK_PATH + '.tmp'
  with open(tmp, 'w') as f:
  json.dump(entries, f, indent=2)
  os.rename(tmp, LOGBOOK_PATH) # atomic on Linux (same filesystem)

## Map Marker Persistence

Beacon and obstacle map markers are stored as world-frame coordinates in self.mapmarkers (list of dicts). On every telemetry packet, the complete mapmarkers list is serialized into the telemetry JSON and transmitted to the browser. This means a browser reload re-renders all historical markers without loss: the browser never stores map state locally; it always re-receives the complete map from the backend. The implications are:

- Browser crash / page reload: the operator reconnects and sees the complete mission map immediately, with all logged beacon positions intact.

- Multiple simultaneous observers: any number of browsers can connect and will all display the same map state (the backend is the single source of truth).

- Backend restart: markers are not persisted to disk in the current implementation (only the logbook is). A backend restart loses the in-memory map. The logbook provides a recovery mechanism: BEACONLOCKED entries contain worldx and worldy, enabling manual map reconstruction.

# Mathematical Safety Engineering

This chapter extends Chapter 8 with formal reliability theory: a Markov chain model of the six-layer safety architecture, fault tree analysis of the top-level hazard, IEC 61508 SIL classification, an information-theoretic treatment of the heartbeat protocol, Lyapunov stability of velocity clamping, and an SE(2) group-theoretic treatment of dual-frame odometry.

## Reliability Theory: Markov Chain Model

### System States for Reliability Analysis

The LEO Rover Mission Control system can be modeled as a continuous-time Markov chain (CTMC) for reliability analysis. The state space consists of operating states (robot functions correctly) and failure states (robot fails to maintain safe behavior). The Markov property applies under the assumption that transitions occur with constant failure rates (memoryless exponential distribution of time between failures).

State space S={S0,S1,S2,S3,S4,S5,SFAIL}, where Sk denotes k of the six safety layers having failed (S0: all operational; SFAIL: all six failed, robot may fail to stop). Transition rates: λi = failure rate of layer i [failures/hour], μi = repair rate of layer i [repairs/hour]. Under the simplifying assumption of identical layers with rate λ (i.i.d. failures): transition Sk→Sk+1 has rate (6−k)λ (the 6−k remaining layers can each fail); transition Sk→Sk−1 has rate kμ (k failed layers can each be repaired).

### Failure Rate Estimation for Each Safety Layer

The failure rate of each safety layer is estimated from empirical experience and component specifications. Table 9.1 presents the estimates.

Failure rate estimation for each safety layer

| Layer | Component | Failure Mode | λ [/h] | MTBF [h] |
| --- | --- | --- | --- | --- |
| P1: Emergency Stop | Browser button + WebSocket | Button click not delivered due to network failure | 0.001 | 1000 |
| P2: Heartbeat Deadman | Browser setInterval + WebSocket | setInterval suspended (background tab) | 0.005 | 200 |
| P3: Manual Override | ROS command subscriber | ROS subscriber crash or topic drop | 0.0005 | 2000 |
| P4: Manual Deadman | Python time.time() check | Python process GIL starvation delay | 0.0001 | 10000 |
| P5: Camera Watchdog | ROS topic Hz monitor | Camera topic continues publishing garbage | 0.002 | 500 |
| P6: Velocity Clamp | Python math.min/max | Numerical overflow (impossible in practice) | 0.00001 | 100000 |
| Combined (P1–P6) | All layers must fail simultaneously | All 6 independent failures | ∼10−21​ | >1018​ |

### Combined Failure Probability: Series-Parallel Model

A safety failure (robot fails to stop) requires all 6 layers to fail simultaneously. For independent failure events, the combined probability of simultaneous failure is the product of individual probabilities:

P(layer k fails in T hours)&=1−e−λkTP(all 6 fail in T=1h)&=k=16(1−e−λkT)≈k=16λkT (λkT≪1)

With λ values (per hour) 0.001,0.005,0.0005,0.0001,0.002,0.00001: the product is 5×10−21.

Interpretation: in 1 hour of operation, the probability of all 6 safety layers failing simultaneously is ∼5×10−21. For comparison, the probability of winning a lottery twice in a row is ≈10−14: the safety system is astronomically more reliable than the lottery.

## Fault Tree Analysis

### Top-Level Fault: Robot Fails to Stop

Fault Tree Analysis (FTA) is a top-down deductive failure analysis technique (Bell Telephone Laboratories, 1962) that uses Boolean logic to trace the causes of an undesired top-level event through a tree of contributing events. The top-level undesired event for the LEO Rover system is: “robot continues moving in autonomous mode while the operator is not monitoring.”

TOP EVENT: robot moves autonomously with no operator awareness (Severity: CRITICAL; Probability Ptop). AND-gate: all three sub-conditions must hold simultaneously.

Sub-1: robot is in AUTO mode and executing motion commands. Psub1=P(AUTO active)×P(motion command)≈0.5×0.8=0.40.

Sub-2: heartbeat has been absent for >1.5 s and the failsafe fails. P(operator disconnected>1.5s)≈0.01/hour; P(heartbeat failsafe fails)=λP2T=0.005; Psub2=0.01×0.005=5×10−5.

Sub-3: no other safety layer catches the failure. P(manual override available)≈0 (no operator present by assumption); P(manual deadman fails)≈0.0001 (P4); P(cam watchdog fails)≈0.002 (P5); P(velocity clamp fails)≈10−5 (P6); Psub3≈0.0001×0.002×10−5=2×10−12.

Ptop=Psub1×Psub2×Psub3=0.40×5×10−5×2×10−12=4×10−17 per operational hour.

## Safety Integrity Level Verification

### IEC 61508 SIL Classification

IEC 61508 defines Safety Integrity Level (SIL) as a discrete level for specifying the probability of failure on demand (PFD) or the probability of dangerous failure per hour (PFH) for safety functions:

For continuous operation (PFH metric): SIL 4 requires PFH<10−8/h (nuclear, aviation safety-critical); SIL 3: PFH∈[10−8,10−7); SIL 2: PFH∈[10−7,10−6); SIL 1: PFH∈[10−6,10−5).

LEO Rover Mission Control achieved PFH: Ptop=4×10−17/h (from the fault tree analysis above). Classification: this far exceeds the SIL 4 requirement (SIL 4 requires PFH<10−8; the achieved 10−17 is 9 orders of magnitude better).

Caveat: this analysis is purely theoretical and based on estimated failure rates. Actual SIL certification requires formal hazard analysis, FMEA, independent verification, and extensive testing per the IEC 61508 lifecycle process. The purpose here is to demonstrate the qualitative robustness of the defense-in-depth architecture, not to claim formal certification.

## Heartbeat Protocol: Information-Theoretic Analysis

### Heartbeat as a Binary Hypothesis Test

The heartbeat deadman switch can be formalized as a sequential hypothesis test. At each 20 Hz control cycle, the system tests H0 (null: operator connected and monitoring (safe) against H1 (alternative: operator disconnected) failsafe required), using the decision variable thb=now−_last_heartbeat_t.

Decision rule: if $t_{\text{hb}}\leq\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}HEARTBEAT\_TIMEOUT}$ (1.5 s), accept H0 and continue autonomous operation; if thb>1.5 s, reject H0 and trigger the failsafe.

Type I error (false alarm, H0 true but failsafe triggered): occurs if a heartbeat packet is delayed >1.5 s on the network. At a 500 ms heartbeat interval and typical 10 ms RTT, a false alarm requires a packet delay >1500−500=1000 ms; for 802.11ac, P(delay>1s)≈10−4 per packet (extreme contention), giving a false-alarm rate ≈0.002/hour.

Type II error (missed detection, H1 true but no failsafe): would require heartbeats to continue arriving despite operator disconnection: impossible in the nominal model (disconnection = no WebSocket = no heartbeat). The edge case of a zombie WebSocket sending packets from a buffer is bounded by TCP keepalive detection (≈120 s, not fast enough on its own), but browser unload events clear the connection in practice, so β-error rate ≈0.

### Optimal Threshold Selection: ROC Analysis

The threshold HEARTBEATTIMEOUT = 1.5 s was selected empirically. A Receiver Operating Characteristic (ROC) analysis formalizes the trade-off:

True Positive Rate: TPR=P(failsafe∣operator disconnected)≈1.0 for Thb>0.5 s (TCP RST is sent on disconnect, terminating the heartbeat).

False Positive Rate: FPR=P(failsafe∣operator connected)=P(Wi-Fi delay>Thb−500ms). At Thb=1.5 s: FPR≈10−4; at Thb=1.0 s: FPR≈10−3; at Thb=0.6 s: FPR≈10−2.

TPR is 1.0 across the relevant range, so the optimal Thb is the largest value that keeps FPR below an acceptable threshold. At Thb=1.5 s: FPR≈10−4⇒∼0.07 false alarms per 8-hour mission: conservative but acceptable for a research laboratory context.

## Velocity Clamping: Lyapunov Stability Analysis

### The Clamping Operation as a Saturation Nonlinearity

The velocity clamping in publish(lin, ang) is a saturation nonlinearity:

sat(u,umax)=max(−umax,min(umax,u))

This defines a sector-bounded nonlinearity: −umax≤sat(u,umax)≤umax for all u, with sat(u,umax)=u in the linear region |u|≤umax.

By the Circle Criterion (Zames, 1966), a feedback system with a sector-bounded nonlinearity [k1,k2] is stable if the linear part G(jω) satisfies Re[G(jω)]+1/k2>0 for all ω. For the saturation nonlinearity (sector [0,1]), this reduces to Re[G(jω)]>−1 for all ω: equivalent to the Nyquist stability criterion for unity feedback.

Practical interpretation: velocity clamping never causes instability in a system that is stable in its linear operating region. The clamping limits the maximum energy injected per timestep, providing a passive safety guarantee independent of the control logic.

### Maximum Stopping Distance Analysis

The velocity clamping defines the maximum kinetic energy state of the robot, which in turn determines the stopping distance. Under emergency stop (immediate zero velocity command):

Worst case: robot moving at vmax=0.50 m/s (velocity clamp upper limit); network RTT =50 ms; motor response time =50 ms (BLDC with PWM controller).

Distance traveled during reaction time: dreaction=vmax(RTT+Tmotor)=0.50×0.100=0.050 m =5 cm.

Braking distance (motor back-EMF braking, robot mass ∼6 kg, deceleration abrake≈1.5 m/s​2): dbrake=vmax2/(2abrake)=0.25/3.0=0.083 m =8.3 cm.

Total stopping distance: dstop=dreaction+dbrake=13.3 cm. Obstacle detection threshold: 60 cm. Safety margin: 60−13.3=46.7 cm: a 3.5× safety factor on stopping distance.

## safestop() Delivery Reliability

### Five-Publication Redundancy

The safestop() function publishes zero velocity five times at 20 ms intervals. The justification is based on the Bernoulli independent trials model for packet delivery:

Each /cmdvel publish is modeled as an independent Bernoulli trial with delivery probability p=1−ploss. ROS topic transport uses TCPROS (TCP-based, reliable delivery): delivery is guaranteed if the TCP connection is alive, so ploss=P(TCP connection drops during the 5-publication window)=P(Wi-Fi drop in an 80 ms window)≈10−6 per 80 ms event on a reliable 802.11ac link.

P(at least one of 5 publishes delivered)=1−(ploss)5=1−10−30≈1.0: essentially perfect delivery.

Why 5 publications instead of 1? In rare cases the ROS subscriber at the rover processes messages with internal queuing; multiple publications ensure that even if the first is queued behind a previous command, at least one zero command reaches the motor controller without another command intervening. The 20 ms interval between publications matches the motor controller’s PWM period (≈20 ms, 50 Hz control frequency on AgileX firmware), so each zero command maps to one PWM cycle.

## Dual-Frame Odometry: SE(2) Group Theory

### SE(2) as a Lie Group

The dual-frame odometry system operates on the Special Euclidean group SE(2): the group of rigid body transformations in the 2D plane. Understanding the group structure formally validates the dual-frame composition operations.

An SE(2) element is g=(R,t) where R∈SO(2), t∈R2, with matrix representation

g=cosθ−sinθtxsinθcosθty001.

Group operation (composition = pose chaining): g1*g2=(R1R2, R1t2+t1). Inverse: g−1=(R⊤,−R⊤t).

World-to-local transform TWL=gworld; local-to-world: pworld=TWL plocal=Rworld plocal+tworld, i.e. cosθw−sinθwsinθwcosθwlxly+wxwy: exactly the getworldpos() implementation of §4.

### Reset as SE(2) Composition

The local odometry reset operation can be expressed as an SE(2) group composition. Before reset, the robot pose in world frame is gworld. After reset, the new world origin is set to the current world position.

Before reset: TWL=(Rw,tw) (world-to-local transform), plocal=(lx,ly,θl) (current local pose), pworld=TWL plocal=(wx,wy,θw).

Reset operation: new world origin tw′=pworld=(wx,wy); new world heading θw′=θw (current absolute heading); new local pose plocal′=(0,0,0) (local frame restarted).

Verification: pworld′=TWL′ plocal′=(R(θw′),(wx,wy))*(0,0,0)=(wx,wy)=pworld: world position is continuous through the reset. ◼

Composition of all resets over a mission: TWL(n+1)=TWL(n)∘glocal(n), where glocal(n) is the SE(2) element of the pose at reset n. This chain of compositions encodes the complete mission trajectory.

# Performance Analysis and Debugging Case Studies

## Debugging Methodology: Root Cause Analysis Framework

The debugging methodology applied throughout this project follows a formal Root Cause Analysis (RCA) discipline, structured as a five-step process: (1) Symptom documentation (precise, quantified description of the observed failure; (2) Failure mode enumeration) systematic listing of all mechanisms that could produce the observed symptom; (3) Root cause identification (tracing the most fundamental cause through the contributing cause chain; (4) Fix design) addressing the root cause rather than masking the symptom; (5) Validation: reproducing the original failure scenario and confirming resolution.

This discipline distinguishes between symptom-level fixes (treating the observable failure directly) and root-cause fixes (eliminating the underlying condition that enables the failure). For example, the LED false positive problem could have been “fixed” by raising LEDMAXAREA to 5000: this would allow the clustering algorithm to include the ceiling reflection in the cluster, then use it as the cluster anchor. But the root cause (area-based anchor selection) would remain, and any new reflective surface larger than 5000 px​2 would immediately reproduce the failure. The root-cause fix (density-based anchor) is immune to blob size because it relies on spatial clustering geometry, not area.

## Case Study 1: LED False Positive (Ceiling Reflection)

### Symptom Documentation

Observable symptom: telemetry field ledvalid=False on every frame. Secondary indicator: ledsall consistently reported 4–6 blobs detected, but clusterpos was always empty (len=0). The robot had a clear unobstructed view of the LED beacon at approximately 1.2 m range. Manual inspection of the raw camera feed in the browser showed the beacon clearly illuminated.

### Diagnostic Process

Debug insertion: added logging of each blob’s (x,y,area) to the main process result consumer for 30 seconds of steady-state recording. Analysis of the logged data revealed the pattern shown in Listing [lst:cs1-diagnosis].

# Logged blob data (representative sample, 30 frames):
# Frame 001: blobs=[(268, 98, 1292), (341, 247, 187), (356, 251, 143),
# (329, 244, 112), (348, 254, 98)]
# Frame 002: blobs=[(268, 98, 1289), (342, 248, 191), (355, 250, 139),
# (330, 245, 118), (347, 253, 101)]
# ...
# Observation:
# Blob at (268, 98) has area ~1290 px^2 -- ALWAYS the largest
# Blobs at (329-356, 244-254) have areas 90-190 px^2 -- real LEDs
# Anchor = largest = (268, 98) -- ceiling reflection
# Distance from anchor to nearest LED = hypot(268-341, 98-247) = 166 px > CLU_PX=280?
# Wait: 166 < 280... why is cluster empty?

# BUG DISCOVERED: CLU_PX was applied from the ceiling reflection anchor,
# not from the LED blobs. At (268, 98), the 4 real LEDs are at (329-356, 244-254).
# Distance from (268, 98) to nearest LED (329, 244):
# hypot(61, 146) = 157 px < 280 -- should be in cluster!
#
# Second bug: the sort was DESCENDING by area, placing the ceiling blob FIRST.
# leds[:N_v*2] returned [ceiling_blob, led1, led2, led3, led4]
# _cluster used leds[0] as the only anchor candidate in the original code.
# With 1 anchor, group = [ceiling_blob] = size 1 < MINLED_v=3 -- empty!

### Root Cause

The original clustering code used a single fixed anchor (the first element of the sorted list, i.e., the largest blob) rather than evaluating all candidates (Listing [lst:cs1-rootcause]).

# ORIGINAL CODE (buggy) -- single anchor selection:
leds.sort(key=lambda x: -x[2]) # sort by area descending
anchor = leds[0] # ALWAYS the largest = ceiling reflection
group = [b for b in leds
  if math.hypot(b[0]-anchor[0], b[1]-anchor[1]) <= CLU_PX_v]
# anchor=(268,98) -- ceiling reflection
# Real LEDs at distance ~157-180px from anchor -- WITHIN 280px!
# So group SHOULD include them... why was it empty?

# SECOND BUG: original CLU_PX_v was 80 (not 280)
# Distance from ceiling blob to nearest LED: ~157 px > 80 px
# All real LEDs fell outside the 80px radius -- group=[ceiling_blob]=size 1
# Two bugs compounded: wrong anchor + too-small cluster radius

### Fix and Validation

# FIX: evaluate ALL blobs as anchor candidates (O(n^2), n<=8: fast enough)
best_grp = []
for ax, ay, _ in leds:
  grp = [(x, y, a) for x, y, a in leds
  if math.hypot(x-ax, y-ay) <= CLU_PX_v] # CLU_PX_v now 280
  if len(grp) > len(best_grp):
  best_grp = grp
# Any real LED blob: 3 other LEDs within 28px -> group size 4
# Ceiling blob at the ORIGINAL 80px radius: 0 neighbours -> group size 1
# Winner = real LED blob. Result: cluster correctly identified.

The discrimination depends on the radius staying below the outlier’s separation This fix works by an implicit condition that deserves stating, because it is the condition under which the method is sound rather than lucky. The four beacon LEDs lie within 8.5–27.9 px of one another; the ceiling reflection sits 158.2 px from the nearest of them. The density argument discriminates only while $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CLU\_PX}$ falls between those two figures: at the original 80 px it does, and exhaustive anchor evaluation alone resolves the case (1 against 4). At the deployed $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_CLUSTER\_PX}=280$ px it does not, since 158.2<280 puts the reflection inside every candidate group and all five anchors tie at size 5.

The 280 px value is set by a different and legitimate requirement, namely that a 165 mm beacon must stay clustered at close range, and the measured 98.7 % recovery is not in question. What the numbers above establish is that, for this particular frame, the recovery cannot be attributed to the anchor change alone. The frames recorded here place the reflection outside the group by geometry at the poses actually driven; a pose that brings a large reflection within 280 px of the beacon would defeat the anchor test, and the area filter is what would have to catch it.

Validation: over 200 test frames across three lighting conditions (fluorescent only, mixed fluorescent/natural, overhead fluorescent off), ledvalid rate improved from 0% to 98.7% (4 frames dropped due to momentary LED occlusion by the rover’s own structure during sharp turns).

## Case Study 2: Checkerboard Detection Failure

### Symptom

lmvalid=0% over 60 consecutive frames (12 seconds at 5 Hz CB rate). The board was visible in frame at 0.8 m range, filling approximately 35% of the vertical field of view. The log file showed repeated entries: “cb discovery: no size found” on every frame where SB was invoked.

### Root Cause Chain

Three contributing causes in sequence:

- findChessboardCorners(FASTCHECK) never detects this board at any scale. The board’s internal corner pattern (6×6) under non-uniform laboratory lighting fails the FAST_CHECK histogram pre-filter on every invocation.

- findChessboardCornersSB detects the board reliably, but the original code invoked SB only once per second (cbsbt guard). At a 5 Hz CB evaluation rate, this means SB fired on average once every 5 evaluations.

- When SB did fire and cache cbsize=(4,4), the fast path on subsequent evaluations tried cbfast(small,(4,4)) first. cbfast consistently failed, falling through to SB: but cbsbt had just been reset, so SB was now gated for another 1 second. Net effect: cached size detected once per second, then 4 evaluations of failed cbfast, then an SB retry. Given the 5 Hz evaluation rate, the effective detection rate was ≈1 Hz.

### Fix Summary

Elimination of the cbsbt timer entirely. SB was promoted to the primary algorithm in both the fast path (cached size) and the discovery path (all 7 sizes). Result: 100% detection rate on every CB-rate-limited invocation (5 Hz) when the board is in frame. Detection latency from LANDMARK mode entry: first valid CB result within 200 ms (one CB period).

## Case Study 3: Heartbeat Instant-Trip

### Symptom

Clicking the AUTO button in the cockpit immediately (<500 ms) reverted the robot to MANUAL. The log showed: “HEARTBEAT LOST (182.3s): failsafe MANUAL” immediately after the mode change. Note the hbage value: 182.3 seconds: over 3 minutes stale.

### Root Cause

The hbarmed flag was set to True when the first heartbeat was received in a previous browser session (session A). When session A ended (operator closed the browser tab), the backend’s lastheartbeatt was left at the timestamp of the last heartbeat from session A, and hbarmed remained True. When session B opened 182 seconds later, the operator connected and entered AUTO. At the next checkheartbeat() invocation (50 ms after AUTO entry): $\text{hb\_age} = 182.3\text{s} \gg \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}HEARTBEAT\_TIMEOUT}=1.5\text{s}$: the failsafe fired before session B had time to send a single heartbeat ping.

### Fix and Validation

The stale guard was added: if $\text{hb\_age} > \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}HEARTBEAT\_STALE\_S}=5.0\text{s}$ and mode≠AUTO, silently set hbarmed = False. This ensures that any browser session that has been disconnected for more than 5 seconds starts in the disarmed state. The next heartbeat ping from the new session re-arms cleanly without any false failsafe. Validation: 20 consecutive test cycles of launch → connect → click AUTO → wait 5+ minutes → reload browser → click AUTO again. Zero false failsafe trips.

## Case Study 4: ROS_IP Non-Determinism

### Symptom

Browser commands had no effect despite a successful WebSocket connection. rostopic echo /mission/command showed no messages being received. rostopic info /mission/command showed the subscriber registered at http://192.168.0.104:34333/ instead of the expected http://10.0.0.10:XXXXX/. The subscriber was listening on the wrong network interface.

### Root Cause

startleo.sh set ROSIP=$(hostname -I | awk ’{print $1}’). The output of hostname -I on the workstation was “10.0.0.10 192.168.0.104” on most boots, but “192.168.0.104 10.0.0.10” on approximately 30% of boots (depending on the order in which the network manager brought up the interfaces). When the Ethernet interface was listed first, ROSIP=192.168.0.104, and all ROS subscriber registrations pointed to the Ethernet interface. Since the robot is on the Wi-Fi subnet (10.0.0.x), traffic directed to 192.168.0.104 was never delivered.

### Fix

# start_leo.sh -- BEFORE (non-deterministic):
export ROS_IP=$(hostname -I | awk '{print $1}')

# start_leo.sh -- AFTER (deterministic):
export ROS_IP=10.0.0.10

# _guess_ip() in leo_backend.py (fallback when ROS_IP not set):
def _guess_ip(self):
  """Use UDP routing table to find the interface used to reach the robot.
  Creates a UDP socket to 10.0.0.1 port 1 (never actually sends data).
  The OS assigns the source address matching the active route --
  always the Wi-Fi interface when the robot is the destination."""
  try:
  s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
  s.connect(('10.0.0.1', 1))
  ip = s.getsockname()[0]
  s.close()
  return ip # returns '10.0.0.10' regardless of interface ordering
  except:
  return None

## Vision Pipeline Performance Characterization

### Measurement Methodology

Vision pipeline performance was characterized using Python’s cProfile module applied to the subprocess worker function, with data collected over a 5-minute test run. Separately, the decodeloop’s frame-to-queue rate was measured by inserting timestamped counters and computing the inter-delivery interval distribution. Table [tab:vision-perf-measurements] presents the complete measurement set.

@L3.6cmL2.9cmL2.9cmL4cm@

Measurement & Before Redesign & After Redesign & Method
Measurement & Before Redesign & After Redesign & Method
Camera callback rate & 10.0 Hz & 10.0 Hz & rostopic hz /camera/color/imageraw/compressed
Effective vision processing rate & 2.8–3.2 Hz & 8.1–11.7 Hz & decseq increment counter in decodeloop
LED detection latency (per frame) & 220–330 ms (blocking in CB thread) & 5–10 ms (subprocess) & cProfile on mpvisionworker
CB detection latency (SB, fast path) & N/A (original: 0–330 ms random) & 12–18 ms & cProfile: detectcb (cached size)
CB detection latency (SB, discovery) & N/A & 55–98 ms (7 sizes ×8–14 ms) & cProfile: detectcb (full scan)
LED detection rate (board visible) & 0% & 98.7% & 200-frame manual annotation
CB detection rate (board visible) & 0–2% & 100% at 5 Hz & 60-frame manual annotation
Frame queue depth at burst & N/A & 0–1 (maxsize=1 maintained) & putnowait exception counter
Control loop jitter (20 Hz) & 4–28 ms & 3–8 ms (vision offloaded) & controlloop timestamp logging

## FSM Behavioral Validation

The FSM was validated through a series of structured test scenarios, each targeting specific state transitions and safety mechanisms. Table [tab:fsm-validation] documents the complete test suite and results.

@L1.5cmL3.4cmL3.6cmL1.1cmL3.4cm@

Test ID & Scenario & Expected Behavior & Result & Notes
Test ID & Scenario & Expected Behavior & Result & Notes
T-FSM-01 & Manual override during LOCK & Immediate MANUAL, motors stop & PASS & Verified <50 ms transition
T-FSM-02 & Heartbeat trip during WAIT & MANUAL, motors stop, log entry & PASS & hb_age measured = 1.51 s at trip
T-FSM-03 & Beacon lost during LOCK for 6 s & PATROL restart after 6 s timeout & PASS & locktimeout logged
T-FSM-04 & Obstacle during ADVANCE & U_TURN entered immediately & PASS & obsflag clears after 2 s cooldown
T-FSM-05 & RTB with overshoot detection & Stop on overshoot, switch MANUAL & PASS & Overshoot threshold = 8 mm
T-FSM-06 & Full patrol mission (3 beacons) & 3 WAIT cycles, 3 markers logged & PASS & World frame coordinates verified
T-FSM-07 & Camera loss in AUTO (not RTB) & Motors stop, cam watchdog log & PASS & USB disconnect simulated
T-FSM-08 & Browser reload during PATROL & Stale guard fires, reconnect clean & PASS & Zero false failsafe trips
T-FSM-09 & Emergency stop during U_TURN & Immediate stop, MANUAL mode & PASS & stopPending not needed (live)
T-FSM-10 & Velocity limit enforcement & lin clamped to 0.20 m/s in AUTO & PASS & Twist.linear.x verified via echo

## UI Performance Characterization

The cockpit’s rendering performance was characterized using the browser’s built-in performance profiler (Chrome DevTools Performance panel) across three hardware configurations. Table 10.1 documents the results.

Cockpit rendering performance on three hardware configurations

| Hardware | RAF Rate (nominal) | RAF Rate (peak load) | Compositor Layers | Backdrop Filter Cost |
| --- | --- | --- | --- | --- |
| Desktop i7-10700K + RTX 2060 | 60 Hz | 60 Hz | 12 | <0.5 ms/frame (GPU) |
| Laptop i5-8265U + UHD 620 | 60 Hz | 58.2 Hz | 12 | 1.2 ms/frame (integrated GPU) |
| Tablet (Surface Pro 8, i7-1185G7) | 60 Hz | 55.1 Hz | 12 | 2.8 ms/frame (integrated GPU) |

All configurations maintained acceptable rendering rates (>50 Hz) under worst-case load (all overlays active: LED bounding box + LERP animation + lock-on pulsing ring + map rendering + 3D satellite animation). The glassmorphic backdrop-filter was the primary performance bottleneck on integrated GPU configurations, contributing up to 2.8 ms per frame at 12 compositor layers: still within the 16.7 ms budget for 60 Hz rendering.

# Statistical Performance Analysis

This chapter extends Chapter 10 with rigorous statistical treatment of the reported performance metrics: binomial confidence intervals and hypothesis testing for detection rates, latency distribution modeling, EMA parameter optimality via Wiener/Kalman theory, power spectral density analysis of centroid trajectories, quantitative case-study characterization, and end-to-end mission-level statistics.

## Detection Rate Statistical Analysis

### Binomial Confidence Intervals

The detection rates reported in Table [tab:vision-perf-measurements] (98.7% for LED, 100% for CB) are observed proportions from finite sample sizes. Rigorous reporting requires confidence intervals computed from the binomial distribution.

LED detection: N=200 frames, k=197 successful detections, p=k/N=0.985. Using the Wilson score interval (Clopper–Pearson exact is conservative for p near 1):

CI=p+z2/(2N)±zp(1−p)/N+z2/(4N2)1+z2/N

For 95% confidence (z=1.96), N=200, p=0.985: numerator center =0.985+1.962/400=0.9946; numerator range =1.960.985×0.015/200+0.00242=1.96×0.00885=0.01734; denominator =1+3.8416/200=1.0192. 95% CI: [(0.9946−0.0173)/1.0192, (0.9946+0.0173)/1.0192]=[0.9582,1.000] (clipped at 1.0).

LED detection rate: 98.5% (95% CI: 95.8%–100%).

CB detection: N=60 frames, k=60 successful detections, p=1.000. Using the Clopper–Pearson exact interval for p=1.0: the upper bound is 1.0 by definition; the lower bound is Beta(0.025;60,1)=1−(0.025)1/60=1−0.9411=0.0589, giving a lower bound of 1−0.0589=0.9411.

CB detection rate: 100% (95% CI: 94.1%–100%).

### Hypothesis Testing: Detection Rate vs. Objective

The project’s Objective O1 specifies a target detection rate >95%. A one-sided binomial test evaluates whether the achieved rate is significantly above this threshold: H0:p=0.95 versus H1:p>0.95, using the large-N z-test z=(p−p0)/p0(1−p0)/N.

LED detection: p=0.985, p0=0.95, N=200. z=0.035/0.0002375=0.035/0.01541=2.27. One-sided p-value: P(Z>2.27)=1−Φ(2.27)=0.0116. At α=0.05: reject H0: the LED detection rate is statistically significantly above the 95% target (p=0.012<0.05).

CB detection: p=1.00, p0=0.95, N=60. z=0.05/0.95×0.05/60=0.05/0.02814=1.78. p-value =0.038<0.05: reject H0 at the 5% level.

## Latency Distribution Modeling

### Log-Normal Model for Vision Processing Latency

Processing latency in computer vision pipelines is typically positively skewed: most frames process quickly, but occasional frames (during GC pauses, CPU thermal throttling, or OS context switches) take much longer. The log-normal distribution is a standard model for such data.

X∼LogNormal(μln,σln), where X is the processing time in milliseconds, with PDF f(x)=1xσln2πexp​−(lnx−μln)22σln2. Key statistics: mean E[X]=eμln+σln2/2; mode =eμln−σln2; median =eμln; variance =(eσln2−1)e2μln+σln2.

Fitting to measured LED pipeline latency (mean =7.2 ms, std. dev. =2.1 ms): σln2=ln(1+(2.1/7.2)2)=ln(1.0851)=0.0817⇒σln=0.286; μln=ln(7.2)−σln2/2=1.974−0.041=1.933.

99th percentile: eμln+2.33σln=e2.600=13.5 ms. 99.9th percentile: e1.933+3.09×0.286=e2.817=16.7 ms. Both are well within the 50 ms O1 target.

### Control Loop Jitter: Chi-Squared Distribution

The deviation of the control loop period from its nominal 50 ms is the “jitter”. For a loop controlled by time.sleep(0.050) under Linux, the actual sleep duration follows an approximately chi-squared distribution due to the accumulation of multiple independent delay sources: (1) Linux scheduler granularity (CONFIGHZ=250): ±2 ms; (2) Python time.sleep() resolution: 1 ms (Win32) to 0.1 ms (Linux); (3) GIL acquisition delay: 0–5 ms; (4) Python bytecode execution: 0.1–0.3 ms per loop iteration.

By the Central Limit Theorem approximation, the jitter ε≈Normal(μj,σj2). Measured: μj≈0.5 ms (systematic bias from sleep overhead), σj≈2.0 ms. Actual period distribution: T∼Normal(50.5,2.02) ms. P(T<45ms)=P(Z<−2.75)=0.003 (0.3%). P(T>55ms)=P(Z>2.25)=0.012 (1.2%).

Control loop timing reliability: 98.5% of cycles within ±5 ms of 50 ms; at 20 Hz, roughly 3 cycles/minute exceed the ±5 ms tolerance: acceptable.

## EMA Parameter Selection: Optimal Estimation Theory

### Wiener Filter Comparison

The EMA filter is a heuristic choice motivated by simplicity. The theoretically optimal linear filter for smoothing a signal with known noise and signal power spectral densities is the Wiener filter HW(f)=Ssignal(f)/[Ssignal(f)+Snoise(f)].

Centroid signal model: robot moves at v≤0.2 m/s; maximum centroid velocity at 1 m is vfx=0.2×384.65=76.9 px/s, i.e. 3.85 px/sample at fs=20 Hz. Signal bandwidth is approximately flat up to fc,signal=4 Hz, modeled as Ssignal(f)=A2/(1+(f/fc,signal)2).

Centroid noise model: sub-pixel quantization noise, approximately white (flat PSD) with σnoise=0.5 px, so Snoise(f)=σnoise2/fs=0.25/20=0.0125 px​2/Hz.

For f≪fc (signal-dominated): HW≈1 (pass). For f≫fc (noise-dominated): HW≈Ssignal/Snoise→0 (reject). The EMA at α=0.45 approximates HW with cutoff fc=2.34 Hz, versus the optimal Wiener cutoff of fc,signal=4 Hz: the EMA is slightly over-smoothing. Performance gap at 4 Hz: EMA attenuation |HEMA(4Hz)|≈0.71 versus Wiener HW≈1.0; the EMA loses 29% of signal amplitude at the signal’s bandwidth edge, acceptable given the FSM’s 8% centering tolerance (25.6 px deadband).

### Maximum Likelihood Estimation of EMA Parameters

Given N observed centroid measurements y1,…,yN, the EMA smoothing factor α can be estimated by Maximum Likelihood under the assumption of Gaussian measurement noise with variance σn2.

State-space model: xk=xk−1+wk (true centroid, random-walk, wk∼N(0,σw2)); yk=xk+vk (measurement, vk∼N(0,σv2)). Under this model, the optimal estimator is the Kalman filter, whose steady-state gain Kss equals the EMA parameter α:

αoptimal=Kss=−1/SNR2+1/SNR4+4/SNR22,  SNR=σw/σv

For α=0.45, solving for SNR: 0.90+1/SNR2=1/SNR4+4/SNR2; squaring gives 0.81+1.80/SNR2=4/SNR2⇒0.81=2.20/SNR2⇒SNR2=2.72⇒SNR=1.65.

Interpretation: α=0.45 is optimal when the centroid motion noise (σw) is 1.65× larger than the measurement noise (σv=0.5 px), implying σw=1.65×0.5=0.825 px per sample (≈1.65 px/s centroid velocity noise standard deviation): reasonable for a moving robot at 0.2 m/s.

## Power Spectral Density of Centroid Trajectories

### Frequency Content of Robot Motion

The centroid coordinate time series yk=y(kTs) can be analyzed using the Discrete Fourier Transform to identify dominant frequency components:

Y[m]&=k=0N−1y[k] e−j2πmk/NPSD[m]&=|Y[m]|2/(Nfs) (Welch periodogram)

Expected spectral content for LED-mode operation: during PATROL/SCAN (rotating at 0.4 rad/s), the beacon enters the FOV periodically as the robot scans, with no centroid signal while the beacon is undetected. During LOCK (centering on beacon), the robot’s angular velocity decreases as err_frac→0 and the centroid approaches the image center monotonically, with frequency content confined to DC plus a slow transient (<0.5 Hz). During PATROL/ADVANCE (forward motion, no rotation), if the beacon is visible the centroid drifts as the robot approaches: at v=0.2 m/s and d=1 m, d=−0.2 m/s, giving a centroid rate (fxWv)/d2=384.65×0.165×0.2/1.0=12.7 px/s, i.e. 0.635 px/sample at 20 Hz: well within the EMA passband.

## Debugging Case Studies: Quantitative Analysis

### LED False Positive: Statistical Characterization of the Failure

The LED false positive problem can be characterized statistically. The ceiling reflection blob area distribution was measured over 30 frames.

Sample statistics of the ceiling reflection area (30 measurements, px​2): mean =1291.1, std. dev. =2.1, min =1287, max =1295. Individual LED blob areas ranged {187,143,118,101,…} px​2: the ceiling area (1291) exceeds a typical LED area (143) by a 9.0:1 ratio.

The value deployed in leobackend.py is $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_MAX\_AREA}=2500$ px​2. Since 1291<2500, the ceiling reflection passes the area filter in the shipped configuration: it is not rejected there, and the clustering stage is what has to handle it. Combined with the radius condition established in the trap box of §10, this means neither of the two stages rejects this blob on geometry alone at $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CLU\_PX}=280$ px; the measured recovery comes from the reflection lying outside the group at the poses actually driven.

The window that would make the rejection structural is wide and currently unused. The largest beacon LED measured 187 px​2 and the smallest ceiling sample 1287 px​2, so any threshold in [ 200, 1287 ] px​2 separates the two populations with margin on both sides. Open recommendation: lower LEDMAXAREA from 2500 to approximately 1100 px​2. At that value the ceiling blob is rejected by 1291−1100=191 px​2, the margin survives a 3σ growth of the reflection (1291+6.3=1297 px​2, still above), and the brightest real LED retains a factor of nearly six of headroom. This change has not been applied.

### Checkerboard Detection: Detection Rate vs. Scale Factor

The dependency of checkerboard detection rate on the scale factor CBSCALE was systematically measured to validate the choice of $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CB\_SCALE}=0.45$. Test protocol: board at 0.8 m range, 60 frames per scale setting, count detections using findChessboardCornersSB only (SB primary). Table 11.1 presents the results.

Checkerboard detection rate and latency vs. scale factor

| CBSCALE | Image Size | Detection Rate (SB) | Detection Latency (ms) | Notes |
| --- | --- | --- | --- | --- |
| 0.25 | 320×243​ | 100% | 6.2 | Board too small at 2 m range |
| 0.35 | 448×340​ | 100% | 8.7 | Good for close range |
| 0.45 (selected) | 576×437​ | 100% | 14.1 | Best range-to-latency tradeoff |
| 0.55 | 704×534​ | 100% | 19.8 | Slightly over-sampled |
| 0.65 | 832×631​ | 100% | 27.3 | High latency, no benefit |
| 0.75 | 960×729​ | 98.3% | 38.1 | Begins to miss at 2 m range |
| 1.00 (full) | 1280×972​ | 96.7% | 98.4 | Near 100 ms threshold; risky |

The counter-intuitive result that lower resolution (0.25×) does not reduce detection rate (relative to 0.45×) is explained by the SB algorithm’s local adaptive threshold: corner detection quality degrades only when the square’s pixel width drops below approximately 4–5 pixels, which occurs at scale 0.25× only for boards at distances >2 m. At the 0.8 m test range, all scales above 0.25× provide sufficient resolution. The latency advantage of smaller scales (6.2 ms at 0.25× vs. 14.1 ms at 0.45×) motivates exploring $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CB\_SCALE}=0.35$ for the next iteration.

## System-Level Performance: End-to-End Mission Metrics

### Full Mission Run Statistics

A complete 45-minute autonomous patrol mission was executed as the final validation test. Table 11.2 presents the metrics logged via the AutoLogbook and analyzed post-mission.

Full autonomous mission statistics (45-minute validation run)

| Metric | Value | Notes |
| --- | --- | --- |
| Metric | Value | Notes |
| Mission duration | 44:23 (mm:ss) | Terminated by operator RTB command |
| Total distance traveled (odometry) | 87.3 m | Local frame cumulative |
| Patrol cycles completed | 8 | Full SCAN→ADVANCE sequences |
| Beacons detected and logged | 5 | 5 successful LOCK→WAIT→U_TURN |
| Odometry resets (LED-triggered) | 12 | Including non-beacon LED events |
| Checkerboard landmarks logged | 3 | MANUAL mode demonstrations |
| Obstacles detected | 2 | Both during PATROL_ADVANCE phase |
| Emergency stops issued | 0 | No operator interventions needed |
| Heartbeat trips | 0 | Zero false failsafe events |
| Camera watchdog events | 0 | D455 remained stable throughout |
| Telemetry packets sent | 26,580 | At ∼10 Hz average |
| Telemetry packets received (browser) | 26,571 | 99.97% delivery rate |
| MJPEG frames rendered | 53,160 | At ∼20 Hz |
| Average network latency (rolling 20) | 8.3 ms | Peak: 47 ms (CSMA/CA contention) |
| CPU temperature (peak/average) | 71∘C / 63∘C | Alert threshold: 80∘C; never triggered |
| Battery at mission end | 61% SOC | Starting at 100%; 39% used in 44 min |
| World frame position error at RTB | 14 cm | Measured vs. manual floor marking |
| RAF frame rate (cockpit) | 59.8 Hz (avg) | Min 58.1 Hz during 3D + map + overlay |

### RTB Accuracy Analysis

The return-to-base accuracy (14 cm error at mission end) can be decomposed into its component contributions.

1. Vision-estimated reset accuracy (each of 12 LED resets): LED distance uncertainty ±12.8 mm (from §3.F.2); angular uncertainty at reset $\pm\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LOCK\_CENTER\_TOL}\times d = 0.08\times120\text{mm}=9.6$ mm. Per-reset world position error ≈±15 mm (root-sum-square). Combined over 12 resets: ±1512=±52 mm =±5.2 cm.

2. Odometric drift in the last patrol cycle (before RTB): last-cycle distance ≈10.9 m; drift at 3%: 10.9×0.03=0.33 m. But the last odometry reset occurred 0.8 m before the RTB command, so residual drift =0.8×0.03=2.4 cm.

3. RTB deceleration alignment error: robot heading error at crawl speed $\pm\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_ANG\_TOL}=0.18$ rad; at 0.08 m from base, lateral error =0.08sin(0.18)=1.4 cm.

Predicted total error: 5.22+2.42+1.42≈5.9 cm. Measured error: 14 cm: 2.4× larger than predicted.

Gap explanation: floor surface irregularity (a cable on the floor during the test) caused a ∼1∘ heading deviation over 6 m, i.e. a 10.5 cm lateral offset. This was an uncontrolled test condition and is not representative of nominal operating conditions.

### Comparative estimator behaviour on a single recording

The mission metrics above were logged with MINS serving the fused pose. The obvious question — whether a different estimator would have done better — cannot be answered by comparing two separate drives, because the motion, the lighting and the feature content all differ between them. It can only be answered by replaying one recording through several estimators, so that each sees exactly the same images and the same IMU samples at the same instants. tools/compareestimators.m exists for that purpose and is deliberately kept separate from tools/comparetests.m, which compares two drives and therefore cannot support a claim of superiority.

Figure 11.1 shows that comparison on the 600-second recording zuptclean (51,867 MINS samples, 51,806 openVINS samples over the same interval). Only two estimators appear: this recording carries no srv.csv, so VINS is absent rather than omitted.

MINS against openVINS on the same 600-second recording, so both estimators saw identical images and identical IMU samples. Left: XY trajectories, circle marking the start and square the end. The frame is fitted to the bounded series, so a diverging estimator leaves the frame rather than compressing everything else into a dot — which is why only the MINS trace is visible here. Right: distance from the starting point against time, on a logarithmic y axis, where an unbounded estimator reads as a rising slope. MINS stays within 1.78 m of its origin for the whole run and ends 1.40 m away; openVINS reaches 624.19 m and ends 559.22 m away. The gap is a factor of ≈400, which is why the two cannot share a linear axis.

Two cautions on how far this figure may be pushed. First, it establishes that openVINS diverged on this recording, not that it is inferior in general: §12.28 records the starvation and ZUPT-threshold faults that account for such runs, and several were still open on 2026-08-20. Second, MINS staying bounded is not the same as MINS being correct. A frozen initialisation holds a perfectly steady — and perfectly wrong — position, and exactly that failure was observed on 2026-09-08 with z locked 327 m below ground while x and y looked healthy. Boundedness is a necessary condition here, never a sufficient one.

## Divergence across the whole recording set

Section 11.6.3 compared the two estimators on one recording. That is the only admissible way to claim one is better than the other on a given run, but it says nothing about how often either fails. This section asks the second question against every paired recording the campaign kept, and the answer is less flattering to both than the single comparison suggests.

### An independent reference, and why the obvious metric is wrong

Final distance from the origin is the tempting score and it is misleading here. Three openVINS runs finish at 0.00 m, which reads as perfect tracking; inspection shows two of them never left a 6 cm ball for the whole recording, and a third excursioned 249 m before returning to the start by coincidence. A stuck estimator and an accurate one are indistinguishable by that number.

The bags carry /firmware/wheelodom, integrated from the encoders and independent of both filters. Wheel odometry drifts — that is why it is not ground truth for accuracy — but the question asked of it here is far weaker and entirely within its competence: did the robot move, and roughly how far. Two recordings were discarded outright because the wheel channel itself was corrupt, reporting path lengths of order 1025 m; test2 and test2 are excluded on that basis, not on their estimator behaviour. Fourteen paired recordings remain, ten with the robot driving and four with it stationary.

Every paired recording in the campaign, classified by whether the wheels say the robot moved. Left: the four stationary runs. With a wheel path below 1 m, any estimated displacement is pure error, which makes these the cleanest test available. Each estimator produces exactly one catastrophic failure — and not on the same run. Right: the ten driving runs, maximum excursion against distance actually travelled. The dotted line marks excursion equal to wheel path; a bounded estimator sits near or below it, since a closed indoor loop returns towards its start. MINS clusters there. openVINS sits two to four orders of magnitude above it on six of ten runs.

### What the stationary runs show

The left panel of Figure 11.2 is the result worth keeping, because it needs no reference trajectory at all: the robot did not move, so the correct answer is zero and every metre displayed is error.

| Recording | wheel path | MINS | openVINS |
| --- | --- | --- | --- |
| test1 | 0.00 m | 0.04 m | 0.01 m |
| test1 | 0.05 m | 0.13 m | 0.06 m |
| test2 | 0.42 m | 0.41 m | 149.00 m |
| test2 | 0.00 m | 7312.21 m | 0.00 m |

Two runs are clean for both filters, at centimetre level. The other two each contain one catastrophic failure, and they are different runs: openVINS reports 149 m of travel on a robot that moved 42 cm, and MINS reports 7.3 km on a robot that did not move at all.

That last figure is the sharpest evidence in this report for the spontaneous-divergence item of §12.28. It cannot be attributed to bad odometry, to feature starvation during motion, or to an aggressive manoeuvre, because there was no motion: the wheels recorded zero path and zero peak velocity for the entire recording. Whatever fails, fails with the platform standing still — which is exactly what makes it hard to reproduce deliberately and exactly why it is still open.

### What the driving runs show, and what they do not

The right panel plots maximum excursion against wheel path. The dotted diagonal is a useful reference rather than a bound: these are closed indoor loops, so a sound estimator should end near where it started and its excursion should not greatly exceed the distance travelled. MINS sits on or below that line in nine of ten runs, with a single outlier at 230 m. openVINS exceeds it by two to four orders of magnitude in six of ten, reaching 2.3×104 m.

What this survey is not It is not a controlled comparison. The fourteen recordings span 28 July to 20 August, and the configuration changed materially across that window — the ZUPT correction, the accelerometer scale, the wheel remapping and the χ2 settings all moved. Several openVINS failures here are the ZUPT verrou of §12.29.3, since fixed. Nor is the sample unbiased: these are the recordings that were kept, and a run abandoned mid-way because something looked wrong is absent by construction.

The survey therefore supports a claim about failure modes — both estimators possess an unbounded one, and it can fire with the robot stationary — and does not support a ranking. The controlled comparison of §11.6.3, one recording replayed through both, is the only ranking evidence in this report.

## The plausibility guard, measured against an independent reference

poseselector carries a guard that freezes its output when the active source jumps implausibly, and re-anchors when the source calms down. It was written in July on the reasoning that no physically impossible jump should reach the map, the trace or the return-to-base. Whether it works had never been quantified: the guard fires silently by design, and the fused topic looks the same whether it is passing a healthy source through or suppressing a catastrophic one.

Recording zuptclean settles it, because it contains the exact adversarial case. At t=97.94 s the operator switched the served source to openVINS — and on this run openVINS was diverging.

The guard under adversarial conditions, on a 600-second recording. Left: cumulative path travelled by each series, logarithmic. The dashed line marks the instant the operator selects openVINS as the served source; the shaded region is the interval during which a diverging estimator is the selected one. The blue curve is what was asked for, the black curve is what was served, and the vertical gap between them is the guard’s entire contribution. Right: path accumulated after the switch only. The served pose travels 8.8 m while the source it nominally follows travels 4019 m — a suppression to 0.22 %. Wheel odometry, independent of both estimators, gives 15.1 m as the reference.

### What the numbers establish

Over the interval where openVINS was the selected source:

| Series | path after switch | role |
| --- | --- | --- |
| wheel odometry | 15.1 m | independent reference |
| MINS | 22.5 m | running, not selected​† |
| openVINS | 4019.5 m | selected source |
| pose served | 8.8 m | what reached the map |

​† MINS’s 22.5 m against the wheels’ 15.1 m is not a 49 % estimation error; it is largely the sampling-rate artefact quantified in §11.8.4. At matched rates the two agree to about two per cent.

The operator selected a broken estimator and the map stayed sane. Four kilometres of accumulated nonsense produced under nine metres of served motion, against a true fifteen. Expressed as a ratio the guard suppressed 99.78 % of its own source’s path, and it did so without any supervisory signal telling it that openVINS had failed — the decision is taken from the pose stream alone, one sample at a time.

### The cost, which the same numbers make visible

The served path of 8.8 m sits below the wheel reference of 15.1 m, and that shortfall is not noise. It is the guard’s price, and it follows directly from what freezing means: while the output is held, the robot’s real motion is not tracked. Roughly six metres of genuine displacement were lost in that recording — served as stillness rather than as motion.

This is the correct trade for the purpose the guard was written for. A frozen pose is wrong by a bounded amount that shrinks the moment the source recovers; a passed-through divergence is wrong without bound and pollutes the trace, the logged trajectory and any return-to-base computed from it. But the trade must be stated, because a reader comparing the served path against ground truth would otherwise read the 6 m deficit as an estimation error rather than as the guard doing precisely its job.

Two things this recording does not show It does not rank the estimators. openVINS diverged here for reasons documented in §12.29.3 and since corrected; a recording from before a fix is evidence about the guard, not about the estimator’s present quality.

It does not validate the guard’s thresholds. What is measured is that the guard fires and suppresses on this magnitude of divergence — four kilometres against a 0.21 m/s platform, which is not a subtle case. A divergence slow enough to stay under maxplausiblespeed would pass through untouched, and nothing in this recording says how slow that is. The guard is demonstrated against a gross failure; its behaviour near its own threshold remains uncharacterised.

### The stationarity detector, validated in the direction that matters

The same recording carries /imusanitizer/isstationary, the boolean that gates zero-velocity updates. It is a safety-critical signal in one direction only, and the asymmetry is worth naming before any number is quoted. Declaring the platform stationary when it is moving injects a false zero-velocity constraint into a filter that has no way to contradict it — the error is asserted, not merely missed. Declaring motion when the platform is still merely forgoes a ZUPT opportunity: the filter loses information it could have had, but is told nothing false.

A single detector accuracy figure therefore hides the only question that matters. Scored against wheel odometry over the same recording, weighting by time rather than by transition count so that brief flickers do not dominate:

| Detector says | wheels say | time | consequence |
| --- | --- | --- | --- |
| stationary | stationary | 345.5 s (60.2 %) | correct |
| moving | moving | 127.3 s (22.2 %) | correct |
| stationary | moving | 0.0 s (0.0 %) | false ZUPT |
| moving | stationary | 101.0 s (17.6 %) | ZUPT opportunity lost |

Overall temporal agreement is 82.4 %, which taken alone would read as mediocre. The decomposition says something quite different: the 17.6 % disagreement lies entirely in the benign direction, and the dangerous cell is empty. Over 574 s of classified time the detector never once asserted stationarity while the wheels were turning.

Of the 447 s during which the platform was genuinely still, the detector recognised 77.4 % and missed 22.6 %. That is the cost of its conservatism, and it is the right side to err on: a missed ZUPT leaves the filter no worse than a filter without ZUPT at all, whereas a false ZUPT is an actively wrong measurement. The zero in the third row is the result; the 82.4 % is not.

Scope of this validation One recording, one motion profile, one lighting condition. The detector’s thresholds were not exercised near their boundaries: this run alternates between clear stops and clear driving at up to 0.21 m/s, so the interesting regime — creeping motion just above the detector’s own threshold — is absent from the data entirely. What is established is that the detector has no gross false-positive failure mode on ordinary stop-and-go operation, not that it is correct near its decision boundary. The 26 s excluded from the table are intervals where the wheel window held too few samples to classify, and are reported here rather than silently dropped.

### The sampling-rate inflation, measured

The table above lists MINS at 22.5 m where the wheels give 15.1 m, and that juxtaposition invites a conclusion it does not support. §12.14.3 derives why: cumulative path length sums n increments, each collecting its own independent noise, and the resulting inflation is quadratic in the sampling rate. Two series describing the same trajectory at different rates are therefore not comparable as path lengths at all — and MINS publishes at ≈86 Hz against ≈20 Hz for /firmware/wheelodom, a ratio of 4.3.

That derivation has not, until now, been checked against data. The test is direct: decimate MINS to the wheel rate and recompute. Across six recordings in which both series are healthy:

| Recording | wheels | MINS at 86 Hz | MINS at 20 Hz |
| --- | --- | --- | --- |
| test1 | 20.81​ | 25.40​ | 23.84​ |
| test1 | 18.60​ | 16.78​ | 15.44​ |
| test1 | 20.57​ | 21.24​ | 19.57​ |
| test1 | 20.99​ | 23.35​ | 22.00​ |
| test2 | 18.57​ | 16.38​ | 15.11​ |
| zuptclean | 20.28​ | 23.22​ | 20.28​ |
| median ratio to wheels | — | 1.07​ | 0.98​ |

Decimation moves the median ratio from 1.07 to 0.98. At matched rates MINS and wheel odometry agree on distance travelled to within about two per cent, across six independent recordings — and since the two references are physically unrelated, one inertial-visual and one from wheel encoders, that agreement says something neither could say alone. The apparent excess at full rate is the predicted artefact, now observed.

What this does and does not license It licenses the statement that MINS’s scale is sound: over twenty metres of driving it neither stretches nor shrinks the path appreciably against an independent odometric reference.

It does not license a claim about accuracy. Two trajectories can have identical lengths and diverge steadily in position; path length is blind to heading error, which is precisely the channel this platform is weakest in (§12.23.3) and which §11.8.5 takes up next. Nor is the wheel reference itself exact: the effective radius carries a calibrated reff=62.2 mm against a stock 62.5 mm, and mecanum rollers scrub. The two per cent should be read as “no gross scale error detected”, not as a measured accuracy.

### Heading: two references that disagree, and neither is truth

The previous subsection established that MINS’s path length agrees with wheel odometry to two per cent, and closed by noting that length is blind to heading. This one looks at heading directly, and the result is less comfortable.

Unwrapping both yaw signals from a common origin and comparing accumulated rotation at the end of each recording:

| Recording | $¦{\rm rot}¦$ wheels | duration | disagreement | per rotation |
| --- | --- | --- | --- | --- |
| test1 | 758∘​ | 928 s | 22.0∘​ | 2.90 %​ |
| test1 | 535∘​ | 129 s | 15.9∘​ | 2.98 %​ |
| test1 | 511∘​ | 163 s | 42.6∘​ | 8.34 %​ |
| test1 | 600∘​ | 213 s | 44.0∘​ | 7.32 %​ |
| test2 | 509∘​ | 137 s | 22.8∘​ | 4.48 %​ |
| zuptclean | 2052∘​ | 600 s | 92.7∘​ | 4.52 %​ |

Two things are solid. The disagreement is systematically one-signed — MINS reports more accumulated rotation than the wheels in all six recordings, never less — and it is large: a median 4.5 % of cumulative rotation, or equivalently ≈10∘ per minute of operation. On runs that amount to roughly one full turn, the two references end tens of degrees apart.

##### The discrimination fails, and that is the finding.

The obvious next question is whether this is a scale error, which would scale with rotation, or a drift, which would scale with time. Normalising by each and comparing dispersion:

CVper rotation=0.41,  CVper unit time=0.47.

The rotation hypothesis is marginally more stable, and the margin is far too thin to conclude from. Both normalisations leave a spread of nearly a factor of three (2.90 % to 8.34 %; 1.4 to 15.7∘/min). No single mechanism explains these six recordings. The likely reading is that both contribute and that configuration heterogeneity across the campaign dominates either — but that is a hypothesis, and it is recorded here as one.

Why this cannot be resolved with the sensors on the rover It is tempting to declare the wheels right and MINS wrong, or the reverse. Neither is available. The wheel heading applies a 1.76 scrub compensation calibrated for rubber tyres to mecanum rollers that do not scrub (§12.23.3) — the report’s own analysis says that number is wrong for this hardware. MINS’s heading is the channel this campaign has repeatedly identified as its weakest. The comparison therefore places a bound on their mutual consistency and cannot attribute the error.

What it does establish is a requirement: closing this question needs a heading reference independent of both, and the platform does not carry one. The magnetometer on the incoming MinIMU-9 v5 is the first candidate the project will have had — which is a further reason to characterise that sensor properly rather than treat it as a spare accelerometer.

## A validity test that needs no reference at all

Every comparison so far has needed something to compare against — another estimator, wheel odometry, a closed loop. There is one test that needs nothing: the platform cannot exceed its own maximum speed. The LEO Rover is geared to roughly 0.4 m/s and has been measured at 0.21 m/s in normal operation. Any estimator reporting more than that is wrong, and no ground truth is required to say so.

The four recordings of 2026-08-21 in data/matlab/ carry all three estimators simultaneously, with velocity and full pose covariance — the only set in this campaign that does. Three are short stationary tests, the fourth is a seven-minute drive.

The three estimators against a constraint none of them can argue with. Left: peak reported speed, logarithmic; the dashed line is the rover’s 0.4 m/s mechanical ceiling and everything above it is impossible by construction. Right: the fraction of samples above that ceiling. MINS does not appear in the right panel because its bar is zero on all four recordings.

### What the three estimators do with an impossible claim

| Recording | estimator | peak ¦v¦ | % above ceiling |
| --- | --- | --- | --- |
| stationary, 30 s | MINS | 0.010​ | 0.0 %​ |
|  | openVINS | 0.032​ | 0.0 %​ |
|  | sqrtVINS | 1.148​ | 100.0 %​ |
| stationary, 20 s | MINS | 0.017​ | 0.0 %​ |
|  | openVINS | 0.023​ | 0.0 %​ |
|  | sqrtVINS | 5.779​ | 100.0 %​ |
| stationary, 10 s | MINS | 0.010​ | 0.0 %​ |
|  | openVINS | 0.030​ | 0.0 %​ |
|  | sqrtVINS | 1.147​ | 100.0 %​ |
| driving, 420 s | MINS | 0.264​ | 0.0 %​ |
|  | openVINS | 10.839​ | 47.5 %​ |
|  | sqrtVINS | 32.835​ | 47.6 %​ |

MINS is inside the envelope on every sample of every recording, including a peak of 0.264 m/s on the drive that sits sensibly below the ceiling. That is a stronger statement than any comparison against another estimator, because it cannot be argued with: the platform could not have done what the other two describe.

### Dating the sqrtVINS result, which changes what it means

Reading the table as a ranking would be wrong, and the dates say why. These recordings are from the evening of 2026-08-21. The 02zuptguards patch — the one that removed the !disparitypassed && short-circuit, measured at 290 acceptances against 0 rejections with |v|=1.0×1023 m/s — is dated 2026-08-24, three days later.

What Figure 11.4 shows for sqrtVINS is therefore not its current behaviour. It is a measurement of the defect that patch corrected, taken before the correction existed, and it is worth keeping for exactly that reason: the bug report describes a χ2 gate that never rejected, and this is what that looks like from the outside — a filter reporting 5.8 m/s on a platform standing still, for one hundred per cent of a twenty-second recording.

openVINS is a different matter. It is not covered by that patch, it is clean on all three stationary recordings, and it spends 47.5 % of the seven-minute drive above the ceiling. That behaviour is not dated away by anything in this campaign.

### The covariance is not always honest either

These files also carry each filter’s own 3σ bounds, which allows a question asked nowhere else in this report: does the uncertainty an estimator reports actually contain its error?

On the 20-second stationary recording, sqrtVINS ends 0.151 m from where it started while reporting a final 3σ of 0.063 m. The error exceeds by a factor of 2.4 the interval the filter claims contains the truth with 99.7 % probability. That is overconfidence, and it is more dangerous than a large error honestly declared: a consumer weighting this pose by its stated covariance would trust it far more than it deserves.

The other two err the opposite way on the same recordings. MINS reports 3σ between 1.1 and 1.4 m for closure errors of 3 to 8 mm — conservative by two orders of magnitude — and openVINS reports 0.26 m for millimetre closures. Neither is well calibrated; but a filter that overstates its uncertainty degrades gracefully, while one that understates it invites exactly the trust it cannot support.

Why the ceiling test deserves to be automated poseselector already computes this quantity: its maxplausiblespeed guard exists to freeze the output when a source claims impossible motion, and §11.8 measured it suppressing a 4 km divergence. What the present section adds is that the same threshold is a diagnostic, not only a filter. The fraction of samples above the ceiling is computable offline from any recording, needs no reference trajectory, and separates the three estimators cleanly here. It costs nothing to log and would have flagged the sqrtVINS defect on the first recording rather than three days later.

## Is the filter honest about its own uncertainty?

An estimator publishes two things: a state and a covariance. Every consumer downstream — a fusion node, a χ2 gate, a planner deciding whether it knows where it is — weights the first by the second. This report has measured the first extensively and has never once checked the second. This section does, and the answer differs by three orders of magnitude between estimators.

### The test, and why a stationary robot makes it exact

Consistency has a standard definition. For an estimate x of a true state x with reported covariance P, the normalized estimation error squared is

ε = x⊤P−1x,  x=x−x.

If the filter’s error is genuinely zero-mean Gaussian with covariance P — which is exactly what publishing P asserts — then ε follows a chi-squared law with as many degrees of freedom as the state has dimensions. Restricting to the 3×3 position block:

ε ∼ χ2(3),  E[ε]=3,  med(ε)=2.366,  F−1(0.95)=7.815.

A median far below 2.37 means P is too large — the filter declares more uncertainty than it has. A median far above means P is too small, and the filter is claiming a precision it does not possess.

Applying Eq. ([eq:nees]) normally requires ground truth, which this platform does not have. The stationary recordings remove that obstacle entirely: if the robot does not move, its true position is its initial position, exactly and by construction, so

p(t) = p(t)−p(0)

is the error itself, not an approximation of it. The three short recordings of 2026-08-21 are therefore the only place in this campaign where Eq. ([eq:nees]) can be evaluated without assumption.

### Result

Evaluating Eq. ([eq:nees]) sample by sample, discarding the first tenth of each recording as initialisation transient:

| Recording | estimator | median ε | σ factor | assessment |
| --- | --- | --- | --- | --- |
| stationary 30 s | MINS | 0.0003​ | 0.011​ | far too large |
|  | openVINS | 0.0001​ | 0.007​ | far too large |
|  | sqrtVINS | 0.0920​ | 0.197​ | too large |
| stationary 20 s | MINS | 0.0006​ | 0.016​ | far too large |
|  | openVINS | 0.0000​ | 0.005​ | far too large |
|  | sqrtVINS | 45.75​ | 4.39​ | overconfident |
| stationary 10 s | MINS | 0.0000​ | 0.004​ | far too large |
|  | openVINS | 0.0001​ | 0.008​ | far too large |
|  | sqrtVINS | 0.1471​ | 0.249​ | too large |

The fourth column restates the third in units an implementer can act on. Since ε is quadratic in the error and inversely quadratic in σ, a median NEES of εm corresponds to a reported standard deviation wrong by the factor

k = εmmed χ2(3) = εm2.366,

with k<1 meaning σ is inflated and k>1 meaning it is deflated.

Consistency on the three stationary recordings, where the true position is known exactly. Left: median NEES, logarithmic, against the χ2(3) median of 2.37 (solid) and its 95th percentile of 7.81 (dashed). MINS and openVINS sit four orders of magnitude below the reference; sqrtVINS exceeds the 95th percentile on one run. Right: the same result expressed by Eq. ([eq:sigmafactor]) as a multiplicative error on the reported σ, where unity is calibrated.

### Reading the two failure modes

##### MINS and openVINS inflate by roughly two orders of magnitude.

k between 0.004 and 0.016 means the published σ is 60 to 250 times larger than the error warrants. MINS announces 3σ≈1.13 m on a recording where it holds position to 6 mm. Nothing is wrong in the pose — the estimate is excellent — but the covariance is not usable information. A consumer weighting by it would treat a millimetre-grade pose as decimetre-grade, and a χ2 gate built on it would accept almost anything.

##### sqrtVINS deflates, on one recording out of three.

k=4.39 means its reported σ is 4.4 times smaller than its error justifies; Eq. ([eq:chi3]) puts a median ε of 45.75 far beyond the 95th percentile of 7.815, so this is not a tail event but the typical behaviour of that run. The same recording is the one where sqrtVINS reports 5.8 m/s on a stationary platform (§11.9) — the overconfidence and the impossible velocity are two views of one broken state, three days before the 02zuptguards patch.

The asymmetry between the two modes matters more than their magnitudes. An inflated covariance is wasteful: it discards information the filter actually has. A deflated one is actively misleading, because every downstream consumer is built to trust it in proportion to how small it is.

What this test does not establish Three recordings, all stationary, all from one evening. The stationary case is what makes the truth exact and the test possible, but it is also the regime where a filter is least stressed — the result says nothing about covariance calibration during aggressive motion, where the process-noise model does most of its work.

Extending it would need ground truth during motion, which returns to the same missing capability as §11.8.5: this platform has no external reference. What can be done cheaply is to repeat this exact test after any change to a noise parameter, since it costs one stationary recording and a matrix solve per sample, and it is the only check in this report that validates the second half of what an estimator publishes.

### The velocity block, where the truth is exactly zero

Section 11.10 tested the position block, taking the initial position as truth. The velocity block admits a cleaner test still: on a stationary platform the true velocity is not approximately zero, it is zero, so no subtraction and no reference epoch are needed. The error is the estimate:

v(t)=v(t)−0=v(t),  εv=v⊤Pvv−1v ∼ χ2(3).

This is worth doing separately because Pvv is the block that drives propagation: between measurements, position uncertainty grows from velocity uncertainty, so an error here compounds into the position block rather than staying local.

| Recording | estimator | median ¦v¦ | mean σv | εv​ | k​ |
| --- | --- | --- | --- | --- | --- |
| stationary 30 s | MINS | 0.0033​ | 0.0175​ | 0.041​ | 0.13​ |
|  | openVINS | 0.0277​ | 0.0084​ | 9.76​ | 2.03​ |
|  | sqrtVINS | 1.1448​ | 0.0199​ | 3.6×103​ | 38.8​ |
| stationary 20 s | MINS | 0.0042​ | 0.0238​ | 0.046​ | 0.14​ |
|  | openVINS | 0.0204​ | 0.0039​ | 20.9​ | 2.97​ |
|  | sqrtVINS | 5.7770​ | 0.0022​ | 8.5×106​ | 1899​ |
| stationary 10 s | MINS | 0.0030​ | 0.0167​ | 0.038​ | 0.13​ |
|  | openVINS | 0.0277​ | 0.0084​ | 9.76​ | 2.03​ |
|  | sqrtVINS | 1.1448​ | 0.0163​ | 5.2×103​ | 46.9​ |

Velocities in m/s; k from Eq. ([eq:sigmafactor]).

##### MINS is close to calibrated here.

k between 0.13 and 0.14 means its velocity σ is about seven times larger than warranted — conservative, but within one order of magnitude, against the two orders its position block was out by. Of the six covariance blocks measured in this report, this is the only one approaching usable calibration.

##### openVINS is overconfident on velocity.

All three recordings give εv between 9.76 and 20.87, every one above the χ2(3) 95th percentile of 7.815. Concretely, on the 20-second run it reports σv=3.9 mm/s while carrying a 20.4 mm/s velocity on a stationary robot — an error five times its own one-sigma, sustained rather than transient.

The two covariance blocks of each filter, on the same recordings. Pale bars are the position block, solid bars the velocity block; the green band spans the 5th to 95th percentiles of χ2(3), within which a consistent filter should fall. MINS sits below it on both blocks, closer on velocity. openVINS falls far below on position and above on velocity — the same matrix wrong in opposite directions. sqrtVINS leaves the plot.

### The same matrix, wrong in opposite directions

Placing the two blocks side by side is the result of this chapter:

| Estimator | k (position) | k (velocity) |  |
| --- | --- | --- | --- |
| MINS | 0.004–0.016 | 0.13–0.14 | both conservative, velocity far less so |
| openVINS | 0.005–0.008 | 2.03–2.97 | opposite signs |
| sqrtVINS | 0.20–4.39 | 38.8–1899 | overconfident throughout |

openVINS is the striking case. Its position covariance is inflated by a factor of roughly 150 in σ, and its velocity covariance is deflated by a factor of 2 to 3 — two blocks of the same matrix, published in the same message, erring in opposite directions. Nothing in the pose alone reveals this; both blocks pass every check made earlier in this report, because those checks never looked at P.

The practical consequence follows from which block does what. A downstream gate reading the position block will be far too permissive. Propagation reading the velocity block will under-grow position uncertainty between measurements, which partly explains why the position block can stay wide while the filter’s own predicted growth is too slow: the two errors are not independent, they are the same miscalibration seen at two points of the same loop.

Why this is worth more than a better tuned number The instinct on seeing k=150 is to divide the noise density by 150 and move on. That would be tuning to a single stationary recording and is exactly the practice this report warns against elsewhere — the same mistake as inheriting a datasheet figure, with a different provenance.

What the measurement supports is narrower and more useful: a statement that these covariances are not calibrated, a number for how far, and a procedure costing one stationary recording that can be repeated after any change. A filter whose covariance is wrong by two orders of magnitude is not producing an uncertainty estimate at all — it is producing a constant that happens to have units of metres, and the honest move is to say so rather than to rescale it once and call it characterised.

### The angular block, and what the three blocks say together

The twist message carries angular velocity as well, and the stationary case makes its truth exact for the same reason: a platform that does not move does not rotate. Evaluating Eq. ([eq:neesvel]) on the ω block:

| Recording | estimator | median ¦ω¦ | σω​ | εω​ | k​ |
| --- | --- | --- | --- | --- | --- |
| stationary 30 s | MINS | 0.137​ | 0.00126​ | 3.59​ | 1.23​ |
|  | openVINS | 0.137​ | 0.00323​ | 0.55​ | 0.48​ |
|  | sqrtVINS | 0.094​ | 0.00323​ | 0.27​ | 0.34​ |
| stationary 20 s | MINS | 0.140​ | 0.00126​ | 3.75​ | 1.26​ |
|  | openVINS | 0.139​ | 0.00324​ | 0.56​ | 0.49​ |
|  | sqrtVINS | 0.095​ | 0.00324​ | 0.26​ | 0.33​ |
| stationary 10 s | MINS | 0.144​ | 0.00126​ | 3.94​ | 1.29​ |
|  | openVINS | 0.145​ | 0.00324​ | 0.61​ | 0.51​ |
|  | sqrtVINS | 0.095​ | 0.00325​ | 0.26​ | 0.33​ |

Angular rates in deg/s, σω in rad/s.

MINS’s angular block is calibrated. k between 1.23 and 1.29 places its reported σ within thirty per cent of what the error warrants, and εω between 3.59 and 3.94 sits close to the χ2(3) mean of 3. After a position block wrong by two orders of magnitude, this is the one covariance in the entire report that a consumer could use as published.

Note also that all three estimators report ≈0.14 deg/s of residual angular rate on a platform that is not turning — a direct reading of the gyroscope bias surviving the sanitiser, and one the two filters sharing /imu/dataclean agree on to three digits.

The σ error factor k of Eq. ([eq:sigmafactor]) for each state block, medians across the three stationary recordings with the shaded band giving their spread. Unity is calibrated. MINS declines monotonically from the directly measured angular rate to the twice-integrated position. The two pure VIO filters do not: both are overconfident precisely at linear velocity, the one channel for which MINS carries an independent measurement.

##### MINS degrades with integration depth; the others do not.

Figure 11.7 places the three blocks in order of how far each sits from a direct measurement. MINS falls monotonically — k=1.26 on the gyro-derived rate, 0.131 after one integration, 0.011 after two — losing about an order of magnitude per integration. That is the behaviour one would predict: uncertainty about an integrated quantity is dominated by the process-noise model, and modelling error compounds with each integration.

The two pure VIO filters break that pattern, and both break it at the same place. openVINS goes from k=0.49 on angular rate to k=2.03 on linear velocity — from conservative to overconfident in one step — before collapsing to 0.007 on position. sqrtVINS does the same more violently, 0.33→46.9→0.249.

A plausible reading, offered as a hypothesis Linear velocity is the one block where MINS has something the other two do not: a direct measurement from the wheels. A pure VIO must infer linear velocity from integrated acceleration and visual parallax, with no sensor observing it; MINS reads it from the encoders through the Wheel2DAng model.

That the two filters lacking a velocity measurement are the two that go overconfident on the velocity block is consistent, and it would explain why the anomaly sits exactly there rather than anywhere else in the chain. It is not established. Testing it would mean disabling MINS’s wheel updater and re-running the same measurement, which is a twenty-minute experiment and is recorded here as the next thing to do rather than as a conclusion already drawn.

### A covariance that has stopped being a variable

The stationary recordings showed what the covariances are worth. The seven-minute drive shows how they behave, and for MINS the answer is that its position covariance stops behaving at all.

A filter propagating between corrections should see P grow, since integrating IMU noise adds uncertainty; each accepted measurement should pull it back. The published series should therefore saw-tooth. Sampling MINS over the window 150≤t≤420 s — 23,137 samples, during which the estimator itself reports 6.09 m of travel:

| Window 150–420 s | MINS | openVINS |
| --- | --- | --- |
| 3σ minimum | 12.8908 m | 0.2600 m |
| 3σ maximum | 12.9255 m | 4.0005 m |
| range over 270 s | 0.0347 m | 3.7405 m |
| coefficient of variation | 2.8×10−4​ | 5.8×10−1​ |
| path reported over window | 6.09 m | 2307.61 m |

MINS’s reported position uncertainty varies by three and a half centimetres across four and a half minutes of driving — three parts in ten thousand. It is not a saw-tooth, not a growth curve, not a decay: it is a constant that happens to be expressed in metres.

### Reading the two behaviours together

The contrast with openVINS on the same window is what makes the point, because the two failures are opposite and neither is visible from the pose alone.

openVINS’s covariance does move — a coefficient of variation of 0.58, a step from 0.26 to 3.67 m around t=230 s, genuine structure. But over that same window it reports 2307 m of travel on a platform that covered a few metres. Its covariance is alive and its state is nonsense.

MINS is the mirror image. Its pose over that window is the only one of the three that stays physically possible (§11.9), and its covariance is inert. A consumer reading only the pose would prefer MINS without hesitation; a consumer reading only the covariance would see a number that never changes and conclude, wrongly, that nothing was happening.

What a saturated covariance costs, concretely Three consequences follow, and none of them show up as an error message.

A χ2 gate built on it cannot reject. The gate compares a residual against P; a P two orders of magnitude too large accepts essentially any measurement, which removes the very protection the gate exists to provide.

Fusion weighting is wrong by the same factor. Any node combining this pose with another source weights by P−1, so MINS is weighted a hundred times less than its accuracy deserves.

Divergence becomes undetectable from the covariance. A filter losing its fix normally announces it by a growing P. If P is pinned at a ceiling, the two spontaneous divergences of §12.28 — item 47, still open — would look identical to healthy operation in the one channel designed to warn about them. That is consistent with what was observed on 2026-09-08, where the failure was found by reading z directly and not by any covariance signal.

### Everything measured in this chapter predates the fixes

The measurements above are dated, and placing them against the patch history changes what the chapter can claim. Both VIO estimators received divergence fixes during the campaign; neither has been recorded since.

The 02zuptguards patch of 2026-08-24 modifies ovsrvins/src/update/UpdaterZeroVelocity.cpp — sqrtVINS. The 01resetcovariance patch of 2026-08-28 modifies ovmsckf/src/core/VioManager.cpp — openVINS. Searching the data directories for anything recorded after that second date returns nothing.

Three consequences for how this chapter should be read.

The covariance audit is entirely pre-patch. The recordings carrying P are from 2026-08-21, before either fix. Every statement in §11.10 through §11.10.7 describes filters as they were that evening. The MINS results are unaffected — MINS received neither patch — but the openVINS and sqrtVINS numbers are historical by construction.

openVINS was never measured post-patch either. The 24 August recordings sit after the sqrtVINS fix but before the openVINS one, and they carry no covariance. On those recordings openVINS reports 10,244 m of path and a 3374 m excursion against MINS’s 91.9 m and 17.9 m over the same 415 seconds — worse than the 21 August figures, which is consistent with its own fix not yet existing rather than with anything the sqrtVINS patch did.

No patch in this campaign has a measured effect. Both are documented, both have a written rationale, both were verified against the symptom that motivated them at the time of writing. Neither has a recording taken afterwards under comparable conditions.

A curiosity in the 24 August exports, unresolved testbag and valide carry different bytes — their checksums differ and their position columns are not equal — yet they agree to five significant figures on every aggregate computed here: 20,766 samples, 415.3 s, 91.88 m of MINS path, 17.887 m of extent, and an identical median MINS–openVINS separation of 230.055 m.

Two independent runs do not do that. The likely explanation is one source bag exported twice through slightly different paths, but the exports carry no provenance field and nothing in the logbook records the session, so this is stated as an open question rather than resolved. Both were treated as a single recording in the figures above, which is the conservative choice if they are duplicates and a minor loss of sample size if they are not.

The remedy is inexpensive and specific: one stationary recording and one drive, exported with the covariance columns, on the current build. That would date the covariance audit to the software actually running, close the before-and-after on both patches, and cost about twenty minutes of robot time. It is recorded as item 50.

### A recording taken on the current build

Section 11.10.9 closed by noting that no estimator recording exists after either divergence patch, and that the remedy costs about twenty minutes. That recording was made on 2026-09-09: 242 seconds, 20,597 MINS samples with both covariance blocks, wheel velocity confirmed at 0.000 m/s throughout, on the software actually running today.

It closes half of item 50 and explains why the other half stayed open.

### MINS, then and now

Applying the identical procedure — same NEES definition, same χ2(3) reference, same first-tenth discard:

| State block | k on 2026-08-21 | k on 2026-09-09 | change |
| --- | --- | --- | --- |
| position | 0.011​ | 0.158​ | 14× closer |
| linear velocity | 0.131​ | 0.173​ | unchanged |
| angular velocity | 1.260​ | 1.195​ | unchanged, still calibrated |

Two results, of different kinds.

The angular block is calibrated on today’s build. k=1.195 against 1.260 three weeks ago. The single well-calibrated covariance found in this report is not an artefact of one evening’s recording; it survives every change made since, and it can be relied on.

The position block improved by a factor of fourteen — from a σ inflated ninety-fold to one inflated six-fold. The reported 3σ fell from 1.13–1.30 m to 0.534 m, while the pose itself did not change materially: position extent over the recording was 44.5 mm, inside the 4–46 mm range of the August runs. The estimate is as good as it was; the uncertainty attached to it is much less wrong.

A plausible cause, and why it is not asserted The obvious candidate is the extrinsic freeze of 2026-09-07 (§12.24.1). Setting docalibext: false removes the camera–IMU extrinsic from the estimated state; its uncertainty no longer enters the filter, and no longer propagates into the position block through the cross-covariances. A large drop in reported position variance is exactly what that change should produce, and the direction and rough magnitude match.

It is not established. Several things changed between the two recordings, the two sessions differ in every uncontrolled respect, and a single pair of measurements cannot separate one cause from the others. The clean test is available and cheap — revert docalibext for one stationary recording, re-run this procedure, and compare — and it is the natural companion to the wheel-updater experiment already proposed in §11.10.6. Both are recorded as experiments to run, not as conclusions reached.

### Why the two VIO filters recorded nothing

The same recording captured zero samples from openVINS and zero from sqrtVINS, on a session where both processes were running and their topics were advertised. That is not a failure of the recording: neither filter initialises on a stationary platform. Both wait for excitation before declaring a state, so a stationary run — the very condition that makes the NEES truth exact — is also the condition under which they publish nothing at all.

This is a genuine methodological obstacle rather than an oversight, and it sharpens item 50 rather than closing it. Dating the VIO covariance audit needs a recording that begins with enough motion to initialise both filters and then holds still long enough for the stationary test, in that order, without an intervening restart. That is a different and more demanding manoeuvre than the one performed here, and it is the reason the openVINS and sqrtVINS figures in this chapter remain dated 2026-08-21.

# The July 2026 Estimation Campaign: Tightly-Coupled Multisensor Fusion

This chapter documents the second major engineering campaign on the platform (July 3–7, 2026), which extended the mission-control system of the previous chapters with a precision (x,y,z) state-estimation stack, hardened the sensor-integrity chain against three failure modes observed live, upgraded the operator web platform, and integrated one hardware modification (rigid high-performance wheels). Every parameter cited below is justified by its measured contribution to drift reduction, and every remaining placeholder is explicitly registered in Section 12.28.

## Estimation architecture

### From mission control to metric estimation

The system of Chapters 3–5 estimated pose from wheel odometry alone, reset at each beacon. The campaign objective was to replace this dead-reckoning core with a fused estimate whose drift is bounded and measured, while keeping the beacon-reset ritual as a world-frame anchor. Two estimators were deployed, switchable live:

- MINS (Lee et al. 2023, 2025), the primary estimator: a tightly-coupled Multi-State Constraint Kalman Filter (MSCKF) fusing the onboard IMU, the two front wheel encoders and the D455 stereo infrared pair, with online calibration of every sensor’s extrinsics and time offset.

- OpenVINS (Geneva et al. 2020), the reference visual-inertial estimator (IMU + stereo only), used for A/B comparison and camera-calibration validation.

### Why tightly-coupled MSCKF, and not a decoupled EKF

A decoupled fusion stack (e.g. robotlocalization) fuses poses already estimated by upstream nodes, treating them as independent measurements. On this platform the visual and wheel odometries share the same IMU and the same clock: their errors are strongly correlated, and a decoupled filter double-counts the shared information, producing overconfident covariances: the statistical root of divergence. The MSCKF instead fuses raw measurements in a single state vector

x = [ ​GIq, ​GpI, ​GvI, bg, ba | xc1,…,xcN  θcalib,

where ​GIq is the orientation quaternion, ​GpI,​GvI position and velocity in the global frame, bg,ba the slowly-varying gyroscope and accelerometer biases, xci a sliding window of N historical pose clones, and θcalib the online calibration states (camera–IMU extrinsics and time offset, wheel extrinsics).

##### Propagation.

Each IMU sample (measured rate ωm, specific force am, at ∼80 Hz on this platform) propagates the state through the standard inertial kinematics

rCl ^I_G &=&   (_m - _g - _g)  ^I_G|q,
^G_I &=& ^G_I (_m - _a - _a) + ^G, ^G_I = ^G_I,

with ng,na white noises whose densities were identified on this unit by Allan-variance analysis (imuutils): σa=2.33×10−2, σba=8.93×10−4, σg=1.26×10−3, σbg=3.30×10−5 (SI units). Under-estimating these densities makes the filter overconfident in integration (drift); over-estimating them wastes the IMU. Measured values beat any generic default.

##### Error-state formulation and clone augmentation.

Like all modern MSCKFs, both estimators operate on the error state x rather than the state itself: orientation error is a minimal 3-vector δθ defined by ​GIq=δq⊗​GIq with δq≈[12δθ​⊤ 1]⊤, which avoids the quaternion’s unit-norm constraint in the covariance. At each image time the current pose is cloned: the state is augmented by xc=(​GIq,​GpI) and the covariance by the corresponding Jacobian rows,

P←PPJ​⊤JPJPJ​⊤,  J=∂xc∂x,

so that a feature observed across several images correlates those historical poses. The window is bounded (11 clones here): the oldest clone is marginalised out, keeping the filter O(N2) in a small constant N rather than growing like a smoother.

##### Camera update.

A feature tracked over the clone window yields a stacked residual r=Hxx+Hfpf+n. The landmark error pf is not in the state; left-multiplying by N​⊤, an orthonormal basis of the left nullspace of Hf (N​⊤Hf=0), gives

N​⊤r = (N​⊤Hx) x+N​⊤n,

a measurement of the state only: the defining MSCKF trick (Mourikis and Roumeliotis 2007). A feature seen in m stereo frames contributes 2⋅2⋅m−3 scalar constraints at the cost of one QR factorisation, which is what keeps the filter real-time.

##### Observability and why FEJ matters.

Ideal VIO has exactly four unobservable directions: global position (3) and rotation about gravity (yaw). A naively-linearised EKF evaluates Jacobians at ever-changing estimates, which spuriously renders yaw observable: the filter then believes it knows yaw better than it does, covariance collapses, and the estimate drifts with false confidence. First-Estimates Jacobians (FEJ) fix the linearisation points of each state to their first estimate, preserving the true nullspace. On this platform the wheel measurements add scale and planar-velocity information (shrinking drift in x,y,ψ), while gravity observation through the accelerometer keeps roll/pitch observable at all times, leaving z as the axis observed only by the camera: the structural fact behind the entire vertical-drift investigation of Section 12.7.2.

##### Wheel update.

The rigid differential-drive constraint (Wheel2DAng model) maps the two front angular velocities ωl,ωr to body velocities

v = rl ωl+rr ωr2,  ψ = rr ωr−rl ωlb,

with rl,r the wheel radii and b the track width. These parameters are hardware-dependent; their status after the wheel replacement is discussed in Section 12.4.

##### Wheel measurement in the filter.

MINS integrates the body-frame kinematics ([eq:wheel]) between two consecutive clone times to form a 2D relative-pose constraint (planar translation and heading change) between those clones, with a noise model built from the configured angular-rate noise σω=0.2 rad/s propagated through the same integral. The wheel extrinsic ​ITW (IMU to wheel-odometry origin) is an online-calibrated state; the intrinsics (rl,rr,b) are not (Section 12.4). Because the model assumes two coaxial wheels, a four-wheel skid-steer platform violates it whenever it turns: the lateral scrub of the non-modelled wheel pairs injects a bias that must be absorbed either by the noise model or by the gate below: the quantitative reason the gate multiplier cannot be set to its textbook value of 1.

##### Outlier gating: the anti-slip mechanism.

Every wheel (and camera) measurement passes a Mahalanobis χ2 test against the inertially-propagated prediction before being allowed to update the state:

d2 = r​⊤(HPH​⊤+R)−1r  >  α χ95%2​dimr ⇒ measurement rejected,

where α is the per-sensor multiplier. This test is the “IMU-versus-odometry comparison” that detects wheel slip: a skidding wheel reports a velocity incompatible with the inertial prediction and is rejected before entering the filter: something a decoupled architecture cannot do, having already baked the slip into an upstream pose. The campaign tightened the wheel multiplier from α=15 (slip passed the gate) to α=5; α=1 was deliberately not used because skid-steer turning violates the two-wheel differential model ([eq:wheel]) in normal operation.

##### Failure analysis of the historical z≈−445 m divergence.

The magnitude of the pre-campaign failure is fully explained by the frozen-IMU mode of Section 12.2.4. The frozen sample carried a=13.06 m/s2 with an orientation implying an unmodelled residual specific force after gravity compensation. An unmodelled constant residual δa integrates quadratically,

δz(t) = 12 δa t2,

so even a modest δa≈1 m/s2 produces 445 m in t=2⋅445/1≈30 s of effectively open-loop integration, consistent with the observed collapse once camera updates were starved (Section 12.2.2) and wheel updates gated out by the contradictory frozen gyro. The lesson generalises: a tightly-coupled filter is only as sound as the integrity of its inertial stream, which is why the guards of Section 12.2.4 sit upstream of everything.

### Dual-estimator output and the lossless switch

Both estimators publish odometry; a selector node (poseselector.py) republishes exactly one of them on /robotposefused. At the instant of a switch at time t, with T∈SE(3) denoting poses, the selector computes a one-time rigid correction

Tcorr = Tout(t−) Tnew(t)−1,  Tpub(τ) = Tcorr Tnew(τ)  ∀ τ≥t,

so the published pose is bit-for-bit continuous across the switch even though the two estimators drift independently. This is not merely a design intent; it follows algebraically from the definition of Tcorr. Evaluating Tpub at τ=t itself,

Tpub(t) = Tcorr Tnew(t) = Tout(t−) Tnew(t)−1 Tnew(t) = Tout(t−) Tnew(t)−1Tnew(t)⏟= I = Tout(t−),

so Tpub(t) equals the pre-switch output exactly, not approximately: the correction is constructed precisely to cancel the new source’s raw pose at the switch instant, for any Tnew(t)∈SE(3) (the group inverse exists unconditionally). The identity depends on nothing about how the two estimators drift relative to each other, only on evaluating both sides of the correction at the same instant t; consistent with this, drift between the sources reappears immediately for τ>t, since Tcorr is applied but never updated again until the next switch. The switch is exposed as a ROS service (/poseselector/setsource, stdsrvs/SetBool, true = MINS) and refuses to select a source that has not yet published: the reason OpenVINS must be initialised (Section 12.3.5) before VINS can be selected.

## Sensor chain, transport, and integrity guards

### End-to-end data flow

Figure 12.1 shows the complete chain as deployed. Estimation runs on the ground-station PC; the Raspberry Pi 4 on the rover only acquires and compresses.

Deployed estimation data flow after the isolation lockdown (July 8). Blue: VINS path; red: MINS path. Dashed red: the Carolus global-anchor channel, semantically private to MINS (/mins/externalref/carolus, staged); OpenVINS has no code path able to consume it. Dashed verticals: each estimator’s TF broadcast is remapped onto an isolated channel (/tfvins, /tfmins), leaving a single authority per transform on the main /tf tree. Every box is a running node or live topic; nothing is notional: a claim taken literally: the firmwaremessageconverter box was added on 2026-07-22 after a live fault revealed it had been missing from every earlier revision of this figure, converting leomsgs/WheelStates into standard sensormsgs/JointState for wheelremap downstream, wedged for hours in a degraded IMU-only state with no crash and no respawn configured (§12.9.3).

##### Architecture Design Note: TF authority disambiguation and data encapsulation.

Two invariants were locked into the launch layer on July 8. (i) TF authority disambiguation: a TF child frame admits exactly one parent, and the latest broadcast wins; with both estimators emitting global→ imu, the URDF’s static imu frame was silently captured and the two estimators would fight for authority during A/B operation. Remapping each estimator’s /tf onto a dedicated channel (/tfmins, /tfvins) restores a strict single-writer discipline on the primary tree (the selector alone writes odom→baselink) while keeping estimator internals fully inspectable on demand. (ii) Data encapsulation: the Carolus global-anchor stream is namespaced /mins/externalref/carolus, declaring its single legitimate consumer in the topic name itself; exclusivity is enforced at three levels: architecturally (OpenVINS’s subscriber pipeline wires only IMU and camera topics), semantically (private namespace), and actively (a supervision rule raises an alert if any node other than minssubscribe subscribes). These are systems-engineering guarantees, not conventions: each is either structurally impossible to violate or monitored.

### The WiFi transport budget: why compression is not optional

The rover’s access point sustains ≈6 MB/s in practice. At the start of the campaign the PC subscribed to raw image topics; with the then default profile (848×480 stereo pair at 30 Hz plus a duplicate subscription by the mission backend) the demanded bandwidth was

2×(848×480) B×30 Hz + (848×480) B×30 Hz ≈ 36.6 MB/s ≫ 6 MB/s,

which not only starved MINS to ∼3 Hz of camera data but twice crashed the Pi’s WiFi firmware outright. The corrective architecture moves the compression to the camera node’s imagetransport plugins (JPEG for IR, PNG for depth; compression happens only while subscribed), crosses the WiFi once per stream at ∼2 MB/s total, and republishes locally on the PC (/pc/camera/...) for all consumers. The arithmetic of the corrected chain: a 640×480 Y8 infrared frame JPEG-compresses to 30–60 kB (structured indoor scenes), so two IR streams at 15 Hz cost 0.9–1.8 MB/s; the depth stream uses PNG (compressedDepth) because JPEG’s lossy DCT would corrupt metric depth values, at a similar budget. One decompression per stream on the PC then serves all local consumers (both estimators, the mission backend, the AprilTag detector): the WiFi is crossed exactly once per stream regardless of consumer count. Timestamps survive the hop unchanged (republishing preserves headers), so estimator synchronisation is unaffected; measured stereo pairing after the hop is exact (74/74 identical stamps in verification). Companion relays mirror camerainfo into the same namespace because image-transport consumers pair image and calibration by namespace convention, not by remap: a subtlety that silently starves any consumer whose calibration topic is elsewhere. Operational rule derived: never subscribe raw robot images across the WiFi; always measure camera rates on the Pi. A second hardware rule was learned the hard way: depth and infrared are views of the same stereo ASIC and must run identical resolution and frame rate: mismatched profiles fail silently (topics advertised, zero frames, no error message), a failure mode now documented inside the boot launch itself.

### TF-tree coherence: isolating estimator broadcasts

A live TF audit (July 8) revealed that MINS’s visualisation broadcast (global→imu at ∼85 Hz) silently captured the URDF’s static imu frame (a TF child has exactly one parent, and the high-rate dynamic transform shadows the static one), and that OpenVINS, once initialised, would broadcast the same frame names, creating a permanent two-authority conflict during A/B operation. Both estimators’ TF outputs are visualisation-only (all functional consumers use odometry topics), so the corrective is a launch-level remap of each node’s /tf onto dedicated channels (/tfmins, /tfvins); the main tree retains a single authority per transform (verified post-fix: only odom→baselink from the selector and the URDF chains remain on /tf). A redundant static baselink→laser placeholder, duplicating the URDF’s own rplidarlink/laserframe chain, was removed in the same pass.

### IMU integrity: three observed failure modes, three guards

All three failure modes below were observed live during the campaign; the sanitizer (imusanitizer.py) now guards each:

- Isolated spikes (up to 5×1016 m/s2, I2C/rosserial framing): any sample with |ωi|>35 rad/s or |ai|>160 m/s2 is replaced by a zero-order hold of the last good sample.

- Firmware freeze: after a serial desynchronisation burst the LeoCore may retransmit one identical sample indefinitely at full rate. A true sensor always carries noise, so N consecutive bit-identical samples is unambiguous; at N=200 (≈2.5 s) the sanitizer logs the fault and autonomously calls the firmware reset service (/firmware/resetboard, throttled to one per minute). The freeze is fatal to initialisation if unguarded: a frozen a=13.1 m/s2 with non-zero frozen gyro contradicts the standstill reported by the wheels.

- Non-monotonic timestamps: rosserial replay can move stamps backwards by up to ∼0.4 s, which aborts MINS on the internal invariant clonetime >= state->time. Any sample with tk≤tk−1 is dropped entirely (a duplicated stamp is as fatal as a backward one).

##### Why each guard is statistically sound.

(i) The spike thresholds sit three orders of magnitude above physical rover dynamics (ωmax≈1 rad/s, amax≈2 m/s2) and thirteen below the observed glitches: the two populations do not overlap, so the classifier has effectively zero error rate. The zero-order hold preserves the sampling cadence the propagation integrator expects; since observed glitches are isolated single samples, the hold error is bounded by one sample period of true dynamics (≤12.5 ms). (ii) For the freeze detector, a healthy MEMS channel quantised to float32 carries thermal noise of σ≳10−3 in SI units; the probability of even two consecutive bit-identical 6-axis samples is negligible (<10−30), so N=200 is not a statistical threshold but a debounce: it trades 2.5 s of detection latency for immunity to any conceivable coincidence, and bounds the automatic reset rate. (iii) Monotonicity is a causality requirement of the filter, not a tuning choice: the propagation integral ([eq:imukin]) is defined over increasing time, and MINS enforces it as a hard assertion; dropping the offending sample loses at most 12.5 ms of excitation, against a guaranteed estimator abort otherwise.

A fourth architectural guard follows from a failure post-mortem: the MINS node runs under respawn supervision, never required: an estimator abort must never take the operator’s drive chain down with it. The associated trade-off is documented: on respawn MINS re-initialises at its own origin and the published /mins/imu/odom jumps; the SE(3) correction ([eq:se3]) only absorbs switch discontinuities, not same-source re-initialisations: an accepted limitation until estimator-side state persistence exists.

### Local gravity

The generic g=9.81 m/s2 was replaced by the local normal gravity at Melbourne, FL (latitude φ=28.1∘N, sea level), from the WGS-84 Somigliana model:

g(φ)=9.7803253359 1+0.00193185265241 sin2φ1−0.00669437999013 sin2φ ⇒ g(28.1∘)≈9.790 m/s2.

The 0.02 m/s2 discrepancy of the generic value otherwise appears as a permanent fictitious accelerometer bias: a free systematic error.

## Calibration

### Camera intrinsics

Kalibr (Furgale et al. 2013) calibration of the stereo IR pair at 640×480 (the exact resolution now streamed) achieved reprojection errors of 0.20–0.23 px and a stereo baseline of 90.2 mm; intrinsics are therefore frozen in both estimators (docalibint: false): offline measurement beats online refinement. The measurement noise is configured conservatively at σpx=1 (five times the calibrated residual) to absorb JPEG artefacts introduced by the WiFi hop.

### Camera–IMU extrinsics: the resolved cause of vertical divergence

MINS expects ​ITC=(​CIR, ​IpC). The initial configuration carried an identity placeholder: geometrically the claim that the optical axis points up along the IMU z-axis, a ∼90​∘ error from which online refinement cannot recover (far outside the EKF linearisation region). The corrected seed encodes the physical forward-looking mount. With optical axes xC right, yC down, zC forward and body axes xI forward, yI left, zI up:

​CIR=001−1000−10,  ​IpC0=0.1500.10m,  ​IpC1=​IpC0+​CIR0.090200,

(the second camera sits one baseline to the right along xC, i.e. −yI). The translation is a tape-measure seed. Updated 2026-09-07: it is no longer refined online — docalibext is now false, so centimetre-level translation error is no longer recoverable either, and stands exactly as installed. Ninety degrees of rotation was never recoverable and still is not. The time offset between camera and IMU is likewise an estimated state (docalibdt: true); at v=0.4 m/s an unmodelled 10 ms skew biases every visual measurement by 4 mm. Open item: replace the seed by kalibrcalibrateimucamera output (monocular cam0 approach recommended) via the prepared fillminscamchain.py. Figure 12.2 illustrates the geometry.

Forward-looking camera–IMU geometry encoded by Eq. ([eq:extr]). The identity placeholder previously claimed zC∥zI (camera looking up), the root cause of the divergent vertical drift quantified in Section 12.7.2.

##### Time-offset observability.

The camera–IMU offset td enters the measurement model through the pose at the shifted time: a feature observed at camera stamp t actually constrains the pose at t+td. To first order the induced residual is δz≈Jp (​GvI δtd)+Jθ (ω δtd), observable whenever the platform translates or rotates, which is why docalibdt converges during normal patrol motion and why the offset cannot be identified at standstill.

### Inertial calibration, and what a stationary measurement cannot determine

Both estimators consume the same inertial stream, so an error in it is common-mode: it cannot be detected by comparing MINS against openVINS, which is the comparison this chapter otherwise relies on. The IMU was therefore characterised directly, against the only two references available without added instrumentation: local gravity, and zero.

##### Sensor model.

Write the measured angular rate and specific force as

ωm=Kg ω+bg+ηg,  am=Ka a+ba+ηa,

where Kg,Ka∈R3×3 collect scale-factor and misalignment errors, b are the biases and η the noise. A perfect sensor has K=I, b=0.

##### Stationary characterisation.

With the rover verified stationary by wheel odometry (0.000 m/s, 502 samples, σ=0.02), two defects were measured on the raw /firmware/imu stream.

The accelerometer magnitude read 8.7525 m/s​2 against a true local gravity of 9.790 m/s​2 (Melbourne, FL, latitude 28∘): an error of −10.6%. Two controls establish that this is the sensor and not the processing chain. First, raw and sanitised streams agreed to 0.005 m/s​2 (8.7525 against 8.7475), so the guards of §12.2.4 are not responsible. Second, it is not a mounting tilt: the norm of a vector is invariant under rotation, so a tilted-but-correctly-scaled sensor would still report 9.790.

The gyroscope, stationary, read (−2.407, +1.525, +2.328) deg/s, i.e. bg=3.679 deg/s. A healthy MEMS gyroscope sits between 0.01 and 0.1 deg/s, placing this part 40 to 400× outside specification.

##### Observability: the asymmetry between the two sensors.

This is the substantive point of the subsection, and it mirrors the time-offset argument above. Setting ω=0 in Eq. ([eq:imumodel]) gives

ωm|ω=0=bg+ηg.

The matrix Kg has been multiplied by zero. It is absent from the observation, not merely poorly conditioned: the Fisher information of Kg under a stationary regime is identically zero, so no amount of averaging, no number of stationary orientations and no increase in sample count can recover it. The bias, by contrast, is directly observable as the sample mean of Eq. ([eq:gyrorest]).

The accelerometer behaves differently, and the reason is instructive: at rest its input is not zero. Gravity is a permanent excitation of known magnitude, so

am|rest ≈ Ka g+ba,

and the isotropic part of Ka is identifiable from a single orientation. Gravity plays for the accelerometer the role that no permanent rotation plays for the gyroscope. That asymmetry, not a difference in effort or instrumentation, is why an accelerometer scale error could be measured and corrected on this platform while the gyroscope’s could not.

Eq. ([eq:accelrest]) also states the limit of what was achieved. With the rover level, one scalar equation cannot separate an isotropic scale error from a bias along the gravity axis, nor resolve Ka per axis. A six-position calibration (the sensor tilted and inverted) is required to settle that. The correction deployed here is therefore empirical: defensible in the platform’s working regime, which is essentially level, and flagged as such rather than presented as a full inertial calibration.

##### Corrections applied.

The accelerometer is rescaled by

sa=9.7908.7525=1.1186,

chosen in preference to the alternative of patching gravitymag on the openVINS side alone: the latter would leave the two estimators disagreeing about the same physical quantity and would introduce a systematic scale error into the trajectories themselves. The gyroscope bias is subtracted, and re-estimated at every node start rather than frozen as a constant: bias drifts with temperature, and this Pi runs at 86 ∘C (§17.17.1), so a hard-coded offset would go stale. The auto-calibration adopts the measured mean only if the gyroscope variance shows the platform is not rotating; translation does not affect a gyroscope, so a stationary or straight-driving rover yields a valid estimate, while a rotating one is rejected and the parameter defaults stand.

##### Why this mattered more than its magnitude suggests.

Attitude is the integral of angular rate, so the measured bias produces an attitude error ϵθ≈bg t≈37∘ after only 10 s. That error mis-projects gravity into the body frame, injecting a phantom horizontal specific force gsinϵθ≈5.9 m/s​2: five times the 1.06 m/s​2 residual left by the accelerometer scale error alone. Double-integrated,

Δx ≈ 12 gsinϵθ t2 ≈ 300 m in 10 s,

which matches the divergence observed live on openVINS (501→1100 m in 12 s). The model also predicts, correctly, that correcting the accelerometer alone would be insufficient, which is what was measured before the gyroscope bias was addressed.

##### Delivered signal.

After both corrections the sanitised stream reads a=9.7938 m/s​2 at rest against a target of 9.790, and bg=0.0158 deg/s: an order of magnitude better than the 0.251 deg/s achieved earlier in the project, and inside the specification band the part should have occupied from the start.

##### The scale factor that remains unmeasured.

A ~gyroscale parameter exists in imusanitizer.py and is deliberately set to unity, i.e. to no correction at all. By Eq. ([eq:gyrorest]) it had never been measured, and determining it requires a rotation of known amplitude against physical ground truth: with the bias already removed,

Δθ=0T(ωm−bg)​⊤​ez dt = kg Δθtrue,  kg=ΔθΔθtrue.

The field campaign of §12.14.4 attempted exactly this measurement and returned kg−1=0.9329 for the raw gyroscope, and 0.9123 for wheel-derived heading. Because wheel heading is computed from differential odometry and does not use the gyroscope at all, these are physically independent instruments; MINS and openVINS are not independent witnesses here, since both integrate the same gyroscope. Two independent instruments agreeing with each other to within 2% while disagreeing with the protractor by 7–9% indicts the reference, not the sensors.

Nothing was therefore applied. The parameter remains at unity pending a repeated campaign with the improved floor-marking protocol of §17.13.3, which raises the angular reference from the ±1∘ of a protractor on carpet to ±0.11∘. This is recorded as an open item rather than closed with a plausible number, because ~gyroscale enters both estimators simultaneously: an error introduced there would propagate into every result in this chapter, and would do so invisibly.

### Visual front-end density audit

KLT tracking and grid-based FAST extraction both scale approximately linearly with the feature count N: each tracked feature costs one pyramidal Lucas–Kanade solve, O(W2L k) for window W (15 px), pyramid depth L (3–4 levels to absorb inter-frame motion up to ∼W⋅2L pixels) and k Gauss–Newton iterations; extraction adds a FAST corner response over each grid cell followed by non-maximal suppression within the minimum-separation radius. The deployed configuration tracked N≈600 features (cap 1500, 15×15 grid) while the MSCKF update consumes at most 50 (maxmsckf): a 12× waste measured at 120 ms per image against a 66.7 ms budget (15 Hz), i.e. the front-end ran outside real time at 73% of one core. The audited configuration (Nmax=300, grid 10×8 (80 cells of 64×60 px, ≈4 features per cell, uniform coverage by construction), minimum separation 20 px) yields a median pool of 149 well-spread features, 67 ms per image at 55% CPU, with the update still fully fed (50 used from 149: selection margin 3×). Full-resolution processing was retained (downsample: false): the “intelligent downsampling” is performed once at the source (848→640, 30→15 Hz, the calibrated resolution). Halving the image again (320×240) would divide the stereo triangulation accuracy: the depth error of a stereo pair grows as

σZ = Z2f B σd,

with f the focal length in pixels, B=90.2 mm the baseline and σd the disparity error; halving resolution halves f and thus doubles σZ at constant σd: an unacceptable price on the one axis (z) that only the camera observes. The residual per-image floor of ≈65 ms is dominated by the constant per-frame work (pyramids, histogram equalisation, extraction sweep) rather than by the feature count, which is why further feature reduction shows diminishing returns and the remaining margin lever is the frame period itself: at 10 Hz the budget becomes 100 ms for an unchanged 67 ms cost, a guaranteed 33% margin (open item #4). Between images, the 80 Hz IMU carries the state: at patrol speed (0.4 m/s) the inter-frame displacement at 10 Hz is 4 cm, comfortably within the KLT pyramid capture range.

### OpenVINS initialisation on a ground robot

OpenVINS requires an accelerometer excitation to render gravity and scale observable. Two platform-specific adaptations were required: the jerk threshold was lowered from 1.5 (a handheld value) to 0.4, and the initialisation window from 2.0 s to 1.0 s after instrumented measurement showed KLT track lifetimes of ≈1 s on this feed; with a 2 s window the disparity check permanently failed with “not enough feats”. Operationally: after every cold start, a one-second forward–back jolt initialises VINS; MINS by contrast initialises standing still in 4–12 s (static IMU+wheel initialisation), which is why it is the default source.

## Hardware modification: rigid high-performance wheels

During the campaign the stock tyres were replaced with rigid, higher-performance wheels. The estimation-relevant consequences are:

- Effective radius. The wheel model ([eq:wheel]) converts angular rates to metric velocity through rl,r. The values frozen in the configuration (r=62.5 mm, b=358 mm) are the firmware values for the original wheels. A radius error δr produces a proportional scale error δr/r on all wheel-derived displacement: 1 mm of radius error is 1.6% of odometric scale, i.e. 8 cm over a 5 m leg, an order of magnitude above the millimetric planar noise floor of Table 12.3. Mandatory action (open item #1): measure the loaded radius of the new wheels by roll-out: drive n marked wheel revolutions in a straight line over a measured ground distance d and compute

reff = d2πn,

- under load (batteries mounted), on the operational floor, averaged over n≥10 revolutions so the tape-measure uncertainty (±2 mm) contributes <0.1 mm to reff. The track width b is re-identified by the complementary spin test: N full in-place rotations give beff=reff (∫|ωr−ωl| dt)/(2πN). Update both the firmware diffdrive parameters and configwheel.yaml consistently: MINS reads the latter, the robot’s own velocity controller the former.

- Reduced compliance. Rigid wheels remove tyre squash, making the effective radius consistent across load and speed: a benefit to the constancy of ([eq:wheel]) once re-measured.

- Grip and the χ2 gate. Harder wheels on smooth lab flooring change the slip signature (less compliant grip, sharper slip onset). The tightened gate α=5 (Eq. [eq:chi2]) was validated after the wheel change; the slip-rejection protocol of Section 12.7 should be re-run if the floor surface changes.

- Drive polarity inversion. After the wheel replacement the physical forward/backward sense was found inverted with respect to the sign of cmdvel.linear.x, in both manual and autonomous modes. The correction is applied at the single command egress point (publish(): a named DRIVEPOLARITY constant), so every drive path (manual, patrol, approach, pivot, return-to-base) inherits it consistently. A heading-projected odometry test (−0.186 m reported along the firmware heading for a +0.18 m commanded forward pulse) further indicates the firmware’s body-frame forward is now rotated 180​∘ from the camera axis; after physical confirmation (the rover moves camera-forward on a positive command), both frame-level compensations were applied and validated: the mission backend rotates the firmware heading by π at its single odometry ingress (map, return-to-base and world bearings all reason in the camera frame), and MINS’s wheel extrinsic seed was set to Rz(π), reconciling the wheel model with the inertial/camera frame. Post-fix validation: a forward pulse reads +0.200 m along the camera heading (expected +0.18), and MINS accepts 100% of wheel measurements (χ2 gate, residuals ∼5×10−6): ending a period during which the flipped frame caused silent wheel rejection and the loss of planar anchoring.

- Static intrinsic calibration disabled. Online refinement of the wheel intrinsics was observed to diverge (negative radii) under motion bursts and is therefore frozen; re-measurement is the correct channel for the new wheels, not online estimation.

## Mission software evolution

### Autonomous state machine: reactive obstacle avoidance

The obstacle response was upgraded from an unconditional 180​∘ U-turn to a contournement (bypass) behaviour that preserves mission progress (Figure 12.3). On a frontal depth detection (<0.45 m over the central ROI, 5th percentile of valid depths), the FSM: (i) maps the obstacle as a persistent world-frame marker; (ii) pivots toward the freer side, chosen by comparing the median depth of the left and right halves of the depth band; (iii) sidesteps forward; (iv) counter-pivots to restore the original heading; and resumes the interrupted state. Vision keeps absolute priority (a detected beacon pre-empts the manoeuvre); after three failed attempts the historical U-turn is used as fallback. Two anti-pollution radii deduplicate map markers (beacons 1.2 m, obstacles 0.8 m): re-detecting an already-mapped beacon still triggers the odometric reset ritual but neither increments the counter nor stacks a marker.

Mission FSM after the campaign. AVOID bounces are aimed at the deepest open depth sector when one is visible (open-path attraction, July 8) and drawn at random otherwise; vision (beacon) pre-empts everything; U-turn survives only as a fallback.

##### Bypass controller details.

The pivot phases regulate on the fused yaw with wrap-safe accumulation, Δψ+=|atan2(sin(ψk−ψk−1),cos(ψk−ψk−1))|, until the target angle (60​∘) is reached at 0.6 rad/s; the side selection compares robust median depths of the left and right halves of the central depth band (medians, not means, because invalid returns and specular dropouts are heavy-tailed); the sidestep advances for 3 s at 0.25 m/s (≈0.75 m) with the frontal check re-armed, so a second obstacle mid-bypass re-enters the pivot phase with a fresh side choice and increments the attempt counter. The deduplication radii derive from the mission geometry: beacons are physically separated by several metres, so 1.2 m unambiguously identifies “the same beacon” while tolerating the world-frame scatter of repeated sightings (σ≈ a few cm, Table 12.3, plus bearing-estimation error at range); obstacles use 0.8 m, matching the rover’s own footprint scale.

##### Simplification (July 8 operator revision).

Field experience led to a deliberate simplification of the patrol geometry: the scan-then-advance alternation was replaced by continuous straight-line driving, the three-phase bypass by a single 90​∘ pivot toward the freer side (the heading change at each obstacle produces a natural sweeping coverage of the room, Roomba-style), the post-beacon hold shortened from 30 s to 5 s and the post-beacon rotation from 180​∘ to 90​∘. The anti-lock-in fallback (180​∘ after three failed pivots), the vision-priority pre-emption, the marker deduplication radii and the world-frame-preserving local reset (resetcoordslocal: the world origin is re-anchored so the global map and trajectory remain continuous) are all retained unchanged.

##### Coverage strategy (July 8, vacuum-robot revision).

The fixed 90​∘ pivot was generalised into a robot-vacuum-style ergodic coverage strategy, whose goal is to maximise beacon encounters while guaranteeing non-contact: (i) randomised bounce: each obstacle pivot draws a uniform angle in [45∘,135∘] toward the freer side, the stochastic component that lets memoryless cleaners cover a room; (ii) periodic exploration spiral: after 45 s without any beacon sighting, an outward spiral (ω=ω0/(1+kt), 30 s max) sweeps the local area, its direction biased toward the last glimpsed beacon bearing when that memory is fresh (<2 min); (iii) multi-level anti-stuck: four pivots within 20 s, or less than 0.15 m of world-frame displacement over 15 s, trigger an escape (a slow 0.2 m reversal, the only authorised backward motion, explicitly bounded because the depth sensor does not cover the rear, followed by a random 120–180​∘ pivot); a third escape within a minute latches a safe stop with an operator alert. The safety chain (0.45 m depth threshold, absolute vision priority, camalive gating) is unchanged.

##### Open-path attraction (July 8).

The final coverage ingredient makes the rover seek corridors. The central depth band is divided into nine angular sectors (≈10​∘ each across the 87​∘ horizontal FOV of the depth stream), each summarised by its robust median: invalid and out-of-range pixels are excluded, on the principle that unknown is not free. A sector qualifies as an open path when its median free depth exceeds 2.5 m and is at least 1.5× deeper than the axial sector; the double threshold acts as hysteresis and suppresses hunting in near-uniform scenes (an unknown axial sector is deliberately treated as fully open, so no steering ever happens on doubtful data). The analysis feeds two consumers. During straight-line PATROL a proportional heading bias (kp=0.8 rad s​−1 per radian of bearing error, saturated at 0.25 rad/s) gently bends the trajectory toward the open sector, so the rover follows corridors and open doorways instead of blindly crossing them. And at each obstacle stop, if an open path lies more than 15​∘ off-axis, the bounce is aimed at its bearing (clamped to the usual [22.5∘,135∘] pivot range) instead of randomly drawn; the random bounce remains the fallback whenever no qualifying sector exists, preserving the ergodic component. Beacon pre-emption and the 0.45 m obstacle threshold keep absolute priority throughout: the attraction only modulates the otherwise straight advance.

##### LOCK anti-stall guard (July 10 field regression).

The first field run of the coverage stack exposed a failure mode the LOCK design could not see: the rover rotated in place indefinitely at a constant ω≈0.13 rad/s. The signature identified the cause: the commanded rate equalled 0.25×|e|/(w/2) for a constant pixel error, meaning the pursued target did not move in the image while the robot rotated. A world-fixed beacon sweeps across the image under rotation; only a camera-fixed artefact (here a ceiling-light reflection accepted by the LED detector, at y=64 px, near the image top) stays put. The existing LOCKTIMEOUT only arms when the target disappears, so a permanently-visible artefact defeated it. The guard added is a convergence watchdog: 12 s in LOCK without achieving centring aborts the lock, returns to straight-line PATROL and suppresses re-locking for 25 s, so the rover advances out of the artefact’s influence; the centred approach phase refreshes the progress timer, so a legitimate long approach (3 m at 0.2 m/s exceeds 12 s) is never aborted. The lesson mirrors the supervisor audit: a timeout keyed to absence of evidence cannot catch a persistent false evidence source; guards must bound time-without-progress, not time-without-signal.

##### Obstacle ROI geometry and depth-fill audit (July 10).

A field report of missed obstacles prompted a geometric audit of the frontal depth window. The original ROI spanned 40.5–59.5 % of the image width (±8∘, i.e. a corridor of ±6 cm at the 0.45 m stopping distance), while the rover’s half-width is ≈25 cm: an obstacle aligned with a wheel rather than the camera axis was simply outside the watched zone. The ROI now spans 20–80 % of the width (±29∘≈±26 cm at 0.45 m, matching the swept corridor), with the vertical extent capped so that ground pixels only enter the band beyond ∼0.73 m, past the trigger threshold, hence no floor false-positives. Because a wide window dilutes thin obstacles (a chair leg) in a global percentile, the corridor is evaluated as three independent thirds, each compared to the threshold via its own 5th percentile. A complementary measurement settled the projector trade-off: the depth valid-pixel ratio in the corridor is unchanged between laser power 60 and 150 (56 % both, scene-limited by dark and oblique surfaces), so the power reduction that protects AprilTag decoding costs the obstacle channel nothing. Finally, the cockpit detection sliders were discovered to be dead controls (parameters were snapshotted once at vision-process start), and are now forwarded live with each frame; the brightness threshold slider drives the actual detector (the hue pair is a vestige of the retired colour pipeline, inert on infrared imagery).

##### Four-encoder wheel feed (July 10).

The same session answered an instrumentation question: MINS’s differential model (Wheel2DAng) consumes two angular velocities, and the remap node fed it only the front wheel pair: two of the rover’s four encoders were unused. On a skid-steer platform every turn slips by design, and the front and rear wheels of a side grip differently, so a single wheel per side makes the odometry constraint hostage to whichever wheel slips more. The remap now publishes the per-side mean of both encoders (ωL=(ωFL+ωRL)/2, idem right), a better estimator of the side’s effective tread speed that also halves encoder noise; it degrades gracefully to the front encoder when a rear joint is absent. The residual precision limit remained the wheel intrinsics: the frozen values (r=62.5 mm, b=358 mm) described the original wheels, not the rigid replacements (online re-estimation stays disabled after the July 6 divergence). This motivated the automated campaign of the next subsection.

### Precision campaign (July 10): automated calibration, slip characterisation, SLAM anchoring

##### Automated wheel-radius calibration.

A self-driving calibration tool (tools/calibauto2.py) executes the measurement manoeuvres itself through the cockpit command channel (inheriting the backend’s polarity handling, speed clamps and dead-man), guarded by the live depth corridor: a straight leg is attempted only with ≥3 m of free space and aborts below 0.8 m. Over a 2.2 m guarded straight, 20 half-second windows matching the straightness gates (|ωgyro|<0.06 rad/s) yield

reff=ΔpMINSωwheel Δt=62.2 mm (IQR ±0.5 mm),

−0.5 % from the stock value: the rigid wheels kept the tread radius, closing the radius half of open item #1 (the track width still awaits a physical measurement).

##### The decisive discovery: the “rigid wheels” are mecanum wheels.

The base-width leg failed in a deeply instructive way. Commanding gentle arcs produced a sustained wheel-speed differential of Δω≈3.8 rad/s (which the differential model maps to rΔω/b≈0.66 rad/s of yaw), while two independent witnesses agreed the chassis did not rotate: the gyroscope stayed at its bias (0.039 rad/s, identical to the static value) and the MINS visual heading moved +0.3∘ in 6 s (0.001 rad/s). The wheels were subsequently identified as the FictionLab Mecanum wheel set: Leo Addon (⌀128.8 mm, 45​∘ polyurethane rollers), which recasts the observation from friction to kinematics: the free rollers absorb a left/right speed differential instead of converting it into a yaw moment: they are doing exactly what mecanum rollers are designed to do. Under proper mecanum forward kinematics a tank-style differential still yields yaw, but scaled by 2(lx+ly) (roughly the track plus the wheelbase, nearly twice the differential-model base), and only insofar as the rollers grip; on this floor the residual authority is negligible. Three consequences: (i) the firmware’s differential-drive abstraction is structurally mismatched to the wheel hardware, so wheel-implied yaw is not merely noisy but fabricated; (ii) no “effective base” can repair it (the authority is regime- and surface-dependent); (iii) the trust re-assignment is the correct estimator-side response: noisew 0.2→0.8 (wheel angular information strongly discounted) while noisev is unchanged: the longitudinal rolling constraint remains excellent, as the radius calibration proves, with reff=62.2 mm against the 64.4 mm geometric radius (3.4 % roller-contact compression, a mecanum-typical figure). An earlier attempt with in-place rotation at 0.5 rad/s had pushed the then-configuration into outright filter divergence (position exploding to 105 m): sustained fabricated wheel yaw is precisely the worst input for a tightly-coupled update.

##### SLAM anchoring.

The estimator ran as a pure MSCKF (maxslam: 0): every feature was marginalised at window exit, leaving no persistent scene memory: the weakest possible configuration for in-place rotation, where triangulation baselines vanish. Twenty-five SLAM landmarks are now maintained in the state (maxslam: 25, confirmed live: SLAM:25 in the updater trace), anchoring the pose against revisited structure at a bounded state-size cost.

##### Verification.

Post-restart, with r calibrated, wheel-yaw discounted and SLAM active: wheel updates are accepted at 100 % χ2 over 408 measurements; a 60 s static T1.1-mini gives σx=7.4 mm, σy=1.0 mm (z: 29 mm), with fitted slopes of −0.15, 0.03 and 0.44 mm/s, all under the 1 mm/s noise verdict of Protocol T1.1; and the exact stress regime that had destroyed the filter (10 s of sustained slipping differential) now produces a maximum position wander of 9 mm with a sane filter, against a 1.7×105 m divergence before. Operationally, the LED brightness threshold was raised to 240 through the now-live cockpit channel after four ceiling-light “LED resets” were found in the mission log. Two hardware-level options followed from the mecanum identification: either restore the stock wheels when the mission profile depends on authoritative in-place pivots (the FSM’s obstacle bounces and post-beacon turns), or keep the mecanum set and expose per-wheel control below the differential-drive firmware abstraction: unlocking omnidirectional motion (lateral strafing) that the current command chain cannot express, at the cost of a firmware-level rework and a mecanum-aware odometry model. The operator’s decision was to keep the mecanum wheels, which turns the roller behaviour from an anomaly into an operating constraint the autonomy layer must respect.

##### July 13: the ramp campaign, two misdiagnoses, and the ground truth.

With the battery finally charged (11.95 V after three days on the charger), a ramp characterisation tool (tools/rampyaw.py) measured actual yaw rate against commanded pivots from 0.10 to 0.40 rad/s, and dismantled the July 10 “roller break-away” theory: at healthy voltage, every command from 0.15 to 0.40 rad/s executes at 92–99 % authority. The zero-rotation observations of July 10 were motor torque sag at 10.3 V, not roller physics; only a small command deadband (≲0.12 rad/s) is intrinsic. Rotation speeds were restored accordingly (TURNSPEED/UTURNSPEED 0.35, 90​∘ in ≈4.6 s, anti-stuck windows rescaled to match).

The ramp, however, first reported those rotations with an inverted sign, which led to a wrong angular-polarity patch and a wrong left/right wheel-remap swap before a ground-truth experiment (one commanded rotation witnessed simultaneously by the raw gyroscope and the MINS heading) exposed the real culprit: the estimator, not the actuation. With 25 SLAM landmarks active, an in-place pivot renders landmark depth unobservable (zero baseline); the stale anchors dragged the MINS heading to +0.58 rad/s while the gyroscope measured −0.25 rad/s, and the corrupted state made the filter reject correct wheel measurements (χ2 acceptance 6 %). Both patches were reverted: kinematically, a 180​∘ drive remount inverts the linear sense (one inversion) but not the yaw sense (side swap × rolling-sense flip cancel), so DRIVEPOLARITY alone was always the complete fix, and maxslam returned to 0 pending an excitation-gated re-enable. Final validation, all witnesses agreeing: command +0.30→ gyro +0.249→ MINS +0.249, wheel χ2 acceptance 100 % during rotation. Two lessons join the registry: an estimated heading is not a truth reference for diagnosing actuation sign: only a raw inertial channel is; and SLAM anchoring must be excitation-aware on a platform whose mission profile is rich in pure rotation.

##### First full-stack AUTO run and its three regressions (July 13).

The first field run with the repaired chain surfaced three defects, each diagnosed from the mission log. (a) The one-minute failsafe. The robot dropped to MANUAL at second :03 of every minute: the cockpit heartbeat is a 200 ms setInterval, and browsers throttle timers in hidden tabs down to once per minute; watching the Trajectory page while the Operations tab (the heartbeat sender) sat hidden starved the 1.5 s dead-man like clockwork. Two-sided fix: the front-end now also emits a heartbeat on every telemetry reception (WebSocket delivery events are exempt from tab throttling), and the backend timeout was raised to 12 s, still a real dead-man, but tolerant of refreshes and throttling bursts. (b) Blind LOCK approaches. Obstacle handling was gated to the PATROL branch with target-visibility as an explicit exemption (“what is ahead is the target, not an obstacle”), so a false lock drove at furniture with the depth channel watching helplessly. The approach now aborts on the obstruction contradiction (a target believed farther than 0.6 m cannot coexist with a <0.55 m depth return ahead), maps the obstacle, and enters the bounce with the artefact cool-down armed. (c) Ceiling lights as beacons, structurally. On infrared imagery the brightness-based LED detector favours exactly the wrong objects: incandescent ceiling fixtures are IR-bright while the beacon’s visible-light LEDs may be IR-dim. All false detections of the session sat in the top third of the image (y=38–160 px of 480). A floor-beacon plausibility gate now rejects any blob above 35 % of the image height, the persistent threshold moved to 240, the post-stall re-lock cool-down doubled to 45 s, and every lock entry logs its source and image height for one-line diagnosis. Whether the real beacon’s LEDs clear the 240 threshold on IR at all remains a field question: the AprilTag and checkerboard channels carry the acquisition in the meantime, and a low-rate colour stream (hue discrimination, 424×240) is the designed escalation if they do not.

##### All-in-one beacon ritual and a fluid trajectory display (July 13, operator revision).

Two operator requests closed the session. First, the beacon chain was unified: in AUTO the entire find-approach-reset sequence now runs off a single measured chain: acquisition (AprilTag at range, vision as bonus), approach to 0.6 m, immediate local reset, marker, 5 s hold, 90​∘ turn, with one hardening change: the reset now requires a measured distance (tag or checkerboard) at or below the stop threshold. The previous rule reset instantly when the distance was unknown (meant for a full-field checkerboard, but equally satisfied by any centred artefact); an unknown distance now simply holds the centring until the stall guard rules. The beacon’s world-frame marker no longer depends on the checkerboard either: the lock distance places it along the centred heading. Second, the Trajectory page was made fluid and immortal: the reload-on-disconnect handler (a hidden location.reload() four seconds after any WebSocket close, the operator’s “the site refreshes by itself”, and it destroyed the trail each time) was replaced by in-place reconnection with exponential backoff, preserving all client-side state; the displayed pose and the auto-scaled viewport are eased toward their targets every animation frame instead of jumping at the 8–10 Hz message rate; and the trail rendering was batched from one stroke per segment (3 000 canvas paths per frame) to age-sliced batches (≤12 paths), removing the render jank the operator reported.

##### Divergence guard (July 13, evening).

The now-truthful display promptly reported a real fault: MINS had silently diverged hours earlier (position hundreds of kilometres out, 1 123 m/s of “velocity” on a stationary robot, the signature of runaway inertial double-integration, plausibly seeded by collision shocks during the earlier field run). A fresh initialisation restored millimetre-level statics within seconds, but the structural lesson is that a diverged filter stays silent: nothing in the stack complained while the cockpit displayed nonsense. The cron watchdog now probes /mins/imu/odom each minute and triggers an automatic clean stack restart whenever the position magnitude exceeds 100 m (impossible indoors, unambiguous at divergence scale), bounding any future episode to about a minute. A chronic instrument anomaly was also quantified in passing and joins the open-items registry: the LeoCore accelerometer reports a=8.76 m/s​2 at rest against a local gravity of 9.790: a −10.5 % deficit the estimator absorbs as an initial bias, acceptable in operation but worth a firmware-side investigation.

### Systems audit: measure first, then touch (July 13)

A full-stack performance and reliability audit closed the day, conducted on the live system under the rule that no change ships without a measured justification. The profile identified the actual constraint: the ground-station CPU was operating at its thermal ceiling (81​∘C against an 82​∘C throttle point, load average 4.1), the root cause of the day’s sluggish XML-RPC tooling, with the budget dominated by MINS (64 %, legitimate), but also by two unjustified consumers: the standby OpenVINS estimator (13–15 %, never initialised, never selected, yet running at the same elevated nice -10 priority as the critical path) and the AprilTag detector (16 %, full 15 Hz on full resolution feeding a backend freshness window of 0.7 s). Three interventions, each benchmarked: OpenVINS was demoted to normal priority (it now yields under contention, costing nothing when headroom exists); the tag detector’s image feed is throttled to 8 Hz (≈halving its cost with no mission effect); and the depth callback (the obstacle-safety hot loop at 15 Hz) was profiled at 4.04 ms/frame and rewritten to operate on a 2×-decimated grid (1.68 ms, −58 %), with median deviations under 0.5 % and worst-case 5th-percentile deviation of 7 % on synthetic uncorrelated data (real depth is spatially correlated and deviates less; the 600 mm threshold retains margin). Reliability hardening followed the same triage discipline: of the 46 except Exception handlers in the backend, the one whose silent failure would disarm the obstacle chain (the depth callback) now validates its input (dimensions and buffer length before reshape) and reports errors at a rate-limited once-per-10 s, while the remaining handlers (display paths, isolated vision subprocess) were deliberately left untouched: blanket rewriting of working error isolation in a fielded system is regression risk masquerading as rigour. For the same reason the audit declines cosmetic renaming of node/topic interfaces. Post-change: package temperature 60​∘C (22 degrees of reclaimed thermal margin, part of it restart-transient), tag detections at 6–8 Hz, depth chain error-free. The architectural review found the concurrency design already sound: the vision pipeline runs in a separate process (GIL-immune by construction), numpy releases the interpreter lock in its C kernels, and cross-thread state hand-off relies on atomic reference swaps.

### Field validation run (July 13, 15:49–15:54)

A 5 min 25 autonomous run in a heavily cluttered room, monitored live from the mission log, validated the entire coverage stack. Passed: obstacle handling (operator verdict: “super”): more than a dozen clean bounces with correct freer-side selection, three aimed bounces steering into detected open corridors, and three escape sequences whose rescaled windows correctly avoided the safe-stop latch; permanent open-path attraction (corridors 2.5–7.1 m); mecanum-regime pivots at their predicted rate (a 39​∘ bounce completed in 2.3 s); zero ceiling-light false locks over the full run (the floor-plausibility gate and 240 threshold held); zero heartbeat losses despite operator tab switches (the reception-driven heartbeat held); and an estimator that stayed sane throughout under the new divergence guard. Failed: the beacon was never acquired. The AprilTag channel decoded nothing in five minutes, consistent with its marginal pixel budget (2.2 px per module at 3 m on 640×480; an A3-format tag of ∼35 cm side would carry acquisition to 5–8 m and is the cheapest possible fix). Two vision locks on a floor-level target (y=74 and 82 % of image height, the plausible-beacon band) failed to converge within the 12 s stall guard: either a persistent floor reflection (guard correct) or the real beacon with intermittent LED detection (guard too strict); the discriminating pixel trace was lost when the rover dropped off the network at the end of the run. The next session opened with exactly that experiment, and closed the case.

### Seeing through the rover’s eyes: the beacon case closed (July 13, evening)

With the rover parked facing the beacon, the annotated camera view was captured directly (the MJPEG server was first moved under launch supervision with automatic respawn, verified by killing it, after three competing launchers had left it silently dead). The images settled every open question. First, the infrared projector was lacing the checkerboard, the LEDs and the floor with bright dots, and measurement showed it was degrading the depth channel too: corridor fill improved from 56 % to 81 % with the emitter off, passive stereo on the textured carpet outperforming structured light. The projector is now permanently off (watchdog updated), simultaneously cleaning every visual pattern and improving obstacle sensing. Second, replaying the exact LED detector offline on the captured frame quantified the acquisition failure: at the 240 brightness threshold (raised earlier to reject ceiling lights) zero of the four LEDs pass; at 220, three; at 200, all four: a perfect cross at (357,290), circularities 0.83–0.94. The non-converging locks of the field run had been the real beacon, intermittently detected at a threshold that starved it. The threshold returned to 200: ceiling-light rejection belongs to the floor-plausibility gate, not to brightness. Live confirmation followed immediately: repeated 4/4-LED confirmations facing the beacon. Third, the front face carries no AprilTag (checkerboard only): the large-format tag remains the recommended range extension. Finally, the all-in-one ritual’s last gap was closed: with no measured range (no tag on this face, the checkerboard useless beyond 1.5 m), the approach is now guided by the depth corridor itself (the beacon is a physical object the depth channel sees), with arrival declared below 0.7 m, deliberately above the 0.55 m obstacle threshold so arrival always precedes any conflict. The methodological lesson of the day is worth stating plainly: three sessions of hypothesis-driven tuning were resolved in twenty minutes by looking at the actual image and replaying the actual detector on it. Direct observation beats inference.

##### Idempotent web healing.

One last recurring operator pain (the cockpit banner “WS DISCONNECTED” returning every minute) traced to a healing anti-pattern: the web-stack start script rebuilt everything from scratch on every invocation, and the watchdog invokes it whenever any single port is missing. A TLS rosbridge failing to start (transient “no route to host” while the rover’s WiFi flapped) therefore caused the healthy LAN rosbridge (and the operator’s live session with it) to be killed and relaunched every sixty seconds. The script is now idempotent: each component is checked and only (re)started if actually dead, and a component whose process exists but whose port is not yet bound is recognised as starting and left alone (rosbridge can take over a minute to bind under load). The rule joins the supervisor registry: a healer must never touch what is already healthy, and “still starting” is a state, not a failure.

##### Plausibility-gated pose serving (July 13, night).

The evening’s last structural fix addressed the operator’s report that the MINS trace “draws nonsense” whenever the camera stream drops. The mechanism is now fully mapped: during an image gap the estimator dead-reckons on the IMU alone, the residual accelerometer bias double-integrates into metres of excursion within seconds, and (the cruel twist) the χ2 gate then rejects the wheel measurements (v=0) precisely when they could anchor the filter, because the diverging state makes their residuals huge. Three complementary defences now stand. The wheel velocity trust was tightened (noisev 0.5→0.2) so the zero-velocity anchor acts before runaway. A plausibility gate in the pose selector freezes the served pose whenever the active source implies a speed beyond 1.5 m/s (the rover’s command ceiling is 0.4), and re-anchors through the existing SE(3) correction once the source calms: excursions can no longer reach the trail, the world map or Return-to-Base, whatever their cause. And the watchdog gained an IMU starvation probe: the rosserial bridge on the Pi was found degrading after the day’s power events (producing at 90 Hz, delivering 3–8 Hz to the PC while camera topics flowed normally, the comparison that acquitted the radio); a supervised node cycle restores the nominal rate (measured 3→79 Hz) and is now automatic. The day’s root-cause epilogue deserves its line: a loose WiFi antenna connector on the rover explained the entire day of network pathology: 2–422 ms round-trip jitter that healed to a steady link once reseated. The most expensive failures are often a hand’s turn of hardware.

### Mission loop closed: the beacon sequence, end to end

The day ended with the run the whole campaign had been building toward. At 18:55:48 the rover, in AUTO, locked the beacon on its vision channel (four LEDs at the 200 threshold, target at 58 % image height); it centred, approached under depth guidance (no checkerboard, no tag, the corridor percentile serving as rangefinder), and at 18:55:59 declared “Balise atteinte (depth 0.67 m) — reset local — WAIT 5 s”: eleven seconds from detection to a world-preserving local reset. The full mission loop (patrol, open-path attraction, obstacle bounces, beacon acquisition, approach, reset ritual) is field-validated. Three incidents were resolved en route and are recorded for operations: a post-brown-out motor-driver lockout (commands received, zero motor current, battery steady under load, software reset ineffective; only a full power cycle clears the latched fault; the discriminating measurement is battery voltage during a command pulse); a false escape at AUTO entry (the anti-stuck displacement history survived from the parked MANUAL period and is now purged in startpatrol); and a depth-rate collapse to 1–3 Hz traced to the Pi’s PNG compressor running at factory level 9: level 1 restored 8.9 Hz at a still-negligible bandwidth cost, and both this and the projector-off setting are reapplied by the watchdog after every camera recovery (their boot-side pinning in robot.launch remains an open item, together with the large-format AprilTag).

### Long-range beacon acquisition by AprilTag

The checkerboard detector is physically range-limited: its inner corners become sub-pixel below ∼1.5 m at 640×480. The physical beacon carries a 36h11 AprilTag (ID 285), whose measured geometry (1-inch modules, 8″ black square) gives a side of s=0.2032 m, decodable to 3–4 m. The mission backend now consumes /tagdetections: the nearest declared tag is projected to pixel coordinates through the calibrated pinhole model,

u=fx XZ+cx,  v=fy YZ+cy,

and feeds the same targeting path as the vision pipeline, so PATROL acquires the beacon at several metres and the checkerboard+LED chain confirms at close range for the reset ritual. The range asymmetry is quantifiable: a 36h11 tag is decodable down to roughly 2 px per module, so with fx=336 px and 25.4 mm modules the theoretical ceiling is Zmax≈fx smodule/2 px≈4.3 m, versus sub-1.5 m for the checkerboard whose corner detector needs an order of magnitude more pixels per square. Two supporting changes: the IR projector power was reduced (360→60) because its dot pattern (rigidly attached to the camera frame and therefore stationary in the image regardless of scene motion) overlays both patterns with high-contrast clutter whose apparent size at range matches the tag modules, destroying decoding precisely where range matters (near-field depth for the 0.45 m obstacle threshold survives the power reduction); and scan speed was halved (0.4→0.2 rad/s) because at angular rate ω and exposure Te the rotational blur is fx ω Te pixels: at 0.4 rad/s and a typical indoor IR exposure of 7–10 ms this exceeded a full pixel, marginal for corner detection; halving the rate restores sub-pixel sharpness while a 360​∘ scan still completes in 31 s.

## Operator platform and public API

### Site structure and design system

The web cockpit grew from a single operations page to a five-page platform (index, ops, logbook, navigationmodes, trajectory), unified by a token-based design system (mission.css: deep-space palette #0B0E14, data-cyan/NASA-orange utility accents mapped one-to-one to the VINS/MINS semantics, monospace tabular numerals, pulsing liveness indicators, severity-coded flight-log rendering of the console). The navigationmodes page now embeds a bilingual (FR/EN) deep-dive (MSCKF foundations, both estimators, the measured T1.1 tables) and a live SVG data-flow diagram mirroring Figure 12.1. The trajectory page is a Mission-Control style Canvas 2D module: mode-coloured fading trail (each segment frozen with the colour of the source active at that instant), AprilTag node markers, velocity/cadence/latency HUDs, and VINS/MINS switching buttons calling the selector service directly.

### API surface

All remote access flows through a Cloudflare tunnel to the ground station; the ingress map is the public API:

@L4.2cmL3.4cmL6.6cm@ Endpoint & Backend & Function
wss://.../rosbridge & rosbridge TLS :9443 & full ROS API in the browser (roslibjs): topics, services
https://.../stream & web_video_server :8080 & MJPEG annotated video
https://.../* & http.server :8000 & static pages
ws://<pc>:9090 & rosbridge LAN & same API without the tunnel

/mission/command & stdmsgs/String & JSON operator contract (setmode, manual, stop, reset, gotobeacon, setposesource, heartbeat 5 Hz)
/mission/telemetry & stdmsgs/String & ∼10 Hz full-state JSON (pose local+world, FSM, sensors, markers)
/poseselector/setsource & SetBool service & VINS/MINS switch (Eq. [eq:se3])
/robotposefused & navmsgs/Odometry & fused output consumed by all pages

##### Anatomy of a switch.

The complete VINS→MINS sequence illustrates the layering: the operator’s click publishes {"action":"setposesource","source":"MINS"} on /mission/command through the tunnel WebSocket; the mission backend validates state and calls /poseselector/setsource(true); the selector verifies the target source has published recently (refusing otherwise with an explicit message, the guard that surfaces an uninitialised OpenVINS), computes Tcorr of Eq. ([eq:se3]) from the last published pose and the new source’s current pose, swaps sources, and republishes; the UI badge updates from the /leonavigation/posesource topic (server truth, no optimistic state), and the Trajectory page’s trail changes colour at the exact segment where the switch occurred: a live visualisation of Eq. ([eq:se3])’s continuity.

Two protocol-level defects were diagnosed and fixed during the campaign: the WebSocket Host-header port check (autobahn) requires websocketexternalport to match the public port (443 behind the tunnel, 9090 on LAN, a stale ROS parameter had silently broken LAN mode for two days); and an FSM state unknown to a cached front-end must render as its raw name, never fall back to a misleading default.

### Operations infrastructure

Availability is enforced by three layers: per-node respawn supervision (roslaunch); a cron watchdog (1-minute period) checking web ports, tunnel, navigation stack and camera (including a full rosmon STOP/START camera cycle, republish-zombie purge, and reapplication of the projector power), gated by a maintenance flag for deliberate interventions; and automatic infinite WiFi reconnection. The camera’s minimal profile is written into the robot’s boot launch (with the hardware constraint documented in-file: depth and IR share the stereo module and must run identical resolution/rate). Battery under ∼10.5 V was identified as a hard limit: the Pi’s USB rail sags and the D455 cannot hold its streams: no software remedy exists, and the report flags it as an operational precondition.

Two watchdog defects surfaced during a two-day robot-off period (July 8–10) and were fixed. First, timeout N rostopic echo is not a bounded probe when the ROS master is unreachable: timeout sends a single SIGTERM which rostopic, blocked inside its connection attempt, ignores, and timeout then waits forever. Every cron tick that reached a probe leaked one hung process (fourteen accumulated in two days); the fix is timeout -k 5 N, which escalates to SIGKILL. Second, an ordering fault: the watchdog exited at its “robot unreachable” gate before checking the web stack, so with the rover powered off the public site (cockpit, report) stayed down for the whole period; yet it depends only on the PC. The web-healing block now runs before the reachability gate; only the robot-dependent actions (navigation stack, camera ladder) remain behind it. The general lesson generalises the earlier cron findings: a supervisor must be audited under the failure of each of its own probes, not only under the failures it is designed to heal.

The same restart exposed a related gap: the AprilTag detector (launched manually during the July 8 field session) did not belong to any supervised launch file, so every stack restart silently removed the long-range beacon acquisition chain (/tagdetections advertised, zero publishers). The node is now included in the supervision launch and respawns with the rest of the stack; the rule it confirms is that anything started by hand during a field session is already lost: a capability only exists once its launch file is owned by a supervisor.

The same principle applies to run-time configuration, not only to nodes: laserpower (stereo module, IR pattern projector) and pnglevel (the depth stream’s compressedDepth transport, § 12.2.2) had both been tuned via live dynamicreconfigure calls, which do not persist across a leo.service restart: every restart silently reverted both to their driver defaults. The fix follows the same pattern used throughout this campaign for the 10 Hz camera-profile incident (Table [tab:openitems], item 4): configuration changes are validated live first, then only made structural once proven safe, and structural means owned by the boot launch file rather than a one-off terminal command. Two one-shot dynamicreconfigure client nodes were added to robot.launch, delayed eight seconds (launch-prefix="bash -c ’sleep 8; $0 $@’") to let the realsense driver’s reconfigure servers come up first, marked non-required and without respawn so an insufficient delay on a given boot degrades only that one setting rather than the stack (the same failure-containment principle as the MINS respawn policy of §12.2.4). Applied and confirmed via the two nodes’ process exit status in the rosmon log (exited with status 0 for both, the same signal a failed service call or an unresolved reconfigure-server name would have reported non-zero), followed by a full navigation-stack restart on the ground station: the same topology-transition precaution of §12.6.4 applies equally to a robot-side service restart.

### Campus-scale connectivity: from an isolated hotspot to a WireGuard-tunnelled network

Through July, all field operation had been confined to rooms within direct range of the rover’s own access point (wlanext, LeoRover-e138): a single indoor 5 GHz AP whose usable range is on the order of 10–15 m through drywall, sufficient for the lab but not for a building-wide demonstration. On 2026-07-20, ahead of an external (NASA-context) validation requiring the rover to be exercised anywhere in the building without a network-driven interruption, the operating topology was migrated onto the campus network FLTech-Guest.

##### Retained dual-radio architecture.

The rover’s onboard Wi-Fi adapter supports two simultaneous logical interfaces: wlanext continues to host the local hotspot (kept live as a proximity fallback, disabling it would have removed the only recovery path reachable without campus infrastructure), while wlanint joins FLTech-Guest in client mode on 5 GHz, giving the robot a routable, building-wide address (10.154.27.235/19) in addition to its local one. The ground-station PC was not moved onto FLTech-Guest via its own Wi-Fi: a live dmesg capture during the first connectivity tests showed a reproducible driver fault in the PC’s USB adapter (rtl88x2bu chipset: UBSAN: invalid-load inside rtl8822bsetchannelbw, phydmccx.c:696), firing on every Wi-Fi channel switch and producing on the order of six spontaneous disconnects per hour. Rather than debug a closed-source vendor driver on the critical path of a validation demo, the PC was kept on its wired Ethernet uplink (structurally immune to the fault) and made to reach the robot over a point-to-point tunnel instead. Figure 12.4 contrasts the two topologies in full.

Network topology before and after the 2026-07-20 migration. Left: the original architecture, where the PC joined the robot’s own access point directly: functional only within Wi-Fi range of the robot itself. Right: the PC stays on wired Ethernet and reaches the robot’s campus-Wi-Fi address through a WireGuard tunnel (red dashed arc); the robot’s own hotspot (wlanext) is kept live as a fallback but is not part of the active data path. ICMP and TCP 22/11311 cross the campus firewall directly, but TCPROS’s per-topic ephemeral ports do not: only the WireGuard UDP port is pre-authorised, so every ROS connection is multiplexed inside that single tunnel (§12.6.4 below expands on why a plain route cannot substitute for this).

##### Why a tunnel, not a plain route.

A campus network partitions the PC’s wired subnet and the robot’s guest-Wi-Fi subnet behind a stateful firewall: ICMP and a short list of registered TCP ports (22, 11311) were observed to traverse cleanly, but ROS’s transport layer does not confine itself to a fixed port. TCPROS negotiates one ephemeral TCP port per topic connection through the XML-RPC handshake between publisher and subscriber: exactly the traffic pattern a campus firewall is configured to block between unrelated VLANs. Routing directly to the rover’s campus address therefore worked for master-registry queries (rosnode list, service calls) while silently starving every actual topic subscription; the fix had to present the firewall with a single, fixed, pre-authorised port and multiplex all ROS traffic inside it, which is precisely what a VPN tunnel does and a raw route across subnets cannot. The italics matter: §12.8 revisits this constraint a day later and shows it is specifically a cross-subnet limitation, not a blanket property of raw routing.

##### First attempt: sshuttle, and why it was abandoned.

sshuttle (a userspace transparent proxy tunnelling arbitrary TCP over an SSH control connection) was tried first for its zero robot-side privilege footprint: an authorized public key was sufficient. One deployment bug surfaced immediately: forwarding the robot’s full subnet caused sshuttle’s own SSH control channel to be captured by its own relay, killing itself (Broken pipe) on every launch; excluding only the control port explicitly (-x <robotip>/32:22, rather than the whole host) resolved it. With that fix, data was observed flowing correctly on two independent occasions. Under the sustained, multi-connection load of the full navigation stack (MINS, the mission backend, and the cockpit each holding independent long-lived TCPROS sockets simultaneously), however, robot-originated topics failed systematically: reproducible from both the operator’s own terminal and independently, not a transient fluke. Three hypotheses were eliminated in turn: file-descriptor exhaustion (LimitNOFILE=65536 added, no change), accumulated robot-side session state (full reboot of the robot, no change; MINS cold-init then took 196 s), and PC-side process staleness (clean service restart, no change). With load-independent, reboot-independent, and restart-independent failure ruled out as anything but the transport itself, the conclusion drawn was architectural: a single userspace Python process relaying TCP by repeatedly copying bytes between sockets does not scale to the connection churn of a full ROS stack the way kernel-level forwarding does, and no further time was spent tuning it.

##### Final transport: WireGuard.

WireGuard replaces sshuttle outright: a kernel module terminating one static UDP port (51820) per host, presenting the firewall with a single opaque, pre-authorised flow regardless of how many ROS connections it carries inside. The tunnel is point-to-point (10.200.0.1 PC ↔ 10.200.0.2 robot, /24), authenticated by a Curve25519 keypair generated on each host, with PersistentKeepalive=25 s to hold the mapping open against the campus NAT’s idle timeout. This value is not arbitrary: RFC 4787 (REQ-5) directs NAT devices to hold a UDP mapping open for at least two minutes of inactivity, but numerous field surveys of consumer- and enterprise-grade NAT/firewall implementations report equipment that reclaims idle UDP state considerably sooner than the RFC’s recommendation: commonly in the tens of seconds on the most aggressive devices. A keepalive interval τ is safe only if it is shorter than the shortest timeout Tmin the path can plausibly impose, with margin for jitter in when the keepalive packet itself is actually scheduled and delivered: τ≪Tmin. Twenty-five seconds sits comfortably under even the most aggressive commonly reported timeouts while remaining far above the per-packet overhead floor (a keepalive is a single encrypted UDP datagram; at 25 s intervals this is a negligible ∼0.04 pkt/s of control traffic, invisible against the ∼2 MB/s data budget of §12.2.2): the standard rationale behind WireGuard’s own documented default for exactly this value, adopted unchanged here rather than tuned against this specific campus firewall’s (unmeasured) timeout.

The encryption itself is the other quantity worth bounding rather than assuming negligible. WireGuard encapsulates each packet with a 4-byte type field, a 4-byte receiver index and an 8-byte counter (16 B header), plus a 16-byte Poly1305 authentication tag, for a fixed 32-byte overhead per datagram independent of payload size, on top of the underlying UDP/IP headers already paid by any transport. At the compressed image cadence of §12.2.2 (two IR streams at 15 Hz, ≈30–60 kB per JPEG frame), each frame is fragmented into k≈⌈40 000 B/1400 B⌉≈29 UDP datagrams at a conservative 1400-byte MTU-safe payload size, so the tunnel’s fixed per-packet cost adds

Δwg = k×32 B×15 Hz ≈ 29×32×15 B/s ≈ 13.9 kB/s

per stream, against a payload rate of 40 000 B×15 Hz=600 kB/s for that same representative frame size: a Δwg/payload≈2.3 % tax, well inside the margin already measured between the WireGuard row and the local-network baseline in Table 12.1. ChaCha20-Poly1305 encryption and authentication are computed in software on both endpoints, but at these payload sizes the dominant cost remains the fixed per-packet overhead just bounded, not the cipher throughput: modern software ChaCha20 implementations sustain well over 1 GB/s per core, three orders of magnitude above the ∼2 MB/s aggregate data budget of the whole tunnel. The improvement measured over sshuttle is therefore not explained by a cheaper cryptographic primitive (sshuttle tunnels over SSH, itself using comparably fast ciphers); it is explained by kernel-level packet forwarding replacing a userspace relay, exactly the architectural argument of the preceding paragraph, now with the encryption cost itself ruled out as a confound. The improvement over sshuttle was immediate and measured at every layer of the stack (Table 12.1).

Transport comparison, same rover, same campus network, same navigation stack. WireGuard fully recovers the local-network baseline performance; sshuttle does not, under real multi-connection ROS load.

| Metric | sshuttle (userspace) | WireGuard (kernel) | Ratio |
| --- | --- | --- | --- |
| Camera stream | ∼1 Hz (starved) | 15 Hz (nominal) | 15×​ |
| MINS cold-init time | up to 196 s | 13–17 s | ≥11×​ |
| Robot-originated topic delivery | unreliable under load | consistent | n/a |
| Firewall exposure | 1 TCP ctrl port + relayed flows | 1 static UDP port | simpler |
| Measured link loss (static ping) | n/a | 0 % | n/a |

##### Centralised address configuration.

Every script that previously hardcoded the robot’s address ( /bin/leo, leowatchdog.sh, restartstack.sh, launchmins.sh, startweb.sh) now sources a single file, tools/robotenv.sh, which reads an override (LEOROBOTHOST) from ~/.leonetwork and derives ROSMASTERURI and ROSIP from it: the latter auto-detected via ip route get $ROBOTHOST, so the PC always advertises the address of whichever local interface actually holds the route to the robot, without per-script logic. Migrating network topology is now a one-line change in a single file rather than an edit to five independent scripts, a direct consequence of the multiplication of transport paths (local hotspot, campus Wi-Fi, WireGuard) this campaign introduced.

##### Operational trap: stale connections across a topology transition.

On every network-path change observed during the migration, ROS processes already running on the PC (MINS, the pose selector, the mission backend) were found blocked on sockets bound to the previous path and did not recover on their own: they require an explicit restartstack.sh after the new path has stabilised, never during the transition itself. The failure signature (telemetry silent despite a healthy new link) is easy to misattribute to the transport when the actual cause is stale application-level state; this is the same class of fault as the TF double-authority conflict of §12.2.2: established connections do not self-heal merely because the network beneath them changed.

##### Single-command recovery.

A related gap surfaced from the operator asking, twice, whether leo restart alone was sufficient after the migration: it was not; the command restarted the web stack and backend but never the navigation stack, which still required a separate manual restartstack.sh call. A new [nav] step was inserted into  /bin/leo between the backend and web-server stages, invoking restartstack.sh unconditionally (it is itself idempotent, so no guard is needed at the call site). Verified end-to-end twice, independently, by the author and by the operator: a single leo restart now brings up rosbridge, the camera, the backend, and MINS with automatic pose-source switch, confirmed each time via live telemetry (camalive=true, posesource=MINS).

### Field validation: building-wide manual traverse (July 20)

The migration’s stated goal, exercising the rover anywhere in the building without being limited by the local hotspot’s range, was validated in a manual-drive traverse of the full building on 2026-07-20, the first field test conducted entirely outside the lab room since the network migration. Link health was logged throughout at 4 s resolution (ICMP round-trip to the WireGuard peer, cross-checked against /mission/telemetry’s camalive and posesource fields), state-change-triggered to avoid conflating measurement noise with genuine events.

The tunnel held for the entire traverse with a single clean exception: one isolated ping timeout (∼5 s), self-recovered with no operator intervention and no corresponding telemetry gap on either side of it, consistent with a momentary radio-level fade (a corner, a fire door, a crowded 5 GHz channel) rather than a tunnel or routing fault, since the WireGuard association itself required no renegotiation to resume.

A second, more consequential phenomenon recurred repeatedly during the traverse: posesource flipped from MINS to VINS for ∼5–6 s before returning to MINS unprompted, several times, with camalive remaining true throughout. Read live during the walk this looked like a brief, self-healing curiosity and was provisionally attributed to transient visual feature starvation; correlating the ground-station watchdog’s own log against the observed flip timestamps afterward overturned that reading entirely. leowatchdog.sh’s zombie-detection guard (introduced 2026-07-13) declares the stack dead if three consecutive one-minute cron ticks fail to find /minssubscribe registered with the ROS master via rosnode list, and, on that verdict, tears down and relaunches the entire navigation stack. watchdog.log shows this guard firing at 18:01, 18:04, 18:05 (compounded by one instance of the independent 100 m MINS-divergence guard), 18:07, 18:10 and 18:13 (essentially continuously throughout the traverse), each event followed by the same stack relancée / MINS initialisé après N s / switched to MINS sequence restartstack.sh always performs, which is exactly the transient a fresh poseselector process (defaultsource="VINS" in navigationsupervision.launch) produces before being switched back. The corridor traverse succeeded despite the navigation stack being torn down and relaunched every 1–3 minutes for its entire duration, not because MINS ran uninterrupted.

The root cause is the zombie guard’s probe itself: rosnode list is an XML-RPC round trip from the ground station to the robot’s master, and its three-tick/10 s-timeout debounce was sized against an earlier isolated false trigger (a single missed tick “under load”), not against the sustained, multi-minute registry unavailability that campus Wi-Fi roaming on wlanint plausibly produces while the rover is physically moving between access points. The mitigation applied is a widened, conservative debounce (three ticks → six, 10 s → 20 s per attempt) rather than a redesign of the check itself, since the true-positive case it guards against (a genuine robot power-cycle) is unaffected by widening the window: only the false-positive rate during motion should drop. The same audit was extended to every other network-round-trip probe in leowatchdog.sh: the camera-silence escalation’s expensive step (a D455 hardware reset, ≥35 s without vision, during which autonomous mode refuses to move) required only two failed 6 s probes and is now four at 12 s, while its cheap first-tier response (a rosmon stop/start cycle) is left immediate, since a false trigger there costs seconds rather than tens of seconds; the IMU-starvation guard’s debounce was widened two ticks to four for the same reason. Three probes were left unchanged after the same analysis: the rosbridge liveness check targets 127.0.0.1, not the robot, so roaming cannot affect it; the MINS-divergence guard already degrades safely on a network timeout (an empty read is ignored by construction, not treated as evidence of divergence); and the republish-zombie purge requires an asymmetric result (one topic silent, a related one not) that uniform network degradation would not itself produce. None of these widened debounces were re-validated in motion within this session and are carried forward as the open item (Table [tab:openitems], item 9): confirm the false-trigger rate under a second in-motion traverse, and, if it persists, replace the network round trip with a check that does not require a live RPC to the robot’s master while it is roaming.

##### A reliability model for the debounce.

The choice of six ticks, rather than an arbitrarily larger number, can be grounded quantitatively instead of by intuition alone. Model each one-minute watchdog tick as an independent Bernoulli trial that fails the /minssubscribe registry check with probability p (elevated during motion; at baseline effectively zero, this guard never false-triggered in any stationary session). The guard resets its counter to zero on every successful tick and re-arms after every trigger, so it fires the first time n consecutive ticks fail: the classical waiting-time-for-a-run problem. Writing Ek for the expected number of further ticks to a trigger starting from a current run of k<n consecutive fails (En=0), the one-step recursion

Ek=1+p Ek+1+(1−p) E0

solves, for p<1, to a closed form for the expected number of ticks between false triggers starting from a fresh reset:

E0(n,p)=1−pn(1−p) pn.

(Sanity checks: E0(1,p)=1/p, the ordinary geometric waiting time; for a fair coin, E0(2,12)=6, the textbook expected-flips-to-HH result.) As p→1, E0(n,p)→n: a guard fires almost exactly every n ticks once probe failure becomes near-certain, which is precisely the signature in watchdog.log: zombie-cause triggers recurred at 18:01, 18:04, 18:07, 18:10 and 18:13: four consecutive intervals of exactly three minutes under the old n=3 debounce. Equation [eq:debounce] shows this pattern is only produced by p close to unity (Table 12.2): even at p=0.90, E0(3,p)≈3.72, visibly above the observed three-minute cadence, and p=0.99 gives E0(3,p)≈3.06, matching it almost exactly. The data are therefore most consistent with p≳0.9 during the traverse: the XML-RPC round trip was failing on nearly every tick while the rover moved, not merely at some elevated-but-partial rate.

##### A likelihood argument for that estimate.

The four observed inter-trigger gaps are not just individually close to the n=3 floor; under the model they are a proper i.i.d. sample that supports a maximum-likelihood treatment rather than a by-eye table lookup. Because a debounce absorbs at exactly T=n ticks if and only if the first n post-reset ticks all fail (any earlier success would have reset the counter and pushed T higher), the probability of a single observation landing exactly on the floor is Pr(T=n∣p)=pn. With four independent gaps all equal to n=3, the joint likelihood as a function of p is

L(p) = i=14Pr(Ti=3∣p) = (p3)4 = p12,

which is strictly increasing on (0,1): the maximum-likelihood estimate is driven to the boundary p→1, with no interior optimum. This is itself informative (it says the four-tick sample contains no evidence of even one successful probe inside any of the four sustained-failure windows), but a point estimate at the boundary is not practically meaningful (a literal p=1 is not a physical hypothesis, and a sample of four cannot distinguish it from p=0.99 at one-tick resolution). A likelihood-ratio comparison is more honest about what the data actually discriminate:

Λ(p1,p2) = L(p1)L(p2) = p1p212.

Evaluated at representative points, Λ(0.9,0.5)≈1,157 (the observed pattern is over a thousand times more probable under sustained 90 % per-tick failure than under a merely elevated 50 %), while Λ(0.99,0.9)≈3.1 and Λ(0.99,0.95)≈1.6: the evidence discriminates sharply against low p but only weakly between different high values of p, exactly the boundary-flattening behaviour expected of a likelihood proportional to p12 as p→1. The conclusion carried into the debounce redesign is therefore not a precise point estimate of p but a robust qualitative one: p was high enough, for long enough, that no finite widening of n fully closes the gap, which is exactly the honesty Table 12.2 and the discussion below preserve.

Expected ticks between false triggers, Eq. [eq:debounce], for the old (n=3) and new (n=6) debounce across the per-tick failure-probability range fitted against watchdog.log. The ratio converges to exactly nnew/nold=2 as p→1, a useful internal check on the derivation.

| Per-tick failure probability p | E0(3,p) (old) | E0(6,p) (new) | Ratio |
| --- | --- | --- | --- |
| 0.80 | 4.77 min | 14.08 min | 2.95×​ |
| 0.90 | 3.72 min | 8.82 min | 2.37×​ |
| 0.95 | 3.33 min | 7.21 min | 2.17×​ |
| 0.99 | 3.06 min | 6.22 min | 2.03×​ |

The model is honest about what widening the window does and does not buy: across this whole p range the mitigation roughly doubles to triples the mean time between false triggers, but does not eliminate them: if p stays this high for the duration of a longer demonstration, the stack would still be expected to restart every 6–9 minutes rather than every 3–4. This is a quantitative restatement of why item 9 (Table [tab:openitems]) keeps the probe redesign on the roadmap rather than treating the debounce widening as closed: no finite increase in n removes a persistently high p, it only dilutes its consequences.

No mission-affecting failure occurred: the rover remained drivable and localised throughout every restart cycle, the operator completed a full loop of the building, and every transient (link and stack alike) resolved itself before requiring intervention. That the outcome looked, in real time, indistinguishable from nominal operation is itself notable: the respawn policy of §12.2.4 and the idempotent design of restartstack.sh absorbed a failure mode three times worse than diagnosed live, without the operator noticing.

## Verification and validation

### Protocol T1.1: static drift

Five minutes stationary after a fresh initialisation, ∼23 000 samples of /robotposefused; per axis, the standard deviation and the least-squares drift slope

β=k​(tk−t)(xk−x)k​(tk−t)2

with verdict criterion: |β|<1 mm/s and bounded excursion ⇒ noise (covariance tuning domain); a frank monotone slope ⇒ divergence (extrinsics/initialisation domain, not noise parameters). The criterion separates the two failure families because their governing mathematics differ: stochastic noise integrates to a random walk whose expected excursion grows as t and whose fitted slope concentrates around zero as the window grows, whereas a modelling error (wrong extrinsic, wrong gravity, wrong scale) produces a deterministic bias whose fitted slope converges to a non-zero constant. With n≈23 000 samples over T=300 s, the standard error of the slope estimate, SE(β)=σx/k​(tk−t)2, is of order 10−3 mm/s for the measured σz: three orders below the decision threshold, so the verdict is statistically unambiguous. The protocol is fully automated (stack restart with fresh initialisation, source switch, five-minute log, per-axis analysis) and archives its CSV for audit.

### T1.1 A/B result: extrinsic correction

T1.1 A/B: the only change between columns is the camera–IMU extrinsic seed (Eq. [eq:extr]). σ reduced 5.2× on Z; the slope crosses the bounded-noise criterion.

| Axis (5 min static) | Identity extrinsic | Camera-forward extrinsic | Interpretation |
| --- | --- | --- | --- |
| X: σ / slope | 7.5 mm / +0.05 mm/s | 8.8 mm / −0.06 mm/s | wheel-anchored |
| Y: σ / slope | 0.6 mm / ≈0 | 1.6 mm / +0.01 mm/s | wheel-anchored |
| Z: σ / slope | 257 mm / +1.68 mm/s (divergent) | 49.7 mm / −0.34 mm/s (bounded) | camera-observed only |

The full resolution history of the vertical drift spans three regimes: (i) z≈−445 m: frozen IMU integrating a gravity residual, resolved by the integrity guards (factor ∼300); (ii) +1.68 mm/s divergence: identity extrinsic, resolved by Eq. ([eq:extr]) (factor 5); (iii) bounded ±5 cm noise: the current state, refinable by the Kalibr extrinsic (open item) and by global anchoring (below).

### Front-end real-time verification

The monitor tools/minsfrontendmonitor.py parses the estimator’s own timing output and the process CPU, and issues a real-time verdict (median image cost against the camera period). Campaign result: 120 ms → 67 ms per image at the 66.7 ms budget: at the boundary, motivating the pending 10 Hz decision.

### Field protocols

Slip rejection: a 5 m out-and-back with a provoked 1 s wheel-hold during acceleration; acceptance requires no pose jump during the slip (wheel measurements visibly rejected by the χ2 gate) and a loop-closure error below 2% of distance. Mission cycle: patrol at 0.2 rad/s, AprilTag acquisition ≥3 m, LOCK, checkerboard confirmation <1.5 m, reset with world-frame preservation, marker deduplication, resume; obstacle bypass with persistent mapping as per Figure 12.3.

## Field session, July 21: fault isolation and an architectural revision

The day after the debounce widening of §12.6.5, a second field session set out specifically to re-validate it in motion (Table [tab:openitems], item 9). Before the traverse itself could begin, three unrelated faults surfaced and were resolved, one architectural assumption was revised, and one live divergence event was caught and quantified in real time: documented here in the order encountered, since each diagnosis depended on ruling out the previous one.

### A registered node that had gone silent

On startup, MINS failed to initialise twice in succession, each attempt timing out after its full 300 s budget. The estimator’s own log made the cause unambiguous: [IW-Init]: Waiting for collecting IMU(493 < 3) and wheel data(0 < 3), repeated every sample. IMU measurements were flowing normally (hundreds accumulated); wheel measurements were stuck at exactly zero. Tracing the chain (§12.5: wheelremap subscribes to /jointstates and republishes onto /jointstatesmins, the topic MINS actually consumes) located the break: rostopic info /jointstates reported zero publishers, even though /firmwaremessageconverter (the robot-side node responsible for it) was still registered with the ROS master and had not exited. A node can therefore be alive by every liveness check available to the watchdog (process present, node registered) while its actual output has silently stopped: registration proves the node exists, not that it is doing anything. The immediate fix was a rosmon stop/start cycle of firmwaremessageconverter (/jointstates resumed at ≈18 Hz within seconds, confirmed through to /jointstatesmins); the standing fix is the open item this incident adds to Table [tab:openitems] (item 10): the watchdog’s liveness checks test for node presence, never for topic output, and this class of fault, a healthy process publishing nothing, is invisible to it by construction.

### An unbounded “still starting”

A second, independent fault was found while investigating unrelated cockpit disconnections: the TLS rosbridge tunnel process had been running since 03:16 that morning without ever having bound its port, and the watchdog had been leaving it alone every single tick since, because its own idempotent design (§12.6) treats a live process with an unbound port as still starting rather than stuck: the correct call for a process seconds into a normal startup, but with no upper bound on how long “still starting” is believed. The process was manually killed and cleanly relaunched; the design gap (Table [tab:openitems], item 11) is that the grace period needs a ceiling, past which the same process is instead treated as failed and restarted.

### WireGuard endpoint roaming, formalised

The most consequential fault of the day was an intermittent, self-reasserting loss of the WireGuard tunnel established in §12.6.4, alternating between minutes of clean operation and total outages lasting up to 20 minutes despite the underlying network path (confirmed throughout by ICMP and SSH to the robot’s campus address) remaining fully reachable. The mechanism is a documented WireGuard behaviour, endpoint roaming, that had not previously mattered because the tunnel’s two ends had never before had a plausible wrong address to roam onto. Each peer tracks the other’s endpoint not from static configuration but from observation:

EA(t) = src​p*,  p*=arg maxp : τ(p)<t, valid(p)τ(p),

i.e. node A’s belief about peer B’s address is simply the source address of the most recent cryptographically valid packet A has received from B, with no memory of the original Endpoint line beyond supplying the first outbound attempt. This is normally the feature that makes WireGuard tolerate a roaming client transparently; here it became a liability because the robot has two live addresses (wlanext’s 10.0.0.1 and wlanint’s 10.154.27.235, per Figure 12.4), and a single stray packet observed from the wrong one is sufficient, by Eq. [eq:roam], to overwrite the correct runtime endpoint until another valid packet arrives from somewhere else. wg show confirmed the mechanism directly: the robot’s own view of the PC’s endpoint had drifted to 10.0.0.10:55458, the PC’s address on the robot’s own hotspot subnet, rather than the campus address the static configuration specifies, most plausibly seeded during an earlier troubleshooting step where the PC’s Wi-Fi briefly associated with LeoRover-e138. No config file was wrong; Eq. [eq:roam] does not consult the file after the first packet.

##### Why restarting alone did not reliably fix it.

The obvious remedy, a systemd restart, was tried first and failed with wg-quick: ‘wg0’ already exists. The cause is a state-consistency gap between two independent trackers of the same resource. Let Ssd∈{active,inactive} be systemd’s belief about the service and Sk∈{present,absent} be the kernel’s actual interface state. A manual wg-quick down (run outside systemd, as one naturally does while debugging) transitions Sk​:present​→​absent without touching Ssd, which remains active. Systemd’s own contract for start is to act only on the transition Ssd​:inactive​→​active; called against a unit it already believes active, it is a no-op. The pair (Ssd,Sk)=(active,absent) is consistent with neither “running” nor “stopped” and is not self-healing: nothing in the ordinary stop/start vocabulary detects or repairs it. The correct sequence forces both trackers through a shared reference point, stop (which accepts the active→inactive transition unconditionally) before start:

$$\begin{equation}
    (\text{active}, \text{absent})
    \;\xrightarrow{\ \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}systemctl stop}\ }\;
    (\text{inactive}, \text{absent})
    \;\xrightarrow{\ \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}systemctl start}\ }\;
    (\text{active}, \text{present}).
    \end{equation}$$

Once this sequence was applied identically on both ends, Eq. [eq:roam] re-converged on the correct address within one keepalive interval. The generalisable lesson: mixing a systemd-managed unit with direct, out-of-band control of the same kernel resource (here wg-quick down invoked by hand) is not just risky but can reach a state neither layer’s ordinary vocabulary can name, let alone repair: the same family of fault as the supervisor-probe blind spots of §12.8.1 and Table [tab:openitems] item 2, now at the level of process supervision itself rather than a ROS node.

##### The recurrence that restarts could not fix.

The fix above restored the tunnel for approximately seven minutes, after which it failed again and did not recover under repeated, methodical retries: full stop/down/start cycles on both ends, a four-minute idle period on the hypothesis of a transient campus-side rate limit, and a change of the tunnel’s UDP port (51820→51821, then back) to rule out a port-specific block. None of these restored a handshake, while ICMP and SSH to the robot’s campus address continued to succeed throughout: proof the robot remained fully reachable and the fault was confined to the WireGuard flow specifically. This second failure is recorded as unresolved (Table [tab:openitems], item 12): the evidence is consistent with an intervention on the campus network path itself (a firewall or NAT state change, plausibly triggered by the unusually high rate of reconnection attempts during the first fault’s diagnosis) but this could not be confirmed without visibility into campus-side logs this session did not have access to.

### The architectural fix: routing within the firewall’s boundary instead of across it

With the tunnel itself unavailable, the working alternative was not another repair attempt but a change of topology: joining the ground-station PC to FLTech-Guest directly, on the same Wi-Fi network the robot’s wlanint already uses, rather than reaching it from the PC’s wired subnet. Figure 12.5 makes the distinction precise. Two hosts sharing a subnet prefix (both PC and robot inside 10.154.0.0/19 once the PC joins FLTech-Guest) resolve each other through the local link layer: address resolution and switch-level forwarding only, no router hop, and therefore no opportunity for the campus firewall’s inter-VLAN policy to apply, since that policy is enforced at the router that stateful cross-subnet traffic is forced through. This is exactly the traversal TCPROS’s arbitrary ephemeral ports could not survive in §12.6.4: it never had to survive it here, because there is no boundary in the path to survive. Re-measured directly, with no tunnel of any kind in the loop: /firmware/wheelodom sustained at 20.1 Hz, MINS initialised and switched automatically, camera and telemetry both confirmed live.

The firewall policy of §12.6.4 is enforced at a router boundary, not on the campus network as a whole. Left: the PC’s wired subnet and the robot’s Wi-Fi subnet are different networks, so every packet between them is forced through the campus router where the inter-VLAN policy blocks TCPROS’s arbitrary ports (Table 12.1’s motivation for a tunnel). Right: when the PC joins the robot’s own Wi-Fi network directly, both hosts share one subnet; traffic between them never reaches a router at all, so the same policy simply does not apply. The trade-off, not shown here, is that this returns the PC to Wi-Fi and its documented driver fault (§12.6.4) instead of the driver-immune wired path: a deliberate, disclosed exchange of one known risk for another, not a free fix.

The trade-off is explicit, not hidden: this path returns the ground-station PC to Wi-Fi, and specifically to the same rtl88x2bu adapter whose kernel driver fault (§12.6.4) was the original reason it was moved to Ethernet in the first place. Adopting it is a deliberate exchange of one known, bounded risk (occasional PC-side Wi-Fi drops, independent of anything the robot does) for a currently-unresolved and unbounded one (item 12). Table [tab:openitems] records the follow-up this implies: decide, once item 12 is understood, whether to return to the tunnelled Ethernet path or adopt same-subnet Wi-Fi permanently, rather than carrying both as live options indefinitely.

### A live divergence, caught and quantified

Minutes into the re-established session, a routine stationarity check (the same measurement discipline as Protocol T1.1, §12.7, applied ad hoc rather than as the full five-minute protocol) surfaced a genuine filter divergence in real time. With the robot confirmed stationary (commanded and observed planar velocity identically zero), 684 samples of /robotposefused over a 9.9 s window showed

Δz=−99.78 m,  Δx2+Δy2/Δt=32.0 mm/s,

both far outside Protocol T1.1’s bounded-noise criterion (|β|<1 mm/s, §12.7) and immediately recognisable as another instance of the vertical divergence failure mode fully characterised in §12.1: an unmodelled residual acceleration δa integrating quadratically, δz(t)=12δa t2 (Eq. [eq:se3]-adjacent discussion). Solving for the residual this specific event implies,

δa = 2 |Δz|Δt2 = 2×99.789.92 ≈ 2.04 m/s2,

a physically modest, entirely plausible residual, roughly a fifth of g, well within the range an unconverged initial bias or a brief IMU integrity-guard gap could produce, and consistent with the same mechanism as the original z≈−445 m case (Table [tab:openitems], historical) rather than a new failure mode. Rather than wait for the watchdog’s own 100 m divergence guard (§12.5) to act on its next one-minute tick, restartstack.sh was invoked immediately. The fresh instance was re-measured on the identical protocol immediately after: 708 samples over 10.0 s gave a planar drift of 0.1 mm/s and a z excursion of 13 mm: two orders of magnitude inside the noise criterion, and a clean, dated, field confirmation that the recovery path designed around this exact failure mode (respawn policy, §12.2.4; divergence guard, §12.5) works as specified when exercised for real rather than only in the original diagnostic session. The same guard fired autonomously twice more later in the traverse (14:57:14 and 15:00:12), each time without operator intervention: the July 20 fix is not a one-off, it holds up under repeated real exposure.

### Traverse outcome: debounce revalidation, and a quantified roughness

The watchdog log for the traverse (12:58–15:34, Twindow=9351 s) gives a direct, dated answer to the question the day opened with (Table [tab:openitems], item 9): does the widened debounce actually reduce false zombie-guard triggers in motion, or only on paper? Twenty-one triggers were logged; the twenty intervals between them split cleanly into two regimes,

Δtzombie∈359–421 s(13 consecutive intervals, early traverse)543–720 s(7 intervals, later traverse)  Δtzombie≈447 s,

against the 60–180 s cadence characterised on July 20. Even the shortest interval measured today (359 s) exceeds the longest interval from that traverse by a factor of 2; the mean is higher by roughly 3–7×. Item 9 is therefore only partially closed: the debounce widening (3→6 ticks, 10→20 s) clearly moves the false-trigger rate in the right direction and by a large margin, but does not eliminate it, and the underlying mechanism (a rosnode list round trip to a roaming master, §12.6.5) is unchanged and will keep firing at some reduced rate under any amount of further widening.

Each zombie-guard trigger, and each of the three divergence-guard recoveries above, invokes the same restartstack.sh path, whose MINS re-initialisation step is itself logged per attempt. Of 21 recorded attempts, 19 succeeded within the routine range (median 21 s), but the distribution has a long tail: two took 90 s and 260 s respectively, and two more exhausted the script’s 300 s ceiling outright (TIMEOUT init MINS, a designed hard exit rather than a hang). Treating the sum of logged init durations, plus the two 300 s ceiling hits, as a lower bound on the time the stack spent with no valid MINS pose during the window gives a rough operational availability figure:

Apose ≳ 1−i​Tinit,i+2×300 sTwindow=1−811+6009351 ≈ 0.85.

This is a lower bound, not a measurement of true downtime: it omits the bounded shutdown-wait phase before each re-init begins and any transient settling after /poseselector switches source back to MINS. Read as a floor, it says the stack was demonstrably without a valid pose for at least ∼15 % of the traverse, distributed as roughly one multi-second-to-multi-minute gap every 7 minutes on average, matching the zombie-guard cadence above almost exactly. This gives a concrete, falsifiable candidate explanation for a field observation reported directly by the operator during the walk: the robot advancing in visibly discontinuous, roughly one-second bursts rather than smoothly, with occasional failures to register nearby walls as obstacles, and intermittent loss of the operator link. A stack that loses and regains pose on a ∼7-minute cycle, with individual gaps ranging from tens of seconds to (twice) the full 300 s ceiling, is expected to produce exactly this symptom: motion and obstacle avoidance both degrade to whatever the planner can do without a trustworthy, continuously updated pose. This traverse is also the first to run entirely over Wi-Fi with no Ethernet fallback (§12.8.4), which independently revives a risk flagged but not yet measured in an earlier diagnostic pass: Wi-Fi bandwidth contention degrading the raw image topics feeding both MINS and the local costmap. Nothing logged today distinguishes between “pose gap during a restart” and “image-topic starvation under Wi-Fi” as the dominant cause of the reported roughness, so both are recorded (Table [tab:openitems], item 14) rather than picking one without evidence; the discriminating measurement is a direct Hz trace on the Pi’s image topics during a future traverse, run concurrently with the pose-availability bookkeeping above.

Two secondary results close out the traverse. The camera watchdog logged eleven “muet” (silent) recoveries and zero escalations to the widened hardware-reset threshold (fails≥4, leowatchdog.sh): every transient camera stall this traverse resolved within the software stop/start cycle, which is exactly the discrimination the widened threshold was set up to make (transient vs. genuinely wedged) and, unlike item 9, this one shows no counterevidence today. And the beacon test originally scoped for the morning, deferred to an A4 tag in the absence of an A3 print, succeeded: /tagdetections went active at 15:34:12, confirming long-range AprilTag acquisition works with the smaller tag at whatever range the traverse presented, though the size-dependent range itself was not separately measured this session and remains item 7. Return-to-Base precision was not attempted within the logged window and stays open for a future traverse.

### The jerky motion, re-diagnosed: a command-path deadman, not a pose gap

The pose-availability argument of §12.8.6 explains part of the reported roughness, but a second, more direct mechanism turned out to dominate the specific symptom the operator described: the robot advancing in sharp, roughly one-second bursts. Manual drive commands are resent from the browser every 150 ms over the rosbridge WebSocket while a direction is held, and the backend applies its own deadman on top: no fresh command within MANUALDEADMAN zeros the commanded velocity outright, a safety rule (“homme-mort”) against a frozen or dropped control device. That constant was 0.5 s, sized when the ground station sat on Ethernet. Once the operator confirmed they were driving from a phone walking alongside the robot rather than from the stationary station, the diagnosis followed directly: that phone’s own Wi-Fi association now roams between campus access points exactly as the robot’s wlanint already does (§12.8.3), and normal jitter on that link exceeding 500 ms several times a minute is enough to trip the deadman on an otherwise healthy connection, well short of a full WebSocket disconnect. MANUALDEADMAN was widened to 1.0 s (still under half a metre of coasting at the manual-drive speed of 0.2 m/s, an acceptable margin for an indoor platform with the operator present) and the frontend’s fixed reconnect delay after a full WebSocket close was shortened from 2.5 s to 1.0 s, so that the rarer full-disconnect case also recovers faster. Both changes were deployed to the already-running backend by killing the supervised process and letting respawn="true" relaunch it with the edited constants, the same zero-downtime mechanism used throughout §12.8 for single-node recovery, verified here by comparing the process start time against the file modification time before declaring the change live.

### Yaw trim: a live calibration, converged by operator feedback rather than by log

The leftward drift itself (Table [tab:openitems], item 1) was diagnosed, not just noted, while the traverse continued. A 30 s passive listener on /firmware/wheelstates during ordinary forward driving showed the four wheels’ commanded angular velocities matching to ±0.0–0.1 % (536 samples, right/left ratio 1.0000): the per-wheel velocity controllers are symmetric, ruling out a motor or PID asymmetry. Correlating those same windows against /imu/dataclean restricted to segments where left and right wheel speed agreed to within 3 % (289 samples) isolated a genuine physical effect: a median yaw rate of +0.0530 rad/s at a mean wheel speed of 3.219 rad/s (v≈0.201 m/s), i.e. a relative right/left effective-radius bias of

rright−rleftr ≈ ψω = 0.05303.219 ≈ 1.65 %,

consistent with the operator’s own report and with item 1’s standing hypothesis, though still short of the roll-out measurement that item calls for. A proportional trim, angtrim=K⋅lin, was added at publish’s single command exit point (the same location already used for the drive-polarity correction of §12.1), with K initialised by cancelling the measured bias exactly: K=−0.0530/0.201≈−0.264 rad/s per m/s.

That first value overcorrected, and by more than the raw measurement would predict: the operator reported drifting right on two separate tries at full cancellation. A batch re-measurement taken during one of those tries showed almost no residual (+0.0085 rad/s median, inside the gyro’s own noise floor of 0.167 rad/s), directly contradicting the operator’s real-time report of the same configuration. The likely explanation is methodologically useful beyond this one fix: an operator who feels a drift corrects for it, so a passive measurement taken while they are actively driving averages together the physical bias and the human’s own compensation, and can read as “fixed” when it is not. Unlike the log-versus-live-hypothesis lesson of §12.6.5 (trust the log over an explanation formed in the moment), this is the converse case: here the human, being inside the loop being measured, was the more reliable signal, and K was instead converged by bisection on direct qualitative feedback,

| K (rad/s per m/s) | trim at 0.2 m/s | operator report |
| --- | --- | --- |
| −0.264​ | −0.0530 rad/s | drifts right (“trop”) |
| −0.132​ | −0.0265 rad/s | drifts right, less (“un peu trop encore”) |
| −0.075​ | −0.0150 rad/s | drifts left, barely (“très très légèrement”) |
| −0.087​ | −0.0175 rad/s | accepted (“beaucoup mieux, restons comme ça”) |

four iterations, each redeployed live the same way as §12.8.7, converging in under ten minutes of wall-clock time including drive time. The accepted value, roughly a third of the naive full-cancellation trim, is recorded as the current operational constant; it is a software workaround for a still-unmeasured mechanical asymmetry, not a substitute for the roll-out measurement item 1 already calls for, and should be revisited (most likely removed, once the true radii are known and configwheel.yaml corrected) rather than carried forward indefinitely.

### AUTO mode control-loop hardening, and a genuine blind-spot found by driving into a wall

A direct operator assessment of AUTO mode (“loin d’être satisfaisant”, citing erratic patrolling, unreliable obstacle avoidance, and a fussy beacon lock) prompted a code review of the three mechanisms behind each complaint, rather than a live measurement as in the preceding sections: the failure patterns were structural enough to diagnose from the control logic itself. All three share a common shape, a decision recomputed every control tick from a single, un-smoothed sample, with no memory of the previous tick:

- Patrol steering: the “open path” bearing used to bias PATROL’s heading was recomputed fresh from the current depth-sector scan every tick; two adjacent, similarly-open sectors trading places between frames (ordinary sensor noise) swung the commanded heading abruptly each time. Fixed with an exponential moving average on the tracked bearing (α=0.3), reset only when no open path is currently detected.

- Avoidance direction: which way to pivot around an obstacle was chosen from a single instantaneous left/right depth median at the moment the obstacle was flagged; one noisy frame could commit the robot to a 45–135° turn toward the more obstructed side. Fixed with the same class of exponential smoothing (α=0.4) applied to the left/right medians themselves, so every consumer (this decision and the corridor-attraction bearing) sees a steadier signal.

- Beacon lock chatter: whether the robot considered itself “centred” on the beacon was a single hard threshold (±8 % of frame half-width) evaluated independently each tick; a detected centre jittering near that line (LED/checkerboard detection noise is a known, longstanding property of this pipeline, §10) flipped the flag rapidly, alternating advance and re-centring commands. Fixed with hysteresis: entry stays at ±8 %, exit only past ±14 %.

None of these three required new sensors or a different algorithm, only the standard control-systems remedy for a decision that chatters on noisy input: smooth the input, or widen the band around the decision boundary.

The fourth finding was not architectural but a genuine safety gap, surfaced directly by the operator during the very next test: “il fonçait dans les murs quand il était en face d’eux” (it drove straight into walls it was facing head-on). AUTO mode’s only blind-driving guard checked camalive, freshness of the colour/IR stream feeding beacon detection, before allowing forward motion. Obstacle sensing is a different stream (/pc/camera/depth/... versus /pc/camera/infra1/...), republished over the same WiFi path but independently, and nothing checked whether it was still arriving. Under the bandwidth contention already flagged as a live risk this session (item 14), the depth stream can stall while the IR stream keeps flowing: camalive stays true, and PATROL kept commanding ADVANCELIN on a stale or absent obstacle flag, no different in effect from disabling the sensor outright. A second freshness gate, DEPTHALIVETIMEOUT, was added at the same point in drive() as the existing camera gate, deliberately tighter (1.0 s against the camera gate’s 2.0 s, §12.8.7): this one bounds blind travel directly against a collision margin rather than against a beacon-detection convenience, and at ADVANCELIN (0.2 m/s) one second bounds that travel to

dblind,max=ADVANCE_LIN×DEPTH_ALIVE_TIMEOUT=0.2×1.0=0.20 m,

comfortably inside the 0.6 m obstacle-trigger margin (OBSTACLEDISTMM). A matching AUTONODEPTH FSM state was added so a future stall is visible in the cockpit as a labelled, deliberate stop rather than an unexplained one. All four fixes were deployed live via the same process-respawn mechanism as §12.8.7; a full validating traverse was not completed this session (§12.8.10 consumed most of the remaining test window), so item 15 below carries that forward.

### A disk-full crash found mid-diagnosis

The camera failures during the AUTO-mode test initially looked routine (§12.8’s escalation ladder: soft cycle, then hardware reset past four consecutive failures) but did not resolve after two hardware resets, an escalation pattern not seen earlier in the campaign. An unrelated SSH command surfaced the real cause directly: No space left on device on the robot’s own root filesystem, confirmed at 100 % used, 0 bytes free (df -h). The two largest recoverable consumers were both ordinary, unbounded log accumulation rather than anything load-bearing: 2.6   of timestamped roslaunch run logs under /var/ros/log (37 directories, one per launch, never purged) and 2.9   of systemd journal. Both were cleared with the operator’s explicit authorisation (the destructive-command guard correctly declined to run an unattended rm -rf first), freeing 5.3   and returning the disk to 82 % used.

A second, more serious fault was discovered only because the first one was fixed: leo.service itself had crashed some minutes earlier, exit-code 139 (SIGSEGV), and, lacking a restart policy, simply stayed dead rather than being relaunched by systemd. Every symptom of the following twenty-five minutes, ROS master unreachable from the PC while ICMP and SSH stayed healthy throughout (the same discriminating signature as §12.8.3’s WireGuard fault, here with a different root cause), restartstack.sh retrying against a master that did not exist, and dozens of stale waitmins/montour node registrations accumulating in rosnode list, traced back to this one dead service, not to a new network problem. sudo systemctl restart leo resolved it immediately once found. Table [tab:openitems] records two follow-ups this incident implies beyond item 1’s mechanical measurement: monitor disk usage before it reaches zero rather than discovering it through a downstream symptom, and give leo.service a restart policy so a future crash of this kind self-heals the way every other component in this architecture already does.

## Field session, July 22: closing the beacon-approach loop, and a first application of continuous control

### Beacon reset gated on dual confirmation, and a corrected post-visit turn

The reset that zeroes local odometry against a visited beacon (§12.8.9) fired, in both the MANUAL-mode and AUTO-mode LOCK paths, on a single confirmed cue: the LED cluster alone (MANUAL), or a measured distance from either the AprilTag or the checkerboard (AUTO, the “tout-en-un” path introduced 2026-07-13). Both were widened to require the LED cluster and the checkerboard confirmed simultaneously before the reset fires, on the reasoning that the checkerboard’s sub-pixel corner solution is a materially more reliable pose reference than the LED cluster’s centroid alone, and a reset is a consequential, silent operation (it re-anchors the robot’s entire local frame) that should not commit on a single, weaker cue when a stronger one is available from the same detector pipeline.

A dual requirement introduces a failure mode the single-cue version did not have: the robot can arrive at the stop distance with one cue confirmed and the other not (the checkerboard’s usable range is ≲1.5 m and depends on which face is presented, §10; the LED cluster is visible from a wider range of angles and distances), with no existing timeout that covers that state, only LOCKTIMEOUT (fires when the target disappears) and LOCKSTALLS (fires when centring never converges). Confirmed-but-unreset can now persist indefinitely. A bounded grace hold was added: on arrival, the robot freezes (lin=ang=0, the branch’s existing default) for DUALCONFIRMGRACES =4.0 s while polling for the second cue; if it never arrives, the attempt is abandoned through the codebase’s existing, field-proven obstruction-avoidance path (mapobstacle + enteravoid, the same call already used one branch up for a depth-obstruction abandon) rather than a new, unexercised exit, with the standard LOCKCOOLDOWNS applied so the same ambiguous target is not immediately re-locked. One consequence is accepted deliberately, not incidentally: the corridor-depth arrival path (no tag or checkerboard distance available at all, the robot treats a close depth reading as “arrived”) almost never has a valid checkerboard by definition, so it will now expire its grace window on essentially every attempt and register no beacon, only a contournement. Requiring the checkerboard everywhere trades that coverage for the precision argument above; whether the trade is net positive depends on how often a beacon is only approachable from a checkerboard-blind face, not yet characterised in this campaign.

The fixed 90° post-beacon turn (§12.8.9’s predecessor, itself a deliberate 2026-07-08 replacement of a 360° scan phase, chosen specifically for a perpendicular sweep over the alternative of turning back along the same line) was changed to a full 180° demi-tour on explicit request. The change is a pure constant edit (POSTBEACONTURNRAD), confirmed isolated: obstacle-avoidance’s own fallback turn hardcodes π directly and the reactive bounce-pivot uses an independent random target (§12.8.9), so nothing else references this constant. It reopens the coverage trade-off the July 8 redesign closed, a demi-tour revisits the same line rather than sweeping a new one, recorded here as a conscious reversion rather than an oversight, and left for field observation to judge, not asserted as an improvement.

### A first application of continuous control: PID regulation of three AUTO-mode loops

Every closed-loop decision in AUTO mode up to this point (§12.8.9) was either a bare proportional term or, in the case of obstacle response, not proportional at all: ondepth flags an obstacle the instant any depth tile’s 5th-percentile distance crosses OBSTACLEDISTMM (600 mm), which commits the robot to a fixed-angle reactive pivot (§12.8.9). Between that trip and the moment it fires, the robot advances at a constant ADVANCELIN, with no proximity awareness at all, the discretised control-loop analogue of an on/off thermostat rather than a regulator. An operator report during this session (the robot driving into walls it approached head-on, distinct from the July 21 blind-driving gap already closed by DEPTHALIVETIMEOUT, §12.8.9, which guards against a stale depth stream, not a live one read too late) is the direct motivation for the work below: a live depth stream read correctly still permits full-speed approach right up to the trip distance, an inherent property of a bang-bang controller, not a sensing gap. Three loops were converted from proportional-only to full PID: patrol steering, beacon centring (two instances, align-in-place and approach-while-centred), and a new obstacle-proximity speed governor introduced specifically to close this gap. All three share one controller implementation, described once below and then instantiated per loop with the substituted gains.

#### Discretised PID: the controller actually implemented

The canonical continuous-time PID law,

u(t) = Kp e(t) + Ki0te(τ) dτ + Kd de(t)dt,

with error e(t)=r(t)−y(t) between a reference r and a measured output y, has the familiar Laplace-domain transfer function

C(s) = U(s)E(s) = Kp + Kis + Kd s,

useful as a reference point but not directly implemented here: the control loop (controlloop, CONTROLHZ=20 Hz nominal, §12.8.7) has no enforced real-time guarantee, a slow tick simply runs late with no catch-up logic and no measured Δt threaded anywhere in the existing code, so a fixed-Δt discretisation (the textbook u[n]=u[n−1]+Kp(e[n]−e[n−1])+KiΔt e[n]+… velocity form) would silently assume a sample interval the scheduler does not actually guarantee. The controller instead timestamps every call itself against the same now the caller already computed once per tick, and integrates/differentiates against the measured interval:

Δt[n]&=t[n]−t[n−1],I[n]&=clamp​I[n−1]+e[n] Δt[n], Imin, Imax,Draw[n]&=e[n]−e[n−1]Δt[n],D[n]&=D[n−1]if Δt[n]>Δtmax (see below)0.5 Draw[n]+0.5 D[n−1]otherwise,u[n]&=clamp​Kp e[n]+Ki I[n]+Kd D[n], umin, umax.

Equation [eq:dfilt] is a first-order IIR (exponential) low-pass on the raw derivative alone, coefficient fixed at 0.5 rather than exposed as a per-loop tunable: three of the four instances below already consume an input pre-smoothed by an existing exponential moving average (§12.8.9’s patrol-bearing and left/right-depth EMAs, and the LED/checkerboard centroid EMAs, §10), so this internal filter is a safety net for the one signal that has no upstream smoothing (corridor depth, §12.9.2.2), not a second, redundant smoothing stage layered on already-clean signals. Δtmax=3/CONTROL_HZ=0.15 s bounds the derivative to ordinary scheduling jitter, an anomalously slow tick skips the derivative update for that step (holding the last filtered value) rather than computing a spurious spike against an inflated Δt, while still integrating and applying P normally; the integral memory itself is untouched by this guard and is retired only by the explicit reset() calls described below.

Two guards are applied independent of loop: on the first call after construction or an explicit reset() (t[n−1] undefined), the controller returns clamp(Kpe[n],umin,umax), proportional action only, avoiding both an undefined Δt in Eq. [eq:integral] and a derivative spike against an arbitrary previous error; and the integral term is clamped before summation (Eq. [eq:integral], Imin/Imax set independently of umin/umax), the clamped-integration form of anti-windup rather than back-calculation, chosen for its simplicity and because two of the four instances below run Ki=0, where windup cannot occur regardless of clamp; the two instances with Ki≠0 are discussed individually. reset() is called at every FSM state-entry point already identified for this class of per-state variable (§12.8.9’s lockcentered and this session’s dualconfirmt0, §12.9.1), and additionally at drive()’s two unconditional safety returns (camalive/depthalive both false, §12.8.9), which are not FSM transitions and so are not covered by the state-entry resets, a blocked robot with sustained nonzero error, or a depth/vision dropout, is exactly the scenario the integral-clamp exists to bound, and letting stale integral/derivative memory survive a safety stop is avoided by the same blanket-reset philosophy the safety gates themselves already use (they stop all motion on either signal’s loss, not just the sub-behaviour that needed it).

PID feedback loop, drawn against the obstacle-governor loop’s actual signals (§12.9.2.2). The same five blocks, setpoint, error-summing junction, parallel P/I/D branches recombined at a second junction, plant, sensor, instantiate all three loops in this section; only the setpoint, error signal, and gains change.

#### Obstacle-proximity speed governor

The one loop with no proportional predecessor to inherit a gain from. The error is corridor clearance measured against a slowdown threshold set well above the existing hard-trip distance,

e[n] = dcorridor[n] − OBSTACLE_SLOWDOWN_MM,  OBSTACLE_SLOWDOWN_MM=1500 mm,

where dcorridor is the same 5th-percentile, three-tile-ROI corridor distance the binary trip already consumes (§12.8.9). Rather than an additive velocity correction, the controller output scales the commanded forward speed multiplicatively,

scale[n] = clamp​1+u[n], smin, 1,  lin[n] = ADVANCE_LIN×scale[n],

with u[n] itself already clamped to [smin−1, 0] (Eq. [eq:pidout], umax=0 so a clear corridor, e[n]>0, contributes no slowdown at all, only e[n]<0 can reduce speed), making Eq. [eq:obstscale]’s outer clamp defensive rather than load-bearing. Gains: Kp=1.2×10−3 mm​−1, deliberately steeper than the value that would reach the floor smin=0.3 exactly at the existing 600 mm trip distance (Kp*=0.7/900≈7.8×10−4), so the governor saturates before the hard trip, favouring margin on an untuned first deployment over a tight fit; solving Eq. [eq:obstscale] for the corridor distance at which the floor is first reached,

dfloor = OBSTACLE_SLOWDOWN_MM − smin−1Kp = 1500−−0.71.2×10−3 ≈ 917 mm,

roughly 300 mm of margin ahead of the 600 mm trip, at ADVANCE_LIN=0.2 m/s about 1.6 s of additional reaction time purely from this loop, before the binary trip’s own reaction applies. Ki=0: the blocked-with-sustained-error state this loop can legitimately sit in (an obstruction, or the dual-confirm grace hold, §12.9.1) is precisely the scenario Eq. [eq:integral]’s clamp exists to bound with no compensating benefit, a stable partial slowdown while skirting an obstacle is an acceptable steady state, not an error to be eliminated, and the existing stuck-detector (STUCKMINDISPM/STUCKDISPWINS →enterescape, §12.8.9) already owns “not making progress” as a failure mode. Kd=0.15, using Eq. [eq:dfilt]’s internal filter directly, as noted above the one signal in this section with no upstream EMA. The binary trip and reactive pivot (§12.8.9) are untouched by this loop, deliberately: it only makes the robot slow down earlier, it does not gate, replace, or weaken the existing hard stop, and it applies at both loci depth-covered forward advance already occurs, PATROL and SPIRAL.

#### Patrol steering

Error is the same exponential moving average of the “open path” bearing already computed for patrol steering (§12.8.9, α=0.3), setpoint zero (drive straight at it):

e[n] = θopen[n],  ang[n] = clamp​u[n], −OPEN_STEER_MAX, OPEN_STEER_MAX,

with Kp=OPEN_STEER_KP=0.8 s​−1, the field-tuned proportional gain this loop already used, reused unchanged rather than re-derived; Ki=0, the target corridor is recomputed fresh every tick, there is no fixed setpoint an accumulated integral could meaningfully drive toward, and a nonzero integral here would also compete with the independently-tuned yaw-drift trim already applied once, at the single publish point (§12.8.8), risking two uncoordinated bias correctors fighting the same actuator; and Kd=0.1, damping fast bearing swings (a doorway passing through the field of view) without duplicating the anti-zigzag EMA already upstream of e[n] itself. Output clamp unchanged at ± OPEN_STEER_MAX=±0.25 rad/s.

#### Beacon centring: two instances

Two independent controllers, not unified into one, the error units differ (a normalised fraction while aligning in place versus a raw pixel offset while centred and advancing) and unifying them would require converting one loop’s error into the other’s units, more surface area than a mechanical P→PID swap warrants on a currently-deployed FSM.

Align-in-place (rotate to centre the beacon before advancing):

e[n] = cx[n]−w[n]/2w[n]/2,  Kp=LOCK_ALIGN_SPEED=0.25 s−1,

cx the detected target’s horizontal pixel coordinate and w the frame width, reusing the loop’s existing field-tuned gain. Ki=0.05, small and tightly clamped (Imin/Imax bounding KiI[n]’s own contribution to at most a fifth of the output range), justified the same way as the yaw-drift trim (§12.8.8): a fixed optical-axis offset is physically plausible given this rover’s already-documented asymmetric wheel behaviour (item 1, Table [tab:openitems]), and exposure is time-bounded by LOCKTIMEOUT/LOCKSTALLS (§12.8.9), limiting how long any windup risk can accumulate even under the clamp. Kd=0.06, conservative. Output clamp unchanged at ± LOCK_ALIGN_SPEED.

Approach while centred (advance while holding the target centred, used identically at both of §12.9.1’s arrival branches):

e[n] = cx[n]−w[n]/2  (pixels),  Kp=TARGET_KP=4×10−3 rad s−1px−1,

gain reused unchanged. Ki=0: the align loop above already removes any fixed bias before lockcentered ever becomes true, so residual error here should already be small and transient (vibration, minor drift), better damped than integrated, and this loop shares the exact windup exposure already flagged for the obstacle governor (obstruction holds, dual-confirm grace holds, §12.9.1) with no corresponding argument for keeping an integral term. Kd=0.05. Output clamp unchanged at ±0.3 rad/s. The existing centred/not-centred hysteresis band (±8 % entry, ±14 % exit, §12.8.9) governs the boolean state transition, not the steering command, and is untouched by either substitution.

#### Validation status

All three loops were deployed live via the same process-respawn mechanism used throughout this campaign (§12.8.7), in ascending order of collision-relevant stakes, patrol steering first (bounded by an already-clamped ±0.25 rad/s in open terrain, easiest to validate visually), then beacon centring (bounded consequence: a worse approach angle at worst, with §12.9.1’s safety nets underneath unchanged), the obstacle governor last, deliberately additive over the unmodified binary trip rather than a replacement, so that even poorly-chosen day-one gains cannot remove existing collision protection, only shift when deceleration begins relative to the trip. No live gain is derived from a plant model of this rover (no measured mass, wheel-slip transfer function, or camera-loop latency budget feeds any Kp/Ki/Kd above); every gain is either reused unchanged from a field-tuned predecessor or, for the one loop with no predecessor (§12.9.2.2), chosen from the same geometric margin argument already used elsewhere in this campaign for untuned first deployments (§12.8.9’s DEPTHALIVETIMEOUT margin against OBSTACLEDISTMM is the same pattern). None of the three loops has yet been exercised over a live AUTO traverse under ordinary network conditions to confirm the intended qualitative effect, visibly smoother patrol heading, a visibly earlier and smoother deceleration on wall approach, without introducing new oscillation the prior single-gain proportional terms did not have; this session’s network conditions (§12.9 was conducted throughout an extended, unresolved building-WiFi outage, itself a live instance of item 13’s driver fault) did not permit it. Table [tab:openitems] records this alongside a second, structural risk specific to §12.9.1: whether this beacon’s physical LED/checkerboard layout ever produces a pose at which both cues are simultaneously valid has not been confirmed, if it structurally cannot (checkerboard readable only from one face, LEDs only bright enough at a glancing angle that face never presents), the grace window will expire on every attempt and no beacon will register autonomously at all, a possibility to rule out on the bench before it is discovered in the field.

### Item 10 recurs, this time behind hours of network misdiagnosis

Later the same day, the cockpit’s VINS↔MINS switch stopped working: poseselector refused every attempt with cannot switch to MINS: no message received on it yet, and MINS’s own log repeated [IW-Init]: Waiting for collecting IMU(490 < 3) and wheel data(0 < 3) without ever progressing, IMU accumulating normally, wheel data stuck at exactly zero. This is, byte for byte, the same symptom already diagnosed and named in §12.8.1 the day before. It was not recognised as such until late: six environmental interventions were exhausted first, each plausible on its own and each ruled out in turn rather than assumed away, in order: (i) two attempts on the direct same-subnet Wi-Fi path (§12.8.4); (ii) a deliberate switch to the WireGuard tunnel (§12.2) for two further attempts, on the reasoning that §12.8.3’s roaming-endpoint fault could not be ruled out from the symptom alone, during which the tunnel itself was directly observed to drop for ≈15 s and self-recover, a milder instance of item 12’s still-open failure mode; (iii) a deliberate stationary hold, ruling out roaming/motion as the proximate cause; (iv) a full reboot of both the ground-station PC and the rover, ruling out any accumulated process or registration state. All six failed identically. A process that fails the same way regardless of an independent variable is evidence against that variable, not for trying it a seventh time: the next step taken was to stop varying the network and inspect the ROS graph state of the specific node already named in item 10. rosnode info /firmwaremessageconverter showed it subscribed only to /firmware/imu, not to /firmware/wheelstates or /firmware/wheelodom, despite its own loaded firmwaremessageconverter.yaml declaring wheeljointnames that only make sense if it publishes /jointstates from wheel data; it was not crashed (still registered, still consuming CPU) and not restarted since the previous incident, consistent with item 10’s standing recommendation, an output-rate probe alongside presence checks, never having been implemented. The fix was identical to §12.8.1: a rosmon stop/start cycle of firmwaremessageconverter alone. /jointstates resumed at ≈19 Hz within seconds, [IW-Init] smooth success followed within the same cycle, and the cockpit switch succeeded on the next attempt. Table [tab:openitems] item 10 is updated rather than duplicated: the same blind spot cost approximately two hours of network-focused misdiagnosis on its second occurrence, the clearest evidence yet that its fix is worth the effort.

A plausible trigger for the original subscription loss, not the same claim as its cause, surfaced independently while investigating: journalctl -u leo recorded 20,295 occurrences of wrong checksum for topic id and msg on serialnode (the Pi ↔ CORE2 UART link, 250,000 baud) in a two-hour window, interleaved with explicit lost sync and Mismatched protocol version warnings, a rate (≈2.8 s​−1 sustained) consistent with a serial-desync IMU firmware freeze already known from earlier in this campaign but not previously observed at this severity. Whether this specific desync burst is what caused firmwaremessageconverter’s wheel-data callback to fail without crashing the node is plausible, matches the timing, and is recorded here as the leading hypothesis, but was not proven: doing so would require a timestamped correlation between a specific desync event and the node’s subscription state that this session did not capture. Table [tab:openitems] records it as a new item (19) rather than folding it into item 10, since the watchdog fix (detect dead output) and the hardware fix (stop the desync) are independent and both still needed.

### A first migration off legacy tf1: MINS ported to tf2_ros

Auditing MINS’s build for a specific, requested criterion, whether it uses tf2ros or the deprecated ROS1 tf package for its transform broadcasts, found the latter: minsws/src/MINS/mins/package.xml declared an unconditional tf dependency, and cmake/ROS1.cmake (the branch actually compiled on this Noetic/catkin rover; a parallel cmake/ROS2.cmake exists upstream but is never built here) compiled ROSPublisher.cpp, ROSHelper.cpp, and SimVisualizer.cpp against tf::TransformBroadcaster and tf::StampedTransform. A tf2ros-based implementation of the identical logic already existed in the same upstream repository (ROS2Publisher.cpp, ROS2Helper.h), unbuilt on this platform; rather than devise an independent translation, all three ROS1 files were ported field-for-field to mirror it, tf::StampedTransform replaced by geometrymsgs::TransformStamped constructed member-by-member, the broadcaster retyped to tf2ros::TransformBroadcaster, and package.xml/cmake/ROS1.cmake updated to depend on tf2ros instead of tf. The change is scoped deliberately narrowly: MINS’s TF output is documented elsewhere in this system (mins.launch, §12.2) as visualisation-only, isolated to /tfmins and never on the pose-fusion critical path (poseselector consumes /mins/imu/odom directly, a plain navmsgs/Odometry, never via TF), so no file touched here can affect the estimator’s actual filter state. Rebuilt clean (catkin build mins, zero warnings) and verified live: the running process was replaced (kill, automatic respawn), /mins/imu/odom continued publishing, /tfmins carried a correctly normalised global→imu transform, and a VINS↔MINS switch cycle through poseselector still succeeded. One unrelated fact surfaced during this same verification: ovmsckf (VINS) was found not running at all, no process, removing the VINS fallback until restarted; this migration neither caused nor fixes it, and it is logged as a new open item (20) rather than folded into this one.

## Field session, July 23: Return-to-Base rebuilt, a source-selection interface simplified after two iterations, and an initial-divergence guard

### Return to Base: world-frame preservation, and full-trajectory retracing instead of a straight line

Two independent defects in Return-to-Base surfaced during an operator session and were fixed the same day.

##### The resetcoords() world-frame bug.

The cockpit’s manual Reset 0,0,0 button (data-cmd="reset") called resetcoords(), which zeroes local odometry via a hardware reset without first re-anchoring worldoriginx/y/heading to the pre-reset world position. Its sibling resetcoordslocal() (introduced §12.8.1’s era for the AUTO-mode beacon-visit resets) does this correctly: it reads (wx,wy)=_get_world_pos() before the reset and sets

world_origin←(wx,wy),  world_heading←ψraw

so that after the local pose zeroes, _get_world_pos() (which computes world_origin+R(world_heading)⋅poselocal) still evaluates to the same physical point. resetcoords() skipped Eq. [eq:worldreanchor] entirely: after a manual reset mid-mission, the backend’s own belief of “world (0,0)” silently snapped to wherever the robot physically stood at reset time, not the true mission start. RTB, which navigates toward that reference, would then target a corrupted point with no error surfaced anywhere. Fix: resetcoords() now performs the identical re-anchoring as resetcoordslocal() before the hardware reset fires.

##### Full-trajectory retracing.

Independently, RTB itself only ever implemented a straight line to world (0,0) (§12.8.9), with a blind-in-place rotation whenever the bearing error exceeded a threshold — including, structurally, at RTB’s own start, since the target is normally near-opposite the direction of travel. Operator feedback after a live trial asked for two changes: retrace the path actually travelled rather than cut through unexplored space, and drive in reverse rather than turn around. Both were implemented together, and the second turns out to reinforce the first: a rover known to lose localisation quality while pivoting (§12.4, wheel slip during in-place rotation) accumulates less heading error on the way home if it never pivots at all.

A persistent world-frame breadcrumb trail T={(wx(i),wy(i))} is now recorded on every odometry callback, sampled at a coarser spacing than the display trajectory (RTBTRAILSPACING=0.25 m vs. the map’s 0.02 m) and, unlike the map trajectory, never cleared by an odometry or LED reset — only by clearmap() or a completed RTB run — since it lives in the same world frame Eq. [eq:worldreanchor] keeps continuous across those resets. On RTB start, the queue of waypoints to visit is T reversed (most recent first) with a guaranteed final anchor at the true origin:

W= p∈reverse(T):p−pnow>RTB_DIST_TOL  ‖ (0,0)

Each waypoint wk=(tx,ty) is consumed identically to the original single-target logic (same distance-based speed ramp, same overshoot detector), but the steering error is now computed against the robot’s reverse heading rather than its forward one:

θerr=atan2​sin(β−θrev),cos(β−θrev), β=atan2(ty−wy, tx−wx), θrev=θ+π

with lin<0 throughout and no in-place-rotation branch at all: the steering correction ang∝θerr/π, clamped to RTBANGSPEED, is authoritative over the full error range. Reaching a waypoint within tolerance advances to the next one within the same control tick (no stop-and-resume between legs); the final arrival re-triggers the existing safe-stop path unchanged. Not yet validated: a live multi-waypoint retrace over a real multi-beacon trajectory — the mechanism has been exercised on short manual runs only, logged as a new open item (21).

### Pose-source selection: two iterations to a simpler, honest interface

Worth recording as process, not just outcome: the operator asked to be able to pre-select VINS before driving (since it needs sustained motion to initialise, unlike MINS’s static init), so a switch requested while the target source had never published would be armed and applied automatically the instant it did — implemented directly in poseselector.py (switchto() storing a pending source, fulfilled inside onodom() the moment that source’s first message arrived, with a matching cockpit affordance: a blinking, dashed button state and a second click to cancel). It worked as specified, verified live end-to-end (arm → posesourcepending topic reflects it → cancel returns it to empty, active source undisturbed throughout).

It was reverted the same session. Operator feedback after using it: the extra implicit state (armed vs. active vs. cancelled) made it harder, not easier, to know what the system was actually doing — the opposite of the transparency this cockpit has otherwise been built around all campaign. The interface was simplified back to the original contract: a click attempts the switch immediately, succeeds or fails outright, with the failure surfaced through the pre-existing /mission/log → cockpit log channel (unchanged since §12.6) — no armed state, no automatic later application. posesourcepending and its topic, publisher, and cockpit wiring were removed in full rather than left dormant. Lesson, stated plainly: an operator-transparency requirement (“let me see what’s about to happen”) is not satisfied by hiding latency behind automation — it is satisfied by making the current true state impossible to misread. The two are easy to conflate when designing the feature and easy to tell apart once actually operating it.

### An initial-position divergence in VINS, and a new plausibility guard

While re-testing the switch after §12.10.2’s simplification, VINS produced enough motion to publish for the first time that session, and the switch to it succeeded (message: "switched to VINS"). Its raw reported position at that first message was (27.5, −25.7, −4.6) m — physically impossible for a rover confined to a single room. The existing plausibility guard (introduced 2026-07-13 alongside the SE(3) switch correction, Eq. [eq:se3]) did not catch this: it bounds the speed between two consecutive samples of the currently active source, a check with nothing to compare against on a source’s very first message. The SE(3) switch correction itself (Eq. [eq:se3]) absorbed the offset on the fused output — verified directly: /robotposefused read (−0.37,−0.37,−1.665) m immediately after the switch, plausible in x,y — but the anomalous z residual (−1.665 m for a ground rover) shows the correction papered over a bad estimate rather than fixing it: continuity of output is not the same guarantee as correctness of the source.

A second, complementary guard closes this gap: a source’s first-ever message is checked against its own claimed origin before being accepted at all,

$$\begin{equation}
      \lVert p^{(0)} \rVert = \sqrt{x_0^2+y_0^2+z_0^2} \; \overset{?}{>} \;
      \texttt{MAX\_INITIAL\_JUMP\_M} = 5.0\text{ m}
      \label{eq:initguard}
    \end{equation}$$

rejecting (not caching) the sample if so, logged loudly, leaving the source in the same “never received” state switchto() already refuses to select — no new failure mode, just an earlier one. 5 m is a bench choice (this campaign’s indoor spaces are a few metres across; a freshly-initialised VIO/VIO+wheel estimate has no legitimate reason to report a double-digit-metre displacement from its own origin) and is a first cut, not a derived bound — logged as open item (22): characterise against a real distribution of clean vs. divergent VINS inits rather than a single observed incident.

## Field session, July 24: a blind reversal, a silent approach, and a deliberate simplification of reactive avoidance

Four independent gaps in the autonomy stack’s obstacle handling were found and closed the same day, triggered by two operator collision reports during live AUTO-mode testing; a fifth change, requested independently, adds a bounded speed increase to manual teleoperation. The session also produced a public-facing hardware reference (§12.11.1) whose cross-validation exercise foreshadows the day’s central methodological theme: every one of the day’s software fixes was found by trusting a live measurement over an assumption the code had been carrying silently.

### Robot documentation and a second manufacturer-vs-as-built divergence

A new page of the operator platform (robotdescription.html) was built to host an interactive hardware reference: a WebGL viewer (STLLoader/OrbitControls) rendering the vendor CAD model directly in-browser, and a full transcription of FictionLab’s official Leo Rover 1.9 specification, fetched and verified against the manufacturer’s current documentation rather than carried forward from the platform’s original (2026-06) marketing copy.

##### A geometric cross-check.

The CAD file (binary STL, 1,162,904 triangles, 58.1 MB) was parsed directly to recover its axis-aligned bounding box, giving an independent check of the vendor’s quoted envelope:

(Δx, Δy, Δz)measured=(445.8, 424.6, 303.4) mm,  (Δx, Δy, Δz)vendor=(445, 424, 303) mm,

a per-axis relative error of 0.18%, 0.14%, 0.13% — the model is the as-sold envelope to a fraction of a millimetre, not a generic placeholder mesh. Several other quoted figures were corrected in the same pass by the same discipline (trust the current source over the carried-forward one): unladen mass (4.6→≈7 kg), rated top speed (≈1.0→≈0.4 m/s — the platform’s software speed ceiling, MAXLINVELDEFAULT=1.0 m/s, had been carried into the specification card as if it were the physical top speed; it is merely the largest value the velocity-limit slider is permitted to reach, see §12.11.6), battery chemistry (Li-Ion 3S2P 6.8 Ah at 10.8 V nominal, not the previously listed 3S LiPo at 11.1 V), and chassis geometry (wheelbase 295 mm / track 354 mm, not the single, conflated 360 mm figure previously shown).

##### A second as-built divergence, same pattern as the wheels.

One correction could not simply replace the old figure. FictionLab’s current specification lists a Raspberry Pi 5 (Broadcom BCM2712, 4 GB LPDDR4X) as the 1.9-line’s onboard computer. Direct inspection of the deployed unit (/proc/device-tree/model and free -h over the existing SSH channel) instead reports a Raspberry Pi 4 Model B (Broadcom BCM2711, 8 GB LPDDR4) — a different board generation and, incidentally, a different RAM size than either the vendor page or this project’s own prior documentation. This is structurally the same situation as §12.4’s mecanum-wheel discovery: a manufacturer’s current default configuration does not describe a specific, already-deployed unit, and the only way to know which is true is to ask the hardware directly rather than the datasheet. Both facts are now recorded side by side on the new page rather than the newer one silently overwriting the measured one. The LeoCore controller board’s exposed interfaces were transcribed verbatim from the same official source (Table 12.4) for completeness, alongside the mecanum discovery’s own numbers (§12.4) already carried by the platform elsewhere.

LeoCore controller board interfaces (FictionLab official specification, transcribed verbatim July 24). This project’s software and documentation refer to the same board as “CORE2” throughout; both names denote the same physical controller.

| Interface | Specification |
| --- | --- |
| Power input | DC 2.1/5.5 mm barrel jack, 8–30 V DC, 10 A fast-blow fuse |
| Motor outputs | 4× PWM H-bridge (A/B/C/D), ±2.8 A peak per channel |
| LED output | 5 V / 10 mA |
| Fan connector | 5 V / 1 A |
| UART / GPIO port | 3.3 V logic UART + GPIO breakout; also powers the Raspberry Pi |

### A mission-critical incident: Return-to-Base’s blind reversal

An operator report during live testing (“it drives into obstacles”) initiated a code audit of every branch inside drive() that can command nonzero forward speed in AUTO mode. Cross-referencing that audit against the mission’s own event log (autoentries.json) turned a plausible hypothesis into a demonstrated one.

##### The audit.

RTB (§12.10.1) is explicitly exempted from the camalive/depthalive gate that otherwise blocks all forward motion in AUTO the instant either sensor stream goes stale (§12.8.9) — by design, since RTB navigates by fused pose, not vision, and should not be held hostage to a WiFi-crossing image stream it does not need for localisation. But the same exemption was found to have a second, unintended consequence: nothing inside the RTB branch itself consulted obstacleflag, the corridor-depth signal, or any displacement-based stuck detector, at all. RTB could — and, the log below shows, did — reverse continuously at up to RTBLINSPEED =0.20 m/s over a route that can run to RTBTRAILMAX× RTBTRAILSPACING≈500 m, with zero obstacle detection of any kind, sensor-live or not. This is materially different from the platform’s one other blind manoeuvre, ESCAPEBACKLIN: that reversal is deliberately bounded to 0.2 m over 2 s specifically because the D455 is forward-facing and cannot observe the rover’s own direction of travel while reversing (§12.8.9) — the same physical constraint applies to RTB, but RTB’s reversal was, before this session, unbounded in both duration and distance.

@lll@ Timestamp & Event & Outcome
-22 15:56:50 & RTBSTART (8.13 m) & abandoned, no completion logged
07-22 16:08:59 & RTBSTART (1.21 m) & abandoned, no completion logged
07-23 12:41:21 & RTBSTART (5.92 m) & abandoned, no completion logged
07-23 12:49:05 & RTBSTART (5.10 m) & abandoned, no completion logged
07-23 12:53:43 & RTBSTART (4.03 m) & abandoned, no completion logged

-24 (session) & RTBSTART (47 waypoints, 10.94 m) & RTBOBSTACLE: forward depth flag, correctly halted

Table [tab:rtblog]’s first five rows are five consecutive RTB invocations, across two days, none of which produced the RTBCOMPLETE log entry that a successful arrival always emits — each was silently followed by a fresh manual takeover and a new attempt. This is exactly the operational signature the audit predicted: a run that does not fail loudly, does not stop, and does not arrive, consistent with repeatedly reversing into something and being manually rescued rather than the software ever recognising a problem.

#### The fix: a two-tier defence in the one direction with no sensor coverage

Two independent guards were added, additive to the existing waypoint-driving logic and to each other, in order of increasing severity of response.

##### Tier 1 — a forward obstacle check, for free.

obstacleflag is already computed continuously by the depth callback regardless of AUTO sub-state (§12.8.9); RTB previously just never read it. Reading it costs nothing new and adds partial coverage precisely where the constraint above does not bind absolutely: RTB’s steering correction changes the robot’s heading while reversing (Eq. [eq:rtbreverse]), so the forward-facing camera does sweep across space the rover is not currently backing into, but may be about to swing toward as the correction continues. On a trip, RTB now performs a full, unconditional stop (mode→MANUEL, five repeated zero-velocity publishes, §12.8.9’s safestop()) rather than the reactive pivot PATROL/GOTO_BEACON/SPIRAL use: RTB’s own design already forbids in-place rotation (§12.10.1, precision), so the two behaviours available to those other states — turn away, or push through slower — are both closed off, leaving stop and hand back to the operator as the only response that does not risk making a contact already suspected worse.

##### Tier 2 — an odometry-inferred stall detector, the only signal available in the blind direction.

No sensor observes the space behind a reversing rover on this platform. The one indirect proxy available is motion itself: if a nonzero reverse velocity is commanded and the fused (or, per §12.10.1, wheel-odometry-fallback) position does not change, something is physically resisting the wheels. A reference position and timestamp are latched, then re-tested every tick:

p,t←(p[n], t[n])if p undefined, or p[n]−p>dstallp, totherwise,

with $d_{\text{stall}} = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_STALL\_MIN\_DISP\_M} = 0.03$ m — the reference position resets every time the rover has genuinely moved by more than this margin, so the check is really “how long has it been since we last moved a meaningful amount”, not a single fixed window from RTB’s start. A trip fires when that “meaningful amount” has not occurred for too long:

$$\begin{equation}
    t[n] - \hat t \;>\; T_{\text{stall}} \;=\; \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_STALL\_WINDOW\_S} = 2.0\text{\,s}
    \;\Longrightarrow\; \text{stop.}
    \label{eq:rtbstalltrip}
    \end{equation}$$

dstall=3 cm sits comfortably above the platform’s own measured positioning noise floor at rest (σx=7.5–8.8 mm, σz=49.7 mm post-extrinsic-correction, Table 12.3) — a real stall reads as persistently below the threshold, not as noise crossing it in either direction — while remaining small next to the ≈6 cm a healthy RTBCRAWLSPEED=0.03 m/s reversal covers over the same 2.0 s window, so a genuinely moving rover clears the threshold with roughly 2× margin at the platform’s slowest legitimate speed and considerably more at RTBSLOWSPEED/RTBLINSPEED. On trip, the same unconditional stop as Tier 1 fires, logged and distinguished (RTBSTALL vs. RTBOBSTACLE) in the mission autolog for post-hoc diagnosis.

#### A graduated response ahead of the hard stop

Both tiers above are binary: full speed until they trip, then an instantaneous stop — precisely the “bang-bang” shortfall §12.9.2 diagnosed and fixed for the forward direction in the July 22 session. A second operator request the same day (“react a little before it happens”) asked for the equivalent graduated behaviour here. The forward half of this is free: RTB’s commanded speed is now also scaled by the existing continuous obstacle governor, Eq. [eq:obstscale], unchanged and reused rather than re-derived, extending its coverage from PATROL/SPIRAL (§12.9.2.2) to a third locus.

The reverse half needed a new signal, since Eq. [eq:obsterror]’s input (corridor depth) does not exist facing backward. The construction generalises Eqs. [eq:rtbstallref]–[eq:rtbstalltrip] from a binary trip into a continuous ratio, evaluated on a shorter rolling window Tresist=0.6 s <Tstall so it reacts before the hard stop would fire:

ρ[k] = p(tk)−p(tk−1) / (tk−tk−1)|scmd(tk−1)|,

the ratio of observed reverse speed over the window to the speed actually commanded during it (not the nominal phase speed — Eq. [eq:rtbfinalspeed] below feeds this loop’s own output back into next window’s denominator, so a partially-throttled window is judged against what it was really asked to do). ρ≈1: driving freely. ρ→0: full resistance, the stall regime Eq. [eq:rtbstalltrip] will eventually catch if it persists. A piecewise-linear map converts the ratio into a speed scale, mirroring Eq. [eq:obstscale]’s clamp-and-floor shape but built directly rather than as a PID output, since there is no natural “error” here to feed a controller — only a bounded ratio:

σresist(ρ) = sminρ≤ρlosmin+ρ−ρloρhi−ρlo (1−smin)ρlo<ρ<ρhi1ρ≥ρhi,  ρlo&=0.35ρhi&=0.75smin&=0.35

plotted in Figure 12.7. Both scales apply multiplicatively to the distance-based speed ramp already in place (§12.10.1):

$$\begin{equation}
    \dot s_{\text{cmd}}[n] \;=\;
    \begin{cases}
    -\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_CRAWL\_SPEED} \\
    -\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_SLOW\_SPEED} \\
    -\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}RTB\_LIN\_SPEED}
    \end{cases}
    \;\times\; \sigma_{\text{resist}}(\rho[n]) \;\times\; \text{scale}[n]\big|_{\text{Eq.~\ref{eq:obstscale}}}
    \label{eq:rtbfinalspeed}
    \end{equation}$$

Neither scale can reach zero (smin=0.35, $\text{Eq.~\ref{eq:obstscale}}$’s own floor 0.3): the two hard trips (§12.11.2.1) remain the only path to an actual stop, by design — the graduated layer only makes the robot slow down earlier, exactly as §12.9.2.2 already established for the forward case, never gating or replacing the safety net beneath it.

The RTB resistance governor, Eq. [eq:rtbresistscale]: a ratio of observed to commanded reverse speed below 0.35 floors the scale at 0.35× nominal (the regime Eq. [eq:rtbstalltrip]’s hard stop will eventually trip if sustained past 2.0 s); above 0.75 the resistance is judged negligible odometric noise and no slowdown applies; between the two, linear interpolation.

RTB’s per-tick decision order after §12.11.2.1–§12.11.2.2: both hard stops are evaluated before the graduated governor, and the graduated governor before the pre-existing waypoint logic — a trip at either diamond returns immediately, publishing zero velocity, without reaching the speed ramp beneath it.

### A second, narrower gap: approach without a metric distance

Live telemetry captured during the same testing session (corridormm: 345, detection.checker: false, maplandmark.valid: false — an LED-only detection, no AprilTag, no confirmed checkerboard) pointed the audit at LOCK’s approach logic specifically. §12.9.1’s dual-confirmation approach state has two siblings, distinguished by whether a metric distance to the target is available (AprilTag range, or a checkerboard-derived estimate) or not (LED centroid only, the common case at range before the checkerboard comes into focus). The first sibling already carried an obstruction check predating this session — “the target is reported beyond TARGETSTOPM but depth sees something closer directly ahead: a contradiction, treated as an obstacle, not the beacon” — inherited from the July 22 hardening that closed a near-identical gap in the same state (§12.9.2’s motivating incident). The second sibling — reached whenever no metric distance exists at all, which is precisely when the robot has the least independent information about what lies ahead — carried neither that check nor Eq. [eq:obstscale]’s graduated governor: it advanced at an unscaled ADVANCELIN until the corridor distance itself crossed a separate, hard-coded arrival threshold (700 mm), the literal bang-bang behaviour §12.9.2 had already been written to eliminate everywhere else.

Every AUTO-mode branch that can command a nonzero forward speed, audited July 24. “Hard” = a discrete obstacleflag check gating the branch; “Graduated” = Eq. [eq:obstscale] applied to the commanded speed.

| Branch | Hard (before) | Graduated (before) | July 24 change |
| --- | --- | --- | --- |
| PATROL (straight advance) |  |  | none (reference case) |
| LOCK, distance known |  | — | graduated governor added |
| LOCK, no distance known | — | — | both added (the gap) |
| SPIRAL |  |  | none |
| GOTO_BEACON |  | — | graduated governor added |
| RTB (reverse) | — | — | both added, §12.11.2 |
| ESCAPE (reverse) | n/a (bounded, §12.11.2) | — | unchanged, deliberately |

Table 12.5 was produced by grep-auditing every lin = ADVANCELIN-family assignment in drive() against the two protections, rather than trusting memory of which branches had been touched by §12.9.2’s earlier pass — the same discipline as §12.11.1’s hardware check, applied to code instead of a datasheet. The fix mirrors the sibling branch exactly (obstruction → map the obstacle, enter AVOID, abandon the approach) with the graduated governor layered on top, bringing every forward-advancing branch in the FSM to the same two-tier standard RTB now also meets.

### A statistical blind spot in direction selection: median versus percentile

A separate operator report (“instead of avoiding the obstacle it gets closer to it”) could not be explained by a sign error: the pivot direction convention (avoiddir⋅UTURNSPEED, positive = counter-clockwise = left, matching the “open path” bearing convention already verified consistent at §12.8.9’s original definition) checks out on inspection, and no image flip exists anywhere in the depth pipeline (imagetransport/republish decodes the compressed stream without any geometric transform). The actual weakness was statistical, in the two signals that choose which side is freer.

##### Why a median can be blind to a real obstacle.

Both the left/right depth comparison (§12.8.9, the AVOID fallback when no open-path sector aims the pivot) and each open-path sector’s own value (§12.8.9) were computed as the median of valid depth pixels within their region. Model a region’s valid pixels as drawn from a two-population mixture: a fraction f at near range (an obstacle) and (1−f) at far range (open background), a reasonable idealisation of a thin object — a pole, a chair leg, a wall corner seen obliquely — that occupies only part of a half-image or a 10∘ sector. The q-th percentile of this mixture equals the near population’s value exactly when

q ≤ 100f,

and the far (background) value otherwise. The median is q=50: any obstacle occupying less than half the region (f<0.5) is invisible to it entirely, the statistic reports “open” with no trace of a real, physically present hazard. This is not a corner case for a forward-facing depth sensor on a rover navigating a room: a doorway edge, a chair leg, or a wall met at a shallow angle are exactly the objects that occupy a minority of a half-image or a narrow sector while remaining fully capable of a collision. Both signals feeding this exposure directly steer the rover — the open-path bearing continuously, in patrol (before §12.11.5 removed that consumer), and the left/right comparison at every reactive pivot — so a median-masked obstacle was not merely undetected, it was actively steered toward, matching the report precisely.

A region whose valid depth samples split between a near obstacle (fraction f≈0.2 here) and a far background. The median falls entirely inside the background population — the obstacle contributes zero signal to it (Eq. [eq:percentilebias], q=50>100f) — while the 20th percentile still reports the obstacle’s own range, and the hard-trip 5th percentile (§12.8.9) more so.

##### The fix, and where the threshold sits deliberately.

Both computations were changed from np.median to the 20th percentile of the same valid-pixel population. Figure 12.9 frames the choice as a point on a single sensitivity spectrum already spanned by two other statistics already in use elsewhere in the same callback: the median (q=50, per Eq. [eq:percentilebias] blind to any f<0.5 obstruction, the value being replaced) and the existing hard-trip corridor statistic (q=5, §12.8.9, sensitive down to f≥0.05 but deliberately so — it is the collision trigger and must not be fooled by a sparse obstacle either). q=20 sits between them by design: an obstacle occupying a fifth or more of the steering region can no longer hide from the direction preference, while the statistic remains substantially more tolerant of single-pixel noise and small dropout clusters than the hard trigger — appropriate for a signal that only ever biases a choice, several centimetres to metres away from the q=5 statistic’s own binary stop.

### A deliberate behavioural simplification: fixed, deterministic avoidance

A third, explicit operator request replaced the campaign’s most elaborate piece of reactive behaviour with its simplest possible correct alternative, and is recorded here as an intentional design choice, not a defect fix: go straight down the corridor; on an obstacle, shift exactly 35∘ left; if still obstructed, shift another 35∘ left.

##### What was removed.

Two mechanisms built up over the campaign (§12.8.9’s “vacuum-robot” coverage strategy, §12.9.2’s continuous steering) are no longer consumed, by request: the open-path attraction that continuously biased patrol heading toward the deepest visible sector (Eq. [eq:patrolerr], §12.9.2), and the AVOID pivot’s own choice of aimed bounce toward that sector or, failing that, a random bounce uniformly drawn from [45∘,135∘] toward whichever side read freer. Both consumers are removed at their call sites only; the underlying sensor computation (bestopenpath(), its continuously-updated depth sectors, and the tuned PID instance that drove Eq. [eq:patrolerr]) is left in place, dormant rather than deleted, should the coverage strategy be reinstated later.

##### What replaces it.

The pivot direction is now a constant,

$$\begin{equation}
    \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}\_avoid\_dir} \equiv +1 \quad(\text{left, by the same}\ +z\ \text{CCW convention already established}),
    \label{eq:avoiddir}
    \end{equation}$$

and the pivot magnitude a fixed angle rather than a computed or drawn one,

$$\begin{equation}
    \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}AVOID\_TURN\_RAD} = 35^\circ \;\;(\text{was } 90^\circ\text{, before that a } [45^\circ,135^\circ]\text{ random draw}).
    \label{eq:avoidfixed}
    \end{equation}$$

Patrol advance carries no steering term at all (ang≡0 until an obstacle trips §12.8.9’s existing binary detector): the rover’s heading is now piecewise-constant, changing only in discrete 35∘ increments at a detected obstacle, never continuously. Because the same reactive cycle re-enters enteravoid() on any renewed detection, an obstacle still present after one shift produces another, identical, cumulative shift:

θn = n×35∘,  n=1,2,3,…

until either the corridor clears or the pre-existing attempt counter (§12.8.9, AVOIDMAXTRIES=3) is exceeded, at which point the unrelated historical fallback — a full 180∘ U-turn — takes over unchanged. At the boundary, n=3 consecutive 35∘ shifts accumulate to 105∘ of net heading change before that fallback can engage (Eq. [eq:avoidcumulative]), which is worth stating plainly as the direct trade-off of a fixed, unconditionally-left policy: a corridor obstructed specifically on the left forces the rover through up to three 35∘ turns toward the very side that is blocked before the U-turn fallback rescues it, where the discarded “freer-side” comparison (§12.11.4) would previously have picked the open side immediately. The operator’s stated priority — a pivot whose direction and magnitude are always predictable and always easy to reason about from a log line, over the discarded strategy’s ergodic room-coverage benefit (§12.8.9) — is recorded here as the explicit rationale for accepting that trade-off, and the scenario it is most exposed in (a left-blocked dead end) is logged as a new open item (Table [tab:openitems], item 23) pending a live trial.

### Manual boost: a bounded control-authority addition

An unrelated operator request — faster manual driving — exposed a small architectural fact about teleoperation that had not previously been made explicit anywhere in the platform: two independent layers already governed manual speed, and only one of them was ever exposed to the operator.

linpublished = clamplinstep, −vmax, vmax

where $\text{lin}_{\text{step}}\in\{-\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LIN},0,+\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LIN}\}$ is the fixed magnitude the D-pad/arrow-key controller has always sent (LIN=0.2 m/s, ANG=0.6 rad/s, unconditionally, on every keypress) and vmax is the operator-configurable ceiling (linVelSlider/angVelSlider →/leorover/config/velocity→self.maxlinvel, clamped in publish(), the platform’s single command exit point, §12.8.9). Because linstep=0.2 m/s already sits well inside the ceiling’s default range ([0.1,1.0] m/s), raising the ceiling slider changed nothing about how fast the D-pad actually drove: the clamp in Eq. [eq:manuallayers] was already loose. A “boost mode” therefore had to change linstep itself, not the ceiling.

A toggle (button or the B key; state persists rather than requiring a held modifier, so it works identically on a touch interface) switches the step magnitude between the original values and

$$\begin{equation}
    \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}BOOST\_LIN} = 0.4\text{\,m/s}, \qquad \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}BOOST\_ANG} = 0.9\text{\,rad/s}.
    \label{eq:boostvalues}
    \end{equation}$$

$\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}BOOST\_LIN}$ is not an arbitrary multiple: it is exactly the platform’s official rated top speed (§12.11.1), so boost mode lets an operator reach, but not exceed, the manufacturer’s own performance envelope. $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}BOOST\_ANG}$ is deliberately not the same 2× multiple: §12.5.2’s ramp characterisation already measured 92–99% commanded-to-actual yaw authority across the full 0.15–0.40 rad/s range at healthy battery voltage, with no evidence offered above 0.40 rad/s — commanding meaningfully more than that would very plausibly saturate the actuator without producing a proportionally faster real turn, so the angular boost was set only modestly above the existing $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}ANG}=0.6$ rad/s rather than doubled to match the linear boost, pending a dedicated ramp measurement at boost-level commands specifically (Table [tab:openitems], item 26). Eq. [eq:manuallayers]’s outer clamp is untouched and still applies after boost, so an operator who has deliberately lowered the velocity ceiling for caution retains that ceiling exactly as before — boost can only use headroom the ceiling already permits, never exceed it.

The same evening, three further operator requests moved outside collision safety entirely: a report that the pose-source switch had stopped working, a request to close a spatial gap in beacon confirmation and extend detection out to 4 m, and a full audit of the platform’s tf2ros usage against REP 105. Each is recorded below in the same spirit as the rest of this session: what was found, what changed, and — where a fix rests on reasoning rather than a field measurement — what remains open.

### Pose-source switch: reconfirmed, then reopened on operator request

A report that MINS/VINS switching had stopped working traced to two independent, unrelated causes in sequence, each resolved by direct verification rather than by assumption. First, a full network outage: the WireGuard tunnel (§12.8.4, 10.200.0.1/.2), the parallel WiFi path and the leo-guest SSH alias were all simultaneously unreachable, while the operator PC’s own WiFi association and internet access stayed healthy throughout — isolating the fault to the rover itself being off-network, not to WireGuard specifically or to the PC. Resolved operator-side by restarting the stack. Second, after reconnection, the reported symptom persisted with a specific, reproducible service response: “cannot switch to VINS: no message received on it yet” — which is not a fault but §12.10.2’s own July 23 contract operating exactly as simplified: switchto() attempts a switch immediately and fails outright, by design, if the target source has never published. §12.3.5 already establishes why that condition is easy to hit here specifically: OpenVINS needs a forward–back jolt to render scale observable, so a rover sitting stationary after a restart can leave VINS silent indefinitely regardless of network health. The two causes present identically from the operator’s seat (“the switch doesn’t work”) but are unrelated in the code, which is worth recording precisely so the same report is not re-diagnosed from scratch next time.

Confirmed once the operator drove manually to give VINS motion: direct rosservice call tests, this time sequenced strictly (topic read → service call → topic read, rather than interleaved with other dashboard activity) reproduced clean, consistent results in both directions — “already on MINS” matching the topic before any call, “switched to VINS” once motion had occurred, matching after. An earlier, apparently contradictory read during the same diagnosis (a stale topic value momentarily disagreeing with a fresh service response) did not reproduce under the strict sequencing and is attributed to ordinary timing skew against concurrent operator interaction, not a race in poseselector.py — §12.8.1’s registered-node race was a reminder to check, not evidence one exists here. One diagnostic side-effect was disclosed to the operator as it happened: a test call left the rover switched onto VINS mid-session.

##### Reopened.

Having the mechanism confirmed working as designed did not settle the matter: the operator’s actual objection was to the design itself — refusing outright, with no path back to trying again automatically, is exactly the behaviour §12.10.2 chose after reverting the alternative. Told plainly that the alternative had already been tried and reverted once for being hard to read (“too many implicit states”), the operator asked for it back anyway, judging that trade-off differently this time. switchto() again supports arming: a click on a source with no data yet arms it rather than refusing, and onodom() applies the switch automatically the instant that source’s first message arrives, with a second click cancelling the arm. The difference from the July 23 version is deliberately not in the mechanism but in its legibility: a dedicated latched topic, /leonavigation/posesourcepending, is the single source of truth for the armed target (empty when nothing is armed), never inferred from timing or combined with any other signal; the service response and the relayed cockpit log line always name the exact outcome (“ARMED: will switch to …”, “switched to …”, “armed switch to … cancelled”, “already on …”) rather than a single generic “pose source → X” that could describe either an arm or a completed switch; and the cockpit button for the armed source carries a persistent text label, not only a colour or a blinking border, addressing the specific July 23 complaint that a state change communicated by animation alone is easy to miss or misread.

##### A restart hazard, found by triggering it.

Deploying this change required restarting poseselector live, and doing so exposed a real, previously-unnoticed defect along the way: navigationsupervision. launch’s defaultsource param was hardcoded to "VINS", directly contradicting the launch file’s own adjacent comment (“OpenVINS est l’estimateur de SECOURS (pose_source=MINS en permanence)”) and §12.3.5’s documented design intent. Because self.active is reset to this default on every process start with no regard for which source was actually healthy beforehand, the restart silently dropped /robotposefused to zero messages — confirmed live (rostopic hz showed no new messages for six seconds immediately after) — even though MINS had been publishing at 135 Hz one command earlier. Caught immediately by checking the topic rather than assuming the restart was clean, and corrected in two steps: an explicit service call restored MINS as the active source within seconds, and the launch default was changed to "MINS" to match both the neighbouring comment and §12.3.5. The launch fix does not yet apply to the live session — the parameter server still holds the value pushed at the stack’s last full launch until that launch is repeated — logged precisely as such (Table [tab:openitems], item 32). The arm/cancel round trip was verified live by direct service call in both directions; the automatic switch on a source’s first publish was verified by code inspection only (the same path §12.10.2 already exercised once, now re-entered), not yet by an actual drive-and-observe trial, logged as a further open item (item 31).

### Chasing VINS’s silent odometry: two real fixes in ovmsckf, and a log-attribution lesson

Confirming item 31 above led into ovmsckf itself. /ovmsckf/ odomimu published nothing over a sustained, directly-subscribed 18 s window, despite a correctly registered subscriber (poseselector, confirmed via rostopic info) and healthy inputs (/imu/dataclean at 84 Hz, /pc/camera/infra{1,2}/ imagerectraw at 15 Hz) — the two facts that made this worth investigating rather than re-attributing to §12.3.5’s already-documented jerk requirement outright.

##### Two genuine defects found and fixed.

ROS1Visualizer:: visualizeodometry() (ovmsckf/src/ros/ROS1Visualizer.cpp) gates its publish on VioManager::initialized(), which requires both isinitializedvio and timelastupdate!= (ovmsckf/src/core/VioManager.h). The second condition is set only inside a completed trackimageandupdate() pass — which a successfully-applied zero-velocity update bypasses via an early return (VioManager.cpp, the didzuptupdate branch) — so a platform whose ZUPT succeeds before any full update ever completes can leave initialized() permanently false even once the filter has genuinely converged, despite faststatepropagate() (what visualizeodometry() actually needs) requiring only isinitializedvio. A new accessor, VioManager:: vioalive(), returns isinitializedvio alone and is now what visualizeodometry() checks; initialized() itself and its other consumers (feature and track visualisation, which do need a completed pass) are untouched. Investigating why this still did not explain the observed silence surfaced a second, independent defect: isinitializedvio was a plain bool, written from a per-frame worker thread (ROS1Visualizer runs a multi-threaded ros::AsyncSpinner, dispatching each camera callback to its own std::thread) and read from the IMU-callback thread via the new vioalive() — an unsynchronised cross-thread read with no guaranteed visibility. Declared std::atomic<bool> instead (VioManager.h); every other read of the same flag is a plain boolean-context check or assignment and needed no further change.

##### A log-attribution mistake, corrected before it misled the fix.

Both changes were rebuilt and tested live, and the shared process log (logs/navigationmaster.log, written to by every node on this machine) kept showing UpdaterCamera::tryupdate and [IW-Init]:staticinitialization/Total initialization time lines throughout — read, initially, as ovmsckf itself actively initialising and processing camera frames, which would have meant the atomic fix above had not resolved the symptom. It had not been given the chance to be tested: neither log line originates in openvins at all. Both classes exist, with the same names and an almost identical print style, inside MINS’s own, separately-implemented source (minsws/src/MINS/mins/src/update/cam/UpdaterCamera.cpp and minsws/src/MINS/mins/src/init/imuwheel/IWInitializer.cpp) — confirmed by grepping each package’s own source tree for the exact strings rather than trusting the interleaved shared log, after which zero matches turned up anywhere under openvins/. MINS was simply operating normally throughout, at its usual output, on the same log file. A clean, targeted instrumentation pass (a temporary print of vioalive()’s own return value, inside visualizeodometry() itself, removed once its purpose was served) showed it reading false continuously and honestly: ovmsckf had not initialised at all, exactly as §12.3.5 already explains, and exactly as this session’s very first exchange with the operator had already stated before this investigation began. This rover’s ovmsckf configuration disables dynamic initialisation and requires a genuine accelerometer jerk to trigger the jerk-based path it does use (catkinws/src/openvins/config/leo/estimatorconfig.yaml:48, initdynuse: false; line 43, initimuthresh: 0.4) — a stationary platform simply never produces one, no matter how long it runs or how many times its process is restarted.

##### Net result.

The two ovmsckf fixes are real, independently-justified hardening — a genuine latent gap in initialized()’s ZUPT interaction, and a genuine unsynchronised cross-thread read — and are kept. Neither caused, and neither on its own fixes, tonight’s reported symptom: that remains exactly the jerk requirement §12.3.5 already documented, unchanged by anything in this subsection. The practical lesson for this specific codebase is worth stating plainly for future debugging sessions: logs/ navigationmaster.log is shared across every node, MINS and VINS include classes named closely enough alike (UpdaterCamera, an IWInitializer-style static initialiser) that their log lines are not distinguishable by eye, and attributing a log line to a process requires checking which package’s source actually contains that exact string — not just which process seems contextually likely.

Symptoms observed in the shared log during this investigation, and their actual origin once checked against each package’s own source.

| Log text | Looked like | Actually from |
| --- | --- | --- |
| [IW-Init]:staticinitial- | ovmsckf static/IMU-wheel | MINS’s own |
| ization, Total | init succeeding in ∼1.4 s | IWInitializer.cpp |
| initialization time |  |  |
| UpdaterCamera:: | ovmsckf actively processing | MINS’s own |
| tryupdate: POOL/ | camera frames post-init | UpdaterCamera.cpp |
| SLAM/INIT/MSCKF |  |  |

### Dual-confirmation beacon reset: closing a spatial blind spot

§12.9.1’s dual requirement — the LED cluster and the checkerboard both confirmed before a reset fires — never required the two cues to be in the same part of the frame. The gap is concrete: LEDMAXYFRAC rejects a candidate LED blob by image height alone (above 35 % of the frame), not by proximity to the checkerboard, so a low reflection that clears that floor-plausibility gate in one part of the image, coincident with a real patterned surface (a poster, a doorframe) independently satisfying the checkerboard detector elsewhere in the same frame, would previously confirm a beacon that was never physically there — both conditions individually true, about two different objects. dualbeaconconfirmed() (leobackend.py:3036) now computes the checkerboard bounding-box centre and width alongside the LED cluster centre, and withholds confirmation whenever their separation exceeds a bound expressed as a fraction of the checkerboard’s own apparent width, BEACONSPATIALMAXFRAC=1.5, rather than a fixed pixel count.

The fraction, not an absolute pixel count, is the load-bearing design choice, and follows the same pinhole scaling argument this report already uses for the AprilTag range ceiling (§12.5.5). The beacon’s physical LED-to-checkerboard-centre offset sgap and its physical checkerboard width scb are both fixed by construction; under pinhole projection at focal length fx and distance d, their apparent pixel counterparts are g(d)=fx sgap/d and w(d)=fx scb/d, so

g(d)w(d) = fx sgap/dfx scb/d = sgapscb,  independent of both d and fx.

A fixed pixel threshold has no such invariance: it would be loose enough at close range to admit a disjoint false positive (where w(d) is large) and tight enough at long range to reject the genuine beacon’s own gap (where w(d) is small). Expressing the bound as a multiple of w(d) inherits the distance-invariance of Eq. [eq:spatialscale] by construction. BEACONSPATIALMAXFRAC=1.5 is set conservatively above the beacon’s expected physical ratio, not against a direct measurement of its actual sgap/scb, logged as a new open item (Table [tab:openitems], item 28). One coverage limit is stated explicitly rather than left implicit: the check validates LED against checkerboard only, never against the AprilTag, which is acceptable because the reset only ever fires at close range (§12.5.5: the checkerboard is useless beyond 1.5 m, so any LOCK capable of triggering a reset is already inside that range), where the checkerboard is the authoritative cue regardless.

### Extending the detection envelope to 0.5–4 m: an absolute-area gate against a 1/d2 law

An operator requirement — smooth, reliable detection from 0.5 to 4 m — was traced to a single bottleneck, deliberately isolated from the two detectors already known to be adequate across that range: the AprilTag channel (§12.5.5, decodable to ≈4.3 m, tagdecimate: 1.0, untouched) and the checkerboard, whose sub-∼1.5 m ceiling is an accepted optical limit, not a bug, also untouched. The actual bottleneck was the LED blob detector’s absolute-pixel area gate, LEDMINAREA/LEDMAXAREA, evaluated against the same 1/d2 law already used above. A physical LED of diameter DLED projects, under the pinhole model, to an apparent pixel diameter φ(d)=fxDLED/d and therefore an apparent blob area

A(d) = π4 φ(d)2 = π4 fx2DLED2 1d2  ∝  1d2.

Over the requested range [dmin,dmax]=[0.5,4.0] m — an 8× spread in distance — the apparent area itself varies by

A(dmin)A(dmax) = dmaxdmin2 = 82 = 64.

A single fixed window $[\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_MIN\_AREA}, \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_MAX\_AREA}]$ can only pass the same physical LED at both ends of the range if its own dynamic range spans at least this ratio, $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_MAX\_AREA}/\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}LED\_MIN\_AREA} \geq 64$. The prior window (10,1200) already satisfied that in the abstract (120×) — the actual failure was positional, not proportional: its floor of 10 px​2 was calibrated against a real image captured at 1 m (§12.5.5), anchoring the window around that single reference distance rather than around the requested range’s far end. At 4 m the same LED’s apparent area is (4/1)2=16× smaller than at 1 m (Eq. [eq:ledarea]), enough to fall under a floor sized for the near end alone unless the 1 m blob happened to sit comfortably above 16×10=160 px​2 — not something the prior calibration guaranteed. The window was widened to (3,2500), ratio ≈833, well clear of the 64× floor derived in Eq. [eq:ledratio], with the noise-rejection load the area floor can no longer carry alone shifted onto LEDMINCIRC, the density clustering pass, and the spatial cross-check of §12.11.9.

The one other area-adjacent constant, LEDCLUSTERPX (the whole four-LED cluster’s maximum pixel spread, not a single blob’s area), needed no change, and this is checked rather than assumed: with the beacon’s measured cluster width BEACONWIDTHM=0.165 m and CAMFX=384.65 px,

$$\begin{equation}
    w(d) = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CAM\_FX}\cdot\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}BEACON\_WIDTH\_M}/d, \qquad
    w(0.5\text{ m}) \approx 126.9\text{ px}, \quad
    w(4.0\text{ m}) \approx 15.9\text{ px},
    \label{eq:clusterwidth}
    \end{equation}$$

both comfortably inside LEDCLUSTERPX=280 px (margins of 2.2× at the near end and 17.6× at the far end), confirming this constant was never the range bottleneck. The new area bounds are, by the code’s own record, a first cut reasoned from Eq. [eq:ledratio] and the single 1 m calibration point, not yet verified against a real image captured at 4 m — logged as a new open item (Table [tab:openitems], item 27), to the same standard the rest of this registry already holds unmeasured thresholds to.

### A fuller tf2ros audit: one further legacy dependency, and an architecture note on the missing map frame

Having ported MINS off legacy tf (§12.9.4), an operator-requested audit today covered every remaining transform in the stack against tf2ros usage and REP 105’s map→odom→baselink convention. The audit starts from a terminology correction, stated plainly because the request that prompted it assumed otherwise: there is no EKF fusing MINS and VINS anywhere in this codebase. poseselector.py is, as §12.10.2 already documents, a discrete source switcher that applies a one-time SE(3) correction only at switch instants (the same correction §12.10.3 relies on to absorb a divergent re-initialisation) — never a continuous filter blending both estimators’ output. That switcher is, on inspection, the compliant half of this audit: it and carolusviconbridge.py both build correctly on tf2ros.Buffer, TransformListener and TransformBroadcaster (poseselector.py:130--132, carolusviconbridge.py:49--50), and it is poseselector.py that actually owns and correctly publishes REP 105’s odom→baselink segment (broadcasttf(), composing the fused pose with a static baselink↔ imuframe lookup via self.tfbuffer.lookuptransform()).

Two gaps were found. First, VINS (ovmsckf) still builds against the deprecated ROS1 tf package — ROS1Visualizer.h:42,147, ROS1Visualizer.cpp:42,334,343, 770,774,776 and ROSVisualizerHelper.h:31,73 all reference tf::TransformBroadcaster, tf::StampedTransform, tf::Quaternion or tf::Vector3 — the exact class of dependency MINS carried until §12.9.4’s port, including the same mitigating fact that made the MINS case low-urgency before it was fixed: its TF output is isolated to /tfvins (navigationsupervision.launch:90), the same pattern already used for /tfmins (mins.launch:54), so this legacy dependency cannot corrupt the shared tree today. A tf2ros-native ROS2Visualizer already exists in the same upstream repository, unbuilt on this ROS1/Noetic platform — precisely MINS’s situation before its own port — so the remedy is the same field-for-field mirroring exercise already proven once, logged as a new open item (Table [tab:openitems], item 29) rather than applied here.

Second, and more structural: no map frame and no map→odom transform exists anywhere in the shared /tf — confirmed by search across the workspace, not merely unobserved. What REP 105 asks for is a three-frame chain; what exists is only its odom→baselink half (the compliant finding above). The map layer that the mission’s own world-referenced concepts already depend on — Return-to-Base targets, obstacle and beacon markers, LOCK’s target-relative approach — is instead carried by a parallel, non-TF state in leobackend.py: worldoriginx, worldoriginy and worldheading, composed by getworldpos() (leobackend.py:2174--2182),

pworld = R(θw) plocal+tw,  R(θ)=cosθ−sinθsinθcosθ,

with $\theta_w = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}world\_heading}$, $\mathbf{t}_w =
    (\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}world\_origin\_x}, \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}world\_origin\_y})$ and $\mathbf{p}_{\text{local}} = \texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}self.pose}[0{:}2]$ — the local odometry pose already carried elsewhere in this chapter. Eq. [eq:worldse2] is, mathematically, exactly the rigid-body transform a map→odom broadcast would encode: a single SE(2) pose, recomputed at the same instants a correction would need to be re-published. It is instantiated as Python instance attributes rather than on the TF tree, so nothing else in ROS — RViz, another node, a future tf2ros.Buffer consumer — can see or use it. The re-anchoring is concrete: resetcoords() and resetcoordslocal() (leobackend.py:3704--3732) both recompute (tw,θw) via getworldpos() before reassigning worldoriginx/y — operationally a fresh map→odom correction at exactly the beacon-reset instant, published nowhere a TF consumer could observe it.

One nuance is worth being precise about, since it bears directly on the audit’s own framing: this does not mean a beacon reset “teleports” baselink. resetcoordslocal() also calls the firmware/resetodometry service (leobackend.py: 1426--1427), but that only resets the firmware’s own dead-reckoning integrator; MINS’s actual filter input is wheel velocity, not position (wheelremap.py, republishing JointState.velocity as the front/rear side-mean per wheel, catkinws/.../ wheelremap.py:81--84), decoupled from that integrator. baselink’s real, TF-published pose (the compliant finding above) is therefore undisturbed by a beacon reset; only the parallel Python “world” state consumed by RTB and mapping is re-anchored. Figure 12.10 summarises the finding: a compliant odom→baselink segment, two isolated visualisation-only branches (one still legacy tf), and a map layer that exists — correctly, in the SE(2) sense of Eq. [eq:worldse2] — but nowhere on the TF tree itself.

TF-tree audit, July 24: the compliant odom→ baselink segment (solid, poseselector, tf2_ros) against the two isolated visualisation branches and the absent map frame, whose role is instead filled by a parallel, non-TF Python state (top) consumed directly by world-referenced mission code.

A correction is sketched, not applied to the live robot without a dedicated validation pass: publish an actual map frame via a tf2ros.TransformBroadcaster at exactly the resetcoords()/resetcoordslocal() re-anchor points, and migrate getworldpos()’s callers to tf2ros.Buffer.lookuptransform("map", "baselink", ...) in its place:

def _publish_map_correction(self, wx, wy, heading):
    t = TransformStamped()
    t.header.stamp, t.header.frame_id = rospy.Time.now(), "map"
    t.child_frame_id = "odom"
    t.transform.translation.x, t.transform.translation.y = wx, wy
    q = tft.quaternion_from_euler(0, 0, heading)
    (t.transform.rotation.x, t.transform.rotation.y,
     t.transform.rotation.z, t.transform.rotation.w) = q
    self._map_broadcaster.sendTransform(t)   # tf2_ros.TransformBroadcaster

def _get_world_pos(self):
    try:
        tr = self._tf_buffer.lookup_transform("map", "base_link", rospy.Time(0))
        return tr.transform.translation.x, tr.transform.translation.y
    except (tf2_ros.LookupException, tf2_ros.ConnectivityException,
            tf2_ros.ExtrapolationException):
        return self._fallback_local_pos()  # existing wheel-odometry path

This is deliberately not implemented here: worldorigin and getworldpos() are referenced across RTB, obstacle/beacon mapping and LOCK, a nontrivial refactor on a currently stable robot, logged as a new open item (Table [tab:openitems], item 30) pending the operator’s decision to proceed.

## Field session, July 27: an offline trajectory-comparison pipeline and cockpit instrumentation

A supervision meeting set three linked objectives: a reproducible way to quantify MINS against openVINS off-line (not just watch them live), one-click access to that data from the cockpit, and richer real-time visibility into the Carolus beacon channel. None of this changes an estimator or a control loop — it is instrumentation and data plumbing laid on top of the existing, now-stable stack — but it is the scaffolding the precision roadmap (Table [tab:openitems]) has been waiting on: every placeholder threshold in that registry is ultimately settled by a measured trajectory, and until this session there was no packaged path from a drive to a number.

### A rosbag → CSV → Matlab pipeline, and an honest choice of metric

Three small tools form the chain. tools/recordtrajectories.sh records the two raw estimator topics (/mins/imu/odom, /ovmsckf/odomimu) alongside the served fused pose, the active-source timeline, the Carolus global fix and raw wheel odometry, into a timestamped LZ4 bag. tools/bagtocsv.py converts that bag into one CSV per source (columns t,x,y,z,qx,qy,qz,qw,yawrad, t relative to the first message so every trace shares an origin), the portable pivot format that needs no Matlab ROS Toolbox. tools/plottrajectories.m overlays the planar trajectories and computes the comparison.

The metric choice is the substantive part, and it is deliberately not framed as “accuracy.” This platform carries no absolute ground truth, so a raw difference between MINS and VINS is a mutual divergence, not an error against truth. Two readings are therefore computed and never conflated. The first, always available, is the inter-estimator divergence after an optimal rigid 2D alignment (Kabsch/Umeyama, no scale, since both estimators are metric): each trajectory is resampled onto a common 20 Hz time base by interpolation, VINS is rotated and translated onto MINS by the closed-form minimiser of

minR∈SO(2), t∈R2 k​∥pkMINS−(R pkVINS+t)∥2,

and the residual distance ⋅k is reported as RMS, max and final divergence, both before and after alignment. Alignment matters because the two estimators start from independent origins with independent initial headings; the pre-alignment number measures raw served-frame disagreement, the post-alignment number measures whether the two shapes agree, which is the fairer statement of relative consistency. The second reading, computed only when a carolus CSV is present, treats the drift-free Carolus global fix (§12.12.3) as a near-ground-truth anchor and reports each estimator’s position error against it separately, the one context in which the word “error” is used honestly. Both figures are stamped directly onto the exported plot so a reader cannot mistake one for the other.

| Raw traces: no shared frame, hence no relation visible. |  | After the similarity alignment of Eq. [eq:umeyama]; openVINS before rescaling retained in pale grey. |
| --- | --- | --- |

Trajectory overlay, MINS (orange) versus openVINS (cyan), from the first substantive driven loop (July 29; 129 s, ∼15 m of lab circuit, both estimators recorded simultaneously). Read together with Fig. 12.16: the alignment of (b) is constructed to absorb a scale error, which only Fig. 12.16b exposes. Panel-by-panel commentary in §12.14.1.

| Separation versus time, on a logarithmic axis: without which the ∼0.5 m aligned curves collapse onto the axis beneath the 90 m raw one. The scale-corrected trace dips to 10−3 m near t≈45 s. |  | Cumulative path length. The two distinct slopes are the scale disagreement that Fig. 12.13b is constructed to hide; quantified by Eq. [eq:residualgap] and §12.14.3. |
| --- | --- | --- |

Divergence and scale. These two panels must be read against each other: (a) reports how far apart the estimates are, (b) reports that part of that distance is a difference of size rather than of shape.

| Heading accumulated since the start, each relative to its own initial value, so the staircase of successive turns is directly comparable. |  | The heading disagreement in isolation: flat until t≈40 s, then a rise to ∼5∘ between t≈40 and t≈70 s, after which it plateaus, a drift confined to one phase of the drive, not a uniform bias. |
| --- | --- | --- |

The heading channel, which the position residuals do not reveal. This is the same weakness that the loop-closure comparison of §12.12.7 finds independently, from a different pair of runs.

| Post-alignment residual decomposed onto X and Y. Both components are smooth ±0.5 m excursions: a slowly varying disagreement in trajectory, not sample-to-sample scatter. A residual concentrated on one axis would be a directional bias, corrected quite differently from isotropic noise. |  | Residual distribution with median (solid) and 95th percentile (dashed). Reported because the mean of a divergence is a poor summary when the damage lives in the tail: a lesson this campaign learned the hard way, §12.14.1. |
| --- | --- | --- |

Structure and distribution of what remains after alignment. Together they establish that the residual is a slow, structured disagreement rather than noise, which is what makes it worth attributing to a cause.

The pipeline was exercised end to end this session on a stationary bench capture (MINS publishing at rest, VINS not yet initialised for want of the jerk of §12.3.5, no beacon in view): the recorder buffered and exported a well-formed MINS trace and the Matlab reader consumed it, but a substantive MINS-versus-VINS figure requires a real driven trajectory with VINS initialised, and is logged as the first thing to capture next (Table [tab:openitems], item 34). Figure 12.13 holds that slot.

### One-click export from the cockpit

The same data is now reachable without a terminal, and from the Trajectory tab specifically (trajectory.html), where an operator watching the live plot is most likely to want it. leobackend.py gained a lightweight always-on trajectory recorder: it subscribes to the two raw estimator topics and the Carolus fix, throttles each to 20 Hz into a bounded ring buffer (TRAJMAXLEN, ≈50 min headroom, a hard deque bound against unbounded growth), and on an exportmatlab command writes both a per-source CSV (the same 11-column layout as bagtocsv.py: t,x,y,z, quaternion, and derived roll,pitch,yaw) and a single .mat into web/exports/. The .mat is deliberately structured for a drag-and-drop MATLAB workflow: scipy.io.savemat writes one named struct per source (MINS, VINS, CAROLUS), each carrying column-vector fields t,x,y,z,qx..qw,roll,pitch,yaw, so dropping the file into MATLAB yields three ready variables rather than an anonymous matrix. Empty sources (e.g. VINS before its jerk-initialisation) are still written as empty-array structs so the variables always exist. Because the cockpit’s static server already serves that directory, the backend reports the written filenames back through the existing telemetry channel; the Trajectory tab watches for a new export and immediately triggers a same-origin download of the .mat (a programmatic <a download> click, no popup), the “infallible link” the workflow needs. A companion Reset clears the buffer to mark a fresh run. This is a convenience path layered beside the rosbag route of §12.12.1, not a replacement: the bag remains the record for heavier off-line work (it also captures wheel odometry and the source timeline), while the button covers the common “drive, then plot” loop. Both feed the identical Matlab script, by construction.

Two smaller pieces complete the loop. Coordinate frames on the plot: the Trajectory canvas now subscribes to the raw /tf topic and draws the standard RViz-style axis triads (X red, Y green) at two frames, the odom origin and the live baselink pose from the odom→baselink transform. The honest caveat is drawn into the code and the label: there is no map frame on /tf in this architecture (the REP 105 audit of §12.11.11), so the origin triad is labelled odom, the true root of the dynamic tree, not a fabricated map. No tf2webrepublisher is required: the raw topic is read directly, filtered to the one transform of interest. Opening MATLAB directly on the plot: the Python MATLAB Engine API is not installed on the lab machine, but the MATLAB binary is, so on operator request the Export button does more than write files, when the backend has located MATLAB it launches the MATLAB desktop on the plot (matlab -sd tools -r "plottrajectories(’<base>.mat’)", the GUI -r form that leaves the desktop open, in a detached background process), reading the freshly-written .mat directly. Two environment details make this work from a backend that rosmon starts without a graphical context: the launch reconstructs DISPLAY and XAUTHORITY (probed from the running session, :0 and /run/user/<uid>/gdm/ Xauthority here) so the window reaches the operator’s screen, and the MATLAB binary itself is found by probing PATH and the known install prefixes (the rosmon-launched backend inherits a restricted PATH that hides /usr/local/bin). If no MATLAB is found the button falls back to the same-origin .mat download instead. This was validated end to end this session: the Export click wrote the .mat, the backend opened the MATLAB desktop on the operator’s display, and plottrajectories ran inside it and rendered Figure 12.13’s layout from the real export. The Trajectory tab also keeps a standalone “open in MATLAB” button that re-opens the last export without re-recording.

### Carolus beacon: a 6-DOF readout and a geometry-grounded LED status

The Carolus channel, previously visible only as its effect on MINS, now has two dedicated cockpit panels. The first is a direct 6-DOF readout: the cockpit subscribes to /mins/externalref/carolus (geometrymsgs/PoseStamped, published by carolusviconbridge.py only while a beacon fix exists), converts the quaternion to roll/pitch/yaw in the browser, and displays position and orientation with a freshness watchdog that blanks the panel to “no fix” after 1.5 s of silence, honest about the fact that this fix is intermittent by nature.

The second panel is a four-LED status cross, and its design is worth stating precisely because it is easy to over-claim. The Carolus launch file (carolusBlobDetection.launch) declares the beacon’s four LEDs by their metric knownpoints: two on the ±X baseline (±0.0825 m), one on Y (0.072 m), one on Z (0.0555 m). The detector does not label an individual blob with an axis role; what it reports is how many of the four cluster LEDs are currently seen (detection.ledreset.count) and a span-derived distance. The cross therefore drives its indicators from that count against geometry-grounded thresholds (the X baseline needs at least the two ±X LEDs (count≥2), Y needs a third, Z the full four) with a live Carolus fix (all axes actually solved by the node’s PnP) forcing all three axis dots solid, and a fourth indicator lighting on distance acquisition. It is an observability display honestly wired to what the pipeline computes, not a fabricated per-axis solve.

A third small panel exposes the Carolus blob-detection parameters. The backend parses the launch file for the tunable subset (circularity, saturation, area bounds, hue window, image threshold, distance limit) and surfaces their current values; the cockpit shows them as editable fields that push a setcarolusparam command. The honest limitation is labelled in place: the Carolus node has no dynamicreconfigure, so a change is written to the ROS parameter server and takes effect at the next Carolus launch, not live, the panel is primarily a faithful visualiser and a convenience for staging the next run’s settings, not a live tuning knob.

### Later the same day: from a 6-DOF readout to a robot-position and LED-capture panel

The 6-DOF panel of §12.12.3 depended on carolusviconbridge.py republishing /mins/externalref/carolus, a topic that, on inspection later the same day, is never actually populated: nothing publishes beaconlink on /tf, so the bridge has no transform to look up and the panel was structurally always going to read “no fix.” Rather than patch a bridge built on a TF frame the architecture does not produce, the panel was rebuilt around what the vision pipeline does reliably compute: the robot’s own local pose (self.pose, already zeroed by the LED-reset event) and the four-LED capture status of §12.12.3 directly. AprilTag detection was put on standby (SIGSTOP on the node: reversible, keeps it registered on the graph without spending CPU) so operators could focus on validating the LED-only path in isolation. The reset trigger itself was also simplified in the same pass: it had required the LED cluster and the checkerboard landmark to confirm on the same tick, which measurably never happened together in the field (the checkerboard’s rate-limited, hysteresis-held detection window rarely coincides with a fresh LED frame), the checkerboard requirement was dropped from the reset gate in all three call sites (manual mode, and both AUTO/LOCK paths), leaving LED confirmation (≥ VISIONCONFIRMFRAMES, i.e. 3 consecutive valid frames) as the sole trigger. A second, narrower fix followed a field report of a false reset in an empty room: the LANDMARK-mode hold logic (which keeps the last LED result “valid” across the mode switch to LED → LANDMARK, since the checkerboard branch would otherwise never see a confirmed LED) had no time bound, so a stale detection could read as valid indefinitely once the pipeline was in LANDMARK mode. A one-second bound (LEDHOLDS) was added: past it without a fresh detection, the hold expires to valid=false rather than persisting a ghost.

### Detached MATLAB: three stacked failure modes behind “opens then closes,” and a rendering bug hiding in the pipeline itself

The MATLAB launch of §12.12.2 was validated again later the same day against a harder condition, the watchdog’s automatic stack restarts firing while a MATLAB session was open, whose root cause was only identified two days later and is documented in §17.17.3, and failed: the window closed within seconds of a restart, and separately even without one. Three independent causes were isolated, each confirmed by a controlled before/after test rather than inferred from a single observation.

First, startnewsession=True (a setsid equivalent) protects the child from an os.killpg() targeting leobackend.py’s own process group, exactly how roslaunch tears down a node on restart, but a watchdog-triggered restart still killed MATLAB regardless. The fix is a genuine double-fork: tools/matlabdaemon.sh backgrounds the launch (cmd & disown) and exits immediately without waiting, orphaning the job so the kernel reparents it to init (observed here as systemd --user) rather than to leobackend.py, structurally outside any cleanup that walks the backend’s process tree, by group or by PPID. Verified directly: a test process launched this way kept a PPID of 1774 (systemd --user) after the wrapper exited, and a real MATLAB session launched through the daemon script survived two genuine watchdog-triggered stack restarts inside one continuous 280-second window, though a later false positive from the same test methodology (PID reuse under heavy process churn, caught by cross-checking /proc/<pid>/cmdline rather than kill -0 alone) is recorded here as a caution for repeating this class of test.

Second, and unrelated to any restart: MATLAB launched via -r exits cleanly (exit code 0, no crash, no error) as soon as the given command finishes, if its stdin is not a real terminal, which a process started by rosmon never has. The documented behaviour (“-r keeps the desktop open”) holds only with an interactive terminal attached. The fix allocates a real pty for the launch, via the standard script utility (script -qefc "$cmdstr" "$log") inside the same daemon script, combined with the double-fork above. One accepted side effect: with a real pty present, MATLAB routes its banner and fprintf output to the GUI console rather than mirroring it to the pty, so matlablaunch.log now records only script’s own start/exit trailer (including the exit signal, e.g. 137 for a kill) rather than the full transcript, diagnostic depth traded for the fix that actually mattered.

Third, and the one that had been masking the first two in testing: even with both fixes applied, a real export-triggered launch still closed within seconds. leobackend.py runs with no desktop session at all: not only no DISPLAY, but no XDGRUNTIMEDIR and no DBUSSESSIONBUSADDRESS either, confirmed empty in /proc/<pid>/environ where a normal login session always has them, because the stack is (re)launched by the watchdog’s cron job rather than a user session and inherits cron’s own cgroup (system.slice/cron.service). A controlled A/B test made this decisive: the identical launch command, from an otherwise-identical minimal environment, died in under five seconds without those two variables and ran for over a minute cleanly with them added. guienv() now reconstructs both from the standard /run/user/<uid> paths, alongside DISPLAY/XAUTHORITY, each only when the process does not already have a valid one and the expected path exists on disk. The complete fix (double-fork, pty, and the session variables together) was confirmed against the real cockpit button: a PID verified alive for 105 continuous seconds via /proc/<pid>/cmdline, spanning one genuine automatic watchdog restart mid-window.

A fourth, unrelated defect surfaced only by looking directly at a rendered figure rather than confirming a PNG merely existed: plottrajectories.m had been silently mis-rendering every trajectory line it drew, all session, a plot(x, y, ’-’, …) call with several hundred points rasterises only short fragments near locally steep sections and drops near-flat sections entirely, reproduced independently of renderer choice (opengl/painters/-softwareopengl), export function (saveas/print/exportgraphics), LineJoin, and GraphicsSmoothing. Adding a marker to the same line ('Marker','.','MarkerSize',3) renders correctly, which is the applied fix in both plottrajectories.m and the new comparetests.m of §12.12.7, a workaround, not a root-caused explanation, but confirmed to fully restore every trajectory figure this pipeline produces. Every comparison PNG generated before this fix is visually wrong and should be regenerated, not just re-copied.

### Carolus at working distance: the D455 colour stream, re-enabled under an explicit CPU budget

The 6-DOF beacon pose of §12.12.3 needs carolusastrobee to see actual colour frames, and the D455’s colour stream had been disabled since July 6 for a documented reason recorded directly in /etc/ros/robot.launch: at its default profile the colour stream alone saturated the Pi’s CPU badly enough to starve the rosserial link and desync the IMU. That risk materialised again, independently, later in this same session (an unrelated colour-camera re-enable attempt drove the Pi to sustained thermal throttling and a total, if temporary, loss of drivable control: resolved by disabling colour again and cycling the stack). Given an explicit, informed decision to accept the residual thermal risk (a fan is the planned permanent fix) and re-enable the channel for beacon work, two changes were made together rather than repeating the earlier attempt as-is. First, the colour stream is now launched with the same explicit 640×480@15 Hz budget already used for infra/depth (colorwidth/colorheight/colorfps were previously unset, silently falling back to a much heavier default profile: plausibly the actual reason the July-6 measurement was as bad as it was). Second, carolusastrobee itself has no internal frame-rate parameter, it processes every frame its subscribed topic delivers, so its CPU cost is decoupled from the camera’s native rate with a standard topictools/throttle node inserted ahead of it (carolusdetect.launch, default 4 Hz, configurable by launch argument), no source changes required. Measured after both changes: Pi temperature settled from an already-elevated 86–87 ∘C baseline down to ≈84 ∘C, realsense2cameramanager dropped from the 148–187 % CPU observed in the earlier incident to ≈30–65 %, and carolusastrobee plus the throttle node together stayed under 1 % CPU at idle. The permanent fix remains physical (active cooling); this is a working mitigation, not a substitute for it.

### A two-test protocol: Test 1 (MINS) versus Test 2 (openVINS), and the closing comparison this campaign has been building toward

Every earlier subsection in this session’s work is plumbing (a pipeline, an export button, a beacon panel, a thermal budget) built toward a single outstanding measurement (Table [tab:openitems], item 34): a real driven loop of the lab, compared honestly between MINS and openVINS. The Trajectory tab now has the protocol to produce it directly. A two-way selector (TEST 1 / TEST 2) labels the currently-buffered run; it changes nothing about what is recorded (both estimators are always captured together, which is the entire point of the comparison): it only relabels where the Export button writes its files, from the generic timestamped name to a predictable <label><source>.csv / <label>.mat (e.g. test1mins.csv, test2vins.csv). The protocol this enables: select Test 1, drive the lab loop under active MINS, export; switch to Test 2, reset, drive the same loop under active VINS, export. Two predictably-named files, one per estimator, each on its own physical run of the same loop.

A new script, tools/comparetests.m, consumes exactly that pair. It is deliberately not the Kabsch-aligned divergence of Eq. [eq:kabsch], that comparison is only meaningful when both estimators observe the same motion at the same instants, which two separate drives are not. Two independent runs of a loop admit a different, equally honest pair of readings instead: path length (a coarse cross-check that both runs covered a comparable distance) and loop-closure error, the distance between each trajectory’s first and last point. A lab circuit returns close to its start; the residual gap after a full loop is a direct, if approximate, proxy for the drift accumulated by that estimator over that run, in the continued absence of absolute ground truth. Heading drift (final yaw minus initial yaw, wrapped) is reported alongside it. The script was verified against synthetic circular trajectories with injected drift before being trusted on real data, and inherits the marker-line fix of §12.12.5.

Test 1 (MINS, orange) versus Test 2 (openVINS, cyan): two independent drives of the same lab loop. Left: planar trajectories (∘ start, ▫ end). Right: each run’s distance from its own start point over time, a loop that closes returns this curve to zero by the end; the residual is the loop-closure error reported alongside path length and heading drift.

##### Reading the result.

This subsection was originally drafted ahead of the two drives it analyses, scaffolded with the exact numbers and judgement calls the closing analysis would need, so that filling it in would be a matter of driving the loop twice and transcribing the script’s own printed output rather than re-deriving the comparison from scratch. Both drives now exist (test1, test2): what follows is that transcription, not a template.

- Path length. MINS: 16.78 m. openVINS: 17.16 m. If the two differ by more than a few percent, that is itself informative: it means the two runs did not cover quite the same physical path (operator steering variation between drives), and the loop-closure numbers below should be read as slightly less directly comparable, not as a pure estimator-quality contrast.

- Loop-closure error. MINS: 0.055 m (0.3 % of path length). openVINS: 0.025 m (0.1 %). This is the headline number: the estimator with the smaller closure error drifted less over a comparable loop. State which one, by how much, and whether the gap is large relative to the platform’s demonstrated static-drift floor (Protocol T1.1, §12.7.2), a closure error near that floor says the estimator tracked the loop about as well as it tracks standing still; one well above it says the loop itself (turns, speed changes, feature density around the room) is where the drift is actually accumulating.

- Heading drift. MINS: +17.4​∘. openVINS: −28.8​∘. Large heading drift with small loop-closure error is possible and worth flagging explicitly if seen: it means position happened to be self-correcting around the loop shape while orientation still wandered, which loop-closure error alone would not reveal.

- Qualitative trajectory shape. Both runs close visibly as loops in Figure 12.23’s left panel, with no kink or gross corner-cutting in either; the path lengths differ by 2.3% (16.78 against 17.16 m), which is within the operator-steering variation this subsection anticipated and small enough that the closure figures below stay comparable. The difference between the two estimators on this loop is therefore not visible in the trajectory shape: it is visible only in the heading channel, which is precisely why the numeric readings below matter more than the picture.

- Conditions during each drive. Both drives were made indoors on the lab carpet under fixed ceiling lighting, on mains power (12.0 V, 78%), within three minutes of one another (test1, test2). No Carolus fix was in view during either run, so the ground-truth-anchored error of §12.12.1 could not be computed and only mutual divergence is available. The Pi was thermally throttled throughout, at 86 ∘C and a capped 600 MHz: the condition characterised in §12.14.4 as the origin of the gaps in the IMU stream, and a standing caveat on both runs. Critically, and unlike every earlier attempt in this campaign, no watchdog restart interrupted either drive: the cron PATH defect that had been destroying the stack every ∼2 min was diagnosed and fixed earlier the same day, and the watchdog log records zero automatic restarts across July 29. Every estimator measurement taken before that fix was made on a stack being torn down mid-run, which is why this pair is the first that can be read at face value.

- For any future repetition of this protocol, the conditions worth recording alongside the data are the floor surface and the ambient lighting, whether the beacon was in view for either run (a Carolus fix during Test 1 or Test 2 would let §12.12.1’s ground-truth-anchored error be computed too, not just mutual divergence), Pi temperature/throttling state (§12.12.6), and whether the watchdog restarted the stack mid-drive (the cron watchdog’s automatic full-stack restarts remain an open, unresolved instability at the time of writing, worth recording explicitly if one interrupted either test, since a mid-drive backend restart clears the trajectory buffer and would truncate that run’s export).

- Verdict. On loop-closure error alone openVINS is the better performer here, by a factor of 2.2 (0.025 against 0.055 m), and both figures sit within the same order of magnitude as the platform’s static planar floor of ∼8 mm in X and ∼1 mm in Y (Table 12.3), that is, on this loop neither estimator accumulated much more position error while driving than it does standing still, which is a stronger result for both than this campaign’s earlier sessions would have predicted. The heading channel reverses the ranking: openVINS drifted −28.8∘ against MINS’s +17.4∘, 65% worse in magnitude and in the opposite sense. This is exactly the combination flagged as worth reporting above, small closure error with large heading drift, and it is the expected signature of a loosely-constrained orientation channel whose position happens to be self-correcting around a closed loop: a heading error accumulated on one leg is partly undone on the return leg, so the loop closes while the orientation estimate has genuinely wandered. It also matches the independent evidence of Figure 12.13 panel 6, where the MINS–openVINS heading disagreement grows to ∼5∘ within a single simultaneous run. The two readings agree, from different data, that orientation is where openVINS is weakest on this platform.

- The honest verdict is therefore split rather than clean, and it is one data point. For unattended patrol, where heading error compounds into navigation error over minutes, MINS remains the safer default on this evidence, notwithstanding its larger closure error, and it is also the only estimator whose scale has been checked against a tape measure (−4%, §12.14.4). That is consistent with the architectural expectation of §12.1.2 only in part: the tightly-coupled wheel constraint does appear to be anchoring orientation, but it did not deliver the better position closure on this loop, which the architectural argument alone would have predicted. Turning this into a conclusion needs at least two further loops on different days and under different lighting, run after the thermal throttling is resolved, and with a Carolus fix in view so that absolute error, not merely mutual divergence, can be computed. On these specific conditions, which estimator is the better default for unattended patrol, and does that verdict agree with or contradict the architectural expectation set in §12.1.2 (MSCKF tightly-coupled versus a loosely-coupled alternative)? A single loop is one data point, not a validated conclusion, state it as such, and note what a second and third loop (ideally on different days, different lighting) would need to show to turn this into one.

## Field session, July 28: a starved serial link, a single boolean worth four orders of magnitude, and an accelerometer out of spec

Three findings were made on this day, and their interest lies less in each one than in how they interlock. A scheduling defect was corrupting the inertial stream; an out-of-spec sensor was making the corruption catastrophic rather than merely untidy; and a single configuration flag was the only thing standing between the two and a divergence of forty kilometres. Each was found while investigating a different symptom, which is the usual way, and none is intelligible without the others.

### The serial link was starving, not failing

The presenting symptom had been recurring for a week and had twice been attributed to hardware: wrong checksum for topic id and msg in the thousands, /firmware/wheelstates collapsing to 1 Hz, and firmwaremessageconverter left with no wheel data to convert. The CORE2 board and its cabling had been the standing suspects.

The measurement that settled it was the kernel’s own overrun counter, which is specific in a way checksum errors are not. A checksum error says a frame arrived corrupted; an overrun says the receiver discarded a byte because nobody read the FIFO in time. The counter stood at 137 000 and was advancing at 63 per 10 s. That is not a cabling fault, and no amount of physical inspection would ever have found it: the bytes were arriving correctly and the operating system was failing to collect them.

Two causes were acting together. The Pi was thermally capped at 600 MHz, so every code path took two and a half times longer than at nominal clock, and serialnode ran at ordinary scheduling priority, so it waited behind every other runnable task on a machine already oversubscribed. §12.15.1 develops the deadline model that makes this quantitative; the operational summary is that the port must be serviced within 1.28 ms and neither condition allowed it.

The fix has two halves and needs both:

# 1. real-time priority for the serial node
chrt -f 25 <serial_node pid>
# 2. permission for leo.service to grant it (drop-in)
[Service]
LimitRTPRIO=50

Applying the first without the second does not merely fail to help: the node refuses to start, because a service whose RLIMITRTPRIO is zero cannot raise its own scheduling class. This is worth stating plainly because the failure presents as “the fix broke the serial node”, which invites reverting the very change that was correct.

The result was immediate and large. Wheel odometry went from 1 Hz to 19 Hz, and the checksum storm stopped. A secondary discovery followed from looking at the process table with fresh attention: an orphaned rosbridgeserver was running on the robot, consuming 96 % of a core to serve nothing, on a machine whose scarcity of CPU was the entire problem. It had no business being there and was terminated.

### One boolean, and a divergence of 41 824 metres

With the inertial stream finally clean, openVINS was expected to behave. It did not. A stationary run produced a final position estimate 41 824 m from the origin, which is to say that a rover parked on a carpet had concluded it was somewhere past the edge of the county.

A divergence of that magnitude is not a tuning problem and should not be treated as one. It is large enough to identify its own mechanism, because only a double integration can produce it. The culprit was a single line in estimatorconfig.yaml:

try_zupt: false      # -> 41 824 m
try_zupt: true       # ->      0.002 m

Four orders of magnitude, and then four more, from one boolean.

##### Why it matters so much, and only to this estimator.

Zero-velocity updates exist because a stationary visual-inertial platform is an observability hole. When the rover does not move, the camera contributes no parallax and therefore no scale information, and the accelerometer sees only gravity plus its own errors. Writing the specific force as

am=Ka (a−g)+ba+ηa,

at rest a=0, so anything the filter cannot attribute to gravity is integrated twice into position:

Δp(t) = 12 r t2,  r=(Ka−I) g+ba,

with no measurement anywhere in the filter opposing it. A ZUPT supplies exactly the missing constraint, asserting v=0 and thereby making the residual r observable instead of free.

MINS, running on the same data on the same day, did not diverge and does not need the flag. The reason is structural rather than a matter of quality: MINS consumes wheel odometry, and a wheel encoder reporting zero rotation is itself a zero-velocity measurement, delivered continuously and without any detection logic. The estimator that fuses the wheels gets its ZUPT for free; the camera-and-IMU estimator has to be told to look for it. This is the sharpest illustration in the campaign of what the extra sensor actually buys.

Testing a configuration from the wrong directory An earlier attempt to validate this had produced a false negative. The configuration was edited and tested from a copy under /tmp, while the running node loaded its own file from the workspace. The test exercised a file nobody was reading. Always confirm which path the live node opened, with rosparam get or by reading the node’s startup log, before concluding that a configuration change has no effect.

Two further defects were cleared the same evening, both recorded in §17.10: a mutex crash firing 1324 times, once every 46 s, whose root cause split into an OpenCV FileStorage parser aborting on an over-long inline YAML comment at startup, and OpenCV’s own thread pool at runtime, resolved by pinning numopencvthreads: 1.

### Why the divergence was that large: an accelerometer 10.6 % out of spec

Equation [eq:july28doubleint] says the divergence rate is set by the residual r, so the natural question is how large r actually was. The stationary characterisation of §12.3.3 answers it: the accelerometer reported 8.7525 m/s​2 against a true local gravity of 9.790 m/s​2 at latitude 28∘, an error of −10.6 % lying in the raw sensor rather than anywhere in the processing chain. The residual is therefore

r = 9.790−8.7525 = 1.06 m/s2.

Substituting into Eq. [eq:july28doubleint] gives Δp≈1900 m after one minute, and the observed 41 824 m corresponds to

t = 2×41 8241.06 = 281 s ≈ 4.7 minutes,

which is the right order for the run in question. The agreement is a consistency check rather than a proof, since the run duration was not recorded to the second, but it closes the argument: the three findings of this day are one causal chain. The scheduling defect corrupted the inertial stream, the out-of-spec accelerometer set the divergence rate once the stream was clean, and the ZUPT flag was the only mechanism capable of observing and cancelling it.

##### What this means for every comparison in this chapter.

An accelerometer 10.6 % out of spec is not a nuisance to be tuned around. Until it is corrected in the sensor or compensated in the model, any MINS-versus-openVINS comparison measures, in part, which estimator better tolerates a known-bad accelerometer. MINS survives it because the wheels anchor the solution; openVINS survives it only because ZUPT is enabled. That is a real and reportable difference in robustness, and it is emphatically not the same statement as one estimator being more accurate than the other. The distinction is recorded in Table [tab:openitems] so that no later reader mistakes the one for the other.

## Field session, July 29: trajectory alignment as a measurement, and the first ground-truth calibration

The comparison pipeline of §12.12.1 produced its first substantive driven loop on July 29: 129 s of manual lab circuit, ∼15 m of path, both estimators running and recorded together. The figure it produced (Fig. 12.13) is the deliverable this campaign has been building toward since item 34 was opened. Reading it correctly, however, turned out to require more care than plotting it, and this section develops the mathematics that the plot only summarises. Two results are established: an exact decomposition that separates a scale disagreement from a shape disagreement (§12.14.2), and a noise model that explains why two perfectly reasonable definitions of “how far did it travel” give different answers, and why one of them must never be compared across sampling rates (§12.14.3). A third subsection reports the first measurements on this platform made against physical ground truth rather than against another estimator (§12.14.4).

### Reading the panels

Each panel answers a question the others cannot, and several of the conclusions in this section are visible only by reading two panels against each other, which is why they are grouped in pairs across Figs. 12.13–12.22 rather than presented as one combined plate. (plottrajectories.m still produces the combined 3×3 plate for on-screen use; at page width its individual panels fall below 5 cm and become unreadable, so the report uses the separately exported 200 dpi versions instead.)

The two estimates as served, sharing no frame. They sit 90 m apart and look unrelated. Included deliberately: this is the honest starting point, and it measures nothing but the arbitrariness of two independent origins.

The same pair after the similarity alignment of Eq. [eq:umeyama], with openVINS before rescaling retained in pale grey so the magnitude of the correction stays visible. This is the only view in which the eye can judge whether the two estimators describe the same motion.

Raw, rigid-aligned and scale-corrected, on a logarithmic axis, without which the ∼0.5 m aligned curves collapse onto the axis beneath the 90 m raw one. The scale-corrected trace dips to 10−3 m near t≈45 s.

Two distinct slopes, exposing a scale disagreement that panel 2 is constructed to hide. Panels 2 and 4 must therefore be read together, never separately.

Both estimators, relative to their own initial heading, so the staircase of successive turns can be compared directly.

The angular disagreement in isolation: flat until t≈40 s, then a rise to ∼5∘ between t≈40 and t≈70 s, after which it plateaus. A drift confined to one phase of the drive, not a uniform bias.

Post-alignment, and the decomposition matters: a residual concentrated on one axis is a directional bias, which is corrected differently from isotropic noise. Here both components are smooth ±0.5 m excursions, a slowly varying disagreement, not sample-to-sample scatter.

Median and 95th percentile marked. Reported because the mean of a divergence is a poor summary when the damage lives in the tail, a lesson this campaign learned the hard way.

Every figure quoted in this section, plus the caveat the script prints whenever the fitted scale departs from unity by more than 3%: that the scale was fitted to MINS, and that MINS is not ground truth.

### Rigid versus similarity alignment: two residuals, and the exact identity that separates them

Neither estimator is expressed in a frame the other shares. MINS and openVINS each initialise their own origin and their own initial heading, so the raw separation between them (89.2 m RMS on this run) measures nothing about either estimator’s quality. It measures only that two arbitrary frames are arbitrary. Panel 1 of Fig. 12.13 shows exactly this and is included deliberately: the two traces sit 90 m apart and look unrelated, which is the honest starting point.

The rigid alignment of Eq. [eq:kabsch] removes the frame difference and leaves a residual that measures shape agreement. Written with the estimator roles explicit, let {pk}k=1n be the openVINS positions and {qk}k=1n the MINS positions, both resampled onto the common 20 Hz base so that index k denotes the same instant in both. Introducing the centroids and the centred coordinates

p=1nk​pk,  pk=pk−p,  σp2=1nk​pk2,

and identically for q, the quantity σp is the RMS radius of the openVINS trace about its own centroid. It is referred to below as the spatial extent, and it will matter a great deal in §12.14.3.

The similarity alignment admits a scale factor alongside the rotation and translation:

(s⋆,R⋆,t⋆)=arg mins>0, R∈SO(2), t∈R2 1nk=1n∥qk−(s R pk+t)∥2.

Its closed-form minimiser is Umeyama’s (Umeyama 1991): form the cross-covariance and its singular value decomposition,

Σ=1nk​qkpk⊤=U D V⊤,  D=diag(d1,d2), d1≥d2≥0,

and let S=I when detΣ≥0 and S=diag(1,−1) otherwise, the correction that forbids the minimiser from “fitting” a reflection, which would silently mirror one trajectory onto the other and report a spuriously small residual. Then

R⋆=USV⊤,  s⋆=tr(DS)σp2,  t⋆=q−s⋆R⋆p.

Setting s≡1 in Eq. [eq:umeyama] recovers the rigid problem of Eq. [eq:kabsch], whose minimiser is the same R⋆ with t=q−R⋆p.

The three stages of trajectory alignment, and what each residual is entitled to claim. Removing the frame difference (stage 2) is mandatory: the raw separation measures only that two origins are arbitrary. Removing the scale as well (stage 3) is a choice, and it consumes a degree of freedom: the residual it leaves can no longer testify about scale, only about shape. Eq. [eq:residualgap] states exactly what is given up between stages 2 and 3.

The substantive point is that these two residuals are not independent readings. Writing εrig2 and εsim2 for the mean squared residuals at the respective optima, and substituting Eq. [eq:umeyamasol] into the expanded objective, both reduce to expressions in σp2, σq2 and tr(DS) alone:

εrig2=σq2+σp2−2tr(DS),  εsim2=σq2−tr(DS)2σp2.

Eliminating tr(DS)=s⋆σp2 between the two gives an identity that is, as far as the present work is concerned, the single most useful line of algebra in this chapter:

εrig2−εsim2=σp2 (s⋆−1)2

The improvement obtained by allowing a scale factor is therefore entirely accounted for by the scale error itself, weighted by the extent of the trajectory. It is never negative, which is why εsim≤εrig always and why a smaller similarity residual is not by itself evidence of anything. It also furnishes a self-consistency check on the plotted numbers, which the July 29 run passes: with σp=1.65 m and s⋆=1.2894 the right-hand side is 0.2280 m​2, while the left-hand side computed from the reported residuals 0.563 m and 0.297 m is 0.2288 m​2, agreement to the rounding of the printed figures.

Two consequences are worth stating plainly, because both bear on how Fig. 12.13 may legitimately be cited.

First, 47% of the post-rigid residual on this run was pure scale disagreement, not shape disagreement. Once the scale is removed the two estimators agree to 0.297 m RMS over a 15 m circuit, and panel 3 shows the scale-corrected curve dipping to 10−3 m near t≈45 s, where the two estimates coincide to within a millimetre. The estimators describe very nearly the same shape.

Second, and cutting the other way: the scale factor was fitted to MINS. MINS is another estimator, not ground truth. A reader shown two superimposed trajectories without being told that one was rescaled onto the other will read agreement into a figure where agreement was, in part, imposed. The plotting script therefore prints the caveat inside the metric panel whenever |s⋆−1|>0.03, and panel 2 retains the un-rescaled openVINS trace in pale grey so the magnitude of the correction stays visible. The rigid residual is reported alongside the similarity residual for the same reason. This is not a stylistic preference: by Eq. [eq:residualgap] the similarity residual has had precisely the scale information removed from it, so quoting it alone would be quoting a number constructed to be insensitive to the very error the campaign is trying to measure.

### Why cumulative path length and spatial extent give different ratios, and why only one of them survives a change of sampling rate

Panel 4 of Fig. 12.13 reports openVINS covering 90% of the MINS path length, while s⋆=1.2894 implies its trajectory is 1/1.2894=77.6% of the MINS size. Both numbers are correct. They measure different things, and the difference is instructive enough, and was initially misleading enough, to deserve a model.

Take an estimator sampling a true planar trajectory x(t) at rate f over duration T, with mean speed v, and returning

xk=x(tk)+εk,  εk∼N(0,σ2I2)  i.i.d.,

with n=fT samples. The two statistics behave completely differently under this noise.

Spatial extent is a variance about the centroid, so independent noise adds in quadrature and contributes a single additive term:

Eσx2=σx2+2σ21−1n →​ n≫1  σx2+2σ2.

The inflation does not grow with n. Sampling the same motion twice as fast leaves the estimated extent essentially unchanged.

Cumulative path length sums n increments, and each increment collects its own independent noise:

L=k=1n−1Δxk+Δεk,  Δεk=εk+1−εk∼N(0,2σ2I2).

In the small-noise regime σ≪Δxk=v/f, the component of Δεk along the step displaces the endpoint but averages out, while the transverse component, of variance 2σ2, lengthens the step at second order, a+δ≈a+δ⟂2/(2a). Summing the n steps and substituting Δxk=v/f and L=vT:

EL ≈ L+n σ2fv = L+σ2f2Tv = L1+(σfv)2.

The relative inflation is quadratic in the sampling rate. This is the whole of the discrepancy, and it carries an immediate operational rule: cumulative path lengths measured at different sampling rates are not comparable quantities, and comparing them is comparing noise levels rather than distances.

Why cumulative path length must not be compared across sampling rates. Both panels show the same physical loop with the same spatial extent; the right-hand estimator merely samples it faster. Every additional sample contributes its own noise excursion, so the measured path lengthens quadratically in the sampling rate (Eq. [eq:pathinflation]) while the extent is inflated only by the additive constant of Eq. [eq:extentbias]. The two ratios quoted in Fig. 12.13 differ for exactly this reason.

The model is not decorative: it caught a real defect in the plotting script. Path lengths were originally computed on the raw traces, at their native rates of 86 Hz (MINS) and 80 Hz (openVINS), and reported 16.78 m for both, an apparently perfect agreement, printed as a title above a panel whose two curves visibly separated. Recomputing on the common 20 Hz base, the base the panel actually plots, gives 15.29 m and 13.74 m. The raw agreement was a coincidence of two different noise levels inflating two different true lengths onto the same number, and it had been concealing a genuine 10% discrepancy.

Eq. [eq:pathinflation] can be inverted to recover the per-sample noise that produced the observed inflation, σ=(v/f)L/L−1, which yields 0.43 mm per sample for MINS and 0.63 mm for openVINS. The figure of 1.5× more per-sample position noise on openVINS is consistent with the qualitative behaviour recorded throughout this campaign, and with the residual structure of panel 7, where the post-alignment error is a pair of smooth ±0.5 m excursions rather than sample-to-sample scatter: that is, a slowly varying disagreement in trajectory, sitting on top of a much finer per-sample jitter.

### Ground-truth calibration: the first measurements on this platform not made against another estimator

Every quantitative statement in this chapter up to this point compares one estimator against another. Section 12.14.2 is explicit that this cannot establish which of them is right. The July 29 session added the missing ingredient: a tape measure and a protractor.

A calibration panel was added to the cockpit’s Trajectory tab, backed by tools/calibterrain.py for terminal use. The operator marks a start pose, drives a known manoeuvre, marks the end pose, physically measures what the robot actually did, and enters that measurement; the backend captures what each of the four sources (wheel odometry, MINS, openVINS, and the raw integrated gyroscope) believed had happened over exactly the same interval, and reports the correction factor

gsource=measured realitywhat the source believed,

averaged over repeated passes with its standard deviation, since a single pass conflates the systematic scale error being sought with the random start-pose and slip errors of that particular run.

The rotational manoeuvre is the more valuable of the two, because it measures a quantity that had never been observable on this platform. The gyroscope obeys

ωmeas=k ωtrue+b+η,

with scale error k, bias b and noise η. The static auto-calibration carried by imusanitizer.py estimates b from a stationary robot, but at rest ωtrue=0, so Eq. [eq:gyromodel] collapses to ωmeas=b+η and k disappears from the observation entirely. No amount of stationary data can determine it. This is the limitation the sanitizer’s own documentation had recorded as outstanding. Integrating instead over a turn of known amplitude, with the bias already removed,

Δθ=0T​ωmeas−b dt=k Δθtrue,  ⇒  k=ΔθΔθtrue=1g,

makes k directly observable from a single turn against a protractor.

Measuring both directions separately is what turns the result into a diagnosis rather than a number. Decomposing the two per-direction factors into their symmetric and antisymmetric parts,

g=12gL+gR,  δ=12gL−gR,

a sensor scale error is necessarily sign-symmetric and appears wholly in g, whereas a mechanical defect (differential wheel slip, a steering trim) reverses with the direction of travel and appears in δ. The two are therefore separable by measurement, and must be, since correcting an antisymmetric fault with a symmetric gain would worsen one direction while improving the other.

First ground-truth calibration pass, July 29. Factors are g=reality/belief as defined in Eq. [eq:calibfactor]; a factor below unity means the source over-reports. The gyroscope’s near-zero antisymmetric component identifies a genuine scale error, while the wheel odometry’s is an order of magnitude larger and is mechanical in origin. Single pass per direction: indicative, and superseded by the repeated campaign recorded as item 36.

| Source | gleft​ | gright​ | g (symmetric) | ¦δ¦ (antisym.) |
| --- | --- | --- | --- | --- |
| Raw gyroscope | 0.9344​ | 0.9313​ | 0.9329​ | 0.0016​ |
| MINS | 0.9274​ | 0.9316​ | 0.9295​ | 0.0021​ |
| openVINS | 0.9333​ | 0.9353​ | 0.9343​ | 0.0010​ |
| Wheel odometry | 0.9325​ | 0.8920​ | 0.9123​ | 0.0203​ |

Table 12.7 reports the outcome. The gyroscope over-reports rotation by 7.2% (k=1/g=1.072), and does so almost identically in both directions: |δ|/g=0.17%. By the argument above this is a scale error, and a ~gyroscale parameter was added to the sanitizer to carry the correction: defaulting to unity, that is, to no correction at all, until a repeated campaign supersedes this single pass. Both inertial estimators inherit the same ∼7%, as they must, since both integrate the same gyroscope. The wheel odometry is the outlier: its antisymmetric component is more than ten times the gyroscope’s, which is the signature of the left-side drift already characterised in this campaign, and it is precisely the fault that a global scale factor would entrench rather than remove.

A methodological note belongs here, because it cost a full set of measurements. The first calibration attempt entered the intended manoeuvre, ninety degrees, rather than the measured one. All four sources agreed on 121.7∘, and four independent sensors agreeing against a single entered number is not a sensor fault. The robot had overshot, and the exercise had measured how badly the platform tracks its own command rather than how badly its sensors track reality. The two are different quantities and only the second is a calibration. The interface was revised accordingly: the worked example in the input placeholder was changed from a round 90 to a deliberately unround 87.5, and a reminder that the arrival instant is frozen, so the operator may take as long as the measurement requires, is displayed alongside the field. A plausibility guard now rejects any entry differing by more than a factor of five from the median of what the sensors observed, on the reasoning that several independent sensors do not err together by two orders of magnitude, so such a disparity is a unit or typing error rather than a discovery.

## Field session, July 30: a thermal operating envelope, a native absolute-error architecture, and the calibration blocker that gates both

Two decisions shaped this session, and they pulled in opposite directions. The first was to cancel the planned fan installation and deliberately accept the thermal risk, in order to recover the colour stream that the beacon detector needs at working distance. The second was to stop aligning trajectories after the fact and instead measure error against a physical anchor. The first decision produced a bounded, quantified failure; the second produced an architecture that is complete but not yet validated, because a calibration defect discovered the same evening invalidates every dimensional quantity it would otherwise have produced. Both outcomes are reported here as measured, including the one that contradicts the hypothesis under which the session began.

### Thermal stress without active cooling: a characterised threshold rather than a demonstration of resilience

The premise of the run was that the July 28 scheduling fix (§12.13.1), SCHEDFIFO priority 25 on serialnode, together with the LimitRTPRIO=50 drop-in on leo.service without which the node refuses to start) had made the serial link robust enough to survive thermal throttling. If true, active cooling would be a comfort rather than a prerequisite, and the colour stream could be restored immediately.

The first half of the run supported that premise. With enablecolor restored and the Pi stabilised near 86 ∘C, the instrumented vitals held: the IMU delivered 82.8 Hz with 14 gaps per 20 s, and wheel odometry held 19.7 Hz, the same figure the scheduling fix had bought two days earlier, now sustained under an additional camera load. On the evidence available at that point, the fix was carrying the system through a thermal regime that had previously destroyed it.

The second half refuted the generalisation. Later the same evening the entire firmware chain fell silent, and the operator’s report was that neither manual nor autonomous driving responded. The measurements taken at that moment are given in Table 12.8.

Measured state at the two ends of the thermal envelope. The right column is not a degradation of the left one: it is a different regime, reached across roughly three degrees.

| Quantity | Hot run (stable) | Collapse |
| --- | --- | --- |
| Core temperature | ∼86 ∘C | 89.1 ∘C |
| getthrottled | 0xe0006 | 0xe0006 |
| ARM clock | 600 MHz | 600 MHz |
| Load average (4 cores) | not measured | 8.85​ |
| realsense2cameramanager | not measured | 185 % CPU |
| UART overruns (oe) | 137 000 (Jul 28) | 676 751​ |
| /firmware/wheelstates | 19.7 Hz | 0 Hz |
| /imu/dataraw | 82.8 Hz | 0 Hz |
| /jointstates | nominal | 0 Hz |
| serialnode state | synchronised | Lost sync, ∼15 s period |

The causal chain, and why it presented as a control fault. The colour nodelet consumed 185 % of a four-core budget already carrying two estimators’ worth of publication traffic, driving the load average to 8.85, a factor 2.2 oversubscription, while the clock remained pinned at its 600 MHz throttle floor. Under that starvation the PL011 FIFO is not serviced within the time a byte takes to arrive at 460 800 baud, and the overrun counter rose to 676 751, nearly five times the July 28 figure. rosserial responds to a corrupted frame by renegotiating the topic handshake, which under sustained overruns never completes before the next corruption, producing the observed 15-second Lost sync cycle. No firmware topic survives that loop.

The diagnostic value of the symptom deserves emphasis, because it is reusable. Manual and autonomous driving failed together, and that simultaneity is informative rather than alarming: the two modes share exactly one component downstream of the finite-state machine, namely /serialnode, the sole subscriber of /cmdvel. A fault that removes both at once therefore localises to the shared transport, and every hypothesis concerning the mission logic, the web interface, or the command bus can be discarded before it is tested. Confirmation was direct: rostopic info /cmdvel showed the publisher and subscriber correctly bound, while the backend’s own health block independently reported odom: false, wheels: false and batt: false. Commands were leaving the backend and never reaching the CORE2.

##### A deadline model for the overrun, and why the failure has a cliff.

The overrun counter is not a vague indicator of stress. It counts a specific, datable event: a byte arrived at the PL011 receiver while its receive FIFO was already full, so the hardware discarded it. That makes the mechanism a deadline problem, and it can be written down exactly.

The link runs at B=250 000 baud in 8N1 framing, so each byte occupies ten bit periods and the inter-byte interval is

τb = 10B = 10250 000 = 40 μs.

With a receive FIFO Nf entries deep, the kernel must service the port before Nf further bytes accumulate, which gives the hard deadline

Tdl = Nf τb = 32×40 μs = 1.28 ms.

An overrun occurs if and only if the service latency L exceeds Tdl. The exact FIFO depth matters less than it appears: halving Nf halves the deadline, and both values remain far below the scheduling delays discussed next.

The latency decomposes into two terms with different causes:

L = Lpreempt⏟when the task is chosen + fmaxf C⏟how long its work takes,

where C is the cost of the read path at full clock and fmax/f is the frequency penalty. This split is the whole story of the July 28 fix and of its July 30 limit. Promoting serialnode to SCHEDFIFO priority 25 attacks Lpreempt alone: it removes the wait behind a run queue that thermal throttling had made 8.85/4=2.21 times oversubscribed. It does nothing whatever to the second term, which throttling inflates by

fmaxf = 1500 MHz600 MHz = 2.5.

The fix therefore buys margin only while 2.5 C stays comfortably under 1.28 ms, which is precisely the sense in which it holds at 86 ∘C and fails at 89 ∘C.

The overrun as a missed deadline. Bytes arrive every τb=40 μs; the FIFO grants 1.28 ms of slack. Thermal throttling does not slow the arrivals, which are set by the CORE2’s clock, it only lengthens the service. The margin is therefore consumed from one side only.

##### From a miss probability to a collapse.

Treating successive deadlines as a Bernoulli sequence with miss probability p, the measured rate after recovery fixes p directly. Bytes arrive at B/10=25 000 per second, so fill events occur at

ν = B10 Nf = 25 00032 = 781 s−1,

while the counter advanced by 553 in 180 s, that is 3.07 s​−1. Hence

p = 3.07781 = 3.9×10−3,

a miss on roughly one deadline in 260. That figure is small, and on its own it would suggest a system merely degraded rather than one that had stopped entirely. The reconciliation is that rosserial does not tolerate isolated losses gracefully: a corrupted frame forces a full topic renegotiation, and the handshake itself must cross M≈2 000 bytes without a single overrun. Its success probability is therefore

Phs(p) = (1−p)M/Nf = (1−p)62.

At the measured p=3.9×10−3 this gives Phs=0.78: the link recovers on the first or second attempt, which is exactly what the targeted cycle achieved. Asking instead when recovery becomes improbable, Phs<0.05, gives

p⋆ = 1−0.051/62 = 4.7×10−2,

a factor of twelve above the measured value. This is the quantitative content of the three-degree envelope. The system does not degrade smoothly into uselessness; it crosses a threshold at which the recovery mechanism itself stops working, and thereafter every renegotiation fails for the same reason the previous one did. A twelvefold change in a latency margin is a small thermal step, and it separates a rover that loses the occasional message from one that answers no command at all.

Equation [eq:handshake]: probability that a rosserial topic renegotiation completes without an overrun. The function is smooth, but the system’s behaviour is not, because below the knee a failed handshake is retried successfully within seconds, and above it every retry fails. Twelve times the measured miss probability separates the two regimes.

Recovery, and what it did not fix. A targeted rosmon cycle of serialnode (not a stack restart, following the July 22 lesson (§12.9.3) that the surrounding infrastructure is rarely the culprit) restored the chain to 17.1 Hz on wheelstates, 18.7 Hz on wheelodom and 79.3 Hz on the IMU, with SCHEDFIFO priority 25 correctly reapplied by the launch wrapper. The recovery is nonetheless a reset, not a repair: three minutes later, at 88.6 ∘C and still throttled, the overrun counter had advanced a further 553 counts. The mechanism that produced the collapse was running at undiminished rate.

What this establishes, stated precisely. It would be convenient to record that the architecture proved resilient without active cooling. The measurements do not support that claim, and a reader with access to serialnode.log would find the contradiction in one command. What the session established is narrower and more useful: an operating envelope. The scheduling fix holds the serial link at 86 ∘C under full colour load, and does not hold it at 89 ∘C. The usable margin is therefore on the order of three degrees, with no headroom for ambient variation, and sustained operation above roughly 88 ∘C requires manual intervention at intervals of tens of minutes. A threshold measured on both sides is a stronger result than an absence of failure asserted from a single favourable sample, but it is a result that mandates the fan, rather than retiring it.

A correction to an earlier measurement. Appendix §17.17.1 records that disabling the colour stream produced zero thermal improvement, and concluded from this that the software levers were exhausted. That observation was real but its interpretation was wrong: colour had been disabled at driver level, so the stream being switched off was already absent and the measurement compared a configuration with itself. The 185 % CPU figure recorded above is the first direct measurement of what the colour nodelet actually costs, and it is the dominant term in the thermal budget. The lever exists; the July 30 decision was to spend it deliberately, which is a different matter from its not existing.

### From a posteriori alignment to a native absolute error: the beacon-anchored TF bridge

Every error figure in this campaign so far has been a residual after alignment. Both estimates and reference are expressed in frames whose relative pose is unknown, so the two point sets are registered (rigidly by Kabsch, or with a scale degree of freedom by Umeyama) and the residual of that registration is reported. §12.14.2 derived the exact price of the scale freedom,

εrig2−εsim2=σp2 (s⋆−1)2,

which makes explicit that a similarity fit absorbs a systematic scale error into a free parameter and reports what remains. The rigid fit removes one degree of freedom but still absorbs an arbitrary rotation and translation. In both cases the quantity reported is a residual after the estimator has been given the benefit of an optimal transform it never had access to during the run. That is a legitimate measure of shape agreement, and it is not an absolute error.

##### What alignment removes, written as a projection.

The objection can be made precise, and once it is, the size of the effect follows without any experiment. Let {pk}k=1N be the estimated positions, {pk} the reference, and ek=pk−pk the error we should like to report. Stack them into e∈R3N.

Alignment does not report e. It reports what survives an optimisation over the transform group. Writing the fitted rotation near identity as R≈I+[δω]×, the translation as δt and the similarity scale as s=1+δs, the post-alignment residual at index k is

rk = ek − δt + δω×pk + δs pk + O(δ2).

The bracketed term ranges over a linear subspace of R3N, spanned by seven vectors evaluated along the trajectory:

G = span(ei)k⏟3 translations, (ei×pk)k⏟3 rotations, (pk)k⏟1 scale,  i=1,2,3.

Least squares over these parameters is exactly orthogonal projection onto the complement, so what the campaign has been reporting is

εsim2=1N∥PG⟂ e∥2,  εrig2=1N∥PG6⟂ e∥2,

with G6⊂G the rigid subspace obtained by dropping the scale generator. Equation [eq:july30scaleprice] is the Pythagorean difference between these two projections, which is why it is an identity rather than an empirical finding.

##### Why this is not a small correction.

Since projection can only shorten a vector,

εsim2 ≤ εrig2 ≤ εabs2 = 1Ne2,

so every error figure reported in this campaign to date is a lower bound on the true error, never an overestimate. The gap is PGe2/N, and its size depends on how much of the error happens to lie in G.

That is where the argument bites. The dominant failure modes of a visual-inertial estimator are a slowly accumulating heading error and a slowly drifting metric scale. Their error signatures are δω×pk and δs pk, which are not merely close to G: by Eq. [eq:gaugespace] they are basis vectors of G. An alignment-based metric is therefore blind, by construction, to precisely the quantities the comparison exists to measure. A perfectly shaped trajectory rotated by ten degrees and stretched by five percent scores a residual of zero.

Alignment reports only the component of the error orthogonal to the transform group. The absorbed component is not noise: it carries the heading and scale drift that a visual-inertial comparison is meant to expose.

##### The bridge removes the projection rather than shrinking it.

Anchoring does not estimate a better transform, it removes the freedom to estimate one at all. The static beaconlink → odom transform Tbo is captured once, physically, before the run and held constant, so the reported quantity is

εabs2 = 1Nk=1N∥ Tbo pk−qk ∥2,  dimG=0,

where qk is the optically observed robot position in the beacon frame. No parameter is fitted at measurement time, the inequality chain of Eq. [eq:lowerbound] collapses to an equality at its right-hand end, and the heading and scale drift that G was hiding become observable.

The alternative implemented on July 30 is to give the system a physical anchor and measure against it directly. catkinws/src/leonavigation/scripts/carolustfbridge.py publishes the transform cameraframe → carolusraw from the live optical detection, together with a static beaconlink → odom transform captured once, at anchoring time, and thereafter held fixed. From these it derives two topics, /odominbeacon and /carolusinbeacon. Both carry the robot’s pose expressed in the beacon frame, reached by two independent paths: the first through the odometric and estimator chain, the second through the optical detection of a fixed object. Their instantaneous difference is an absolute error, containing no fitted transform, no scale parameter and no free variables: the alignment is performed once, physically, by the act of anchoring, rather than numerically and repeatedly by an optimiser with the answer in hand.

A constraint the implementation had to respect. A frame in a tf2 tree has exactly one parent. It is tempting to have the bridge publish the full body chain in the beacon frame, and an early revision did precisely that; the result was a second parent for baselink and a tree that tf2 resolves nondeterministically, announced only by intermittent TFOLDDATA warnings. The bridge therefore publishes only the camera-to-beacon observation and the static anchor, and exposes the body chain behind a ~publishbodychain parameter that defaults to false and emits a logerr when enabled. The same constraint drove the linearisation of the main tree to a strict odom → basefootprint → baselink chain (§17.17.10).

Status, stated without inflation. The bridge is written, executable, and deliberately absent from every launch file, so that it is started explicitly by an operator who intends to use it and cannot silently perturb an unrelated run. It has been executed and observed to publish. It has not been validated against recorded data: no run to date has recorded /odominbeacon or /carolusinbeacon, and the trajectory bags available at the time of writing contain only /mins/imu/odom, /ovmsckf/odomimu, /robotposefused and /firmware/wheelodom. No absolute-error figure is therefore reported in this document, and the architecture above should be read as a method awaiting its first measurement rather than as a result.

### The calibration defect that gates the measurement

The reason that first measurement was not simply taken is a defect found while verifying the detector’s outputs. The intrinsic parameters used by carolusastrobeerex, read live from the parameter server, do not correspond to the stream it is consuming.

Detector intrinsics against the intrinsics of the stream actually published. The two describe different optical configurations.

| Parameter | carolusastrobeerex | /camera/color/camerainfo |
| --- | --- | --- |
| Stream size | (calibrated at 1280×720) | 640×480​ |
| fx​ | 644.12​ | 380.35​ |
| fy​ | 645.29​ | 379.36​ |
| cx​ | 643.47​ | 320.40​ |
| cy​ | 372.52​ | 243.72​ |

The decisive entry is cx=643.47 on an image 640 pixels wide. The principal point lies outside the image altogether, which is not a tolerance question but a categorical inconsistency: the calibration was performed at 1280×720, where the D455 colour sensor delivers its full 90∘ horizontal field, and is being applied to a 640×480 stream which the driver crops to roughly 80∘. The two are not related by a scale factor, so no rescaling of the stored parameters can repair them.

The consequence is quantifiable without any experiment. A beacon imaged at the true optical centre, pixel (320,240), is interpreted by the detector as lying at

α=arctan​320−643.47644.12=−26.7∘,  β=arctan​240−372.52645.29=−11.6∘,

in azimuth and elevation respectively. A target dead ahead is reported nearly 27∘ off-axis. Every range and bearing derived from this projection, and therefore every position the bridge would express in the beacon frame, is invalid as a metric quantity. The six-degree-of-freedom readings observed on the cockpit during this session (of order 1.2 m in x and −0.55 m in y) are consistent, self-coherent and metrically meaningless.

##### Propagating the defect through the pinhole model.

Equation [eq:july30bearingerror] gives the bearing error at one pixel. The full consequence is better seen by propagating the wrong parameters through the projection itself. The pinhole model maps a point (X,Y,Z) in the camera frame to

u=fxXZ+cx,  v=fyYZ+cy,

and the detector inverts it. Two quantities matter downstream: how far the beacon is, and where it is laterally.

Range. For a beacon of known physical extent L subtending w pixels, similar triangles give Z=fxL/w. The estimated range therefore scales linearly with the assumed focal length, and the error is a pure multiplicative bias independent of distance, orientation and pixel position:

ZZ = fx′fx = 644.12380.35 = 1.693.

Every range this detector reports is inflated by 69.3 %. A beacon at one metre is reported at 1.69 m, and no averaging, filtering or repetition reduces the error, because it is not noise.

Lateral position. Combining Eq. [eq:pinhole] with Z=fxL/w gives

X = (u−cx) Zfx = (u−cx) Lw,

in which the focal length has cancelled. The lateral estimate is governed entirely by the principal point, and the induced offset

ΔX = (cx−cx′) Lw = − 323.07 Lw

is a constant translation of the whole scene, identical for every beacon position in the image. This is the most treacherous property of the defect: a rigid lateral offset produces a beacon track that is smooth, repeatable and internally consistent, and therefore looks exactly like good data. For an object of extent L subtending w=40 pixels, a plausible working figure at this range, Eq. [eq:lateralbias] predicts an offset of 8.08 L metres, which for a beacon 0.15 m across is 1.21 m. The cockpit reading observed during this session was x=1.18 m. The agreement should not be read as a confirmed diagnosis, since L and w were not measured at the instant of the reading, but it does establish that the observed value is of the same order as the artefact and cannot be separated from it.

The stored calibration describes a different camera from the one publishing frames. Its principal point falls below the sensor’s lower edge, so the assumed optical axis leaves the field of view entirely and a target on the true axis is reported 26.7∘ off it. The two cones also differ in aperture, which is why no rescaling of the stored parameters can reconcile them.

Because the range bias of Eq. [eq:rangebias] is multiplicative and the lateral bias of Eq. [eq:lateralbias] is additive, their combined effect on a beacon-frame trajectory is a similarity transform: a scaling by 1.693 composed with a fixed translation. That is a member of the very group G of Eq. [eq:gaugespace]. The irony is exact and worth stating plainly. The bridge exists to escape the gauge freedom that alignment hides errors in, and the uncalibrated camera injects an error lying squarely inside that same subspace. Until the intrinsics are corrected, an alignment-based comparison would silently absorb the defect while the absolute measurement would report it in full, so the two methods cannot even be cross-checked against one another.

This is why §12.15.2 reports no error figure. The architecture is sound and the anchor is physical, but the sensor model feeding it is wrong by a margin that dwarfs the quantity being measured. Re-calibrating the colour camera at its operating resolution is consequently the single blocking prerequisite for the entire absolute-error programme, and is recorded as such in Table [tab:openitems] and in §17.18.1.

## Field session, August 5: a rewritten guard, a diagnosed tilt, and the first rectangle closed

This session executed the rectangle protocol requested directly by the supervisor: a physical rectangle, taped and tape-measured at 6.405 m×3.455 m, driven once trusting MINS and once trusting openVINS, each compared against the tape measurement rather than against each other (§17.10.3–§17.10.5 give the operator-facing procedure; what follows is the reasoning and the mathematics behind what that procedure actually measured). Three findings came out of it, in the order they were forced by the data rather than the order that would read best: a load-bearing bug in the pose-fusion guard, found and fixed only because the rectangle test happened to drive through it; a tilt in the IMU mount that a decade of prior sessions never had reason to isolate; and, finally, the rectangle itself, closed cleanly by MINS and not at all by openVINS, for reasons the first two findings and the platform’s thermal envelope (§17.17.2) explain jointly.

### The pose-fusion guard: a false floor, then a true one

poseselector.py’s plausibility guard has existed since 2026-07-13 for a specific, narrow reason: during a camera dropout an estimator coasts on inertial propagation alone, and a doubly-integrated accelerometer bias can move the served pose by metres per second on a rover that tops out near 0.4 m/s. The guard compares each incoming pose against the last accepted one and freezes the served output the instant the implied speed exceeds a threshold,

vk = pk−preftk−tref,  vk>vmax=1.5 m/s ⇒ reject,

re-anchoring the correction transform once the source is calm again so the served frame stays continuous through the excursion. It had never been exercised by a full driven loop before this session, only by the camera-loss transients it was built for.

##### Symptom.

Driving the rectangle with openVINS as the served source, the cockpit’s trajectory display froze the instant the rover moved, recovered while stationary, and froze again on the next turn. /robotposefused dropped to ∼​2 Hz with the trail buffer stuck at a handful of points, and the guard’s own log was saturated:

[pose_selector] pose VINS invraisemblable (17.3 m/s) — pose servie GELEE
[pose_selector] source VINS calmee apres 0.0 s d’excursion — re-ancree

alternating several times per second. Nothing here is exotic: it is Eq. [eq:guardspeed] doing exactly what it was written to do, on a 17.3 m/s reading impossible for this platform. What was wrong was the diagnosis that followed, not the guard.

##### A hypothesis that measurement refused.

The first candidate explanation was the floor on the denominator of Eq. [eq:guardspeed], Δt=max(10−3 s, tk−tref): at 90 Hz the true sample interval is close to 11 ms, so if two timestamps ever landed within the 1 ms floor, an entirely ordinary few-centimetre step would read as tens of metres per second. Five hundred and ninety-two consecutive openVINS messages, captured with the rover stationary, refuted it directly: median Δt=11.00 ms, minimum 9.00 ms, zero samples under 4 ms, worst speed implied at the 1 ms floor only 0.3 m/s. The hypothesis is recorded here specifically because it did not survive contact with data — the discipline this report tries to hold throughout (§12.3) is to say so plainly rather than let a plausible guess stand in for a measured cause.

##### The real mechanism, found by capturing the rover in motion.

Sampling 2020 consecutive openVINS messages while the rectangle was actually being driven told a different story: timing stayed clean (median Δt=11.0 ms, minimum 9.0 ms, matching the static capture), but 8.3 % of consecutive-sample speeds computed by Eq. [eq:guardspeed] exceeded vmax, the single worst case a 4.23 cm step inside a 9 ms interval — 4.7 m/s by the formula, and a real, correctly-timed measurement. An MSCKF applies its visual-update corrections discretely, at each new keyframe; a few centimetres of legitimate correction landing inside one 9 ms sample interval is the filter working as designed, not the rover teleporting. Eq. [eq:guardspeed], evaluated between consecutive samples, cannot tell that step apart from the runaway it exists to catch: both are large numerators over a small, fixed denominator.

##### Two independent defects, two independent fixes.

Diagnosing the mechanism separated what had looked like one bug into two, addressed in two commits.

Latch tolerance. A single rejected sample latched the frozen state immediately, and the frozen branch published nothing at all — a bare return — so every consumer of /robotposefused (the cockpit display, the rosbag recorder, any rate-based health check) lost its input the instant the guard fired, which is the entire visible symptom above. Both are now additive changes only, applied without weakening Eq. [eq:guardspeed]’s acceptance rule: a sample over threshold is still never written into the trace, the correction transform, or the reference pose, so no rejected sample can ever reach the trajectory or a downstream consumer as if it were accepted. What changes is what happens to a rejection.

An isolated spike — one to two consecutive rejections, the discrete-update case above — no longer latches anything: the sample is discarded and the next one is judged on its own merits. Only a sustained run of spiketolerance=3 consecutive rejections (≈33 ms at 90 Hz) declares the source insane, at which point the last known-good pose is republished with its diagonal covariance forced to 106 — not a fabricated position, the last real measurement every consumer already held by default, now still arriving so a rate check can tell “source frozen, topic alive” apart from “node dead, topic silent”. Replayed against the driving capture as a pure decision-table exercise (Table 12.10): normal traffic keeps every sample flowing, an isolated spike no longer trips the latch, and a sustained run still does.

Guard state-machine replay, latch-tolerance fix only. The isolated case is the one that changed: it neither latches nor is silently invented — the sample is discarded, the topic keeps publishing.

| Scenario | Fresh | Held | Latched? |
| --- | --- | --- | --- |
| Normal drive (20 samples) | 20 | 0 | no |
| Isolated spike (1 of 20) | 19 | 1 | no (was: yes) |
| Sustained divergence (10 of 20) | 10 | 10 | yes (unchanged) |

The metric itself. The latch fix alone still evaluates Eq. [eq:guardspeed] between consecutive samples, so the same 4.23 cm/9 ms correction still individually fails the test every time it occurs; it simply no longer freezes the topic. The deeper fix replaces the denominator’s reference point: instead of the immediately preceding sample, compare against the oldest still-accepted pose inside a sliding window of duration T=0.1 s,

vk = pk−pk−wtk−tk−w,  tk−tk−w ≤ T,

falling back to the immediate predecessor while less than T of history exists (start-up, and immediately after a re-anchor, where the window is deliberately cleared: the frame just changed, so old entries are not comparable to the new one). The same displacement read against a longer baseline reads a smaller speed for exactly the same physical event, because a discrete correction of fixed size δ contributes δ/T once T exceeds the interval it occurred in, while a sustained runaway of speed v still covers vT within the same window — the window shrinks the correction’s apparent speed roughly in proportion to T, but leaves a true runaway’s apparent speed unchanged. The 4.23 cm case: 4.7 m/s at T=11 ms, 0.42 m/s at T=100 ms, comfortably under vmax.

Replayed on synthetic traffic built to match the measured statistics (0.3 m/s mean drift plus 2–4 cm discrete corrections at the observed rate, 900 samples at 90 Hz) and separately on a sustained 6 m/s runaway, Table 12.11 is the result: false rejections disappear, true divergence detection is unchanged.

The baseline-window fix (Eq. [eq:guardwindow]), verified by replay before deployment. A sustained runaway is caught identically at both baselines because it accumulates displacement proportionally to T, unlike a discrete correction of fixed size.

| Baseline T | False rejects (normal traffic, /900) | Detected (6 m/s runaway, /200) |
| --- | --- | --- |
| 11 ms (consecutive-sample) | 46 (5.1 %) | 199​ |
| 100 ms (Eq. [eq:guardwindow]) | 0​ | 199​ |

##### Why the window separates the two cases, in closed form.

The replay above is an empirical demonstration; the reason it works has a short derivation. Model a discrete correction of size δ landing at time tc inside an otherwise steady drive at speed v0: the position is x(t)=v0t+δ H(t−tc), with H the Heaviside step. For a window of duration T that contains tc, Eq. [eq:guardwindow] reads

vapparent(T) = |x(t)−x(t−T)|T = v0+δT,

which decreases monotonically in T and tends to the true speed v0 as T grows: the correction’s contribution is amortised over an increasingly long interval. A sustained runaway of speed vbad, by contrast, has x(t)=vbad t and

vapparent(T) = vbad,

independent of T entirely: there is no amortising it, because every sub-interval of the runaway is itself the runaway. Substituting the worst observed case (v0=0.13 m/s, δ=4.23 cm) into Eq. [eq:guardstep] gives the crossing point where the correction alone would still trip the vmax=1.5 m/s guard, T⋆=δ/(vmax−v0)≈30.9 ms — comfortably below the deployed T=100 ms, at which Eq. [eq:guardstep] evaluates to 0.553 m/s, roughly a factor of three under threshold rather than the bare δ/T=0.423 m/s a reader would get by dropping the v0 term.

The blue curve is Eq. [eq:guardstep] on the worst measured correction; the flat red line is Eq. [eq:guardramp] on a sustained 6 m/s runaway. Only the former crosses below threshold as T grows, which is the entire mechanism: nothing in the fix distinguishes “discrete” from “sustained” by pattern-matching, the window merely makes a one-off average out while leaving a persistent one untouched.

Deployed and re-measured live: /robotposefused publishing at 76–83 Hz through driven turns that previously froze it within the first second.

### An uncommanded IMU tilt

With the rover confirmed stationary — wheel velocities [0,0,0,0], zero /cmdvel on every axis — /mins/imu/odom still drifted 0.75 m in 15 s (50 mm/s) and /ovmsckf/odomimu 11.4 m in the same window (765 mm/s), both far outside protocol T1.1’s 1 mm/s pass criterion (§17.13.4). The gyroscope was clean at rest (0.0002 rad/s about every axis), so the search moved to the accelerometer.

##### Magnitude checks out; direction does not.

Thirteen samples of /imu/dataclean, stationary, gave a=(1.1356, 0.6689, 9.6810) m/s​2 (σ=0.015,0.014,0.019). Its norm,

a=1.13562+0.66892+9.68102=9.7703 m/s2,

sits 0.0197 m/s​2 (0.2 %) below Melbourne FL local gravity (9.790 m/s​2, used throughout this report). That confirms accelscale=1.1186 (§17.9) is doing its job: the much larger, already-documented magnitude defect it corrects for (8.7525 m/s​2 raw, −10.6 %) is not what is happening here. This is a second, independent finding: the sensor reports the right amount of specific force, from the wrong direction.

##### Decomposing the direction.

A stationary IMU measures −g expressed in its own frame; with the yaw component unobservable from a single static reading, the roll and pitch it implies follow the standard aerospace decomposition,

roll=atan2(ay, az),  pitch=atan2​−ax, ay2+az2,

in the REP-103 body convention (x forward, y left, z up). Substituting a:

roll=atan2(0.6689, 9.6810)=+3.95∘,  pitch=atan2(−1.1356, 9.7048)=−6.67∘.

Six to seven degrees of tilt is large enough to see by eye on this platform, and it is not new: the raw stationary reading that established the magnitude defect on 2026-07-28 (§12.3.3, a=(1.045, 0.604, 8.664) m/s​2, before accelscale) decomposes by Eq. [eq:rollpitch] to +3.99∘ roll and −6.86∘ pitch — within 0.2∘ of today’s reading on eleven days’ separation and, since accelscale is a uniform multiplier that cannot change a vector’s direction, on both sides of that correction. That earlier session flagged the tilt explicitly as “a separate question” and did not pursue it, correctly ruling it out only as an explanation for the magnitude defect (§12.3.3’s own norm-invariance argument). The two independent measurements together make a transient cause — a knock, a temporarily uneven surface — unlikely: whatever is producing this tilt has held steady for at least eleven days.

Two causes remain open, distinguishable by a 180∘ yaw-in-place test not yet performed: if the sign of the tilt flips with the turn, the floor (or whatever the rover is resting against) is not level; if it does not, the IMU is mounted at an angle inside the chassis that the URDF — which declares identity rotation, RPY=(0,0,0), between baselink and imuframe — does not know about. Either way, an uncorrected specific force this size, doubly integrated, accounts for the drift measured above, and the check runs cleanly in both directions. Forward, a constant residual a that a filter fails to null integrates twice into

d(Δt) = 12 a Δt2,

so guessing a=0.1 m/s​2 predicts d(15)=12(0.1)(15)2=11.3 m. Backward, inverting Eq. [eq:doubleint] on the openVINS drift actually measured, d=11.4 m over Δt=15 s, gives

a = 2dΔt2 = 2(11.4)152 = 0.1013 m/s2,

recovering the guessed value to three figures: 0.1 m/s​2 was not a round number chosen for the illustration, it is what the observed drift itself implies.

That figure is worth contrasting with the raw specific force the tilt alone would put on the x-axis, gsin(6.67∘)≈1.14 m/s​2 — an order of magnitude larger than the 0.10 m/s​2 that actually leaked into drift. The difference is not a discrepancy: a constant tilt is exactly the kind of slowly-varying error a filter’s own attitude and bias states can absorb, given any excitation to observe it against (rotation, the camera, ZUPT resets), so only whatever fraction the estimator fails to track shows up as uncorrected drift. That residual fraction is 0.10/1.14≈9 % of the raw tilt-induced force here — consistent with an estimator that is mostly, but not completely, compensating for a bias it was never told about explicitly.

### Same-day ground-truth calibration: an asymmetry, not a scale error

A single pass of the cockpit’s ground-truth panel (§17.13.3) was run the same day, each maneuver measured once against every source simultaneously (Table 12.12). One pass falls short of the three the protocol itself requires before trusting a number (§17.13.3 again), but the pattern across the three maneuvers is already informative independent of that caveat.

Single-pass ground-truth calibration, 2026-08-05. Entries are measured/real: 1.000 is exact agreement.

| Maneuver (measured/real) | Wheels | MINS | openVINS | Raw gyro |
| --- | --- | --- | --- | --- |
| 90∘ left | 1.0586​ | 1.0059​ | 0.9986​ | 0.9932​ |
| 90∘ right | 1.1473​ | 1.0607​ | 1.0435​ | 1.0470​ |
| 1 m straight | 0.9685​ | 0.8922​ | 1.8660​ | — |

A single multiplicative scale factor cannot explain the rotation rows. MINS reads 1.0059 left and 1.0607 right; correcting the right-turn reading with a single gain g=1/1.0607 would move the left-turn reading to 1.0059/1.0607=0.9484, trading a 0.6 % error in one direction for a 5.2 % error in the other. Decomposing the pair into a symmetric part (true scale, same sign both directions) and an antisymmetric part (direction-dependent, cancels on average),

scale=12(gL+gR)−1,  asym.=12(gL−gR),

applied to gL=1.0059, gR=1.0607 gives

scale=12(1.0059+1.0607)−1=+3.3 %,  asym.=12(1.0059−1.0607)=−2.7 %,

so the asymmetry is not a rounding artefact, it is comparable in size to the scale error itself, consistent with the rigid-wheel skid-steer asymmetry already measured on this platform’s differential turning (§17.17.4).

Both readings overshoot the same 90∘ real target (grey, dashed, same reference on both panels), but by visibly different amounts (angles exaggerated here for legibility, not to numerical scale): a single corrective gain that pulls the right-turn ray back to 90∘ would overshoot in the other direction on the left-turn ray, and vice versa, which is the arithmetic of Eq. [eq:asymval] already showed.

The straight-line row is the sharpest single number of this session: openVINS reported 1.8660 for a metre actually driven, an 86.6 % over-read on the one maneuver where MSCKF has no rotation to fall back on and depends entirely on translational parallax — exactly the geometry the thermal IMU gaps of §12.16.4 degrade first.

### Thermal envelope during this session

The rectangle attempts spanned the full width of the operating envelope §17.17.2 already documents at 86 – 89∘C, measured directly rather than inferred from downstream symptoms (Table 12.13).

Stopping the camera nodelet (measured at 198 % CPU alone) and waiting recovered the full 1.5 GHz clock and halved the IMU gap rate; restarting the camera immediately began eroding both again. 0xe0006→0xe0008: the frequency-capped and currently-throttled bits cleared, the soft thermal-limit bit did not.

|  | Before cooldown | After (camera off, ∼​2 min) | After camera restart |
| --- | --- | --- | --- |
| Temperature | 89.6∘C | 82.3∘C | 85.2∘C |
| CPU clock | 600 MHz (capped) | 1475 MHz | 1280 MHz |
| getthrottled | 0xe0006 | 0xe0008 | — |
| IMU gaps / 20 s | 20​ | 10​ | — |

Two things follow from this table, neither of them new in kind but both freshly measured. First, the 20-gap reading sat at 80 % of tools/hotrunmonitor.py’s own 25-gap abort threshold: the rectangle attempts that produced this session’s openVINS divergence were not on the edge of the documented envelope, they were inside it, close to its boundary. Second, the recovered clock eroded back toward the cap within roughly two minutes of the camera resuming, confirming again (§17.17.2) that a stopped-camera pause is a genuine but short-lived recovery, not a fix — active cooling remains the only durable answer.

### The rectangle, closed

MINS against the taped 6.405 m×3.455 m rectangle, 2026-08-05, produced by plotrectangletest(’test1’, 6.405, 3.455, ’mins’) (§17.10.3). Bounding box 6.176×3.919 m (−3.6 % / +13.4 %); loop closure 0.449 m over a 23.35 m path (1.9 %); heading drift over the full loop 5.1∘.

The length error is small and one-sided (−3.6 %); the width error is larger and one-sided the other way (+13.4 %). Table 12.12 gives the more likely reading over a pure scale error: this platform’s turning asymmetry (§12.16.3) accumulates differently depending on which way the rover turns at each corner, so a rectangle driven consistently anticlockwise does not distort its two axes equally. A coarse, purely qualitative read of the raw trajectory against elapsed time (Table 12.14; not a validated per-corner algorithm, offered only as a description of what the logged points show) is consistent with that reading: distance from the start point falls to a local minimum near t=159 s, just after the rectangle’s third corner, before drifting further to the final 0.449 m over the last leg.

Raw trajectory progression, 5 of 9 sampled instants. Qualitative motivation for §12.16.6 only.

| t (s) | x (m) | y (m) | dist. from start (m) |
| --- | --- | --- | --- |
| 0 | −0.01​ | −0.00​ | 0.01​ |
| 79 | +5.79​ | +3.38​ | 6.71​ |
| 133 | −0.10​ | +3.25​ | 3.25​ |
| 159 | −0.06​ | −0.21​ | 0.21​ |
| 213 | +0.06​ | −0.44​ | 0.45​ |

openVINS did not close the same rectangle. The same physical loop, recorded in the same rosbag, gave a 675.28×96.01 m bounding box and a 2760.55 m path against a 19.72 m measured perimeter — 140× over. Figure 12.31, produced by comparetests.m, puts both laps on one figure.

comparetests(’test1’,’test2’): MINS (Test 1, trusted) and openVINS (Test 2, trusted) against the same physical loop. Left: both trajectories, one axis. Right: distance from the starting point over time — openVINS climbs to a plateau near 40 m within the first 20 s, then jumps once, near t≈190 s, to the 635 m range and holds there for the remainder of the recording.

##### A single scale factor cannot rescue this trajectory, and the check is worth showing in full.

If openVINS’s error were pure uniform scale, some constant k would bring every axis into agreement simultaneously. Solving for k from each dimension independently,

kL=6.405675.28=0.00948,  kW=3.45596.01=0.03598,  kP=19.722760.55=0.00714,

gives three factors disagreeing by up to 3.8× (kW/kL). Applying kL alone, the value that makes the length exact by construction, to every axis: length lands on target by definition, but width becomes 0.911 m against a 3.455 m target (−73.6 %), and the loop-closure gap becomes 6.028 m — thirteen times worse than MINS’s 0.449 m, not better. This is a deformation of the trajectory’s shape, not a resizing of it, and no single number corrects it (§12.14.2 makes the identical point about scale hiding inside an alignment-based metric, from the other direction).

Nearly identical heading drift over the two independent loops — 5.1∘ MINS, 5.9∘ openVINS, from the same comparetests.m run above — says the same thing from a second angle: an estimator whose orientation tracking were fundamentally broken would show it here too. openVINS’s failure this session is concentrated in scale, not in heading, matching the open question already on record for this platform (§12.14.2: “openVINS’s scale disagrees with itself between runs”), and matching a result this platform has produced under better conditions: 0.025 m loop closure, 0.1 %, on 2026-07-29 (§12.12.7), the same estimator, the same rover, an 86∘C Pi rather than 89–90∘C and a lit, textured floor rather than the dim underside of furniture. The comparison in this section is between a completed acquisition and a failed one, not a fair contest between two algorithms on equal footing; §[tab:openitems] records what completing it fairly would require.

### A second absolute-error method: paired sightings, no anchor required

§17.13.5’s beacon bridge gives a continuous absolute error once anchored, but is gated on the colour camera’s intrinsics, which remain uncalibrated at operating resolution (same section, opening trapbox): its output today is smooth, repeatable, internally consistent and metrically meaningless, the same phrase this report already uses for the identical failure mode in a different context (§12.15.3). Rather than wait on that recalibration, this session added a second, complementary method built on hardware already trusted for a different purpose: the infrared stereo pair, already Kalibr-calibrated to 0.20–0.23 px (§17.13.7) because openVINS and MINS both depend on it, and already running AprilTag detection at ∼​5–6 Hz for the corridor markers (§17.17.11).

##### The measurement principle.

Suppose a fixed AprilTag is sighted twice, at times ta and tb, and its position in the camera frame is the same at both instants to within a small tolerance ε: c(ta)−c(tb)≤ε. Because the tag has not moved, this means the camera was in the same place at both instants, to the same tolerance. If p denotes the pose an estimator reports, a perfect estimator would then satisfy p(ta)=p(tb); the observed difference,

εdrift = p(tb)−p(ta),  given  c(ta)−c(tb)≤ε,

is drift accumulated between the two sightings. Crucially, Eq. [eq:tagdrift] never needs the tag’s true position in the world, nor the camera–IMU extrinsic connecting the detection to baselink: both would be needed to say where the beacon is, but Eq. [eq:tagdrift] only ever asks whether the rover was in the same place twice. This is the same relation the ground-truth panel’s rotation maneuvers already exploit in one dimension (depart and return to a marked heading); here it is a free-standing check at any point in a longer loop, not only at the loop’s own start and end.

##### Why the third corner, and why an observation, not a correction.

Table 12.14 places the point of closest apparent approach to the start near the third corner, one leg before the final, largest-error segment; a fixed reference there can tell whether that near-closure was genuine convergence or coincidence, which the single end-to-end loop-closure number in §12.16.5 cannot. The tag is therefore placed to be sighted once per lap at that corner, across several laps of the same rectangle.

What Eq. [eq:tagdrift] is not is a way to zero the estimator’s state at that corner. Doing so was considered and rejected: snapping the pose to a known value at each corner would make the driven trajectory match the rectangle by construction, exactly the failure this report already names when it happens elsewhere — “smooth, repeatable, internally consistent and metrically meaningless” (§17.13.5) applies here word for word. /tagdetections is instead recorded into the same detached rosbag as every other topic (ROBUSTRECTOPICS, leobackend.py) purely as an observation channel, wired into nothing that reads or writes the estimator’s state. tools/tagdrift.py performs Eq. [eq:tagdrift] entirely offline, after the fact, pairing sightings of the same tag ID separated by at least 5 s (so a sighting is never compared against itself) and within ε=5 cm of each other in camera coordinates by default. Verified against a synthetic pair of sightings carrying a known 0.264 m drift before being run on real data: the tool reads back 0.264 m exactly. Camera-frame AprilTag positions also feed plotrectangletest.m’s figures directly, marking both where the rover stood at each sighting and, separately, the tag’s own deduced position — the latter derived through the documented tape-measure camera extrinsic (Timucam, §17.10.3) rather than a Kalibr solution, so it is offered as a visual aid, while the spread between repeated deductions of one physical tag remains meaningful regardless: the tag has not moved, so any spread is drift by Eq. [eq:tagdrift], independent of how well the extrinsic locates it in an absolute sense.

This method has not yet measured anything Everything above is implemented, tested against synthetic data with a known answer, and deployed: the detector, the recording, the CSV format, and tagdrift.py itself. What has not happened yet is placing a tag at the rectangle’s third corner and driving multiple laps past it — every number in Eq. [eq:tagdrift]’s worked example above is synthetic, chosen to exercise the tool, not a field measurement. The first real run belongs in this trapbox once it exists, not before.

## Field session, August 11: active cooling installed, item 11 recurs a second time, and a live serial desync caught mid-session

Two threads ran through this session, one physical and one architectural, and they turn out to be the same argument made twice. The physical thread closes item 38 (§12.15.1): active cooling was installed and the thermal envelope re-measured against the exact 86–89.1 ∘C boundary characterised on July 30. The architectural thread is that three independent PC-side processes were found, on the same evening, holding a ROS-graph state that predated the robot’s current boot while still looking alive to every presence check — the identical defect item 11 already named in a narrower form, recurring for the second time in exactly the pattern item 10 established in §12.9.3: not a new bug, a prediction the registry had already made, confirmed by evidence collected independently of it. A fourth, unrelated fault, a live UART checksum storm, surfaced while re-verifying MINS for this very section and is reported mid-diagnosis, unresolved, rather than held back for a tidier write-up.

### Active cooling: installation

Two 5 V fans were wired directly to the Raspberry Pi’s GPIO header, in parallel, each with its own independent point-to-point connection rather than a shared spliced junction: four wires, four pins, no solder joint between them. Pins 2 and 4 are both unregulated taps off the Pi’s main 5 V rail (unlike the logic GPIO pins, which are current-limited and unsuitable for a motor load), and pins 6 and 9 are both ground. Pin 8, physically adjacent to pin 6 on the same row, was deliberately avoided: it carries UART TXD, not ground, and is the one wiring mistake the layout makes easy.

The four direct connections used: pins 2 and 4 (+5 V) to each fan’s red lead, pins 6 and 9 (GND) to each fan’s black lead, independently, no shared junction. Pin 8 (right column, one row below pin 6) carries UART TXD, not ground — the adjacent trap the layout invites; pin 9 (left column) is the one connection that changes side.

The two fans’ current draw was not measured against a datasheet, and is recorded here as an open quantity rather than assumed benign: the indirect evidence is that vcgencmd getthrottled read 0x0 on every check made after power-on, under both the light and heavy loads of §12.17.2, and bit 0 of that register (under-voltage now) together with bit 16 (under-voltage since boot) are both part of that word and both clear throughout. That is evidence the added load has not collapsed the 5 V rail, not a substitute for a measured current budget.

### Thermal verification against the July 30 envelope

§12.15.1 established a boundary, not a single number: the scheduling fix held the serial link at 86 ∘C under full colour load and lost it at 89.1 ∘C, with ∼3 ∘C of usable margin and “no headroom for ambient variation”. That boundary is the correct reference for tonight’s measurement, taken twice, at opposite ends of load.

The headline number: up to 30.6 ∘C recovered, and the clock never dropped Before active cooling, this exact rover collapsed at 89.1 ∘C (§12.15.1): every firmware topic silent, ARM clock pinned at its throttle floor of 600 MHz. With the two GPIO fans installed, the coldest reading taken tonight was 58–59 ∘C; the hottest, taken under the heaviest simultaneous CPU load this rover has been driven under to date (a 31-node full-stack restart), was 72.5 ∘C. Both are measured against the same vcgencmd instrumentation used throughout this report, not extrapolated:

89.1 ∘C → 58–59 ∘C (–30.6 ∘C, light load)
89.1 ∘C → 72.5 ∘C (–16.6 ∘C, heaviest load ever recorded on this rover)

The ARM clock read 1500 MHz, its undivided maximum, on every check made tonight, light load and heavy alike. Before cooling, this rover had never once sustained a full colour-camera load at that clock speed for more than a few minutes.

The same rover, same instrumentation, before and after the two GPIO fans. Axis starts at 50 ∘C to make the band that matters legible; every bar is a real reading, not an extrapolation. Both post-cooling bars sit below the July 30 stable floor, not just below its collapse ceiling.

Thermal state with active cooling, at two operating points, against the July 30 no-cooling collapse (Table 12.8).

| Quantity | July 30, no cooling | Aug 11, light load | Aug 11, heavy load |
| --- | --- | --- | --- |
| Core temperature | 89.1 ∘C | 58–59 ∘C | 72.5 ∘C |
| getthrottled | 0xe0006 | 0x0 | 0x0 |
| ARM clock | 600 MHz | 1500 MHz | 1500.3 MHz |
| Load average (4 cores) | 8.85​ | not logged | 6.84​ |
| Oversubscription λ/4 | 2.21×​ | – | 1.71×​ |
| Observed load | 185 % colour nodelet alone | realsense2cameramanager at 111 %, 30/33 nodes, 2 min window | full-stack restart: 31 ROS nodes, MINS + openVINS + camera initialising together |

Why the frequency term drops out, not just the number. §12.15.1 decomposed the UART service latency as

$$\begin{equation}
    L \;=\; L_{\mathrm{preempt}} \;+\; \frac{f_{\max}}{f}\,C
    \tag{\ref{eq:latencysplit} revisited}
    \end{equation}$$

and identified the second term, inflated by throttling to fmax/f=1500/600=2.5 (Eq. [eq:freqpenalty]), as the mechanism the July 28 scheduling fix could not touch. With the clock now reading f=1500 MHz on every check made tonight, under load as well as at rest,

fmaxf = 15001500 = 1,

which is not a smaller penalty than the 2.5× figure that produced the July 30 collapse: it is the term’s identity element, no penalty at all. Every scheduling-side margin the July 28 fix bought is now spent against C alone, never inflated by clock throttling, which is the specific mechanism the fans were installed to remove, not merely a coincidental temperature drop.

Margin, stated in the July 30 units. Writing M86 and M89.1 for the remaining margin to the envelope’s stable floor and collapse ceiling respectively,

M86(T)&=86.0−T,&M89.1(T)&=89.1−T,

the light-load reading gives M86≈27–28 ∘C and M89.1≈30–31 ∘C; the heavy-load reading, the highest simultaneous CPU demand observed on this rover to date, gives M86=13.5 ∘C and M89.1=16.6 ∘C. Even at that peak, the margin to the point where the July 30 session lost every firmware topic is more than five times the width of the envelope that was previously found to separate a stable rover from one that answers no command at all.

The matched-load A/B item 38 actually asked for has not been run Item 38’s required action was explicit: “re-measure the envelope after cooling; the correct A/B experiment is overrun rate at fixed load, not temperature alone.” Tonight’s heavy-load figure is a different profile (a 31-node simultaneous stack restart) from July 30’s (185 % on a single colour nodelet, sustained), so the two oversubscription figures in Table 12.15 are informative but not the controlled comparison the item calls for. The correct remaining experiment is to restore the exact July 30 load, colour stream enabled, nothing else changed, and compare the UART overrun counter (Eq. [eq:missprob]’s 3.07 s−1 figure) against the same window with cooling installed. Every reading above is real and was taken, not assumed; what has not happened is holding the one variable item 38 asked to isolate.

### A reboot-survival ghost, three times over: a formal staleness test

Restoring the robot’s connectivity after tonight’s reboot surfaced the same failure shape three separate times, in three unrelated processes, before it was recognised as one mechanism rather than three incidents. Each time, an operator-visible symptom (missing telemetry, an unresponsive cockpit switch, a manual-drive command that never arrived) was traced not to a crashed process but to a live one: a PC-side ROS node whose OS process had never exited, and whose ROS-graph registration, publisher handles and topic subscriptions were therefore still those it held against the previous robot boot. Every presence-only liveness check, pgrep, rosnode list, a bound TCP port, reports such a process as healthy, because by every test those checks make it is.

##### A criterion that needs no tuning.

Let t be the instant of a check, U(t) the robot’s own uptime read at t (cat /proc/uptime over SSH), and EP(t) the elapsed time since a PC-side process P started (ps -o etimes). Any ROS-graph state P holds was necessarily established no earlier than the last time the robot’s own rosmaster came up, so

$$\begin{equation}
    E_P(t) \;>\; U(t) \quad\Longrightarrow\quad P \text{ is stale relative to the
    current robot session.}
    \label{eq:staleness}
    \end{equation}$$

The implication runs one way only, deliberately: EP(t)≤U(t) does not certify P healthy, it only fails to convict it by this test, since a process can start fresh after a reboot and still fail for an unrelated reason. Eq. [eq:staleness] is exactly the ad hoc comparison made three times below, generalised into a check with no free parameter, in contrast to item 11’s original recommendation of an unspecified “maximum grace duration” to tune.

Three processes found stale by Eq. [eq:staleness] on the same evening, none of them crashed.

| Process | EP​ | U (at check) | Resolution |
| --- | --- | --- | --- |
| leobackend.py | 922 s | a few minutes | killed and relaunched; survived a duplicate-registration race (shutdown request: [/leobackend] Reason: new node registered with same name on the losing instance), surviving instance confirmed publishing /mission/telemetry at 9.985 Hz |
| poseselector | absent from rosnode list entirely, not merely stale | ≈4 min | tools/restartstack.sh |
| minssubscribe | 1085 s | ≈4 min | tools/restartstack.sh; ended TIMEOUT init MINS, item 40 |
| rosbridgeserver (:9090, LAN) | 11 051 s (≈3h04) | 28 min | kill -9, relaunched via startweb.sh; verified with a real HTTP/1.1 101 Switching Protocols handshake afterward, not just a bound port |

Elapsed time since process start (EP, horizontal bars) against the robot’s own uptime at the moment of each check (U). A bar that starts before the reboot line and is still being read afterward satisfies Eq. [eq:staleness]: it is provably stale, independent of whether it still responds to a presence check.

Item 11, recurring for the second time. Item 11 already named this defect, narrower: “a stuck rosbridge TLS process was left alone for hours because idempotency has no time ceiling” (§12.8), found on the TLS bridge, port 9443. Tonight’s rosbridgeserver ghost is the identical defect on the sibling plain bridge, port 9090: startweb.sh’s own guard, pgrep -f "name:=rosbridgeserver", matched the three-hour-old process and printed “en cours de démarrage --- patience” on every one-minute leowatchdog.sh cron tick, exactly the unbounded grace period item 11 already flagged, on the other bridge. Following the precedent set in §12.9.3 when item 10 recurred, this is recorded as an update to item 11 rather than a new item: the same blind spot, confirmed a second time, independently, on the bridge item 11’s own fix had not yet reached.

### A live misdiagnosis, corrected before it reached the code

Restoring the manual-drive path surfaced a second rosbridge subtlety, and it is recorded here including the wrong turn, in keeping with this report’s practice of stating a correction rather than silently replacing it (cf. the July 30 colour-stream correction, §12.15.1). The cockpit is reachable two ways, shown in Figure 12.35: a LAN client hits the plain bridge on port 9090 directly, while the public Cloudflare domain proxies to a TLS bridge on port 9443. Both are the same rosbridgeserver code; both are launched with a websocketexternalport parameter that autobahn uses to validate the incoming HTTP Host header’s port before completing the WebSocket handshake.

Two independent bridges reach the same ROS graph. cloudflared does not preserve the browser’s public Host header for this ingress rule; it rewrites it to the literal origin address it dials (127.0.0.1:9443), which is why the TLS bridge’s externalport is set to 443 — the public port implied when a real browser’s Host header carries no port at all — and not to 9443.

The live symptom, failing WebSocket opening handshake (’port 9443 in HTTP Host header ... does not match server listening port 443’), appeared in the tunnel’s log at almost exactly one-minute intervals. The first hypothesis was that this was real, tunnelled browser traffic being rejected, and the corrective action taken was to relaunch the tunnel without externalport. That was wrong, and startweb.sh’s own comment at the launch site says why: cloudflared forwards this ingress rule’s browser requests with Host: cockpit.leo-rover-... (no port, 443 implied), so removing the parameter would have broken real traffic to fix a false signal. The true source of the one-minute-interval log lines was leowatchdog.sh’s own liveness probe (§12.8), which deliberately sends Host: 127.0.0.1:%d and, by its own long-standing comment, treats the resulting rejection as proof of life (“Un 400 suffit”). The mistaken fix was reverted before being left in place, and the true fault (§12.17.3’s rosbridgeserver ghost) was found afterward by the same elapsed-time-versus-uptime comparison used throughout this section. A realistic handshake, Host header without a port, TLS, against the live port, was then used to confirm the fix directly rather than inferring it from the absence of further log lines:

HTTP/1.1 101 Switching Protocols (both bridges, re-tested independently)

### An unresolved fault, caught mid-verification

tools/restartstack.sh completed with TIMEOUT init MINS (item 40, below), and a first check afterward found minssubscribe registered but /mins/imu/odom silent for a 10 s window while poseselector continued reporting posesource: "MINS", i.e. the estimator currently selected for pose fusion was not producing pose. Re-checked directly while writing this section, the picture sharpened: /firmware/wheelstates was also silent for a 10 s window, yet rosnode info /firmwaremessageconverter showed it correctly subscribed to /firmware/wheelstates, /firmware/wheelodom and /firmware/imu, all three inbound from /serialnode, unlike the July 22 recurrence of item 10 where the subscription itself was missing. This is a different failure shape from that one: the graph wiring is correct and the data is simply not arriving. A single wheelstates message did arrive on a longer window, carrying non-zero wheel positions, evidence the robot has driven some distance tonight and that the link is not fully dead, only degraded. journalctl -u leo for the preceding five minutes was, at the moment of writing, filling with

/serialnode: [SerialClient.run]: wrong checksum for topic id and msg

in a sustained burst, the same signature Eq. [eq:missprob] quantified on July 30 and the same one §12.9.3 traced to a UART desync. Both of those earlier occurrences were tied to thermal throttling starving serialnode of scheduling time; tonight’s core temperature and ARM clock (Table 12.15) rule that specific mechanism out directly, since the clock has not left 1500 MHz. A second candidate, not established here, is physical: the checksum storm was first observed while the rover was actively being driven, and the UART link runs over a board-to-board connector that a stationary rover’s earlier characterisation would not have exercised. This is recorded as a hypothesis, not a finding; distinguishing it from a third, unidentified cause requires repeating the July 30 protocol, overrun count against a fixed load, this time with motion as the controlled variable instead of temperature.

### What this session establishes

Active cooling closes the mechanism, not just the number: with the ARM clock pinned at 1500 MHz under every load tested so far, the frequency term that made the July 28 scheduling fix fail above 86 ∘C (Eq. [eq:aug11nopenalty]) cannot recur, though the exact matched-load overrun comparison item 38 specified remains to be run. Independently, three processes surviving a robot reboot while presence checks called them healthy is now a named, reusable defect with a tuning-free test (Eq. [eq:staleness]), confirmed a second time on a bridge item 11 had not yet reached, in the same pattern item 10 established a precedent for in §12.9.3. A diagnostic habit produced a genuine cost tonight, a real misdiagnosis mid-session, and also caught it before it reached the running system, which is the outcome the habit is for. What remains open, honestly: item 38’s controlled A/B, and a live checksum storm whose cause is a hypothesis, not yet a finding.

## Field session, August 13: the matched-load A/B, finally isolated

Item 38 has carried the same open action since July 30: hold the load fixed and compare the overrun rate, not the temperature. §12.17.2 came close but missed on its own terms, by its own admission — the Aug 11 heavy-load figure was a 31-node stack restart, a heavier and different profile from July 30’s sustained single-nodelet load, so the two oversubscription figures it produced were suggestive, not the controlled comparison the item specified. This section runs the comparison the item actually asked for.

### Instrumentation gap, closed first

Neither existing script measured the right quantity. tools/checkserialhealth.sh (item 19) counts wrong checksum lines in journalctl — a protocol-level symptom, one layer removed from the event of interest. Every number this report’s UART analysis is actually built on, Eq. [eq:missprob]’s 3.07 s−1 and the 137 000→676 751 progression of Table 12.8, comes from the kernel’s own FIFO overrun counter instead, and nothing in tools/ read it in a scripted, repeatable way: every past reading was an ad hoc SSH one-liner, typed once and not saved. tools/checkuartoverruns.sh closes that gap: it reads /proc/tty/driver/ttyAMA’s oe: field twice, a chosen number of seconds apart, and reports the delta as a rate in s−1 directly comparable to Eq. [eq:missprob]. Confirmed live before use: irq:14, uart:PL011, the same interrupt already implicated in §12.15.1’s deadline model.

### Matching the load

Checked live immediately before the measurement: realsense2cameramanager at 164.7 % CPU, /camera/color/imageraw/compressed publishing at 15.0 Hz (not the 4 Hz throttle applied for the Test1/Test2 protocol elsewhere in this report — reverted, whether deliberately or as a side effect of a later restart, and confirmed via rostopic hz rather than assumed), and both estimators registered and running simultaneously (minssubscribe, ovmsckf). That is the same qualitative load July 30 described: “the colour nodelet consumed 185 % of a four-core budget already carrying two estimators’ worth of publication traffic.” The CPU percentage itself is not expected to match: July 30’s 185 % was measured with the clock throttled to 600 MHz, and the same code path against a 1500 MHz clock (Eq. [eq:aug11nopenalty], no frequency penalty) does less relative work for the same wall-clock output, so a lower percentage at full clock is consistent with an equal or heavier absolute load, not evidence of a lighter one. Core temperature, read before and after both windows, held at exactly 73.0 ∘C throughout with getthrottled at 0x0 and the clock at 1500.4 MHz on every check — stable across the full ∼13-minute span from the initial process check to the last reading, which is circumstantial support that the same background load persisted through the measurement, not a per-second confirmation of it.

Two runs, one number: 2.07 s−1, not 3.07 Two independent 180 s windows, run back to back, no configuration change between them:

| Window | Δoe | Duration | Rate |
| --- | --- | --- | --- |
| Run A | 383​ | 180.8 s | 2.118 s−1​ |
| Run B | 367​ | 180.8 s | 2.030 s−1​ |
| Pooled | 750​ | 361.6 s | 2.074 s−1​ |

The two runs agree to within 4 % of each other, which is what makes the pooled figure a measurement rather than a single noisy sample. Against the July 30 reference,

$$\begin{equation}
    \frac{2.074}{3.07} = 0.676, \qquad\text{a } 32.4\,\%\text{ reduction, and a }
    67.6\,\%\text{ remainder.}
    \label{eq:aug13reduction}
    \end{equation}$$

Restating in the miss-probability form of Eq. [eq:missprob],

pAug13=2.074781=2.66×10−3  versus  pJul30=3.9×10−3.

### What this actually settles

Item 38, read literally, is closed: the matched-load A/B it specified has been run, on the load it specified, and produced a number. What that number says is not what a reader hoping to close the whole UART story would want. Active cooling recovers roughly a third of the overrun rate at this load, not the whole of it. Two out of every three overruns that occurred under July 30’s throttled, no-cooling condition are still occurring under a condition with no throttling at all, clock pinned at its maximum on every single check taken across the session. Eq. [eq:aug11nopenalty] already established that the frequency-penalty term in Eq. [eq:latencysplit] is at its identity element under tonight’s conditions; a majority of the original overrun rate surviving that removal means a majority of L in that equation is coming from somewhere Eq. [eq:freqpenalty] never touched — Lpreempt itself, or a mechanism the decomposition does not model at all, such as the physical connector hypothesis §12.17.5 raised and explicitly did not establish. This result does not distinguish between those; it rules out temperature as sufficient, which is a narrower and more useful claim than ruling it out as relevant.

What this is not Two runs from one session, one physical robot, one evening. It is a real controlled comparison — same load, same instrumentation, same 180 s window as the reference it is measured against — not a large sample, and it says nothing about whether the remaining 67.6 % is constant, load-dependent, or itself thermal in a way this experiment did not vary (ambient temperature, connector temperature specifically rather than core temperature, time since boot). The next step this result points to, not yet run, is interrupt-latency tracing on irq 14 itself (ftrace’s irqsoff tracer, confirmed available on this robot without installing anything, against cyclictest, which is not installed) to see whether the residual 2.07 s−1 traces to Lpreempt directly rather than staying a subtraction.

## IRQ 14 starvation, caught directly: the MMC/USB chain that blocks the UART

The trapbox above named the next step; it was run the same evening. ftrace’s irqsoff tracer records the single longest span in which hardware interrupts were disabled, with a full function-call trace of what the CPU was doing throughout. Two independent captures, buffer widened to 2 MB/core first, tracer reset to nop and the buffer size restored to its default afterward in both cases:

Both captures exceed the FIFO deadline by more than double, and neither implicates the UART Capture 1 (90 s window, no load applied beyond the robot’s own idle state): tracingmaxlatency =2829 μs. The trace is a single MSI interrupt, xhciirq, entering at t=22 μs and not releasing interrupts until t=2829 μs: seventy-six separate swiotlbbounce calls, each preceded by dmadirectsyncsingleforcpu/dmadirectunmappage, draining a burst of completed USB transfer descriptors (handletxevent → finishtd) one at a time. Not one line of this capture mentions pl011 or uart.

Capture 2 (45 s window): 3064 μs, worse than capture 1. This one brackets the UART directly. pl011int fires at t=10 μs (a normal service, draining its FIFO via pl011fifototty); at t=131 μs and again at t=183 μs, bcm2835mmcirq (the Pi’s own SD-card controller) runs, each invocation spinning on udelay inside the handler; at t=240 μs xhciirq begins the same swiotlb-bound descriptor drain as capture 1 (this time 380 swiotlb occurrences across the whole trace). pl011int does not fire again until t=2889 μs.

What capture 2’s bracket means, in the deadline model’s own units. The gap between the two pl011int services is

Δtpl011=2889−10=2879 μs,

against Tdl=1.28 ms (Eq. [eq:fifodeadline]), a factor of 2.25×. At the byte arrival rate of Eq. [eq:fillrate] (τb=40 μs), that gap spans

Δtpl011τb=287940≈72

byte-arrival opportunities against a 32-entry FIFO: not a near miss, a gap wide enough for roughly 40 bytes to have nowhere to go, in this one occurrence alone. Both captures’ worst-case latency exceeds Tdl by a comparable margin (2.21× and 2.39×), which is the first time this report has a single, named, directly-observed mechanism wide enough to miss the deadline on its own, rather than an inferred one.

What this does and does not establish Both captures name the same class of culprit — swiotlb-bound xHCI descriptor draining, in one case preceded by SD-card interrupt handling that itself busy-waits with udelay — and both are consistent with the checksum storm’s known properties: present with the rover stationary (§12.17.5), unaffected by serialnode’s scheduling priority or the kernel’s RT budget (item 40’s ruled-out hypotheses), and only partially remedied by removing thermal throttling (item 38, §12.18) since a faster clock drains the same 76–380 swiotlb calls faster but does not remove them. What two captures do not establish: that this is the only mechanism (a longer or repeated capture could surface a different worst case), or that the task context shown in both traces’ header (python3-43371, rtprio:25 — almost certainly serialnode itself, the only userspace process at that priority) means serialnode is implicated in causing the delay, rather than simply being whichever process the CPU happened to be running when a hardware interrupt — necessarily non-preemptible, regardless of which process it interrupts — arrived. Candidate next steps, none attempted tonight: check whether the xHCI controller’s DMA mask forces swiotlb unnecessarily (a 64-bit-capable mask would skip the bounce buffer entirely); reduce the SD-card driver’s use of busy-wait udelay inside its IRQ handler if a kernel or DTS option controls it; check /proc/irq/*/smpaffinity for whether the xHCI and MMC interrupts (MSI-based, unlike IRQ 14’s fixed GIC line) can be pinned away from CPU0 entirely, which would remove the contention without touching the UART side at all.

### Two of three candidates checked, the same evening

##### DMA mask: ruled out directly, not inferred.

The trapbox above named the xHCI controller’s DMA mask as a candidate. Checked directly rather than assumed: lspci -vv reports the VL805 controller (PCI 01:00.0) with dmamaskbits =64, and the platform’s own dma-ranges property (/proc/device-tree/scb/dma-ranges, decoded by hand against #address-cells=2/#size-cells=2) maps a 16 GiB inbound window starting at physical address 0 — double this robot’s installed 7.6 GiB, so nothing on this board is DMA-unreachable. This rules out address-range restriction as the reason §12.19’s trace shows swiotlb bouncing; the cause is elsewhere in the driver/allocator path, not identified tonight.

##### IRQ affinity: xHCI is hard-blocked by the kernel, MMC moved but the test was inconclusive.

/proc/interrupts confirmed the exact contention pattern §12.19 inferred: IRQ 14 (UART), IRQ 27 (mmc0/mmc1) and IRQ 43 (xhcihcd) had each delivered essentially 100 % of their interrupts to CPU0 (2.2 M, 18.6 M and 354 k respectively), despite an smpaffinitylist of 0-3 on all three — nothing was actively rebalancing them. Writing a narrower mask to IRQ 43 failed outright, twice, with I/O error on both the list and bitmask interfaces: this specific Broadcom STB PCIe MSI controller does not support runtime affinity changes for this interrupt at all, closing off the most direct version of this mitigation. IRQ 27 (MMC) accepted the same change. The before/after overrun-rate comparison this enabled was inconclusive: three 30 s samples taken in sequence (1.579 s−1 baseline, 2.632 s−1 with MMC moved, 2.928 and 2.961 s−1 after reverting to the original affinity) drift upward across the whole window regardless of which configuration was active, meaning a single sample before and after cannot separate a real effect from ordinary time-varying load — exactly the discipline item 38’s own matched-load protocol (§12.18) was built to enforce, and a single quick test here did not. Configuration fully restored (0-3 on all three, confirmed) before moving on.

##### SD-card udelay: no runtime knob exists.

The third candidate, checked last: bcm2835mmc (the driver bound to mmc0, the eMMC2 controller carrying this Pi’s SD card and root filesystem, confirmed via /sys/bus/mmc/devices/ and matching bcm2835mmcirq in §12.19’s trace) exposes exactly two module parameters, both read as 0: /sys/module/bcm2835mmc/parameters/{mmcdebug,mmcdebug2} — debug-logging verbosity flags by every naming convention in the kernel, not timing controls. Nothing in this driver’s exposed surface lets the busy-wait be reduced or disabled without patching and rebuilding the module itself, which was not attempted: this Pi’s SD card is also its boot device, and testing a rebuilt storage driver on the only one this robot has, live, was judged the wrong trade for tonight. All three candidates this item named are now checked, not guessed at: one ruled out, one hard-blocked by the kernel, one with no accessible knob.

##### MMC affinity, re-tested properly: still not confirmed, now for a better reason.

The gap this section left open — a single before/after sample cannot separate a real effect from drift — was closed the same evening with an alternating protocol matching item 38’s own discipline (§12.18): two 120 s windows with IRQ 27 on CPU1 and two with it restored to 0-3, in B–A–B–A order, configuration confirmed restored after each switch.

|  | Round 1 | Round 2 |
| --- | --- | --- |
| B (CPU1) | 2.616 s−1​ | 1.744 s−1​ |
| A (0-3) | 2.450 s−1​ | 2.558 s−1​ |

Mean B =2.180 s−1, mean A =2.504 s−1, a nominal 12.9 % reduction — but B’s own two samples span 0.872 s−1 (standard deviation 0.617), nearly three times the 0.324 s−1 gap between the two means, while A’s two samples agree far more closely (spread 0.108, standard deviation 0.076). A difference smaller than the noise within one of the two groups it is drawn from is not a confirmed effect, whichever direction it points. Four windows, honestly, is not enough power to settle this either way; what this run adds over the earlier single-sample attempt is knowing precisely how much power is missing; not that there is none. What remains genuinely open is the same experiment run longer, or repeated over more than one evening.

## sqrtVINS on the Raspberry Pi: installed, debugged, and diverged — a complete answer to a new request

A new request arrived from Prof. Gutierrez the same evening: MINS is too large to run in real time on the Raspberry Pi 4B, so it now runs on the ground PC (already the case throughout this report, confirmed live: ps aux on the PC shows minssubscribe at 84–85 % CPU, none of it on the robot). Carolus stays local to the Pi. A new candidate, VINS (Peng et al., ICRA 2024 / TRO 2025, https://github.com/rpng/sqrtVINS), was proposed as a lighter alternative that could run locally on the Pi instead of shipping camera frames to the PC: install it, test it, and compare its update rate and accuracy against MINS on the lab’s rectangular trajectory, with the caveat — stated in the request itself — that VINS does not fuse wheel encoders and is therefore expected to be less accurate than MINS on its own terms.

This section reports the complete result: installation succeeded, two real defects were found and fixed the same evening, the system ran and processed live sensor data in real time on the Pi, and then diverged severely on an actual driven lap of the lab. All three are reported, not just the parts that flatter the outcome.

### Installation

VINS (package ovsrvins) is an extension of openVINS from the same lab (RPNG, University of Delaware), replacing the conventional EKF/MSCKF covariance update with a Cholesky (LLT) square-root filter and running in 32-bit single precision (USEFLOAT=1, confirmed directly in the compiler invocation — not merely claimed by the README). It ships ovcore, ovdata and oveval packages with the same names as the openVINS installation already running on this robot, so it was built in a fully isolated workspace (/home/pi/sqrtvinsws) rather than the existing catkinws, to guarantee zero risk to the estimator this report already relies on. Every system dependency (libceres-dev, libopencv-contrib-dev, libeigen3-dev) was already present on the Pi. Build time: 104 minutes (catkin build -j2, deliberately capped to leave headroom on a robot that was live throughout), zero errors.

### Two real defects, found by bisection, not by guessing

##### Defect 1: an undocumented, incompatible config schema.

A first config, built by porting this robot’s proven openVINS values (§12.15.1 and throughout this report) onto the classic openVINS estimatorconfig.yaml schema, crashed on every launch before a single frame was processed:

terminate called after throwing an instance of
’boost::wrapexcept<boost::lockerror>’
what(): boost: mutex lock failed in pthreadmutexlock: Invalid argument

Bisected by rebuilding the config from the one file confirmed to run without crashing (config/eurocmav/estimatorconfig.yaml, the package’s own example) and re-adding this robot’s tuned values one field at a time, retesting after each addition. Every numeric value — calibration locks, measured gravity, KLT tuning, initialization thresholds — proved harmless in isolation and in combination. The schema itself was the problem: ovsrvins silently requires many fields absent from classic openVINS configs (initprior, useimuavg/userk4int in place of integration:"rk4", and others), undocumented in the public README, discovered only by diffing against the one config known to work. A second, narrower trap surfaced while writing an explanatory comment about this very fix: a code-review-style comment containing the literal text integration:"rk4" (colon, no space, quoted value — resembling a real YAML assignment even though it was inside a # comment) reintroduced the identical crash. OpenCV’s FileStorage parser is not a full YAML parser; comments describing a bug can apparently retrigger it. The deployed config avoids the pattern entirely rather than relying on a fully characterised safe subset.

Defect 2: tryzupt: true diverges catastrophically at rest, on this platform, with this filter This robot’s proven openVINS value is tryzupt: true — on the PC, it is documented elsewhere in this report as the correction that took a divergence from 41 824 m down to 0.002 m (§12.13.2). Porting it unchanged to VINS on the Pi, robot stationary, produced the opposite: state velocity growing from |vIinG|=1780 to 6015 m/s across nine ZUPT updates in a few seconds, each one logged as accepted by the filter’s own χ2 consistency gate:

[ZUPT]: accepted |vIinG| = 6015.597 (chi2 165137575521026048.000 < 65.171)

A χ2 residual of 1.65×1017 against a threshold near 65 is not a near-miss gate failure, it is the gate reporting success on a value seventeen orders of magnitude past it — consistent with a numerical representation problem rather than a modelling one, and the one architectural fact that distinguishes this filter from every other estimator in this report is single-precision arithmetic (USEFLOAT=1, confirmed directly in the build above). Confirmed by bisection to be caused by tryzupt alone: identical on every other setting to a configuration that ran stably for the full length of every test conducted (up to 70 Hz, repeatedly, including a rebuild-from-scratch reproduction of the crash and its fix the same evening). Deployed config: tryzupt: false, the opposite of the PC’s proven setting, with the contrast documented in-line in the config file itself.

### Running, locally, on real sensor data

With both defects fixed, VINS subscribed directly to this robot’s own camera and IMU topics (/camera/infra1|infra2/imagerectraw, no /pc/camera/... prefix, no republish hop — it runs on the same machine the D455 publishes from, so the WiFi bandwidth constraint that shapes §12.15.1’s republish architecture does not apply to it at all) via a second, independently-named instance of this report’s own imusanitizer.py (imusanitizerpi, publishing /imu/datacleanpi, coexisting on the same ROS master as the PC’s instance without collision). Processing held between 30 and 70 Hz per frame with the robot stationary, correctly reported “no jerk detected, platform is stationary” while waiting for motion to initialize — the identical behaviour already documented for openVINS on this robot (tools/recordtrajectories.sh’s own header note).

An unscheduled power interruption, mid-session, recovered cleanly The Pi lost power entirely partway through this investigation (confirmed: operator checked physically, “il était pas allumé”). Recovery required exactly two commands (relaunch imusanitizerpi, relaunch ovsrvins) since the build and config live on disk, not memory. Separately, MINS on the PC survived the robot’s reboot as a live process but stopped appearing in the ROS graph — the identical staleness class already named in §12.17.3 (EP(t)>U(t), Eq. [eq:staleness]): the process outlived the robot session that established its ROS-graph registrations. tools/restartstack.sh, already built for exactly this failure mode, resolved it in 8 s.

### The driven test: three estimators, one lap, one clear result

Same lap of the lab, three estimators recorded simultaneously in one bag (415 s, 104701 messages). Loop closure error = distance between the first and last recorded position (a proxy for accumulated drift, same methodology as comparetests.m elsewhere in this report).

| Estimator | Messages | Position range (m) | Loop closure error |
| --- | --- | --- | --- |
| MINS (PC, wheels+cam+IMU) | 32 802​ | x∈[0.0,11.8], y∈[−0.2,4.7] | 1.02 m​ |
| openVINS (PC, cam+IMU) | 31 889​ | x∈[−168,3366], y∈[−652,0] | 541.7 m​ |
| VINS (Pi, cam+IMU) | (diverged before capture​*) | x,y,z in the thousands | unbounded |

​*Correction, 2026-08-18. This footnote previously read that /ovsrvins/odomimu “never reached the PC-side rosbag record process despite confirmed local publication on the Pi” — an explanation formed during the session and never checked afterwards. Re-reading the bag settles it: rosbag info lists five topics, and that one is not among them. Not recorded-and-empty, never requested. tools/recordtrajectories.sh simply did not carry /ovsrvins/odomimu in its topic list, so no network explanation was ever needed. The line is added, with a preflight check that treats its absence as normal rather than alarming, since sqrtVINS runs on the Pi on demand and not as part of the PC stack. The divergence figures quoted above remain reconstructed from VINS’s own node log, timestamped and un-editable after the fact — that part was never in doubt. This is the third time in this campaign that an explanation given live proved wrong on re-reading; the lesson is recorded in §17.17.15’s neighbourhood and holds here too.

VINS’s own log lets the divergence be dated precisely against distance travelled, not just against a final number:

| Frame # | 1 | 700 | 900 | 1000 | 2000 | 6293 (last) |
| --- | --- | --- | --- | --- | --- | --- |
| Reported dist. (m) | 0.0​ | 19.3​ | 128.8​ | 275.4​ | 6093.6​ | 43 742.6​ |

Up to frame ∼​700 (≈19–20 m), the reported distance is consistent with a real lab lap. Past that point the growth rate itself accelerates rather than staying linear — a runaway, not a step failure: once the estimate is wrong enough to corrupt the feature geometry it depends on, each subsequent update makes the next one worse, with nothing left to catch it. This is the specific failure mode absent zero-velocity updates predicts, and Defect 2 above is exactly why they were absent here.

### What this settles, and what it does not

Every part of the original request now has a real answer, not a promise of one. VINS does install and run on a Raspberry Pi 4B in real time (the README’s claim is not merely repeated here, it is independently confirmed at 30–70 Hz on this robot’s own sensors). Its update rate, while numerically healthy, was never the limiting factor. Its accuracy, on the one lap this evening produced, was not just lower than MINS’s — expected, stated in the request itself, and consistent with every other camera+IMU-only estimator in this report — it was categorically worse than even vanilla openVINS on the identical lap (43 000 m of accumulated reported distance against 541 m of final displacement error). The most defensible reading is not “VINS is unusable,” it is “the specific configuration deployed tonight, running without zero-velocity updates because the alternative crashed outright, was not enough to keep this filter bounded through a real, moderately dynamic lap on this hardware.”

The disparity-only path was tried, the same evening: worse, not better Two candidate directions were named above as untried. The first, a write-up of Defect 2 for RPNG’s own issue tracker, is drafted and ready to submit, now covering both ZUPT sub-modes below. The second, the disparity-only path (zuptchi2multipler: 0) as a middle ground between “no velocity anchoring” and “the IMU-based setting that diverges,” was tested at rest the same evening — and is not a middle ground. The deployed, stable configuration already carried zuptchi2multipler: 0; the entire change tested was tryzupt: false → true. Within roughly a dozen updates, well under one second, the filter reached

[ZUPT]: accepted |vIinG| = 203734831702474752.000 (chi2 inf < 0.000)

— not merely a large finite χ2 as in Defect 2’s IMU-path capture, but literal floating-point infinity, against a printed threshold that itself collapsed to 0.000, consistent with the consistency check’s denominator vanishing rather than its numerator merely overflowing. Reverted immediately, config restored and confirmed. Neither ZUPT sub-mode is usable in this build; a real driven test of the disparity-only path was not attempted, since a mode that reaches inf in under a second standing still has nothing left to test by moving.

### A source patch: reading UpdaterZeroVelocity.cpp instead of just reporting it

The write-up mentioned above as “drafted and ready to submit” was finished by actually reading the file it describes, rather than stopping at the printed symptom. Two things stood out in ovsrvins/src/update/UpdaterZeroVelocity.cpp:

- The χ2 consistency check runs S.llt().solve(res) without ever inspecting .info(). Eigen’s documented behaviour, if the LLT decomposition of the innovation covariance S fails (not positive-definite — plausible under single precision as terms accumulate), is to return silently from .solve() with garbage rather than signalling failure. Nothing downstream checks for it.

- In the disparity-only path, the reject condition is written if (!disparitypassed && (chi2 > thresh || vel > maxvel)) reject;. When disparitypassed is true — the common case at rest, since disparity stays small while stationary — the entire χ2/velocity gate short-circuits out and the update is accepted unconditionally. The printed (chi2 inf < 0.000) in §12.20’s log is cosmetic in this branch: nothing ever compares against it.

A minimal, additive guard, tested against the real symptom it targets The patch adds an unconditional numerical-validity check ahead of the existing reject block, changing no tuning value or threshold:

Eigen::LLT<MatX> llt_of_S(S);
DataType chi2 = res.dot(llt_of_S.solve(res));
bool chi2_numerically_valid = (llt_of_S.info() == Eigen::Success) &&
                               std::isfinite(static_cast<double>(chi2)) &&
                               (chi2 >= 0);
...
if (!chi2_numerically_valid) {
  last_zupt_state_timestamp_ = 0.0;
  last_zupt_count_ = 0;
  PRINT_WARNING(RED "[ZUPT]: rejected -- chi2 numerically invalid "
                    "(llt_success=%d, chi2=%.6e, disparity_passed=%d)\n" RESET, ...);
  return false;
}

Applied on the Pi’s own sqrtVINS clone (git diff: one file, 38 insertions, 1 deletion — attached in full as docs/sqrtvinszuptnumericalguard.patch), rebuilt (catkin build ovsrvins, 5 min 29 s, “All 2 packages succeeded”), and run live for 60 s against real stereo+IMU data, robot stationary, disparity-only path, tryzupt: true: 588 updates correctly rejected as numerically invalid, 241 correctly accepted with sane χ2 values, zero acceptances of an inf/garbage χ2. The specific bug this section opened with — silent acceptance of an astronomically-out-of-threshold residual — no longer reproduces, verified against the actual log, not inferred from the absence of a crash.

The patch is real; it does not close the case With every garbage ZUPT update now correctly rejected, the same 60 s stationary run still diverges: pIinG grows smoothly — no longer via ZUPT chatter, since ZUPT is now provably not contributing it — from roughly 1015 m to roughly 3.4×1017 m over the window. That rules out the ZUPT updater as the divergence source and narrows it to a second, independent numerical-stability issue elsewhere in the pipeline — propagation or the MSCKF update are the natural suspects, since ZUPT is now demonstrably not the vector. That second issue has not been diagnosed: it would mean reading Propagator.cpp, UpdaterMSCKF.cpp and StateHelper.cpp with the same discipline applied here, a distinct, larger piece of work not yet started. Per this investigation’s own stated condition — a driven test only once the estimator is stable at rest — it is not: no driven test was run against the patched build, and none should be claimed as validated until the second source is found. The patch is offered to RPNG as a real, tested fix for the logic bug it targets, not as a fix for the platform’s divergence as a whole.

## The Carolus quaternion, traced to source: why sixteen trials were never going to converge on a general answer

The supervisor’s question, prompted by two independent deployments (LIMO, §12.21.1; this platform) each finding a working sign convention by exhaustive trial, was direct: is there a general solution, one that would also work on a Robomaster with no prior Carolus deployment at all? The honest answer is not a single quaternion, portable across mounts, that number cannot exist, since three different physical camera mountings have three different correct answers by construction. What this section delivers instead is stronger: the exact mechanism, traced into Carolus’s own source rather than inferred from its behaviour, that made sixteen-trial search the only available method until now, and a closed-form procedure that replaces the search with one measured calibration motion, on any robot, Robomaster included.

### Two prior deployments, both solved by search

Two lab-internal guides document independent Carolus integrations, both predating this section. “How to use Carolus to make the LIMO follow the beacon” (T. Brezins, LIMO rover, ROS Noetic) found, by exhaustive sign search over 24=16 combinations with the axis-letter permutation already fixed, the working conversion Xrobot=−Zc, Yrobot=−Xc, Zrobot=−Yc for translation and rotation.x,y,z,w =(−oz, ox, −oy, −ow) for orientation, and states plainly: “if you not use this Rover, you must find your own quaternion.” A companion guide for this Leo Rover (§17.10.3’s Carolus chain) independently found the identical translation letters and signs, but a different orientation sign combination, (−oz, ox, oy, ow), reporting the same 24-combination search and the same warning. Two of four rotation signs differ between the two working conventions despite an otherwise near-identical forward-facing colour-camera mount, concrete, sourced evidence that no fourth robot can safely reuse either number without its own verification.

### What is genuinely universal: the optical-to-body rotation

Carolus (package carolusastrobee, derived from NASA’s Astrobee free-flyer code) solves a P4P problem in Ceres and publishes a geometrymsgs/PoseStamped on /pose. Its cost function, read directly from ceresP4P.hpp:120--150, is the textbook calibrated pinhole model, nothing platform-specific in it:

ceres::AngleAxisRotatePoint(camera_R.data(), target_point, camera_point);
camera_point[0] += camera_T[0]; camera_point[1] += camera_T[1];
camera_point[2] += camera_T[2];
double xp = camera_point[0] / camera_point[2];   // pinhole: x/z
double yp = camera_point[1] / camera_point[2];   // pinhole: y/z
predicted_point[0] = xp*focalx_length_ + cx_;
predicted_point[1] = yp*focaly_length_ + cy_;

This is Xcam=R Xworld+T followed by division by the third coordinate, i.e. the standard OpenCV/optical convention: X right, Y down, Z forward (out of the lens, increasing with depth). With the known beacon points defined about their own origin, Xworld=0 gives Xcam=T: the published translation is genuinely the beacon’s position in the camera’s own optical axes, no hidden library convention, verified by reading the optimiser, not assumed from the output.

ROS body-style frames (REP 103, the convention baselink and every TF in this project’s tree already use) place X forward, Y left, Z up. A point with optical coordinates (Xo,Yo,Zo) has body-style coordinates obtained by matching what each axis physically means: forward is the optical depth axis, left is minus optical right, up is minus optical down:

XbYbZb=RobXoYoZo,  Rob=001−1000−10.

Rob is orthogonal (RobRob⊤=I) with detRob=+1: a proper rotation, not a reflection, exactly as a fixed relabelling of one rigid camera’s own axes must be. This matrix is universal. It follows from the two convention definitions alone (REP 103, REP 104/OpenCV) and depends on neither the beacon, the solver, nor the robot. It is the entire content of the “letter correspondence” both the LIMO and this platform’s own documentation found empirically (Xrobot=±Zc, Yrobot=±Xc, Zrobot=±Yc, §12.21.1): Eq. [eq:rob] is the reason the letters always matched across two unrelated mounts, and derives in three lines what both predecessor documents found by pattern-matching two independent trial-and-error searches against each other.

The one universal piece: optical depth (Zo, red) becomes body forward (Xb, red) on every camera, on every robot, because both conventions are fixed definitions, not measurements. Nothing about this diagram is platform-specific.

### What is not universal, traced to the exact line that breaks it

If Eq. [eq:rob] were the whole story, applying it to both translation and rotation would settle the question on any robot in one derivation, no search required. It does not, and the reason is a specific, quotable inconsistency in carolusastrobee.cpp between how translation and rotation are published, not a deeper property of optics.

Immediately after the solve, translation is published as the solver’s raw output, PoseAstrobeeMsgs.pose.position = bestPose.t verbatim (carolusastrobee.cpp:1180--1182). Rotation is not:

// TRANSPOSE TO GET THE CORRECT ROTATION MATRIX -- IF NOT TRANSPOSED,
// THE ROTATION MATRIX IS INVERTED
// DOUBLE CHECK THIS ITS 5AM
Eigen::Quaterniond q(bestPose.R.transpose());

(carolusastrobee.cpp:1185--1187, comments verbatim). This is not this project’s guess about upstream intent: it is the original authors’ own, timestamped uncertainty, left in the code. For a rigid transform Xcam=R Xworld+T, the self-consistent inverse (“where is the camera, expressed in the world/beacon frame”) is

Rinv=R⊤,  Tinv=−R⊤T.

carolusastrobee.cpp publishes Rinv=R⊤ but Tpub=T, not −R⊤T. The published pair is a hybrid: an inverted rotation paired with a non-inverted translation, which is not one consistent SE(3) transform in either direction. This is precisely why no single textbook rotation, applied uniformly to both position and orientation, was ever going to reconcile the empirical sign findings: the two components are not related by the same convention to begin with, so searching for one 4×4 (or one quaternion) that fixes both at once is searching for something that, given the source above, does not exist. Both predecessor documents’ sixteen-combination sweeps (§12.21.1) were the correct response to this exact bug, not a shortcut around a cleaner answer that was available and missed.

### The general method: one measured calibration, not sixteen trials

The mismatch above is specific to this one upstream file. A Robomaster integration would not inherit this exact bug, it might have none, or a different one in whatever P4P/PnP code it uses. A method that first diagnoses this file would not be general. The method that is general treats Carolus (or any beacon detector) as an uncalibrated black box and solves for whatever fixed correction it needs directly from measurement, which is correct regardless of what is or is not consistent inside the box.

##### Setup.

Let (Rc(i),tc(i)) be Carolus’s raw published rotation and translation at calibration pose i, and let (Rg(i),tg(i)) be the same rover pose measured by ground truth (tape measure and a marked heading, as already established for this platform’s floor protocol, §17.13.3). The unknown is a single, fixed calibration rotation C such that

Rg(i)=C Rc(i) C⊤ or Rg(i)=C Rc(i)

for every i (the first form if C conjugates a rotation expressed in Carolus’s frame into the ground frame; the second if C instead re-expresses a rotation already acting on ground-frame coordinates, which of the two applies is itself determined, not assumed, by the residual in step 3 below).

##### Procedure.

- One reference pose is not enough for rotation, only for translation’s axis permutation and per-axis sign (three components, three independent equations, generically enough with one non-degenerate sighting once the beacon is not on an axis of the rover). Rotation about the optical axis itself is unobservable from a single sighting at a generic angle: take a second pose related to the first by a known, commanded rotation (this platform already has a scripted primitive for exactly this, 90° G/90° D on the Trajectory tab, §17.13.3).

- Solve for C in closed form. With two or more (Rc(i),Rg(i)) pairs, Eq. [eq:calibgeneral] is a Wahba’s problem: minimise i​Rg(i)−CRc(i)F2 over the rotation group. The closed-form solution is the same SVD construction already implemented in this project’s own Matlab tooling for trajectory alignment (localkabsch2d, §17.13.1, here in 3D rather than 2D and applied to orientations rather than positions): H=i​Rc(i)⊤Rg(i), H=UΣV⊤, C=V diag(1,1,det(VU⊤)) U⊤. Two poses related by a rotation about a vertical axis already fully constrain C; a third, non-coplanar pose is a check, not a requirement.

- Translation calibration is then a linear solve, not a search. With C fixed, tg(i)≈C tc(i)+d for a fixed offset d (the beacon-to-ground-origin displacement); two or more sightings give an over-determined linear system, solved by least squares.

- Validate, don’t assume convergence. Apply (C,d) to a held-out sighting not used in the fit; residual position error should be within the beacon’s own known measurement noise (∼5 cm, configvicon.yaml, §17.10.3), not simply small relative to the uncorrected error.

This is the answer to the supervisor’s actual question. It requires no prior knowledge of Carolus’s internal bug, no inspection of anyone’s source code, and generalises unchanged to Robomaster or any other platform: two measured poses, one closed-form SVD, one linear solve, one held-out check. Sixteen (or 24=16, or 24×16=384, §12.21.1) blind trials are not a step of this procedure at any point.

The general method: two measured poses fix C in closed form (no search), a linear solve fixes the translation offset, a third pose validates. Portable to any beacon detector on any robot, since it never inspects the detector’s internals.

Not yet run on this platform, or on Robomaster Everything above is derived and, for the optical-to-body rotation (Eq. [eq:rob]) and the source-code inconsistency (§12.21.3), directly verified against ceresP4P.hpp and carolusastrobee.cpp. The calibration procedure of §12.21.4 has not yet been executed, on this rover or any other: it requires two physical, tape-measured poses and the 90° scripted primitive already available on the Trajectory tab, a field session this report does not yet contain. The existing empirical sign convention already locked into carolustfbridge.py’s ~quatsigns default ("-+++", §12.21) continues to be what this platform actually runs until that calibration session replaces it with a measured, closed-form value.

## Relocalization theory: why an absolute fix corrects accumulated drift, and by how much

Item 5’s VICON-slot integration (§17.10.3, configvicon.yaml) is implemented and unenabled (§12.17.5 already established this precisely). What follows is the theory the supervisor’s requested straight-line experiment (§12.22.3 below) will measure against: not a description of what should happen, but a derivation of what a correctly-tuned filter is mathematically required to do, so the eventual field result can be judged against a real prediction rather than a vague expectation of “it should snap back.”

### Drift as a growing covariance, not a growing error

Between absolute fixes, MINS propagates position purely by integrating IMU and wheel measurements, each corrupted by process noise. For a filter tracking position error as a random walk driven by noise intensity q (the propagation step of any Kalman-family estimator, configestimator.yaml’s own noise densities being exactly this q, already documented as measured via Allan variance rather than datasheet, §17.10.3), the estimator’s own belief about its uncertainty evolves as

P(t)=P(t0)+q (t−t0),

variance growing linearly in the time since the last correction, not the position error itself, which is a single unknown realisation, not a distribution. This distinction matters for reading the experiment correctly: a single straight-line run’s actual drift is one sample from a distribution whose spread grows by Eq. [eq:covgrowth], so one run confirms an order of magnitude, not the growth law itself, which would need repeated trials at matched duration to fit.

### The correction is a precision-weighted average, not a reset

When a Carolus-derived fix z=x+v arrives, v∼N(0,R) (this platform’s own configvicon.yaml: noisep=0.05 m, so R≈(0.05)2=2.5×10−3 m2 per axis), the scalar Kalman update gives the corrected estimate and its variance as

K&=PP+R,x+&=x−+K (z−x−),P+&=(1−K) P−=P−RP−+R=1P−+1R−1.

Eq. [eq:pupdate]’s last form, precisions (inverse variances) summing rather than variances averaging, is the exact reason a beacon fix does not merely nudge the estimate but effectively overrides it once drift has run long enough: if P−≫R (accumulated uncertainty far exceeds the beacon’s own 2.5×10−3 m2), Eq. [eq:gain] gives K→1 and Eq. [eq:pupdate] gives P+→R, independent of how large P− actually was. The correction magnitude is therefore not a tuned behaviour, it is what the arithmetic in Eq. [eq:xupdate] does automatically once the beacon’s precision dominates: the “jump” the supervisor’s protocol asks the student to observe (§12.22.3) is the innovation term K(z−x−), which for K≈1 is approximately the entire accumulated error at that instant, not a fraction of it.

The predicted shape of the straight-line experiment (§12.22.3), derived, not measured: an uncertainty envelope opening as t while no fix arrives (dashed red), an abrupt correction sized by the innovation at the instant a fix lands (blue arrow, Eq. [eq:xupdate]), and a tight, beacon-precision envelope resuming immediately after (dashed green). No data from this rover appears in this figure; it is what Eq. [eq:covgrowth]–[eq:pupdate] require, against which a real run can be checked.

### The straight-line experiment this predicts, not yet run

The supervisor’s protocol, precisely: record a rosbag through tools/recordtrajectories.sh (§17.13.1) while driving a long straight line with the beacon initially out of view, exposing it partway through, either by manual driving or a scripted straight-line primitive (the encoder-calibration scripts, §17.17.4, already show the pattern: subscribe to odometry, integrate distance, gate a Twist publisher on a threshold). tools/plotrelocalization.m (written this session, §12.22.4 below) locates the correction instant from the first timestamp in *carolus.csv and reports the jump magnitude directly, the same K(z−x−) quantity Eq. [eq:xupdate] predicts.

Not yet run, on this platform or any other Everything in this section is a derivation from standard linear-Gaussian estimation theory (Eq. [eq:covgrowth]–[eq:pupdate]) applied to this platform’s own measured numbers (configvicon.yaml’s R≈2.5×10−3 m2), not a report of anything observed. It is gated on the same prerequisites already recorded in §12.17.5 and §12.22.4: vicon.enabled is false, the beacon’s global coordinates are unsurveyed, and the live UART checksum storm found the same night this theory was written (§12.17.5) is an open, unresolved threat to /firmware/wheelstates, one of MINS’s three fused inputs, which would corrupt exactly the propagation phase Eq. [eq:covgrowth] models. Figure 12.38 is a prediction to test against, not a result.

### Code audit, same session: one real bug, one missing tool, one name correction

Before extending the theory above, the code it depends on was audited directly rather than assumed current from earlier documentation. carolusviconbridge.py and configvicon.yaml (§17.10.3) were re-read in full: both already met the standard this audit checked for (explicit exception handling with no crash path when the beacon is out of view; every configuration field commented), no change made. Two real gaps were found and closed:

- tools/plotrelocalization.m, referenced by name in §12.22.3 and in earlier drafts of this guide, had only ever been drafted as chat text, never written to tools/. Written for real this session, with explicit error handling for every missing- or empty-CSV case (a bare file-not-found stack trace is not an acceptable failure mode for a script meant to run unattended after a field session); verified against a nonexistent path, confirmed to raise a named, readable error rather than a raw one.

- leobackend.py’s AprilTag long-range fallback (§12.22.5) carried its 0.7 s freshness threshold as a bare literal, duplicated in two call sites. Promoted to a named constant, TAGFALLBACKMAXAGES, both sites updated, pycompile clean.

A separate, independent cross-check (Table [tab:openitems]’s item 1 requirement, “the TF2 node handling the beaconlink anchor”) found that the file matching this description in this project’s own tree is carolustfbridge.py, not the generic example this guide’s Section 1 originally illustrated the mechanism with: the generic example (modelled on the LIMO document, §12.21.1) republishes baselink→cameraframe as a static transform, which this platform’s own TF tree already carries (verified by listening to /tf directly, §17.11), so republishing it would give baselink two parents and break the tree. The real node’s own ~publishbodychain guard (default false) exists specifically to prevent this, with an explicit rospy.logerr if enabled without verification first, a stronger and more accurate artefact for this guide than the illustrative example it replaces.

### Foundation for Section 2: the existing Carolus/AprilTag arbitration, re-verified

The motion-control appendix that follows (§17.19) issues commands through the same target-acquisition path already live in leobackend.py, not a parallel one, so it is verified here rather than assumed. targetcenter(det) returns Carolus’s own per-frame centroid whenever present, and only consults the AprilTag long-range fallback (ontagdetections, reprojecting the nearest tag through the already-Kalibr’d cam0 intrinsics, px=336.372 (x/z)+310.200) on the frames where Carolus’s own detection is absent, gated by $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}TAG\_FALLBACK\_MAX\_AGE\_S}=0.7$ s. Strict priority, not arbitration by comparison, is what prevents the two sources from ever competing: no code path evaluates both and picks one, so no comparison logic exists to oscillate. What is confirmed not to exist is any hysteresis on the handoff itself: near the boundary of Carolus’s own reliable range (≈0.5–4 m), a frame-to-frame flicker in Carolus’s own detection would switch sources every such frame, and the two sources do not report pixel-identical positions for the same physical beacon (Carolus: direct detection; AprilTag: a computed reprojection). This is recorded as an identified, currently unguarded risk, not a proven field problem, since no live AUTO traverse has yet exercised this boundary (§12.9.2.5).

## Standard versus mecanum wheels: two firmware kinematics, one of them switched off

§12.4 established, in July, that the wheels on this rover are mecanum and not the rigid skid-steer set the configuration assumed, and drew the estimator-side consequences (wheel yaw discounted, reff=62.2 mm calibrated). What follows does not repeat that discovery. It documents four things learned since, three of which come from reading the running parameter server on 2026-08-12 rather than any configuration file: that the firmware carries two complete kinematic profiles and runs the wrong one; what the wheel choice costs in metres, written out; the full three-degree-of-freedom kinematics the hardware supports and this configuration cannot express; and a cockpit control added the same day so that recorded data finally states which wheels produced it.

### Two profiles, read live

rosparam on the running robot returns two disjoint parameter trees under /firmware/, plus a boolean selecting between them:

|  | diffdrive (active) | mecanumdrive (unused) |
| --- | --- | --- |
| wheelradius | 0.0625 m | 0.0635 m |
| wheelseparation | 0.358 m | 0.370 m |
| wheelbase | — | 0.3052 m |
| angularvelocitymultiplier | 1.76​ | 1.0​ |

with /firmware/mecanumwheels = false and, on the robot’s own /etc/ros/setup.bash, export MECANUMWHEELS=false — the single switch FictionLab’s own integration guide names. The rover therefore drives mecanum hardware through the differential model, and the mecanum profile, geometry and all, sits unused beside it. Three separate mismatches follow, and they are independent of one another:

- the radius in use (0.0625) is the stock rubber-tyre figure, not this rover’s measured reff=0.0622 (§12.23.2);

- the yaw multiplier in use (1.76) is a scrub compensation derived for rubber tyres, applied to wheels whose rollers do not scrub (§12.23.3);

- the model in use has two degrees of freedom where the hardware has three (§12.23.4).

### Effective radius, and what a wrong one costs

A rubber tyre rolls on its outer surface, so its rolling radius is essentially geometric. A mecanum wheel rolls on polyurethane rollers that compress under load, so the distance covered per revolution corresponds to a radius smaller than a calliper reading on the unloaded wheel:

δ=rgeo−reff=64.4−62.2=2.2 mm,  δrgeo=2.264.4=3.4 %.

The 2.2 mm of Eq. [eq:rollercomp] drawn to scale on the wheel, where it is invisible, and again at ×10, where it is not. That ratio is the whole reason this quantity is easy to leave wrong in a configuration file for months.

Wheel odometry turns encoder angle into distance through a single multiplication, so the radius is a pure scale factor: an error in it does not average out over a long run, it grows with the run.

d&=rused∑Δθwheel,  dtrue=rtrue∑Δθwheel,ε&=rused−rtruertrue=0.0625−0.06220.0622=+0.48 %.

On the 23.35 m taped-rectangle lap of §12.16.5 that is +0.11 m of pure over-report. Stated honestly, this is not the dominant error: the loop-closure gap actually measured on that lap was 0.449 m, four times larger. It is, however, the only part of that gap available for free, by editing one number, where the remainder needs estimator work.

### Why a pivot costs 1.76× on rubber and 1.0× on rollers

A four-wheel skid-steer platform has no steering linkage. To rotate, its tyres must slide sideways across the ground: the wheels roll forward while the chassis turns, so all four contact patches scrub. That scrub absorbs part of the commanded rotation, and the firmware compensates by over-driving the wheel differential. Mecanum rollers, free to turn about their own axes, convert the same lateral demand into rolling instead of sliding, and need no such compensation — which is exactly what the two profiles of §12.23.1 encode in one number each.

The same commanded pivot, produced two different ways. Red arrows are enforced sliding; green arrows are free rolling. The firmware states the difference as a single scalar per profile, and both scalars were read from the live parameter server.

The differential model relates yaw to the wheel-speed difference through the track b; the firmware inserts a gain k on top:

ωz=r (ωR−ωL)b,  ωz,cmd=k r (ωR−ωL)b.

A gain is indistinguishable from a change of track, so k=1.76 is exactly the statement that the platform rotates as though its wheels were

$$\begin{equation}
    b_{\mathrm{eff}} = k\,b = 1.76 \times 0.358 = 0.630\;\text{m}
    \label{eq:beff}
    \end{equation}$$

apart, against a geometric 0.358 m. Differentiating Eq. [eq:diffyaw] gives the sensitivity that makes both geometric parameters matter at once, and in opposite directions:

δωzωz=δrr−δbb.

The 1.76 in use was derived for wheels this rover does not have 1.76 is the stock FictionLab value, shipped with the rubber-tyre configuration, and it compensates a scrub mechanism that mecanum rollers do not produce. Running it against the fitted mecanum set is therefore a third mismatch on top of the radius and the model, and it is consistent with what §12.4 already measured the hard way in July: a sustained Δω≈3.8 rad/s of wheel differential that produced 0.001 rad/s of real yaw. No scalar gain repairs that, because the quantity being scaled is not merely mis-scaled but, in that regime, fabricated — which is why the estimator-side answer was to discount wheel yaw (noisew: 0.2→0.8) rather than to retune k.

### The third degree of freedom, written out

Each roller is set at 45∘ to the wheel axis, so a mecanum wheel can transmit ground force along one diagonal only. Mounted in the standard X pattern the four diagonals span the plane, and the platform gains a genuine third controllable degree of freedom. With this firmware’s own mecanum numbers,

lx=12(0.3052)=0.1526 m, ly=12(0.370)=0.185 m, L=lx+ly=0.3376 m,

the inverse kinematics mapping a body twist to the four wheel rates is

ωFLωFRωRLωRR=1r1−1−L1−1−L1−1−L1−1−Lvxvyωz.

Three degenerate cases make the structure legible. Pure advance (vy=ωz=0) drives all four wheels alike, ωi=vx/r. Pure lateral translation (vx=ωz=0) gives ωFL=ωRR=−vy/r and ωFR=ωRL=+vy/r: two wheels forward, two backward, at equal magnitude, the forward components cancelling and the lateral ones adding. Pure rotation (vx=vy=0) gives |ωi|=L|ωz|/r on all four, so that

ωz=r (ωR−ωL)2L,  2L=0.675 m,

i.e. mecanum yaw obeys the same shape as Eq. [eq:diffyaw] but with 2L in place of b — an effective base 2L/b=1.89 times the differential one. The numerical proximity of 1.89 to the differential profile’s k=1.76 is a coincidence and is flagged as such: the two arise from unrelated mechanisms (roller geometry against tyre scrub), no derivation links them, and treating the match as evidence would be exactly the kind of numerology this report avoids elsewhere.

Left: the roller diagonals in the X pattern, with the two geometric lengths of Eq. [eq:mecgeom]. Right: the second degenerate case of Eq. [eq:mecik] — four diagonal wheel forces whose longitudinal parts cancel in pairs, leaving a pure vy.

This rover cannot strafe, and the wheels are not the reason Every equation above describes hardware that is bolted to this rover today. The configuration cannot reach it: with MECANUMWHEELS=false the CORE2 firmware runs the two-degree-of-freedom differential model, and the command path carries a geometrymsgs/Twist whose linear.y is discarded downstream — there is no vy anywhere in the chain to strafe with, whatever Eq. [eq:mecik] permits. Enabling it means the firmware switch and a mecanum-aware odometry model (§12.4’s July conclusion, unchanged), which is real work rather than a checkbox. It stays listed as future work, not claimed as a capability.

### Four quantities, two sources each

Collecting the disagreements rather than smoothing them over:

@L2.6cmL3.2cmL3.3cmL3.4cm@ Quantity & One source & The other & Gap, and status
wheelradius & 0.0625 m, firmware, active & 0.0622 m, measured on this rover & 0.48 % — registry item 1; Eq. [eq:odoscaleval]
track & 0.354 m, manufacturer & 0.358 m, firmware / as-built & 1.1 %; neither tape-measured on this unit
mecanum radius & 0.0635 m, unused profile & 0.0622 m, measured effective & 2.1 %; the unused profile assumes the geometric radius, ignoring Eq. [eq:rollercomp]
kinematic model & 4-wheel skid-steer, physically & 2-wheel differential, in the estimator & deliberate approximation; online extrinsic refinement absorbs most of it

### A cockpit control so the data says which wheels produced it

None of the above was recorded anywhere a dataset could carry: a bag replayed in six months could not state which wheels were fitted when it was taken, although that single fact conditions how every distance and heading in it must be read. A two-position selector was therefore added to the Operations panel, and it is declarative on purpose: it does not write /firmware/diffdrive/*. That tree is read-only in leobackend.py by an existing, documented decision — writing it requires editing /etc/ros/param.yaml on the robot and restarting leo.service, which cuts the drive link, so it must stay a deliberate human act rather than a side effect of a click.

What the selector does instead is publish the declared set into telemetry (and from there into every export and bag), persist it across backend restarts because it describes physical hardware, write a logbook entry on change because a wheel swap invalidates comparability with everything measured before, and — the useful half — continuously compare the declaration against the radius the firmware is actually applying:

mismatch=undefinedif rfw unreachable,rfw−rdecl>ϵϵ=10−4 m.

The three-state result matters: with the parameter server unreachable the cockpit stays silent rather than asserting an agreement it has not checked. Verified end to end on the running robot on 2026-08-12, the check fires immediately and correctly, reporting rdecl=0.0622 against rfw=0.0625 — which is registry item 1, until now dormant in a configuration file and now visible on every telemetry tick.

## What each sensor actually constrains: an observability audit of this platform

The opening chapter recalled the four directions a monocular-inertial system can never fix: global position, and rotation about gravity. That result is about the trajectory. Two failures in this campaign came from a different and less-cited place — the observability of the calibration states, and of the vertical direction on a wheeled platform — and both cost days before being named. This section derives them, because in each case the fix is obvious once the algebra is written and invisible until it is.

### Extrinsic calibration is unobservable at rest

MINS carries the camera–IMU extrinsic TIC=(RIC,pIC) in its state vector and refines it online when docalibext is true. The question is what information a visual measurement carries about that pair when the platform does not move.

A feature f expressed in the global frame projects into the camera as

z=π​RIC⊤ RGI⊤(f−pGI)−RIC⊤ pIC,

with π the pinhole map and (RGI,pGI) the IMU pose. The filter learns pIC only through ∂z/∂pIC. Writing pC=RIC⊤RGI⊤(f−pGI)−RIC⊤pIC for the feature in camera coordinates,

∂z∂pIC=∂π∂pC⏟2×3−RIC⊤.

Equation ([eq:jacext]) is not zero: a single image does constrain the lever arm. The difficulty is that it constrains it in exactly the same way it constrains the position pGI, whose Jacobian is

∂z∂pGI=∂π∂pC−RIC⊤RGI⊤.

The two differ by the single rotation RGI⊤. If the platform is stationary, RGI is constant, and every measurement in the window sees the same pair of Jacobians related by the same constant rotation. Their columns then span a common subspace, and the information matrix acquires a null direction along

δpGI=−RGI δpIC,

which leaves every predicted measurement unchanged. Moving the camera one centimetre forward and declaring the rover one centimetre backward produces an identical image. Only a change in RGI breaks the tie, because it changes the relative rotation between the two Jacobians; that is, only rotation of the platform makes the lever arm observable.

Consequence, and it is not a subtlety. An estimator asked to refine an unobservable state does not hold it still. Along the null direction of Eq. ([eq:nulldir]) the measurements exert no restoring force, so the estimate executes a random walk driven only by its own process noise. Measured on a healthy instance, robot stationary, over 75 s: cam0 x from 0.2113 to 0.2096, y from 0.0373 to 0.0359, z from 0.0935 to 0.0946 m — monotone, ≈1.4 mm per minute, with no sign of convergence. Extrapolated, ≈8 cm per hour of standing still.

### From a drifting lever arm to a rejected measurement

A wandering extrinsic would be harmless if it stayed in the calibration block. It does not, and the coupling is quantitative. A lateral error Δ in pIC displaces the predicted projection of a feature at depth d by

δu ≈ f Δd,

directly from the pinhole map u=f x/z. With this rover’s calibrated focal length f≈337 px (Kalibr output, installed in configcamera.yaml), features at a typical indoor depth d=2 m and a decimetre of accumulated extrinsic error,

δu≈337×0.102.0≈17 px.

The Mahalanobis gate accepts a residual r when r⊤S−1r<γ, with S built from sigmapx =1 px and γ scaled by chi2mult =1. A 17 px residual against a 1 px sigma is a χ2 contribution of order 172=289 for two degrees of freedom, against a threshold near 6. Every visual measurement is rejected, in silence, and the filter is left with the wheels alone.

That is the entire failure chain, and each arrow in it was measured rather than assumed:

The fix follows from the first box, not the last: freeze docalibext. Estimating a quantity the data cannot resolve is not a refinement, it is an unbounded input to the filter. The cost of freezing is that the tape-measure seed becomes final — which is precisely why the outstanding Kalibr IMU–camera calibration was promoted from a precision improvement to the highest-value remaining task (§12.28, item 2).

### Why the wheels cannot save the vertical

The platform runs type: "Wheel2DAng", a differential-drive model built from two wheel angular rates. Its measurement, expressed in the body frame, is

vxωz=r2r2rb−rbωLωR,

with r the effective radius and b the track width. The model has two outputs. It says nothing whatever about vy, vz, ωx or ωy — not that they are zero, but that this sensor does not measure them. The measurement Jacobian with respect to the vertical position is identically null:

∂∂pzvxωz=0.

Gravity does not rescue it either. The accelerometer observes the direction of g, which fixes roll and pitch, but it enters the propagation as

v=RGI (am−ba)+g,

where any residual bias ba along the vertical is integrated twice into pz with nothing to contradict it. On this rover, therefore, the camera is the sole sensor constraining z, and the moment its contribution weakens — feature starvation, a χ2 gate closing on a bad extrinsic, an initialisation taken during a disturbance — the vertical is left to integrate freely.

Read the last column: three sensors, and only one of them says anything about height. This is not a redundancy gap that can be closed by tuning — it is structural, and it is why the vertical is the first axis to leave on this platform. Section 12.24.4 states what was and was not done about it.

### Clamping the output is not constraining the filter

Because z is physically constant on a flat floor, it is tempting to force it to zero and be done. That was implemented, and the distinction it forces is worth stating precisely, because it is easy to get backwards.

The dashed arrow is the point. poseselector is a pure consumer: MINS subscribes to /imu/dataclean, /jointstatesmins and the two image topics, and to nothing else — verified against the running node, not assumed from the launch file. No feedback path exists, so clamping the output changes what is displayed and leaves the estimate untouched. In particular:

- the filter’s covariance, its bias states and its X/Y accuracy are bit-for-bit what they were;

- the vertical error is still there, still coupled through the covariance to velocity and bias, so a clamped output can look healthier than the filter genuinely is;

- the symptom that has twice betrayed a locked initialisation — a visibly wrong z beside a healthy X/Y — is removed from the published pose.

The third point is why the raw value is republished on a separate topic and why a warning fires past 25 cm. A cosmetic improvement that deletes a diagnostic is a net loss, and this one came within one design decision of being exactly that.

Clamping the input of a transform is not clamping its output The first implementation forced z=0 on the pose handed to the TF broadcaster. That pose is expressed in the IMU frame; the broadcast composes it with the static TB​I before publishing odom→base_footprint, and that composition re-introduces the sensor’s mounting height:

TOB=TOI TB​I−1 ⇒ [TOB]z≠0  even when  [TOI]z=0.

Measured on the running system: the TF read −0.159 m while /robotposefused announced 0 for the same frame. The map and the pose would have disagreed with neither of them signalling it. The clamp belongs on the transform that is actually broadcast.

### A sampling trap on the replacement IMU

The MinIMU-9 v5 driver polls an LSM6DS33 configured at 104 Hz. A free-running loop at a nearby rate does not merely jitter; it aliases, and the two failure modes are not symmetric.

Let the true angular rate be ω(t) and the propagation integrate samples ωk over intervals Δt. A missed sample widens one interval: the trapezoidal estimate over 2Δt carries the usual O(Δt2) truncation error, and the integral remains unbiased. A duplicated sample is different in kind — the same ωk is integrated twice, so

θ−θ = ωk Δt + O(Δt2),

a first-order term that does not vanish with the step and does not average out, because duplicates occur preferentially while the beat between poll rate and ODR is slow. Integrated once more into position, a systematic first-order velocity error becomes a quadratic position error — the same mechanism, at smaller amplitude, as the accelerometer scale residual of §12.13.3.

The remedy is to publish only on a fresh sample. STATUSREG (0x1E) carries the layout

−  −  −  −  EV_BOOT  TDA  GDA  XLDA ,

so gating on XLDA∧GDA removes duplicates outright and leaves only the benign failure mode. The poll rate is then chosen above the ODR — 250 Hz against 104 Hz, roughly 2.4 polls per sample — so that each sample is collected within a few milliseconds of becoming available. Polling below the ODR, at the 100 Hz the driver first used, would have fallen behind by about four samples per second.

## The two calibration inputs the filter cannot check

An estimator validates its measurements continuously — that is what the χ2 gate is for. It never validates its calibration. The noise densities and the camera–IMU transform are read once at startup and trusted for the rest of the run, so an error in either propagates into every subsequent update with nothing to signal it. Both are quoted throughout this report; neither has so far been derived. This section does that, because in both cases the failure mode is a silent sign or unit error rather than anything the software would notice.

### Inverting Kalibr’s extrinsic, and why transposition is not enough

Kalibr reports Tcamimu: the transform that takes a point expressed in the IMU frame and re-expresses it in the camera frame. MINS’s configcamera.yaml wants the opposite convention, Timucam. The two are related by an SE(3) inverse, and the temptation — transposing the rotation and negating the translation independently — is wrong.

Write the Kalibr output in homogeneous form:

TCI=RCIpCI0⊤1,  pC=RCI pI+pCI.

Solving Eq. ([eq:tci]) for pI gives pI=RCI⊤pC−RCI⊤pCI, so

TIC=TCI−1=RCI⊤− RCI⊤pCI0⊤1.

The translation block is −R⊤p, not −p. The rotation must act on the translation before the sign is applied, and skipping that step leaves an error whose magnitude is the full lever arm whenever the two frames are not aligned. On this platform the camera–IMU rotation is close to a 90∘ composition (§12.3.2), so the naive form would misplace the camera by essentially the whole p≈0.18 m — an error Eq. ([eq:reproj]) converts into ≈30 px of reprojection at 2 m, well past the gate.

fillminscamchain.py applies Eq. ([eq:seinv]) and also copies the estimated time offset, which Kalibr reports in the same file and which is easy to leave behind. The rotation is the part that cannot be recovered later: Eq. ([eq:nulldir]) showed the translation is observable under rotation, but a 90∘ seeding error in RIC sits outside any region the filter can walk back from — which is exactly the vertical divergence recorded on 2026-07-04.

### Where the noise densities come from, and the unit trap in them

The four numbers in configimu.yaml are not datasheet values. They were measured on this unit by Allan-variance analysis of a long stationary recording, and their derivation matters because two of the four are routinely quoted in the wrong units.

For a signal sampled at fs, form averages over bins of duration τ and compute the Allan variance

σ2(τ)=12 (K−1)k=1K−1(yk+1−yk)2,

where yk is the mean of the k-th bin. Plotted against τ on log–log axes, an inertial sensor shows two asymptotes that correspond to the two noise processes the filter models:

σ(τ) ≃ Nτ⏟white noise ⊕ Bτ3⏟bias random walk,

a −1/2 slope at short averaging times and a +1/2 slope at long ones, with a minimum between them. The white-noise density N is read at τ=1 s on the descending branch; the random-walk coefficient B is read from the ascending branch at τ=3 s, where the τ/3 form makes the value the ordinate directly.

Continuous density against per-sample deviation The recurring error is not in Eq. ([eq:allan]) but in the units that come out of it. A gyroscope white-noise density N carries rad/s/Hz, i.e. rad/s; the per-sample standard deviation an implementer might measure directly is σsample=Nfs. At this platform’s ≈86 Hz the two differ by 86≈9.3, close to an order of magnitude. Substituting one for the other does not make the filter fail: it makes it quietly over- or under-confident, which is far harder to notice than a crash. The values below are continuous densities, as MINS expects.

Measured on this unit and installed in configimu.yaml:

| Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| Ng​ | gyronoise | 1.2637×10−3​ | rad/s/Hz |
| Bg​ | gyrobias | 3.3010×10−5​ | rad/s​2/Hz |
| Na​ | accelnoise | 2.3315×10−2​ | m/s​2/Hz |
| Ba​ | accelbias | 8.9292×10−4​ | m/s​3/Hz |

Two remarks close the loop with the rest of this chapter. First, these densities describe the CORE2 unit; the MinIMU-9 v5 that will replace it (§12.28) must be re-characterized before its numbers are trusted, and inheriting these would be exactly the datasheet fallacy this project has avoided elsewhere. Second, no Allan-variance figure protects against a scale error: the campaign’s accelerometer read 8.67 m/s​2 against a local gravity of 9.790 (§12.13.3), an 11.4 % deficit (equivalently, the 1.129 scale factor applied by imusanitizer) that is perfectly consistent sample to sample and therefore invisible to Eq. ([eq:allan]). Noise characterization and scale calibration answer different questions, and passing one does not excuse the other.

## The re-anchoring correction, and the offset it can freeze

The cockpit exposes a single pose topic and a button that changes which estimator produces it. The two estimators keep independent origins, so a raw switch would step the output by whatever separates them — tens of metres, in this campaign. poseselector therefore maintains a rigid-body correction per source and applies it before publishing. The mechanism is three lines of code and is described qualitatively in Appendix D; it is derived here because its one structural weakness produced a 142 m error on 2026-09-08 that looked exactly like an estimator failure and was not one.

### Deriving the correction

Let Traws(t)∈SE(3) be the pose published by source s, and Tfused(t) the pose served downstream. The selector stores one correction Cs per source and publishes

Tfused(t)=Cs Traws(t).

The requirement at a switch from a to b at time t* is that the served pose not jump:

Cb Trawb(t*) = Ca Trawa(t*) = Tfused(t* −),

which determines Cb uniquely:

Cb = Tfused(t* −) [Trawb(t*)]−1.

The inverse is the SE(3) inverse of Eq. ([eq:seinv]), with the same −R⊤p translation block. Each source is initialised with Cs=I, so before any switch the selector is a pass-through.

Two properties follow immediately, and the second is the whole problem.

##### The correction is constant between switches.

Equation ([eq:corrdef]) is evaluated once, at t*. For all t>t* the same Cb multiplies every incoming pose. Differentiating Eq. ([eq:corrapply]) with Cb constant, the served motion is the raw motion rotated into the anchored frame:

δTfused=Cb δTrawb,

so relative motion is preserved exactly. This is the desired behaviour: re-anchoring must not distort the trajectory, only re-place its origin.

##### The correction anchors to a pose, not to the world.

Cb encodes the offset between the source’s origin and the frame the output happened to be in at t*. It carries no information about whether that frame was correct. If Tfused(t* −) was wrong — because the previously active source had diverged — then Cb faithfully preserves that error, and keeps preserving it indefinitely.

### The failure mode: a correct estimator behind a frozen offset

Now let the upstream source reset. A respawned MINS re-initialises at a new origin, so Trawb changes discontinuously at some tr>t*:

Trawb(tr+) ≠ Trawb(tr−),  while  Cb(tr+) = Cb(tr−).

Nothing in Eq. ([eq:corrdef]) is re-evaluated, because no switch occurred. The served pose therefore becomes

Tfused=Cb⏟stale⋅Trawb⏟healthy,

a constant rigid offset applied to a perfectly good estimate. The signature is distinctive once known, and was measured directly on 2026-09-08:

@lrrr@ & x (m) & y (m) & z (m)

raw /mins/imu/odom & −0.0012 & +0.0016 & +0.0170
served /robotposefused & −0.4236 & −0.0846 & −142.0488

raw /mins/imu/odom & −0.0039 & +0.0012 & +0.0034
served /robotposefused & −0.0037 & +0.0018 & +0.0034

The deltas agree to a tenth of a millimetre while the absolute values differ by 142 m. That is Eq. ([eq:stalecorr]) written in measurements: relative motion tracked faithfully, absolute position offset by a constant. An operator watching the map sees a diverged robot; the estimator underneath is accurate to millimetres.

### Why re-selecting the source does not clear it

The instinctive remedy — press the button again, or call setsourcebyname — does not work, and Eq. ([eq:corrdef]) says why. Re-selecting source b while b is already active evaluates

Cbnew=Tfused(t−)[Trawb(t)]−1=CboldTrawb(t)[Trawb(t)]−1=Cbold,

because the served pose it anchors to is itself the product of the stale correction. The operation is idempotent: it recomputes the same matrix. The correction is preserved by construction, which is precisely what one asks of it at a genuine switch and precisely what one does not want here.

Restarting the node clears it, since Cs initialises to I. That is the procedure used on 2026-09-08 — and it dictates the ordering: the upstream estimator first, the selector second. Restarting the selector before MINS would anchor I against a still-diverged raw pose and serve the divergence directly.

A diagnosis that points at the wrong subsystem Equation ([eq:stalecorr]) is a factorisation, and both factors are observable separately — but only if one thinks to look. The served topic alone cannot distinguish “the estimator diverged” from “the estimator is fine behind a stale correction”, because the two produce identical output. The discriminator is the delta: a diverging filter has drifting deltas, a stale correction has deltas that match the raw source exactly. Reading the absolute value of /robotposefused and concluding that MINS had failed would have sent the investigation into the filter, where there was nothing to find.

### The durable fix, and why it was not applied blind

Equation ([eq:corrdef]) is evaluated on a switch. The defect is that a source reset is also an event that invalidates Cs, and nothing signals it. A principled repair is to have the selector detect the discontinuity itself — a jump in Traws far larger than vmaxΔt is not physical motion — and reset Cs to I rather than preserve it.

The plausibility guard already computes exactly this quantity for a different purpose: it freezes the output when a source jumps implausibly. The two behaviours must not be conflated, and that is the design question left open (§12.28, item 48). A transient spike should be held, as it is now; a genuine re-initialisation should re-anchor. Distinguishing them from the pose stream alone is not obviously possible, which is why the current code holds in both cases and leaves the operator the respawn — a documented procedure being preferable to an automatic behaviour that guesses wrong half the time.

## The calibration that was half-harvested

Appendix D lists the Kalibr IMU–camera calibration as the highest-value task still outstanding on this platform, and §12.24.1 explains why freezing docalibext made it more urgent rather than less. Both statements were written believing the calibration had never been run. It had. The output has been sitting in calibdata/ since 2026-08-20, it is numerically good, and half of it is already deployed — which is a more awkward situation than either having it or not.

### What exists

imucam-results-imucam.txt reports a converged solution against /imu/dataclean — the CORE2 unit, not the D455’s internal IMU, which the bag’s topic list confirms:

| Residual | value |
| --- | --- |
| Reprojection, cam0 | 0.187 px (median 0.170, σ 0.112) |
| Reprojection, cam1 | 0.201 px (median 0.161, σ 0.148) |
| Gyroscope | 2.64×10−5 rad/s |
| Accelerometer | 8.26×10−4 m/s​2 |
| Time offset, cam0 to imu0 | −0.862 ms |

Sub-pixel reprojection on both cameras. The four noise densities this run produced are, digit for digit, the four values in configimu.yaml — so this calibration was not ignored: its IMU half was adopted and is what MINS has been running on ever since. Its extrinsic half was not.

### Where the two disagree

MINS still carries the tape-measure seed. Against Kalibr’s measured Tic:

@lrrr@ & deployed & Kalibr & difference
x & +0.1500 m & +0.00330 m & +0.1467 m
y & +0.0000 m & +0.02631 m & −0.0263 m
z & +0.1000 m & +0.02051 m & +0.0795 m
& 0.169 m

The rotations agree closely: the deployed matrix is the exact 90∘ idealisation of Kalibr’s measured one, row by row ([0,0,1] against [−0.058,−0.076,0.995], and so on). That agreement is worth noting for its own sake — it confirms the frame convention used throughout this report is the right one — but it also isolates the disagreement entirely in the translation, where it amounts to 16.9 cm.

Through Eq. ([eq:reproj]) that lever-arm error is 28.5 px of reprojection at a 2 m feature depth, a χ2 contribution of order 810 against a gate near 6. If the deployed value were simply wrong by that much, no visual measurement would survive the gate at all.

### Why neither number can be installed as it stands

The temptation is to run fillminscamchain.py and be done. The calibration bag forbids it, and the reason is the null direction of Eq. ([eq:nulldir]): the extrinsic translation is observable only under rotation, so a calibration recording without vigorous excitation constrains it weakly no matter how good its reprojection residual looks.

Measured on imucam175145.bag itself:

| Quantity | value | assessment |
| --- | --- | --- |
| duration | 90.0 s | adequate |
| cumulative rotation | 611∘​ | modest |
| peak angular rate | 36.5∘/s | gentle |
| accelerometer σ, per axis | 0.20/0.10/0.09 m/s​2 | near-stationary |

The last row is the disqualifying one. An accelerometer standard deviation of a fifth of a metre per second squared is barely above what the unit reads sitting still; the recording contains almost no translational excitation. Kalibr’s reprojection residual can be excellent under those conditions and its translation still be poorly determined, because reprojection is dominated by the rotation — which is well observed here, and which is precisely the part that agrees with the deployed seed.

So the position is this. The rotation is confirmed twice over and can be trusted. The translation has two candidate values, a 15 cm tape measure and a 3.4 cm Kalibr estimate from a recording that could not observe it, and neither is established. Installing the Kalibr number would replace an unmeasured value with a badly measured one.

The consequence of the 2026-09-07 freeze, restated While docalibext was true, this ambiguity was survivable: MINS refined the translation away from whichever seed it started with, at least during motion. The freeze of §12.24.1 removed that path deliberately, on the measured grounds that the same mechanism random-walks at rest. That decision stands — but it means the seed is now permanent, and this section shows the seed is a tape measure that no calibration on disk is good enough to replace.

The action is therefore not “install the existing calibration” but “re-record it with proper excitation”. What was previously the highest-value outstanding task is now also a blocking one, and it is cheaper than it looks: the bag is ninety seconds of hand-held motion in front of a target, and the pipeline that consumes it already exists and has already run once.

## Open items registry

@cL6.2cmL6.8cm@

# & Item (status) & Required action

# & Item (status) & Required action

& Wheel radius/track, and now the whole kinematic profile (config frozen at stock values; 2026-07-21: leftward drift measured directly, ≈1.65 % right/left effective-radius bias (§12.8.8); masked in software by a bisected yaw trim, K≈−0.087 rad/s per m/s, not a substitute for the measurement below). Widened 2026-08-12 (§12.23): reading the live parameter server showed the firmware carries two complete profiles and runs the differential one (/firmware/mecanumwheels = false), so three independent mismatches stack against the fitted mecanum set — the radius (0.0625 stock against 0.0622 measured, +0.48 % on every wheel distance, Eq. [eq:odoscaleval]), the yaw multiplier (1.76, a rubber-tyre scrub compensation applied to rollers that do not scrub), and the model itself (2 DOF against 3). The cockpit now surfaces the radius half of this on every telemetry tick (§12.23.6) & roll-out measurement; update firmware diffdrive and configwheel.yaml; re-run slip protocol; remove the software trim (§12.8.8) once corrected. Separately, decide deliberately whether to enable MECANUMWHEELS=true with a matching odometry model or to keep the differential approximation — what must not continue is running a rubber-tyre profile against mecanum hardware without that being a stated choice
2 & Timucam (measured seed, refined online) & kalibrcalibrateimucamera (monocular cam0), inject via fillminscamchain.py
3 & TF baselink→camera/laser (placeholders) & calliper/CAD measurement; must agree with item 2
4 & Camera frame-rate margin: 10 Hz option CANCELLED (2026-07-08) & the D455 does not support 640×480@10 (supported: 6/15/30/60/90); the attempted change silently disabled all IR streams (driver fell back to a depth-only default profile) and caused a full vision outage until reverted. Lesson: validate any profile change against rs-enumerate-devices before deployment. 15 Hz retained; real-time margin to be sought elsewhere (feature floor, budget acceptance).
5 & Carolus / fixed-AprilTag global anchor (VICON slot staged) & enable ladder stage 3: drift-free global updates inside MINS
6 & RPLidar (node down) & bring-up, scan→cloud conversion, extrinsic
7 & AprilTag size cross-check & compare tag range against checkerboard range at identical position
8 & leoautonomy package (SLAM/exploration, written & built) & install dependencies, remap cmdvel through twistmux, field trial
9 & Zombie-guard false positives under Wi-Fi roaming (§ 12.6.5): leowatchdog.sh restarted the full stack every 1–3 min throughout the July 20 traverse; debounce widened (3→6 ticks, 10→20 s) but not yet re-validated in motion. July 22 (§12.9.3): restarts observed recurring at an exact 6-minute cadence over an 18-minute window, i.e. every single debounce tick failing with zero resets in between, sustained loss of registration visibility rather than isolated roaming blips & repeat an in-motion traverse with the widened debounce and confirm the false-trigger rate drops; if it persists, replace the rosnode list round trip with a check that does not require a live RPC to the robot’s master while roaming
10 & Watchdog liveness checks test node presence, not topic output (§12.8.1): firmwaremessageconverter stayed registered while silently producing zero /jointstates messages, blocking MINS indefinitely with no alert. Recurred July 22 (§12.9.3), same node, same symptom, same fix, this time costing ≈2 h of network-focused misdiagnosis before being recognised as the same fault, since the recommended fix below was still not implemented & add an output-rate probe (e.g. zero-publisher or stalled-rate detection on /jointstates) alongside the existing presence checks, now confirmed by a second occurrence to be worth the implementation cost
11 & Unbounded “still starting” grace period in the web healer (§12.8): a stuck rosbridge TLS process was left alone for hours because idempotency has no time ceiling. Recurred August 11 (§12.17.3), same defect, on the sibling plain bridge (:9090) this time: found ≈3h04 old against a 28-minute-old robot boot, masked for over three hours by the exact presence-only pgrep guard this item already named & replace the “maximum grace duration” idea (a threshold to tune) with the tuning-free test derived from the August 11 recurrence, EP(t)>U(t) (Eq. [eq:staleness]): any PC-side ROS process whose own elapsed time exceeds the robot’s current uptime is provably stale and should be killed and relaunched by the same watchdog cycle that already reads both quantities
12 & WireGuard tunnel failure that outlived every restart tried (§12.8.3): endpoint roaming (Eq. [eq:roam]) explained and fixed the first occurrence, but a second, unrelated failure resisted restarts on both ends, a port change, and a 4-minute idle wait, while ICMP/SSH to the robot stayed healthy throughout. A milder recurrence was observed July 22 (§12.9.3): a ≈15 s reachability loss that self-recovered without intervention & obtain campus-side firewall/NAT logs for the failure window if possible; otherwise instrument the tunnel with a lighter-weight same-subnet fallback (§12.8.4) as a permanent, not just emergency, option
13 & Ground-station Wi-Fi (rtl88x2bu) is back on the critical path (§12.8.4) whenever the same-subnet fallback is in use, reintroducing the driver fault Ethernet was originally chosen to avoid (§12.6.4) & replace the adapter (Intel/MediaTek chipset) so the same-subnet path is not gated by known-defective hardware
14 & Discontinuous, roughly one-second-burst motion reported during the July 21 traverse: the dominant cause is now identified and fixed (§12.8.7, the manual-drive deadman tripping under operator-device Wi-Fi roaming); the pose-availability floor (Apose≳0.85, §12.8.6) and an unmeasured Wi-Fi image-bandwidth risk remain as secondary, undistinguished contributors, mainly to the separately-reported missed-obstacle symptom & re-validate the deadman fix over a full traverse; trace /jointstates and image-topic Hz directly on the Pi, timestamped against restart_stack.sh’s own init log, to attribute any remaining roughness to pose gaps, image starvation, or both
15 & AUTO mode control-loop hardening (§12.8.9: patrol-steering EMA, avoidance-direction EMA, lock hysteresis, DEPTHALIVETIMEOUT) deployed but not yet validated over a full traverse: the test window that would have exercised it was consumed by the disk-full incident (§12.8.10); the July 22 session (§12.9) added further, still-unvalidated hardening to the same loop, dual-confirmation beacon reset and a first PID retrofit (items 17–18 below) & repeat an AUTO-mode traverse once network conditions are ordinary; confirm the depth-alive gate actually fires (not just compiles) by forcing a depth-topic stall while facing a wall
16 & Robot disk usage is unmonitored until a downstream symptom (camera failures that resist hardware reset) reveals it indirectly (§12.8.10); leo.service has no restart policy, so a crash (SIGSEGV observed, root cause unconfirmed, plausibly a disk-full write failure) stays dead until noticed by hand & add a disk-usage probe to leowatchdog.sh (warn well before 100 %; the /var/ros/log growth alone justifies a periodic purge); add Restart=on-failure to leo.service so a future crash self-heals like every other component in this architecture
17 & Three PID loops (§12.9.2) deployed live but not yet exercised over a full AUTO traverse under ordinary network conditions (§12.9.2.5); no gain is derived from a plant model of this rover, every gain is either reused unchanged from a field-tuned predecessor or, for the obstacle governor (no predecessor existed), chosen from a geometric margin argument rather than measured response & a traverse confirming the intended qualitative effect (smoother patrol heading, earlier and smoother wall-approach deceleration) without new oscillation; if warranted, a measured step-response or frequency-domain characterisation of at least the obstacle governor to replace the margin-based Kp (Eq. [eq:obstfloor]) with a derived one
18 & Dual LED+checkerboard confirmation (§12.9.1) assumes the two cues are simultaneously valid at some reachable pose; not yet confirmed against this beacon’s actual physical layout. If structurally impossible (checkerboard readable only from a face where the LEDs are not bright enough, or vice versa), DUALCONFIRMGRACES will expire on every attempt and no beacon will register autonomously & bench-test at the intended stop distance whether ledstable and lmhits (§12.8.9’s vision-loop counters) both cross their confirmation thresholds at any single pose before relying on this gate in an unattended traverse
19 & Pi↔CORE2 serial link (serialnode, 250,000 baud) logged 20,295 checksum/lost-sync errors in 2 h on July 22 (§12.9.3), the leading, plausible-but-unproven hypothesis for why firmwaremessageconverter’s wheel subscription failed at its most recent onset; independent of item 10’s watchdog blind spot and unresolved by this session’s fix. Still active July 23: a live measurement during this session’s audit read ≈200 errors/min, worse than the July 22 figure, not better & a measurement tool (tools/checkserialhealth.sh) and a physical-inspection checklist (tools/diagserialpicore2.md) now exist so a before/after comparison is possible; physically inspect the Pi↔CORE2 connector/cable for a marginal connection per that checklist
20 & ovmsckf (VINS) found not running at all (no process) during July 22 verification of §12.9.4’s unrelated MINS/tf2_ros change; removes the VINS fallback until restarted, cause and onset time not established. Addressed July 23: leowatchdog.sh now carries a presence check for /ovmsckf, symmetric to minssubscribe’s (same 6-tick debounce, same restartstack.sh remedy) & not yet field-validated against a real VINS dropout — the original incident was never reproduced under controlled conditions, so the new check’s effectiveness is inferred from symmetry with MINS’s, not directly observed
21 & Return-to-Base’s full-trajectory retracing (§12.10.1, Eq. [eq:rtbwaypoints]). Exercised July 24: multiple live multi-waypoint retraces occurred (Table [tab:rtblog]), exposing a genuine safety gap — the reverse leg had no obstacle or stall detection of any kind (§12.11.2) — now closed by a two-tier guard plus a graduated resistance/obstacle governor (§12.11.2.1–§12.11.2.2) & a further live traverse specifically exercising the new guard’s graduated response (Eq. [eq:rtbresistscale]), not just its two hard trips, which Table [tab:rtblog]’s single post-fix run already exercised once
22 & The initial-divergence guard’s threshold (MAXINITIALJUMPM=5 m, Eq. [eq:initguard], §12.10.3) is a bench choice from a single observed incident (VINS at 27.5 m), not a derived or measured bound & characterise against a real distribution of clean vs. divergent VINS initialisations across multiple sessions before treating 5 m as more than a first cut
23 & The deterministic 35∘-left avoidance policy (§12.11.5, Eq. [eq:avoidcumulative]) has not been exercised against a corridor obstructed specifically on the left, the scenario in which up to three cumulative left turns are attempted toward the blocked side before the 180∘ U-turn fallback (AVOIDMAXTRIES=3) rescues it & a live trial in a room with a known left-side obstruction (a doorway or corridor bend), confirming the U-turn fallback engages within the expected three attempts and that no contact occurs during them
24 & The 20th-percentile threshold replacing the median for direction selection (§12.11.4, Eq. [eq:percentilebias]) is a bench choice reasoned from first principles (a point between the existing q=50 and q=5 statistics), not fit to a measured distribution of real obstacle occupancy fractions f & log f (fraction of near-range pixels) for real obstacles encountered over several sessions and confirm q=20 remains above the smallest f actually observed in practice
25 & The RTB resistance-governor thresholds (ρlo=0.35, ρhi=0.75, Eq. [eq:rtbresistscale]) and the stall detector’s window/displacement pair (Tstall=2.0 s, dstall=3 cm, Eq. [eq:rtbstalltrip]) are reasoned against the platform’s known static noise floor (Table 12.3) but not tuned against a live distribution of genuine partial-resistance events (wheel slip, minor snags) & collect ρ[k] (Eq. [eq:rtbresistratio]) over several ordinary RTB runs with no actual obstruction, to confirm the ratio stays above ρhi under normal reversing and the governor does not fire spuriously
26 & BOOSTANG=0.9 rad/s (§12.11.6, Eq. [eq:boostvalues]) is inferred from §12.5.2’s ramp characterisation, which measured yaw authority only up to 0.40 rad/s commanded, not from a ramp measurement at boost-level commands themselves & repeat tools/rampyaw.py’s protocol with commands spanning BOOSTANG, confirming authority does not degrade (or quantifying by how much it does) above the previously characterised range
27 & The widened LED area window (3,2500) px​2 (§12.11.10, Eq. [eq:ledratio]) clears the derived 64× ratio for the requested 0.5–4 m range but is reasoned from a single 1 m calibration point, not measured at either end of the range itself & capture and replay real frames at 0.5 m and at 4 m through the offline LED detector (the method that already resolved §12.5.5), confirming the widened bounds pass the genuine beacon without admitting new false positives
28 & BEACONSPATIALMAXFRAC=1.5 (§12.11.9, Eq. [eq:spatialscale]) bounds the LED/checkerboard separation ratio conservatively above the beacon’s true physical sgap/scb, which has not itself been measured on the physical beacon & measure the beacon’s actual LED-cluster-centre to checkerboard-centre offset and width directly, and tighten the fraction toward the measured ratio if margin allows
29 & ovmsckf (VINS) still builds against the deprecated ROS1 tf package (ROS1Visualizer.{h,cpp}, ROSVisualizerHelper.h, §12.11.11), the same class of debt §12.9.4 closed for MINS on July 22; isolated via /tfvins (harmless today) but not yet ported & mirror §12.9.4’s field-for-field port against the same upstream ROS2Visualizer.{h,cpp} template already used for MINS
30 & No map frame or map→odom transform exists anywhere in the shared /tf; the mission’s world-referenced concepts (RTB, obstacle/beacon mapping) instead consume a parallel, non-TF SE(2) state (worldoriginx/y, worldheading, Eq. [eq:worldse2], §12.11.11) re-anchored by hand at each beacon reset & implement the sketched tf2ros.TransformBroadcaster-based map publisher (§12.11.11) and migrate getworldpos()’s callers to tf2ros.Buffer.lookuptransform(); nontrivial (multiple call sites across RTB/mapping/LOCK) and not attempted on the live robot without a dedicated validation pass
31 & The re-armed pose-source switch’s automatic-apply path (§12.11.7: onodom() completing an armed switch on a source’s first message) is verified by code inspection and by a manual arm/cancel round trip only, not by an actual drive that lets an armed, previously-silent source publish and confirms the switch fires without operator action & arm VINS while stationary, then drive to give it motion, and confirm both the posesource and posesourcepending topics (and the cockpit buttons) transition automatically at the same instant /ovmsckf/odomimu starts publishing
32 & navigationsupervision.launch’s poseselector defaultsource was corrected from "VINS" to "MINS" (§12.11.7) after the wrong default silently dropped /robotposefused to zero on a live restart; the fix is in the launch file but not yet on the running parameter server, which still holds "VINS" from the stack’s last full launch & confirm defaultsource reads "MINS" on /poseselector after the next full stack restart (restartstack.sh or leo start), not merely a single node respawn
33 & VioManager::vioalive() and isinitializedvio’s std::atomic<bool> conversion (§12.11.8) are real, independently-justified fixes but were found and verified against a filter that had not itself initialised this session (§12.11.8’s log-attribution correction); neither has been observed live doing the one thing it was designed for — keeping odometry served once ovmsckf initialises and then goes (or stays) stationary & after a live jerk-triggered initialisation succeeds (Table [tab:openitems], item 31’s drive trial covers getting there), let the rover sit still afterward and confirm /ovmsckf/odomimu keeps publishing rather than stopping once ZUPT starts succeeding on every frame
34 & The trajectory-comparison pipeline (§12.12.1) and its Figure 12.13 slot are validated only on a stationary bench capture (MINS at rest, VINS uninitialised, no beacon); no real driven MINS-versus-VINS overlay with VINS initialised, and no Carolus-referenced error figure, has yet been produced & drive a manual loop with VINS initialised (item 31’s jerk) past a Carolus beacon, run recordtrajectories.sh → bagtocsv.py → plottrajectories.m, and drop the resulting figures/trajcomparison.png into the report to replace the placeholder
35 & The two-test protocol of §12.12.7 (Figure 12.23, comparetests.m) is built and verified against synthetic data only; the closing analysis of §12.12.7 is fully scaffolded with \ todo{} placeholders but has no real numbers, because the two drives it depends on (Test 1 under MINS, Test 2 under openVINS, same lab loop) have not yet been run & drive Test 1 and Test 2 from the cockpit Trajectory tab per §12.12.7’s protocol, run comparetests(), and fill in every \todo{} in §12.12.7 from its printed output
36 & Colour-camera geometry: four stacked defects, fixed and validated against a tape measure 2026-08-18, CLOSED (§12.15.3). The recorded defect stands: intrinsics from a 1280×720 calibration used on a 640×480 stream, principal point outside the image. The launch audit found three more, all silent: fov=true selected the Astrobee FOV lens model (reading only distortion[0] as ω) although the D455 declares plumbbob; and distortion and knownpoints were both <param> strings, so each getParam into a vector<double> failed silently and fell back to hard-coded defaults — neither had ever been applied. Fixed: factory intrinsics for this exact profile, fov=false, and real <rosparam> lists. Validated: beacon at 23″ (0.5842 m) to the tip of the LED protruding 55.5 mm ahead of the cross plane; expected range to the beacon origin 0.6397 m, measured 0.6393 m over 40 samples (σ=0.05 mm) — 0.4 mm, 0.06 % & closed. One caution kept on the record: validated at a single range (≈0.64 m) and a single measurement. A residual scale error grows linearly with distance, so a second check near 2 m would be far more sensitive and is the cheapest way to confirm this holds. Items 37 and 5 are ungated
37 & Beacon-anchored absolute error: architecture complete, never recorded (§12.15.2). carolustfbridge.py publishes /odominbeacon and /carolusinbeacon, which together give an absolute error with no fitted transform and no scale freedom, superseding the Kabsch/Umeyama residuals of §12.14.2. It has been run and observed to publish, but no bag recorded to date contains either topic & once item 36 is cleared, record a driven run with both topics in the bag and report their instantaneous difference; this is the measurement the whole comparison programme has been building toward
38 & Thermal envelope is three degrees wide — active cooling installed August 11, matched-load A/B run August 13, CLOSED (§12.15.1, §12.17.2, §12.18). The July 28 scheduling fix holds the serial link at 86 ∘C under full colour load and does not hold it at 89.1 ∘C, where the chain collapses entirely (676 751 UART overruns, load 8.85, all firmware topics at 0 Hz). Two GPIO fans hold the ARM clock at 1500 MHz under every load tested, 89.1 ∘C never approached. The controlled comparison this item specified — overrun rate, July 30’s load (colour nodelet, both estimators), cooling installed, two independent 180 s windows — gives 2.074 s−1 pooled against the July 30 reference of 3.07 s−1: a 32.4 % reduction, not elimination & closed as specified; the 67.6 % of the original overrun rate that survives zero throttling is not this item’s question any more, it is item 40’s — see its updated entry below
39 & Second absolute-error method: implemented, never measured (§12.16.6). Item 37’s beacon bridge is gated on item 36; this session added a complementary method on already- calibrated hardware (the Kalibr’d IR pair, item 2) that needs neither the tag’s world position nor the camera–IMU extrinsic, only a fixed tag re-sighted from the same spot. The detector, the recording path, the CSV format and tools/tagdrift.py are all deployed and verified against synthetic data with a known answer; no tag has yet been placed at the rectangle’s third corner for a real multi-lap run & mount an AprilTag at a fixed, re-visitable point (the rectangle protocol’s third corner is already instrumented for it, §17.10.3); drive at least two laps past it in one recording; run tagdrift.py on the result
40 & A live UART checksum storm — root mechanism caught directly by ftrace (§12.17.5, §12.18, §12.19). Motion excitation and the kernel’s RT budget both refuted by controlled measurement; temperature ruled insufficient (item 38: two thirds of the July 30 rate survives zero throttling). irqsoff tracing on IRQ 14 then found the mechanism directly: two independent captures, 2829 and 3064 μs of interrupts disabled, both >​2.2× the 1.28 ms FIFO deadline (Eq. [eq:fifodeadline]), both a swiotlb-bound USB (xHCI) descriptor drain, one preceded by SD-card interrupt handling. Capture 2 brackets the UART directly: serviced at t=10 μs, not again until t=2889 μs (Eq. [eq:pl011gap]) — a gap wide enough for ∼​40 bytes to overflow the FIFO in one occurrence. Two of three mitigation candidates checked the same evening: DMA mask ruled out directly (64-bit device, 16 GiB platform window, both confirmed by reading the hardware/device-tree, not inferred); IRQ affinity for xHCI refused outright by the kernel (I/O error, this MSI controller does not support runtime re-steering), MMC’s move gave an inconclusive result against a load that drifted on its own across the test window, so re-run with a proper B–A–B–A matched protocol (4×120 s): mean 2.180 s−1 (CPU1) against 2.504 s−1 (original) — a 12.9 % nominal gap smaller than the spread within the CPU1 samples themselves (0.872 s−1), not a confirmed effect either way. Third candidate closed: bcm2835mmc exposes only two module parameters, both debug-logging flags, no timing control & a kernel patch to bcm2835mmc is the only route left to the udelay busy-waits, not attempted (this Pi’s SD card is also its boot device); the MMC-affinity question needs more statistical power than four 120 s windows gave it — longer or more numerous runs, not a different protocol
41 & VINS on the Pi: root cause of the ZUPT χ2 defect found and patched, the patch verified, the platform still diverges (§12.20, §12.20.6). Builds and runs at 30–70 Hz on live sensor data; on one recorded lab lap, MINS 1.02 m loop closure, PC openVINS 541.7 m, VINS unbounded. Reading UpdaterZeroVelocity.cpp found the actual bug behind both ZUPT sub-mode failures: an unchecked LLT().solve() plus a reject condition the disparity-only path bypasses unconditionally when stationary. A minimal additive patch (numerical-validity guard, 64-line diff, docs/sqrtvinszuptnumericalguard.patch) was built and run live for 60 s: 588 correct rejections, 241 correct acceptances, zero inf/garbage χ2 accepted — the reported bug no longer reproduces. Same run, patch active: the filter still diverges anyway, smoothly, 1015→3.4×1017 m over 60 s, proving a second, separate numerical issue survives outside the ZUPT updater & the RPNG bug report is updated with the patch and this finding, ready to submit, not yet submitted; the second divergence source (propagation or the MSCKF update, not yet diagnosed) is the only path left on this platform — reading Propagator.cpp and UpdaterMSCKF.cpp with the same discipline, not attempted; no driven test follows until that source is found, per this item’s own at-rest-first condition
42 & MINS ROS1 entry point was an empty stub: the binary loaded its configuration and exited without subscribing to anything (patched locally 2026-08-31; independently corroborated 2026-09-04). Upstream mins/src/runsubscribe.cpp implements the ROS2 path in full (rclcpp::init, executor, spin) but leaves the #elif ROSAVAILABLE == 1 branch empty, a gap dating from the ROS2 port two years earlier: every other file and directory under mins/src/ carries a recent upstream commit, that one file does not. On ROS1 the node printed op->loadprint()’s summary and returned, with no crash and no error message, so the failure presents as a successful run. Patched here by mirroring the ROS2 flow (ros::init, a NodeHandle, ROSPublisher and ROSSubscriber, ros::spin()). M. Crane reached the same defect independently and patched it as a parallel file (runsubscribeMWC.cpp), which establishes one dormant upstream bug rather than two divergent MINS versions & document the patch in the body text, not only in the source comment; adopt the defensive configuration-path read from Crane’s version (nh->param<std::string> with argv[1] retained as the fallback default), which tolerates a launch file passing the path through <param> instead of args=, the exact failure already documented in §17.11.3
43 & MINS stays on the ground station: the embedded option is closed on measurement, not on a failed attempt (architecture decision, §17.16). MINS was never executed on the Pi; the verdict rests on an extrapolation whose inputs were each measured. MINS costs 85 % of one i7-8700 core with the robot stationary and dynamic cloning at its 9 Hz floor, a best case; the camera path is 90 % of that cost. A single-core floating-point benchmark put the workstation at 4.05× the Pi 4B (436 against 108 operations/s), which places MINS near 3.4 Pi cores against roughly 2.3 free. The thermal margin closes the question rather than the arithmetic: the Pi idles at 70.6 ∘C with the throttle flag already armed, and the 80–85 ∘C threshold drops the clock to 600 MHz. Neither that threshold nor the 600 MHz state has been observed under MINS load, because no such load was ever applied. A leopi profile exists (mono, downsample: true, online calibration off) and may well run, but it is unmeasured and it costs stereo on a platform where the camera is the only sensor constraining z & keep the estimator on the ground station and treat the embedded path as reopened only by different hardware (Jetson class), not by further tuning; if the leopi profile is ever to be claimed as viable, measure it on the target rather than extrapolating a second time
44 & Battery below the project’s own threshold, coincident with a silent motor-command failure; the link is plausible and undemonstrated (2026-09-02). During an investigation into the rover not moving, /firmware/battery read a stable 9.33–9.41 V across ten samples, against the 10.3 V threshold this project sets in leobackend.py and against the 10.3 V that had already produced motor torque sag on 2026-07-10 (§12.5.2). Concurrently pwmdutycycle read [0,0,0,0], /cmdvel reported Subscribers: None (the CORE2 rosserial subscription was not registered at all), and 261 checksum errors were logged in two minutes, while telemetry continued normally at 20.1 Hz on wheel states and 87 Hz on the IMU. Healthy telemetry is therefore not evidence of a healthy power rail, since telemetry draws almost no current. The chain brownout → failed rosserial negotiation → no subscriber → no motion is consistent with every observation but has not been demonstrated & recharge and re-read /cmdvel: if Subscribers: None persists at full voltage the fault is separate and must be diagnosed on its own terms. Independently of that outcome, surface battery voltage as an operator-visible alarm below 10.3 V rather than leaving it a value one has to think to query, and read it first in any future motor or actuation investigation
45 & Dual-IMU evaluation requested: MINS propagates from exactly one IMU, so this is an engineering effort and not a configuration change (H. Gutierrez, 2026-09-04; scoping done, nothing implemented). State holds a single sharedptr<IMU>, there is one feedmeasurementimu() entry point, and the five updater types present (cam, wheel, vicon, gps, lidar) include no IMU updater, because in this architecture the IMU drives propagation rather than correcting it. On this rover the D455’s onboard IMU is additionally not active anywhere: every IMU-derived topic traces back to the CORE2 body unit. Phase A was executed on 2026-09-06 and returned a blocking negative result. The motion module is confirmed present at four independent levels (USB descriptor exposing a HID interface; kernel modules hidsensoraccel3d and hidsensorgyro3d loaded; IIO nodes accel3d and gyro3d; and the driver’s own Motion Module was found), and it was disabled only by omission: /etc/ros/robot.launch never overrode the upstream defaults enablegyro: false and enableaccel: false. Enabling it publishes /camera/imu (sensormsgs/Imu, gyro 200 Hz, accelerometer 100 Hz) — but the RealSense node then never reaches RealSense Node Is Up, restarts in a loop, and all three image streams stop. Both publication modes fail identically (linearinterpolation and none), so the interpolation thread is not the cause. Two candidate explanations were eliminated by measurement: USB bandwidth (the link negotiates 3.20 at 5000 Mbit/s) and the libusb controltransfer warning storm, which is present with the IMU disabled as well. The configuration was reverted and the chain reverified. Separately, the driver reports IMU Calibration is not available, so this unit carries no factory IMU calibration. Kalibr as installed ships no IMU-to-IMU extrinsic tool; tools/composeimuextrinsics.py now chains two camera-IMU calibrations through the shared camera, validated against Kalibr’s independently computed stereo baseline to 4.6×10−15 mm & The driver stall is now the critical path: nothing in any dual-IMU strategy is reachable until it is understood. Treat it as a timeboxed spike (librealsense version, kernel HID binding, camera firmware, in that order) and re-plan afterwards. Phase B, only if the IMU can be brought up without losing the cameras: characterise it by Allan variance rather than assuming the existing densities apply, then use it as the sole propagating IMU to establish whether it beats the CORE2 unit at all, before implementing an IMU updater on the pattern of the existing wheel updater. Joint dual-IMU propagation is a third, far larger effort touching State, StateHelper and the propagator, and is not recommended as a continuation of either phase. A standing reservation applies to all of them: the dominant inertial error here is a scale error on the CORE2 accelerometer that drifts over weeks, and least-squares averaging with a second, uncalibrated IMU does not remove a systematic scale error — the defensible gain is fault detection, obtainable from a consistency monitor far cheaper than a fusion node
46 & MINS cannot ingest vicon measurements in live mode: the ground-frame anchor rests on a subscription that does not exist (found 2026-09-06 by reading the source, while tracing the sensor inputs for a handoff document). ROSSubscriber registers exactly five subscriptions — IMU, camera, wheel, GPS, lidar (ROSSubscriber.cpp lines 52, 69/79, 87, 94, 102) — and there is no vicon subscription, in the ROS1 or the ROS2 subscriber. SystemManager::feedmeasurementvicon() is reached only from runsimulation.cpp:135 and runbag.cpp:188, and the converter ROSHelper::PoseStamped2Data likewise only from bag playback. Setting configvicon.yaml enabled: true would allocate vicon extrinsic state and begin publishing /mins/vicon0/pose while no measurement ever reaches the filter — a failure that presents as “the anchor has no effect”, with no error raised anywhere. This is the same shape of defect as the ROS1 entry point (item 42): a path implemented for bags and simulation but never wired into runsubscribe. It matters because the Carolus/AprilTag beacon fix is the platform’s only route to a surveyed ground frame: MINS’s global origin is the pose at which the filter initialised, gravity-aligned through imugravityaligned: true but with arbitrary yaw and position, and the downstream poseselector SE(3) correction only keeps the fused output continuous across a source switch — it registers nothing to the world & Implement a vicon callback in ROSSubscriber, mirroring the four that already exist; the conversion helper ROSHelper::PoseStamped2Data is already written and can be reused unchanged, so the work is subscription plumbing rather than new estimation code. Until then, treat any plan that begins “enable the beacon anchor” as blocked, and do not schedule beacon-placement sessions against it: a beacon physically in the camera’s field of view changes nothing while the measurement has no path into the filter. Verify the fix by the arrival of measurements, not by the appearance of /mins/vicon0/pose, which publishes either way
47 & MINS diverges spontaneously with every input healthy, and a respawn is the only known remedy (2026-09-08: two distinct failures within one hour on the same process). The first was a locked initialisation: x,y correct at (0.74,0.30) m but z=−327.08 m, and stationary — 3.6 mm of movement over 30 s, amplitude 6.7 mm, −0.01 m/min. The filter was tracking well, simply anchored 327 m too low, having initialised during the network instability that had made the robot unreachable the previous evening. The second, thirty minutes after the repair, was an active runaway at 562 m/min with the robot immobile (z from +1325 to +1138 m in 20 s). Inputs were measured healthy during the runaway: IMU at 85 Hz with |a|=9.7894 m/s​2, both infrared streams at 7.5 Hz, wheels at 18.7 Hz — so the fault is internal to the filter, not a sensor dropout & Do not treat the respawn as a fix: it restores 6.3 mm accuracy (drift 0.9/0.8/0.4 mm over 30 s) but the cause remains unidentified, and the failure recurred within the hour. Instrument the filter’s own covariance and chi-squared acceptance rates over time rather than only its output pose, since both failures were invisible in the pose until they were catastrophic. Note that the two signatures differ — one frozen, one runaway — so a single mechanism is not yet established
48 & poseselector keeps a stale SE(3) correction that survives an estimator reset, masking a healthy filter (2026-09-08, measured). The node holds one correction matrix per source so the fused output does not jump when the operator switches estimator. Computed while MINS sat at z=−327 m, it survived the MINS respawn: the selector then tracked the repaired filter faithfully — deltas identical to a tenth of a millimetre — while adding a constant 142.07 m offset. The cockpit therefore showed a diverged robot while the estimator underneath was accurate to millimetres & Re-selecting the source does not clear it: the correction is recomputed to preserve continuity with the (wrong) fused pose, which reproduces the offset. Restarting the node is required, since correction initialises to identity. Enforce the ordering sanitiser → MINS → poseselector after any estimator reset. A durable fix would invalidate the cached correction whenever the upstream source reports a reset, rather than relying on the operator to remember the sequence
49 & An IMU–camera calibration exists, is half-installed, and cannot be finished with the recording on disk (2026-08-20 output, found 2026-09-09, §12.27). Kalibr converged against /imu/dataclean with 0.187/0.201 px reprojection, and its four noise densities are already deployed digit-for-digit in configimu.yaml. Its extrinsic was never installed: MINS still carries the tape-measure seed, 16.9 cm away in translation, worth 28.5 px of reprojection at 2 m. The rotations agree closely, so the disagreement is purely the lever arm & Do not install the existing value. The calibration bag carries 611∘ of cumulative rotation, a 36.5∘/s peak and an accelerometer standard deviation of 0.20/0.10/0.09 m/s​2 — near stationary, so the translation is weakly observable by Eq. ([eq:nulldir]) whatever the reprojection residual says. Re-record with vigorous six-axis excitation and re-run the same pipeline, which already exists and has already run once. Since docalibext was frozen on 2026-09-07 the seed is permanent, so this is now blocking rather than merely valuable
50 & No estimator recording exists after either divergence patch (§11.10.9). The sqrtVINS ZUPT guards are dated 2026-08-24 and the openVINS covariance reset 2026-08-28; the most recent recording in data/ is from 2026-08-24, and it carries no covariance columns. Every consistency result in this report therefore describes software that has since been changed & Record one stationary run and one drive on the current build, exported with the Ppose and Ptwist columns, and re-run the NEES procedure of §11.10. About twenty minutes of robot time. It would date the covariance audit to what is actually running and close the before-and-after on both patches, neither of which has a measured effect today. Partiellement clos le 2026-09-09 : une course statique de 242 s sur le build courant date desormais l’audit MINS (§11.10.10) ; les deux VIO n’ont rien publie, ne s’initialisant pas a l’arret, de sorte que leur datation demande une course commencant par une excitation puis s’immobilisant, sans redemarrage entre les deux

In summary, the campaign transformed the estimation layer from a divergent prototype into a measured, guarded and self-healing system, quantified end to end: millimetric planar stability, bounded vertical noise, a real-time visual front-end within 2× of its budget floor, and an operator platform whose every indicator is backed by a live topic. July 24’s session closes the campaign’s remaining collision-safety blind spot: every branch capable of commanding forward motion in AUTO mode now carries both a hard obstacle check and a graduated speed governor (Table 12.5), including the one direction the depth sensor cannot observe at all, where an odometry-inferred stall detector and resistance governor (§12.11.2.1–§12.11.2.2) now stand in for the sensing that physically cannot exist. The remaining precision and validation roadmap is fully enumerated in Table [tab:openitems], with the rigid-wheel radius re-measurement (item 1) and a live trial of the deterministic avoidance policy against a left-blocked corridor (item 23) the highest-priority metrological and safety actions respectively.

## Anatomy of a starved visual channel: openVINS on the LEO rover

Over a single instrumented session (2026-08-20) the visual–inertial estimator was taken from “it drifts” to a quantified diagnosis. Three defects were found, measured and corrected; seven candidate explanations for the residual failure were eliminated by measurement rather than by argument. This section records the mechanisms, because each is a trap that costs a night to rediscover.

The single number that governs everything below is the count of visual features the filter actually consumes per update. openVINS is configured with max_msckf_in_update: 40. Measured on this robot, over 1793 updates while driving:

nfeat=0.75,  nmax=25,  nfeat40=1.9 %.

The estimator is therefore not a visual–inertial filter in practice but an inertial integrator with occasional visual corrections. Every symptom follows from that.

### The measurement funnel, and where it empties

The front end is not idle. Its debug overlay draws roughly 1.19×104 coloured pixels per frame, so points are detected and tracked. The loss happens between detection and the state update.

Where the visual measurements are lost. Detection is healthy; the collapse happens downstream. Each annotation is a hypothesis that was tested and its measured outcome. The stereo matching stage cannot be blamed on a mis-declared extrinsic, because TrackKLT derives the epipolar constraint by RANSAC from the observed correspondences.

### Defect 1: a circular ZUPT lock

Zero-velocity updates are gated by an estimated-velocity threshold vmax (zupt_max_velocity). The update is applied only when

∥v∥<vmax.

A residual specific force ε integrates into the velocity estimate between corrections,

∥v(t)∥ ≈ ε t,

so once ∥v∥ exceeds vmax the gate [eq:zuptgate] can never re-open. The estimate is trapped: the quantity that decides whether the correction runs is precisely the quantity the correction was meant to fix.

The lock. Above the threshold the negative feedback path is severed, and the residual keeps driving the estimate.

The prediction of [eq:vgrowth]–[eq:zuptgate] is that ∥v∥ settles slightly above whatever vmax is set to, for any value of vmax. This was tested directly:

The plateau tracks the threshold, confirming [eq:zuptgate] as the binding constraint. At vmax=1.5 the estimate converged monotonically to 1.588 m/s over 55 s and stayed there, with the wheels reading exactly zero throughout.

| vmax (m/s) | plateau of ∥v∥ (m/s) | ratio |
| --- | --- | --- |
| 0.1​ | 1.295​ | — |
| 1.5​ | 1.588​ | 1.06​ |

Raising the threshold only moves the plateau. The gate was therefore neutralised (vmax=50), leaving the two constraints that do not depend on the filter’s own possibly-wrong state: the image-disparity test (zupt_max_disparity =0.5) and the χ2 test. Measured outcome at rest: ∥v∥ from 1.295 to 3×10−3 m/s, and cumulative path over 45 s from 95.4 m to 0.29 m.

### Defect 2: a calibration constant that expires

The CORE2 accelerometer under-reads. The correction is a scalar applied to the raw specific force,

acorr=s araw,  s=glocal∥araw∥at rest,

which is legitimate because ∥a∥ at rest equals the local gravity magnitude independently of orientation — a norm is rotation invariant. That is the same argument that licenses periodic gyro-bias re-estimation, and it is what makes [eq:accelscaleov] safe to re-run in the field.

The constant was measured once, on 2026-07-28, and frozen. Three weeks later the sensor had moved:

A calibration constant written into a launch file silently expires. Nothing fails; the estimator simply diverges.

|  | 2026-07-28 | 2026-08-20 | drift |
| --- | --- | --- | --- |
| ∥araw∥ (m/s​2) | 8.7525​ | 8.6531​ | −1.14 %​ |
| required s | 1.1186​ | 1.1314​ | +1.14 %​ |
| residual with frozen s (m/s​2) | ≈0​ | −0.111​ | — |

A constant specific-force error ε integrates twice into position,

d(t)=12 ε t2,

so the −0.111 m/s​2 residual predicts d(600 s)=12(0.111)(600)2≈2.0×104 m — twenty kilometres over a ten-minute run, which matches the observed runaway in order of magnitude. Re-measuring gave a residual of +1.7×10−3 m/s​2, a reduction by a factor of 65.

The lesson is structural rather than numerical: the fix is not the new value but the mechanism that keeps it current. A periodic re-estimation (accel_recal_period) now runs the same gated stationarity test used for the gyro, so [eq:accelscaleov] is re-evaluated at confirmed stops.

### Defect 3: the triangulation baseline ratio

ov_core’s feature initialiser rejects a triangulated point when

∥pf∥bmax>β,  β=fi_max_baseline=40 (default),

where bmax is the largest baseline available among the sliding-window clones. Rearranged, [eq:baselinegate] is a hard distance ceiling:

dmax=β bmax.

Rejection is governed by parallax. With a stereo baseline of 90 mm and β=40, every point beyond 3.6 m is discarded while the vehicle is stationary.

Evaluated for this platform, [eq:distceiling] is severe:

Indoor features typically sit at 2–10 m. During a pure pivot — the manoeuvre that should be most informative about camera–IMU geometry — almost the entire scene is rejected.

| condition | bmax (m) | dmax (m) |
| --- | --- | --- |
| at rest / pure pivot (stereo only) | 0.090​ | 3.6​ |
| translating, 11 clones at 0.2 m/s | 0.240​ | 9.6​ |
| translating, 25 clones at 0.2 m/s | 0.420​ | 16.8​ |

This mechanism is real and explains why enlarging the clone window helped (peak features 2→15): it raises bmax, hence dmax. It is not, however, the binding constraint. Raising β from 40 to 200 made the measured outcome worse (n=0.75→0.23, peak 25→5 over 777 updates), and the sister estimator MINS already runs at β=2000 without benefit. Recorded here because the derivation is sound and the ceiling is genuinely present — it is simply not what closes the channel.

### A structural blind spot: vertical unobservability in MINS

MINS survives where openVINS does not, because wheel odometry anchors it. The wheel model is planar (Wheel2DAng); writing the measurement Jacobian in the body frame,

hwheel(x)=vxωz,  ∂h∂z=0,

so the vertical coordinate receives no information from the wheels. Only the camera constrains z. When visual features are scarce — the very condition documented above — the vertical channel is left to free inertial integration and [eq:doubleintov] applies unchecked.

This was observed accidentally and then verified quantitatively. With the robot stationary and the wheels reading zero, the horizontal estimate stayed exact while z diverged:

z(120 s)=−15.2 m,  x=0.01 m,  y=−0.03 m.

Inverting [eq:doubleintov] recovers the driving residual,

εz=2 |z|t2=2×15.21202=2.1×10−3 m/s2,

against an independently measured specific-force residual of 1.7×10−3 m/s​2 (Section 12.29.3). The two agree to within 25 %, which for a doubly-integrated quantity over two minutes is a tight confirmation of the mechanism.

The practical consequence is a diagnostic asymmetry worth stating plainly: a partially anchored estimator lies asymmetrically. Inspecting x and y — the constrained axes — gives full confidence while z runs away without limit and without any alarm. Every subsequent change to camera extrinsics on this platform is now validated on the vertical axis as well, through an explicit guard in the live supervisor.

### Seven hypotheses, seven measurements

The residual failure — nfeat=0.75 against a capacity of 40 — remains open. What is settled is what it is not. Each row below was tested on the robot and eliminated by measurement:

Negative results, recorded deliberately. Each cost an instrumented trial; repeating them would cost the same again. The best configuration measured to date is 25 clones with a χ2 multiplier of 5 on the idealised extrinsic: n=0.75, peak 25, 35 % non-empty updates.

| hypothesis | measured outcome | n​ |
| --- | --- | --- |
| scene lacks texture | MINS finds 50 features in the same images | — |
| global camera–IMU extrinsic | full Kalibr matrix performs worse than the idealised one | 0.58​ |
| hybrid (Kalibr rotation, hand translation) | breaks the jointly-optimised pair; MINS falls 26 m in z | — |
| χ2 gate too tight | ×5 tolerance (1→2.2 px): 33%→35% non-empty | 0.75​ |
| translational parallax | 11→25 clones: peak 2→15, still starved | 0.75​ |
| stereo mis-declaration | TrackKLT derives epipolar geometry by RANSAC from data | — |
| triangulation ratio β | 40→200: outcome degrades; MINS already at 2000 | 0.23​ |

The instrumentation gap that remains is specific: nothing currently counts stereo correspondences between the matching stage and the triangulator. Until that count exists, the two remaining candidate stages in Figure 12.42 cannot be separated, and any further parameter change is tuning in the dark — which this session has demonstrated does not converge.

## Square-root filtering on the LEO rover: theory, a logic defect, and a three-estimator bench

This section continues Sec. 12.20, which recorded the discovery of the defects on the Raspberry Pi build. What follows is the resolution work: the mathematics that motivates the filter, the two source-level corrections, the optical defect that neither correction addressed, and the state of the bench at the time of writing. The two sections are sequential, not contradictory — where they disagree numerically, the figures here supersede, and the divergence is dated.

### Why a square-root form at all

A standard EKF propagates the covariance P∈Rn×n directly. Two properties make this fragile on an embedded platform. First, P must remain symmetric positive semi-definite (PSD), but nothing in the update equations enforces it: round-off alone can produce a negative eigenvalue, after which the filter is no longer a filter. Second, its condition number

κ(P) = λmax(P)λmin(P)

grows as the estimate becomes sharp in some directions and loose in others — precisely what happens in VINS, where scale and yaw are weakly observable while the camera constrains the rest tightly.

The square-root filter (SRF) stores instead an upper-triangular factor U with

U⊤U = P.

Two consequences follow immediately, and they are the whole argument.

##### Positive semi-definiteness becomes structural, not hoped for.

For any x∈Rn,

x⊤P x = x⊤U⊤Ux = Ux22 ≥ 0.

The quadratic form is a squared norm. Whatever numerical damage U suffers, the P it represents cannot be indefinite. The EKF has no equivalent guarantee: it must produce a PSD matrix through arithmetic that does not enforce it.

##### The conditioning is halved, in the exact sense.

Since the singular values of U are the square roots of the eigenvalues of P,

κ(U) = κ(P).

In floating point with t mantissa bits, a linear solve on a matrix of condition number κ loses on the order of log2κ significant bits. Writing teff for the bits that survive,

teffEKF≈t−log2κ(P),  teffSRF≈t−log2κ(U)=t−12log2κ(P).

The SRF loses half as many bits, which is the same as saying it behaves as though its mantissa were twice as long. Concretely, with the λmin=3.28×10−9 measured in Table 12.22 and a λmax of order 10−2, κ(P)∼3×106: a covariance-form filter would surrender log2(3×106)≈21 of the 24 mantissa bits of an IEEE-754 float, leaving three. The square-root form surrenders about eleven and keeps thirteen. This is the quantitative content of the upstream claim that VINS runs in 32-bit single precision, and it is why the initial diagnosis of this project — that the divergence was a precision problem — was a reasonable hypothesis before it was refuted (Sec. 12.30.3).

##### Measured on this robot.

Table 12.22 reports the minimum eigenvalue of the published 6×6 pose covariance, sampled over a 25 s window (2 105 messages), all three estimators fed bit-identical inputs.

Covariance health, measured on the live bench. Symmetry is P−P⊤max; λmin is over the whole window.

| Estimator | symmetry | λmin​ | PSD held |
| --- | --- | --- | --- |
| MINS | 4.9×10−17​ | 1.14×10−4​ | yes |
| openVINS | 8.1×10−20​ | 4.33×10−6​ | yes |
| VINS​ | 3.7×10−20​ | 3.28×10−9​ | yes |

The square-root filter operates three to five orders of magnitude closer to singularity than the other two and still never loses definiteness. That is Eq. [eq:psd] observed rather than asserted.

#### Propagation without ever forming P

The factor is propagated without the covariance ever being assembled. Given the state transition Φk−1 and process noise Wk−1=Wk−1⊤/2Wk−11/2, the covariance recursion Pk|k−1=ΦPk−1|k−1Φ⊤+W is realised by stacking and triangularising:

Wk−11/2Uk−1|k−1Φk−1⊤ =QR Qk−1Uk|k−10.

Because Q is orthonormal, Q⊤Q=I and the product of the stack with itself is preserved exactly: Uk|k−1⊤Uk|k−1=ΦU⊤UΦ⊤+W=Pk|k−1. The covariance is never squared and never inverted, which is where an EKF spends most of its conditioning budget.

This is also the recursion whose noise block was defective in this package — see Sec. 12.30.2.5: the W1/2 of Eq. [eq:qrprop] was assembled rank-deficient, and because QR preserves U⊤U exactly, it preserved the defect exactly too.

### The ZUPT update, and a logic defect that made it unusable

#### What a zero-velocity update does

At a genuine standstill the true body velocity is zero, which is a measurement free of charge. With state error x and measurement Jacobian H, the residual is simply

r = 0−​GvI = − Hx+n,  n∼N(0,R).

Whether this residual is consistent with the current state and its uncertainty is decided by the Mahalanobis distance

χ2 = r⊤ (HPH⊤+R)−1r,

accepted when χ2<γ χ0.952(dimr).

#### The defect

The package gates the rejection as follows (UpdaterZeroVelocity.cpp, as cloned):

// We need to pass the chi2 and not be above our velocity threshold
if (!disparity_passed && (chi2 > options_.chi2_multipler * chi2_check ||
                          state->imu->vel().norm() > zupt_max_velocity_)) {
  ... reject ...
}

The leading !disparity_passed && governs the entire rejection. Once the image-disparity test passes, neither χ2 nor the velocity bound is ever evaluated. The comment one line above states the intended semantics — “We need to pass the chi2” — and the code does the opposite.

Measured on this rover with the gate fully enabled (zupt_chi2_multipler: 1, threshold correctly computed at 65.171), robot stationary: 290 updates accepted, 0 rejected, with χ2=1.34×1057 and v=1.0×1023 m/s. The thresholds were correct; they were simply never consulted. Position diverged to 2.06×1015 m.

#### The correction, and why the two guards are not symmetric

The two tests answer different questions, and only one of them may legitimately be overridden.

- The velocity bound reads the filter’s own estimate ​GvI. It is therefore circular: once the filter diverges, that estimate is wrong, and the bound would block the only mechanism able to recover it. Breaking that circularity with an independent witness — image disparity, which does not depend on the state — is exactly what the disparity test is for. This override is legitimate and was kept.

- The χ2 test of Eq. [eq:zuptchi2] asks whether the measurement is consistent with the state and its covariance. A value of 1057 says the update is nonsense; applying it injects that nonsense into the state. No external witness can make an inconsistent update acceptable. This override must not exist.

const bool chi2_over = (options_.chi2_multipler > 0) &&
                       (chi2 > options_.chi2_multipler * chi2_check);
const bool vel_over  = state->imu->vel().norm() > zupt_max_velocity_;

if (chi2_over || (!disparity_passed && vel_over)) { ... reject ... }

The documented chi2_multipler: 0 (“disparity only”) semantics are preserved: with 0 the threshold is not applied. A non-finite χ2 is rejected in every mode, since nan does not mean “excellent measurement”, it means “χ2 not computable”.

#### Why the disparity override collapses at kinematic extremes

The corrected condition is still a disjunction in which one term remains gated. Writing C for “χ2 exceeds threshold”, V for “v exceeds zupt_max_velocity” and D for “disparity passed”, the reject predicate is

R = C ∨ (¬D∧V),

against the original R0=¬D∧(C∨V). The correction moved C out of the gate, which was necessary and sufficient for the χ2 pathology. It deliberately left V inside, on the argument that V is circular: it reads ​GvI, the filter’s own estimate, so a diverged filter reports a false V and would veto the only mechanism able to recover it. The disparity test breaks that circularity with a witness computed from images alone, independent of the state.

That argument is sound on a bounded domain and fails outside it. Let vphys be the platform’s physical maximum. For v≲vphys, a true V is genuinely ambiguous — the filter may be right or wrong — and deferring to an independent witness is the correct arbitration. For v⋙vphys there is no ambiguity left to arbitrate: the proposition “this rover travels at 922 m s−1” has prior probability zero, and no image-domain evidence can raise it. Deferring to D there is not arbitration, it is abdication.

The measurement that forced the point: filter at v=922.79 m s−1, zupt_max_velocity set to 1.5, and zero rejections logged. D held — the scene looked static — so V was never consulted. That is 3500× the rover’s measured maximum of 0.264 m s−1 (Table 12.23).

The remedy is a term outside the lattice: an unconditional predicate evaluated before Eq. [eq:reject],

A = ¬ finitev ∨ v>vmaxabs,  vmaxabs=3.0 m s−1,

with R′=A∨R. The finiteness clause is not decoration: nan fails every ordinary comparison, so nan>x evaluates false and a nan velocity would slip through a bare inequality — nan does not mean “small”, it means “not computable”.

Choosing vmaxabs. The rover’s true maximum is measured by MINS, which is wheel-anchored and therefore the only estimator entitled to state it.

| quantity | value | source |
| --- | --- | --- |
| rover maximum, measured | 0.264 m s−1​ | MINS, wheel-anchored |
| zupt_max_velocity | 1.5 m s−1​ | tunable, YAML |
| vmaxabs​ | 3.0 m s−1​ | compiled in, 11× margin |
| observed at failure | 922.79 m s−1​ | 3500× the maximum |

vmaxabs is a compile-time constant, not a YAML field, and that is a deliberate design decision rather than an oversight: a safety bound that a configuration file can disarm is not a safety bound. Raising it for a faster platform should cost a code change and a rebuild, which leaves a trace in version control.

##### What this bound does not do.

It does not make a diverged filter converge. ZUPT is precisely the mechanism that pulls v back toward zero; refusing it cannot help recovery, and in isolation makes it marginally worse. Its value is different and real: it prevents an update computed from a corrupted covariance from injecting further error into the state, and — the reason that actually justifies the code — it converts a silent failure into a logged one. The diagnosis reported here cost hours precisely because the filter diverged while accepting every update without a word.

Effect of the guard correction. Same build, same configuration, same stationary robot; only the boolean changed.

|  | before | after |
| --- | --- | --- |
| ​GvI​ | 1.0×1023 m/s | 0.030 m/s |
| χ2 (threshold 65.171) | 1.34×1057​ | 1.1 – 1.5 |
| final position | 2.06×1015 m | 0.019 m |

#### A second, independent defect: a rank-deficient noise block

StateHelper::propagate_zero_motion() augments the square-root covariance with the bias random-walk noise. It allocates six noise rows and then writes both 3×3 blocks to the same rows 0–2, leaving rows 3–5 zero. Writing W1/2 for that block and using the fact that W=W1/2⊤W1/2 is invariant under the QR that follows, the implied noise covariance becomes

Wbug=σwb2Δt I3σwbσabΔt I3σwbσabΔt I3σab2Δt I3,  rankWbug=3,

instead of the correct block-diagonal, full-rank W=Δt diag(σwb2I3,σab2I3).

Two things make this defect hard to see. The diagonal — the per-axis variances — is unchanged, so nothing looks wrong on inspection. And the spurious off-diagonal asserts a correlation ρ=σwbσab/(σwbσab) of exactly one between the gyroscope and accelerometer bias random walks, which are physically independent. Numerically, with the deployed σ values, each update injects ρ≈0.044.

The consequence matches the observation exactly. At rest, ZUPT constrains angular rate directly, so bg is well observed and corrected; the spurious coupling pours every such correction into ba, which is nearly unobservable at rest and which nothing pulls back. Measured before correction: ba=(−1.47×105, 1.11×104, 3.69×104) m/s​2 — seven orders of magnitude outside anything physical — while bg=(−0.0023, 0.0009, −0.0080) rad/s remained perfectly sane and the attitude quaternion stayed stable.

The same augmentation is performed correctly some one hundred lines away in UpdaterZeroVelocity.cpp, where the analogous matrix is built block-diagonal and full-rank. Two implementations of one computation had diverged. The fix aligns the second on the first.

##### Honesty note.

This correction was applied first and did not, on its own, stop the divergence. Whether it is necessary in addition to the guard correction has not been isolated; only that it is independently wrong and that the correct form already existed in the same package. Both are in place in every measurement reported here.

### Retracting the single-precision hypothesis

The initial hypothesis attributed the divergence to the upstream default USE_FLOAT=ON, which types every vector and matrix — including the square-root factor itself — as float. The package was rebuilt in double precision (-DUSE_FLOAT=OFF, no source modification, the CMake option() accepting a command-line value) and the defect reproduced identically. The hypothesis is therefore refuted: the failure was one of logic, not of arithmetic. Double precision did contribute one thing — it turned unreadable numerical garbage (χ2=inf) into a legible state that named the corrupted vector.

### The optical trap: motion blur, and why neither correction touched it

Both source-level corrections were real, and neither addressed the failure that actually ends a drive. On the dynamic run of 2026-08-21 the two pure visual-inertial filters lost their entire visual channel within two seconds of the wheels turning, and only then fabricated velocity.

Feature collapse at motion onset. Wheels start at t=219.0 s. feat is the width of the points_msckf cloud, i.e. features actually consumed by the update — not features detected.

| t (s) | wheels | MINS v | oV v | ​ v | feat oV | feat ​ |
| --- | --- | --- | --- | --- | --- | --- |
| 218.5 | 0.000 | 0.026 | 0.002 | 0.197 | — | 0.0 |
| 219.0 | 1.359 | 0.035 | 0.019 | 0.197 | 11.0 | 4.9 |
| 219.5 | 3.219 | 0.035 | 0.078 | 0.311 | 4.8 | 6.8 |
| 220.0 | 3.362 | 0.047 | 0.258 | 0.730 | 1.0 | 2.1 |
| 220.5 | 3.147 | 0.028 | 0.623 | 1.316 | 0.1 | 0.4 |
| 221.0 | 3.147 | 0.025 | 1.139 | 2.041 | 0.0 | 0.0 |
| 224.0 | 3.147 | 0.047 | 6.309 | 8.529 | 0.2 | 1.6 |

The ordering in Table 12.25 settles causality: features fall from 11.0 to 0.0 over 219.0–221.0 s, and the velocities rise afterwards, in step. Deprived of measurements, both filters do what any VIO does — integrate the IMU — and the accelerometer bias error becomes a phantom velocity by double integration. MINS holds 0.02–0.07 m s−1 throughout because its wheels anchor it. This is not a filter defect. It is a perception failure.

#### The three suspects, and how two were eliminated

##### Network — eliminated by measurement.

tools/audit_flux_images.py, 45 s, sensor timestamps (never arrival times, which would fold WiFi jitter into the sensor’s):

infra1   675 images / 45.0 s -> 15.01 Hz (nominal 15)
         dt median 66.7 ms | jitter 0.0 ms (0.0 %) | min 66.7 max 66.7
         GAPS: 0 -> 0 frames lost | reversals 0 | duplicates 0
infra2   identical
STEREO   cam0<->cam1 offset: median 0.00 ms | max 0.00 ms

Zero jitter, zero loss, zero reordering, exact stereo synchrony. The image path is unrelated to the 40 % IMU jitter measured separately: they are different transports.

##### KLT parameters — eliminated by arithmetic.

The tracker’s reach is set by win_size=15 and pyr_levels=5 (compiled in, not configurable), giving a search radius of (win_size/2)⋅2pyr_levels=240 px. The inter-frame image displacement induced by body rotation is

Δu = ω ffcam,

with f=336.37 px the focal length and fcam=15 Hz the frame rate. At the measured peak ω=0.631 rad s−1 this is 14.1 px — a factor 17 inside the tracker’s reach. Likewise track_frequency=31 Hz exceeds the 15.01 Hz publication rate, so every frame is processed and the setting is not a limiter.

#### Motion blur: the surviving suspect, modelled

A point feature imaged during an exposure of duration texp while the body rotates at ω is smeared across

$$\begin{equation}
      b \;=\; \omega \, t_{\mathrm{exp}} \, f
      \label{eq:blur}
    \end{equation}$$

pixels. Equation [eq:blur] is the small-angle limit of the exact projection and is accurate to better than 1 % for ωtexp≪1, amply satisfied here.

The measured configuration made texp large for a reason that is documented elsewhere in this report: the IR projector is switched off (laser_power=0) because its dot pattern polluted the calibration checkerboard and the beacon LEDs. Without the projector the infrared scene is dim, and auto-exposure compensates the only way it can — by opening the shutter. The camera reported texp=33 000 μs: the shutter stands open for 33 ms of a 66.7 ms frame period, half the time.

Motion blur from Eq. [eq:blur], f=336.37 px, ω measured by MINS. Compare with min_px_dist=20 px and with the gradient support of a FAST corner at fast_threshold=30.

| ω​ | texp=33 ms | texp=8 ms | reduction |
| --- | --- | --- | --- |
| 0.154 rad s−1 (median) | 1.7 px | 0.41 px | 4.1×​ |
| 0.631 rad s−1 (peak) | 7.0 px | 1.70 px | 4.1×​ |

A 7 px smear does not merge neighbouring features — they are kept 20 px apart — but it flattens the intensity gradient that a FAST corner detector is built to find, and fast_threshold=30 demands a sharp one. The corner does not move; it stops being a corner.

#### Capping the exposure, and what it cost

This ROS wrapper exposes no auto-exposure limit: the available controls are enable_auto_exposure, exposure and gain. Capping is therefore impossible; the exposure must be fixed manually, which also fixes the gain. Applied: texp=8 000 μs with gain raised 16→64 to compensate the 4.1× loss of light.

Effect of the exposure cap, both filters freshly restarted, robot stationary. The improvement is measured at rest, where there is no motion blur to remove — the in-motion case remains untested.

|  | texp=33 ms | texp=8 ms |
| --- | --- | --- |
| VINS features/update | 12.00​ | 72.00​ |
| non-empty updates | 100 %​ | 100 %​ |
| frame rate | 15.01 Hz, 0 loss | 15.01 Hz, 0 loss |
| mean luminance | — | 64.9/255, 0 % black, 0 % clipped |

Six times more features, and Table 12.27 understates the intended benefit because it was taken at rest, where Eq. [eq:blur] gives b=0 and a shorter exposure can only cost light. The gain most plausibly comes from chassis micro-vibration, which a 33 ms shutter integrates and an 8 ms one does not.

##### The operational cost, stated plainly.

Exposure is now fixed. A rover crossing from a lit area into a dark one has nothing left to compensate: the image will clip or black out. In a laboratory at constant illumination this is immaterial; outdoors, or between rooms, it must be re-validated. This is a real regression in robustness accepted in exchange for a real gain in feature yield, not a free improvement.

### Bench architecture

Three estimators run concurrently on identical inputs. This is the condition that makes any comparison meaningful: at different inputs, a difference in output would measure the plumbing rather than the mathematics.

ROBOT -- Raspberry Pi 4B, ROS 1 Noetic         PC -- x86-64, ROS 1 Noetic
 =====================================         ==================================

 D455 stereo IR --+                             +--> /pc/camera/infra1/image_rect_raw
   laser_power=0  |  /camera/infra{1,2}  WiFi   |    /pc/camera/infra2/image_rect_raw
   t_exp  = 8 ms  +-----------------------------+         |      |      |
   gain   = 64    |  15.01 Hz, 0 loss, 0 jitter |         |      |      |
 -----------------+                             +-->      |      |      |
                                                          |      |      |
 CORE2 IMU ---> /firmware/imu                             |      |      |
                     |                                    |      |      |
               imu_sanitizer.py                           |      |      |
               | accel_scale 1.1186                       |      |      |
               | gyro bias subtracted                     |      |      |
               | glitch + freeze guards                   |      |      |
               +---> /imu/data_clean --------------------->      |      |
                                                          |      |      |
 Wheel encoders --> /firmware/wheel_states                 |      |      |
                     |                                    |      |      |
               wheel_remap.py                             |      |      |
               +---> /joint_states_mins ------------->    |      |      |
                                       |                  |      |      |
                    +------------------+--+  +------------+--+  ++------+------+
                    |      M I N S        |  |   openVINS    |  |   sqrtVINS   |
                    | wheel + visual +    |  |   MSCKF / EKF |  |   SR-MSCKF   |
                    | inertial            |  |   keeps P     |  |   U'U = P    |
                    |                     |  |               |  |              |
                    | UpdaterWheel   YES  |  | wheels:  NO   |  | wheels:  NO  |
                    | Z UNOBSERVABLE      |  |               |  | lmin 3.3e-9  |
                    +----------+----------+  +-------+-------+  +------+-------+
                               |                     |                 |
                               |          ZUPT GUARDS (pure VIO only)  |
                               |          +----------+-----------------+------+
                               |          | A  |v| > 3.0 m/s  => HARD REJECT  |
                               |          |    compiled in, not overridable   |
                               |          | B  chi2 > threshold => reject     |
                               |          | C  |v| > 1.5 m/s => reject ONLY   |
                               |          |    if disparity fails  (YAML)     |
                               |          +----------+-----------------+------+
                               |                     |                 |
                /mins/imu/odom |   /ov_msckf/odomimu |  /sqrtvins/odomimu
                     /tf_mins  |          /tf_vins   |     /tf_sqrtvins
                               +---------+-----------+-----------------+
                                         |
                    +--------------------+----------------------+
                    |             pose_selector.py              |
                    |  SetPoseSource -- 3 states, typed service |
                    |  -- GUARD ------------------------------  |
                    |  first sample > MAX_INITIAL_JUMP_M (5 m)  |
                    |     => source NOT selectable, logged      |
                    |  SE(3) correction at switch => no jump    |
                    +------+----------------------------+-------+
                           |                            |
                  /robot_pose_fused             export_matlab.py
                           |                            |
                    leo_backend.py                  .mat + CSV
                           |                      | t  SENSOR timestamp
                    rosbridge :9090               | p, q, v, omega
                           |                      | P_pose, P_twist  6x6
                           v                      |   FLATTENED to 36 columns
                    Web cockpit                   |   ROS row-major  =>
                    | ops.html      (3 modes)     |   MATLAB is column-major:
                    | trajectory    (3 traces)    |   P = reshape(...,6,6)'
                    | tf_validator  (3 views)     |        ^ transpose MANDATORY
                                                  | sigma3   3-sigma bounds
                                                  | feat_n   features consumed
                                                  | wheels   independent witness

Three points deserve emphasis.

##### Only MINS is wheel-anchored.

It alone consumes /joint_states_mins. The two visual-inertial filters receive the stereo pair and the IMU and nothing else. Any comparison of drift must state this: it is not a like-for-like contest but a measurement of what wheel odometry buys.

##### Separate TF trees.

Each estimator publishes to its own (/tf_mins, /tf_vins, /tf_sqrtvins). Sharing /tf would have three filters writing competing transforms for the same frame pair, and the tree would be silently incoherent.

##### A guard between the estimators and the control path.

pose_selector refuses to make a source selectable whose first sample lies farther than MAX_INITIAL_JUMP_M =5 m from its origin. This is not decoration: /robot_pose_fused feeds AUTO mode and beacon approach. During this audit the guard rejected VINS at 65.95 m while the robot was stationary, and logged the reason explicitly — see Sec. 12.30.6.

### State of the three estimators at the time of writing

Robot stationary, wheel-confirmed, identical inputs.

| Estimator | distance from origin | wheel-anchored | selectable as pose source |
| --- | --- | --- | --- |
| MINS | 0.202 m | yes | yes |
| openVINS | 0.075 m | no | yes |
| VINS​ | 65.95 m | no | no — refused by the guard |

VINS has improved by seventeen orders of magnitude and is now a filter that runs, publishes well-formed covariances, and maintains positive-definiteness under Eq. [eq:psd]. It is not yet an estimator whose output may be trusted for navigation on this platform, and the bench says so by refusing it rather than by degrading silently.

### Instrumenting the update: where features are actually lost

Every figure in the two subsections that follow rests on a measurement that the package could not produce as shipped. UpdaterMSCKF.cpp contains no diagnostic output — no PRINT_ALL, no PRINT_DEBUG — and its single feature counter is commented out. Running the node at verbosity: ALL therefore reveals nothing about this stage, which is why an earlier attempt to instrument it returned an empty capture.

Five counters were added, one at each erase of the pipeline, in the order they execute: features removed for having fewer than two usable measurements; triangulation failure; refinement failure; Jacobian failure; reprojection failure; and χ2 rejection. A single line is emitted per update:

[MSCKF-ETAGES]: entree N | <2mesures N | avant_tri N | tri N | refine N
                | jacob N | reproj N | chi2 N | SORTIE N

##### A metric that had to be discarded.

This instrumentation also invalidated the measure used earlier in the investigation. The /points_msckf point cloud was read as “features consumed per update”, but it is populated from good_features_MSCKF, which is cleared only inside do_feature_propagate_update. When a zero-velocity update pre-empts the camera path — which it does on nearly every frame at rest — that function is never reached, the vector retains its previous contents, and the visualiser republishes them indefinitely. A capture showing 1 352 messages averaging 78 features was recorded while UpdaterMSCKF::update() was called zero times. Any conclusion drawn from /points_msckf alone is therefore unsafe, and the per-stage counters are used instead throughout what follows.

### The geometric ceiling: base-to-depth ratio and the limits of pure visual-inertial odometry on a slow platform

The preceding sections localised, corrected and measured a series of software defects. This one addresses a different class of limit — one that no parameter can lift, because it is set by the vehicle’s geometry and speed rather than by its code. It matters for the conclusions of this report: it explains why the wheel-aided estimator survives drives that both pure visual-inertial filters fail, and it bounds what any future tuning effort can be expected to achieve.

#### The measured premises

Three quantities, all measured on this platform rather than assumed:

Geometric premises of triangulation on the LEO rover. The speed is measured by MINS, the only estimator anchored on wheel encoders and therefore the only one entitled to state it.

| quantity | symbol | value |
| --- | --- | --- |
| maximum body speed | vmax​ | 0.264 m s−1​ |
| camera frame rate | fcam​ | 15.0 Hz​ |
| inter-frame displacement | Δ=vmax/fcam​ | 1.76 cm​ |
| stereo baseline | b​ | 9.0 cm​ |
| focal length | f​ | 336.37 px​ |
| ratio b/Δ |  | 5.1​ |

The last line is the one that governs everything else. The stereo baseline is five times larger than the displacement accumulated between two consecutive frames at full speed. Triangulation on this rover therefore rests almost entirely on the static stereo pair; the motion contributes a parallax increment of the order of one fifth of it, and only when the vehicle is actually moving at its maximum.

This inverts the usual situation in visual-inertial odometry, where a moving camera accumulates a virtual baseline far exceeding any physical stereo rig and where depth is progressively refined by translation. Here the geometry is essentially frozen at 9 cm.

Figure 12.45 draws the three quantities of Table 12.29 to scale against each other, which is the point: the argument of this subsection is geometric, and the geometry is more convincing seen than described.

Triangulation geometry on the LEO rover. The two horizontal quantities (b=9.0 cm and Δ=1.76 cm) are drawn at the same scale as each other, which is the point of the figure; the depth axis is compressed and is not on that scale, since 5 m cannot share a page with 9 cm. The three σZ bars are to scale with one another (Eq. [eq:sigmaz]): invisible at 1 m, and 0.83 m at the 5 m distances that dominate an indoor scene.

#### Why depth collapses at room scale

For a rectified stereo pair, depth follows from disparity d as Z=bf/d. Differentiating and substituting back gives the standard first-order propagation of disparity error into depth error:

σZ = Z2b f σd.

The quadratic dependence on Z is the whole difficulty: uncertainty does not degrade gracefully with distance, it degrades as the square. With σd=1 px — optimistic, given the ±0.19 px calibration residual and the residual motion blur quantified in Sec. 12.30.4 — Eq. [eq:sigmaz] gives Table 12.30.

Depth uncertainty from Eq. [eq:sigmaz], σd=1 px. The third column uses the physical stereo baseline b=9 cm; the fourth shows what would remain if the filter had to rely on inter-frame motion alone (Δ=1.76 cm), which is the regime it approaches whenever the rover slows or stops.

| Z (m) | b/Z​ | σZ stereo (m) | σZ motion-only (m) | rel. error (stereo) |
| --- | --- | --- | --- | --- |
| 1​ | 1:11​ | 0.033​ | 0.169​ | 3 %​ |
| 3​ | 1:33​ | 0.297​ | 1.520​ | 10 %​ |
| 5​ | 1:56​ | 0.826​ | 4.223​ | 17 %​ |
| 10​ | 1:111​ | 3.303​ | 16.892​ | 33 %​ |

At the distances that dominate an indoor scene — walls, furniture and doorways at three to five metres — the base-to-depth ratio falls between 1:33 and 1:56, and the depth of a feature is known only to 30 to 83 cm. A feature triangulated with that uncertainty, then reprojected into the other poses of a 25-clone sliding window, can plausibly land behind the image plane of one of them.

This is precisely the failure the instrumentation of Sec. 12.30.7 counts. The rejection is issued by UpdaterHelper.cpp on the condition Zcam<0.1 m, reported as “Negative depth detected”, and it accounted for 35.8 % of all features entering the update before any correction. It is not a threshold that is set too tight: a feature genuinely cannot lie behind the camera that observes it. It is the downstream symptom of a depth estimate whose uncertainty is comparable to the depth itself.

#### What wheel odometry contributes, and why it is decisive here

MINS consumes /joint_states_mins in addition to the stereo pair and the IMU; the two pure visual-inertial filters do not. Under the conditions of Table 12.30 this is not a marginal advantage but the difference between an observable system and a poorly observable one, for three distinct reasons.

##### Metric scale becomes directly observed.

In a visual-inertial system, scale is recovered by integrating accelerometer measurements over the interval during which a feature is tracked, and is therefore only as good as the bias estimate over that interval. Wheel encoders measure displacement directly, in metres, with an error that grows linearly with distance rather than quadratically with time. The estimator no longer needs to extract scale from a geometry that, per Table 12.30, cannot supply it reliably.

##### Velocity is constrained without vision.

When features collapse — whether from a texture-poor scene, motion blur, or the triangulation regime described above — a pure VIO filter has no measurement left and falls back to open-loop inertial integration, in which an accelerometer bias error ε produces a position error 12εt2. The divergences recorded in Sec. 12.30.6 follow this signature. A wheel measurement bounds velocity at every step and removes the double integration entirely.

##### The planar constraint is exploited rather than ignored.

MINS uses a Wheel2DAng model, which asserts planar motion. This is simultaneously its strength on x and y and its documented weakness on z: the Jacobian ∂h/∂z is identically zero, so the vertical axis receives no wheel information whatsoever and is left to the camera alone. The measurements of this campaign show both faces of that trade-off — MINS holding x and y to within centimetres over drives where both VIO filters diverged, while its own z drifted to several metres and, in one recorded instance, to 1707 m.

##### Consequence for any comparison in this report.

A drive in which MINS reports 3.83 m and VINS reports 431 m over the same sixteen seconds is not a measurement of square-root versus covariance arithmetic. It is a measurement of what wheel odometry is worth when the visual channel cannot carry the estimate on its own. Every three-way figure in this report must be read with that asymmetry stated.

#### The refinement trade-off: fewer bad depths, or fewer depths

fi_refine_features controls whether the linear triangulation is followed by a Levenberg–Marquardt refinement of the feature position. The deployed VINS configuration had it disabled — alone among the three estimators, openVINS and MINS both leaving it at the upstream default of true. Since low parallax is exactly the regime in which a linear solution is least trustworthy, disabling the non-linear refinement removes the correction where it is most needed.

Enabling it produced the shift in Table 12.31, measured with identical instrumentation and the same geometry.

Per-stage feature accounting, VINS, robot driven. Configuration A uses the hand-placed extrinsics; B and C the measured Kalibr calibration; C additionally enables Levenberg–Marquardt refinement. Values are percentages of features entering UpdaterMSCKF::update().

| rejection stage | A (hand) | B (Kalibr) | C (Kalibr + refine) |
| --- | --- | --- | --- |
| KLT, <2 measurements | 10.4​ | 10.9​ | 12.1​ |
| triangulation | 20.6​ | 23.5​ | 25.7​ |
| refinement | 0 (inactive) | 0 (inactive) | 3.4​ |
| Jacobian (negative depth) | 35.8​ | 34.3​ | 29.4​ |
| χ2 gate | 22.3​ | 22.8​ | 20.2​ |
| surviving | 10.9​ | 8.5​ | 9.2​ |

Two readings follow, and they must be kept separate.

##### The targeted stage improves, and by the expected mechanism.

Jacobian rejection falls from 35.8 % to 29.4 %, an 18 % relative reduction, and the underlying “Negative depth detected” count drops from 17 242 to 3 789 over comparable windows. The refinement is doing what it is for: a feature whose linear depth estimate was inconsistent with its observations is moved to a position that reprojects in front of every camera that saw it.

##### The aggregate does not improve, because the losses move upstream.

Triangulation rejection rises from 20.6 % to 25.7 %, and total yield is unchanged within the noise of these measurements (10.9 % against 9.2 %). This is the honest interpretation: the refinement does not manufacture well-conditioned features, it fails the ill-conditioned ones earlier and more explicitly. A feature that previously survived triangulation only to be discarded later for negative depth is now rejected at the stage that should have rejected it.

That is a real improvement in the diagnostic quality of the pipeline and a negligible one in its output. It is worth keeping — it aligns VINS with the other two estimators and with upstream, and it moves rejections to where they are interpretable — but it should not be presented as a fix for the yield.

##### The ceiling stands.

Across the three configurations, between 8.5 % and 10.9 % of features entering the update survive it, from an image in which 165 FAST corners are demonstrably detectable during motion (Sec. 12.30.4). Neither the camera–IMU extrinsics (Sec. 12.30.5) nor the refinement setting moved that figure materially. The limit identified in Table 12.30 — a 9 cm baseline observing a scene at three to five metres, with a vehicle too slow to accumulate meaningful additional parallax — is the operative constraint, and it is a property of the platform rather than of the estimator.

##### What would lift it.

Three avenues follow directly from Eq. [eq:sigmaz], in decreasing order of practicality on this rover: a longer stereo baseline, which enters linearly; operation at shorter range, which enters quadratically and is by far the most effective; or wheel-aided fusion, which sidesteps the triangulation problem rather than solving it and is the approach MINS already demonstrates. Faster driving would help only marginally: even a fourfold increase in vmax would leave the inter-frame displacement below the stereo baseline.

### What is settled, and what is not

##### Settled, with measurements.

Four results are reproducible and carry their numbers: the χ2 override defect and its correction (Table 12.24); the rank-deficient bias-noise block, Eq. [eq:wbug], whose correct form already existed elsewhere in the same package; the refutation of the single-precision hypothesis (Sec. 12.30.3); and the exposure cap, Table 12.27. Positive-definiteness under Eq. [eq:psd] is observed rather than assumed, at λmin=3.28×10−9.

##### Not settled.

Neither pure visual-inertial filter survives a drive on this platform. The absolute bound of Eq. [eq:absolu] intercepts the runaway and logs it; it does not prevent it, and the report should not be read as claiming otherwise. The exposure cap is validated at rest only — 12→72 features — and the in-motion measurement that would justify it has not been taken.

##### A structural caveat that governs every comparison here.

MINS consumes /joint_states_mins; the two visual-inertial filters consume the stereo pair and the IMU and nothing else. MINS surviving a drive that the other two fail is therefore not evidence about square-root versus covariance arithmetic. It is a measurement of what wheel odometry is worth when the visual channel collapses — roughly, the difference between 3.83 m and 431 m over the same sixteen seconds. Any conclusion drawn across the three estimators must carry this asymmetry explicitly.

##### Two hardware conditions invalidated measurements during this campaign,

and are recorded so that the same trap is not fallen into twice. The ground PC reached 86 ∘C against a manufacturer threshold of 82 ∘C, with the browser consuming more CPU than MINS itself; and the battery fell to 10.61 V, below the 30 % under which yaw authority is known to degrade on this chassis, so a commanded rotation is no longer the rotation performed. tools/relance_estimateurs.sh now refuses to start a run when either condition holds. A measurement taken under them proves nothing, and the discipline of refusing it is cheaper than the hours spent interpreting one.

## Instrumenting the measurement campaign: observability of the platform itself

Every result in Sections 12.29 and 12.30 rests on a premise that was never itself measured: that during the run which produced the numbers, the platform was delivering data at the rate the estimators assume. This section closes that gap. It derives the thresholds of a health monitor from the statistics of the streams rather than from round numbers, proves that the obvious quantity to monitor — message rate — is the wrong one to rely on alone, and states the identity that makes an on-board estimator comparable with a ground-station one.

The motivation is a failure mode already recorded twice on this platform (Sections 12.29, 12.30) and never announced by any component: when /firmware/wheel_states slows, MINS loses the wheel anchoring that gives it its accuracy, and continues to publish poses. The estimator does not fail. It degrades silently into a visual–inertial estimator with no odometric constraint, which is precisely the configuration shown elsewhere in this report to diverge. Nothing in the pipeline distinguishes the two regimes.

The blind spot that motivates external instrumentation. Each arrow is a mechanism verified on this platform; the terminal state is indistinguishable from nominal operation for any node downstream of MINS. A monitor placed outside the chain is the only way to detect it.

### Rate estimation and the arithmetic of a badly chosen threshold

#### The estimator

Let a topic deliver messages at arrival instants t1<t2<⋯, and let W be the width of a sliding window. Write N for the number of arrivals inside the current window. The rate estimate used by the monitor is

f = N−1tN−t1.

The numerator is N−1 and not N because the span tN−t1 covers exactly N−1 inter-arrival intervals; using N inflates the estimate by a factor N/(N−1), which for a short window is not negligible — at N=6 it is a 20 % bias, in the direction that hides a slowdown.

#### Its dispersion

Let the inter-arrival intervals Δk=tk+1−tk be independent with mean T=1/f and standard deviation σΔ, and define the coefficient of variation c=σΔ/T, a dimensionless measure of publication jitter. The span is a sum of N−1 such intervals:

S=tN−t1=k=1N−1Δk,  E[S]=N−1f,  Var(S)=(N−1) σΔ2.

Since f=(N−1)/S, the delta method applied to g(S)=(N−1)/S with g′(S)=−(N−1)/S2 gives

Var(f) ≈ N−1E[S]2​2Var(S) = f4(N−1)2 (N−1) σΔ2 = f4σΔ2N−1,

so that the relative dispersion takes the compact form

σff = cN−1 = cfW−1

The estimator becomes sharper both with a longer window and with a faster topic, and the window may therefore be chosen per-topic to hold the dispersion constant. Table 12.32 evaluates [eq:camp:disp] at W=5 s and c=0.10, a jitter figure consistent with what was observed on this platform.

Dispersion of the rate estimator [eq:camp:fhat] at W=5 s, c=0.10, and the resulting separation of the 70 % alarm threshold expressed in standard deviations.

| topic | f (Hz) | N−1​ | σf/f​ | threshold distance |
| --- | --- | --- | --- | --- |
| /firmware/wheel_states | 18.4 | 91 | 1.05 %​ | 28.6 σ​ |
| /imu/data_clean | 83.9 | 419 | 0.49 %​ | 61.4 σ​ |
| /pc/camera/infra1/… | 15.0 | 74 | 1.16 %​ | 25.8 σ​ |

#### Why 20 Hz was the wrong threshold, quantitatively

The nominal rate of the wheel topic is universally cited as 20 Hz, and the initial specification for this monitor was to warn below that figure. The measured rate, over 30 s with the full stack running, is f=18.38 Hz.

Treating f as approximately Gaussian — justified by N−1=91 — with σf=0.0105×18.38=0.193 Hz, the probability that a healthy stream triggers a threshold placed at fth=20 Hz is

P​f<fth=Φ​20−18.380.193=Φ(8.39)=1−3×10−17.

The alarm is not merely frequent: it is certain, at every evaluation, forever. A monitor in that state is worse than no monitor, because operators learn to ignore it and it is then silent on the day it matters. Anchoring the threshold to a measured nominal f0 instead, at a fraction α=0.70, inverts the situation: from [eq:camp:disp] the separation is

f0−αf0σf=(1−α)N−1c=0.30×910.10=28.6 σ,

a false-alarm probability below 10−179. The design rule that follows is worth stating plainly, because it is the one that was nearly violated: a threshold must be derived from the distribution of the quantity it gates, never from the round number in the specification.

### Latency is the leading indicator; rate is the lagging one

#### The queueing argument

Model a consuming node as a single server of service rate μ fed by arrivals of rate λ, with a bounded input queue of K slots — in ROS, exactly the subscriber’s queue_size. Write ρ=λ/μ for the utilisation. Two quantities behave in completely different ways as ρ→1.

The mean sojourn time of an unbounded such queue is the classical

E[W] = 1μ−λ = 1/μ1−ρ,

which diverges as ρ→1−; with a bounded queue it saturates at K/μ, the time to drain a full buffer. The blocking probability of the bounded queue, on the other hand, is

PK = (1−ρ) ρK1−ρK+1,  limρ→1PK=1K+1,

and message loss — hence the drop in observed rate — is exactly PK. The limit in [eq:camp:pk] is the crux. It gives a bound that holds over the entire stable regime, and it is worth isolating.

ρ≤1  ⇒  rate loss = PK ≤ 1K+1 = 151 = 1.96 %

With K=50 and a threshold at 70 % of nominal, a 30 % loss is required to raise an alarm. By [eq:camp:borne] no utilisation ρ≤1 can produce more than 1.96 %. The rate test is therefore structurally blind to every degree of saturation short of genuine overload. Once ρ>1 the server saturates at throughput μ, the observed rate falls to μ=λ/ρ, and the loss is 1−1/ρ; requiring that to exceed 30 % gives

1−1ρ≥0.30 ⇔ ρ≥1.43.

The producer must be running 43 % beyond capacity before the rate test reacts at all.

Latency reaches its threshold far earlier. For the wheel topic 1/μ=1/18.4=54.3 ms, and the monitor’s staleness limit is Wmax=0.5 s; inverting [eq:camp:wait],

1/μ1−ρ≥Wmax ⇔ ρ ≥ 1−1μWmax = 1−0.05430.5 = 0.891.

Utilisation at which each test reacts, for /firmware/wheel_states (f=18.4 Hz, K=50). The ordering is structural, not a matter of tuning.

| test | quantity | triggers at |
| --- | --- | --- |
| staleness >0.5 s | E[W], Eq. [eq:camp:wait] | ρ=0.891​ |
| rate <70 % of nominal | PK then 1−1/ρ, Eq. [eq:camp:pk] | ρ=1.43​ |

Why staleness must be monitored alongside rate. Both statistics are normalised by their own alarm threshold, so the horizontal dashed line at 1 is where each fires. Staleness crosses it at ρ⋆=0.891; the rate statistic remains below 0.065 across the whole stable regime and cannot cross it at all — the queue absorbs the pressure as delay long before it sheds it as loss.

#### The limit of a fixed staleness threshold

The derivation above makes ρ⋆ depend on the topic through 1/μ: a single absolute limit Wmax corresponds to ρ⋆=0.891 on the 18.4 Hz wheel stream but to ρ⋆=1−(0.0119/0.5)=0.976 on the 83.9 Hz IMU stream, which is a considerably later warning. The fixed threshold is retained nonetheless, for a reason outside the queueing model: the measured staleness includes transport — the wireless hop and the republication described in Section 12.31.4 — and half a second of stale odometry is unusable for control whatever its cause. A per-topic ρ⋆ would be the refinement to make if the monitor were ever used for diagnosis rather than for vetoing a run.

### Load average is not utilisation

The decision to run an on-board estimator hinges on how much CPU the Raspberry Pi has left. The instinct is to read load average; on this platform that instinct gives the wrong answer by a factor of two.

Linux maintains the load average as an exponentially weighted moving average, sampled every Δ=5 s, over the number n(t) of tasks that are either runnable or in uninterruptible sleep:

ℓ(t) = ℓ(t−Δ) e−Δ/τ + n(t)1−e−Δ/τ,  τ∈{60, 300, 900} s.

The set counted by n(t) is the whole difficulty. A task in state D — blocked on disk, on USB, on a wireless transfer — contributes to ℓ while consuming no CPU whatsoever. Utilisation, by contrast, is read from the jiffy counters of /proc/stat over an interval:

U = 1−ΔidleΔtotal,  cores free = ncpu (1−U).

Measured on the rover on 2026-08-24, with the full stack running and no on-board estimator:

ℓ=4.18  on ncpu=4,  U=0.54,  cores free=4×0.46=1.84.

Read as utilisation, ℓ/ncpu=1.045 says the machine is saturated and the campaign is impossible. The jiffy counters say 46 % of four cores is idle. The gap is the population of blocked tasks,

nD ≈ ℓ−ncpuU = 4.18−4×0.54 = 2.02,

about two tasks permanently waiting on I/O — consistent with the RealSense USB pipeline and SD-card traffic, neither of which competes for CPU with an estimator. Since sqrtVINS runs single-threaded on this build (num_opencv_threads=1, Section 12.29), it requires approximately one core against the 1.84 available. The margin is real but thin, which is why the campaign script measures [eq:camp:util] before every run rather than assuming it, and refuses to start below 25 % idle — one core, the strict minimum with no reserve.

### Comparability: separating the estimator from the architecture

Deploying sqrtVINS on the robot changes what it consumes. The two ground estimators subscribe to republished streams that have crossed a wireless link; the on-board instance reads the sensors directly.

The two data paths, and why the ground instance of sqrtVINS is retained even though it is redundant as an estimator. Holding one of the two factors fixed at a time is what makes the decomposition [eq:camp:decomp] available.

Let pE,A(t) denote the trajectory produced by estimator E on architecture A, with E∈{sqrt,ov,mins} and A∈{Pi,PC}. Comparing the on-board square-root filter against the ground openVINS gives a difference in which two independent causes are superposed. Inserting the ground instance of sqrtVINS — the same code on the other architecture — splits them by a telescoping sum:

psqrt,Pi−pov,PC⏟observed = psqrt,Pi−psqrt,PC⏟architecture, code fixed + psqrt,PC−pov,PC⏟estimator, inputs fixed

The identity is trivial; its consequence is not. Without psqrt,PC the two terms on the right are not separately observable, and any discrepancy can be attributed to whichever of the two the reader prefers. This is the reason the campaign keeps both instances of sqrtVINS running and records them as distinct sources, and the reason the trajectory resampler carries four keys rather than three. It is also a caution about the campaign’s headline comparison: an on-board estimator measured against ground-based ones is not an estimator comparison unless the middle term is available.

### Resampling onto a common time base

The four series arrive on independent clocks at unequal rates. Comparison requires a common grid, which requires interpolation, which for the rotational part is not a vector-space operation.

#### Rotational interpolation, and the error that actually matters

Between two unit quaternions qi, qi+1 separated by Ω=arccos⟨qi,qi+1⟩, spherical linear interpolation is

slerp(qi,qi+1;u) = sin​(1−u)ΩsinΩ qi + sin(uΩ)sinΩ qi+1,  u∈[0,1],

which traverses the geodesic of S3 at constant angular velocity. Normalised linear interpolation follows the same path but at a non-uniform rate; writing θ(u)=atan2​usinΩ, (1−u)+ucosΩ for the angle it actually reaches, a third-order expansion in Ω gives

θ(u)−uΩ = Ω3u22−u6−u33+O(Ω5),  maxu|θ(u)−uΩ|≈0.0161 Ω3.

At Ω=π/2 this is 0.062 rad, or 3.6∘ — the familiar figure that motivates slerp. At the rates in play here it is nothing at all: the rover’s yaw rate stays below 0.5 rad s​−1 and odometry is published at 84 Hz, so Ω≤6×10−3 rad and the bound in [eq:camp:nlerp] evaluates to 3.5×10−9 rad.

The consequential subtlety is elsewhere. The unit quaternions double-cover SO(3): q and −q denote the same rotation, and a publisher may emit either. If consecutive samples straddle a sign change, ⟨qi,qi+1⟩<0, then [eq:camp:slerp] interpolates along the long arc, sweeping 2π−Ω instead of Ω. The remedy is one line — replace qi+1 by −qi+1 whenever the inner product is negative — and the asymmetry of the two errors is the point:

sign-flip errorslerp-versus-lerp error ≈ π3.5×10−9 ≈ 9×108.

Nine orders of magnitude separate the failure everyone guards against from the one that is easy to omit. The validation written for this pipeline initially tested qw≈1 at the origin, which a legitimately negated quaternion fails; the correct invariant is |qw|≈1.

#### Origin alignment, and why its order is not free

Each series is expressed in its own initialisation frame. Alignment to a common origin is the rigid transformation

p′(t)=R0⊤p(t)−p0,  q′(t)=q0−1⊗q(t),

with (p0,q0) the pose at the origin of the comparison. The operation must be applied after resampling, not before, and the reason is a timing argument rather than an algebraic one.

Let series X have raw samples beginning at t0X, and let the common window begin at T=maxXt0X, since the intersection of the supports — not their union — is the only interval on which every series is a measurement rather than an extrapolation. Aligning before resampling sets (p0,q0)=(pX(t0X),qX(t0X)), so that on the common grid

pX′(T)=R0⊤pX(T)−pX(t0X) ≠ 0  whenever t0X<T,

which is every series except the latest to start. The trajectories then begin at different points, and since the campaign’s primary metric is loop closure — the norm ∥p(tend)−p(tstart)∥ for a path returning to its start — an origin offset enters the result directly and silently. Applying [eq:camp:se3] after resampling, with (p0,q0) taken at T, makes every series start at the origin by construction.

### The campaign protocol

#### Ordering, and the coverage condition

The launch order is not a convenience. Write [s,e] for the temporal support of the recording and [ms,me] for that of the health monitor. The recording is admissible as evidence only if

ms≤s and me≥e,

that is, only if every instant of the bag is covered by health evidence. Violating the left inequality is the realistic failure: starting the monitor after the recorder leaves an initial interval — exactly when nodes are starting and contention peaks — with no observation, and a starvation occurring there would corrupt a file believed to be clean. Since the monitor additionally needs one window W to fill before it can judge, the operational requirement is ms≤s−W.

Campaign sequence. Stages 1 and 1bis are vetoes. The monitor’s support strictly contains the recording’s, with at least one window W of lead so that its sliding estimators are converged before the first message is written — condition [eq:camp:couverture].

#### Admission thresholds

Table 12.34 collects the conditions under which the campaign script refuses to start, each with the mechanism that justifies it. Two of them — ground temperature and battery voltage — were already established in Section 12.30.9 after they invalidated earlier runs; the three robot conditions are added here.

Admission conditions. A run failing any of them is refused outright.

| quantity | veto | mechanism |
| --- | --- | --- |
| ground CPU temperature | ≥82∘C | thermal throttling; the recorder falls behind and the loss is written into the bag indistinguishably from a sensor fault |
| browser CPU | ≥50 %​ | measured cause of the above; the cockpit overlay alone reached 109 % |
| battery | <11.1 V | below ≈30 % charge yaw authority degrades, so a commanded rotation is not the rotation performed |
| robot idle | <25 %​ | 25 % of four cores is one core; sqrtVINS is single-threaded and needs it, Eq. [eq:camp:util] |
| robot throttled | ≠ 0x0 | the SoC has clocked down; this is the mechanism behind the UART FIFO overruns of 2026-07-28 |
| robot disk | ≥92 %​ | a full card has already crashed leo.service on this platform |

#### Where each observation is made

The monitor runs on the ground station and not on the robot. The reasoning is that a monitor built to detect CPU starvation must not contribute to the load it measures: subscribing to an 84 Hz stream costs real CPU on the very machine under test. All topics are visible from the ground through the shared master, so nothing is lost by observing from outside.

That argument does not extend to the robot’s own resource counters, and the distinction is worth making explicit because it looks like an exception. Reading /proc/stat and the SoC’s throttling register every two seconds costs no measurable CPU — there is no subscription, no deserialisation, no callback. It is also the only place the cause can be seen: the ground monitor observes effects, a rate that falls, while saturation and clock throttling are visible only on the machine that suffers them. The campaign therefore samples both, and the post-run summary reports the minimum idle fraction alongside the rate transitions so the two can be correlated.

The final verification closes the loop on the recording itself. Rates are recomputed from the bag rather than taken from any live counter: a topic can publish at 18.4 Hz and still be written at 12, if the recorder was the component under pressure. Only the file’s own contents establish what was captured, which is the whole purpose of an admissibility check — Eq. [eq:camp:fhat] applied to the recording, with N the message count and tN−t1 the bag duration.

# Conclusion and Perspectives

## Achievement Against Stated Objectives

The five formally stated objectives of this internship (§1) have been addressed as documented in Table 13.1.

Objective achievement summary

| Objective | Target | Achieved | Evidence |
| --- | --- | --- | --- |
| O1: Vision reliability | >95% detection, <50 ms latency | 98.7% LED / 100% CB; 5–20 ms | Table [tab:vision-perf-measurements] performance measurements |
| O2: Autonomous FSM | Safe operation in all failure modes | 10/10 validation tests passed | Table [tab:fsm-validation] test results |
| O3: Professional cockpit | >30 Hz, clean reconnect, FSM display | 55–60 Hz on all tested hardware | Table 10.1 + operator sessions |
| O4: System integration | Single-command launch, documentation | startleo.sh + technical reference | Delivered to lab supervisor |
| O5: Carolus contribution | Quantitative performance data | Tables [tab:vision-perf-measurements]–10.1 produced | Submitted to comparative study |

## Extension Beyond the Original Objectives: The July–August 2026 Estimation Campaign

The five objectives above governed the internship as originally scoped, built around brightness-threshold LED and checkerboard beacon localization. Chapter 12 documents a substantial extension undertaken once that system was operational and validated: replacing beacon-only, intermittent localization with a continuously-running, tightly-coupled MSCKF-based state estimator (MINS, with OpenVINS as a hot-swappable secondary source), and hardening the resulting platform for operation outside the laboratory. Table 13.2 summarizes this extension against the same evidence standard as Table 13.1.

Extension objectives achievement summary (July–August 2026 campaign)

| Objective | Target | Achieved | Evidence |
| --- | --- | --- | --- |
| Table (continued) |  |  |  |
| Objective | Target | Achieved | Evidence |
| continued on next page |  |  |  |
| E1: Continuous pose estimation | Replace intermittent beacon fixes with an always-available fused pose | MSCKF-based MINS/VINS with lossless SE(3) switch | §12.1, Table 12.3 |
| E2: Quantified estimator stability | Statistically distinguish sensor noise from modelling divergence | σZ reduced 5.2× (257 mm → 50 mm) via extrinsic correction | Table 12.3, §12.7 |
| E3: Field-scale connectivity | Operate beyond the lab room without a network-driven interruption | WireGuard tunnel over campus Wi-Fi (partial: an unresolved second failure, §12.8.3, forced a same-subnet Wi-Fi fallback, §12.8.4); two full building traverses completed, one on each path | §12.6.4, §12.6.5, §12.8.3 |
| E4: Autonomy & safety hardening | Guard the estimator and drive chain against observed failure modes | Three IMU integrity guards, respawn policy, watchdog reliability model | §12.2.4, Eq. [eq:debounce] |
| E5: Operator platform maturity | Production-grade public API and self-healing infrastructure | Five-page cockpit, Cloudflare tunnel, cron watchdog, single-command leo restart | §12.6, §12.6.2 |

These were not part of the original five objectives (§1) and are reported separately for that reason, but they account for the majority of the report’s length and, by the quantitative measures in Chapter 12, its most demanding engineering: fusing two independent state estimators without a discontinuity in the served pose, and making that estimate (and the operator’s ability to reach it) survive a real building’s Wi-Fi topology rather than a lab bench.

## Synthesis of Technical Contributions

The principal original contributions of this internship, beyond the resolution of the documented defects, are:

- Density-based LED cluster anchor algorithm: a formally provable, O(n2) algorithm for correctly identifying beacon LED clusters in the presence of larger-area spurious reflections. This algorithm is independently transferable to any LED-based optical beacon system.

- Two-stage checkerboard detection architecture: the systematic promotion of findChessboardCornersSB to the primary detection path, with rate-limited discovery scans and explicit result caching, resolves a class of checkerboard detection failures that is not documented in the OpenCV literature.

- Session-aware heartbeat stale guard: a minimal state machine addition that resolves the session-boundary edge case in deadman switch implementations, enabling reliable reconnection behavior in web-based robot control interfaces.

- UDP routing-based IP discovery (guessip()): a portable Python technique for deterministic local IP detection that is immune to interface ordering, applicable to any multi-homed robotics workstation.

- Full-stack operator cockpit: a production-quality browser-based mission control interface combining glassmorphism design language, RAF-decoupled canvas overlay, O(1) ring buffer trajectory rendering, and a six-condition alert system.

- Lossless dual-estimator switch: an SE(3) re-anchoring correction, computed at the instant of a VINS/MINS switch, that makes the served pose bit-for-bit continuous across a change of estimator despite each running its own independent, separately-drifting reference frame.

- Statistically-grounded calibration validation: the T1.1 static-drift protocol separates sensor noise from a modelling error by the sign and statistical significance of a least-squares drift slope rather than by inspection, fully automated and archived per run.

- Closed-form debounce reliability model: an expected-time-between-false-triggers formula (Eq. [eq:debounce]) for run-length-based supervisor debounces, fitted against field data to both justify a specific parameter choice and honestly bound what that choice does and does not fix, and is independently transferable to any supervisor guarding against a flaky liveness probe.

- Firewall-constrained network migration: a WireGuard-tunnelled architecture that turns ROS’s arbitrary-port TCPROS traffic into a single pre-authorised UDP flow, restoring local-network-equivalent performance (15 Hz camera, single-digit-second MINS initialization) across a campus network that blocks the underlying protocol directly; a second, architecturally similar tunnel failure later in the campaign proved unresolvable by the same remedies (§12.8.3) and prompted a same-subnet Wi-Fi fallback (§12.8.4) as a documented, evidence-based alternative rather than a replacement.

## Lessons Learned

### System Engineering Lessons

The most important lesson of this project is the compounding nature of architectural deficiencies. Each of the five initial defects was individually resolvable, but their combination made the system appear completely non-functional. The LED detection bug produced 0% valid detections, so the FSM never left the PATROL state: making the LOCK, WAIT, and U_TURN code paths completely untestable. This cascading untestability is a pathology of tightly coupled systems without interface mocks. Future system development will include isolated subsystem validation with mock data sources before integration.

### Computer Vision Lessons

The checkerboard detection failure illustrates a general principle in computer vision system design: the empirically-validated best algorithm for a specific physical target should be used from the first day of development, not added as a fallback after the simpler approach fails in production. In hindsight, the discovery that findChessboardCorners fails on the laboratory board should have prompted its immediate deprecation in favor of findChessboardCornersSB, rather than the multi-week incremental integration that was actually followed. This principle generalizes: for any custom fiducial or beacon target, the detection algorithm selection should be validated against the specific physical hardware before integration, not inferred from OpenCV documentation defaults.

### Robotic HMI Design Lessons

The design of the operator cockpit revealed that the requestAnimationFrame / WebSocket decoupling is not merely an optimization but an architectural necessity. An initial prototype that redrew the canvas only on telemetry arrival produced a jagged, twitchy overlay that made the operator feel disconnected from the robot’s actual motion. The switch to a 60 Hz RAF loop with LERP smoothing transformed the operator’s perception of the system from “unresponsive” to “smooth and confident”: despite zero change in the underlying telemetry rate. This demonstrates that perceived system performance is as architecturally important as measured system performance in human-operated robotic systems.

### Reliability Engineering Lessons

The campaign’s most consequential lesson was methodological rather than technical: a real-time explanation formed while a field test is in progress is a hypothesis, not a finding. The transient pose-source flips observed during the building traverse (§12.6.5) were attributed, live, to visual feature starvation: a plausible-sounding story that turned out to be entirely wrong. Correlating the ground-station watchdog’s own log against the flip timestamps after the fact revealed the true cause (a debounce sized for an isolated fault, not sustained Wi-Fi-roaming latency) within minutes. The general rule this confirms: a causal claim made without consulting the system’s own logs should be held provisionally, however plausible it sounds in the moment, and re-derived from evidence before it is written down as a conclusion, a rule this report tries to apply to itself by documenting the wrong hypothesis alongside its correction rather than silently replacing one with the other. A second, related lesson generalizes an earlier finding on the same watchdog (§10): a supervisor’s own probes are themselves failure-prone, and each one needs to be audited for what happens when it is wrong, not only for whether it correctly detects the faults it was written to catch.

The same rule recurred, independently, sixteen days later (§12.16.1). A pose-fusion guard was rejecting real, correctly-timed measurements on the rectangle protocol’s driven turns, and the first candidate explanation, a floor on the guard’s time-delta denominator that could inflate an ordinary displacement into an impossible speed, was plausible enough to have been written down as the finding. It was not: five hundred and ninety-two samples captured with the rover stationary refuted it directly. The true mechanism (a filter’s own discrete visual-update corrections misread as divergence, §12.16.1) only appeared once the rover was captured in motion, not at rest — the static capture that refuted the first hypothesis was, by construction, unable to reproduce the symptom it was meant to explain. Two independent episodes of the same failure mode, sixteen days apart, on two different subsystems of the same estimator stack, is enough to state the rule generally rather than as a one-off: a hypothesis about a live system’s behaviour should be tested under the same conditions that produced the symptom, not merely against whatever data happens to be convenient to capture first.

## Future Development Roadmap

### SLAM Integration for Metric Mapping

Continuous pose tracking no longer depends on beacon visibility (§12.1): the MSCKF-based estimators run at all times, and the LED/checkerboard/AprilTag beacon infrastructure now serves a narrower, still-important role as an absolute, drift-free global anchor rather than the sole source of position (leonavigation’s open Carolus/AprilTag anchoring item, Table [tab:openitems]). What remains environment-specific is exactly that anchor: without it, MINS/VINS accumulate the ordinary unbounded drift of any dead-reckoning estimator, so metric mapping in an arbitrary, unprepared environment still requires a SLAM system. The built, not-yet-deployed leoautonomy package (Table [tab:openitems], item 8) is the concrete step toward this; a lightweight system such as hectorslam (no odometry required, suitable for the D455’s depth stream projected to a 2D occupancy grid) remains an appropriate choice, and the dual-frame odometry architecture is already compatible with it: a SLAM pose estimate would replace or augment the beacon anchor in the world-frame position computation, not the wheel odometry, since that role is now filled by MINS/VINS.

### Machine Learning-Based Beacon Detection

The brightness-threshold LED detection is robust in the laboratory but would fail in outdoor environments (sunlight is brighter than 220/255) or in rooms with multiple similar bright sources (projector screens, large monitors). A YOLOv8-nano model trained on a dataset of ∼500 annotated frames (beacon present vs. absent) would enable deployment-agnostic detection at <5 ms inference time on the laboratory GPU. The existing vision subprocess architecture is directly compatible with a torch.nograd() inference call in place of the brightness threshold pipeline.

### ROS 2 Migration

ROS Noetic reached End-of-Life in May 2025 (already past at the time of writing). The migration to ROS 2 Humble is planned for the Carolus framework’s next generation. The key migration impacts for the LEO Rover Mission Control system are: (1) rospy subscriber callbacks become rclpy executor-managed callbacks (async-compatible); (2) the rosbridge protocol v2 is compatible with both ROS 1 and ROS 2 via rosbridgeserver’s ROS 2 branch; (3) the multiprocessing spawn architecture is unchanged (it is ROS-agnostic). The estimated migration effort is 2–3 weeks.

### Multi-Robot Coordination

The Carolus framework’s ultimate objective is the coordination of the three robot platforms (LIMO, LEO, RoboMaster S1) in a shared environment. The LEO Rover Mission Control system’s world-frame coordinate system is already defined in the same reference frame as the other platforms’ coordinate systems (all originate at their respective start positions). A rendezvous protocol (where the LEO rover navigates to a known world-frame position while a second robot navigates to the same position from a different starting point) would require: (1) a shared ROS master or ROS 2 domain for cross-robot topic exchange; (2) a common beacon infrastructure at known world-frame positions; (3) collision avoidance between platforms (the depth-based obstacle detection is already implemented).

### Mobile Device Interface Extension

The current cockpit is optimized for desktop browsers (16:9 displays, keyboard + mouse). A mobile-optimized interface (responsive layout, touch-based joystick, swipe gesture for mode changes) would significantly expand the operational scenarios for which the system is suitable. The CSS grid layout already includes a responsive breakpoint at 1200 px; extending this to a single-column mobile layout and implementing a touch-event joystick (replacing the pointer-event-based implementation) is estimated at 1 week of front-end development.

### Architectural Debt: Findings of a Senior-Level Optimality Audit

The preceding subsections describe features not yet built. This one is different in kind: a post-hoc audit of choices already shipped, conducted against the question a senior architect asks of a working system — not “does it work” but “what would infinite time have bought instead.” Four items survived that scrutiny as genuine technical debt rather than reasonable engineering trade-offs. Each is stated as a debt with a proposed repayment, not as a retroactive criticism of decisions that were, in every case, correct given the time available.

##### Network transport and zero-trust connectivity.

The browser-facing bridge (§17.6.5) carries ∼60 telemetry fields as text JSON at 10 Hz, a format chosen for its simplicity and human-readability during development but never reconsidered once the system stabilised. The cost is measured, not assumed: 1–3 ms of serialisation per packet (Table 1.1), on top of the bandwidth text encoding always costs over a binary one. Future work should replace it with a binary encoding, CBOR or Protocol Buffers, over the same WebSocket, or migrate the live telemetry path to a WebRTC DataChannel outright. Separately, and this is the more consequential item: the ROS 2 migration costed in §13.5.3 (2–3 weeks) is scoped only for the rospy → rclpy API surface. It does not account for what FastDDS actually requires on this network: DDS’s default discovery relies on multicast, which is precisely what campus NAT and the existing WireGuard tunnel were fighting throughout this campaign (registry items 12–13, §17.6). WireGuard supplied an encrypted point-to-point pipe and nothing above it — no NAT traversal, no endpoint healing, no relay of last resort — which is why every roaming failure had to be diagnosed and patched by hand. A Software-Defined Network overlay (Tailscale or ZeroTier, both built on WireGuard’s own primitives) resolves multicast discovery and NAT traversal natively and should be treated as a prerequisite for the ROS 2 migration, not an independent, lower priority item. Attempting the DDS migration on the current transport risks reproducing the exact discovery failures already spent a field campaign diagnosing, one layer higher in the stack.

##### Omnidirectional kinematics.

§12.4 documents the mid-campaign discovery that this rover’s wheels are not rigid skid-steer wheels but a FictionLab mecanum roller set, and the deliberate decision to keep them without building the matching odometry model. The cost of that decision is now fully quantified and it is larger than a rounding error: the current Wheel2DAng differential approximation, fed by a firmware topic (leomsgs/WheelOdom) that collapses two planar degrees of freedom into a single scalar velocitylin, forces the χ2 gate (Eq. [eq:chi2], α=5) to reject the large majority of wheel measurements during rotation — the acceptance rate falls from 100 % on straight runs to 6 % under rotation (§12.4). The platform therefore drives on four independently actuated omnidirectional wheels and extracts full odometric value from perhaps one axis of that freedom. The differential approximation must be replaced with the complete mecanum forward kinematics, resolving the roller-slip vectors at each wheel’s ±45∘ contact angle into the full planar velocity (vx,vy,ω) rather than the scalar MINS currently receives, and coupling that corrected signal into the MSCKF wheel-update step in place of the present two-wheel fiction. MINS itself offers no native omnidirectional wheel type (only Wheel2D{Ang,Lin,Cen} and Wheel3DAng), so this is not a configuration change: it requires either an upstream contribution to MINS or a pre-processing stage that resolves the mecanum kinematics before the Wheel2DAng interface. leomsgs already defines WheelOdomMecanum, carrying the lateral term the current pipeline discards; it is published by no node in this system. The Hardware Specifications table (Table 15.1, Appendix B) has been corrected as part of this revision: it previously read “4-wheel differential drive (skid steering)”, a description the mecanum discovery of §12.4 directly contradicts, and which would have sent the next reader to the wrong physical model on their first consultation of the document.

##### Vision subprocess IPC.

The multiprocessing spawn architecture (§1.5.5) does exactly what it was built to do: GIL contention on the vision path measured at 0 %, down from 100 % in the single-process baseline (Table 4.1). That result is real and should not be walked back. What it does not report is the price paid for it. Every frame crossing the process boundary is JPEG-encoded on the main thread, queued, decoded again in the subprocess, and OpenCV-processed from there — two full codec passes spent purely on inter-process transport, uncounted anywhere in this document. The IPC channel should move to a zero-copy multiprocessing.sharedmemory ring buffer, passing raw frame data behind a lightweight semaphore handoff instead of a pickled, JPEG-wrapped queue payload. At infinite engineering budget, the vision node belongs in C++ or Rust, where no interpreter lock exists to work around in the first place and the entire IPC question is moot.

##### Front-end rendering and streaming latency.

Two independent limits sit in the browser layer, and both were measured along the wrong axis. First, the video path (webvideoserver, MJPEG over HTTP) was validated on bandwidth alone (4.17 Mbps at 20 Hz, well inside the 802.11ac budget, §7.4.1) and never on end-to-end latency, the metric that actually governs closed-loop teleoperation. MJPEG-over-HTTP inherits TCP’s head-of-line blocking: a single dropped segment, on the same Wi-Fi link whose fragility this report documents at length elsewhere (§17.6), stalls the entire frame rather than degrading gracefully. WebRTC, over RTP/UDP with hardware-accelerated decode, should replace MJPEG for the live feed, trading TCP’s guaranteed-delivery-at-any-latency for UDP’s bounded-latency-at-the-cost-of-an-occasional-frame, which is the correct trade for a human closing a control loop on live video. Second, the trajectory view’s Canvas 2D batch-rendering fix (§7.8, six age-quantised strokes per frame) reduces rasterisation calls from Θ(S) to Θ(B), but the per-frame vertex-append loop remains Θ(S) in the trail length, so a long-enough mission eventually re-encounters the cost the fix was written to remove. A WebGL vertex buffer, uploaded incrementally and drawn in a single call regardless of history length, removes the growth term entirely rather than amortising it. The irony is available in the same codebase: 3dengine.js already drives a WebGL context via three.js for the cockpit’s decorative satellite and capsule renders (§2.4.1) — the infrastructure exists and is exercised nightly for ornament, one page over from the data view that would benefit from it most.

### Near-Term Field Engineering (Campaign Open Items)

The campaign maintains its own explicit open-items registry (Table [tab:openitems]), cross-referenced here rather than duplicated. Its thirty-nine entries have grown with the campaign, and the ordering of priorities has changed materially since the earlier revisions of this section. Four items now gate the work ahead of any further measurement. The colour-camera intrinsics (item 36) are blocking: the beacon detector runs with parameters calibrated at 1280×720 against a 640×480 stream, placing the principal point outside the image, inflating every range by 69.3 %, and rendering unpublishable any dimensional quantity derived from that camera. The beacon-anchored absolute-error measurement (item 37) is implemented, instrumented and never recorded, so the entire MINS-versus-openVINS comparison remains a statement about mutual divergence rather than accuracy; it is gated on item 36. A second, ungated absolute-error method was added on 2026-08-05 (item 39, §12.16.6): it needs neither the colour camera nor a placed-and-frozen anchor, only a fixed AprilTag re-sighted on the already-calibrated infrared pair, but is in the identical state as item 37 for a different reason — built and verified on synthetic data, not yet exercised on a real multi-lap drive. The thermal envelope (item 38) was measured on both sides and is roughly three degrees wide, the serial link holding at 86 ∘C under full colour load and collapsing entirely at 89 ∘C, which makes active cooling a prerequisite rather than a refinement; the same session that added item 39 measured it again independently (§12.16.4) and found the same three-degree width, plus that a stopped-camera pause recovers it for roughly two minutes only. The rigid-wheel radius and track re-measurement (item 1) remains the highest-priority metrological action, independently corroborated both by the field-observed steering drift on commanded-straight motion and, on 2026-08-05, by a measured left/right turning asymmetry on the same rover that a single scale correction cannot explain (§12.16.3). Of the network items, the second unresolved WireGuard failure (item 12) and the ground-station Wi-Fi driver fault (item 13) both remain open, and the discontinuous one-second-burst motion (item 14) still has two undistinguished candidate causes. The watchdog debounce of Eq. [eq:debounce] (item 9) should now be read with care: its widening was measured to lengthen the false-trigger interval by roughly 3–7×, but the proven root cause of the restart storms was found later, on July 29, to be a PATH defect in the cron invocation that made both liveness guards fail unconditionally. The debounce work was therefore treating a symptom, and any estimator measurement taken while a storm was in progress is invalid — the same lesson recurred on 2026-08-05 at a different layer of the same estimator stack (§12.16.1): a plausibility guard’s own single-sample latch was silently starving its output, and the fix required capturing the platform while actually being driven rather than at rest, after a first, plausible-sounding hypothesis about the guard’s arithmetic did not survive that measurement. The remaining items (the Kalibr camera-IMU extrinsic refinement, TF placeholder measurement, RPLidar bring-up, AprilTag size cross-check, and the built-but-undeployed leoautonomy SLAM package) are lower urgency.

## Closing Statement

This internship represents a comprehensive full-stack systems engineering exercise, from the embedded motor control firmware interfaced via ROS topics to the JavaScript rendering engine drawing 60 fps annotations over a live video feed, and, in its extension, to a tightly-coupled multisensor estimator and the campus network infrastructure needed to reach it from outside a single room. The five architectural defects identified at the outset were each independently capable of preventing the system from functioning; their resolution required understanding the system as an integrated whole rather than as a collection of separable components, a discipline the campaign confirmed again and again at different layers, from IMU integrity guards through to a firewall that silently discarded most of ROS’s own transport protocol.

The resulting LEO Rover Mission Control system achieves production-quality reliability for the Carolus laboratory’s research objectives: fully autonomous patrol missions with reliable beacon detection, continuous MSCKF-based pose tracking independent of beacon visibility, safe operation under all tested failure modes, campus-wide connectivity validated by two complete building traverses on two different network architectures, and a professional operator interface that provides genuine situational awareness rather than a collection of raw data fields. The system is deployed and operational in Dr. Gutierrez’s laboratory as of the completion of this internship, and serves as the reference implementation against which the LIMO Rover and RoboMaster S1 platforms’ implementations will be benchmarked in the Carolus comparative study.

From a personal engineering perspective, this project has reinforced the critical importance of root cause analysis over symptom masking, the architectural value of clean layer separation (particularly the vision subprocess isolation), the human factors dimension of robotic system design (a dimension that is easily neglected in favor of “real robotics” work but is ultimately the dimension that determines whether an operator can reliably use the system under real operational conditions), and, from the later campaign, that a field engineer’s own real-time explanation of a live anomaly is only ever a provisional hypothesis until it survives contact with the system’s logs.

# Technical Glossary

Table [tab:glossary] defines all acronyms and specialized terms used throughout this report.

@L3.6cmL10cm@

Term / Acronym & Full Form / Definition
Term / Acronym & Full Form / Definition
AGV & Autonomous Ground Vehicle (self-propelled wheeled robot operating without continuous human teleoperation)
ArUco & Augmented Reality University of Córdoba (fiducial marker system for pose estimation)
CALIBCBFASTCHECK & OpenCV flag enabling coarse histogram pre-check in findChessboardCorners; fails on non-standard boards
CB & Checkerboard (high-contrast printed calibration and localization target)
Chi-squared (χ2) gate & Statistical outlier-rejection test comparing a measurement’s Mahalanobis distance against its expected covariance; used by the estimators to reject wheel/visual measurements inconsistent with the current state
Curve25519 & Elliptic-curve Diffie–Hellman key-agreement function; the asymmetric primitive underlying WireGuard’s key exchange
EKF & Extended Kalman Filter (recursive Bayesian estimator linearizing a nonlinear model at each step; MSCKF is a variant restricted to a sliding window of past poses)
EMA & Exponential Moving Average (first-order IIR low-pass filter for signal smoothing)
FIT & Florida Institute of Technology, Melbourne FL
FSM & Finite State Machine (formal behavioral specification with enumerable states and guarded transitions)
GIL & Global Interpreter Lock (CPython mutex preventing true multicore parallelism in threaded Python)
HMI & Human-Machine Interface (system enabling operator monitoring and control of a robotic system)
IIR & Infinite Impulse Response (digital filter whose current output depends on past outputs, i.e. recursive)
IMU & Inertial Measurement Unit (sensor measuring linear acceleration and angular rate)
LERP & Linear Interpolation (smooth blending between two values: x(t)=x0+t(x1−x0), t∈[0,1])
MINS & Multisensor-aided INertial navigation System (open-source tightly-coupled EKF estimator, RPNG/University of Delaware, fusing IMU with camera and other aiding sensors in one filter, §12.1)
MJPEG & Motion JPEG (video format where each frame is individually JPEG-compressed; no inter-frame prediction)
MSCKF & Multi-State Constraint Kalman Filter (EKF variant that augments the state with a sliding window of past camera poses instead of 3D landmark positions, avoiding per-feature state growth, §12.1)
NAT & Network Address Translation (router/firewall function rewriting private addresses to a shared public one; its UDP mapping timeout motivates WireGuard’s keepalive, §12.6.4)
OpenVINS & Open-source MSCKF-based Visual-Inertial Navigation System (RPNG/University of Delaware; the “VINS” side of this report’s VINS/MINS switch, ROS node ovmsckf)
PnP & Perspective-n-Point (algorithm computing camera pose from n 3D-2D point correspondences)
RAF & requestAnimationFrame (Web API for scheduling visual updates synchronized to display refresh rate)
ROI & Region of Interest (rectangular subset of an image processed independently of the full frame)
ROS & Robot Operating System (middleware framework for robotic software: pub/sub, drivers, tools)
rosmon & ROS process monitor (drop-in replacement for plain roslaunch node supervision, used on the robot; exposes a startstop service for external start/stop control, used by the watchdog’s camera-recovery ladder)
RTB & Return To Base (autonomous navigation back to the mission start position, i.e. world origin)
RTK-GPS & Real-Time Kinematic GPS (differential correction achieving cm-level outdoor positioning)
SA & Situation Awareness (Endsley’s three-level cognitive model: perception, comprehension, projection)
SB & Sub-Pixel-Befitting (OpenCV’s enhanced checkerboard detector using locally adaptive binarization)
SE(3) & Special Euclidean group in three dimensions: the set of rigid-body transformations (rotation + translation); the pose selector’s source-switch correction (Eq. [eq:se3]) is an SE(3) element
SLAM & Simultaneous Localization and Mapping (algorithm building a map while localizing within it)
TCPROS & ROS’s default transport protocol: topic data flows over a plain TCP socket whose port is negotiated per connection through an XML-RPC handshake (§12.6.4)
UBSAN & Undefined Behavior Sanitizer (compiler instrumentation detecting undefined-behavior violations at runtime; surfaced the PC Wi-Fi driver fault of §12.6.4 via dmesg)
UGV & Unmanned Ground Vehicle (remotely operated or autonomous wheeled/tracked platform)
UWB & Ultra-Wideband: radio technology (3.1–10.6 GHz) enabling cm-level indoor ranging
VPN & Virtual Private Network (encrypted point-to-point or site-to-site network overlay; WireGuard, §12.6.4, is the VPN implementation used in this report)
WebSocket & RFC 6455 (full-duplex TCP-based protocol for real-time bidirectional browser communication)
WireGuard & Kernel-space VPN protocol using Curve25519 key exchange and ChaCha20-Poly1305 encryption over a single UDP port; replaces sshuttle as this report’s off-site transport (§12.6.4)
XML-RPC & Remote Procedure Call protocol encoding requests/responses as XML over HTTP; used by the ROS master for node registration and by TCPROS to negotiate each topic’s data port

# Hardware Specifications

## LEO Rover Platform

Table 15.1 summarizes the mechanical and electrical specifications of the LEO Rover platform.

LEO Rover mechanical and electrical specifications

| Parameter | Value |
| --- | --- |
| Manufacturer | Husarion (Warsaw, Poland) |
| Kinematics | 4-wheel mecanum (FictionLab roller set); driven through a differential-drive odometry approximation, not native omnidirectional kinematics — see §12.4 and §13.5.6.0.2 |
| Drive motors | 4× BLDC with Hall-effect encoders |
| Max speed (linear) | ∼0.5 m/s (firmware-limited; operational limit 0.2 m/s) |
| Max speed (angular) | ∼1.5 rad/s (operational limit 0.5 rad/s) |
| Chassis material | Aluminium alloy, IP44 rated |
| Battery | 14.4 V Li-ion, 7800 mAh (112 Wh) |
| Nominal run time | ∼4 h at slow patrol speed |
| Communication | ROS Noetic over Wi-Fi; robot AP (wlanext, 10.0.0.1) or, since 2026-07-20, campus Wi-Fi + WireGuard tunnel for building-wide range (Ch. 12, §12.6.4) |
| Firmware | AgileX leofirmware on Raspberry Pi CM4 (onboard) |
| Odometry | Wheel encoder ticks → /firmware/wheelodom at ∼8 Hz |

## Intel RealSense D455 Camera

Table 15.2 summarizes the D455 depth camera specifications relevant to this project.

Intel RealSense D455 specifications

| Parameter | Value |
| --- | --- |
| Depth technology | Active IR stereo (structured light projector + 2× IR imagers) |
| IR stereo baseline | 95 mm |
| Depth range | 0.4 m–6 m (nominal); 0.1 m–8 m (extended) |
| Depth resolution | 848×480 px |
| Depth accuracy | <2% at 4 m under controlled illumination |
| RGB resolution | 1280×720 px (subscribed) / 640×480 (compressed) |
| RGB frame rate | 30 Hz maximum; 15 Hz nominal (lab config; measured 14.9 Hz) |
| RGB intrinsics (measured) | fx=380.25, fy=379.26, cx=320.40, cy=243.72 px at 640×480 |
| IR intrinsics (measured) | fx=fy=386.79, cx=320.06, cy=236.77 px at 640×480 |
| Constant used in code | $\texttt{\renewcommand{\_}{\textunderscore\penalty 0\hskip 0pt\relax}CAM\_FX}=384.65$ px (see note below) |
| Interface | USB 3.2 Gen 1 (5 Gbps) |
| SDK | Intel RealSense SDK 2.0 (librealsense2) |
| ROS driver | realsense2camera (ROS Noetic, realsense2cameramanager node) |

Do not set the colour stream to 10 Hz Earlier revisions of this table listed 10 Hz as the nominal frame rate. That configuration does not exist on this sensor. The D455 supports 6/15/30/60/90 Hz at 640×480, and the attempted change on 2026-07-08 was accepted silently by the driver, which then fell back to a depth-only default profile and disabled every infrared stream. The result was a complete vision outage that persisted until the change was reverted, with no error message at any point. See registry item 4 (Table [tab:openitems]). Validate any profile change against rs-enumerate-devices before deploying it.

##### On the 384.65 constant, and why it differs from both rows above.

leobackend.py estimates beacon range with a single focal length, CAMFX =384.65 px, together with an optical centre placed at the frame centre (320.0, 240.0). Neither figure is the measured calibration of either stream. It sits 1.16 % above the measured colour fx and 0.55 % below the measured infrared fx, which is consistent with its having been derived from the infrared sensor at a time when beacon detection ran on the infrared stream, and then carried over unchanged when the colour stream was adopted.

The consequence is bounded and the code says so in its own comment: the approximation is good to roughly 1–2 % for navigation at ≤3 m, which is well inside the other error sources in the beacon pipeline. It is nonetheless a constant with no single defensible provenance, and 384.65 propagates into derived results elsewhere in this report, including the LOCK controller’s loop gain and the range estimate. Treat it as a historical working value, not as a calibration.

This is a different problem from the detector intrinsics Do not confuse the 1–2 % approximation described here with the defect of §12.15.3, where carolusastrobeerex runs with fx=644.12 and cx=643.47, calibrated at 1280×720 and applied to a 640×480 stream. That one places the principal point outside the image and inflates every range by 69.3 %. The two are unrelated in cause and differ by nearly two orders of magnitude in effect.

## Operator Workstation

Table 15.3 summarizes the operator workstation configuration used for all development and validation.

Operator workstation specifications

| Parameter | Value |
| --- | --- |
| OS | Ubuntu 20.04.6 LTS (Focal Fossa), kernel 5.15 |
| ROS distribution | Noetic Ninjemys (Python 3.8) |
| CPU | Intel Core i7-10700K (8 cores, 16 threads, 3.8–5.1 GHz) |
| RAM | 32 GB DDR4-3200 |
| GPU | NVIDIA GeForce RTX 2060 (6 GB VRAM; unused for vision) |
| Network | Intel Wi-Fi 6 AX200 (802.11ax); dedicated AP for robot link |
| Python packages | rospy, cv2 (4.2.0), numpy (1.17.4), psutil, threading, multiprocessing |
| Browser (cockpit) | Google Chrome 124+ / Firefox 125+ (tested); any modern WebKit/Blink |

# Bibliography, Figure Index, and Notation

## Annotated Bibliography

The following bibliography lists all works referenced in this report, organized thematically. Citations follow the IEEE reference format; the full formatted reference list appears in the References section at the end of this document. Each entry below is accompanied by a brief annotation describing its specific relevance to the LEO Rover Mission Control project.

### Robot Operating System and Middleware

(Quigley et al. 2009) Quigley et al. (2009) introduced the Robot Operating System (ROS), the middleware framework upon which the entire LEO Rover Mission Control system is built. The paper establishes the publish-subscribe communication model, the rosmaster name resolution service, and the package management paradigm that define ROS’s architecture. The /mission/telemetry, /mission/command, and /cmdvel topics described in Chapter 3 directly implement the inter-node communication protocol defined in this foundational work.

(Crick et al. 2012) The rosbridge paper introduced the bidirectional JSON-over-WebSocket protocol that enables browser-based operator interfaces to subscribe and publish to ROS topics without requiring a ROS installation on the client. The rosbridgewebsocket server used in this project is the production implementation of the protocol described by Crick et al. The generation counter pattern (currentGen) and the /mission/command topic subscription are direct implementations of the rosbridge protocol specification (version 2.0).

(Open Robotics 2020) This release document specifies ROS Noetic as the final Long-Term Support (LTS) release of ROS 1, supported on Ubuntu 20.04 with Python 3.8. The platform constraints documented here: particularly the Python 3.8 compatibility requirement: governed the choice of multiprocessing.getcontext(’spawn’) over alternatives that may behave differently under Python 3.9+. The EOL date of May 2025 motivates the ROS 2 migration discussion in Chapter 13.

(Open Robotics 2013) The rosbridge protocol specification defines the JSON message format for topic advertisement, subscription, publication, and service calls over WebSocket. The /mission/command message format (action, mode, lin, ang fields) and the telemetry subscription JSON are direct implementations of the rosbridge v2.0 protocol message schema.

### Computer Vision and Image Processing

(Otsu 1979) Otsu’s seminal 1979 paper formalized the inter-class variance maximization criterion for automatic global threshold selection, establishing the theoretical benchmark against which the LED pipeline’s fixed threshold (LEDBRIGHTTHRESH=220) is compared. The analysis demonstrates that Otsu’s method is sub-optimal for the trimodal intensity histogram produced by LED beacons under fluorescent laboratory lighting.

(Niblack 1986) Niblack’s 1986 textbook chapter on locally adaptive thresholding introduced the formula T(x,y)=μ(x,y)+kσ(x,y) that forms the mathematical foundation of the adaptive binarization used inside OpenCV’s findChessboardCornersSB function.

(Sauvola and Pietikäinen 2000) Sauvola and Pietikäinen (2000) refined Niblack’s formula with a normalization that makes the threshold less sensitive to absolute intensity levels and more responsive to local contrast: the specific property that enables robust checkerboard corner detection under the non-uniform fluorescent illumination in the FIT laboratory.

(Viola and Jones 2001) While primarily known for its face detection cascade, the Viola–Jones paper’s central contribution to this project is the integral image data structure, which reduces the computation of any rectangular region sum from O(|W|) to O(1) after an O(N) preprocessing pass.

(Bradski 2000) The OpenCV library, documented in Bradski’s 2000 publication, provides all the computer vision primitives used in the leobackend.py vision subprocess: cv2.threshold, cv2.morphologyEx, cv2.findContours, cv2.findChessboardCornersSB, cv2.moments, and cv2.imdecode.

(OpenCV Contributors 2023) The OpenCV 4.x documentation for findChessboardCornersSB describes the Sub-Pixel Befitting algorithm, which uses locally adaptive binarization and direct corner-finding rather than the saddle-point detector used by findChessboardCorners.

(Suzuki and Abe 1985) Suzuki and Abe’s border-following algorithm is the basis for OpenCV’s findContours function used in the LED detection pipeline, tracing external boundaries in a single O(N) pass.

(Matheron 1975) Matheron’s 1975 monograph established mathematical morphology as a rigorous set-theoretic framework. The formal definitions of dilation, erosion, opening, and closing follow Matheron’s original notation and proofs.

(Serra 1982) Serra’s 1982 textbook provided the computational framework for applying Matheron’s abstract morphological theory to digital images, including the discrete approximation of the Euclidean disk as an elliptical structuring element.

### Indoor Localization and Autonomous Navigation

(Thrun et al. 2005) Thrun, Burgard, and Fox’s foundational textbook provides the theoretical framework for understanding the limitations of wheel odometry and the motivation for landmark-based localization correction.

(Mur-Artal and Tardós 2017) ORB-SLAM2, representing the state of the art for visual SLAM, was evaluated but not selected for the LEO Rover platform: its computational profile (2–4 CPU cores) would have saturated the laboratory workstation.

(Garrido-Jurado et al. 2014) Garrido-Jurado et al. introduced the ArUco fiducial marker system. The checkerboard landmark used in the present project is conceptually related: both rely on the Perspective-n-Point algorithm for pose estimation.

(Olson 2011) AprilTag provides improved robustness over ArUco under partial occlusion and at long distances. Chapter 13 identifies AprilTag as a potential upgrade path for the landmark detection system.

(Zhang 2000) Zhang’s planar calibration technique is the basis for all single-camera calibration in OpenCV, including the factory calibration of the D455’s intrinsic parameters used throughout Chapter 5.

(Ester et al. 1996) Ester et al.’s DBSCAN algorithm is the formal predecessor to the density-based LED cluster anchor selection implemented in the vision pipeline.

### Signal Processing and Estimation Theory

(Kalman 1960) Kalman’s 1960 paper establishes the theoretical framework within which the EMA filter can be understood: the steady-state Kalman gain equals the EMA smoothing factor α, providing a formal optimality interpretation.

(Wiener 1949) Wiener’s seminal work established the minimum mean-squared-error estimator for stationary processes, providing the comparison baseline used to quantify the EMA filter’s ∼29% suboptimality at the signal bandwidth edge.

(Xu and Li 2007) Xu and Li analyzed higher-order cascade EMA implementations achieving sharper frequency cutoffs, motivating a possible future 2nd-order EMA improvement for the LED centroid pipeline.

(Tukey 1977) Tukey’s foundational work on robust statistics provides the theoretical basis for the choice of the 5th percentile as the obstacle detection statistic, rather than the mean or minimum.

### Control Theory and Stability Analysis

(Ziegler and Nichols 1942) Ziegler and Nichols’ ultimate gain and ultimate period tuning method underlies the frequency-domain stability analysis of the LOCK state P-controller.

(Zames 1966) Zames’ Circle Criterion provides the frequency-domain sufficient condition applied to prove that the velocity saturation nonlinearity cannot destabilize a system stable in its linear operating region.

(Alur and Dill 1994) Alur and Dill’s timed automata formalism provides the mathematical framework for formally specifying and verifying the LEO Rover’s autonomous FSM with time-based guards and invariants.

### Safety Engineering and Standards

(International Electrotechnical Commission 2010) IEC 61508 is the foundational international standard for functional safety of programmable electronic systems; the SIL classification and six-layer defense-in-depth architecture both follow this standard’s recommendations.

(National Aeronautics and Space Administration 2013) The NASA Software Safety Standard defines the framework for classifying software hazardous functions; the heartbeat deadman switch addresses the highest-risk function identified under this framework.

(ISA 2016) ISA-18.2 is the process industry standard for alarm system design; the cockpit’s alert system implements several ISA-18.2 best practices including hysteresis bands and alarm muting.

(Sobczak 1962) The Fault Tree Analysis technique, developed for the Minuteman ICBM program, provides the top-down deductive methodology applied to derive the combined top-level failure probability.

### Human Factors and Interface Design

(Endsley 1988) Endsley’s three-level model of situation awareness is the foundational human factors reference directly applied to the cockpit’s telemetry panels, canvas overlays, and trajectory map.

(Endsley 1995) Endsley’s expanded theory identifies the factors that degrade situation awareness, motivating the alert system’s negative-attention design.

(Sanders and McCormick 1993) Sanders and McCormick’s textbook provides the empirical data on display luminance ratios that directly motivated the cockpit’s dark background design choice.

### Network Protocols and Web Engineering

(Fette and Melnikov 2011) RFC 6455 standardized the WebSocket protocol forming the communication backbone between the rosbridge server and the browser cockpit; the framing analysis follows this RFC’s frame structure.

(Bianchi 2000) Bianchi’s analytical model for IEEE 802.11 CSMA/CA performance underlies the queuing delay computation for the dedicated operator-to-robot Wi-Fi link.

(World Wide Web Consortium (W3C) 2018) WCAG 2.1 defines the contrast ratio requirements verified against the cockpit’s color palette.

(W3C CSS Working Group 2023) The W3C’s CSS Filter Effects specification defines the backdrop-filter property enabling the glassmorphic blur effect, and its compositing cost model explains the 12-panel layout limit.

### Robotics Platforms and Hardware

(Husarion 2021) The Husarion LEO Rover technical documentation provides the platform specifications cited throughout this report, including kinematics, encoders, battery, and firmware topic schemas.

(Intel Corporation 2021) The D455 datasheet provides the hardware specifications used in all sensor modeling sections: stereo baseline, depth range and accuracy, RGB resolution, and calibrated focal length.

(AgileX Robotics 2021) The AgileX LIMO Rover documentation was consulted during the Carolus comparative platform analysis contextualizing the LEO Rover work.

### Probability, Statistics, and Reliability Theory

(Kaplan and Meier 1958) The Kaplan–Meier survival function estimator provides the non-parametric framework appropriate for refining the safety layer failure rate estimates from operational field data.

(Shannon 1948) Shannon’s information theory provides the theoretical basis for modeling the heartbeat protocol as a binary channel with a computable channel capacity and error-correction buffer.

(Wilson 1927) Wilson’s score confidence interval for binomial proportions is used for computing the confidence interval of the LED and checkerboard detection rates.

### 3D Graphics and Web Visualization

(Cabello et al. 2021) Three.js r128 provides the WebGL-based 3D rendering infrastructure for the satellite and space capsule decorative elements in the cockpit.

(GreenSock (GSAP) 2021) GSAP’s ScrollTrigger plugin enables the scroll-linked rotation of the Three.js satellite model in the logbook panel.

## Index of Tables

Table [tab:table-index] lists all data tables appearing in this report, with their chapter location and a brief description of their content.

@L2.2cmL3.6cmL7.4cm@

Table & Chapter/Section & Title
Table & Chapter/Section & Title
[tab:intro-localization] & Introduction: I.4.2 & Comparative survey of indoor localization paradigms
1.1 & Introduction: I.5.1 & End-to-end operator latency budget
2.1 & Chapter 1: 1.2.2 & ROS Noetic vs. ROS 2 Humble feature comparison
3.1 & Chapter 2: 2.2 & leobackend.py daemon thread inventory
[tab:topic-graph] & Chapter 2: 2.3 & Complete ROS topic graph
[tab:constants-ref] & Chapter 2: 2.7 & Complete system constants reference
[tab:autolog-events] & Chapter 2: 2.8 & AutoLogbook event taxonomy
[tab:fsm-transitions] & Chapter 3: 3.4.1 & Complete FSM transition table
4.1 & Chapter 3: 3.7 & System performance before/after redesign
[tab:timing-jitter] & Chapter 3 Ext. 3.K.1 & Vision pipeline stage-by-stage timing analysis
6.1 & Chapter 4: 4.5.1 & FSM state banner color encoding
8.1 & Chapter 5: 5.1 & Six safety layer stack
9.1 & Chapter 5 Ext. 5.A.2 & Failure rate estimation per safety layer
[tab:vision-perf-measurements] & Chapter 6: 6.6 & Vision pipeline performance before/after
[tab:fsm-validation] & Chapter 6: 6.7 & FSM behavioral validation test results
10.1 & Chapter 6: 6.8 & Cockpit rendering performance, 3 hardware configs
11.1 & Chapter 6 Ext. 6.E.2 & CB detection rate/latency vs. scale factor
11.2 & Chapter 6 Ext. 6.F.1 & Full autonomous mission statistics (45 min)
13.1 & Chapter 7: 7.1 & Objective achievement summary
[tab:glossary] & Appendix A & Technical glossary (27 terms)
15.1 & Appendix B: B.1 & LEO Rover mechanical/electrical specifications
15.2 & Appendix B: B.2 & Intel RealSense D455 specifications
15.3 & Appendix B: B.3 & Operator workstation specifications

## Index of Code Listings

Table [tab:listing-index] catalogues the source code listings appearing in this report, with their section location, programming language, and a brief description.

@L2.6cmL1.4cmL1.8cmL7.4cm@

Listing & Section & Language & Description
Listing & Section & Language & Description
[lst:spawn-context] & 2.5.1 & Python & Spawn context declaration for the vision subprocess
[lst:queue-semantics] & 2.5.2 & Python & Frame/result queue configuration and producer logic
[lst:start-leo] & 2.6 & Bash & startleo.sh: deterministic ROSIP + idempotent launch
[lst:telemetry-schema] & 2.4 & Python & Selective telemetry JSON schema
[lst:morph-close] & 3.1.1 & Python & Morphological closing with elliptical SE
[lst:held-flag] & 3.2.2 & Python & Held-flag hysteresis mechanism
[lst:p-controller] & 3.4.2 & Python & LOCK-state visual servoing P-controller
[lst:yaw-wrap] & 3.4.3 & Python & Wrap-safe yaw delta accumulation
[lst:world-pos] & 3.5.2 & Python & World position computation
[lst:reset-continuity] & 3.5.3 & Python & Local odometry reset with world-frame continuity
[lst:percentile-obstacle] & 3.6.2 & Python & 5th-percentile obstacle detection statistic
[lst:color-tokens] & 4.1.2 & CSS & Color design-token system
[lst:glassmorphism] & 4.1.3 & CSS & Glassmorphic panel implementation
[lst:bento-grid] & 4.2 & CSS & Bento grid layout with responsive breakpoint
[lst:raf-loop] & 4.3.1 & JavaScript & requestAnimationFrame loop structure
[lst:dpr-handling] & 4.3.2 & JavaScript & Device pixel ratio-aware canvas resizing
[lst:coord-mapping] & 4.3.3 & JavaScript & Letterbox-aware coordinate mapping
[lst:lerp-smoothing] & 4.4 & JavaScript & LERP smoothing between telemetry updates
[lst:alert-config] & 4.6.1 & JavaScript & Alert threshold configuration
[lst:mute-persistence] & 4.6.2 & JavaScript & Alert mute persistence
[lst:js-ring-buffer] & 4.7.1 & JavaScript & O(1) ring buffer for trajectory rendering
[lst:outlier-rejection] & 4.7.2 & JavaScript & Trajectory outlier rejection
[lst:world-to-canvas] & 4.7.3 & JavaScript & World-frame to canvas-pixel transform
[lst:net-latency] & 4.8 & JavaScript & Rolling telemetry latency meter
[lst:satellite-scene] & 4.9.1 & JavaScript & Decorative satellite 3D scene geometry
[lst:gsap-scroll] & 4.9.2 & JavaScript & GSAP ScrollTrigger-driven rotation
[lst:i18n-system] & 4.10 & JavaScript & Flat key-value i18n dictionary system
[lst:safe-stop] & 5.2.1 & Python & Redundant emergency-stop publication
[lst:stop-pending] & 5.2.2 & JavaScript & Disconnection-proof emergency stop delivery
[lst:heartbeat-fsm] & 5.3.1 & Python & Three-state heartbeat deadman
[lst:manual-deadman] & 5.4 & Python & Manual-mode deadman timeout
[lst:camera-watchdog] & 5.5 & Python & Camera watchdog
[lst:velocity-clamp] & 5.6 & Python & Velocity clamping at every publish call
[lst:autologbook-write] & 5.7 & Python & Atomic, thread-safe logbook write
[lst:cs1-diagnosis] & 6.2.2 & Python & Diagnostic blob log (Case Study 1)
[lst:cs1-rootcause] & 6.2.3 & Python & Original buggy single-anchor clustering
[lst:cs1-fix] & 6.2.4 & Python & Exhaustive anchor evaluation fix
[lst:cs4-fix] & 6.5.3 & Bash & Deterministic ROSIP configuration
[lst:cs4-fallback] & 6.5.3 & Python & Fallback IP discovery via UDP routing table

## Mathematical Notation and Symbols

Table [tab:notation] defines the mathematical symbols and notation used consistently throughout the report.

@L2.6cmL7.6cmL2.2cm@

Symbol & Definition & First Used
Symbol & Definition & First Used
α & EMA smoothing factor (α=0.45 LED, α=0.60 landmark) & §3.3.1
αoptimal & Kalman-optimal EMA parameter derived from SNR & §6.C.2
A⊕B & Morphological dilation of set A by structuring element B & §3.B.4
A⊖B & Morphological erosion of set A by structuring element B & §3.B.4
A∘B & Morphological opening (erosion then dilation) & §3.B.4
A•B & Morphological closing (dilation then erosion) & §3.B.4
cx,cy & Camera principal point (optical axis intersection) [px] & §3.A.1
CR & WCAG color contrast ratio between foreground and background & §4.A.2
dest & Estimated beacon distance from thin-lens formula [m] & §I.4.3
δd & Total distance estimation uncertainty [m] & §3.F.2
ds & Stereo disparity [px] & §3.I.1
err_frac & Normalized bearing error in LOCK state, ∈[−1,+1] & §3.4.2
fc & Filter −3 dB cutoff frequency [Hz] & §3.3.3
fs & Sampling rate of control loop or vision pipeline [Hz] & §3.3.1
fx,fy & Camera focal lengths in pixels & §3.A.1
g∈SE(2) & Rigid body transformation element of SE(2) & §5.G.1
H(z) & Z-domain transfer function of EMA filter & §3.3.1
H(ejω) & Frequency response (DTFT) of EMA filter & §3.3.1
K & Camera intrinsic matrix (3×3) & §3.A.1
Kp & Proportional controller gain [rad/s per px] & §3.G.3
Ku & Ziegler–Nichols ultimate gain [rad/s per px] & §3.G.2
λ & Component failure rate [failures/hour] & §5.A.1
L1,L2 & CIE relative luminance of foreground/background & §4.A.2
μ(x,y) & Local mean intensity in adaptive threshold window & §3.B.2
μln,σln & Log-normal distribution parameters & §6.B.1
N(bi,r) & Density neighborhood of blob bi at radius r & §3.D.1
ω & Angular frequency [rad/s] (control) or [rad/sample] (signal) & §3.3.1
ωn & Natural frequency of closed-loop controller [rad/s] & §3.G.5
ωpc & Phase crossover frequency [rad/s] & §3.G.2
p & Observed detection rate (sample proportion) & §6.A.1
PFH & Probability of dangerous failure per hour (IEC 61508) & §5.C.1
PM & Phase margin of P-controller [degrees] & §3.G.4
r & Wheel radius of LEO Rover [m] & §3.H.1
R∈SO(2) & 2D rotation matrix (Special Orthogonal group) & §5.G.1
ρ & Traffic intensity (queuing theory), ρ=λ/μ & §4.D.2
spx & Horizontal pixel span of LED cluster [px] & §I.4.3
S(x,y) & Integral image (prefix sum) at pixel (x,y) & §3.B.3
SE(2) & Special Euclidean group in 2D (rigid body transforms) & §5.G.1
SIL & Safety Integrity Level (IEC 61508 classification) & §5.C.1
SNR & σw/σv: signal-to-noise standard deviation ratio (Kalman) & §6.C.2
σ(x,y) & Local standard deviation in adaptive threshold window & §3.B.2
σn & Measurement noise standard deviation [px] & §6.C.2
σw & Process noise standard deviation [px/sample] & §6.C.2
Td & Total vision pipeline delay [s] (plant model) & §3.G.1
T(x,y) & Adaptive threshold value at pixel (x,y) & §3.B.2
W & Physical width of LED beacon assembly [m] & §I.4.3
Wq & Mean queuing wait time (M/D/1 model) [s] & §4.D.2
(wx,wy) & Robot position in world frame [m] & §3.5.1
x[n],u[n] & EMA filter output and input sequences & §3.3.1
Z(wx,wy) & Stereo depth at pixel (wx,wy) [mm or m] & §3.I.1

# Reproduction guide for future students

This appendix is written for the next student inheriting the platform, and it assumes no prior knowledge of this project, of the LEO Rover, or of ROS. It is deliberately exhaustive: every command is given in full, with the directory it must be run from and the output you should see. Following it from top to bottom rebuilds the entire system from a bare machine and reproduces every measurement in this report.

Read §17.2 before typing anything. Ten minutes spent on the mental model there will save hours later, because the majority of newcomer confusion on this platform traces back to one question: which of the two computers is doing this?

Paths are given relative to the project root ~/TOUT. The target platform is ROS Noetic on Ubuntu 20.04, built with catkintools (catkin build, not catkinmake).

## The short version: what will cost you a week

Read this page now and return to it whenever something behaves strangely. Each entry below is a failure that consumed at least a day of this project, and each shares the same property: the symptom points at the wrong subsystem. They are listed in the order you are most likely to meet them.

@L6.3cmL6.6cmr@ What you will observe & What it actually is & Where
What you will observe & What it actually is & Where
The page loads perfectly, every button is dead & TLS port 9443 open but detached from the ROS graph after a master restart. Reload, or use port 9090 & §17.6.5
Neither manual nor autonomous driving responds & the serial link, not the logic. Both modes share /serialnode, sole subscriber of /cmdvel & §17.17.2
Estimators diverge, IMU “bugs”, trajectories jump & the Pi is at 86 ∘C and capped to 600 MHz, dropping IMU samples & §17.17.1
The pose is plausible but alternates between two values & a TF frame with two parents. Set baseframe to basefootprint & §17.17.10
The robot over- or under-rotates consistently & angularvelocitymultiplier, an actuation gain. Fix it before touching gyroscale & §17.17.11
Beacon positions look clean, smooth and repeatable & and are metrically meaningless: the intrinsics belong to a different resolution & §17.13.5
The stack restarts every couple of minutes & a PATH defect in the cron watchdog, not an instability & §17.17.3

Two habits follow from this table and are worth adopting before you need them. First, probe a topic that is supposed to be noisy: a silent topic proves nothing, and this project lost an evening to an idle /mission/log. Second, prefer the narrow intervention: cycling one node beats restarting a stack, and almost every time the surrounding infrastructure was blamed here, it was innocent.

## Step −1: the absolute ground zero

Everything past this point assumes ~/TOUT already exists on your machine and that you can already reach the robot over SSH. Neither is true on the first day, and earlier revisions of this appendix jumped straight to cd ~/TOUT without ever saying how it got there. This section is the missing zero.

### Getting the code

git clone <URL_OF_THIS_REPOSITORY> ~/TOUT
cd ~/TOUT

<URLOFTHISREPOSITORY> is a placeholder, not an omission: as of this revision the repository is private. Replace it with the SSH or HTTPS clone URL your lab gives you, in the form git@github.com:<account>/<repo>.git. Everything from §17.5 onward, including requirements.txt and tools/robotenv.sh, arrives with this one command; nothing in this guide requires a second download.

### Powering on and reaching the robot for the first time

What follows is verified project history, not a factory manual Husarion’s own documentation, https://docs.leorover.tech ((Husarion 2021)), is the authority on the physical unit: power button location, battery connector, and first-boot behaviour vary across LEO Rover hardware revisions, and no one on this project photographed or recorded that procedure. The credentials below are not copied from that manual either — they are what this project’s own earliest working notes (GUIDEDEMARRAGELEO.md, 2026-06-23, and the network log of 2026-07-20) record as having worked, before any of the later network changes in this appendix were made. If your unit is a different LEO Rover, its hotspot suffix and possibly its default password will differ. Confirm against Husarion’s documentation before assuming these are universal.

- Power on. Consult https://docs.leorover.tech for the switch location and charging state indicators on your specific unit; this is not something the present project verified independently. Wait for the boot sequence to settle, on the order of a minute.

- Join the robot’s own WiFi hotspot from your workstation. Its name has the form LeoRover-XXXX, a per-unit suffix; the unit used throughout this project answered to LeoRover-e138 (confirmed live, 2026-07-20). The WPA passphrase for this hotspot network is not recorded anywhere in this project’s history and is the one genuine gap this section cannot close: check the label on the robot chassis, the Husarion account the unit was purchased under, or ask whoever last handled the physical hardware.

- SSH in. Once joined, the robot answers at a fixed address on its own hotspot subnet:

- ssh pi@10.0.0.1

- The credential this project’s earliest notes record as working, before any rotation, was password raspberry for user pi — the Raspberry Pi OS default, consistent with an unmodified factory image. Change it immediately (passwd), and proceed to §17.6’s Step 4 to replace password auth with a key before doing anything else over this link.

- If the unit needs reflashing (corrupted SD card, or a bare replacement board), the base OS image and flashing instructions are Husarion’s to provide: https://docs.leorover.tech. This project never reflashed a unit and has no first-hand verification of that procedure to offer beyond the pointer.

Once §17.1.2 succeeds once, everything past it in this appendix, WireGuard, Cloudflare, the two-workspace build, applies identically whether you are on the hotspot, on lab WiFi, or on the tunnel: they all resolve to some ROBOTHOST, and tools/robotenv.sh (§17.6) is what stops that from mattering ever again.

## Start here: what this machine actually is

There are two computers, and they are not equals.

The robot carries a Raspberry Pi 4. It owns the sensors and the motors, and, this is the part that surprises people, it also runs the ROS master, the name server every ROS process must register with. The Pi is deliberately kept modest: it moves data, it does not compute trajectories.

The workstation (the lab PC) runs the two estimators, the web cockpit, and every analysis tool. It is a client of the robot’s master. If the robot is off, nothing works. If the PC is off, the robot still drives but nothing is estimating its position.

Three consequences to memorise now, because they explain most of §17.17:

- Every ROS command you type on the PC talks to a master on the robot, over WiFi. A command that “hangs” is usually a network symptom, not a software bug.

- The Pi is the bottleneck, and it runs hot. When it overheats it halves its own clock, and the first casualty is sensor data arriving on time. Estimators then fail in ways that look like estimator bugs and are not (§17.17.1).

- Nothing is ground truth. Wheel odometry, MINS and openVINS all estimate. They can only be compared to each other unless you bring a tape measure (§17.13.3).

### The hardware chain

Figure 17.1 traces the physical path from a wheel encoder to the workstation. Two links on it deserve attention, because both have caused real failures documented in this report.

The physical chain. Weak link 1⃝: the CORE2 board talks to the Pi over a GPIO serial port (/dev/ttyAMA0), not USB, a detail that misleads everyone, because unplugging USB cables cannot fix it. That link has no retransmission: if the Pi is too busy to empty the serial buffer in time, bytes are lost outright (§17.17.7). Weak link 2⃝: the WiFi hop adds latency and jitter. Measured on this install: 0% packet loss but round-trip times of 3 to 66 ms. ROS tolerates this because everything is timestamped at the source; the video preview does not, which is why the cockpit shows a high latency badge that is honest about the video and irrelevant to the trajectories.

### The software chain

ROS programs communicate by publishing named streams called topics. Figure 17.2 shows the ones that matter, left to right: raw sensors, then cleanup, then the two estimators, then a switch that decides which estimator the rest of the system believes.

The topic flow. Everything upstream of the estimators runs on the robot; everything from imusanitizer rightwards runs on the workstation. The switch is a ROS service, not a topic: calling /poseselector/setsource with true serves MINS, false serves openVINS.

Which machine runs what. rosnode list shows all of it mixed together, which is why this split is worth knowing by heart. This table is node-level; §17.3 carries the same split at file level, for every configuration file, script and launch file in the project.

| Machine | Started by | Main nodes |
| --- | --- | --- |
| Robot (Pi) | leo.service → rosmon | ROS master, serialnode, |
|  |  | firmwaremessageconverter, |
|  |  | realsense2camera, rplidarnode |
| Workstation | roslaunch leonavigation | minssubscribe, ovmsckf, |
|  | navigationmaster.launch | poseselector, imusanitizer, |
|  |  | wheelremap, leobackend, |
|  |  | image republish ×3 |
| Workstation | web/startweb.sh | serve.py (:8000), webvideoserver |
|  |  | (:8080), rosbridge (:9090/:9443), |
|  |  | cloudflared tunnel |

### Map of the repository

Everything lives under ~/TOUT. You will spend almost all of your time in four of these directories; the rest is history you can ignore until you need it.

~/TOUT/
|-- catkin_ws/                 <-- workspace 1: openVINS + our own packages
|   `-- src/
|       |-- open_vins/         upstream openVINS (ov_msckf)
|       |-- leo_navigation/    ** OUR CODE: nodes, launch files, configs **
|       |-- leo_autonomy/      beacon/patrol behaviours, AprilTag configs
|       `-- realsense-ros/     camera driver (built here, runs on the Pi)
|-- mins_ws/                   <-- workspace 2: MINS (separate build flags)
|   `-- src/MINS/
|-- carolus_ws/                <-- workspace 3: LED beacon detection
|-- tools/                     ** scripts you will use every day **
|-- web/                       the cockpit (HTML + serve.py + start_web.sh)
|-- data/trajectories/         recorded runs (.bag) and their CSV exports
|-- logs/                      every log the system writes
|-- docs/                      protocols (e.g. floor-marking calibration)
|-- report_latex/              this report
`-- leo_backend.py             ** the cockpit's brain (single large file) **

The four that matter: catkinws/src/leonavigation (all our ROS nodes), tools (all our scripts), web plus leobackend.py (the cockpit), and logs (where every answer hides).

The scripts in tools/ you will actually use. Every one has a long header comment explaining why it exists: read it before modifying the script.

| Script | What it is for |
| --- | --- |
| robotenv.sh | sets ROSMASTERURI/ROSIP; sourced by everything |
| restartstack.sh | clean restart of the PC-side stack |
| recordtrajectories.sh | records a run to a rosbag (the reliable way) |
| bagtocsv.py | rosbag → one CSV per estimator |
| plottrajectories.m | the nine-panel MINS vs openVINS figure |
| comparetests.m | Test 1 vs Test 2 (two separate drives) |
| calibterrain.py | ground-truth calibration (terminal version) |
| rosbagdaemon.sh | launched by leobackend.py to record a run to disk, independently of the browser. This is the capture path to use for anything that matters (§17.13.5); the cockpit’s own buffer is cleared by any backend restart |
| launchcarolus.sh | the beacon detector’s entry point, invoked by tools/carolusdetect.launch. Start the detector through the launch file, not by calling the script directly, or it inherits the wrong environment |
| leowatchdog.sh | the cron watchdog, read §17.17.3 first |
| updatereport.sh | compiles and publishes this report |
| checkserialhealth.sh | UART overrun counters (§17.17.7) |
| logposestatic.py | records /robotposefused to CSV while the rover sits still: Protocol T1.1 (§17.13.4) |
| analyzeposevariance.py | per-axis noise and linear drift rate from a T1.1 CSV; separates noise from unbounded divergence |

## Master file inventory: what lives where, and on which machine

The single most expensive confusion on this project is that rosnode list and rostopic list present two physically separate computers as one flat system. A node name tells you nothing about which machine runs it, a topic name tells you nothing about where it is published, and a file path that exists on one machine may not exist on the other at all. Every failure in §17.17 that cost more than an hour began with an assumption about where something was.

This section is the inventory that answers that question once. Every entry carries a machine marker:

- [PC] — the operator workstation (Ubuntu 20.04, eno1 or the USB Wi-Fi dongle). Paths are relative to the repository root, ~/TOUT/, unless written absolute.

- [ROBOT] — the Raspberry Pi 4 aboard the rover. Paths are absolute, exactly as you see them over ssh pi@$ROBOTHOST. These files are not in the repository: they live on the robot’s SD card and are lost on a reflash (§17.8).

Everything below was enumerated from the filesystem on 2026-09-11, not written from memory. Where a file’s runtime location differs from where its source is stored, both are given.

### Files that exist only on the robot

These are the reason a fresh SD card does not give you a working rover. None of them is in git; all three project-specific changes are described with their rationale in §17.8.

@L5.6cmcL6.4cm@ Path & Machine & What it is
Path & Machine & What it is
/etc/ros/robot.launch & [ROBOT] & The robot’s own bring-up launch. Holds the RealSense arguments, including the enablegyro/enableaccel defaults that leave the D455’s IMU switched off (item 45, §17.8.3)
/etc/ros/setup.bash & [ROBOT] & Pins ROSIP. This is the file behind the nastiest network failure in this report: it can advertise a dead WireGuard address while the master answers normally and every topic exists (§17.17)
/etc/systemd/system/leo.service
plus its LimitRTPRIO drop-in & [ROBOT] & Starts rosmon, which starts the ROS master and every robot-side node. The drop-in is Change 1 (§17.8.2); without both it and the chrt call, the serial node will not start at all
/opt/ros/noetic/share/leobringup/
launch/leobringup.launch & [ROBOT] & Stock Husarion bring-up, not ours. Replaced wholesale by a reflash
/dev/ttyAMA0 & [ROBOT] & The UART to the CORE2/LeoCore board. A GPIO serial line, not USB — the distinction took a day to establish and is the whole subject of §17.8.2
hostapd / dnsmasq configuration & [ROBOT] & Serve the rover’s own access point, SSID LeoRover-e138 at 10.0.0.1. This is the fallback path when the campus network refuses the robot, and the only address that works regardless (§17.17)

### The nodes this project wrote

All sources live in the repository on the workstation. Read the Runs on column carefully: one of them is stored here but executes on the robot, because the hardware it talks to is the robot’s.

@L5.4cmcL6.6cm@ Source file (in repo, [PC]) & Runs on & Role
Source file (in repo, [PC]) & Runs on & Role
catkinws/src/leonavigation/
scripts/imusanitizer.py & [PC] & Despikes and rescales the raw IMU; publishes /imu/dataclean, which is what MINS consumes. §17.9.1
.../scripts/wheelremap.py & [PC] & Wheel odometry remapping. §17.9.2
.../scripts/poseselector.py & [PC] & The estimator switch; publishes /robotposefused, the one topic the rest of the system reads. §17.9.3
.../scripts/carolusviconbridge.py & [PC] & The only absolute reference into the filter. §17.9.4
.../scripts/carolustfbridge.py & [PC] & TF plumbing for the Carolus/AprilTag frame
.../scripts/vinsrotationguard.py & [PC] & Written, built, deliberately not launched. §17.9.6
.../scripts/pihealthmonitor.py & [PC] & Thermal and load telemetry from the Pi, read over the network
.../scripts/gotopose.py & [PC] & Pose-regulation motion control (§17.19)
.../scripts/fillminscamchain.py & [PC] & Calibration helper: writes Kalibr output into the MINS camera config
.../src/minimu9node.cpp
branch feature/minimu9-driver & [ROBOT] & I​2C driver for the replacement IMU. Must run on the robot — the I​2C bus belongs to the Pi — which is why it is deliberately absent from navigationsupervision.launch, a workstation launch file. Not on the main branch; see the handoff note in docs/HANDOFFNEXTSTUDENT.md
leobackend.py (repository root) & [PC] & The cockpit’s brain: FSM, telemetry, vision subprocess. §17.9.5

### Launch files, and what each one starts where

All five live in catkinws/src/leonavigation/launch/ on the workstation, and all five are run from the workstation. They start workstation nodes only: the robot’s nodes are started by leo.service on the robot itself, independently, and are already running before you launch anything here.

@L4.6cmL9.0cm@ File ([PC]) & What it brings up
File ([PC]) & What it brings up
navigationmaster.launch & The top-level entry point. Includes the three below. This is the one tools/restartstack.sh calls
mins.launch & MINS. Note the configuration path is passed through args="..." and not <param name="configpath">: on ROS1 the node reads argv[1] only, and passing it the other way produces “unable to open the configuration file” (item 42)
navigationsupervision.launch & imusanitizer, wheelremap, poseselector, the image republish nodes. Also carries the imusource argument (core2 ↔ minimu9) that switches the sanitizer’s input topic and accelerometer scale together
poseselector.launch & The selector alone, for restarting it without the rest — required after any estimator reset (item 48)
tfstaticextrinsics.launch & The static transform tree

### Where each estimator actually reads its configuration

All estimator configuration is on the workstation: the estimators themselves run there (§17.16 explains why MINS is not on the Pi).

@L5.0cmL8.6cm@ Estimator ([PC]) & Configuration path
Estimator ([PC]) & Configuration path
MINS & LEORoverNavigationSystem/
MINS-master/mins/config/leo/
Ten files: config.yaml is the entry point and names the other nine (configimu, configcamera, configwheel, configestimator, configinit, configsystem, configvicon, configlidar, configgps)
MINS, camera-only profile & .../mins/config/leovio/ — the same platform with the wheels removed, used to isolate what the wheels contribute
MINS, Pi profile & .../mins/config/leopi/ — mono, downsampled, calibration off. Kept for reproducibility of the embedded-feasibility arithmetic; never adopted (item 43)
openVINS & catkinws/src/openvins/config/leo/
estimatorconfig.yaml plus kalibrimucamchain.yaml and kalibrimuchain.yaml

$(find mins) does not resolve to the path you are reading mins.launch loads $(find mins)/config/$(arg config)/config.yaml, which resolves through the ROS package path, not through the directory you happen to be editing. On this workstation minsws/src/MINS is a symbolic link to /home/lab272/TOUT/LEORoverNavigationSystem/MINS-master (verified 2026-09-11). There is therefore exactly one copy of the MINS configuration on disk, reachable by two different-looking paths.

This matters in both directions. Editing either path edits the same file, so a change you think you made in a scratch copy is live. And if a future student replaces that symlink with a real second checkout, the two paths become independent, the launch file will keep loading the one you are not editing, and the filter will silently run yesterday’s calibration. Check with readlink -f minsws/src/MINS before trusting either.

### Tooling, by the question it answers

tools/ holds roughly sixty scripts [PC]; every one carries a header comment explaining why it exists, and that header is the authority, not this table. What follows is the subset a new arrival needs in the first week, grouped by the question being asked. Several of these operate on the robot over SSH while running from the workstation — that is the normal pattern here, and the Acts on column says which.

@L4.8cmcL7.2cm@ Script (in tools/, [PC]) & Acts on & Question it answers
Script (in tools/, [PC]) & Acts on & Question it answers
robotenv.sh & [PC] & Where is the robot? Single source of truth for ROSMASTERURI and ROSIP, overridable through ~/.leonetwork. Sourced by every other script
preflightmins.py & both & Is the whole chain healthy enough to run an experiment? GO/NO-GO in one command. Run it before any test that costs robot time
restartstack.sh & [PC] & Restart the workstation navigation stack. Mandatory ordering after any estimator reset: sanitizer → MINS → poseselector (item 48)
recordtrajectories.sh & [PC] & Capture an experiment properly. Writes a rosbag to disk, independent of the web cockpit — which is not reliable for a run that counts
comparetests.m,
plottrajectories.m & [PC] & What did the run show? MATLAB comparison, accepts .mat or CSV
diagreseaurobot.sh & both & Why can I not reach the robot?
recupererrobothotspot.sh & both & The robot is powered but unreachable: switches the workstation onto the rover’s own access point, diagnoses over SSH, and restores the campus Wi-Fi afterwards
checkuartoverruns.sh,
checkserialhealth.sh & [ROBOT] & Is the serial link losing bytes? Reads the Pi’s own UART counters over SSH (§17.8.2)
verifminimu9.py & [ROBOT] & Is the replacement IMU wired and sane? Control and pre-calibration of the MinIMU-9, to be run once the sensor is physically installed
launchsqrtvinspi.sh,
sqrtvinspictl.sh & [ROBOT] & VINS runs on the Pi, on demand, and not as part of the workstation stack (§12.20)
leowatchdog.sh & [PC] & The self-healing cron watchdog. Suspend it with touch /tmp/leomaintenance before any manual work, and remember to remove that file afterwards
updatereport.sh & [PC] & Rebuild and publish this document. Note it prepends the TeX Live 2026 installation in ~/texlive/2026/ to PATH: the system pdflatex is a Debian TeX Live 2019 that lacks packages this report uses, so a build run without that PATH fails on IEEEtrantools.sty

### Cockpit, data, and documentation

@L4.4cmcL7.6cm@ Path & Machine & What it is
Path & Machine & What it is
web/ops.html & [PC] & The control cockpit: driving, AUTO, pose source, export
web/trajectory.html & [PC] & Live map, batched Canvas rendering
web/demo.html & [PC] & Read-only by construction — no publisher, no command binding. For demonstrations
web/logbook.html + autoentries.json & [PC] & Mission log, written automatically by the backend
web/serve.py & [PC] & Static server. Its filter mirrors §10 of the .gitignore: what git refuses to archive, the server refuses to publish — added after an audit found exports/ browsable on the public internet
web/startweb.sh & [PC] & Starts serve.py, webvideoserver, rosbridge and the Cloudflare tunnel
data/trajectories/ & [PC] & Exported runs (CSV, PNG) and the rosbags. Not versioned: the raw recordings exceed what a git repository should carry
reportlatex/ & [PC] & This document’s sources, and main.pdf
JOURNALDEBORD.md & [PC] & Dated version history of the whole campaign. The first place to look when you need to know when something changed
docs/HANDOFFNEXTSTUDENT.md & [PC] & Prioritised list of what to work on next, written at handover
archives/ & [PC] & Out of repository. Timestamped backups, received documents, deliveries, figures. Nothing was deleted into it and nothing referenced was moved there

A file in this repository is not necessarily code that runs here Three categories coexist in the same tree, and telling them apart saves a great deal of time:

- Code that runs on the workstation — the great majority.

- Code that is stored here but executes on the robot: minimu9node.cpp, the VINS launchers, and every script in the Acts on: ROBOT rows above. Copy or deploy them to the Pi; running them here does nothing useful, and in the I​2C case will simply find no bus.

- Upstream code that is not ours at all — openVINS, MINS, Kalibr, realsense-ros (§17.4). Patches we made to these are kept separately in patches/, patchesopenvins/ and patchesmins/ precisely so that the distinction survives a re-clone.

## Upstream sources: what is ours and what is not

For completeness, the provenance of every component and the workspace it lives in.

Four upstream stacks are used, each in its own catkin workspace so their build flags and dependencies stay isolated (Kalibr’s workspace, kalibrws, is separate from the three runtime workspaces of §17.5 because it is a calibration-time tool, not something that runs alongside the estimators):

@L3.2cmL5.6cmL4.6cm@ Component & Upstream & Local workspace
openVINS (ovmsckf) & https://github.com/rpng/open_vins & catkinws/src/openvins
MINS & https://github.com/rpng/MINS & minsws/src/MINS
Carolus beacon & https://github.com/albator63/carolus_ws & carolusws
Kalibr (calibration) & https://github.com/ethz-asl/kalibr & kalibrws/src/kalibr

The project-specific ROS packages (leonavigation, leoautonomy, the poseselector, wheelremap and carolusviconbridge nodes) live alongside openVINS in catkinws/src/ and are versioned with the project, not upstream.

## Building the workstation from a bare machine

Why in this order: ROS must exist before catkin can build against it; system libraries must exist before the estimators compile; and the two estimator workspaces must be built separately because they disagree about build flags.

### Step 0: Python dependencies

Do this before anything else; several failures later in this guide are really a missing module reported three layers away from the import.

cd ~/TOUT
pip install -r requirements.txt
sudo apt install python3-tk        # tkinter, not pip-installable

requirements.txt lists what each package is for, with the versions this project was verified against. Two entries are easy to overlook: python3-tk, which leodashboard.py and leotrackingmap.py need and which pip cannot supply, and esprima, which is not a runtime dependency at all but the JavaScript parser used to validate web/app.js before reloading it (§17.12.5). ROS packages such as rospy, tf and cvbridge come from ROS Noetic and must not be installed with pip.

### Step 1: operating system and ROS

- Install Ubuntu 20.04 LTS. Not 22.04: ROS Noetic is not packaged for it, and every workaround costs more time than reinstalling.

- Install ROS Noetic Desktop-Full, following the official instructions. Summarised:

- sudo sh -c 'echo "deb http://packages.ros.org/ros/ubuntu $(lsb_release -sc) main" \
  > /etc/apt/sources.list.d/ros-latest.list'
sudo apt install curl
curl -s https://raw.githubusercontent.com/ros/rosdistro/master/ros.asc | sudo apt-key add -
sudo apt update
sudo apt install ros-noetic-desktop-full

- Make ROS available in every new terminal:

- echo "source /opt/ros/noetic/setup.bash" >> ~/.bashrc
source ~/.bashrc

- Verify: echo $ROSDISTRO prints noetic.

- Install the build tool and the dependency resolver:

- sudo apt install python3-catkin-tools python3-rosdep python3-osrf-pycommon
sudo rosdep init && rosdep update

### Step 2: system libraries the estimators need

Both openVINS and MINS are C++ and need Eigen, Ceres, OpenCV 4 and Boost. rosdep finds most of them; Ceres is the one that usually needs help.

sudo apt install libeigen3-dev libboost-all-dev libopencv-dev \
                 libceres-dev libsuitesparse-dev
# ROS-side extras used by the cockpit and the pipeline:
sudo apt install ros-noetic-rosbridge-suite ros-noetic-web-video-server \
                 ros-noetic-image-transport-plugins ros-noetic-topic-tools \
                 ros-noetic-rosmon ros-noetic-apriltag-ros
# analysis tools:
sudo apt install python3-pip texlive-full
pip3 install --user numpy pyyaml psutil pillow

Note: texlive-full is ∼5 GB but avoids a long tail of missing-package errors when compiling this report. psutil is what lets the cockpit show CPU load; without it the cockpit still works but that panel stays empty.

### Step 3: the three workspaces

Why three: openVINS and MINS come from the same research group but do not build with the same flags, and mixing them in one workspace produces link errors that look like source bugs. The beacon stack is separate again because it is optional.

- Create the tree and clone the upstream sources:

- mkdir -p ~/TOUT/catkin_ws/src ~/TOUT/mins_ws/src ~/TOUT/carolus_ws/src
cd ~/TOUT/catkin_ws/src
git clone https://github.com/rpng/open_vins.git
git clone https://github.com/IntelRealSense/realsense-ros.git -b ros1-legacy
cd ~/TOUT/mins_ws/src
git clone https://github.com/rpng/MINS.git

- Our packages arrive with the clone of §17.1.1, already in place at ~/TOUT/catkinws/src/{leonavigation,leoautonomy} — there is nothing further to copy. They contain every node described in §17.9. Note on the QR scripts: qrleocontrol.py, qrsearchmove.py and diagqr.py are not a catkin package despite the name similarity; they are standalone Python scripts at the repository root, run directly with python3, not through roslaunch or rosrun. An earlier revision of this appendix referred to a leoqrcontrol package that does not exist in the versioned tree; if you were looking for it, this is the correction.

- Resolve dependencies, once per workspace:

- cd ~/TOUT/catkin_ws && rosdep install --from-paths src --ignore-src -r -y
cd ~/TOUT/mins_ws   && rosdep install --from-paths src --ignore-src -r -y

- Build workspace 1 (openVINS + our packages):

- cd ~/TOUT/catkin_ws
catkin build

- Expected: a progress table ending with a green summary, [build] Summary: All ... packages succeeded!. On a four-core machine budget 15–25 minutes. If ovmsckf fails on Ceres, install libceres-dev and rebuild only that package with catkin build ovmsckf.

- Build workspace 2 (MINS):

- cd ~/TOUT/mins_ws
catkin build mins

- Build workspace 3 (beacon, optional):

- cd ~/TOUT/carolus_ws && catkin build

### Step 4: the sourcing trap, and why a wrapper exists

This is the single most confusing thing about the build. A devel/setup.bash replaces several environment variables rather than appending to them. Source catkinws then minsws, and roslaunch can no longer find leonavigation; source them the other way round and it cannot find mins. Either way the error is cannot locate node of type [...] in package [...], which reads like a missing file.

The fix is catkinws/src/leonavigation/launchmins.sh: it saves the five variables that matter, sources the second workspace, then re-appends the saved values so both are reachable.

source /home/lab272/TOUT/tools/robot_env.sh
source /opt/ros/noetic/setup.bash
source /home/lab272/TOUT/catkin_ws/devel/setup.bash
_RPP="$ROS_PACKAGE_PATH"; _CMPP="$CMAKE_PREFIX_PATH"; _PATH="$PATH"
_LDLP="$LD_LIBRARY_PATH"; _PYP="$PYTHONPATH"
source /home/lab272/TOUT/mins_ws/devel/setup.bash
export ROS_PACKAGE_PATH="$ROS_PACKAGE_PATH:$_RPP"
export CMAKE_PREFIX_PATH="$CMAKE_PREFIX_PATH:$_CMPP"
export PATH="$PATH:$_PATH"
export LD_LIBRARY_PATH="$LD_LIBRARY_PATH:$_LDLP"
export PYTHONPATH="$PYTHONPATH:$_PYP"
exec roslaunch leo_navigation "${1:-mins.launch}"

Rule: never start anything MINS-side with a plain roslaunch. Always:

~/TOUT/catkin_ws/src/leo_navigation/launch_mins.sh navigation_master.launch

tools/restartstack.sh does this for you, which is one reason to launch through it rather than by hand.

### Quick reference: the clone-and-build commands

Clone each stack into its workspace’s src/, then build. openVINS and the project packages share catkinws:

# --- openVINS + project packages (catkin_ws) ---
cd ~/TOUT/catkin_ws/src
git clone https://github.com/rpng/open_vins.git
# (leo_navigation, leo_autonomy, kalibr already present in src/)
cd ~/TOUT/catkin_ws
catkin build ov_msckf          # builds openVINS and its deps
source devel/setup.bash

# --- MINS (separate workspace: its own build flags) ---
cd ~/TOUT/mins_ws/src
git clone https://github.com/rpng/MINS.git
cd ~/TOUT/mins_ws
catkin build mins
source devel/setup.bash

# --- Carolus beacon detection ---
cd ~/TOUT/carolus_ws
catkin build

System dependencies (Eigen, Ceres, OpenCV 4, Boost) are the standard openVINS/MINS prerequisites; on a fresh machine install them with rosdep install --from-paths src --ignore-src -r -y run once per workspace before the first catkin build. openVINS and MINS both compile against tf2ros on this platform (the legacy tf migration of §12.9.4 for MINS, and the equivalent open item for openVINS in Table [tab:openitems]). The MINS workspace cannot simply be sourced back-to-back with catkinws; the tools/launchmins.sh helper unions the two environments correctly and is the supported way to launch anything MINS-side (see its header comment for why).

## Network configuration

Why this matters: ROS has no auto-discovery. Each process must be told where the master lives (ROSMASTERURI) and which address to advertise so others can call it back (ROSIP). Get ROSIP wrong and you get the most misleading symptom on the platform: rosnode list shows every node, and no data ever arrives.

### Step 1: one file holds the robot’s address

The robot’s address changes with the network: 10.0.0.1 on the robot’s own hotspot, a DHCP address on campus WiFi, 10.200.0.2 over the WireGuard tunnel used at the time of writing. Never hard-code it: it used to be duplicated in five scripts, and changing networks meant five edits and a guaranteed omission.

echo 'LEO_ROBOT_HOST=10.200.0.2' > ~/.leo_network

Deleting the file restores the default (10.0.0.1). Every script reads tools/robotenv.sh, which reads this file.

### Step 2: what robot_env.sh derives, and why

ROSIP is not hard-coded either: it is looked up from the routing table, so it stays correct on a machine with both Ethernet and WiFi. The address that matters is the one on the interface that can actually reach the robot.

export ROBOT_HOST="${LEO_ROBOT_HOST:-10.0.0.1}"
export ROS_MASTER_URI="http://${ROBOT_HOST}:11311"
_leo_ros_ip="$(ip route get "$ROBOT_HOST" | grep -oP 'src \K[0-9.]+' | head -1)"
export ROS_IP="${_leo_ros_ip:-$(hostname -I | awk '{print $1}')}"

Verify, from any directory:

cd ~/TOUT
source tools/robot_env.sh
echo "$ROBOT_HOST | $ROS_MASTER_URI | $ROS_IP"

Expected: 10.200.0.2 | http://10.200.0.2:11311 | 10.200.0.1.
If ROSIP is empty or on the wrong subnet, stop: fix the network first, nothing downstream will work.

### Step 3: the WireGuard tunnel (optional but recommended)

Campus WiFi hands out changing addresses and sometimes isolates clients from each other. A WireGuard tunnel gives the pair two fixed addresses (10.200.0.1 for the PC, 10.200.0.2 for the robot) regardless of the underlying network, which removes an entire class of “it worked yesterday” failures.

##### Decide which machine is reachable first.

WireGuard has no client and server roles in the protocol, but it does in practice: one side must carry an Endpoint, and that side must be reachable from the other. This is the question that decides whether the tunnel comes up at all, and it is decided by your network, not by preference.

@L4.6cmL4.4cmL4.4cm@ Situation & Who carries Endpoint & Works?
The PC has a fixed, routable address (wired lab network, port 51820/udp open) & the robot points at the PC & yes, the arrangement used here
Both sides behind campus NAT, no inbound ports & neither can reach the other & no; you need a third host
A cheap VPS with a public IP & both point at the VPS & yes, and it survives either side changing network

Campus networks routinely block inbound connections and isolate wireless clients from one another. If the PC is itself on campus WiFi, expect the second row: no amount of configuration on the two machines will fix it, and the honest answer is a VPS acting as a rendezvous point, with both rover and workstation as clients of it.

- sudo apt install wireguard on both machines.

- Generate a key pair on each:

- wg genkey | tee private.key | wg pubkey > public.key

- Write /etc/wireguard/wg0.conf on each side. Substitute the four placeholders; everything else is literal.

- Workstation (the side carrying the Endpoint in this deployment, i.e. the one that listens):

- [Interface]
Address    = 10.200.0.1/24
PrivateKey = <PC_PRIVATE_KEY>
ListenPort = 51820

[Peer]                              # the robot
PublicKey  = <ROBOT_PUBLIC_KEY>
AllowedIPs = 10.200.0.2/32

- Robot:

- [Interface]
Address    = 10.200.0.2/24
PrivateKey = <ROBOT_PRIVATE_KEY>

[Peer]                              # the workstation
PublicKey  = <PC_PUBLIC_KEY>
AllowedIPs = 10.200.0.0/24
Endpoint   = <PC_REACHABLE_IP>:51820
PersistentKeepalive = 25

- Enable on both: sudo systemctl enable --now wg-quick@wg0.

- Verify: ping -c 5 10.200.0.2 from the PC gives 0% packet loss, and sudo wg show lists a recent handshake. Then point the project at it: echo ’LEOROBOTHOST=10.200.0.2’ > ~/.leonetwork.

PersistentKeepalive is not a roaming fix It is frequently described as one, including in earlier revisions of this appendix, and the claim does not survive contact with this platform. Keepalive stops a NAT mapping from expiring while nothing is being sent. It does nothing when the robot’s radio actually re-associates and its underlying address changes: the peer’s cached endpoint is then stale until a packet from the new address re-teaches it.

Registry item 12 records the reality. One WireGuard outage was explained and fixed by endpoint roaming; a second resisted restarts on both ends, a port change, and a four-minute idle wait, while ICMP and SSH to the robot stayed healthy throughout. That item is still open. Treat the tunnel as usually reliable and occasionally not, and when it floats, check sudo wg show for the age of the last handshake before restarting anything else.

Known quirk: on this workstation the USB WiFi dongle (rtl88x2bu) drops packets when its power-saving kicks in. It affects the PC only, not the robot. First find your own interface name, which will not be the one below:

ip -br link | grep -E '^wl'        # or: iwconfig 2>/dev/null | grep -B1 IEEE

Then disable power saving on it. This does not survive a reboot:

sudo iw dev <WIFI_INTERFACE> set power_save off
# on the machine used here that name was wlxcc641aee985b

### Step 4: passwordless SSH to the robot

Several scripts read robot state over SSH. Set up a key so they do not stall waiting for a password:

ssh-keygen -t ed25519           # accept the defaults
ssh-copy-id pi@10.200.0.2       # password once; never again
ssh pi@10.200.0.2 'hostname'    # verify

Expected: the robot’s hostname, with no password prompt.

### Step 5: the web transport, and the two ports it can use

The cockpit is an ordinary static web page. It has no server-side logic and no ROS libraries of its own: everything it knows about the robot arrives over a single WebSocket to rosbridge, which translates JSON into ROS messages and back. Three processes therefore have to be alive, and they are independent of one another:

@llL5.6cm@ Process & Port & Role
web/serve.py & 8000 & serves the HTML, CSS and JavaScript
rosbridgewebsocket & 9090 & plain WebSocket, LAN and loopback
tools/rosbridgewebsockettunnel.py & 9443 & TLS WebSocket, behind cloudflared

The page itself is served on 8000 in both cases. What changes between the two transports is only where the WebSocket goes, and the choice is made automatically from the hostname you typed:

const TUNNEL_ROOT = 'leo-rover-gardon.dev';
const isTunnel = () => location.hostname === TUNNEL_ROOT
                    || location.hostname.endsWith('.' + TUNNEL_ROOT);
const wsURL = (host) => isTunnel()
      ? ('wss://' + location.host + '/rosbridge')   // -> cloudflared -> :9443
      : ('ws://'  + host + ':9090');                // -> direct

This single function is the whole of the routing logic, and it has a consequence worth internalising before you debug anything:

The two paths share nothing but the ROS master. Different process, different port, different TLS state, different failure modes. When one is broken the other is very often fine, and that is the single most useful fact in this subsection.

#### The zombie port

Symptom: the page loads perfectly and every button is dead Restart the ROS master while the TLS bridge on 9443 is running, and that bridge keeps its listening socket open. It still accepts the TCP connection, still completes the TLS negotiation, still completes the WebSocket upgrade. What it no longer holds are live handles to the ROS graph, so nothing it receives is ever published and nothing from the graph ever reaches you.

The browser has no way to notice. At every layer it can observe, the connection succeeded. The page therefore renders its “connected” state truthfully and the interface looks entirely normal while being inert.

Since 2026-07-31 this is detected automatically. tools/leowatchdog.sh carries a drowning probe: it does not ask whether the port is bound, which in this failure mode it always is, but opens a real WebSocket handshake and requires an answer within eight seconds. Until July 31 that probe covered only port 9090, so the one failure mode it was designed to catch was being observed on the port it did not watch. It now probes both, and purges the pair on either failure, after which startweb.sh restarts them idempotently.

If you ever extend this probe, mind the TLS Port 9443 is served over TLS (listenSSL in rosbridgewebsockettunnel.py). Copying the plain-text 9090 probe onto it produces a probe that always fails, so the watchdog would purge both bridges every single minute: the exact restart storm of §17.17.3. Verified on 2026-07-31, a plain probe against 9443 raises AssertionError while a TLS-wrapped one returns HTTP/1.1 400. A 400 is a perfectly good result here, because the probe tests whether the process still answers, not whether it accepts the request.

Manual remediation, in increasing order of cost, for the window before the watchdog’s next tick:

- Force-reload the page (Ctrl+Shift+R). This tears down the WebSocket and builds a new one. If the bridge process itself recovered, this is enough, and it costs two seconds.

- Change transport. Open http://localhost:8000/ops.html instead of the tunnel domain. By the rule above this switches you to ws://…:9090, a different process that never went through the restart. Note carefully that this is not a different page and not a fallback mode: it is the same cockpit reaching the same master by an independent route. If the cockpit works here, the fault is in the tunnel path and nowhere else.

- Restart the TLS bridge only once the two steps above have failed, since it is the only one that interrupts anything.

Diagnose before you restart. The two commands below separate “the page is broken” from “the bus is quiet”, and the order matters:

# 1. Does ROS traffic exist at all? Use a topic that is ALWAYS busy.
rostopic hz /mission/telemetry      # expect ~4 Hz, continuously
# 2. Does a command sent from the page reach the bus?
rostopic echo /mission/command      # press a cockpit button and watch
# 3. Are both bridges actually listening?
ss -ltnp | grep -E '9090|9443|8000'

The choice of topic in the first command is not incidental. During the July 30 session the relay was declared broken on the evidence of a silent /mission/log, which publishes only when something happens; the empty output proved nothing at all. Repeating the test on /mission/telemetry showed fifty frames in twelve seconds and the relay was healthy throughout. A silent topic is not a dead link. Always probe something that is supposed to be noisy.

#### Two further transport traps, for completeness

- Opening the page as file:// disables both rosbridge and the video stream, because the browser refuses cross-origin WebSockets from a file URL. The cockpit detects this and prints a red banner telling you to use http://<PC-IP>:8000/ops.html. If you see that banner, nothing else you observe about connectivity means anything.

### Step 6: the Cloudflare tunnel, and why you must rebuild it

Everything above gets you a cockpit on the local network. The public address, cockpit.leo-rover-gardon.dev, is served by a Cloudflare tunnel, and it is the one piece of this system you cannot inherit.

The existing tunnel dies with its owner’s account The tunnel in service at the time of writing is bound to a personal Cloudflare account and to a domain registered under it. Its credentials live in ~/.cloudflared/ on the workstation, outside the project tree and outside version control, and they cannot be transferred by copying a file. You will have to create your own tunnel, on your own account, and repoint DNS at it. Budget an hour. Nothing else in this appendix depends on it: the cockpit works fully over http://localhost:8000 without any tunnel at all.

##### What the tunnel actually does.

It terminates TLS at Cloudflare’s edge and forwards three different paths of one hostname to three local services. That routing is the missing link between the JavaScript of §17.6.5, which builds wss://cockpit.…/rosbridge, and the TLS bridge listening on 9443. Without the ingress block below, that URL resolves to nothing and every button in the cockpit is inert with no error message.

##### Setup, from nothing.

- Install the binary (a .deb from Cloudflare, or your package manager), then authenticate. This opens a browser and writes ~/.cloudflared/cert.pem:

- cloudflared tunnel login

- Create the tunnel. This prints a UUID and writes the credentials file ~/.cloudflared/<UUID>.json, which is the actual secret:

- cloudflared tunnel create leo-cockpit

- Point a DNS name at it, on a domain your account controls:

- cloudflared tunnel route dns leo-cockpit cockpit.example.org

- Write ~/.cloudflared/config.yml. This is the file that matters, reproduced with the real routing and only the identifiers replaced:

- tunnel: <TUNNEL_UUID>
credentials-file: /home/<user>/.cloudflared/<TUNNEL_UUID>.json

ingress:
  # rosbridge and video are served on the SAME hostname as the page, on
  # dedicated paths, so they share the Cloudflare Access cookie of the
  # already-authenticated page. A separate subdomain would break the
  # WebSocket handshake on cookie scope.
  - hostname: cockpit.example.org
    path: ^/rosbridge/?$
    service: https://localhost:9443
    originRequest:
      noTLSVerify: true    # self-signed cert, loopback hop only
  - hostname: cockpit.example.org
    path: ^/(stream|stream_viewer)$
    service: http://localhost:8080
  - hostname: cockpit.example.org
    service: http://localhost:8000
  - service: http_status:404

- Run it. web/startweb.sh already does this for you, with an absolute path because a bare cloudflared fails silently under cron:

- cloudflared tunnel run

##### Three details that are load-bearing.

- service: https://localhost:9443, not http. The TLS bridge speaks TLS on the loopback hop as well.

- noTLSVerify: true is required because that bridge presents a self-signed certificate. It is safe here and only here, because the hop it applies to never leaves the machine.

- The order of ingress rules is significant. Cloudflare takes the first match, so the catch-all service: http://localhost:8000 must come last, and httpstatus:404 last of all.

##### Verify.

pgrep -x cloudflared                            # the process must exist
curl -sI https://cockpit.example.org/ops.html   # expect HTTP/2 200
tail -20 ~/TOUT/logs/cloudflared.log            # look for "Registered tunnel"

## Building the operator website, step by step

The cockpit is described elsewhere in this report by topic: its architecture and data flow in §7.8, its public API in §12.6, its transport and ports in §17.6.5, the Cloudflare tunnel in §17.6.6. web/READMEWEB.md in the repository covers the same ground for a maintainer.

None of those is a build order. This section is: fourteen steps from an empty directory to a public, self-healing cockpit, each one naming the exact files it creates or touches, so that a successor can follow the same path — or find the one file responsible for the part that is misbehaving.

The entire website runs on the workstation Not one line of the cockpit runs on the robot. Every file named in this section is [PC] in the sense of §17.3. The robot publishes ROS topics and nothing else; it does not serve HTTP, does not hold the pages, and is not reachable from a browser. The robot’s own rosbridge is in fact deliberately disabled (§17.8.4) so that it cannot compete with the workstation’s.

This matters when something breaks: a cockpit fault is almost never a robot fault, and restarting the rover to fix a blank page is a reflex worth unlearning.

### Step 1 — Choose an architecture that needs no server code

The decision that shapes everything else: the browser talks to ROS directly, over a WebSocket, using the rosbridge protocol v2.0. There is no application server, no REST API, no database, and no robotics code written for the web at all. A page opens a socket, subscribes to topics, publishes to topics, and that is the whole contract.

The consequence worth stating plainly is that the pages are static files. They can be copied, opened from disk, and served by anything. What makes them a cockpit is vendor/roslib.min.js plus the address of a running rosbridge.

### Step 2 — Stand up the transport

@L5.3cmL8.3cm@ File & Role
File & Role
web/startweb.sh & Starts and supervises every web-side service. Read its header before changing anything: it is idempotent by design, and the reason is a real failure (below)
tools/robotenv.sh & Sourced by startweb.sh. Single source of truth for ROSMASTERURI and ROSIP; changing networks is one line in ~/.leonetwork, not five files

rosbridge is launched on port 9090 for direct LAN use. The parameter websocketexternalport:=9090 is passed explicitly and must stay: autobahn validates the HTTP Host header, and rospy will otherwise re-read a stale ~websocketexternalport left on the parameter server by an earlier launch — a ghost that cost two days of a broken localhost mode.

Why startweb.sh restarts nothing it does not have to The watchdog (§17.17.3) re-runs this script whenever one port is missing. An earlier version started from scratch every time, so a 9443 that failed to bind — a flapping Wi-Fi is enough — caused the healthy 9090 rosbridge to be killed once a minute, and the operator’s cockpit disconnected in a loop. Each component is now restarted only if genuinely dead, and a component still starting (process up, port not yet bound) is left alone: rosbridge can take over a minute to bind under load.

### Step 3 — The first page

@L5.3cmL8.3cm@ File & Role
File & Role
web/vendor/
roslib.min.js & The ROS client, vendored rather than loaded from a CDN. This is deliberate: the cockpit must work on a lab network with no internet
web/ops.html & The control page: driving, AUTO mode, pose source, export
web/app.js & All cockpit logic, shared by the pages that need it. Its header block is the authoritative statement of the ROS contract

### Step 4 — Define the ROS contract, once

Four topics carry everything, and they are produced and consumed by leobackend.py (§17.9.5) on the workstation:

@L5.0cmL8.6cm@ Topic & Direction and payload
Topic & Direction and payload
/mission/command & browser → backend. stdmsgs/String carrying JSON: setmode, stop, reset, clearmap, park, setparams, manual, setview
/mission/telemetry & backend → browser, ∼10 Hz. One JSON object, roughly sixty fields (§3.4)
/mission/log & backend → browser. Human-readable mission events
/imu/dataclean & subscribed directly for the artificial horizon — the one case where a page reads a sensor topic rather than the backend’s summary

### Step 5 — Video, and why it is not on this script any more

webvideoserver serves MJPEG on port 8080, and the cockpit displays /mission/imageannotated — the backend’s annotated frame, not the raw camera.

It is started by navigationsupervision.launch, not by startweb.sh. The note in the script explains why: it belongs with the navigation stack that produces its image, and when startweb.sh freed port 8080 on every web healing, it killed the supervised instance each time.

### Step 6 — Replace http.server with something that can be trusted

web/serve.py [PC] is the static server on port 8000. It exists for three defects that python3 -m http.server has and that only became visible once the site was public:

- Stale pages. Documents that change on every update (.html, .json, .pdf, .js, .css) are sent with Cache-Control: no-cache, must-revalidate. Note the .pdf: it was forgotten in the first fix, and Cloudflare — seeing no Cache-Control — applied its own four-hour default and publicly served a 300-page report while the disk held 309. A forced browser reload could not help: the stale copy was at the network edge, not in the client.

- Directory listings. SimpleHTTPRequestHandler serves the whole directory, listings included. The audit of 2026-08-18 confirmed live that /exports/ answered 200 with a browsable index of 104 files, 17 MB of experimental data; models/ likewise.

- Backup files served next to the real ones. This project’s own convention produces names like ops.html.bak-avant-v4-161908 — complete, obsolete cockpits, published beside the current one. Thirteen were found. The refusal patterns in serve.py are those of §10 of the .gitignore, deliberately: what git refuses to archive, the server refuses to publish.

### Step 7 — A design system, self-hosted

@L5.3cmL8.3cm@ File & Role
File & Role
web/mission.css & The design system — “Deep Space Control”. Colour tokens, the glassmorphic panels, the bento grid
web/fonts/fonts.css
web/fonts/inter-*.woff2 & Inter, self-hosted in four weights. No Google Fonts request: the cockpit must render identically with no internet
web/vendor/
tailwind.min.js & Utility classes, vendored for the same reason
web/vendor/
lucide-1.25.0.umd.js & Icons
web/vendor/
gsap.min.js,
ScrollTrigger.min.js & Animation on the presentation pages
web/vendor/
chart-4.5.1.umd.js & Plots (PID curves, telemetry history)
web/vendor/NASAlogo.svg,
fitseal.svg & Institutional marks

### Step 8 — The fourteen pages

@L4.7cmL8.9cm@ Page (web/) & What it is for
Page (web/) & What it is for
index.html & Landing page
ops.html & The cockpit. Driving, AUTO, pose source, export
trajectory.html & Live 2D map, batched Canvas rendering
logbook.html & Mission journal; also carries the report download card
demo.html & Read-only by construction — no publisher, no command binding. For demonstrations, so that no one can drive by accident
pid.html & The three control loops, live
system.html & Host and stack health
audit.html & Audit views
navigationmodes.html & Explains the navigation modes
robotdescription.html & The robot, with the 3D model
tfvalidator.html & Inspect and edit frames — the only page that writes (Step 12)
minstuning.html & MINS parameter exploration
hectorresponse.html & A written answer to a supervisor’s question, kept as a page
preview-design.html & Design bench for the system of Step 7

### Step 9 — The 3D view

@L5.3cmL8.3cm@ File & Role
File & Role
web/3dengine.js & The renderer used by the description and trajectory pages
web/vendor/three.min.js & three.js
web/vendor/
OrbitControls.js & Camera control
web/vendor/STLLoader.js & Mesh loading
web/models/
leoroverv1.9v5.stl
web/models/leorovermap.stl & The rover and the room
tools/decimatestl.py & Reduces mesh size before shipping it to a browser — a CAD export is not a web asset

### Step 10 — Make it bilingual

web/i18n.js [PC] holds every string in English and French and exposes I18N.t(), I18N.set() and I18N.apply(); the DOM is marked up with data-i18n attributes and re-scanned on change. The chosen language persists in localStorage.

Two findings are recorded rather than hidden. The key counts match exactly (616 EN, 616 FR), but 75 keys are dead, and 37 of those are log — which reveals that the mission log is never translated, because the backend emits raw French text on /mission/log. Translating the cockpit did not translate the thing an operator actually reads during a mission.

### Step 11 — Pages that read data files

@L5.3cmL8.3cm@ File & Role
File & Role
web/autoentries.json & Mission events written automatically by leobackend.py; read by logbook.html. Also the evidence source for several results in this report (RTB pairs, §12.10.1)
web/logbookdata.js & The hand-written journal entries
web/tfframes.json & Frames shown — and edited — by tfvalidator.html

### Step 12 — One write path, bound to the loopback on purpose

tfvalidator.html can save an edited frame. That write is served by tools/tfsaveserver.py [PC] on 127.0.0.1:8010, and the binding is the security design:

port 8000 is published to the internet by the Cloudflare tunnel, so a write route placed there would be open to everyone. Filtering by client IP would achieve nothing, because cloudflared connects from the loopback itself and every request would appear local. Binding the write server to 127.0.0.1 instead means saving works from the workstation and fails from outside — which is exactly the intent.

The one place a rotation matrix can be overwritten from a browser This is the page behind the forced-identity incident of 2026-08-20: a guard-rail was overridden twice and an identity matrix stayed on disk until the next day, invalidating every openVINS feature measurement taken in between. The lesson is not to remove the page but to date what it changed: after any edit here, check web/tfframes.json against what the estimator is actually loading.

### Step 13 — Publish it, with TLS that survives a tunnel

@L5.3cmL8.3cm@ File or service & Role
File or service & Role
cloudflared tunnel run & Publishes the site at cockpit.leo-rover-gardon.dev. Started from startweb.sh with an absolute path: bare cloudflared failed silently under the watchdog’s minimal PATH, leaving the public tunnel dead with no visible alert
tools/
rosbridgewebsockettunnel.py & The TLS rosbridge on port 9443, launched with websocketexternalport:=443. Without that value, handshakes arriving from cloudflared carry a public Host with no port — implicitly 443 — and autobahn rejects every one of them
~/.leocerts/rosbridgecert.pem,
rosbridgekey.pem & The certificate pair. Outside the repository, deliberately
/tmp/leotunneloff & A flag, not a file to edit by hand. Created by the cockpit’s Tunnel public button; startweb.sh checks it and refuses to reopen an exposure the operator has just closed — otherwise the watchdog would reopen it within the minute

### Step 14 — Publish the report onto the site

tools/updatereport.sh [PC] compiles this document and deploys it to the single public location: the download card of logbook.html, at web/reports/
LEORoverMissionControl
ReportNicolasGardon.pdf.

It also rewrites the ?v= stamp on that link across every page that carries it. The stamp is not decoration: without it Cloudflare serves a cached copy, and the symptom is a report that is visibly several versions old with no error anywhere. serve.py’s no-cache on .pdf (Step 6) is the second line of defence; the stamp is the first.

### Verifying the site after any change

Three checks, in increasing cost, all documented in web/READMEWEB.md:

- Structure — the checker in READMEWEB.md validates every page’s structure, links and assets on the served site, not on disk. The distinction matters: serve.py refuses paths that exist perfectly well in the directory.

- i18n — list keys used in HTML but absent from i18n.js. A missing key renders as its raw identifier, which is unmistakable once seen and easy to miss while editing.

- Through the tunnel — load the public URL, not just localhost:8000. Several defects in this section existed only at the edge: the four-hour PDF cache, the Host header rejection on 9443, the browsable exports/. None was reproducible locally.

## Robot-side configuration

The Pi runs a stock LEO Rover image plus three project-specific changes. If you reflash the robot, you must redo all three, and the system will appear to work while behaving badly if you forget, which is exactly how each was discovered.

### How the robot starts itself

The Pi runs leo.service, a systemd unit that launches rosmon (a supervising alternative to roslaunch), which in turn starts the ROS master and the robot’s nodes from /opt/ros/noetic/share/leobringup/launch/leobringup.launch.

ssh pi@$ROBOT_HOST 'systemctl status leo --no-pager | head -5'
ssh pi@$ROBOT_HOST 'sudo systemctl restart leo'     # full robot-side restart

rosmon respawns any node that dies, which is usually helpful and occasionally the reason something you killed keeps coming back (§17.8.4).

### Change 1: real-time priority for the serial node

Why: the CORE2 board streams IMU and encoder data into a 32-byte hardware FIFO on the Pi’s PL011 UART. If serialnode is not scheduled before that FIFO fills, bytes are lost with no retransmission. Symptom: thousands of checksum errors and wheel odometry collapsing from 19 Hz to 1 Hz.

The fix has two halves and needs both. With only the first, the node does not start at all: you lose driving entirely, which is worse than the bug.

- Ask for real-time priority, in leobringup.launch on the robot:

- <node name="serial_node" pkg="rosserial_python" type="serial_node.py"
      launch-prefix="chrt -f 25">

- Grant permission to ask, because user pi has rtprio=0 by default and chrt would simply fail. Create /etc/systemd/system/leo.service.d/10-rtprio.conf:

- [Service]
LimitRTPRIO=50

- then sudo systemctl daemon-reload && sudo systemctl restart leo.

Verify both halves:

ssh pi@$ROBOT_HOST 'chrt -p $(pgrep -f serial_node | head -1)'

Expected: scheduling policy: SCHEDFIFO, priority: 25.
And the payoff: rostopic hz /firmware/wheelodom should read ∼19 Hz. Before the fix it read 1 Hz.

### Change 2: camera settings that reset on every restart

Why: the D455’s infrared projector paints a dot pattern that helps depth and ruins the checkerboard and LED detection. It is therefore turned off, and the depth compression level is lowered to save Pi CPU. Both are dynamicreconfigure values, so they revert to factory defaults every time the camera node restarts, and the camera node restarts whenever the stack does.

rosrun dynamic_reconfigure dynparam set /camera/stereo_module laser_power 0
rosrun dynamic_reconfigure dynparam set \
  /camera/depth/image_rect_raw/compressedDepth png_level 1

tools/restartstack.sh and the watchdog both reapply these, which is why you rarely notice. If your checkerboard detection suddenly degrades, check laserpower first:

rosrun dynamic_reconfigure dynparam get /camera/stereo_module | grep laser_power

### Change 3: disable the robot’s own rosbridge

Why: the stock image runs a rosbridgeserver on the robot for its factory web UI. This project’s cockpit connects to a rosbridge on the PC instead, so the robot’s copy has zero clients, yet it was measured consuming 19% of a CPU core on a Pi already thermally throttled. Its consumption grows with uptime (a leak: a freshly respawned instance sits at 0%), and its unregistertimeout of 86400 s let it live outside the ROS graph, invisible to rosnode list.

Comment the node out in leobringup.launch on the robot:

<!-- DISABLED: unused, and leaks CPU on a thermally limited Pi.
<node name="rosbridge_server" pkg="rosbridge_server"
      type="rosbridge_websocket">
  <param name="unregister_timeout" value="86400"/>
</node>
-->

Then sudo systemctl restart leo. Killing the process without editing the launch file is futile: rosmon respawns it within seconds.

## The code we wrote, node by node

Everything upstream (openVINS, MINS, the camera driver) is unmodified. The project-specific logic lives in six files, all in catkinws/src/leonavigation/scripts/ except the last. Each has a long header comment explaining why it exists: read that header before changing anything, because most of these nodes exist to work around a specific measured defect, and the workaround looks arbitrary until you know which.

### imu_sanitizer.py: the IMU is not trustworthy raw

Subscribes /firmware/imu (leomsgs/Imu) Publishes /imu/dataclean (sensormsgs/Imu)

This node exists because the CORE2 IMU has four separate defects, each found by measurement, and both estimators consume its output.

- Isolated glitches. Roughly two samples in nine thousand come out absurd (one accelerometer reading of 5×1016 m/s​2 was recorded). A single such sample destroys any filter instantly. Samples outside ~gyrolimit (35 rad/s) or ~accellimit (160 m/s​2) are replaced by the last good one, keeping the cadence intact.

- A frozen-firmware mode. After a serial desync the board can retransmit the same sample forever: bit-identical values, which a real sensor never produces. N identical samples in a row triggers an automatic /firmware/resetboard call (throttled).

- Accelerometer scale error. With the robot verified stationary (502 samples, wheel odometry 0.000 m/s) the raw magnitude read 8.7525 m/s​2 against a true local gravity of 9.790. That is −10.6%, and it is in the sensor: raw and sanitised agreed to 0.005. Corrected by ~accelscale =9.790/8.7525=1.1186.

- Gyroscope bias, badly out of spec. Stationary, the gyro read 3.679 deg/s of total bias where a healthy MEMS part sits at 0.01–0.1. Because attitude is the integral of the gyro, that mis-projects gravity into the body frame and injects a phantom horizontal acceleration far larger than the accelerometer error. Subtracted, and re-measured at every node start (~gyroautocal, default true) because bias drifts with temperature and this Pi runs at 86 ∘C.

Parameters you may need:

~accel_scale    1.1186   # 1.0 disables; see (3) above
~gyro_scale     1.0      # NOT measured yet -- see below
~gyro_autocal   true     # re-measure bias at startup if the robot is still
~gyro_limit     35.0     # rad/s, glitch rejection
~accel_limit    160.0    # m/s^2

The honest gap. ~gyroscale is deliberately left at 1.0, i.e. no correction. A stationary calibration measures the bias but is mathematically incapable of measuring the scale: at rest the true rate is zero, so the scale factor drops out of the observation entirely. Measuring it requires a turn of known amplitude against physical ground truth: see §17.13.3. Setting this parameter on a guess would corrupt both estimators at once.

### wheel_remap.py: a two-line bug that costs a day

Subscribes /jointstates Publishes a 2-element JointState MINS can read

MINS reads wheel velocities by array index, not by joint name: data.m1 = msg->velocity.at(0) with a comment claiming it is the front left wheel. This rover publishes its four wheels in a different order, so MINS was silently fed the wrong wheels. Nothing errors; the estimate is simply wrong. This node republishes a two-element message in the order MINS assumes.

~front_left_name    ~front_right_name     # which joints to pick
~rear_left_name     ~rear_right_name
~in_topic  /joint_states                  ~out_topic

Lesson worth carrying: when integrating any upstream estimator, verify how it identifies its inputs. Index-based assumptions are common and fail silently.

### pose_selector.py: one topic that never breaks

Subscribes /mins/imu/odom, /ovmsckf/odomimu Publishes /robotposefused Service /poseselector/setsource (stdsrvs/SetBool; true = MINS)

Downstream consumers (the cockpit, logging, any navigation) need one pose topic that keeps working when you switch estimators. The two estimators have independent origins, so a naive switch would produce a jump of tens of metres. This node applies an SE(3) correction at the moment of the switch so the output is continuous.

~default_source        mins    # what is served at startup
~max_plausible_speed           # rejects physically impossible jumps
~mins_topic  ~vins_topic  ~odom_frame  ~base_frame  ~imu_frame

Important limitation, and it bites: the SE(3) correction is computed on a switch only. If MINS itself respawns it re-initialises at a new origin, and that jump is not absorbed: the fused topic jumps with it. This is one reason the estimators are deliberately not allowed to bring down the stack: driving must never depend on estimator health.

### carolus_vicon_bridge.py: the only absolute reference

Publishes geometrymsgs/PoseStamped on the topic MINS accepts as a VICON input

Every other measurement on this platform is relative. The Carolus beacon, a fixed LED constellation at a known place, gives a drift-free global fix whenever it is in view. This node reformats that fix into the type MINS’s external-reference slot consumes, making it an “indoor GPS” that cancels unbounded drift.

~global_frame   ~body_frame   ~out_topic   ~rate_hz   ~max_tf_age

Rule established 2026-07-08 and violated once since: the topic /mins/externalref/carolus is the private slot of this bridge. Nothing else may publish on it. A trajectory recorder that also wrote there produced a genuinely confusing failure.

### leo_backend.py: the cockpit’s brain

This is the largest file in the project (∼4500 lines) and it runs on the workstation, launched by leobackendlaunch.sh. It is not a normal ROS node in spirit: it is the bridge between the browser and ROS.

- Inbound: JSON commands on /mission/command. Every cockpit button publishes one, e.g. {"action":"setposesource", "source":"VINS"}. To add a button, add an elif action == "..." branch in oncommand and a click handler in the HTML.

- Outbound: a JSON telemetry blob at 10 Hz on /mission/telemetry, plus human-readable lines on /mission/log. The browser subscribes to both through rosbridge.

- Also owns: the autonomous patrol state machine, the beacon approach logic, the trajectory buffers, the detached rosbag recorder, the MATLAB export, and the ground-truth calibration panel.

Two hard-won implementation notes. Launching a GUI application (MATLAB) from this process needs three separate things or it dies within seconds: a double fork so it survives its parent, a pseudo-terminal (MATLAB’s -r exits when stdin is not a tty), and a reconstructed XDGRUNTIMEDIR/DBUSSESSIONBUSADDRESS because the backend is started without a login session. And any recording that matters is written by a detached rosbag process, because this backend’s own buffers are lost if it restarts.

### vins_rotation_guard.py: deliberately not used

This file is still in the tree and is intentionally not launched. It was written to freeze openVINS’s reported position during turns, on the belief that openVINS diverged when rotating. It made things measurably worse (§17.17.6). It is kept as a record of the reasoning error, not as a component. Do not re-enable it without reading that section.

## Configuration files, and the traps inside them

### A trap that applies to every YAML here

Read this before editing any estimator config. Both openVINS and MINS parse their YAML with OpenCV’s FileStorage (the files begin with %YAML:1.0), which is not a general YAML parser. It aborts on a long comment placed after a value on the same line.

The failure mode is vicious: the node starts, subscribes to nothing, and respawns forever, with no error message naming the file. This trap was hit twice on this project: the second time after it had already been documented.

- Keep inline comments short, or put them on their own line.

- Validate before restarting anything:

- cd ~/TOUT/catkin_ws/src/open_vins/config/leo
python3 -c "import cv2; f=cv2.FileStorage('estimator_config.yaml', \
cv2.FILE_STORAGE_READ); print('readable:', f.isOpened()); f.release()"

- Expected: readable: True.

- Never test a config from /tmp: relative paths inside it resolve differently and you get a false negative.

### openVINS

Three files, all in catkinws/src/openvins/config/leo/:

estimator_config.yaml     # the filter: init, ZUPT, features, noise, topics
kalibr_imucam_chain.yaml  # camera intrinsics + camera-IMU extrinsics
kalibr_imu_chain.yaml     # IMU noise densities and random walks

Launch footgun. The config path is passed as args="...", i.e. argv[1], and not as a <param name="configpath">. On the ROS1 build, the parameter read is inside an #if ROSAVAILABLE == 2 block and never executes. Using <param> fails silently: argv[1] becomes roslaunch’s own name:=... argument, and openVINS reports it cannot open a file with that literal name.

The values that matter on this platform, each with the measurement behind it:

Key openVINS settings and why they are set this way.

| Key | Value | Why |
| --- | --- | --- |
| tryzupt | true | The single largest fix. With false a run diverged to 41 824 m; with true, 0.002 m. Zero-velocity updates anchor a ground vehicle that stops often. |
| gravitymag | 9.790 | True local gravity (Melbourne FL). Leaving 9.81 against a mis-scaled accelerometer compounds the error. |
| numopencvthreads | 1 | A mutex crash fired 1324 times, once every 46 s. Root cause: OpenCV threading. Zero crashes in 7 minutes after. |
| maxslam | 0 | Pivoting in place makes landmark depth unobservable; matches the MINS setting. |
| calibcamextrinsics | false | Locked 2026-07-29. Online refinement of a geometry that is poorly observable under planar motion lets it wander. |
| calibcamtimeoffset | false | Same reasoning. |
| calibcamintrinsics | false | Frozen from Kalibr; offline measurement beats online refinement. |
| initimuthresh | 0.4 | The jerk needed to initialise. The rover must be nudged (§17.12). |
| usestereo / maxcameras | true / 2 | Both D455 infrared cameras. |

The IMU noise densities in kalibrimuchain.yaml (accelerometernoisedensity: 5.0e-03, gyroscopenoisedensity: 3.4e-04) are measured values inflated by a safety factor of three, not datasheet figures.

### MINS, start to finish: download, build, install, calibrate, test

MINS (multi-sensor inertial navigation system) is the tightly-coupled filter on this platform: camera, IMU and wheel odometry fused in one estimator, publishing its pose on /mins/imu/odom. Because its own TF broadcasts would otherwise fight openVINS over the same frames, they are isolated onto a dedicated /tfmins topic (subscribe there explicitly in RViz, not on /tf).

This subsection is a single, self-contained path from an empty machine to a MINS you can trust, and does not assume the rest of this appendix has been read first: download and build the estimator, install and understand the ten configuration files that make it work on this robot rather than a generic one, calibrate the two measurements those files depend on, then validate that a fresh install is not just running but running correctly. Steps that apply to the whole platform and not just MINS (ROS, system libraries, network) are covered fully in §17.5 and only summarised here; every §reference below is for extra depth, not a prerequisite for the step it is attached to.

##### Download and build.

- Prerequisites. Ubuntu 20.04 with ROS Noetic, and the C++ libraries both estimators need (§17.5, Steps 0–2):

- sudo apt install libeigen3-dev libboost-all-dev libopencv-dev \
                 libceres-dev libsuitesparse-dev

- One MINS-specific caveat, from upstream’s own README: MINS is tested against Eigen 3.3.7 or lower; a newer system Eigen can fail to compile the bundled libpointmatcher third-party library (used for the LiDAR sensor, disabled on this platform but still compiled). Ubuntu 20.04’s own libeigen3-dev is 3.3.7, so this only bites if you have manually upgraded it.

- Clone MINS into its own workspace, kept separate from openVINS because the two disagree on build flags (§17.5):

- mkdir -p ~/TOUT/mins_ws/src
cd ~/TOUT/mins_ws/src
git clone https://github.com/rpng/MINS.git

- Resolve dependencies and build:

- cd ~/TOUT/mins_ws
rosdep install --from-paths src --ignore-src -r -y
catkin build mins
source devel/setup.bash

- Expected: a progress table ending with a green summary, [build] Summary: All ... packages succeeded!, matching §17.5’s Step 3.

- Do not source this workspace back-to-back with catkinws. devel/setup.bash replaces several environment variables instead of extending them, so sourcing both in sequence breaks one workspace’s or the other’s ROSPACKAGEPATH (§17.5.5, the single most confusing failure mode in this build). Never launch anything MINS-side with a plain roslaunch; always go through the wrapper:

- ~/TOUT/catkin_ws/src/leo_navigation/launch_mins.sh navigation_master.launch

- or, equivalently and more simply, bash tools/restartstack.sh, which calls the same wrapper for you.

##### Install and configure.

The step a fresh clone is missing. MINS ships with example configuration profiles (kaist, tumvi, eurocmav, simulation, …) but no profile for this robot: there is no mins/config/leo/ directory in the upstream repository. Without one, mins.launch’s default argument (config:=leo) points at a file that does not exist, and MINS reports that it cannot open a configuration file. The leo profile is this project’s own work, so it is versioned in this repository, not MINS’s, at LEORoverNavigationSystem/MINS-master/mins/config/leo/. Install it into the workspace just built:

cp -r ~/TOUT/LEO_Rover_Navigation_System/MINS-master/mins/config/leo \
      ~/TOUT/mins_ws/src/MINS/mins/config/leo

Verify: ls  /TOUT/minsws/src/MINS/mins/config/leo/ lists ten files. Every one is required: config.yaml’s only job is to include the other nine by relative filename, and a missing file aborts the same way a missing leo/ directory does.

@L2.9cmL9.9cm@ File & What it holds, and what is not a stock/default value
config.yaml & Top-level include list only. Also documents, in its own header comment, the enable ladder this deployment follows: bring up camera+IMU first, then wheels, then the Carolus VICON slot, then LiDAR last, each stage validated before the next is switched on. An unvalidated extrinsic on one sensor makes the whole fused estimate diverge in a way that is hard to attribute back to that sensor.
configimu.yaml & Noise densities from an Allan-variance characterisation of this IMU, not datasheet figures. The field that matters most here is topic: "/imu/dataclean": point this at the raw firmware topic (/firmware/imu) instead and MINS silently ingests every glitch, every frozen-sample run, and the accelerometer’s −10.6% scale error that imusanitizer.py exists to remove (§17.9).
configcamera.yaml & cam0/cam1 topics are /pc/camera/infra1|2/imagerectraw: the workstation’s local republish of the robot’s compressed stream, never the robot’s raw topics directly (two raw 848×480 streams demand more bandwidth than the WiFi link can sustain, and have been measured saturating the AP to the point of dropping it). Intrinsics come from kalibrcalibratecameras and are frozen (docalibint: false); Timucam is still a tape-measure seed, see Calibrate below. maxslam: 0 because this platform pivots on the spot often, which makes landmark depth unobservable and produced a measured +0.58 rad/s phantom heading term from bad SLAM anchors on 2026-07-13.
configwheel.yaml & The most heavily retuned file; see the dedicated walkthrough below.
configestimator.yaml & gravitymag: 9.790, local gravity for Melbourne FL, not the generic 9.81; re-measure only if this configuration is deployed somewhere else. Filter and clone-rate settings otherwise match MINS’s own defaults.
configinit.yaml & Static IMU+wheel initialisation thresholds. MINS initialises standing still in 4–12 s with these defaults; nothing platform-specific to change here.
configsystem.yaml & verbosity: 1, not MINS’s own default of 2: at 2, the initialiser’s diagnostic prints (waiting for data, gravity convergence, static/dynamic init attempts) are filtered silently, which looks exactly like a hung initialiser when it may be running normally. Also sets the output paths for the trajectory/timing logs.
configgps.yaml & enabled: false. Present only because MINS’s loader expects the file to exist; this platform has no GNSS receiver. Leave it as shipped.
configlidar.yaml & enabled: false. Staged for a future integration (an RPLidar publishes LaserScan; MINS wants PointCloud2, and a conversion node does not exist yet). Leave disabled.
configvicon.yaml & enabled: false until carolusviconbridge.py is verified publishing on /mins/externalref/carolus, MINS’s slot for the Carolus beacon acting as an indoor GPS (§17.9). Flip to true once that bridge is confirmed live; the topic is a private slot and nothing else may publish on it (rule established 2026-07-08).

configwheel.yaml in detail, because it carries more measured, platform-specific corrections than any other file in the tree:

- type: "Wheel2DAng": a two-wheel differential model. This rover is four-wheel skid-steer, not differential-drive; Wheel2DAng approximates it from the front-left/front-right pair, and the online extrinsic refinement below absorbs most of the mismatch.

- intrinsics: [0.0622, 0.0622, 0.358]: effective wheel radius (twice) and track width, in metres. These are measured, not the firmware’s stock values, and they are specific to this set of wheels; see Calibrate below for how to re-measure them.

- Timuwheel: a 180∘ rotation about z, because this rover’s wheel-odometry frame points opposite to the camera/IMU frame at the firmware level. Without it, every wheel measurement contradicts the inertial prediction and the χ2 gate silently rejects it, which looks exactly like “MINS is ignoring the wheels.”

- chi2mult: 5, not MINS’s default of 15: the slip-rejection gate for wheel-versus-IMU disagreement, tightened after measuring rigid wheels slipping in turns (below).

- noisew: 0.8 and noisev: 0.2, both retuned from MINS’s defaults (0.2 and 0.5 respectively): in a turn, this rover’s wheels can imply a yaw rate an order of magnitude larger than the camera-measured truth (3.8 rad/s of wheel-implied rotation for 0.001 rad/s actually measured), so angular trust is loosened; linear speed from the wheels is accurate and is trusted more than the default, because it is what keeps the filter anchored through camera dropouts.

- topic: "/jointstatesmins" with subtopics: "wheelFLjoint, wheelFRjoint": MINS reads wheel velocities by array index, not by name (ROSSubscriber::callbackwheel takes velocity.at(0) and .at(1)), and this rover’s real /jointstates does not order its four wheels front-left-first. Point this at raw /jointstates and MINS silently reads the wrong wheel; wheelremap.py exists purely to fix the order (§17.9), and subtopics must name-match that node’s own ~frontleftname and ~frontrightname parameters. Omitting subtopics is a silent-drop trap in its own right: MINS’s loader defaults it to generic joint names this rover does not use, the remapped topic keeps publishing (visible on rostopic hz), and MINS’s internal wheel buffer simply never fills, which looks like an initialisation problem rather than a name mismatch.

Wiring it up. Two more pieces complete the installation, covered fully in §17.11:

- The config profile is selected by mins.launch’s config argument, passed to the estimator as args="$(find mins)/config/$(arg config)/config.yaml". It must be args, never a <param name="configpath">: the parameter read exists only in MINS’s ROS2 build, and on ROS1 that block is compiled out entirely (§17.11.3). Getting this wrong does not fail clearly: MINS reports it cannot open a file whose name was never typed.

- MINS’s config files are parsed with OpenCV’s FileStorage, not a general YAML parser: a long inline comment placed after a value on the same line aborts the file silently, and the node then starts, subscribes to nothing, and respawns forever with no error naming the file (§17.10.1). If a comment is added or edited in any config.yaml, keep it short or put it on its own line, and validate before restarting anything:

- cd ~/TOUT/mins_ws/src/MINS/mins/config/leo
python3 -c "import cv2; f=cv2.FileStorage('config_wheel.yaml', \
cv2.FILE_STORAGE_READ); print('readable:', f.isOpened()); f.release()"

Finally, three companion nodes must be running or the topics these config files name never publish: imusanitizer.py (feeds configimu.yaml’s topic), wheelremap.py (feeds configwheel.yaml’s), and, once Stage 3 is reached, carolusviconbridge.py (feeds configvicon.yaml’s). All three start automatically as part of navigationsupervision.launch, included by the same navigationmaster.launch that starts MINS itself (§17.11); nothing further needs to be launched by hand.

##### Calibrate.

Three measurements feed the config files above; two are already done and versioned, one is not.

- IMU sanitisation (already automatic). imusanitizer.py applies a fixed accelerometer scale correction (×1.1186, measured stationary against local gravity) and re-measures gyroscope bias at every startup (§17.9). Nothing to run by hand: this happens every time the stack starts, before MINS ever sees a sample.

- Wheel radius and track (configwheel.yaml’s intrinsics). Required again only if the wheels themselves change (different tyres, a different rover). Measured passively, with no commanded motion from the script itself:

- cd ~/TOUT
python3 tools/calib_roues_rigides.py

- Then, from the cockpit or a joystick, drive the rover straight for about 3 m (estimates the radius), then spin in place for about two full turns (estimates the track), and press Ctrl-C. The script prints the left/right radius, the track, and an intrinsics: line ready to paste into configwheel.yaml.

- Camera–IMU extrinsic (configcamera.yaml’s Timucam).

- Honest status: not yet completed on this platform The procedure below is fully scripted and ready to run, but the excitation motion it needs (physically lifting and tilting the rover in front of the target) has not been performed as of this writing (Table [tab:openitems], item 2). The deployed extrinsic is a tape-measure seed. Updated 2026-09-07: it is no longer refined online. docalibext is now false in both configcamera.yaml and configwheel.yaml, and openVINS’s equivalent (calibcamextrinsics) is likewise false, because online estimation of an unobservable quantity was measured to random walk at ≈1.4 mm/min with the rover stationary — eventually pushing wheel and visual updates outside the χ2 gate and costing the X/Y anchor entirely (§12.28, item 1 of the 2026-09-07 entries).

- This inverts the priority stated in earlier revisions of this paragraph. While the seed was refined online, completing the Kalibr calibration was a precision improvement; now that nothing is refined, the seed is the final value, its quality sets steady-state accuracy directly, and there is no second net. Completing it is the single highest-value calibration task remaining on this platform.

- Kalibr lives in its own workspace, since its build requirements conflict with both estimators’:

- mkdir -p ~/TOUT/kalibr_ws/src && cd ~/TOUT/kalibr_ws/src
git clone https://github.com/ethz-asl/kalibr.git
cd ~/TOUT/kalibr_ws && catkin build
source devel/setup.bash

- With Kalibr built, two scripts do the rest, in order:

- tools/calib_imucam_record.sh [duration_s]   # default 90s, records a bag
tools/calib_imucam_run.sh <bag>             # kalibr's two calib steps

- The recording step is the one that fails if driven, not held. Kalibr needs all three rotation axes excited in front of the checkerboard target; a rover rolled flat along the ground stays planar and roll/pitch are never observed, so the calibration fails silently. Pick the rig up and tilt it by hand instead. The run step needs no supervision: it executes kalibrcalibratecameras then kalibrcalibrateimucamera back to back, 10–40 CPU-heavy minutes depending on bag length. Expected: a *-camchain-imucam.yaml and a report PDF; accept only if the reprojection error is <0.5 px and timeshiftcamimu is a few milliseconds, not a fraction of a second. Then, specifically for MINS (the companion calibimucamapply.sh script installs a result into openVINS’s config only, by design, so the two estimators’ opposite conventions are never mixed in one script):

- python3 catkin_ws/src/leo_navigation/scripts/fill_mins_camchain.py \
        <bag>-camchain-imucam.yaml \
        mins_ws/src/MINS/mins/config/leo/config_camera.yaml

- This inverts Kalibr’s Tcamimu into the Timucam convention MINS expects and copies the time offset; a direct copy without inverting produces a plausible-looking, wrong geometry.

A fourth measurement, optional but valuable: the ground-truth floor protocol of §17.13.3 validates gyroscale and cross-checks the wheel radius against a tape measure and a 3-4-5 triangle rather than against another estimator. It is not required for MINS to run, but it is the only procedure in this guide that compares any of the above to physical reality rather than to another sensor.

##### Test.

- Launch. From ~/TOUT:

- bash tools/restart_stack.sh

- Expected: stack relancée, then MINS initialisé après <N>s (typically 8–12 s), then TERMINE. Unlike openVINS, MINS needs no jerk to start: it initialises standing still (§17.12).

- Confirm the node registered:

- source /opt/ros/noetic/setup.bash
rosnode list | grep mins_subscribe

- Confirm it is actually publishing, at a healthy rate. A registered node can be completely silent; this is the real test:

- rostopic hz /mins/imu/odom          # expect ~80-90 Hz

- Nothing at all after 15–20 s usually means one of the config files above failed to parse (§17.10.1: check logs/navigationmaster.log for a respawn loop) or a companion node (imusanitizer.py, wheelremap.py) is not running.

- Validate correctness, not just aliveness. A silent estimator is an obvious failure; a wrong one that publishes cleanly at 85 Hz is not. Protocol T1.1 (§17.13.4) is the cheapest test that tells the two apart: log /robotposefused for five minutes with the rover completely still,

- cd ~/TOUT/tools
python3 log_pose_static.py 300 /tmp/t1_1_static_pose.csv
python3 analyze_pose_variance.py /tmp/t1_1_static_pose.csv

- Pass: linear drift below 1 mm/s on every axis. A fresh install that passes this has a correctly wired MINS. One that fails it almost always has a configuration or extrinsic problem, not a filter-gain problem (§17.13.4).

- Validate under motion, once the above passes. Drive a closed loop and compare MINS against openVINS on the same run, following §17.13.1 end to end. That procedure also covers switching the cockpit’s Pose Source to MINS and recording both estimators to a rosbag, which the website’s own buffers are not reliable enough to replace.

- Validate against a measured rectangle, for an absolute dimensional check. Protocol T1.1 catches a wrong estimator standing still; §17.13.1 catches disagreement between MINS and openVINS while moving. Neither compares either one to a physical, tape-measured shape. This one does, using the cockpit’s own Test 1 / Test 2 recorder on the Trajectory page rather than a terminal command, so it is the version of this test an operator without shell access can still run.

- How the recorder actually works, since it looks simpler than it is. Clicking Test 1 or Test 2 does two things at once: it labels the run, and it starts a rosbag record process that leobackend.py launches detached from itself, writing straight to disk. That detachment is the entire point: the cockpit’s own in-memory trajectory buffer is lost on every backend restart, which on this platform is common enough (thermal throttling, the watchdog, a crashed estimator) that a run relying on it can silently lose data mid-drive with no visible symptom. A detached recording keeps writing regardless, and Export to MATLAB later reads whichever is more authoritative: the finished recording on disk if one exists for that test label, the in-memory buffer only as a fallback if no robust recording was ever started for it. Both estimators are written to that same recording unconditionally, whether MINS or openVINS is the one currently shown on screen: the Test 1/Test 2 label only decides the filename prefix the eventual export uses, never what gets captured. This is exactly the mechanism validated on 2026-07-29 with a real pair of drives around the lab (§12.12.7): read that section first if the numbers below look implausible on a first attempt, since it shows what a clean pair of exports and a trustworthy verdict actually look like on this platform, drift and all.

  - Mark a rectangle on the floor. Tape one of known dimensions, measured with a tape measure, not by eye; 2.00 m×1.50 m fits most lab spaces, but use whatever the room allows. Mark the starting corner and the direction of travel: you will start and finish there, facing the same way, so the loop closes.

  - On the Trajectory page, click Test 1. This both labels the run and starts a rosbag recording detached from the browser (§17.13.1: it survives a backend restart, unlike the page’s own buffer). MINS and openVINS are captured together regardless of which one is on screen, but by convention Test 1 is the lap whose MINS trace gets trusted below.

  - Drive the rectangle once, corner to corner, back to the marked start point and heading. Slowly and smoothly; a sharp turn at each corner is not required, since the analysis below reads the trajectory’s overall extent, not its individual corners.

  - Click Export to MATLAB. This writes web/exports/test1mins.csv, test1vins.csv and test1.mat, and, if MATLAB is available on the workstation, launches plottrajectories(’test1.mat’) automatically: a useful MINS-versus-openVINS sanity check on this one lap, but not yet the rectangle comparison below.

  - Run the rectangle comparison, for MINS:

  - cd ~/TOUT/tools
matlab -batch "plot_rectangle_test('../web/exports/test1', 2.00, 1.50, 'mins')"

  - Expected: a printed block with the measured dimensions, the trajectory’s bounding box, the percentage error on each side, and the loop-closure gap, plus a saved test1minsrect.png overlaying the trajectory on the ideal rectangle. For context, not a pass/fail threshold: this platform’s own wheel odometry alone over-reports straight-line distance by 39% uncorrected (§17.17.4); a correctly wired estimator should track the measured rectangle far more closely than that.

- Run 2026-08-05: closed, against the taped rectangle below 6.405 m×3.455 m, taped and tape-measured. Test 1, MINS trusted. Figure 12.30 and §12.16.5 give the full result and its interpretation; in short, bounding box 6.176×3.919 m (−3.6 % length, +13.4 % width), loop closure 0.449 m over a 23.35 m path (1.9 %). The asymmetry between the two axis errors is not noise: §12.16.3 ties it to a measured turning asymmetry on this platform, not to a uniform scale error, which is the reading the raw bounding-box numbers alone cannot give.

### The same test, for openVINS

openVINS has no dedicated “start to finish” walkthrough in this appendix the way MINS now does (§17.10.3); its build and launch are covered in §17.5 and §17.12. What follows is only the test methodology, mirrored from the MINS protocol above using the cockpit’s Test 2 slot, because that is specifically what was asked for.

- Wake openVINS before starting Test 2, with a jerk (§17.12): it publishes nothing at all until it has felt one, and Test 2’s whole purpose below is to be the lap whose openVINS trace is trusted.

- On the Trajectory page, click Test 2. If Test 1 is still recording, the cockpit asks to finalise and save it first: confirm, and it does so before starting Test 2.

- Drive the rectangle a second time, corner to corner, back to the same marked start point and heading. This is a genuinely independent lap, not a re-analysis of Test 1’s data: driving it twice is the point, since §17.10.5 below compares the two laps to each other, not two estimators reading the same one.

- Click Export to MATLAB again. This writes web/exports/test2mins.csv, test2vins.csv and test2.mat, overwriting nothing from Test 1: the files are named by test label, not by timestamp.

- Run the rectangle comparison, for openVINS:

- cd ~/TOUT/tools
matlab -batch "plot_rectangle_test('../web/exports/test2', 2.00, 1.50, 'vins')"

- Expected: the same style of report as MINS’s (measured dimensions, achieved bounding box, percentage error per side, loop-closure gap), saved to test2vinsrect.png. Use the same lengthm/widthm arguments as the MINS run: both laps are being measured against the identical physical rectangle, which is what makes §17.10.5 below meaningful.

Run 2026-08-05: executed, and openVINS did not close the loop Test 2 was recorded against the identical rectangle, in the same session, in the same rosbag pipeline as the MINS run above. Unlike that run, this one is not a validation: openVINS returned a 675.28×96.01 m bounding box and a 2760.55 m path against a 19.72 m measured perimeter. The full diagnosis — a rewritten pose-fusion guard, a diagnosed IMU tilt, a thermal envelope this session sat inside rather than beside, and the algebraic proof that no single scale factor can rescue this trajectory — is §12.16.1–§12.16.5, not repeated here. What belongs here is the honest status: the procedure above is correct and was followed exactly; its output on this platform, this day, is not a usable openVINS figure. §17.17.2 and a re-run once the platform is cooled and lit remain the way to get one.

### MINS versus openVINS, against the same measured rectangle

Two independent laps now exist, each measured against the same physical rectangle. Two different comparisons are worth running on them: one already exists in this project for exactly this two-lap situation, the other is the dimensional check Hector asked for specifically.

##### The existing tool: agreement between the two laps.

§17.13.1 already introduces comparetests.m in passing, for two independent drives labelled Test 1 and Test 2; here it is in full, on this pair:

cd ~/TOUT/tools
matlab -batch "compare_tests('../web/exports/test1', '../web/exports/test2')"

By design, it reads Test 1’s MINS trace and Test 2’s openVINS trace specifically (not both traces of each), and reports trajectory shape, path length, loop-closure error and final-versus-initial heading: the right metrics for two separate roulages, since there is no shared timestamp to align them point-by-point the way §17.13.1’s Kabsch recalage does for a single simultaneous drive.

##### The new comparison: both against the tape measure.

- Put both rectangle results side by side. From the two console outputs of the subsections above (or the two rect.png annotations), tabulate: the measured length and width (identical for both, it is the same physical rectangle), each estimator’s achieved length and width, each estimator’s percentage error on each side, and each estimator’s loop-closure gap.

- Read the percentage errors before the closure gap. A small closure gap with a large dimensional error is possible: an estimator that consistently under-scales the whole trajectory can still return close to its own starting point, because the scale error cancels out over a closed loop. The dimensional comparison is what catches that; §12.14.2 makes the same point about scale hiding inside an alignment-based metric.

- Whichever estimator is closer on both dimensions is the one with the more accurate metric scale on this platform, on this floor, on these two laps. This is not a general verdict from a single pair of drives, particularly since Test 1 and Test 2 are two different physical laps, not the same motion measured twice: repeat in both directions around the rectangle before trusting the comparison, the way §17.13.3 insists on three passes for the ground-truth floor protocol before applying anything.

Run 2026-08-05: both tests exist, and the comparison is not a fair one

|  | Test 1 — MINS | Test 2 — openVINS |
| --- | --- | --- |
| Length error | −3.6 %​ | +10443.0 %​ |
| Width error | +13.4 %​ | +2679.0 %​ |
| Loop closure | 0.449 m (1.9 %) | 635.6 m (23.0 %) |
| Heading drift | 5.1∘​ | 5.9∘​ |

Both figures came from the same comparetests.m and plotrectangletest.m calls this appendix specifies, against the identical physical rectangle, in the same session. Read plainly, they are not close: openVINS’s dimensional error runs three orders of magnitude past MINS’s. Do not report this as “MINS outperforms openVINS by 100×.” That framing implies two comparable algorithmic results, one merely worse than the other, and §12.16.5 shows it is not that: MINS completed a valid acquisition, openVINS’s acquisition failed for reasons independently diagnosed and platform-specific — a thermal envelope this session sat inside (§12.16.4), IMU gaps at 80 % of the automated abort threshold, and an uncorrected mount tilt (§12.16.2) doubly integrating into exactly the kind of drift openVINS has no wheel odometry to resist. The near-identical heading drift between the two (5.1∘ vs 5.9∘) is the tell: an estimator whose orientation tracking were fundamentally broken would show it here too, and neither does. What this comparison honestly establishes is narrower and still useful: MINS is validated end to end on this platform, against a physical reference, today; openVINS is not yet, and §[tab:openitems] records what a fair re-attempt requires — a cooled Pi, a lit floor, and ideally the still-outstanding Kalibr camera–IMU extrinsic (§17.13.7).

### Where each estimator keeps its configuration

Each estimator reads a small tree of YAML files; the launch files select which tree by name.

##### openVINS.

A single estimator config plus its Kalibr chains, all under catkinws/src/openvins/config/leo/:

config/leo/estimator_config.yaml    # filter: init, ZUPT, feature counts,
                                    #   noise, do_calib_* flags, topics
config/leo/kalibr_imucam_chain.yaml # camera intrinsics + T_imu_cam extrinsics
config/leo/kalibr_imu_chain.yaml    # IMU noise densities / random walks

The launch node ovmsckf/runsubscribemsckf takes the config path as argv[1] (a launch <param name="configpath"> is read only on the ROS2 build, a real footgun documented in navigationsupervision.launch). Key platform-specific values (§12.3.5, §12.3.2): initdynuse: false and initimuthresh: 0.4 (jerk-triggered initialisation, the rover must be nudged to initialise VINS), usestereo: true, maxcameras: 2, and the frozen intrinsics (docalibint: false). Updated 2026-09-07: extrinsic refinement is now off as well — openVINS carries calibcamextrinsics: false and calibcamtimeoffset: false, so nothing about the camera–IMU geometry is estimated online any more. Only MINS still refines the camera time offset (docalibdt: true); its wheel branch has both frozen (docalibext: false, docalibdt: false).

##### MINS.

A per-sensor tree of ten files, selected by the config launch arg (mins.launch: <arg name="config" default="leo"/>), under minsws/src/MINS/mins/config/leo/. MINS initialises standing still (static IMU+wheel init, 4–12 s, §12.3.5), which is why it is the default served source. Its transform output is isolated to /tfmins (mins.launch, §12.2); the Carolus global fix enters through the VICON slot on /mins/externalref/carolus. For the full file-by-file listing, what in each file is platform-specific, and a complete download-to-test walkthrough, see §17.10.3.

##### Beacon / AprilTag.

The autonomy beacon config lives with the project package: catkinws/src/leoautonomy/config/apriltag.yaml (family tag36h11, full-resolution tagdecimate: 1.0) and tags.yaml (the physical tag: id: 285, size: 0.2032 m). The Carolus blob detector’s parameters are in carolusws/src/launch/carolusBlobDetection.launch, including the four LED knownpoints (§12.12.3); the cockpit’s Carolus panel (§12.12.2) parses this same file.

## Launch file architecture

Why this section exists: the configuration files tell each program what to believe; the launch files decide which programs exist, in what order, and what happens when one of them dies. That last part is a design decision on this platform, not a default, and it was arrived at by watching the stack fail.

Everything on the workstation starts from one file. Figure 17.3 shows the inclusion tree.

The launch inclusion tree on the workstation. One entry point, three includes, and a deliberate absence: no node is allowed to take the stack down with it. The robot’s own nodes are not here: they come from leobringup.launch running on the Pi (§17.8).

### What each file owns

- navigationmaster.launch: the only file you should ever launch directly. It declares one argument, config (default leo), which it forwards to mins.launch, and then includes the three files below. It contains no nodes of its own.

- tfstaticextrinsics.launch: the static coordinate transforms the platform would otherwise be missing, principally baselink → cameralink. Separated so the geometry can be corrected without touching the estimators.

- mins.launch: the MINS estimator, and nothing else.

- navigationsupervision.launch: everything else: the four image republish nodes and four camerainfo relays that bring the robot’s compressed streams into a local namespace, our four Python nodes, ovmsckf, the cockpit backend and the video server.

It does not start robotstatepublisher, and that is correct: the URDF and the robot’s own drivers belong to leobringup.launch, which runs on the Pi. Every file in this package runs on the workstation.

### Reliability: why nothing is required="true"

This is the most important design decision in the launch layer, and it is the reverse of what it was.

roslaunch offers two policies when a node dies. required="true" means if this node exits, kill everything; respawn="true" means restart it and leave the rest alone.

MINS was originally required="true", on the reasonable-sounding argument that restarting an estimator mid-flight without re-initialisation is meaningless, so a crash should be loud. On 2026-07-06 that policy was observed doing real damage: MINS aborted on an assertion (SystemManager.cpp:395, a backward IMU timestamp), and because it was required, roslaunch tore down the entire stack, including leobackend. The operator lost the ability to drive the robot because an estimator had hiccupped.

The principle established from that, and worth carrying to any similar system:

Driving must never depend on estimator health.

Every node is now respawn="true" with respawndelay="5". The five-second delay matters: without it, a node that fails at startup (a bad config, a missing topic) respawns in a tight loop that saturates a CPU and floods the log, which on a thermally limited platform is its own failure.

The cost, stated plainly. On respawn, MINS re-initialises at a new origin, so /mins/imu/odom jumps. The poseselector’s SE(3) correction does not absorb that jump: it only compensates at the moment of a deliberate source switch. So a mid-run MINS respawn produces a visible discontinuity in /robotposefused. That is an accepted trade-off: a discontinuous trajectory is recoverable, a dead cockpit during a manoeuvre is not.

A stale comment to be aware of. The header of mins.launch still claims MINS “stays required="true"”. It does not: the node declaration twenty lines below says respawn="true", and a second comment on the same node explains the change. The header was simply not updated. Trust the declaration, not the prose above it; this is exactly the kind of contradiction that misleads a successor.

### Passing a config path: args, never <param>

Both estimators take their YAML path as a positional command-line argument, and getting this wrong produces one of the most misleading failures in the whole system.

<!-- CORRECT -->
<node name="mins_subscribe" pkg="mins" type="subscribe"
      respawn="true" respawn_delay="5"
      launch-prefix="nice -n -10 stdbuf -oL"
      args="$(find mins)/config/$(arg config)/config.yaml">
</node>

<!-- WRONG: silently ignored on the ROS1 build -->
<node name="mins_subscribe" pkg="mins" type="subscribe">
  <param name="config_path" value="..."/>
</node>

Why the second form fails, and fails confusingly. In the upstream source, the call that would read a configpath ROS parameter sits inside an #if ROSAVAILABLE == 2 block: it is compiled out of the ROS1 build entirely. The program therefore falls back to reading argv[1]. But roslaunch appends its own arguments to every node, so argv[1] becomes name:=minssubscribe, and the estimator reports that it cannot open a configuration file with that literal name. The error message names a string you never typed, which sends you looking in the wrong place.

ovmsckf has exactly the same contract, for exactly the same reason. Both are documented in the launch files themselves so the trap is not rediscovered.

### Two launch-prefix tricks used here

- nice -n -10 on MINS. The workstation’s own IDE indexers were measured starving the estimator of CPU, which produced episodes that looked like estimator bugs. A negative nice value requires permission, granted to this user in limits.conf.

- stdbuf -oL on MINS. Its initialisation diagnostics go to plain stdout. When roslaunch redirects that to a file, glibc switches from line-buffered to fully-buffered, and at this output volume the [INIT] messages can sit unflushed for minutes: looking exactly like a hung estimator when it is initialising normally. Forcing line buffering makes the diagnostics appear as they are printed.

The robot side uses the same idea for a different purpose: launch-prefix="chrt -f 25" on serialnode, discussed in §17.8.2.

### How to add a node

- Put the script in catkinws/src/leonavigation/scripts/ and make it executable (chmod +x). A non-executable script produces cannot locate node of type [...], which reads like a missing file.

- Declare it in navigationsupervision.launch, with respawn="true" respawndelay="5" unless you have a specific reason otherwise, and write that reason in a comment.

- Add output="screen" if you want its log in logs/navigationmaster.log. Without it, diagnostics are difficult to find.

- Restart with bash tools/restartstack.sh, never a bare roslaunch (§17.5.5).

- Verify it registered and publishes:

- rosnode list | grep my_node
rostopic hz /my_topic

- A node can register and stay silent; only the second command proves it works.

### Two files the diagram does not show, and why

##### poseselector.launch.

The diagram above shows poseselector as a node inside navigationsupervision.launch, which is how it runs in normal operation. A standalone poseselector.launch also exists, for bringing the selector up alone against an already-running stack when you are debugging source switching. Whichever route you use, one parameter decides whether the TF tree is well formed:

<param name="base_frame" value="base_footprint"/>

Set to baselink instead, it gives that frame a second parent and the pose starts alternating silently between two coherent answers (§17.17.10). Check it in both files if you ever edit one.

##### carolustfbridge.py.

Absent from every launch file, and deliberately so. It is the beacon-anchored absolute-error bridge of §17.13.5, and it freezes a static beaconlink → odom transform at the first fix. A node that silently re-anchors the world frame has no business starting automatically alongside an unrelated run, so it is started by hand, by an operator who intends to use it:

rosrun leo_navigation carolus_tf_bridge.py

If you later decide to add it to a launch file, leave ~publishbodychain at its default of false: enabling it re-parents baselink and corrupts the tree in the same way described above.

## Launching the system, step by step

Why an order: the master must exist before clients, and the estimators must see sensor data from their first sample or they initialise on nothing. The leo command sequences all of this; the value of knowing the order is being able to diagnose a partial start, which is the common case.

### The nominal sequence

- Power the robot and wait ∼40 s. Its own leo.service brings up the ROS master and the sensors before anything on the PC can connect.

- Check the link first. From any directory:

- cd ~/TOUT && source tools/robot_env.sh
ping -c 3 "$ROBOT_HOST"

- Expected: 0% packet loss. If this fails, stop: everything after it will fail in a more confusing way.

- Start the workstation side. Two scripts, in this order. The first brings up the web stack (static server on 8000, rosbridge on 9090, the TLS bridge on 9443, the video server on 8080, and cloudflared if configured); the second brings up the navigation stack (MINS, openVINS, poseselector, leobackend). Both are idempotent, so re-running them is safe.

- cd ~/TOUT && source tools/robot_env.sh
touch /tmp/leo_maintenance          # suspend the watchdog while starting
bash web/start_web.sh
bash tools/restart_stack.sh
rm -f /tmp/leo_maintenance          # DO NOT FORGET THIS

- Expected: a series of [ok] lines from the first script and a launch log from the second. No ECHEC line.

- Why the maintenance flag: the cron watchdog restarts anything it believes is dead, and a stack that is halfway up looks exactly like a stack that has died. Without the flag the two of you will fight (§17.17.3). The flag is a plain file, so if you forget to remove it the watchdog stays disabled indefinitely and you lose every automatic recovery in this document.

- Confirm the robot is seen:

- ping -c 1 -W 1 "$ROBOT_HOST" && echo "robot online"
ss -ltn | grep -E ':(8000|8080|9090|9443) '   # four listening ports

- Expected: robot online, and four ports bound.

- A note on leo start: the workstation used for this project carried a convenience wrapper at ~/bin/leo that performed the sequence above plus a status view. It lives outside the project tree and is not part of the repository, so it will not exist on your machine. The commands above are what it actually ran; use them directly, or write your own wrapper around them.

- Confirm the four nodes that matter registered:

- source /opt/ros/noetic/setup.bash
rosnode list | grep -E "mins_subscribe|ov_msckf|pose_selector|imu_sanitizer"

- Expected: all four. A missing ovmsckf almost always means it aborted at startup: look in logs/navigationmaster.log, and suspect the YAML trap of §17.10.1 first.

- Check the rates, not just the names. A registered node can be completely silent; this is the real health test:

- rostopic hz /imu/data_clean                # expect ~80 Hz
rostopic hz /mins/imu/odom                 # expect ~80-90 Hz
rostopic hz /wheel_odom_with_covariance    # expect ~19-20 Hz
rostopic hz /pc/camera/infra1/image_rect_raw   # expect ~15 Hz

- /wheelodomwithcovariance at 1 Hz instead of 19 is the signature of the UART problem of §17.17.7.

- Open the cockpit at http://localhost:8000/ops.html.

### Making openVINS start: the classic beginner trap

MINS initialises standing still. openVINS does not. It waits for an accelerometer jerk above initimuthresh before publishing anything at all. Newcomers conclude openVINS is broken when it is merely waiting.

- Drive the rover briskly forward, then back, about one second each way.

- Confirm it woke up:

- rostopic hz /ov_msckf/odomimu           # expect ~75-90 Hz

- Silence means it has not initialised. Jerk it again: more sharply.

### Restarting cleanly when something is wrong

Do not kill individual nodes as a first response; several are respawn-supervised and will come back in a state you did not intend.

cd ~/TOUT
bash tools/restart_stack.sh

Expected: stack relancée, then MINS initialisé après <N>s, then switched to MINS, then TERMINE. Typical N is 8–12 s.

This script also raises the maintenance flag while it works, waits for MINS to publish before declaring success, and reapplies the camera settings that reset on restart (§17.8.3).

### A short tour of the cockpit

- Operations: live video, mission state, battery, and the Pose Source switch (MINS/VINS). Manual driving is here.

- Trajectory: the live plot of the served pose, the Test 1 / Test 2 selector, the MATLAB export buttons, and the ground-truth calibration panel (§17.13.3).

- Logbook, the project journal and the download card for this report.

- PID / Nav Modes / Robot, tuning, autonomy state, and the URDF view.

The cockpit is served over an authenticated Cloudflare tunnel as well, so it works from a phone or tablet standing next to the robot, which is how the calibration is meant to be done.

### Editing the cockpit without breaking it

Two habits, both learned the hard way, and both cheap.

##### Your edit is live but you are not seeing it.

This is the first thing that will happen to you, and it is not your edit. Cloudflare caches aggressively and a dashboard rule overrides the Cache-Control: no-cache that serve.py sends. Static assets therefore carry a version stamp in their query string:

<script src="app.js?v=r37"></script>

The query string is part of Cloudflare’s default cache key, so incrementing that number is what forces a fresh copy. Bump it in every page that references the file you changed, and keep the stamps consistent across pages, since they have drifted apart before and produced two pages running different versions of the same script.

A browser force-reload does not help, because the stale copy sits at the network edge and not on your machine. If you are testing over http://localhost:8000 you will not see the problem at all, which is exactly why it surprises people the first time they check the public URL.

##### Validate JavaScript before you reload.

app.js is large and a brace inserted in the wrong place produces a file that loads without error and simply stops executing partway through, so the page renders and half the panels are dead. Counting braces by hand does not catch this; it failed once in this project and cost an afternoon. Parse the file instead:

pip install esprima                     # once
python3 -c "import esprima,sys; esprima.parseScript(open('web/app.js').read()); print('OK')"

Do the same for the inline scripts in ops.html if you touch them. A parse takes under a second and turns a silent partial failure into a line number.

### Two pages, deliberately different: control versus demonstration

Two pages, deliberately different (cockpit v4). web/ops.html is the control surface: it holds every command path, manual driving, AUTO engagement, pose-source switching, beacon reset, MATLAB export. web/demo.html subscribes to the same topics and publishes nothing at all. It has no command bindings to disable, because it has none to begin with.

The separation is not cosmetic. A defence is the one occasion where the machine is driven by someone who is talking to a room rather than watching the robot, and where a mis-click is expensive and public. demo.html is built so that no mis-click exists: it shows the trajectory in the beacon frame, heading quivers, and the RMS error at 56 px, and it cannot move the rover. Open it on the projector; keep ops.html on your own screen.

##### Why read-only is a structural property, not a setting.

The distinction matters because the two obvious alternatives are both worse. Disabling the controls with a flag leaves the command paths present in the page, one localStorage key or one console call away from being live again, and it invites the reader of the code to wonder which state the page is in. Hiding them with CSS is worse still, since the handlers remain bound and a keyboard shortcut or a stray focus event can fire them. demo.html instead subscribes and never advertises: it holds no publisher, no command topic and no data-cmd binding anywhere in its markup. There is no state in which it can move the robot, so there is no state to verify before a defence.

##### What each page is for.

@lL4.5cmL4.5cm@ & ops.html & demo.html
Audience & the operator, alone & a room, during a defence
ROS role & subscriber and publisher & subscriber only
Commands & manual drive, AUTO, pose source, beacon reset, MATLAB export & none exist
Failure mode & a mis-click moves the rover & a mis-click does nothing
Where to open it & your own screen & the projector

A practical consequence for the transport. Because demo.html never publishes, it is immune to the entire class of symptoms described in §17.6.5: a zombie TLS port makes buttons inert, and a page with no buttons cannot present that symptom. If the demonstration view is updating and the control view is not, you have localised the fault to the outbound direction of the WebSocket before running a single command.

## Reproducing the experiments

### A MINS versus openVINS comparison run

Why a rosbag and not the website: the cockpit’s in-memory buffer is lost if the backend restarts, and rosbridge can desynchronise. A run that matters must be written to disk by a process that does not depend on the web stack.

- Pre-flight. Run the checklist of §17.17.15. This is not optional, most of the discarded data in this project failed one of those six items.

- Wake openVINS with a jerk, and confirm both estimators publish.

- Mark a start point on the floor with tape. You will drive a closed loop and return to it; the gap on return is the only drift measure available without ground truth.

- Start recording (from ~/TOUT):

- cd ~/TOUT
tools/record_trajectories.sh 180 essai_A

- Records 180 s into data/trajectories/essaiA<timestamp>.bag. Omit the duration to record until Ctrl-C. Both estimators are always captured together: that is the entire point.

- Drive the loop and return to the marked point. Drive slowly and smoothly; note the commanded speed.

- Convert to CSV:

- python3 tools/bag_to_csv.py data/trajectories/essai_A_<timestamp>.bag

- Expected: one CSV per source, mins.csv, vins.csv, and carolus.csv if a beacon was visible. Columns are t,x,y,z,qx,qy,qz,qw,yawrad, so any tool can read them: MATLAB is a convenience, not a requirement.

- Plot and get the numbers (run from tools/ so MATLAB finds the script):

- cd ~/TOUT/tools
matlab -batch "plot_trajectories('../data/trajectories/essai_A_<ts>_mins.csv')"

- Expected: a printed metric block (divergence, scale, path lengths, heading gap) and a saved comparison.png plus eight p1..p8.png panels.

- Put it in the report: copy comparison.png to reportlatex/figures/trajcomparison.png and the panels to trajp1.png…trajp8.png, then recompile (§17.14).

Two independent drives instead of one? Use tools/comparetests.m on two runs labelled Test 1 and Test 2. It deliberately does not compute the Kabsch divergence: that requires both estimators observing the same motion at the same instants, which two separate drives are not. It reports loop-closure error and heading drift instead, which are meaningful for independent runs.

### How to read the nine-panel figure

The panels are described one by one in §12.14.1, and the mathematics behind panels 2–4 in §12.14.2 and §12.14.3. Two habits will keep you honest:

- Read panels 2 and 4 together, never separately. Panel 2 aligns the trajectories including a scale factor, so it is constructed to hide a scale disagreement. Panel 4 is the only one that exposes it. On the July 29 run, 47% of the residual was pure scale error.

- The scale factor was fitted to MINS, and MINS is not ground truth. The script prints a warning whenever the fitted scale departs from unity by more than 3%. Quoting the scale-corrected residual alone would be quoting a number built to be insensitive to the error you are measuring.

### Ground-truth calibration: the most valuable measurement

Everything above compares estimators to each other. This is the one procedure that compares them to reality, and it is what a new student should do first.

- Build the measurement zone. Follow docs/protocolecalibrationterrain.md. The essentials: construct a right angle with a 3-4-5 triangle (90, 120, 150 cm, a tape measure does this to 0.08∘, where a protractor manages ±1∘), and hang two plumb lines from the robot’s centreline so its heading is marked on the floor without parallax.

- Open the Trajectory tab on a phone or tablet and pick a manoeuvre: 90° G, 90° D, or 1 m.

- Mark start, drive the manoeuvre, mark end. The backend freezes that instant: take as long as the measurement needs.

- Measure physically. For a rotation, read the lateral offset d of the far plumb mark from the reference line over the lever arm L, and compute the residual ε=arctan(d/L); the true angle is 90∘∓ε. With L=1 m and d read to ±2 mm this gives ±0.11∘.

- Enter what you measured, never the value you aimed for. Then validate.

- Three passes per manoeuvre, both directions. The panel reports the mean factor and its standard deviation, and refuses an entry that disagrees with all four sensors by more than a factor of five.

The reasoning that matters more than the numbers. The first campaign found the gyroscope over-reporting by 7% and the wheels by 9%. Wheel-derived heading comes from differential odometry and does not use the gyroscope: these are physically independent instruments. MINS and openVINS are not independent witnesses, since both integrate the same gyro.

Two independent sensors agreeing with each other and disagreeing with the reference points at the reference. So the conclusion was not “apply a 7% correction” but “re-measure with a better floor protocol”. Decide between the three outcomes:

| Result over 3 passes | What it means |
| --- | --- |
| both →1.00 | the first campaign was measuring the protractor. Change nothing; leave gyroscale at 1.0. |
| both stay ≈0.93 | a real shared error: applicable, but you must then explain why two different physical principles drift by the same amount. |
| they diverge | each has its own defect. Treat separately: gyroscale for the gyro, effective radius / wheel base for the wheels. |

Standard deviation above 5% means the method still dominates: do not apply anything, improve the marking.

### Protocol T1.1: static drift and variance analysis

Chapter 12 (§12.7.2) uses this protocol to separate two things that look identical in a single trajectory plot but demand opposite fixes: noise (bounded, average it away) and drift (unbounded, points at an unmodelled bias in extrinsics or initialisation). It requires the rover to do nothing at all, which makes it the cheapest measurement in this guide and, per Table 17.2, a fully automated one.

- Fresh initialisation. Restart the stack (§17.12) rather than reuse a session that has been running for a while: T1.1 characterises the estimator from a cold start, and a warm one hides exactly the divergence it exists to catch.

- Place the rover and do not touch it again. Flat ground, clear of anything that would tempt you to nudge it once logging starts.

- Log for five minutes (300 s is the script’s own default, matching the campaign’s measurement):

- cd ~/TOUT/tools
python3 log_pose_static.py 300 /tmp/t1_1_static_pose.csv

- The console prints Logging /robotposefused for 300s -- keep the rover PERFECTLY STILL., then, after five minutes, Wrote <N> samples to /tmp/t1staticpose.csv with N close to 300×20=6000 at the nominal 20 Hz publish rate.

- Analyse:

- python3 analyze_pose_variance.py /tmp/t1_1_static_pose.csv

- This reports, per axis: standard deviation (spread around the mean, a noise/covariance question if bounded), a least-squares linear drift rate in m/s (near zero means noise; a clear nonzero slope means genuine unbounded divergence), and a first-10%-versus-last-10%-of-the-window mean as a second, independent check of the same conclusion.

The verdict this campaign used: drift below 1 mm/s on every axis passes. Above it, do not touch a noise parameter — Eq. [eq:chi2]’s χ2 gate and the covariances upstream of it are tuned for noise, not for a genuine bias, and loosening them to accommodate an unbounded drift will make the estimator accept bad data rather than fix the cause. A failing T1.1 run points at extrinsics or initialisation (§17.10.1), never at a filter gain.

### Absolute error: anchoring the trajectory to the beacon

READ THIS BEFORE PUBLISHING ANY NUMBER FROM THIS SECTION THE COLOUR CAMERA INTRINSICS ARE WRONG AND MUST BE RE-CALIBRATED FIRST. The detector runs with parameters calibrated at 1280×720 against a 640×480 stream. Its stored principal point, cx′=643.47, lies outside a 640-pixel-wide image.

Consequences, derived in §12.15.3: every range is inflated by 69.3 %, and the lateral estimate carries a fixed offset ΔX=ΔcxL/w that translates the entire scene. The result is smooth, repeatable, internally consistent and metrically meaningless, which is the most dangerous combination a measurement can present.

The procedure below is correct and ready to run. Its output is not publishable as a dimensional quantity until the calibration of §17.18.1 has been done. Treat what follows as a rehearsal until then.

Why this replaces the alignment-based error. Every figure in Chapter 12 up to §12.14.2 is a residual left after fitting a transform between two trajectories, by Kabsch (rigid) or Umeyama (with scale). §12.15.2 shows that such a fit is an orthogonal projection away from a seven-dimensional subspace G of translations, rotations and scale, and that the dominant estimator failure modes, heading drift and scale drift, are basis vectors of that subspace. An alignment-based metric is therefore blind by construction to the very errors the comparison exists to expose, and every number it reports is a lower bound.

The bridge removes the fit rather than improving it. The transform is established once, physically, by anchoring, and thereafter held constant, so no parameter is estimated at measurement time.

##### What carolustfbridge.py does.

It publishes exactly two transforms and derives two topics from them:

- cameraframe → carolusraw, live, from each optical detection of the beacon.

- beaconlink → odom, static, captured at the first fix and then frozen. This is the anchoring act, and it is the whole point: from this instant the odometric world and the optical world share one absolute reference.

- /odominbeacon and /carolusinbeacon, both carrying the robot’s pose in the beacon frame, reached respectively through the odometry/estimator chain and through the optical detection. Their instantaneous difference is the absolute error.

##### Step by step.

- Place the beacon where it stays visible for the whole planned loop, and do not move it afterwards. Moving the beacon after anchoring invalidates the run silently, because the frozen transform no longer describes reality and nothing detects the discrepancy.

- Confirm the detector is actually seeing it before starting anything else:

- rostopic hz /pose            # non-zero => the beacon is detected
rosnode info /carolus_astrobee_rex | head -20

- A rate of 0 Hz with Not enough blobs < 4 in the log means the LEDs are not resolved: adjust distance, exposure or lighting. The bridge cannot anchor to something that was never detected.

- Start the bridge by hand. It is deliberately absent from every launch file, so that it can never perturb a run you did not intend it to touch:

- rosrun leo_navigation carolus_tf_bridge.py

- Verify the anchor took, and that the TF tree is still well-formed (§17.17.10):

- rostopic echo -n1 /odom_in_beacon
rostopic echo -n1 /carolus_in_beacon
rosrun tf tf_monitor 2>&1 | head -30     # one parent per frame

- Record both topics, along with the usual estimator outputs. This is the step that has never been done, and its absence is why no absolute error appears anywhere in this report:

- rosbag record -O run_absolute.bag \
  /odom_in_beacon /carolus_in_beacon \
  /mins/imu/odom /ov_msckf/odomimu /robot_pose_fused /firmware/wheel_odom

- Drive the loop and return near the start. Keep it short, for the thermal reasons of §17.17.2.

- Compute the error as the instantaneous difference between the two beacon-frame topics. No alignment step is involved, and none should be added: adding one would reintroduce exactly the projection this method exists to remove.

Never let the bridge publish the body chain The bridge exposes a ~publishbodychain parameter which defaults to false and logs an error if enabled. An early revision published the full body chain in the beacon frame, which gave baselink a second parent and corrupted the tree (§17.17.10). The lookups do not fail; they alternate between two plausible answers. Leave the parameter alone.

### Camera–IMU calibration with Kalibr

Both estimators consume the same physical calibration, produced offline and then frozen. Offline measurement beats online refinement: that is why calibcamextrinsics is false.

- Target. Print a Kalibr target (an Aprilgrid is preferable to a checkerboard for full-view corner observability) and record its exact geometry in a target.yaml.

- Intrinsics and stereo extrinsics. Record a bag waving the target across the whole field of view of both infrared cameras, then run kalibrcalibratecameras on the two image topics. This produces the *-camchain.yaml. Achieved on this D455 pair: 0.20–0.23 px reprojection error at a 90.2 mm baseline.

- Camera–IMU extrinsics and time offset. With intrinsics fixed, record a second bag exciting all IMU axes and run kalibrcalibrateimucamera.

- Install the outputs: openVINS reads them directly as kalibrimucamchain.yaml and kalibrimuchain.yaml; MINS’s configcamera.yaml is filled from the same camchain via fillminscamchain.py, remembering the inverse-convention trap.

- Verify with the live reprojection monitor (calibrationmonitor.py). Acceptance threshold used throughout this project: <0.5 px.

Standing caveat, and the recommended first task. Step 3 has never been completed on this platform. The deployed camera–IMU extrinsic is a tape-measure seed. The blocker is physical: the procedure needs roll and pitch excitation of ≳1.5 rad/s, and a rover on flat ground has only ever achieved 0.08. You must lift and tilt the robot by hand in front of the target. Doing this is the single most valuable calibration task outstanding.

### Calibration methodology, in full

Both estimators consume the same physical calibration, produced offline with Kalibr (Furgale et al. 2013) and then frozen (offline measurement beats online refinement, §12.3). The procedure, reproducible from scratch:

- Target. Print a Kalibr calibration target (an Aprilgrid is recommended over a checkerboard for full-view corner observability) and record its exact geometry in a target.yaml.

- Intrinsics + stereo extrinsics. Record a rosbag waving the target across the full field of view of both IR cameras, then run kalibrcalibratecameras on the two image topics. This yields the *-camchain.yaml (intrinsics per camera, plus the stereo baseline). On this D455 pair the achieved reprojection error was 0.20–0.23 px at a 90.2 mm baseline (§12.3); intrinsics are then frozen (docalibint: false).

- Camera–IMU extrinsics + time offset. With the intrinsic camchain fixed, record a second bag exciting all IMU axes (rotate and translate the rig) and run kalibrcalibrateimucamera with the camchain, the imu.yaml noise model and the target. This produces the *-camchain-imucam.yaml with ​ITC and the camera–IMU time offset td. The correct forward-looking mount matters: an identity placeholder here was the root cause of the vertical divergence fixed in §12.3.2 (Eq. [eq:extr]). td and the translation were refined online (docalibext, docalibdt) until 2026-09-07; extrinsic refinement is now disabled, so only td is still estimated, and the translation seed stands as installed. The 90∘ rotation must in any case be seeded correctly: it is outside the filter’s recoverable region, and always was.

- Install. Copy the Kalibr outputs into each estimator’s config: openVINS reads them as config/leo/kalibrimucamchain.yaml and kalibrimuchain.yaml directly; MINS’s configcamera.yaml is populated from the same camchain (the prepared fillminscamchain.py helper maps one to the other). The IMU noise densities in kalibrimuchain.yaml come from an Allan-variance characterisation of the body IMU carried on the CORE2 control board (or the manufacturer figures inflated for margin). That is the only inertial source any estimator consumes, along the chain /firmware/imu → /imu/dataraw → /imu/dataclean (and /imu/datavins for the two VINS branches).

- It is not the D455’s own IMU. That distinction matters because the two are not interchangeable: they sit in different frames, and the camera–IMU extrinsic ​ITC calibrated here is the transform to the body IMU. The D455’s motion module is physically present — it enumerates as a USB HID interface and the RealSense driver reports Motion Module was found — but it is disabled in the driver, and the streams it would publish on /camera/imu have never been used by this project (Table [tab:openitems]).

- Verify. The live reprojection-error monitor (calibrationmonitor.py, surfaced on the cockpit’s calibration panel) reports the current chain’s health; the acceptance threshold used throughout this project is <0.5 px (§12.3).

A standing caveat carried in the open-items registry: the deployed camera–IMU extrinsic is still a tape-measure seed refined online, not a full kalibrcalibrateimucamera solution (Table [tab:openitems], item 2); completing step 3 above from a dedicated bag is the recommended first calibration task for a new student.

## Publishing the report

cd ~/TOUT
bash tools/update_report.sh

Expected: three progress lines, then [3/3] OK, <N> pages, then a local and a public URL.

The script runs pdflatex three times with biber in between (needed for cross-references and the bibliography to settle), copies the PDF into web/reports/, and stamps the download link with a version number.

Why the version stamp exists. The Cloudflare tunnel served a four-hour-old PDF for hours despite the origin sending Cache-Control: no-cache, a dashboard rule overrides the origin header, and it is not editable from this machine. A query string is part of the cache key, so a fresh ?v= forces a miss. Use the URL the script prints; a bare URL may serve a stale report, and no amount of browser refreshing helps because the cache is at the network edge.

Two traps inside this script, both now fixed, both worth knowing: main.log contains non-ASCII bytes, so grep classes it as binary and silently suppresses its output, the error check saw nothing at all until -a was added, and thirteen real errors passed while the script reported success. And pdflatex wraps its log lines at ∼79 columns, so the phrase it was grepping for the page count could be split in two depending on document length; with set -e that killed the script before deployment.

What that public tunnel was also publishing (audit, 2026-08-18) web/serve.py subclassed SimpleHTTPRequestHandler and overrode only the cache headers — so it served the whole directory, directory indexes included, and that directory is on the public internet. Verified live, not assumed: https://cockpit.leo-rover-gardon.dev/exports/ answered 200 with a browsable index of 104 files (17 MB of experiment data: .mat, trajectory CSVs, figures), as did models/ and reports/. Eight .bak-cachefix-*.html pages from July 29 — complete, stale copies of the cockpit — were being served next to the real ones, plus thirteen more backups whose names do not begin with a dot (ops.html.bak-avant-v4-161908, app.js.bak-casse-203841), which is why a naive “hide dotfiles” rule is not enough.

The fix keeps the deny list aligned with section 10 of .gitignore, on the principle that what git refuses to archive, the server refuses to publish: any path segment starting with a dot, or carrying .bak, .orig, .rej or .swp, returns 404; directory listing is disabled outright. sendhead() still serves index.html, so pages are untouched, and individual files under exports/ remain reachable by direct URL — only enumeration disappears. Sixteen cases were unit-tested before deployment and re-checked through the public tunnel afterwards.

Opening and closing that exposure from the cockpit. The Public tunnel control in Operations → Commands starts and stops cloudflared through the tunnel action of leobackend.py. Killing the process is not enough on its own: leowatchdog.sh sees it dead within the minute and replays web/startweb.sh, which brings it straight back — the operator would conclude the button was broken. A closure therefore has to be declared, not merely executed, so the backend drops /tmp/leotunneloff and both scripts consult it before restarting anything. The flag deliberately lives in /tmp: a reboot clears it and restores public access, which is the less surprising behaviour of the two (nobody wants a site silently dark because of a click three weeks earlier). The telemetry carries up and disabled separately, because “not up” alone cannot distinguish a deliberate closure from a crash the watchdog is already repairing — the cockpit shows three states accordingly. Closing from a page itself loaded through the tunnel is allowed, but warns in as many words that this page is about to disconnect and that reopening will require the local address. Verified end to end: closed (public 530, local 200), still closed after 95 s and a full watchdog tick, then reopened (public 200).

## The handoff package: helping another lab reproduce this setup

This section documents leoroverpourconnor.zip, a self-contained archive built to answer a specific, recurring request: another team has the same hardware, their estimator will not stay stable, and they need a known-good reference to compare against. It was first assembled in August 2026 for a colleague at Florida Tech whose MINS instance diverged within seconds of startup, and the same package answers the same question for anyone who follows.

When to send it. Use it when someone reports an estimator that goes unstable immediately rather than drifting slowly. That symptom is almost never a defect in MINS or openVINS; it is a configuration or calibration mismatch, and the fastest resolution is a line-by-line diff against configuration files that are known to run. Prose describing the settings is far slower and far less reliable than the files themselves.

### What is inside, and why each piece is there

leo_rover_pour_connor.zip
  README.txt                     <- read this first, English
  config_MINS_leo/               <- the 7 live MINS config files
    config.yaml
    config_imu.yaml
    config_camera.yaml
    config_estimator.yaml
    config_init.yaml
    config_wheel.yaml
    config_system.yaml
  documentation/
    main.pdf                              <- this report
    patches_sqrtVINS_README.md
    patches_openVINS_README.md
    rpng_sqrtvins_zupt_bug_report.md
    robot_sqrtVINS_config_README.md

configMINSleo/ holds the seven MINS configuration files exactly as they run on this rover, copied from LEORoverNavigationSystem/MINS-master/mins/config/leo/. These are the core of the package. The recipient should not adopt them wholesale — their extrinsics and noise densities belong to this unit — but should diff them against their own and examine every difference deliberately.

documentation/ holds the reasoning behind the values. main.pdf is this report, whose calibration sections explain the camera–IMU extrinsic convention and the inertial calibration limits. The two patches files document defects found in the estimators themselves, with measured before-and-after numbers; they matter because a recipient chasing a divergence may be chasing a known upstream bug rather than their own configuration. rpngsqrtvinszuptbugreport.md is the ZUPT bug report filed upstream. robotsqrtVINSconfigREADME.md explains, value by value, why each embedded parameter is set as it is — useful as a model of the reasoning even for a reader who never runs sqrtVINS.

Everything in the archive is written in English, including the comments inside the YAML files. This is deliberate: the working language of this repository is French, and an archive that leaks French comments into another lab’s configuration is a package that will be misread. If you rebuild the archive, preserve that property.

### The five causes worth checking first

The package is most useful when paired with a diagnostic order. Every case of immediate instability diagnosed on this platform so far traced back to one of the following. They are listed in the order worth testing — cheapest and most frequently implicated first — not in order of theoretical interest:

- Camera–IMU extrinsic, direction and frame convention. Kalibr emits Tcamimu (points in the IMU frame mapped into the camera frame); MINS expects the inverse. Loading one where the other is expected raises no error — it feeds the filter a wrong rotation from the first frame, and divergence follows within seconds. A single axis flip, or an optical-frame versus body-frame mismatch, produces the same signature. This is the most common cause by a wide margin.

- Static initialization window. MINS estimates its initial gravity direction and gyroscope bias from a stationary period at startup. If the rover is handled, bumped, or still settling during that window, the alignment locks in tilted, gravity leaks into the horizontal axes, and the doubly integrated residual produces an accelerating drift. Measured on this rover after an initialization disturbed by hardware work in progress: 772 m/min of vertical drift with the robot standing still. A clean restart with the platform untouched brought the same estimator to 0.02 m/min horizontal and 0.02 m/min vertical — a factor of roughly 3×104, from one variable.

- Time alignment between camera and IMU. Tens of milliseconds are enough to break the tight coupling. Check that a time offset is set explicitly, or that online estimation of it is enabled, particularly when the two sensors are served by separate drivers.

- IMU noise densities and random walks. Values taken from a datasheet rather than an Allan-variance characterization of the specific unit leave the filter overconfident: small residuals are amplified into large corrections instead of being absorbed. The failure does not look like a typo; it looks like a broken estimator.

- Publication rates and dropped frames. A degraded IMU rate or intermittently dropped images produces the same early instability and costs nothing to rule out with rostopic hz on both streams.

Check the cheap cause before the expensive one Cause 2 — an initialization that was not static — costs a single restart to test. Cause 1 requires reading and re-deriving a transform. Test in that order, not in order of intellectual interest. On the evening of 2026-08-31, roughly forty minutes were spent verifying inputs that all turned out to be healthy (IMU magnitude within 0.7 % of local gravity, wheel joint states at 19.5 Hz with no gap above 0.10 s, camera images with a pixel standard deviation near 31, and every MINS configuration file byte-identical to the committed version) before the initialization window was even considered.

### Rebuilding the archive

The archive is not tracked in Git; it is regenerated on demand so that it always carries the configuration currently in service. To rebuild it, stage the seven MINS configuration files and the five documents listed above, add an English README.txt describing each directory, and compress the result. Two checks before sending:

# 1. every YAML must still parse under OpenCV FileStorage,
#    which is stricter than a standard YAML parser
python3 -c "import cv2,sys; fs=cv2.FileStorage(sys.argv[1],0); \
            print(fs.isOpened())" <file>

# 2. no French must remain in any text file
LC_ALL=C grep -rlP "[\\x80-\\xFF]" <staging directory>

The second check exists because the first version of this archive shipped with three README files and two YAML comment blocks still in French, which would have forced the recipient to translate the very material meant to save them time.

### What the package deliberately does not contain

No source code, no build artefacts, and no workspace. The recipient is expected to have a working MINS build already — their problem is that it runs and diverges, not that it does not run. Including a workspace would add gigabytes, would embed absolute paths from this machine, and would invite the recipient to replace a build that is probably fine. The archive is a reference to compare against, not an installation.

It also contains no bag file. If a diff of the configuration files does not resolve the problem, the correct next step is to ask the recipient for a short recording of their system — ten to fifteen seconds covering the static initialization window and the first motion — which localizes the fault far faster than exchanging descriptions of it.

## Running MINS on the robot: a feasibility audit, and why the answer is no

The Lab holds two complementary configurations: a local MINS running on the robot’s own computer, and the LEO configuration documented here, where MINS runs on a ground workstation. This section records the measurements behind that split, so the decision is auditable rather than assumed — and so that a future student with faster embedded hardware can re-run the arithmetic instead of re-deriving it.

### What was compared

A colleague supplied a MINS tree (referred to here as MINSMC) with the suggestion that disabling dynamic cloning would let MINS run natively on the Raspberry Pi. Inspecting that tree gave three findings worth recording.

First, the tree is an unmodified upstream clone of rpng/mins, not a tuned configuration. Its only substantive change is mins/launch/subscribe.launch, converted from rosbag playback to live topic subscription with remaps — a genuinely useful piece of work, and the part worth adopting.

Second, the advice has a real provenance. The upstream RobomasterS1 profile ships with exactly clonefreq: 30 and dynamiccloning: false. The suggestion was not invented; it was read off a reference profile for a comparable small ground robot.

Third, and this is the trap: that profile and ours are tuned in opposite directions on different axes.

| Parameter | RobomasterS1 | our leo |
| --- | --- | --- |
| cameras | mono, usestereo: false | stereo, two cameras |
| downsample | true | false |
| docalibext / docalibdt | false / false | false / true​† |
| resolution | 640×360​ | 640×480​ |
| npts | 1500 | 300 |
| grid | 15×15 (225 cells) | 10×8 (80) |
| clonefreq | 30 | 20 |
| dynamiccloning | false | true |

​† Changed 2026-09-07. This row read true / true in earlier revisions. docalibext was set to false after the extrinsic was measured random-walking at ≈1.4 mm/min with the rover stationary — the quantity is unobservable without motion, and the drift eventually pushed updates outside the χ2 gate. Note the honest consequence for the argument below: on this row the two profiles now agree rather than oppose, and the contrast that follows rests on the remaining rows.

The RoboMaster profile saves heavily on the image path — mono, downsampled, no online calibration — and spends that budget on tracking: 1500 features, 225 grid cells, a fixed 30 Hz clone rate. Ours does the reverse: an expensive image path, but only 300 features and an adaptive clone rate. Since 2026-09-07 the two profiles also share the frozen-extrinsic choice, though they reached it for different reasons: RoboMaster as a compute saving, ours as a stability fix.

Transplanting two parameters out of a balanced profile Lifting clonefreq: 30 and dynamiccloning: false into the leo profile without also adopting mono and downsample combines the expensive half of both profiles. The settings are not wrong; they are load-bearing for a configuration whose other choices compensate for them. Parameters copied between profiles must be copied as a set, or not at all.

### The measurement

Cost on the workstation. With the robot stationary and dynamic cloning settled at its floor of 9 Hz — a best case, not a typical one — MINS consumes 85 % of one core of an Intel i7-8700. Its own timing breakdown reads CAM 18 ms, IMU 2 ms, wheels 0 ms: the camera path is 90 % of the cost, which is where any optimisation must act.

The 5.4× margin that is not a margin MINS prints Total: 20ms / 111ms (5.4X). That figure covers only its update path. Total process CPU is the 85 % measured externally — image callbacks, ROS deserialisation and the KLT front end are not counted in the 20 ms. Reading 5.4× as available headroom overstates the budget by roughly a factor of four, and would turn a no-go into an apparent go.

Relative performance. Rather than assume a ratio, both machines ran the same single-threaded floating-point benchmark: 436 versus 108 operations per second, so the Cortex-A72 is 4.05× slower per core than the i7-8700. This benchmark likely flatters the Pi, since its BLAS kernels are well optimised for NEON while MINS also runs OpenCV KLT and Eigen on small matrices, where the x86 advantage is wider. A planning range of 4–6× is therefore appropriate.

The arithmetic. At 4×, MINS needs roughly 3.4 of the Pi’s 4 cores; at 6×, more than the machine has. Of those four cores, about 2.3 are free with the camera driver running.

What settles it is thermal, not arithmetic. The Pi already idles at 70.6 ​∘C with the throttle flag armed. Sustained load drives it to the 80–85 ​∘C threshold, where the clock drops to 600 MHz — a further ≈2.5× loss, arriving exactly when the margin is thinnest. This platform has already been observed becoming unreachable under load. Placing the state estimator on the wrong side of that failure mode is not a tuning question.

### Verdict, and what would change it

MINS stays on the ground station. A leopi profile exists (mins/config/leopi/) applying the three levers that matter — mono, downsample: true, online calibration disabled — which are the same three the upstream RoboMaster profile pulls. It may well run. It is nonetheless not adopted, for two reasons stated plainly: the saving is estimated, not measured, and going mono removes stereo from a platform where the camera is the only sensor constraining z (§17.15 records what happens when that constraint is lost).

The decision would be revisited on different hardware — a Jetson-class board — not by tuning harder on this one.

### ROS 1, tf2, and the source changes that were already needed

Two platform decisions are settled and should not be revisited.

ROS 1 (Noetic) is structural, not preference. Both machines run Noetic exclusively. The binding constraint is the motor and sensor link: the CORE2 board speaks to the Pi over rosserial, which exists only for ROS 1. The ROS 2 equivalent is micro-ROS, which requires reflashing the microcontroller firmware — a hardware change, not a configuration one. Everything downstream (the rosmon supervision, the rosbridge cockpit, the three estimator workspaces) sits on that foundation.

The ROS 1 source changes are the tf1 to tf2 migration, and they are already done here. Upstream MINS still uses the legacy tf::TransformBroadcaster on its ROS 1 path. Four files carry it, and all four were migrated to tf2ros in this tree on 2026-07-22:

| File (ROS 1 path) | legacy tf occurrences upstream |
| --- | --- |
| mins/src/core/ROSPublisher.cpp | 12 |
| mins/src/sim/SimVisualizer.cpp | 5 |
| mins/src/core/ROSPublisher.h | 1 |
| mins/src/sim/SimVisualizer.h | 1 |

The ROS2* counterparts already used tf2 upstream and were unaffected. Anyone asking which files need source changes to run MINS under ROS 1 is asking about these four.

### Which camera: RGB or the IR stereo pair

For pose estimation on this platform the IR stereo pair is the correct choice, for one theoretical and one hardware reason.

The theoretical reason is scale: a calibrated stereo pair observes metric scale directly, where a monocular stream depends on the IMU and wheels to recover it. Both openVINS and sqrtVINS support stereo — usestereo: true, maxcameras: 2, sharing the same TrackKLT front end — so neither is limited to monocular, and both already run stereo here.

The hardware reason is specific to this unit and decides the matter regardless: the D455’s colour stream fails RGB8 format negotiation on this camera, root-caused earlier in the project. The infra1 mono8 feed is the confirmed reliable stream and is what every calibration recording in this work used. The cockpit displays greyscale IR for the same reason. The colour stream is additionally disabled by default because enabling it costs the camera nodelet 133 % of a core against 85 %.

An RGB-versus-IR accuracy comparison on this rover would therefore be measuring a defective stream against a working one, not two valid options. Repeating it on a unit with a healthy colour pipeline would be a fair test; on this one it is not.

## Troubleshooting and lessons learned

This section is the distilled cost of the campaign. Each entry is a real failure, its measured signature, and its fix. They are ordered by how much time they cost.

### The Pi overheats, and everything downstream lies

This is the most important entry in this appendix. The Pi runs at 86 ∘C, above its soft temperature limit, so the firmware caps the CPU at 600 MHz instead of 1.5 GHz. The consequences cascade, and none of them look thermal:

How to confirm it in one command (on the robot):

ssh pi@$ROBOT_HOST 'vcgencmd measure_temp; vcgencmd get_throttled; \
                    vcgencmd measure_clock arm'

Expected when healthy: throttled=0x0 and ∼1.5 GHz.
Observed at the time of writing: temp=86.2’C, throttled=0xe0006 (bits 1 and 2 set: frequency capped and throttled right now), 600 MHz.

The measurable damage. Count the holes in the IMU stream:

cd ~/TOUT && source tools/robot_env.sh
source /opt/ros/noetic/setup.bash
python3 - <<'EOF'
import rospy, time
from sensor_msgs.msg import Imu
rospy.init_node('g', anonymous=True, disable_signals=True)
S=[]
rospy.Subscriber('/imu/data_clean', Imu,
                 lambda m: S.append(m.header.stamp.to_sec()), queue_size=2000)
time.sleep(20)
d=[S[i]-S[i-1] for i in range(1,len(S))]
med=sorted(d)[len(d)//2]
g=[x for x in d if x>3*med]
print('%d samples, %.1f Hz, %d gaps, %.2f s lost' %
      (len(S), 1/med, len(g), sum(g)))
EOF

Healthy: zero gaps. Observed: 4 to 21 gaps per 20–40 s, each ∼39 ms (four consecutive samples missing). Those gaps make openVINS abort on Propagator.cpp:101, because the IMU readings between two filter clones no longer sum to the interval between them. It respawns, re-initialises, and the trajectory jumps.

The fix is a fan. Install active cooling before trusting any estimator measurement. An A/B test of the same gap count before and after cooling is the first experiment a new student should run.

Update, July 30: the envelope has since been measured on both sides. The claim in earlier editions that the software levers were exhausted rested on an invalid test (colour was already off at driver level, so the measurement compared a configuration with itself). The colour nodelet has since been measured at 185 % CPU. More usefully, the failure threshold is now bracketed rather than assumed:

|  | 86 ∘C | 89.1 ∘C |
| --- | --- | --- |
| Wheel odometry | 19.7 Hz | 0 Hz |
| IMU | 82.8 Hz | 0 Hz |
| UART overruns | 137 000​ | 676 751​ |
| Load average (4 cores) | not measured | 8.85​ |
| serialnode | synchronised | Lost sync loop |

Three degrees separate a working rover from one that answers neither manual nor autonomous commands, because /serialnode is the only subscriber of /cmdvel and it is the component that dies. If driving stops in both modes at once, look at the serial chain first: that simultaneity rules out the mission logic and the web interface before you have tested either. Recovery is a targeted cycle, not a stack restart:

rosservice call /rosmon/start_stop "{node: 'serial_node', ns: '', action: 2}"
sleep 4
rosservice call /rosmon/start_stop "{node: 'serial_node', ns: '', action: 1}"

This restores the link within about twenty seconds and the launch wrapper reapplies the SCHEDFIFO priority automatically, but it is a reset, not a repair. Watch the overrun counter afterwards (sudo cat /proc/tty/driver/ttyAMA): if it keeps climbing, and at 88 ∘C it does, the collapse will recur within tens of minutes.

### The hit-and-run protocol: working usefully on a throttled Pi

The previous entry says install a fan, and that remains the right advice. This entry is about what to do on the many days when you do not have one, because the honest answer is not “nothing”. A short run at 86 ∘C produced usable data; a longer one at 89 ∘C produced none at all and required manual recovery. The difference is worth managing deliberately rather than discovering halfway through an acquisition.

##### What was actually measured, on both sides.

With enablecolor = true and the Pi settled near 86 ∘C, the system was fully throttled (throttled=0xe0006, 600 MHz) and yet the critical chain held: wheel odometry at 19.7 Hz, IMU at 82.8 Hz with 14 gaps per 20 s, serial link synchronised throughout. That is a working rover, and it is the regime in which a short acquisition is worth attempting.

Three degrees higher the same configuration collapsed completely. Every firmware topic dropped to 0 Hz, the overrun counter reached 676 751, and serialnode entered a Lost sync loop from which it did not return unaided. §12.15.1 derives why the transition is a cliff rather than a slope: the recovery handshake needs a clean window, and past a threshold it stops finding one.

Do not read the first result as a guarantee “The link survived thermal throttling” is true of the 86 ∘C run and false of the 89 ∘C run. The correct statement is that the margin is about three degrees wide with no headroom for ambient variation. Plan for the collapse, do not assume the survival.

##### The protocol.

- Start cold. Power the Pi and leave the perception stack down for two or three minutes. Starting at 70 ∘C instead of 86 ∘C buys most of the useful window.

- Open the vitals monitor before anything else, in its own terminal, and leave it running for the whole session:

- python3 tools/hotrun_monitor.py        # one line every 20 s

- It prints temperature, throttle word, clock, IMU gap count and wheel-odometry rate side by side, and flags its own stop criteria.

- Abort on any one of three signals, without negotiating with yourself: temperature above 88 ∘C, more than 25 IMU gaps per 20 s, or wheel odometry below 10 Hz. The third is the important one, because it means the serial link is going and control authority with it.

- Keep the run short. Everything measured in this campaign that was worth keeping came from acquisitions of a few minutes. Ambition about duration is what turns a good run into a recovery exercise.

- Record to disk, not to the browser. Use tools/recordtrajectories.sh, which writes a rosbag locally and is indifferent to the web stack. The cockpit’s own buffer is cleared by any backend restart, and a thermal event is precisely the circumstance that causes one.

##### Recovery when it does collapse.

Cycle the serial node alone. Do not restart the stack: the July 22 lesson (§12.9.3) is that the surrounding infrastructure is almost never the culprit, and a full restart costs minutes and destroys the run’s context.

rosservice call /rosmon/start_stop "{node: 'serial_node', ns: '', action: 2}"
sleep 4
rosservice call /rosmon/start_stop "{node: 'serial_node', ns: '', action: 1}"

Expect the topics back within about twenty seconds, and expect the launch wrapper to reapply the SCHEDFIFO priority automatically; verify it with chrt -p <pid> rather than assuming. Then watch the overrun counter:

ssh pi@$ROBOT_HOST 'sudo cat /proc/tty/driver/ttyAMA' | grep -o 'oe:[0-9]*'

If it is still climbing, and at 88 ∘C it will be, the cycle has bought you minutes rather than fixed anything. Use them to save your data, not to start a new run.

##### Why both modes of driving fail together, and what that tells you.

When the serial link dies, manual and autonomous control stop at the same instant. This looks alarming and is in fact the most useful diagnostic signal the platform offers: the two modes share exactly one component downstream of the mission logic, namely /serialnode, the sole subscriber of /cmdvel. A fault that removes both at once is therefore in the serial transport, and you may discard the web interface, the command bus and the finite-state machine without testing any of them. Confirm it in one command:

rostopic info /cmd_vel     # publisher /leo_backend, subscriber /serial_node
rostopic hz /firmware/wheel_states   # 0 Hz => the link, not the logic

### A cron script that destroyed the stack every two minutes

tools/leowatchdog.sh runs from cron every minute and restarts things it believes are dead. It sourced /opt/ros/noetic/setup.bash at line 141, but its two node-presence checks call rosnode at lines 100 and 123. Under cron the PATH is minimal, so rosnode did not exist yet: both checks failed every minute regardless of the robot, their counters reached the 6-tick threshold, and the whole stack was torn down, measured at six restarts in fourteen minutes.

Why this matters to you even though it is fixed: openVINS never had more than two minutes to converge. Every estimator measurement taken before the fix was made on a stack being destroyed mid-run, and had to be discarded: including a “rotation causes divergence” conclusion that evaporated once the stack was stable.

- Check before any measurement campaign:

- tail -5 ~/TOUT/logs/watchdog.log
cat /tmp/leo_zombie_fails /tmp/leo_vins_fails    # both should read 0

- What else it guards, none of which was documented before 2026-07-31. The watchdog is not only a restart loop, and knowing what it does prevents you from fighting it:

  - Drowning probe on both rosbridge ports (2026-07-13, extended to 9443 on 07-31). A real WebSocket handshake against 9090 in clear and 9443 over TLS; a bridge that holds its port but has stopped answering is purged and relaunched. See §17.17.12, and read the TLS warning there before touching it.

  - D455 self-healing with an escalation ladder. A silent camera is first cycled through rosmon; if it stays silent, the watchdog issues a hardware reset via /firmware/resetboard. A persistent counter in /tmp debounces this across ticks so that one bad minute does not trigger a board reset. If you are power-cycling the camera by hand, set the maintenance flag first or the two of you will take turns resetting it.

  - Battery alert bound to the camera fault. Past three consecutive silent minutes it logs the battery voltage alongside the fault, because a rail sagging below roughly 10.5 V drops the USB camera and presents exactly as a software failure. A camera that will not come back is a battery symptom until proven otherwise.

- Suspend the watchdog during any deliberate manipulation:

- touch /tmp/leo_maintenance     # watchdog does nothing while this exists
rm -f /tmp/leo_maintenance     # DO NOT FORGET THIS

- General lesson: a script tested by hand runs with a good PATH. Bugs of this class only appear under cron. Test with env -i PATH=/usr/bin:/bin bash -c ’...’.

### Wheel odometry is a reference, not the truth

The mecanum wheels slip. Measured against a tape measure over 1 m, the wheel odometry over-reports by 39%. It is still useful (it never errs by a factor of fifty, so it catches gross estimator divergence) but it is not ground truth and must never be quoted as accuracy.

Worse, it is asymmetric: the correction factor is 0.9325 turning left and 0.8920 turning right. Decomposing per-direction factors into a symmetric and an antisymmetric part, g=(gL+gR)/2 and δ=(gL−gR)/2, separates the two causes: a sensor scale error is sign-symmetric and lands in g, whereas a mechanical defect reverses with direction and lands in δ. The wheels’ |δ| is ten times the gyroscope’s. Correcting that with one global gain would improve one direction and worsen the other.

### When two independent sensors agree against your reference

A subtle trap, and a genuinely useful piece of reasoning. The first calibration campaign found the gyroscope over-reporting rotation by 7% (g=0.9329), and the wheels over-reporting by 9% (g=0.9123). Wheel-derived heading comes from differential odometry and does not use the gyroscope at all: these are physically independent instruments. MINS and openVINS are not independent witnesses here, since both integrate the same gyroscope.

Two independent sensors agreeing with each other and disagreeing with the reference points at the reference, not at both sensors. The conclusion was therefore not “apply a 7% gyro correction” but “re-measure with a better floor protocol”. The corresponding parameter, ~gyroscale in imusanitizer.py, was added and deliberately left at 1.0, no correction, until three repeated passes in each direction settle the question. It affects MINS and openVINS simultaneously, so an error there propagates everywhere.

### Do not filter an estimator’s output

A cautionary tale, included because it cost a full evening and the mistake is tempting. Believing openVINS diverged during turns, the author added a node that froze its reported position whenever the gyroscope exceeded a threshold.

It made things worse, and the measurement that proved it is simple: with the robot verified stationary (wheel odometry 0.000 m over 30 s), raw openVINS wandered 16.99 m of cumulative path but ended 0.021 m from where it started. It was oscillating around a stable point, not drifting. The guard rejected 12% of the increments of a symmetric oscillation, which does not remove a zero-mean signal: it rectifies it. Its own counters showed 87.10 m of accumulated offset for zero real motion.

- Distinguish cumulative path from net displacement before diagnosing drift. They answer different questions and only the second one is drift.

- Removing samples one-sidedly biases a sum. If you must smooth, use a filter that preserves the mean (a low-pass), never a selective freeze.

- Fix the estimator’s inputs, not its outputs.

### Serial data loss: it is a scheduling problem

Symptom: thousands of checksum errors and wheel odometry collapsing from 19 Hz to 1 Hz. It is not a cable and not the CORE2 board. The Pi’s PL011 UART FIFO overruns when serialnode is not scheduled in time.

ssh pi@$ROBOT_HOST 'sudo grep -o "oe:[0-9]*" /proc/tty/driver/ttyAMA'

Expected: oe:0 and stable. A climbing oe counter is overrun. The fix, already deployed, needs both halves or the node will not start at all: real-time priority on serialnode (chrt -f 25) and a LimitRTPRIO=50 systemd drop-in on leo.service granting permission to request it. Verify with chrt -p <pid>; expect priority 25.

### Configuration files that fail silently

Both estimators parse their YAML with OpenCV’s FileStorage (%YAML:1.0), which aborts on a long comment placed after a value on the same line. The node then starts, subscribes to nothing, and respawns forever: with no error naming the file. This trap was hit twice, the second time after it had already been documented.

- Keep inline comments in these files short, or put them on their own line.

- Validate before restarting anything:

- python3 -c "import cv2; f=cv2.FileStorage('estimator_config.yaml', \
cv2.FILE_STORAGE_READ); print('readable:', f.isOpened()); f.release()"

- Never test a config from /tmp: relative paths inside it resolve differently and produce false negatives.

### Smaller traps, each of which cost real time

- pgrep -f matches your own shell. A command containing the pattern appears in its own search results, so kill $(pgrep -f mynode) can kill the shell running it. Encountered three times. Use a bracket trick (pgrep -f "myno[d]e") or filter by parent.

- kill -0 lies about liveness, because PIDs are recycled. Verify identity with /proc/<pid>/cmdline.

- Timestamps, not arrival times. Measure rates and velocities from header.stamp. Arrival time mixes in network jitter and produces confident nonsense.

- Medians hide tails. A divergence with a median of 0.69 had a worst case of 19.96 in the same data set. Report a high percentile as well; what ruins a trajectory lives in the tail.

- Path length is not comparable across sampling rates. Per-sample noise inflates cumulative length as (σf/v)2 (Eq. [eq:pathinflation]). Two raw lengths at 86 and 80 Hz once read identically while the true lengths differed by 10%. Resample to a common rate first.

- Restart the estimators after changing calibration. Configs are read once. Changing the IMU calibration mid-run once cost 34 minutes of recorded data.

- matlab -batch needs a live licence session. Online licensing fails with error 5001 in batch mode when the account token has lapsed, even while an already-open interactive session keeps working. Test with matlab -batch "disp(’ok’)" before blaming a script.

- The published PDF may be cached. The Cloudflare tunnel served a stale report for hours despite a correct origin header. updatereport.sh now stamps the download link with a version; use the link it prints, not a bare URL.

### The TF tree accepts only one parent per frame

Symptom: intermittent TF_OLD_DATA, poses that jump between two plausible values, and no error anywhere A frame in a tf2 tree has exactly one parent. Nothing enforces this: publish baselink from two nodes and both transforms are accepted, after which lookups resolve to whichever arrived last. The result is not a crash but an estimate that silently alternates between two coherent answers.

The fault is structural, not numerical. On the left, baselink has two parents, so every lookup silently returns one of two coherent answers. Nothing errors; the pose simply alternates. On the right, the single-line change that restores a well-formed tree.

This bit twice on July 30. First, poseselector was publishing with baseframe set to baselink while the URDF chain already supplied basefootprint → baselink, giving baselink two parents. The fix is one line in navigationsupervision.launch:

<param name="base_frame" value="base_footprint"/>

which restores a strictly linear odom → basefootprint → baselink chain. Second, an early revision of carolustfbridge.py published the full body chain in the beacon frame, re-parenting baselink a second time; it now publishes only cameraframe → carolusraw plus the static anchor, and hides the body chain behind a parameter that defaults to false.

Check it in one command:

rosrun tf view_frames && evince frames.pdf
# or, faster: any frame appearing twice as a child is a fault
rosrun tf tf_monitor 2>&1 | head -30

Expect one parent per frame and no TFOLDDATA in the logs. Do this before trusting any pose, because the failure is invisible in the numbers: they stay plausible.

### The differential-drive angular multiplier, and where it is not

The firmware applies a scalar to every angular velocity command it receives:

ωapplied=kω⋅ωcommanded,  kω=1.76.

Together with the wheel radius (0.0625 m) and separation (0.358 m), this is what converts /cmdvel into wheel setpoints, so it sits directly in the yaw-error path: a rover that consistently over- or under-rotates by a fixed ratio is far more likely to be showing this multiplier than a sensor fault. Read the live values: never the file:

rosparam get /firmware/diff_drive/angular_velocity_multiplier   # 1.76
rosparam get /firmware/diff_drive/wheel_radius                  # 0.0625
rosparam get /firmware/diff_drive/wheel_separation              # 0.358

Two ways to look for this value and find the wrong answer The file is /etc/ros/params.yaml: params, plural; there is no param.yaml. And inside it, the entire diffdrive block including angularvelocitymultiplier: 1.76 is commented out. Those lines document the firmware defaults; they do not set them. A reader who greps the file finds the value inert and may conclude it is not applied, when in fact 1.76 is live because it is the firmware default. Conversely, editing the commented line changes nothing until the # is removed and sudo systemctl restart leo is run.

The cockpit exposes these three values read-only in the Firmware kinematics panel, precisely so that they are visible during a run without anyone being tempted to change them from a browser.

##### The order of operations for a yaw discrepancy.

This is the part a successor most needs, because getting the order wrong produces a rover that appears calibrated and is not. If the robot turns by a consistently wrong amount, there are two places the discrepancy can live, and they are not interchangeable:

$$\begin{equation}
    \underbrace{\omega_{\mathrm{applied}} = k_\omega\,\omega_{\mathrm{cmd}}}
               _{\text{what the robot \emph{does}}}
    \qquad\text{versus}\qquad
    \underbrace{\hat\omega = s_g\,\omega_{\mathrm{true}}}
               _{\text{what the robot \emph{believes}}}
    \label{eq:yawtwoplaces}
    \end{equation}$$

The first is an actuation gain, set by kω= angularvelocitymultiplier. The second is a sensing gain, set by ~gyroscale in the IMU sanitiser. Correct the actuator first, always. The reasoning is that sg is a property of a physical sensor, so changing it to compensate for an actuation error stores a lie in the one channel every downstream estimator trusts. MINS, openVINS and the wheel odometry all consume the sanitised IMU; a wrong sg corrupts them simultaneously and in a way no later measurement can distinguish from a real inertial defect.

- Measure, do not infer. Command a known rotation, for instance 90∘, and measure the physical result with a protractor or floor markings. Use a deliberately unround target such as 87.5∘: a round number invites the operator to report the number they commanded rather than the one they observed.

- Separate the two gains with one extra observation. Compare the commanded angle, the physically observed angle, and the angle the gyroscope reported. If the robot turned too far and the gyroscope agrees with the floor, the fault is in kω. If the robot turned correctly but the gyroscope disagrees, the fault is in sg. Only the second case justifies touching the sensor.

- Adjust kω multiplicatively: kωnew=kω⋅θtarget/θobserved, then re-measure.

- Leave gyroscale at 1.0 until the ground-truth calibration of §17.13.3 has settled the outstanding 7–9 % rotation discrepancy. Until that question is answered, any change to sg is a guess dressed as a correction.

A worked cautionary case is in §12.14.4: an early calibration attempt compared the angle the robot was told to turn against a single entered number, and so measured how badly the platform tracks its own command rather than how badly its sensors track reality. Those are different quantities and only the second is a calibration.

A third gain, easy to forget A yaw error can also come from the wheel geometry itself, since ω depends on the effective radius and the track width. Item 1 of the open-items registry records a measured 1.65 % right/left effective-radius bias, currently masked by a software yaw trim rather than corrected. If kω needs an implausibly large change to fit, suspect the geometry before accepting the number.

### A TLS port that stays open and stops listening

Symptom: the page loads, the layout is perfect, and every button is inert After a ROS master restart, the tunnel’s TLS port 9443 can remain open, it accepts the TCP connection and completes the WebSocket handshake, while no longer being bound to a live rosbridge behind it. Nothing in the browser reports an error. The interface looks connected because, at the socket level, it is.

Recovery, in order of cost: reload the page; if that fails, connect to the local port 9090 instead, which does not traverse the tunnel; if that also fails, the fault is upstream of the transport and you should stop looking at the network.

Diagnosis before you touch anything. Two independent checks separate “the page is broken” from “the bus is quiet”, and they are worth running in that order:

# 1. Does the browser's relay carry ROS traffic at all?
#    Use a topic that is ALWAYS busy. /mission/telemetry publishes ~4 Hz.
rostopic hz /mission/telemetry
# 2. Does a command sent from the page reach the bus?
rostopic echo /mission/command

The first check matters more than it looks. During this session the relay was declared broken on the evidence of an idle /mission/log, which publishes only when something happens: an empty topic proved nothing, and the same test on /mission/telemetry showed 50 frames in 12 s. A silent topic is not a dead link. Choose a topic that is supposed to be noisy before concluding anything.

### “The robot is on but completely invisible”: four faults stacked

An outage on August 18 cost several hours and is worth reproducing in full, because the robot was healthy throughout: uptime 4 days 22 h, battery 12.3 V, 60.8 ∘C with throttled=0x0, and /firmware/wheelstates publishing at 15.8 Hz locally. Four independent faults were stacked, and each one masked the next.

- The rover falls back to its own access point. When wlanint fails to join the campus network it stays UP with no address, and the robot is then reachable only on br0, 10.0.0.1, behind SSID LeoRover-e138. An ICMP sweep of all 8192 addresses of the campus /19, then a TCP sweep for ports 22 and 11311, found nothing at all — and mDNS cannot help, because that guest network blocks client-to-client multicast. The test that settles it in five seconds is nmcli dev wifi list on the workstation: if LeoRover-e138 is there at full strength, stop scanning. Note that 10.0.0.1 — already the default in tools/robotenv.sh — is in fact the most robust address of the three, since br0 is up regardless of which network the robot did or did not manage to join.

- The saved Wi-Fi profile was pinned to a replaced dongle. nmcli connection up LeoRover-e138 failed with a misleading message naming eno1, because connection.interface-name still held the name of the previous USB dongle. Cleared once and for all with nmcli connection modify LeoRover-e138 connection.interface-name "".

- /etc/ros/setup.bash on the robot pins ROSIP="10.200.0.2", the WireGuard address. This is the nastiest of the four: once the rover is reachable over its access point the master answers, rostopic list works and the topics exist, yet every robot-side node advertises a dead address, so no TCPROS connection can be made and everything looks mute. Confirm with lookupNode: an URI of the form http://10.200.0.2:... is the signature. The fix that avoids restarting the robot’s entire stack is a persistent route on the workstation, nmcli connection modify LeoRover-e138 +ipv4.routes "10.200.0.2/32 10.0.0.1"; the robot accepts packets addressed to its own wg0 address arriving on br0 (0 % loss measured).

- Long-lived nodes keep publishing but never re-subscribe. After the network transition serialnode still had every outbound connection alive (wheels, IMU, odometry) and no inbound /cmdvel — the operator’s report was precisely “the commands no longer work” while telemetry looked healthy. A brand-new publisher did not attract it either. rosbridgeserver failed the same way one layer up: the cockpit showed ONLINE, video ran (MJPEG over plain HTTP, independent of rosbridge), and every telemetry-driven field stayed blank — MISSION 00:00 against a backend counting 462 s, and, most visibly, no button selected in either the pose source or the wheel selector. Both were cured by a targeted restart, the same rosmon stop/start the watchdog already performs for the camera.

Two measurement traps met while diagnosing this rostopic hz buffers its output when redirected or piped: it prints nothing while data is flowing perfectly. Always confirm with rostopic echo -n1 (add --noarr for images) before declaring a topic dead — this alone produced a false “camera mute” diagnosis here.

And pkill -f "rosbridgewebsocket" kills its own shell: the pattern appears in that shell’s own command line. Use explicit PIDs.

The decisive end-to-end test, when several unrelated cockpit fields are blank at once, is to open the WebSocket yourself rather than to read JavaScript: connect to ws://localhost:9090 and send

{"op":"subscribe","topic":"/mission/telemetry","type":"std_msgs/String"}

then wait. A 101 Switching Protocols handshake followed by silence means the transport is at fault, not the rendering. When several unrelated fields fail together it is never the view layer.

### Nothing rotated any log, on either machine

Found by the audit of August 18 and worth stating plainly, because it had been quietly true since the beginning: no log in this project had any rotation, neither on the workstation nor on the robot, and nothing capped the systemd journal either.

On the robot that was close to costing the stack. The 30 GB card was at 97 %, 934 MB free: 2.8 GB of uncapped systemd journal, 3.9 GB of ROS logs across four finished sessions, and a single rosmon.log at 783 MB. And a full disk is not a slow degradation here — it kills leo.service, which was still at Restart=no, so the robot would have stayed powered and mute exactly as it did on July 21. Both halves are now fixed: SystemMaxUse=200M in journald.conf, a cap-ros-logs systemd timer truncating ROS logs hourly, and a 20-restart.conf drop-in giving the unit Restart=on-failure with StartLimitBurst=5 over 5 minutes so a config that fails instantly cannot become a restart storm. Result: 97 %→79 %, 934 MB →6.1 GB free. All four files are versioned under tools/robot/, since a card reflash would otherwise lose them without trace.

Two robot-side facts that break the obvious fix cron does not exist on this Pi: /usr/bin/crontab is absent and cron.service is inactive, so a crontab entry fails silently — the first attempt did exactly that. systemd timers, on the other hand, already run there (man-db, apt-daily), which is why the cap is a timer.

And these logs must be truncated, never deleted: rosmon.log is held open continuously by rosmon, so unlinking it would not return the space while the process lives. Its descriptor carries OAPPEND (flags 02402001, checked before touching anything), which is what makes truncate -s 0 safe: the writer resumes at the end of the emptied file, with no sparse hole and no phantom space.

On the workstation the same gap was real but far less urgent (231 GB free): navigationmaster.log had reached 621 MB in five days and takes back ∼​84 MB in a few hours. The guard lives at the top of leowatchdog.sh, which cron already runs every minute — placed before the early exit taken when the robot is unreachable, because that is precisely when those logs grow fastest. It fired on its first pass, reclaiming 230 MB from carolusdetect.log.

### A checklist before any measurement campaign

- vcgencmd measuretemp on the robot, below 80 ∘C, and throttled=0x0.

- tail logs/watchdog.log, no automatic restarts in the last hour.

- IMU gap count over 20 s, zero.

- rostopic hz on the four key topics, all at nominal rate.

- ps -o etime= -p $(pgrep -f runsubscribemsckf), openVINS older than the intended run duration, i.e. it has not respawned.

- Battery on mains, or voltage noted; yaw performance depends on it.

- Clock offset between the two machines, if the robot has been running on its own access point. Measured August 18: the robot ran +0.16 s ahead of the workstation and NTPSynchronized was no — on the fallback AP it reaches no time server, and this Pi has no RTC. The drift was measured over 16.7 min at +12.3 ms (σ=4.4 ms, so significant), i.e. ≈​+1 s per day. Nothing was broken by it at that magnitude — TF resolved 20/20 across the PC/robot boundary and no extrapolation warning appears in any log — but it accumulates for as long as the robot stays off the campus network, and it clears itself the moment wlanint rejoins it. Check it without SSH round-trip noise by reading the age of a robot-stamped message from the workstation:

- rostopic echo -n1 /firmware/wheel_states   # comparer stamp et date du PC

If any of these fails, the data you are about to record will not survive scrutiny. The single most expensive lesson of this campaign is that a measurement taken on an unstable stack is worse than no measurement, because it is believed.

The sections above are the operating manual. Those that follow are the technical reference: where the source comes from, how it is built, and how each configuration tree is organised.

## Where to go next

The platform is stable and instrumented; what it lacks is not more code. This is the author’s ordered recommendation, with the reasoning, so you can disagree with it deliberately rather than by accident.

### Priority 1: re-calibrate the colour camera intrinsics

Cheapest item on this list, and it blocks the headline deliverable. The detector runs with intrinsics calibrated at 1280×720 against a 640×480 stream (§12.15.3); the stored principal point, cx=643.47, lies outside a 640-pixel-wide image. A beacon at the true image centre is reported 26.7∘ off-axis. No rescaling repairs this, because the two configurations differ in field of view and not merely in sampling.

Run the calibration at 640×480, the resolution actually published, and load the result into carolusastrobeerex. Until then, treat every range, bearing and beacon position from this camera as a relative indicator only. Do not publish a dimensional metric derived from it, including the absolute error of Priority 4.

### Priority 2: install active cooling

Not software. The Pi sits at 86 ∘C and is frequency-capped to 600 MHz whenever it is working, and that single fact taints every estimator measurement on the platform (§17.17.1).

An earlier edition of this appendix stated that the software levers were exhausted, on the grounds that disabling the colour stream gave zero thermal improvement. That measurement was invalid: colour was already disabled at driver level, so the test compared a configuration with itself. The colour nodelet was subsequently measured at 185 % CPU (§12.15.1), making it the dominant thermal term. The lever exists, but spending it costs the working-distance beacon detection, which is why the July 30 session chose to keep colour and accept the risk instead. The measured envelope is roughly three degrees wide: the serial link holds at 86 ∘C and collapses at 89 ∘C. A fan is what buys back that margin without giving up colour.

The experiment to run immediately after fitting a fan is the A/B that was never possible: count IMU gaps over 20 s before and after, and count Propagator.cpp:101 aborts over a ten-minute run. If the gaps go to zero, a whole class of “estimator bug” disappears, and several conclusions in Chapter 12 should be re-measured on the healthier machine.

### Priority 3: finish the ground-truth calibration

Three passes per direction, with the floor protocol, and settle the question posed in §17.13.3: is the 7–9% rotation discrepancy in the sensors or in the reference measurement? This is cheap, needs no hardware, and is the only measurement on the platform anchored to physical reality.

Until it is settled, ~gyroscale must stay at 1.0.

### Priority 4: get an absolute reference into a driven run

Every number in Chapter 12 except the tape-measure calibration is mutual divergence: it says whether MINS and openVINS agree, never which is right. The Carolus beacon is the only drift-free anchor available, and it was in view during neither of the two comparison runs.

Drive a loop with the beacon visible. Nothing needs to be written: plottrajectories.m already computes each estimator’s error against the Carolus fix the moment a carolus.csv exists. That single run converts the whole chapter from relative to absolute.

Since July 30 there is a better path than a post-hoc CSV. carolustfbridge.py (§12.15.2) publishes /odominbeacon and /carolusinbeacon, both carrying the robot’s pose in the beacon frame by two independent routes, odometric and optical. Their instantaneous difference is the absolute error, with no Kabsch or Umeyama fit in between, so nothing is absorbed into a fitted transform or a scale parameter. Record both topics in the bag.

Two cautions. The bridge is deliberately in no launch file: start it by hand, so it can never perturb a run you did not intend it to touch. And it is gated on Priority 1: with the present intrinsics its output is self-consistent and metrically meaningless, which is the most dangerous combination a measurement can have, because it looks like data.

### Priority 5: the Kalibr camera–IMU calibration

Item 2 of the open-items registry, never completed, blocked on producing ≳1.5 rad/s of roll and pitch excitation with a rover that has only ever managed 0.08. It requires lifting and tilting the robot by hand in front of the target. Do it after the three above, because a thermally throttled Pi will drop frames during the very manoeuvre the calibration depends on.

### Open questions the author did not resolve

Recorded honestly, because a successor deserves the loose ends rather than a tidy narrative:

- firmwaremessageconverter publishes an altered gyro bias. It reports 6.867 where its own source reads 3.668. No estimator consumes that topic, so it was never chased, but it is a real defect and suggests something wrong in the conversion path.

- ∼20 000 serial checksum errors were logged on the Pi before the real-time fix. The fix cured the rate collapse; whether it cured the checksum errors entirely was never re-measured.

- openVINS’s scale disagrees with itself between runs. One run gave a trajectory 10% shorter than MINS; a different run gave a straight-line distance 10.5× longer than reality. Both were measured properly. This inconsistency is unexplained and is the most interesting open question in the estimator work.

- The wheel odometry’s 4.3% left/right asymmetry is characterised but not corrected. It is mechanical, so it needs a mechanical or model-level answer, not a gain.

- MATLAB’s batch licensing was failing at the time of writing (error 5001), which is why the per-panel figures in this appendix’s companion chapter were produced by cropping rather than re-rendering. Purely an account/network issue, but it blocks the analysis pipeline until fixed.

### Habits worth inheriting

- Measure before believing, and measure the thing itself. Almost every wrong conclusion in this project came from measuring a proxy: arrival time instead of the timestamp, cumulative path instead of net displacement, the median instead of the tail.

- Write down why, not what. Every script here has a header explaining the defect it works around. Six weeks later that comment is the only thing that distinguishes a deliberate workaround from an accident.

- A measurement taken on an unstable system is worse than no measurement, because it will be believed. The watchdog episode (§17.17.3) invalidated two days of estimator conclusions.

- Fix inputs, not outputs. The one time this project filtered an estimator’s output instead of its inputs, the filter became the fault (§17.17.6).

- Keep the fallback path working. Every convenience here has a manual equivalent, the cockpit calibration panel has tools/calibterrain.py, the one-click export has bagtocsv.py. When the convenient path breaks mid-session, and it will, the campaign survives.

## Pose-regulation motion control: Carolus and AprilTag, to an arbitrary (X, Y, Z, θ)

Every autonomous behaviour documented elsewhere in this report (§12.9.2, §12.22.5) centres on a target, it drives the target’s pixel offset toward zero, with no notion of a specific commanded (X,Y,Z,θ) to stop at. This appendix specifies, derives, and implements the missing capability the supervisor requested: given an arbitrary desired ground-frame pose, drive there from an arbitrary start, using either Carolus or AprilTag as the position sensor, through the same ground frame Section 1 already established (§12.21.2, beaconlink).

### Why a three-phase controller, not a single smooth law

A single, continuously-blended control law that drives (x,y,θ) to a goal simultaneously exists in the literature (polar-coordinate feedback, Aicardi et al. 1995) and was evaluated for this appendix. It was not chosen, for a reason worth stating rather than hiding: its stability proof requires a specific coupling between the heading-error gain and the final-orientation-error gain that this report could not independently re-derive with full confidence in the time available, and shipping an unverified stability argument to a rover with a live, unresolved UART fault (§12.17.5) is a worse mistake than shipping a simpler controller with a plain per-phase proof. The controller specified below uses three sequential phases, each individually a single-degree-of-freedom proportional regulation whose convergence is a one-line argument, and mirrors the exact pattern already implemented and field-validated on this lab’s LIMO platform (LimoUltimateMission’s TURN→MOVEL→TURN state machine, §12.21.1), rather than introducing an unvalidated technique where a validated one already exists in the same lab.

### Geometry and control law

Let (x,y,θ) be the current pose in the ground frame (beaconlink) and (xg,yg,θg) the commanded goal (Z is not independently controlled, this platform’s motion is planar, so a requested Z is only meaningful as a height at which the target was sighted, not a controllable degree of freedom, and is not carried into the control law below). Define

Δx=xg−x, Δy=yg−y, ρ=Δx2+Δy2, γ=atan2(Δy,Δx).

Phase 1, TURN_1 (rotate to face the goal). Error e1=wrap(γ−θ)∈(−π,π], control ω=clamp(kθe1, ±ωmax), this project’s own PID class (§12.9.2.1) instantiated with Ki=0 exactly as the existing pidlockalign loop is. A single scalar error driven by proportional feedback toward zero is Lyapunov-trivial: V=12e12, V=e1e1=−kθe12≤0 for kθ>0. Exit: |e1|<εθ.

Phase 2, MOVE_L (drive to the goal, correcting bearing en route). v=vcruise (constant, this platform’s obstacle governor, §12.9.2.2, remains active underneath and unmodified: this phase adds a goal, it does not remove the existing safety layer), with a small proportional bearing correction re-evaluating Eq. [eq:posegeom] every control tick, ω=clamp(kϕ wrap(γ(t)−θ(t)), ±ωcorr), ωcorr≪ωmax so the robot corrects heading without re-entering a full stop-and-turn. Exit: ρ<ερ.

Phase 3, TURN_2 (rotate to the commanded final heading). Error e3=wrap(θg−θ), identical control and convergence argument as Phase 1. Exit: |e3|<εθ, mission complete.

### Sensor front end: Carolus and AprilTag feed the same state machine

The pose (x,y,θ) Eq. [eq:posegeom] consumes is, deliberately, sensor-agnostic: it is whichever of /odominbeacon (carolustfbridge.py, Carolus, §12.22.4) or an equivalent AprilTag-derived ground-frame pose is freshest, following the same strict-priority pattern already verified for centring (§12.22.5) rather than a second, separate arbitration scheme. AprilTag’s own detection message already carries a full 6-DOF pose (d.pose.pose.pose in ontagdetections, currently reprojected to a 2D pixel centre and a scalar distance only, for the centring use case); this appendix’s implementation reads the same message’s full pose and passes it through the identical optical-to-body rotation already derived and verified (Eq. [eq:rob]), rather than a second, independently-tuned conversion.

# catkin_ws/src/leo_navigation/scripts/goto_pose.py -- NEW, this session.
# Three-phase pose regulation (SS D.15.2), sensor-agnostic front end (SS D.15.3).
# NOT wired into any .launch file and NOT added to leo_backend.py's FSM --
# see the trapbox below for why.

class GotoPose:
    def __init__(self, goal_x, goal_y, goal_theta):
        self.goal = (goal_x, goal_y, goal_theta)
        self.phase = "TURN_1"
        self.pid_turn = PID(Kp=K_THETA, Ki=0.0, Kd=0.08,
                             out_min=-OMEGA_MAX, out_max=OMEGA_MAX)

    def update(self, x, y, theta, now):
        gx, gy, gtheta = self.goal
        dx, dy = gx - x, gy - y
        rho = math.hypot(dx, dy)
        gamma = math.atan2(dy, dx)

        if self.phase == "TURN_1":
            e = wrap(gamma - theta)
            omega = self.pid_turn.update(e, now)
            if abs(e) < EPS_THETA:
                self.phase = "MOVE_L"; self.pid_turn.reset()
            return (0.0, omega)

        if self.phase == "MOVE_L":
            e = wrap(gamma - theta)
            omega = clamp(K_PHI * e, -OMEGA_CORR, OMEGA_CORR)
            if rho < EPS_RHO:
                self.phase = "TURN_2"; self.pid_turn.reset()
                return (0.0, 0.0)
            return (V_CRUISE, omega)  # obstacle governor (S:obstacleloop) still applies

        if self.phase == "TURN_2":
            e = wrap(gtheta - theta)
            omega = self.pid_turn.update(e, now)
            if abs(e) < EPS_THETA:
                self.phase = "DONE"
            return (0.0, omega)

        return (0.0, 0.0)  # DONE

Written this session, deliberately not deployed gotopose.py is new, real code, not pseudocode, and every equation in §17.19.2 is verified by hand (each phase’s Lyapunov argument is one line, checked directly, not cited without derivation). It is not added to leobackend.py’s FSM, not referenced from any .launch file, and has not been run against the real robot, for one concrete reason: §12.17.5 documents a live, unresolved UART checksum storm on this exact platform, found the same night this controller was written, capable of silently dropping /firmware/wheelstates. Deploying a new autonomous motion-control path on top of a known-degraded sensor input, before that fault is even diagnosed, would risk producing a field result that measures the fault rather than the controller. The three-phase design (§17.19.1) and the reused, already-verified building blocks (the PID class, the optical-to-body rotation, the strict-priority sensor front end) mean integration, once the UART fault is resolved, is wiring GotoPose into the existing FSM as one more state alongside LOCKBEACON and GOTOBEACON, not new development.

AgileX Robotics. 2021. LIMO Rover ROS Packages Documentation. Dongguan, China. https://github.com/agilexrobotics.

Alur, Rajeev, and David L. Dill. 1994. “A Theory of Timed Automata.” Theoretical Computer Science 126 (2): 183–235.

Bianchi, Giuseppe. 2000. “Performance Analysis of the IEEE 802.11 Distributed Coordination Function.” IEEE Journal on Selected Areas in Communications 18 (3): 535–47.

Bradski, Gary. 2000. “The OpenCV Library.” Dr. Dobb’s Journal of Software Tools. https://opencv.org.

Cabello, Ricardo et al. 2021. Three.js: A JavaScript 3D Library. https://threejs.org.

Crick, Christopher, Graylin Jay, Sarah Osentoski, Benjamin Pitzer, and Odest Chadwicke Jenkins. 2012. “Rosbridge: ROS for Non-ROS Users.” Proc. International Symposium on Experimental Robotics (ISER) (Quebec City, Canada).

Endsley, Mica R. 1988. “Design and Evaluation for Situation Awareness Enhancement.” Proc. Human Factors Society Annual Meeting 32: 97–101.

Endsley, Mica R. 1995. “Toward a Theory of Situation Awareness in Dynamic Systems.” Human Factors 37 (1): 32–64.

Ester, Martin, Hans-Peter Kriegel, Jörg Sander, and Xiaowei Xu. 1996. “A Density-Based Algorithm for Discovering Clusters in Large Spatial Databases with Noise.” Proc. 2nd International Conference on Knowledge Discovery and Data Mining (KDD) (Portland, OR), 226–31.

Fette, Ian, and Alexey Melnikov. 2011. The WebSocket Protocol. https://tools.ietf.org/html/rfc6455.

Furgale, Paul, Joern Rehder, and Roland Siegwart. 2013. “Unified Temporal and Spatial Calibration for Multi-Sensor Systems.” Proc. IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS) (Tokyo, Japan), 1280–86. https://doi.org/10.1109/IROS.2013.6696514.

Garrido-Jurado, Sergio, Rafael Muñoz-Salinas, Francisco J. Madrid-Cuevas, and Manuel J. Marín-Jiménez. 2014. “Automatic Generation and Detection of Highly Reliable Fiducial Markers Under Occlusion.” Pattern Recognition 47 (6): 2280–92.

Geneva, Patrick, Kevin Eckenhoff, Woosik Lee, Yulin Yang, and Guoquan Huang. 2020. “OpenVINS: A Research Platform for Visual-Inertial Estimation.” Proc. IEEE International Conference on Robotics and Automation (ICRA) (Paris, France). https://doi.org/10.1109/ICRA40945.2020.9196524.

GreenSock (GSAP). 2021. GSAP (GreenSock Animation Platform) with ScrollTrigger Plugin. https://greensock.com/scrolltrigger/.

Husarion. 2021. LEO Rover Technical Documentation. Warsaw, Poland. https://docs.leorover.tech.

Intel Corporation. 2021. Intel RealSense Depth Camera D400 Series Product Family Datasheet. Santa Clara, CA. https://www.intelrealsense.com/download/20132.

International Electrotechnical Commission. 2010. Functional Safety of Electrical/Electronic/Programmable Electronic Safety-Related Systems. IEC 61508. IEC.

ISA. 2016. Management of Alarm Systems for the Process Industries. ISA-18.2. ISA.

Kalman, Rudolf E. 1960. “A New Approach to Linear Filtering and Prediction Problems.” Journal of Basic Engineering 82 (1): 35–45.

Kaplan, Edward L., and Paul Meier. 1958. “Nonparametric Estimation from Incomplete Observations.” Journal of the American Statistical Association 53 (282): 457–81.

Lee, Woosik, Patrick Geneva, Chuchu Chen, and Guoquan Huang. 2023. “MINS: Efficient and Robust Multisensor-Aided Inertial Navigation System.” arXiv Preprint arXiv:2309.15390.

Lee, Woosik, Patrick Geneva, Chuchu Chen, and Guoquan Huang. 2025. “MINS: Efficient and Robust Multisensor-Aided Inertial Navigation System.” Journal of Field Robotics, ahead of print. https://doi.org/10.1002/rob.22546.

Matheron, Georges. 1975. Random Sets and Integral Geometry. Wiley.

Mourikis, Anastasios I., and Stergios I. Roumeliotis. 2007. “A Multi-State Constraint Kalman Filter for Vision-Aided Inertial Navigation.” Proc. IEEE International Conference on Robotics and Automation (ICRA) (Rome, Italy), 3565–72. https://doi.org/10.1109/ROBOT.2007.364024.

Mur-Artal, Raúl, and Juan D. Tardós. 2017. “ORB-SLAM2: An Open-Source SLAM System for Monocular, Stereo, and RGB-D Cameras.” IEEE Transactions on Robotics 33 (5): 1255–62.

National Aeronautics and Space Administration. 2013. Software Safety Standard. NASA-STD-8719.13C. NASA.

Niblack, Wayne. 1986. An Introduction to Digital Image Processing. Prentice-Hall.

Olson, Edwin. 2011. “AprilTag: A Robust and Flexible Visual Fiducial System.” Proc. IEEE International Conference on Robotics and Automation (ICRA) (Shanghai, China), 3400–3407.

Open Robotics. 2013. Rosbridge Protocol Specification V2.0. https://github.com/RobotWebTools/rosbridge_suite.

Open Robotics. 2020. ROS Noetic Ninjemys Release Notes and Migration Guide. Open Robotics. https://www.ros.org/reps/rep-0003.html.

OpenCV Contributors. 2023. findChessboardCornersSB: Chessboard Detection Using a Sub-Pixel Befitting Algorithm. https://docs.opencv.org/4.x/d9/d0c/group__calib3d.html.

Otsu, Nobuyuki. 1979. “A Threshold Selection Method from Gray-Level Histograms.” IEEE Transactions on Systems, Man, and Cybernetics 9 (1): 62–66.

Quigley, Morgan, Ken Conley, Brian Gerkey, et al. 2009. “ROS: An Open-Source Robot Operating System.” Proc. ICRA Workshop on Open Source Software (Kobe, Japan).

Sanders, Frederick T., and Ernest J. McCormick. 1993. Human Factors in Engineering and Design. 7th ed. McGraw-Hill.

Sauvola, Jaakko, and Matti Pietikäinen. 2000. “Adaptive Document Image Binarization.” Pattern Recognition 33 (2): 225–36.

Serra, Jean. 1982. Image Analysis and Mathematical Morphology. Academic Press.

Shannon, Claude E. 1948. “A Mathematical Theory of Communication.” Bell System Technical Journal 27: 379–423.

Sobczak, D. P. 1962. Fault Tree Analysis.

Suzuki, Satoshi, and Keiichi Abe. 1985. “Topological Structural Analysis of Digitized Binary Images by Border Following.” Computer Vision, Graphics, and Image Processing 30 (1): 32–46.

Thrun, Sebastian, Wolfram Burgard, and Dieter Fox. 2005. Probabilistic Robotics. MIT Press.

Tukey, John W. 1977. Exploratory Data Analysis. Addison-Wesley.

Umeyama, Shinji. 1991. “Least-Squares Estimation of Transformation Parameters Between Two Point Patterns.” IEEE Transactions on Pattern Analysis and Machine Intelligence 13 (4): 376–80.

Viola, Paul, and Michael Jones. 2001. “Rapid Object Detection Using a Boosted Cascade of Simple Features.” Proc. IEEE Conference on Computer Vision and Pattern Recognition (CVPR) (Kauai, HI), 511–18.

W3C CSS Working Group. 2023. CSS Filter Effects Module Level 2: Backdrop-Filter Property. https://www.w3.org/TR/filter-effects-2/.

Wiener, Norbert. 1949. Extrapolation, Interpolation, and Smoothing of Stationary Time Series. MIT Press.

Wilson, Edwin B. 1927. “Probable Inference, the Law of Succession, and Statistical Inference.” Journal of the American Statistical Association 22 (158): 209–12.

World Wide Web Consortium (W3C). 2018. Web Content Accessibility Guidelines (WCAG) 2.1. https://www.w3.org/TR/WCAG21/.

Xu, Xiaojian, and Victor O. K. Li. 2007. “Enhanced Cascade Implementation of the Exponential Moving Average Filter.” IEEE Signal Processing Letters 14 (10): 657–60.

Zames, George. 1966. “On the Input-Output Stability of Time-Varying Nonlinear Feedback Systems.” IEEE Transactions on Automatic Control 11 (2): 228–38.

Zhang, Zhengyou. 2000. “A Flexible New Technique for Camera Calibration.” IEEE Transactions on Pattern Analysis and Machine Intelligence 22 (11): 1330–34.

Ziegler, John G., and Nathaniel B. Nichols. 1942. “Optimum Settings for Automatic Controllers.” Transactions of the ASME 64: 759–68.